3GPP TSG-RAN WG2 Meeting #95															R2-165278

Gothenburg, Sweden, 22nd – 26th August 2016

Agenda Item:		8.1.4

Souce:	Samsung

Title:					IDC issue in eLAA

Document for:		Discussion and Decision

# 1. Introduction

The hardware conflict issue when UE is configured for LAA operation and user preference is to connect to home AP was discussed in RAN2#93Bis meeting based on [1]. Following was the RAN2 common understanding on the issue [2]:

RAN2 understanding that the UE does not trigger re-establishment when given a LAA configuration that the UE can't use due hardware conflict for LAA and WLAN operations. UE can use the IDC mechanism, if configured, to indicate the problem to the eNB. If IDC is not configured then the only standardised approach is to perform detach/re-attach.

In Rel-14 LAA operation in UL is been specified. Assuming a UE implementation with separate RF componenets for DL/UL LAA operation and WLAN operation, there is possibility of in-device interference (IDC) issue when LAA UL transmission on an unlicensed carrier frequency causes interference to WLAN background scanning for another adjacent unlicensed carrier frequency.

Since configuration of IDC mechanism is upto eNB decision and has NOT yet been manadated in Stage-2 specification there is concern from UE vendors that if IDC indication is not configured then UE performing detach/re-attach with changed capabilities is not an efficient way to resolve the issue.

To address the hardware conflict issue as well as real the IDC problem, we therefore propose mandating the existing IDC mechanism to resolve the issue. By mandating the eNB to configure IDC indication upon either receiving the LAA capability or configuring the UE for LAA operation addresses the user-preference issue (hardware conflict due to shared hardware) and real IDC problem (separate hardware for LAA and WLAN).

Proposal: RAN2 is requested to agree on the stage-2 CR proposing to mandate the configuration of IDC indication for LAA capable UE.

# 2. References

[1] R2-162301, IDC issue for LAA, Samsung, BlackBerry, Intel, Sony.

[2] R2-163343, Chairman’s Notes, RAN2#93Bis meeting.

[3] R2-165281, Stage-2 draftCR, IDC support in eLAA, Samsung.