3GPP TSG-RAN WG2 Meeting #95	R2-165467

Gothenburg, Sweden, 22 - 26 August 2016

Agenda item:	8.10.5

Source:	Nokia, Alcatel-Lucent Shanghai Bell

Title:	Shared MBMS Network and Receive Only Mode considerations based on SA2 LSs

WID:	MBMS_LTE_enh2 - Release 14

Document for:	Discussion and Decision

# 1	Introduction

Two LSs were received by RAN2 from SA2 concerning eMBMS Work Item in [1] and [2]. SA2 is requesting RAN2 feedback concerning two functionalities they designed to enhance eMBMS framework, i.e.:

- Shared eMBMS functionality aka. Shared eMBMS Network (SEN)

- Receive Only Mode and Receive Only Mode with independent unicast

In this paper we analyze those solutions from RAN2 perspective in order to provide background for a reply LS.

# 2	Shared eMBMS Network

The first feedback, which is requested from RAN2 concerns Solution 1, which is described in [3]. The LS asks:

“SA2 would like to request feedback from RAN2/RAN3 regarding the feasibility of MBSFN synchronization area spanning within and across PLMNs (Option A) from RAN perspective.”

We will not go into architectural considerations, which is in RAN3 area of expertise (we provide some thoughts in a related RAN3 contribution in [4]), but rather focus on the meaning of an eNB belonging to a specific MBSFN Synchronization Area and its implications for multi-PLMN MBMS service deployment. The definition of MBSFN Synchronization Area can be found in TS 36.300 [5] and we paste it below underlining the fragments we find most important:

“MBSFN Synchronization Area: an area of the network where all eNodeBs can be synchronized and perform MBSFN transmissions. MBSFN Synchronization Areas are capable of supporting one or more MBSFN Areas. On a given frequency layer, a eNodeB can only belong to one MBSFN Synchronization Area. MBSFN Synchronization Areas are independent from the definition of MBMS Service Areas”

First of all eNBs, which contribute to a single MBSFN transmission need to be synchronized with each other. Technically there is nothing that prevents eNBs belonging to different PLMNs to be synchronized. In the real deployments it would most probably require an arrangement between the operators to e.g. use a common synchronization source, but such details are out of scope of 3GPP.

Another point we underlined is that eNB may belong to only one MBSFN Synchronization Area at a time. This would again require pre-arrangement between operators operating particular PLMNs to agree on a single MBSFN Synchronisation Area Id to be used for their eNBs configuration, but there are no technical impediments to do so.

Observation 1: From RAN2 perspective MBSFN synchronization area spanning within and across PLMNs is feasible.

# 3	Receive Only Mode

In their first LS SA2 mentioned some prerequisites, which have to be met for the Receive Only Mode and Receive Only Mode with independent unicast to work:

“For these to be feasible, the UE should be able to discover, select, and obtain the necessary system information, including the support for above modes, from the eMBMS cell without any signalling over unicast cells. In addition, a mechanism to prohibit legacy UEs trying to acquire and camping on this cell should be supported.”

In their second LS they put additional pressure on the concern they have about the “impact on the UE radio performance and other issues” of requiring “Receive Only Mode UE to read the broadcast channel to verify if the broadcast TMGI matches the pre-configured one”.

What has to be noted before exploring the topic further is that eMBMS service is in fact “receive only” from RAN perspective already. It is already supported for UEs in RRC IDLE mode and all the information required to receive it are broadcasted inside System Information and broadcast control channels (MCCH). The only procedures requiring unicast communications with the network from RAN perspective are MBMS Interest Indication and Counting Procedure, but these are not required for reception of the MBMS service.

Therefore, the requirements to connect to the network can be only imposed by higher layers, e.g. to perform authentication procedure (but as we indicated in our paper to the previous meeting (see [6]) this procedure is RAN transparent). We also understand that Receive Only Mode UEs will be pre-authenticated and be pre-configured with all the information required for them to receive the eMBMS service, which according to [7] will be PLMN ID(s) that provide the eMBMS service, TMGI(s), RAN specific information. This information is sufficient for the UEs to follow the exactly same procedures as legacy eMBMS UEs, i.e. they can combine it with the information signalled in SIB15 and SIB13 to choose the proper frequency and receive the service of their interest. We assume that Receive Only Mode UE will be pre-configured with the information (TMGIs) only of the services, which can be received by such UEs and do not see any need for additional “Receive Only Mode support” indication from the network. Receive Only Mode UEs will have to perform the same procedures as any other MBMS UE and if these procedures are found inefficient, then this is a broader issue.

Observation 2: Receive Only Mode UEs can reuse the same procedures as legacy eMBMS UEs and there are no additional inefficiencies compared to legacy UEs behaviour from RAN perspective. There is no need for additional indication from RAN about Receive Only Mode support.

Another issue, which is mentioned in the LS is that “a mechanism to prohibit legacy UEs trying to acquire and camping on this cell should be supported.” First of all, in our opinion there is no problem of supporting Receive Only Mode on legacy MBMS carriers from RAN perspective, since as mentioned above the UEs can follow legacy procedures and it is only the matter of proper configuration of UEs and network. In consequence we think that for Receive Only Mode itself, there is no need to prohibit legacy UEs from camping on a cell delivering such service. However it seems that SA2 is assuming that Receive Only Mode will be supported solely on dedicated/standalone MBMS carriers, where in our opinion such mechanism is needed as we analyse in [8]. But once again, this is not caused by Receive Only Mode support, but for other reasons described in that paper.

Observation 3: There is no need to prohibit legacy UEs from camping on a cell providing Receive Only Mode MBMS service. It may be required to prohibit legacy UEs from camping on further enhanced MBMS carrier for other reasons, which is under RAN2 study.

# 4	Summary

In this contribution we analyzed the solutions and questions mentioned in LSs received from SA2 concerning Shared eMBMS Network and Receive Only Mode operation for eMBMS. We observed the following:

Observation 1: From RAN2 perspective MBSFN synchronization area spanning within and across PLMNs is feasible.

Observation 2: Receive Only Mode UEs can reuse the same procedures as legacy eMBMS UEs and there are no additional inefficiencies compared to legacy UEs behaviour from RAN perspective. There is no need of additional indication from RAN about Receive Only Mode support.

Observation 3: There is no need to prohibit legacy UEs from camping on a cell providing Receive Only Mode MBMS service. It may be required to prohibit legacy UEs from camping on further enhanced MBMS carrier for other reasons, which is under RAN2 study.

Based on these observations we propose a reply LS in [9].

# References

- R2-164640 (S2-163212), LS out on RAN impacts from enTV study of SA WG2, SA2

- R2-164621 (S2-164141), LS on RAN impacts from Receive Only Mode UE, SA2

- S2-162944, Correction/update to Solution 1, Shared MBMS, Ericsson, Qualcomm

- R3-161657, Discussion on RAN3 impacts from enTV study of SA WG2, Nokia, Alcatel-Lucent Shanghai Bell

- 3GPP TS 36.300, Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network; Overall description; Stage 2

- R2-163534, Non-authenticated UEs support, Nokia, Alcatel-Lucent Shanghai Bell

- 3GPP TR 23.746 V0.2.0, Study on System Architecture Enhancements to eMBMS for Television Video Service

- R2-165463, Impact of additional MBSFN sub-frames support, Nokia, Alcatel-Lucent Shanghai Bell

- R2-165469, [DRAFT] Reply LS on RAN impacts of Shared MBMS Network and Receive Only Mode in eMBMS, Nokia