3GPP TSG-RAN WG2 Meeting #95	R2-165469

Gothenburg, Sweden, 22 - 26 August 2016

Title:	[DRAFT] Reply LS on RAN impacts of Shared MBMS Network and Receive Only Mode in eMBMS

Response to:	LS (S2-163212) on “LS out on RAN impacts from enTV study of SA2” from SA2

LS (S2-164141) on “LS on RAN impacts from Receive Only Mode UE” from SA2

Release:	Rel-14

Work Item:	SA2: Study on System Architecture Enhancements to eMBMS for TV Video Service

RAN2: eMBMS enhancements for LTE

Source:	Nokia [to be RAN2]

To:	SA2, RAN3

Cc:	-

Contact Person:

#### Name:	Dawid Koziol

Tel. Number:

E-mail Address:	dawid.koziol@nokia.com

Attachments:	-

1. Overall Description:

RAN2 would like to thank SA2 for their LSs concerning RAN impacts of their enTV study provided in S2-163212 and S2-164141. RAN2 has analysed the mentioned issues and would like to provide the following feedback:

For “SA2 Requested feedback 1 (RAN2/RAN3):

SA2 would like to request feedback from RAN2/RAN3 regarding the feasibility of MBSFN synchronization area spanning within and across PLMNs (Option A) from RAN perspective.” from LS S2-163212

From RAN2 perspective MBSFN synchronization area spanning within and across PLMNs is feasible. It has to be noted that two conditions need to be met:

- eNBs need to belong to only one MBSFN Synchronization Area at a time meaning that eNBs belonging to different PLMNs need to be configured with the same MBSFN Synchronization Area ID

- eNBs belonging to different PLMNs need to be synchronized with each other, which RAN2 finds technically feasible, but the details of how to achieve that are out of 3GPP scope

For “SA2 request (RAN2):

SA2 would like to request feedback on the feasibility of  Receive Only Mode and Receive Only Mode with independent unicast from RAN perspective.” from LS S2-163212 and for feedback requested in LS S2-164141

From RAN perspective Receive Only Mode UEs can reuse the same procedures as legacy eMBMS UEs and there are no additional inefficiencies compared to legacy UEs behaviour. There is no need of additional indication from RAN about Receive Only Mode support. There is no need to prohibit legacy UEs from camping on a cell providing MBMS services to Receive Only Mode MBMS UEs. It may be required to prohibit legacy UEs from camping on further enhanced MBMS carrier for other reasons, not related to Receive Only Mode operation, which is under RAN2 study as part of “eMBMS enhancements for LTE” Work Item.

2. Actions:

To SA2, RAN3 groups.

ACTION: 	RAN2 respectfully asks SA2 and RAN3 to take the above information into account in their further work within related Work Items.

3. Date of Next TSG-RAN2 Meetings:

TSG-RAN WG2 Meeting #95bis	10th – 14th October 2016	Kaohsiung, Taiwan

TSG-RAN WG2 Meeting #96	14th – 18th November 2016	Reno, Nevada, US