3GPP TSG-RAN WG2 Meeting #95bis	     R2-166501

Kaohsiung, Taiwan, October 10- 14, 2016               	(Update of R2-165311)

Agenda item:	 9.4

Source:	Samsung

Title:	Co-existence scenarios for LTE and NR

Document for:	Discussion & Decision

# Introduction

In last RAN plenary requirements for co-existence and interworking with legacy RATs were agreed in RAN TR 38.913 [1]. RAN1 and RAN2 need to study the co-existence scenarios for LTE RAT and NR RAT to address the requirements captured in [1]. Some main requirements for co-existence with LTE as re-produced below:

Co-existence with LTE The New RAT should be able to support flexible allocation of resources (e.g. time, frequency) between the new RAT and LTE operating in the same block of spectrum (with possible bandwidths overlap). The solution should work whether LTE is supported by the same base station as the New RAT or the two RATs are supported by different base stations. The solution should not affect backward compatibility with legacy LTE terminals. The New RAT shall be able to co-exist with LTE on adjacent spectrum in the same band in the same geographical area, including collocated deployments of the same or different operators. These requirements are applicable for paired and unpaired spectrum.

In this contribution we want to take a first look at co-existence scenarios and judge which scenarios could be easily supported with existing LTE frameworks and which other scenarios need further study to develop solutions for efficient co-existence.

# Discussion

We assume that there are two main deployments which NR needs to support:

- NR-standalone deployments

A new operator has green field deployment of NR RAT connected to NR core for providing service. Since the operator does not have legacy LTE UEs to serve, there is no co-existence issue for such standalone green field NR deployment. A legacy operator deploys NR RAT connected to NR core for providing service. NR capable UE performs idle mode and connected mode operations in NR RAT. If operator has legacy network then there may be LTE RAT connected to EPC and/or NR core serving legacy LTE UEs in overlapped NR coverage, resulting in following two co-existence scenarios:

Scenario 1: NR Cells and LTE Cells are served by separate carrier frequencies:

As shown in Figure 1 operator has two separate carriers for LTE and NR. In such scenario UE having both LTE and NR radio capability will experience inter-frequency handover between NR RAT and LTE RAT if there is a restriction of single UL from UE perspective. Since there is dedicated carrier frequency which keeps serving the legacy LTE UEs, apart from proper coverage planning and appropriate frequency planning, we do not anticipate major technical issues that need further standardization other than inter-RAT handover and inter-RAT cell-reselection to support this co-existence scenario. During initial deployment since NR traffic demand is expected to be low a smaller bandwidth may be allocated to NR (X2 < X1) as shown in Figure 1. As LTE traffic demand starts becoming less when NR penetration starts to grow then higher bandwidth can be allocated to NR. However, such semi-static allocation of frequency resources is inflexible and inefficient.

NR on F1

LTE on F2

X1 MHz

X2 MHz

When LTE is being phased out or LTE traffic demand becomes low

LTE on F1

NR on F2

X1 MHz

X2 MHz

When NR is initially deployed or NR traffic demand is low

Figure 1

Scenario 2: NR Cells and LTE Cells are served by same carrier frequency:

To avoid the inflexibility of Scenario 1 operator would prefer to pool all the frequency resources together and dynamically allocate resources towards LTE traffic and NR traffic. This is a likely scenario during initial NR deployments which allows for smooth technology migration from LTE to NR without the burden of additional spectrum allocation. For this co-existence scenario we anticipate couple of technical challenges that need to be overcome by investigating mechanisms that allow legacy LTE UEs and new NR capable UE to be served on the same carrier frequency.  In this scenario LTE Cells and NR Cells are different in overlapping coverage areas but served by the same carrier frequency as shown in Figure 2. This deployment scenario resembles co-channel HetNet deployment of LTE and NR. Interference co-ordination and interference avoidance techniques need to be employed to support such co-existence scenario.

X1 MHz

Separate LTE Cell and NR Cell on same frequency

NR Cell#2 on F1

LTE Cell#1 on F1

Figure 2

Existing LTE mechanisms like TDM ICIC and Almost blank subframes (ABS) can be applied on the LTE cells to protect co-channel transmissions from neighbouring NR cells. This also includes dynamic resource adaptation between LTE and NR depending on respective traffic demand. There may be other possibilities in this scenario depending on LTE carrier aggregation (CA) deployment as shown in Figure 3. In such situation LTE and NR co-existence would need to be supported on LTE PCell carrier i.e. F1 or LTE SCell carrier i.e. F2.

Figure 3

- Non-standalone deployments based on LTE+NR aggregation also have two scenarios:

Scenario 3: Deployment where LTE RAT acting as MeNB is providing at least the PCell (typically on lower carrier frequency), and in addition one or more NR SCells in SeNB can be configured (typically on higher carrier frequencies).

Scenario 4: Deployment where NR RAT acting as MeNB is providing at least the PCell (eg. on carrier frequency F1), and in addition LTE SCell in SeNB can be configured (eg. on carrier frequency F2) where both F1 and F2 are from lower frequency band.

Figure 4

For a UE having both LTE and NR capability such deployments is another kind of co-existence scenario. Such scenario is expected to be supported with LTE+NR aggregation based on existing frameworks like Dual Connectivity (DC). From a single UE perspective having LTE and NR radio capability; dual connectivity based aggregation is straight forward if the UE requirement of having 2 UL’s is considered. Therefore this contribution does not focus on the co-existence scenario based on LTE+NR aggregation.

Proposal 1: RAN2 is requested to study co-existence of LTE and NR on same carrier or adjacent carrier is same band.

## Scenario 2: LTE and NR on same carrier

As mentioned above, co-existence scenario 2 enables gradual migration from LTE to NR in standalone mode on existing operating frequency. In scenario 2 it should be possible to provide dynamic resource adaptation between LTE and NR in TDM manner depending on the respective traffic demand.

Following existing LTE frameworks based on TDM could be useful to address the co-existence of LTE and NR on the same carrier:

- MBSFN subframe configuration can be used for scheduling NR UEs:

As shown in Figure 5 below subframes 0, 4, 5 and 9 (FDD case) are required for scheduling legacy physical control signals and channels. Other subframes can be used for LTE or NR scheduling depending on respective traffic demand. In case of TDD subframes 0, 1, 2, 5 and 6 cannot be configured as MBSFN subframes. Enhancements allowing for a larger number of MBSFN subframes are being considered in the Rel-14 eMBMS WI. With this enhancement, more resources can be supported for NR operations on the same carrier.

For measurement purposes like RLM, RRM and CSI the LTE UE needs to be configured with measurement restriction patterns where LTE UE can expect transmission of legacy CRS. Measurement restriction patterns are already supported in LTE specifications. How NR UEs are indicated on which subframes they can expect physical control signals, reference signals and data need to be studied during the study item phase. One design option for co-existence of LTE and NR on same carrier would be that the LTE TTI is multiple of NR TTI then resource adaptation in TDM manner may be possible.

Observation 1: The NR numerology discussions in RAN1 need to address the co-existence of LTE and NR in scenario 2.

Figure 5

- In case of TDD, the LTE UL subframes can be used for scheduling NR UEs

As shown in Figure 6 below some UL subframes of existing UL heavy TDD configurations can be used for scheduling NR UEs. Legacy LTE UEs perform measurements only on DL and special subframes of the indicated TDD configuration. For dynamic adaptation of LTE and NR traffic existing eIMTA framework can be exploited. Based on the eIMTA command legacy LTE UE performs PDCCH monitoring and applying the DL/UL reference configurations it performs HARQ operations in DL and UL. For NR UEs it is not only important to know the scheduled subframes and in which subframes to look reference signals for measurement purpose but also how to perform HARQ operations for DL and UL.

Figure 6

Observation 2: Regardless of MBSFN subframe configuration or utilization of LTE UL subframes, how to configure the subframes where NR UE can expect physical signals, reference signals for measurement purpose and data scheduling need to be studied in RAN1 and RAN2. Further, how to support HARQ operations for NR UE need to be studied.

- LTE-NR coexistence using LAA operations

Other existing LTE frameworks like LAA operation can also be studied for LTE and NR co-existence on same carrier frequency. LTE-NR coexistence on unlicensed bands can be supported without any specification support as long as NR is designed to operate on unlicensed bands. However, currently LAA operation is restricted to SCell from unlicensed band. In LAA operation, CRS is not transmitted when there is no data transmission. If LTE’s LAA operations are extended to licensed bands, eNB would be able to dynamically control the ON/OFF switching of LTE transmission even on licensed bands. With this specification support, it would be possible to dynamically switch between NR and LTE transmissions in the time domain.

Proposal 2: RAN2 is requested to consider existing LTE frameworks (eg MBMS subframes, TDD UL subframes, ABS subframes etc) to study co-existence of LTE and NR on same carrier in addition to mechanisms that need to be supported in NR.

# Conclusion

We conclude the contribution with following observations and proposals:

Observation 1: The NR numerology discussions in RAN1 need to address the co-existence of LTE and NR in scenario 2.

Observation 2: Regardless of MBSFN subframe configuration or utilization of LTE UL subframes, how to configure the subframes where NR UE can expect physical signals, reference signals for measurement purpose and data scheduling need to be studied in RAN1 and RAN2. Further, how to support HARQ operations for NR UE need to be studied.

Proposal 1: RAN2 is requested to study co-existence of LTE and NR on same carrier or adjacent carrier is same band.

Proposal 2: RAN2 is requested to consider existing LTE frameworks (eg MBMS subframes, TDD UL subframes, ABS subframes etc) to study co-existence of LTE and NR on same carrier in addition to mechanisms that need to be supported in NR.

# References

[1] RP-161927, 3GPP TR 38.913 V0.4.0 1 (2016-0609).