3GPP TSG-RAN WG2 Meeting #95bis	R2-167153

Kaohsiung, Taiwan, October 10 – October 14, 2016

# Agenda Item: 8.5.1

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.360 | CR | YYYY | rev | - | Current version: | 13.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | LWAAP running CR for LWA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics Inc. (LWAAP rapporteur) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_WLAN_aggr-Core |  |  |  |  |  |  | Date: |  | 2016-10-10 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | Intention of this CR is to capture agreements made for enhanced LWA in Rel-14. The main agreements potentially impacting LWAAP specification are listed below. In RAN2#93bis => In Rel-14 LWA, multiple UL LWA bearers per UE on WLAN link should be supported. => LWAAP layer is also used to support UL LWA bearer transfer for Rel-14 LWA. => It is proposed to adopt figure 3 as the updated overall protocol model for LWAAP in TS36.360 |  |  |  |  |  |  |  |  |
| Summary of change: |  | To be filled in later |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | LWAAP specification does not support UL LWA. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | To be filled in later |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

1	Scope

The present document specifies the E-UTRA LTE-WLAN Aggregation Adaptation Protocol (LWAAP).

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 36.300: "E-UTRA and E-UTRAN Overall Description; Stage 2".

[3]	3GPP TS 36.323: "E-UTRA PDCP specification".

[4]	3GPP TS 36.331: "E-UTRA RRC Protocol specification".

3	Definitions, symbols and abbreviations

3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

LWA bearer: a bearer used in LTE-WLAN Aggregation, as defined in TS 36.300 [2].

3.2	Abbreviations

For the purposes of the present document, the following abbreviations apply:

DL	DownLink

DRB	Data Radio Bearer

eNB	E-UTRAN Node B

E-UTRA	Evolved UMTS Terrestrial Radio Access

E-UTRAN	Evolved UMTS Terrestrial Radio Access Network

LWAAP	LTE-WLAN Aggregation Adaptation Protocol

PDCP	Packet Data Convergence Protocol

PDU	Protocol Data Unit

RRC	Radio Resource Control

SAP	Service Access Point

SDU	Service Data Unit

UE	User Equipment

UL	UpLink

4	General

4.1	Introduction

The objective is to describe the LWAAP architecture and the LWAAP entities from a functional point of view.

4.2	LWAAP architecture

4.2.1	LWAAP entities

The description in this sub clause is a model and does not specify or restrict implementations.

RRC is generally in control of the LWAAP configuration.

Functions of the LWAAP sublayer are performed by LWAAP entities. For an LWAAP entity configured at the eNB, there is a peer LWAAP entity configured at the UE and vice versa. For all LWA bearers, there is one LWAAP entity in the eNB and one LWAAP entity in the UE.

An LWAAP entity receives/delivers LWAAP SDUs from/to upper layers (i.e. PDCP) and sends/receives LWAAP PDUs to/from its peer LWAAP entity via WLAN:

-	At the transmitting side, when an LWAAP entity receives an LWAAP SDU from upper layers, it constructs the corresponding LWAAP PDU and delivers it to lower layers;

-	At the receiving side, when an LWAAP entity receives an LWAAP PDU from lower layers, it reassembles the corresponding LWAAP SDU and delivers it to upper layers.

Figure 4.2.1-1 illustrates the overview model of the LWAAP sublayer.

Figure 4.2.1-1: Overview model of the LWAAP sublayer

An LWAAP entity delivers/receives the following LWAAP PDU to/from a lower layer entity:

-	LWAAP data PDU.

4.3	Services

4.3.1	Services provided to upper layers

The following services are provided by LWAAP to upper layers (i.e. PDCP):

-	transfer of user plane data;

4.3.2	Services expected from lower layers

The following services are expected by LWAAP from lower layers:

-	transfer of user plane data;

4.4	Functions

The following functions are supported by the LWAAP sublayer:

-	transfer of user plane data;

-	identification of the LWA bearer to which the LWAAP SDU belongs.

5	Procedures

5.1	Data transfer procedures

5.1.1	DL data transfer procedures

When receiving an LWAAP data PDU from lower layers, the LWAAP entity in the UE shall:

-	identify the upper layer entity to which the LWAAP SDU is destined based on the DRB ID included in the LWAAP header;

-	reassemble the LWAAP SDU from the LWAAP data PDU by removing the LWAAP header from the LWAAP data PDU;

-	deliver the reassembled LWAAP SDU to the upper layer entity identified by the DRB ID.

5.1.2	UL data transfer procedures

When receiving an LWAAP data SDU from upper layers, the LWAAP entity in the UE shall:

-	identify the DRB ID of the upper layer entity from which the LWAAP SDU is received;

-	assemble the LWAAP data PDU from the LWAAP SDU by adding the LWAAP header to the LWAAP SDU;

-	deliver the assembled LWAAP data PDU to the lower layers.

5.2	Handling of unknown, unforeseen and erroneous protocol data

When an LWAAP entity receives an LWAAP PDU that contains reserved or invalid values, the LWAAP entity shall:

-	discard the received PDU.

6	Protocol data units, formats and parameters

6.1	Protocol data units

6.1.1	General

An LWAAP PDU is a bit string that is byte aligned (i.e. multiple of 8 bits) in length. In the figures in subclause 6.1, bit strings are represented by tables in which the most significant bit is the leftmost bit of the first line of the table, the least significant bit is the rightmost bit on the last line of the table, and more generally the bit string is to be read from left to right and then in the reading order of the lines. The bit order of each parameter field within an LWAAP PDU is represented with the first and most significant bit in the leftmost bit and the last and least significant bit in the rightmost bit.

An LWAAP SDU is a bit string that is byte aligned (i.e. multiple of 8 bits) in length. An LWAAP SDU is included into an LWAAP PDU from the first bit onward.

The LWAAP entity shall ignore the value of Reserved bits in downlink LWAAP PDUs.

Only one type of LWAAP PDU is defined, i.e. LWAAP data PDU.

6.1.2	LWAAP data PDU

An LWAAP PDU consists of an LWAAP header and a Data field, as described in Figure 6.1.2-1.

An LWAAP header is one byte, and consists of the four header fields R/R/R/DRBID.

A Data field is of variable size, and consists of an LWAAP SDU.

Figure 6.1.2-1: LWAAP data PDU

6.2	Formats and parameters

6.2.1	LWAAP header

The LWAAP header consists of the following fields:

-	R: Reserved bit, set to "0". The R field size is 1 bit;

-	DRBID: The Data Radio Bearer ID field indicates the RRC configured DRB identity to which the corresponding LWAAP SDU belongs. The DRBID field size is 5 bits.