3GPP TSG-RAN2 Meeting #96	R2-167637

# Reno, Nevada, United States, 14th Nov 2016 - 18th Nov 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 25.331 | CR | 5892 | rev | - | Current version: | 14.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of HS-SCCH DRX feature in enhanced Cell Fach |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | FACH_DTXDRX-Core |  |  |  |  |  | Date: |  |  | 2016-11-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | To introduce UE procedures on reception of an HS-SCCH order during the HS-SCCH DRX operation in CELL FACH |  |  |  |  |  |  |  |  |
| Summary of change: |  | Identifies actions related to HS_SCCH_DRX_STATUS and related operations to enable DRX with HS-SCCH in CELL FACH |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification for HS-SCCH DRX operation will be incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.1.6, 8.2.2.3, 8.3.1.6, 8.4.1.6.2, 8.4.1.6.3, 8.4.1.9.2, 8.4.1.9.3, 8.5, 8.6.6,10.2.7a,10.2.8, 10.2.27, 10.2.60, 10.3.3.25, 10.3.6, 11.2, 11.3, 13.1, 13.4.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | 25.212, 25.214, 25.300, 25.306, 25.308 |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

****************************************** TEXT START *****************************************

#### 8.1.1.6	Actions upon reception of system information blocks

The UE may use the scheduling information included within the master information block and the scheduling blocks to locate each system information block to be acquired. For System Information Block type 15.1bis, 15.2bis, 15.2ter, 15.3bis, 15.6, 15.7 and 15.8 the scheduling information shall be used to associate a system information block with a GNSS.

The UE should only expect one occurrence of the scheduling information for a system information block in the master information block and any of the scheduling blocks except for System Information Block type 16, System Information Block type 15.2 and System Information Block type 15.3, which may have multiple occurrences. System Information Blocks 15.1bis, 15.6, 15.7 and 15.8 have one occurrence for each GANSS supported while System Information Blocks 15.2bis, 15.2ter and 15.3bis may have multiple occurrences for each GANSS. However, to enable future introduction of new system information blocks, the UE shall also be able to receive system information blocks other than the ones indicated within the scheduling information. The UE may ignore contents of such system information block.

If the UE:

1>	receives a system information block in a position according to the scheduling information for the system information block; and

1>	this system information block uses a value tag; or

1>	this system information block uses a value tag and configuration or multiple occurrence identity:

the UE shall:

1>	store the content of the system information block together with the value of its value tag or the values of configuration and multiple occurrence identity and the associated value tag in the scheduling information for the system information block; and

1>	consider the content of the system information block valid until, if used, the value tag in the scheduling information for the system information block is changed or at most for 6 hours after reception.

If the UE:

1>	receives a system information block in a position according to the scheduling information for the system information block; and

1>	this system information block does not use a value tag according to the system information block type:

the UE shall:

1>	store the content of the system information block;

1>	if HS_SCCH_DRX_CELL_FACH_STATUS is set to TRUE and if the system information block corresponds to System Information Block type 7; or

1>	if HS_DSCH_DRX_CELL_FACH_2CYCLE_STATUS is set to TRUE and if the system information block corresponds to System Information Block type 7; or

1>	if HS_DSCH_DRX_CELL_FACH_STATUS is set to TRUE and if the system information block corresponds to System Information Block type 7:

2>	start an expiration timer using a value set to 800.

1>	else:

2>	start an expiration timer using a value as defined in Table 8.1.1 for that system information block type.

1>	consider the content of the system information block valid until, the expiration timer expires.

**************************************** TEXT OMITTED ***************************************

#### 8.2.2.3	Reception of RADIO BEARER SETUP or RADIO BEARER RECONFIGURATION or RADIO BEARER RELEASE or TRANSPORT CHANNEL RECONFIGURATION or PHYSICAL CHANNEL RECONFIGURATION message, TTI switch HS-SCCH order or Target cell HS-SCCH order by the UE

The UE shall:

1>	be able to receive any of the following messages:

2>	RADIO BEARER SETUP message; or

2>	RADIO BEARER RECONFIGURATION message; or

2>	RADIO BEARER RELEASE message; or

2>	TRANSPORT CHANNEL RECONFIGURATION message; or

2>	PHYSICAL CHANNEL RECONFIGURATION message;

1>	be able to perform a hard handover and apply physical layer synchronisation procedure A as specified in [29], even if no prior UE measurements have been performed on the target cell and/or frequency.

For FDD: in case a measurement report was triggered by intra frequency event 1d and if the table "Target cell preconfigurations" in the variable TARGET_CELL_PRECONFIGURATION includes the cell that triggered the event, or in case a measurement report was triggered by intra frequency event 1c requesting the serving HS-DSCH cell change and if the table "Target cell preconfigurations" in the variable TARGET_CELL_PRECONFIGURATION includes the target cell, as per the definition of Target cell HS-SCCH order

the UE shall be able to receive:

1>	Target cell HS-SCCH order.

For FDD: in case a filtered UPH report [15] was triggered the UE shall be able to receive:

1>	TTI switch HS-SCCH order.

In case the reconfiguration procedure is used to remove all existing RL(s) in the active set while new RL(s) are established the UE shall:

1>	For FDD:

2>	if the UE has a pending "TGPS reconfiguration CFN" at the activation time received in the reconfiguration message and the reconfiguration requests a timing re-initialised hard handover (see subclause 8.3.5.1), the UE may:

3>	abort the pending CM activation;

3>	set the CM_PATTERN_ACTIVATION_ABORTED to TRUE.

2>	otherwise:

3>	set the CM_PATTERN_ACTIVATION_ABORTED to FALSE.

If the UE will enter a target RRC state as defined in the OTHER_STATE_CONFIGURATION variable after successful transmission of the SIGNALLING CONNECTION RELEASE INDICATION message; or

If the UE receives:

-	a RADIO BEARER SETUP message; or

-	a RADIO BEARER RECONFIGURATION message; or

-	a RADIO BEARER RELEASE message; or

-	a TRANSPORT CHANNEL RECONFIGURATION message; or

-	a PHYSICAL CHANNEL RECONFIGURATION message; or

-	a TTI switch HS-SCCH order; or

-	a Target cell HS-SCCH order

**************************************** TEXT OMITTED ***************************************

If after state transition the UE enters CELL_FACH state, the UE shall, after the state transition:

1>	if the UE was in CELL_DCH state upon reception of the reconfiguration message:

2>	clear the variable E_RNTI.

1>	if the IE "new Primary E-RNTI" is included in the received reconfiguration message:

2>	store the "new Primary E-RNTI" according to subclause 8.6.3.14.

1>	determine the value for the READY_FOR_COMMON_EDCH variable and take the corresponding actions as described in subclause 8.5.47;

1>	determine the value for the COMMON_E_DCH_TRANSMISSION variable and take the corresponding actions as described in subclause 8.5.46;

1>	for FDD, determine the value for the READY_FOR_COMMON_ERGCH variable and take the corresponding actions as described in subclause 8.5.75;

1>	for 1.28 Mcps TDD, determine the value for the MU_MIMO_STATUS variable and take the corresponding actions as described in subclause 8.5.61;

1>	if the IE "Frequency info" is included in the received reconfiguration message:

2>	select a suitable UTRA cell according to [4] on that frequency;

2>	if the UE finds a suitable UTRA cell on that frequency:

3>	if the received reconfiguration message included the IE "Primary CPICH info" (for FDD) or "Primary CCPCH info" (for TDD), and the UE selects another cell than indicated by this IE or the received reconfiguration message did not include the IE "Primary CPICH info" (for FDD) or "Primary CCPCH info" (for TDD):

4>	initiate a cell update procedure according to subclause 8.3.1 using the cause "Cell reselection";

4>	when the cell update procedure completed successfully:

5>	if the UE is in CELL_PCH or URA_PCH state:

6>	if variable READY_FOR_COMMON_EDCH is set to TRUE:

7>	move to CELL_FACH state and proceed as below.

6>	else:

7>	if variable H_RNTI and variable C_RNTI are set:

8>	proceed as below.

7>	else:

8>	initiate a cell update procedure according to subclause 8.3.1 using the cause "Uplink data transmission" and proceed as below.

3>	else:

4>	for 1.28Mcps TDD, decide the working frequency according to subclause 8.6.6.1.

2>	else, if the UE can not find a suitable UTRA cell on that frequency but it finds a suitable UTRA cell on another frequency:

3>	initiate a cell update procedure according to subclause 8.3.1 using the cause "Cell reselection";

3>	when the cell update procedure completed successfully:

4>	if the UE is in CELL_PCH or URA_PCH state:

5>	if variable READY_FOR_COMMON_EDCH is set to TRUE:

6>	move to CELL_FACH state and proceed as below.

5>	else:

6>	if variable H_RNTI and variable C_RNTI are set:

7>	proceed as below.

6>	else:

7>	initiate a cell update procedure according to subclause 8.3.1 using the cause "Uplink data transmission" and proceed as below.

1>	if the IE "Frequency info" is not included in the received reconfiguration message:

2>	select a suitable UTRA cell according to [4];

2>	if the UE finds a suitable UTRA cell on the current frequency:

3>	if the received reconfiguration message included the IE "Primary CPICH info" (for FDD) or "Primary CCPCH info" (for TDD), and the UE selects another cell than indicated by this IE or the received reconfiguration message did not include the IE "Primary CPICH info" (for FDD) or "Primary CCPCH info" (for TDD):

4>	initiate a cell update procedure according to subclause 8.3.1 using the cause "Cell reselection";

4>	when the cell update procedure completed successfully:

5>	if the UE is in CELL_PCH or URA_PCH state:

6>	if variable READY_FOR_COMMON_EDCH is set to TRUE:

7>	move to CELL_FACH state and proceed as below.

6>	else:

7>	if variable H_RNTI and variable C_RNTI are set:

8>	proceed as below.

7>	else:

8>	initiate a cell update procedure according to subclause 8.3.1 using the cause "Uplink data transmission" and proceed as below.

3>	else:

4>	for 1.28Mcps TDD, decide the working frequency according to subclause 8.6.6.1.

2>	else, if the UE can not find a suitable UTRA cell on the current frequency but it finds a suitable UTRA cell on another frequency:

3>	initiate a cell update procedure according to subclause 8.3.1 using the cause "Cell reselection";

3>	when the cell update procedure completed successfully:

4>	if the UE is in CELL_PCH or URA_PCH state:

5>	if variable READY_FOR_COMMON_EDCH is set to TRUE:

6>	move to CELL_FACH state and proceed as below.

5>	else:

6>	if variable H_RNTI and variable C_RNTI are set:

7>	proceed as below.

6>	else:

7>	initiate a cell update procedure according to subclause 8.3.1 using the cause "Uplink data transmission" and proceed as below.

1>	start timer T305 using its initial value if timer T305 is not running and if periodical update has been configured by T305 in the IE "UE Timers and constants in connected mode" set to any other value than "infinity" in the variable TIMERS_AND_CONSTANTS;

1>	if variable READY_FOR_COMMON_EDCH is set to FALSE:

2>	select PRACH according to subclause 8.5.17;

1>	else:

2>	configure the Enhanced Uplink in CELL_FACH state and Idle mode as specified in subclause 8.5.45 for FDD and 8.5.45a for 1.28 Mcps TDD.

1>	for 3.84 Mcps and 1.28 Mcps TDD; or

1>	for FDD, if the UE does not support HS-DSCH reception in CELL_FACH state; or

1>	if the IE "HS-DSCH common system information" is not included in System Information Block type 5 or System Information Block type 5bis; or

1>	for 1.28 Mcps TDD, if the IE "Common E-DCH system info" is not included in System Information Block type 5:

2>	select Secondary CCPCH according to subclause 8.5.19;

2>	use the transport format set given in system information;

2>	take the actions related to the HS_DSCH_RECEPTION_GENERAL variable as described in subclause 8.5.37a.

1>	else:

2>	for FDD if variable READY_FOR_COMMON_EDCH is set to FALSE:

3>	if the RBs have the multiplexing option with transport channel type "HS-DSCH" for the DL and transport channel type "RACH" in the UL; and

3>	if variable H_RNTI and variable C_RNTI are set:

4>	start to receive HS-DSCH according to the procedure in subclause 8.5.36.

3>	else:

4>	clear variable C_RNTI and delete any stored C-RNTI value;

4>	clear variable H_RNTI and delete any stored H-RNTI value;

4>	clear any stored IE "HARQ Info".

2>	else:

3>	if the RBs have the multiplexing option with transport channel type "HS-DSCH" for the DL and transport channel type "E-DCH" in the UL; and

3>	if variable E_RNTI, variable H_RNTI and variable C_RNTI are set:

4>	start to receive HS-DSCH according to the procedure in subclause 8.5.36.

3>	else:

4>	clear variable C_RNTI and delete any stored C-RNTI value;

4>	clear variable H_RNTI and delete any stored H-RNTI value;

4>	clear variable E_RNTI and delete any stored E-RNTI value;

4>	clear any stored IE "HARQ Info".

1>	if the IE "UTRAN DRX cycle length coefficient" is included in the same message:

2>	ignore that IE and stop using DRX.

1>	if the UE supports RAN-assisted WLAN interworking:

2>	stop the timer T330 if it is running;

2>	if the variable WLAN_OFFLOAD_INFO is not empty:

3>	use the information stored in the variable WLAN_OFFLOAD_INFO as specified in [4], and forward it to the upper layers.

2>	else:

3>	use the information stored in the variable SYSTEM_INFO_WLAN_OFFLOAD_INFO as specified in [4] and forward it to the upper layers.

1>	if the contents of the variable C_RNTI is empty:

2>	perform a cell update procedure according to subclause 8.3.1 using the cause "Cell reselection";

2>	when the cell update procedure completed successfully:

3>	if the UE is in CELL_PCH or URA_PCH state:

4>	initiate a cell update procedure according to subclause 8.3.1 using the cause "Uplink data transmission";

4>	proceed as below.

1>	determine the value for the HS_DSCH_DRX_CELL_FACH_STATUS variable and take the corresponding actions as described in subclause 8.6.6.xx.

If the UE was in CELL_FACH state upon reception of the reconfiguration message and remains in CELL_FACH state, the UE shall:

1>	determine the value for the READY_FOR_COMMON_EDCH variable and take the corresponding actions as described in subclause 8.5.47;

1>	determine the value for the COMMON_E_DCH_TRANSMISSION variable and take the corresponding actions as described in subclause 8.5.46;

1>	for FDD, determine the value for the READY_FOR_COMMON_ERGCH variable and take the corresponding actions as described in subclause 8.5.75;

1>	for 1.28 Mcps TDD, determine the value for the MU_MIMO_STATUS variable and take the corresponding actions as described in subclause 8.5.61;

1>	if the IE "Frequency info" is included in the received reconfiguration message:

2>	select a suitable UTRA cell according to [4] on that frequency;

2>	if the UE finds a suitable UTRA cell on that frequency:

3>	if the received reconfiguration message included the IE "Primary CPICH info" (for FDD) or "Primary CCPCH info" (for TDD), and the UE selected another cell than indicated by this IE or the received reconfiguration message did not include the IE "Primary CPICH info" (for FDD) or "Primary CCPCH info" (for TDD):

4>	initiate a cell update procedure according to subclause 8.3.1 using the cause "cell reselection";

4>	when the cell update procedure completed successfully:

5>	if the UE is in CELL_PCH or URA_PCH state:

6>	if variable READY_FOR_COMMON_EDCH is set to TRUE:

7>	move to CELL_FACH state and proceed as below.

6>	else:

7>	if variable H_RNTI and variable C_RNTI are set:

8>	proceed as below.

7>	else:

8>	initiate a cell update procedure according to subclause 8.3.1 using the cause "Uplink data transmission" and proceed as below.

2>	else, if the UE can not find a suitable UTRA cell on that frequency but it finds a suitable UTRA cell on another frequency:

3>	initiate a cell update procedure according to subclause 8.3.1 using the cause "Cell reselection";

3>	when the cell update procedure completed successfully:

4>	if the UE is in CELL_PCH or URA_PCH state:

5>	if variable READY_FOR_COMMON_EDCH is set to TRUE:

6>	move to CELL_FACH state and proceed as below.

5>	else:

6>	if variable H_RNTI and variable C_RNTI are set:

7>	proceed as below.

6>	else:

7>	initiate a cell update procedure according to subclause 8.3.1 using the cause "Uplink data transmission" and proceed as below.

1>	if the IE "Frequency info" is not included in the received reconfiguration message:

2>	if the IE "Primary CPICH info" (for FDD) or "Primary CCPCH info" (for TDD) is included the UE shall either:

3>	ignore the content of the IE "Primary CPICH info" (for FDD) or "Primary CCPCH info" (for TDD) and proceed as below;

2>	or:

3>	if the received reconfiguration message included the IE "Primary CPICH info" (for FDD) or "Primary CPCH info" (for TDD), and it is different from the current cell:

4>	initiate a cell update procedure according to subclause 8.3.1 using the cause "Cell reselection";

4>	when the cell update procedure completed successfully:

5>	if the UE is in CELL_PCH or URA_PCH state:

6>	if variable READY_FOR_COMMON_EDCH is set to TRUE:

7>	move to CELL_FACH state and proceed as below.

6>	else:

7>	if variable H_RNTI and variable C_RNTI are set:

8>	proceed as below.

7>	else:

8>	initiate a cell update procedure according to subclause 8.3.1 using the cause "Uplink data transmission" and proceed as below.

1>	determine the value for the HS_DSCH_DRX_CELL_FACH_STATUS variable and take the corresponding actions as described in subclause 8.6.6.xx.

If after state transition the UE leaves CELL_FACH state, the UE shall:

1>	stop timer T305.

**************************************** TEXT OMITTED ***************************************

#### 8.3.1.6	Reception of the CELL UPDATE CONFIRM/URA UPDATE CONFIRM message by the UE

When the UE receives a CELL UPDATE CONFIRM/URA UPDATE CONFIRM message; and

-	if the message is received on the CCCH, and IE "U-RNTI" is present and has the same value as the variable U_RNTI; or

-	if the message is received on DCCH:

the UE may:

1>	maintain a list of the set of cells to which the UE has Radio Links if the IE "Cell ID" is present.

the UE shall:

1>	stop timer T302;

1>	set variable HS_DSCH_RECEPTION_OF_CCCH_ENABLED to FALSE;

1>	in case of a cell update procedure and the CELL UPDATE CONFIRM message:

2>	includes "RB information elements"; and/or

2>	includes "Transport channel information elements"; and/or

2>	includes "Physical channel information elements"; and

2>	if the variable ORDERED_RECONFIGURATION is set to FALSE:

3>	set the variable ORDERED_RECONFIGURATION to TRUE.

1>	act upon all received information elements as specified in subclause 8.6, unless specified otherwise in the following:

2>	if the IE "Frequency info" is included in the message:

3>	if the IE "RRC State Indicator" is set to the value "CELL_FACH":

4>	select a suitable UTRA cell according to [4] on that frequency;

4>	if the UE finds a suitable UTRA cell on that frequency:

5>	if the received CELL UPDATE CONFIRM message included the IE "Primary CPICH info" (for FDD) or "Primary CCPCH info" (for TDD), and the UE selects another cell than indicated by this IE or the received CELL UPDATE CONFIRM message did not include the IE "Primary CPICH info" (for FDD) or "Primary CCPCH info" (for TDD):

6>	act as specified in subclause 8.3.1.12.

5>	else:

6>	for 1.28Mcps TDD, decide the working frequency according to subclause 8.6.6.1.

4>	else, if the UE can not find a suitable UTRA cell on the indicated frequency but it finds a suitable UTRA cell on another frequency:

5>	act as specified in subclause 8.3.1.12.

3>	if the IE "RRC State Indicator" is set to the value "CELL_PCH" or "URA_PCH":

4>	select a suitable UTRA cell according to [4] on that frequency;

4>	if the UE finds a suitable UTRA cell on that frequency:

5>	if the received CELL UPDATE CONFIRM message included the IE "Primary CPICH info" (for FDD) or "Primary CCPCH info" (for TDD), and the UE in CELL_PCH state selects another cell than indicated by this IE or the received CELL UPDATE CONFIRM message did not include the IE "Primary CPICH info" (for FDD) or "Primary CCPCH info" (for TDD); or

5>	if the received CELL UPDATE CONFIRM message included the IE "URA identity", and the UE in URA_PCH state selects cell being part of another URA than indicated by this IE or the received CELL UPDATE CONFIRM message did not include the IE "URA identity":

6>	act as specified in subclause 8.3.1.12.

4>	else, if the UE can not find a suitable UTRA cell on the indicated frequency but it finds a suitable UTRA cell on another frequency:

5>	act as specified in subclause 8.3.1.12.

3>	if the IE "RRC State Indicator" is set to the value "CELL_DCH":

4>	act on the IE "Frequency info" as specified in subclause 8.6.6.1.

2>	if the IE "RRC State Indicator" is set to the value "CELL_PCH" or "URA_PCH":

3>	if the IE "Wait time"<> 0 is included in the message and the UE supports timer T320:

4>	start timer T320 with value set to the time stated in the IE "wait time".

2>	use the transport channel(s) applicable for the physical channel types that is used; and

2>	if the IE "TFS" is neither included nor previously stored in the UE for that transport channel(s):

3>	use the TFS given in system information.

2>	if none of the TFS stored is compatible with the physical channel:

3>	delete the stored TFS;

3>	use the TFS given in system information.

2>	if the IE "RLC re-establish indicator (RB2, RB3 and RB4)" in the CELL UPDATE CONFIRM message is set to TRUE:

3>	re-establish the RLC entities for signalling radio bearer RB2, signalling radio bearer RB3 and signalling radio bearer RB4 (if established);

3>	clear all entries in the table "Processed transactions" in the variable TRANSACTIONS;

3>	if the value of the IE "Status" in the variable CIPHERING_STATUS of the CN domain stored in the variable LATEST_CONFIGURED_CN_DOMAIN is set to "Started":

4>	set the HFN component of the respective COUNT-C values for AM RLC entities with RB identity 2,RB identity 3 and RB identity 4 (if established) equal to the START value included in the latest transmitted CELL UPDATE message for the CN domain stored in the variable LATEST_CONFIGURED_CN_DOMAIN.

NOTE:	UTRAN should not send a CELL UPDATE CONFIRM message with a target state CELL_PCH or URA_PCH, and including C-RNTI, and setting the IE "RLC re-establish indicator (RB2, RB3 and RB4)" to TRUE, otherwise the UE behaviour is not specified.

2>	if the IE "RLC re-establish indicator (RB5 and upwards)" in the CELL UPDATE CONFIRM message is set to TRUE:

3>	for radio bearers with RB identity 5 and upwards:

4>	re-establish the AM RLC entities;

4>	if the value of the IE "Status" in the variable CIPHERING_STATUS of the CN domain as indicated in the IE "CN domain identity" in the IE "RAB info" in the variable ESTABLISHED_RABS is set to "Started":

5>	set the HFN component of the respective COUNT-C values for AM RLC entities equal to the START value included in this CELL UPDATE message for the CN domain as indicated in the IE "CN domain identity" in the IE "RAB info" in the variable ESTABLISHED_RABS.

NOTE:	UE actions, in case IE "Downlink counter synchronisation info" is included and either IE "RLC re-establish indicator (RB2, RB3 and RB4)" or IE "RLC re-establish indicator (RB5 and upwards)" are set to TRUE, are not defined.

1>	if the CELL UPDATE CONFIRM / URA UPDATE CONFIRM message contained the IE "Ciphering mode info" or contained the IE "Integrity protection mode info":

2>	set the IE "Status" in the variable SECURITY_MODIFICATION for all the CN domains in the variable SECURITY_MODIFICATION to "Affected".

1>	if the variable ESTABLISHMENT_CAUSE is set:

2>	clear the variable ESTABLISHMENT_CAUSE.

1>	enter a state according to subclause 8.6.3.3 applied on the CELL UPDATE CONFIRM / URA UPDATE CONFIRM message.

If the UE after state transition enters CELL_DCH, CELL_FACH, CELL_PCH or URA_PCH state, the UE shall, after state transition:

1>	determine the value for the HSPA_RNTI_STORED_PCH variable and take the corresponding actions as described in subclause 8.5.56;

1>	determine the value for the READY_FOR_COMMON_EDCH variable and take the corresponding actions as described in subclause 8.5.47;

1>	determine the value for the COMMON_E_DCH_TRANSMISSION variable and take the corresponding actions as described in subclause 8.5.46.

1>	for FDD, determine the value for the READY_FOR_COMMON_ERGCH variable and take the corresponding actions as described in subclause 8.5.75;

1>	for 1.28 Mcps TDD, determine the value for the MU_MIMO_STATUS variable and take the corresponding actions as described in subclause 8.5.61;

1>	for 1.28 Mcps TDD, if the UE supports the non-rectangular resource allocation, the UE shall:

2>	determine the value for the NON_RECTANGULAR_ALLOCATION STATUS variable and take the corresponding actions as described in subclause 8.5.82;

1>	determine the value for the HS_DSCH_DRX_CELL_FACH_STATUS variable and take the corresponding actions as described in subclause 8.6.6.xx.

If the UE after state transition enters CELL_DCH state, it shall:

1>	perform the physical layer synchronisation procedure A as specified in [29] (FDD only);

1>	not prohibit periodical status transmission in RLC;

1>	if the IE "Default DPCH Offset Value" is not included:

2>	the UE behaviour is not specified.

If the UE after state transition remains in CELL_FACH state, it shall

1>	start the timer T305 using its initial value if timer T305 is not running and periodical cell update has been configured by T305 in the IE "UE Timers and constants in connected mode" set to any other value than "infinity";

1>	if variable READY_FOR_COMMON_EDCH is set to TRUE:

2>	configure the Enhanced Uplink in CELL_FACH state and Idle mode as specified in subclause 8.5.45 for FDD and 8.5.45a for 1.28 Mcps TDD.

1>	else:

2>	select PRACH according to subclause 8.5.17;

1>	for 3.84 Mcps TDD and 7.68 Mcps TDD; or

1>	for FDD and 1.28 Mcps TDD, if the UE does not support HS-DSCH reception in CELL_FACH state; or

1>	if the IE "HS-DSCH common system information" is not included in System Information Block type 5 or System Information Block type 5bis; or

1>	for 1.28 Mcps TDD, if the IE "Common E-DCH system info" is not included in System Information Block type 5:

2>	select Secondary CCPCH according to subclause 8.5.19;

2>	take the actions related to the HS_DSCH_RECEPTION_GENERAL variable as described in subclause 8.5.37a.

1>	else:

2>	For FDD, if the RBs have the multiplexing option with transport channel type "HS-DSCH" for the DL and transport channel type "RACH" in the UL; and

2>	if the UE does not supports E-DCH transmission in CELL_FACH state and Idle mode or the IE "Common E-DCH system info" is not included system information block type 5 or 5bis; and

2>	if variable H_RNTI and variable C_RNTI are set:

3>	start to receive HS-DSCH according to the procedure in subclause 8.5.36.

2>	else:

3>	if the RBs have the multiplexing option with transport channel type "HS-DSCH" for the DL and transport channel type "E-DCH" in the UL; and

3>	if variable READY_FOR_COMMON_EDCH is set to TRUE; and

3>	if variable H_RNTI and variable C_RNTI and variable E_RNTI are set:

4>	start to receive HS-DSCH according to the procedure in subclause 8.5.36.

3>	else:

4>	clear variable C_RNTI and delete any stored C-RNTI value;

4>	clear variable H_RNTI and delete any stored H-RNTI value;

4>	clear variable E_RNTI and delete any stored E-RNTI value;

4>	clear any stored IEs "HARQ info";

4>	set the variable INVALID_CONFIGURATION to TRUE.

1>	not prohibit periodical status transmission in RLC;

1>	if the IE "UTRAN DRX cycle length coefficient" is included in the same message:

2>	ignore that IE and stop using DRX.

1>	determine the value for the HS_DSCH_DRX_CELL_FACH_STATUS variable and take the corresponding actions as described in subclause 8.6.6.xx.

**************************************** TEXT OMITTED ***************************************

##### 8.4.1.6.2	Inter-frequency measurement

Upon transition from CELL_DCH to CELL_FACH/ CELL_PCH/URA_PCH state, the UE shall:

1>	stop the inter-frequency type measurement reporting assigned in a MEASUREMENT CONTROL message;

1>	if the transition is due to a reconfiguration message which included the IE "Primary CPICH info" (for FDD) or "Primary CCPCH info" (for TDD), and the UE selects a cell other than that indicated by this IE on the current frequency (in case the IE "Frequency info" is not received) or other than that indicated by this IE on the frequency indicated by the IE "Frequency info" (when the IE "Frequency info" is included); or

1>	if the transition is due to a reconfiguration message which does not include the IE "Primary CPICH info" (for FDD) or "Primary CCPCH info" (for TDD); or

1>	if the transition is not due to a reconfiguration message:

2>	delete the measurements of type inter-frequency associated with the variable MEASUREMENT_IDENTITY and delete the corresponding compressed mode pattern stored in the variable TGPS_IDENTITY.

1> for remaining compressed mode patterns, set the IE "TGPS Status Flag" to "deactivate" and the IE "Current TGPS Status Flag" to "inactive" in the variable TGPS_IDENTITY.

1>	for FDD and 3.84/7.68 Mcps TDD:

2>	begin monitoring cells listed in the IE "inter-frequency cell info list" received in System Information Block type 12 (or System Information Block type 11, according to subclause 8.1.1.6.11 and System Information Block type 11bis, according to subclause 8.1.1.6.11a if scheduled on BCH);

1>	for FDD:

2>	begin monitoring cells listed in the IE "inter-frequency cell info list" received in System Information Block type 11ter, according to subclause 8.1.1.6.11b if scheduled on BCH);

1>	in CELL_FACH state:

2>	for FDD if variable HS_DSCH_RECEPTION_CELL_FACH_STATE is set to TRUE and the variable HS_DSCH_DRX_CELL_FACH_STATUS is set to TRUE:

3>	perform measurements on other frequencies according to the requirements in [19], during the frame(s) with the SFN value not fulfiling the inequality specified in subclause 8.5.49.

2>	for FDD, if variable HS_DSCH_RECEPTION_CELL_FACH_STATE is set to TRUE and the variable HS_DSCH_DRX_CELL_FACH_2CYCLE_STATUS is set to TRUE:

3>	perform measurements on other frequencies according to the requirements in [19], during the frame(s) with the SFN value not fulfiling the inequality specified in subclause 8.5.49b.

2>	for FDD if variable HS_DSCH_RECEPTION_CELL_FACH_STATE is set to TRUE and the variable HS_SCCH_DRX_CELL_FACH_STATUS is set to TRUE:

3> perform measurements on other frequencies according to the requirements in [19], during the subframe(s) with the Subframe_Number and SFN value not fulfiling the inequality specified in subclause 8.5.xy.

2>	otherwise:

3>	perform measurements on other frequencies, according to the IE "FACH measurement occasion info", as specified in subclause 8.5.11.

2>	for TDD:

3>	perform measurements on other frequencies according to the IE "FACH measurement occasion info".

1>	for 1.28 Mcps TDD:

2>	if after state transition the UE enters CELL_FACH state and is working on the primary frequency:

3>	begin monitoring cells listed in the IE "inter-frequency cell info list" received in System Information Block type 12 (or System Information Block type 11, according to subclause 8.1.1.6.11).

2>	else if after state transition the UE enters CELL_FACH state and is working on the secondary frequency:

3>	if the cell in which the UE transited from CELL_DCH state is not included in the active set for the CELL_FACH state; or

3>	if the working frequency changes after the state transition:

4>	the measurement shall be started when a MEASUREMENT CONTROL message is received with the measurements of type inter-frequency.

##### 8.4.1.6.3	Inter-RAT measurement

Upon transition from CELL_DCH to CELL_FACH/CELL_PCH/URA_PCH state, the UE shall:

1>	stop the inter-RAT type measurement reporting assigned in a MEASUREMENT CONTROL message;

1>	delete the measurements of type inter-RAT associated with the variable MEASUREMENT_IDENTITY and delete the corresponding compressed mode pattern stored in the variable TGPS_IDENTITY;

1>	begin monitoring cells listed in the IE "inter-RAT cell info list" received in System Information Block type 12 (or System Information Block type 11, according to subclause 8.1.1.6.11);

1>	in CELL_PCH or URA_PCH state:

2>	begin monitoring E-UTRA frequencies listed in the IE "E-UTRA frequency and priority info list" received in System Information Block type 19.

1>	in CELL_FACH state:

2>	if the IE "CELL_FACH Absolute Priority Measurement Indicator" is present in System Information Block type 19:

3>	begin monitoring E-UTRA frequencies listed in the IE "E-UTRA frequency and priority info list" received in System Information Block type 19.

2>	for FDD if variable HS_DSCH_RECEPTION_CELL_FACH_STATE is set to TRUE and the variable HS_DSCH_DRX_CELL_FACH_STATUS is set to TRUE:

3>	perform measurements on other systems according to the requirements in [19], during the frame(s) with the SFN value not fulfiling the inequality specified in subclause 8.5.49.

2>	for FDD, if variable HS_DSCH_RECEPTION_CELL_FACH_STATE is set to TRUE and the variable HS_DSCH_DRX_CELL_FACH_2CYCLE_STATUS is set to TRUE:

3>	perform measurements on other systems according to the requirements in [19], during the frame(s) with the SFN value not fulfiling the inequality specified in subclause 8.5.49b.

2>	for FDD if variable HS_DSCH_RECEPTION_CELL_FACH_STATE is set to TRUE and the variable HS_SCCH_DRX_CELL_FACH_STATUS is set to TRUE:

3> perform measurements on other frequencies according to the requirements in [19], during the subframe(s) with the Subframe_Number and SFN value not fulfiling the inequality specified in subclause 8.5.xy.

2>	otherwise:

3>	perform measurements on other frequencies according to the IE "FACH measurement occasion info";

3>	perform measurements on other systems, according to the IE "FACH measurement occasion info", as specified in subclause 8.5.11.

**************************************** TEXT OMITTED ***************************************

##### 8.4.1.9.2	Inter-frequency measurement

Upon transition from idle mode to CELL_FACH state, the UE shall:

1>	begin or continue monitoring cells listed in the IE "inter-frequency cell info list" received in System Information Block type 12 (or System Information Block type 11, according to subclause 8.1.1.6.11, and System Information Block type 11bis, according to subclause 8.1.1.6.11a if scheduled on BCH, and System Information Block type 11ter, according to subclause 8.1.1.6.11b if scheduled on BCH);

1>	for FDD, if variable HS_DSCH_RECEPTION_CELL_FACH_STATE is set to TRUE and the variable HS_DSCH_DRX_CELL_FACH_STATUS is set to TRUE:

2>	perform measurements on other frequencies according to the requirements in [19], during the frame(s) with the SFN value not fulfiling the inequality specified in subclause 8.5.49.

1>	for FDD, if variable HS_DSCH_RECEPTION_CELL_FACH_STATE is set to TRUE and the variable HS_DSCH_DRX_CELL_FACH_2CYCLE_STATUS is set to TRUE:

2>	perform measurements on other frequencies according to the requirements in [19], during the frame(s) with the SFN value not fulfiling the inequality specified in subclause 8.5.49b.

1>	for FDD, if variable HS_DSCH_RECEPTION_CELL_FACH_STATE is set to TRUE and the variable HS_SCCH_DRX_CELL_FACH_STATUS is set to TRUE:

2> perform measurements on other frequencies according to the requirements in [19], during the subframe(s) with the Subframe_Number and SFN value not fulfiling the inequality specified in subclause 8.5.xy.

1>	otherwise:

2>	perform measurements on other frequencies, according to the IE "FACH measurement occasion info", as specified in subclause 8.5.11.

1>	for TDD:

2>	perform measurements on other frequencies according to the IE "FACH measurement occasion info".

##### 8.4.1.9.3	Inter-RAT measurement

Upon transition from idle mode to CELL_FACH state, the UE shall:

1>	begin or continue monitoring cells listed in the IE "inter-RAT cell info list" received in System Information Block type 12 (or System Information Block type 11, according to subclause 8.1.1.6.11);

1>	if the IE "CELL_FACH Absolute Priority Measurement Indicator" is present in System Information Block type 19:

2>	continue monitoring the list of E-UTRA frequencies assigned in the IE "E-UTRA frequency and priority info list" in System Information Block type 19.

1>	else:

2>	stop monitoring the list of E-UTRA frequencies assigned in the IE "E-UTRA frequency and priority info list" in System Information Block type 19;

1>	for FDD, if variable HS_DSCH_RECEPTION_CELL_FACH_STATE is set to TRUE and the variable HS_DSCH_DRX_CELL_FACH_STATUS is set to TRUE:

2>	perform measurements on other systems according to the requirements in [19], during the frame(s) with the SFN value not fulfiling the inequality specified in subclause 8.5.49.

1>	for FDD, if variable HS_DSCH_RECEPTION_CELL_FACH_STATE is set to TRUE and the variable HS_DSCH_DRX_CELL_FACH_2CYCLE_STATUS is set to TRUE:

2>	perform measurements on other systems according to the requirements in [19], during the frame(s) with the SFN value not fulfiling the inequality specified in subclause 8.5.49b.

1>	for FDD, if variable HS_DSCH_RECEPTION_CELL_FACH_STATE is set to TRUE and the variable HS_SCCH_DRX_CELL_FACH_STATUS is set to TRUE:

2> perform measurements on other frequencies according to the requirements in [19], during the subframe(s) with the Subframe_Number and SFN value not fulfiling the inequality specified in subclause 8.5.xy.

1>	otherwise:

2>	perform measurements on other systems, according to the IE "FACH measurement occasion info", as specified in subclause 8.5.11.

1>	for TDD:

2>	perform measurements on other frequencies according to the IE "FACH measurement occasion info".

**************************************** TEXT OMITTED ***************************************

### 8.5.11	FACH measurement occasion calculation

For 3.84 Mcps TDD and 7.68 Mcps TDD, when in CELL_FACH state and when the variable C_RNTI is non-empty, or
for FDD and 1.28 Mcps TDD, when in CELL_FACH state, when the variable C_RNTI is non-empty and when variable HS_DSCH_RECEPTION_CELL_FACH_STATE is set to FALSE, 
then the UE shall perform measurements as specified in subclauses 8.4.1.6 and 8.4.1.9 during the frame(s) with the SFN value fulfilling the following equation:

SFN div N = C_RNTI mod M_REP + n * M_REP

where

-	N is the TTI (in number of 10ms frames) of the FACH having the largest TTI on the SCCPCH selected by the UE according to the procedure in subclause 8.5.19.  FACHs that only carry MBMS logical channels (MTCH, MSCH, or MCCH) are excluded from measurement occasion calculations.

-	C_RNTI is the C-RNTI value of the UE stored in the variable C_RNTI

-	M_REP is the Measurement Occasion cycle length. According to the equation above, a FACH Measurement Occasion of N frames will be repeated every N * M_REP frame, and M_REP = 2k.

where,

-	k is the FACH Measurement occasion cycle length coefficient.
The value of the FACH Measurement occasion cycle length coefficient is read in system information in "System Information Block type 11" or "System Information Block type 12" in the IE "FACH measurement occasion info".

-	n = 0,1,2… as long as SFN is below its maximum value

The UE is allowed to measure on other occasions in case the UE moves "out of service" area or in case it can simultaneously perform the ordered measurements.

A UE in TDD mode shall use the frame(s) with the SFN value fulfilling the above equation for neighbour cells measurements.

For FDD when in CELL_FACH state, when the variable C_RNTI is non-empty, when variable HS_DSCH_RECEPTION_CELL_FACH_STATE is set to TRUE, when variable COMMON_E_DCH_TRANSMISSION is set to FALSE, when the variable HS_DSCH_DRX_CELL_FACH_STATUS is set to FALSE, when the variable HS_DSCH_DRX_CELL_FACH_2CYCLE_STATUS is set to FALSE and when the variable HS_SCCH_DRX_CELL_FACH_STATUS is set to FALSE then the UE in FDD mode shall perform measurements as specified in subclauses 8.4.1.6 and 8.4.1.9 during the frame(s) with the SFN value fulfilling the following equation:

SFN = H-RNTI mod M_REP + n * M_REP

**************************************** TEXT OMITTED ***************************************

### 8.5.48	Actions related to HS_DSCH_DRX_CELL_FACH_STATUS variable (FDD and 1.28 Mcps TDD only), HS_DSCH_DRX_CELL_FACH_2CYCLE_STATUS variable (FDD only) and HS_SCCH_DRX_CELL_FACH_STATUS variable (FDD only)

The HS_SCCH_DRX_CELL_FACH_STATUS variable shall be set to TRUE only when the following conditions are met:

1>	the UE supports HS-SCCH DRX operation in CELL_FACH state;

1> the UE is in CELL_FACH state;

1>	the UE has a dedicated H-RNTI configured;

1>	for FDD the IE HS-SCCH DRX in CELL_FACH Information has been received or stored for the current cell;

1>	the IE HS-DSCH DRX in CELL_FACH Information has been received from System Information Block Type 5 or System Information Block Type 5bis.

If any of the above conditions is not met and the HS_SCCH_DRX_CELL_FACH_STATUS variable is set to TRUE, the UE shall:

1>	stop any ongoing HS-SCCH DRX operation in CELL_FACH state;

### 1>	set the HS_SCCH_DRX_CELL_FACH_STATUS to FALSE;

### 1>	stop the timer T321, if it is ongoing.

If the HS-SCCH_DRX_CELL_FACH_STATUS variable is set to FALSE, the HS-DSCH_DRX_CELL_FACH_2CYCLE_STATUS variable shall be set to TRUE only when the following conditions are met:

1>	the UE supports HS-DSCH DRX operation with second DRX cycle in CELL_FACH state;

1>	the UE is in CELL_FACH state;

1>	the UE has a dedicated H-RNTI configured;

1>	the IE’s "HS-DSCH DRX in CELL_FACH Information" and "Common E-DCH system info" have been received from System Information Block Type 5 or System Information Block Type 5bis and the IE "HS-DSCH DRX in CELL_FACH with second DRX cycle Information" has been received from System Information Block Type 22.

If any of the above conditions is not met and the HS_DSCH_DRX_CELL_FACH_2CYCLE _STATUS variable is set to TRUE, the UE shall:

1>	stop any ongoing CELL_FACH HS-DSCH DRX operation with second DRX cycle;

1>	set the HS_DSCH_DRX_CELL_FACH_2CYCLE _STATUS to FALSE;

1>	stop the timer T328, if it is ongoing;

1>	stop the timer T329, if it is ongoing.

If the HS-SCCH_DRX_CELL_FACH_STATUS variable is set to FALSE and the HS-DSCH_DRX_CELL_FACH_2CYCLE_STATUS variable is set to FALSE, the HS_DSCH_DRX_CELL_FACH_STATUS variable shall be set to TRUE only when the following conditions are met:

1>	the UE supports HS-DSCH DRX operation in CELL_FACH state;

1>	the UE is in CELL_FACH state;

1>	the UE has a dedicated H-RNTI configured;

1>	the IE "HS-DSCH DRX in CELL_FACH Information" for FDD or IE "HS-DSCH DRX in CELL_FACH Information 1.28 Mcps TDD" for 1.28 Mcps TDD has been received from System Information Block Type 5 or System Information Block Type 5bis;

1>	for 1.28 Mcps TDD, UE with dedicated H-RNTI configured, after transitting from CELL_PCH to CELL_FACH state, has detected its dedicated H-RNTI on HS-SCCH indicating HS-DSCH reception;

1>	for FDD, either of the following conditions is met:

2>	the value of the IE "DRX Interruption by HS-DSCH data" received from System Information Block Type 5 or System Information Block Type 5bis has been set to TRUE; or

2>	the UE supports common E-DCH transmission and the IE "Common E-DCH system info" is included in System Information Block type 5 or System Information Block Type 5bis.

If any of the above conditions is not met and the HS_DSCH_DRX_CELL_FACH_STATUS variable is set to TRUE, the UE shall:

1>	stop any ongoing CELL_FACH HS-DSCH DRX operation;

1>	set the HS_DSCH_DRX_CELL_FACH_STATUS to FALSE;

1>	stop the timer T321, if it is ongoing.

**************************************** TEXT OMITTED ***************************************

### 8.5.xy	HS-SCCH DRX operation in CELL_FACH state (FDD only)

The HS-SCCH DRX operation in CELL_FACH state determines the occasions in which the UE is allowed to discontinuously receive only HS-SCCH orders and not have to decode HS-DSCH in CELL_FACH state. This is possible only in a UE that is capable of supporting HS-SCCH DRX operation in CELL_FACH state and HS_SCCH_DRX_CELL_FACH_STATUS is set to TRUE.

If the variable HS_SCCH_DRX_CELL_FACH_STATUS is set to TRUE, the UE shall:

1>	if E-DCH resource index information is received from lower layers and the "E-DCH resource index" received from MAC with the CMAC-STATUS primitive is not equal to -1; or

1>	if "Fallback transmission initiation" is received from MAC with the CMAC-STATUS primitive:

2>	stop the timer T321, if it is ongoing;

2>	continuously receive HS-DSCH.

1>	if "Fallback transmission termination" is received from MAC with the CMAC-STATUS primitive; or

1>	if E-DCH enhanced random access process termination information is received from lower layers:

2>	start the timer T321.

1>	if the UE does not have an E-DCH resource assigned and data on HS-DSCH or HS-SCCH order for NodeB triggered HS-DPCCH transmission is received:

2>	start or, if the timer is running, restart the timer T321 at the end of the HS-SCCH subframe addressed to this UE.

2> If HS-SCCCH based DRX operation is active and received valid HS-SCCH order as defined by [27]

3>	start receiving HS-SCCH/HS-DSCH continuously 8 ms after the end of the subframe carrying the HS-SCCH order

2> else

3>	continue to receive HS-DSCH.

Upon timer T321 expiry:

1>	the UE shall only receive the HS-SCCH order during the subframe(s) with the Subframe_Number and SFN value fulfilling the following equation:

((5*SFN) + Subframe_Number - H-RNTI + 65536) mod (DRX_cycle) < Rx burst

Where:

-	H-RNTI is the value stored in the variable H_RNTI;

-	Rx burst is the period in subframes within the DRX_cycle, signalled by the IE "HS-SCCH RX burstFACH".

-	DRX_cycle is the length of the DRX cycle in subframes, calculated as follows,

DRX_cycle = 5 * value signalled by the IE "HS-SCCH DRX cycleFACH"

For FDD when in CELL_FACH state, when the variable HS_DSCH_RECEPTION_CELL_FACH_STATE is set to TRUE and the variable HS_SCCH_DRX_CELL_FACH_STATUS is set to TRUE, 
then the UE in FDD mode shall perform measurements as specified in subclauses 8.4.1.6 and 8.4.1.9, according to the requirements in [4] and [19].

**************************************** TEXT OMITTED ***************************************

#### 8.6.6.xx	HS-SCCH order based DRX operation (FDD only)

1>	if in CELL_FACH state:

2>	if IE “HS-DSCH DRX in CELL_FACH” is included in Radio bearer reconfiguration or Cell Update Confirm, the UE shall:

3>  if UE supports HS-SCCH DRX operation in CELL_FACH state:

4>	determine the value of the HS_SCCH_DRX_CELL_FACH_STATUS variable according to the procedure in subclause 8.5.48:

3> else:

4>	set the variable INVALID_CONFIGURATION to TRUE.

2>	else:

3> 	if IE “HS-DSCH DRX in CELL_FACH” is absent in Radio bearer reconfiguration or Cell Update Confirm, the UE shall:

4>	if HS_SCCH_DRX_CELL_FACH_STATUS is set to TRUE:

4>	determine the value of the HS_DSCH_DRX_CELL_FACH_STATUS variable according to the procedure in subclause 8.5.48 based upon IE’s received in System Information Block Type 5 or System Information Block Type 5bis.

**************************************** TEXT OMITTED ***************************************

### 10.2.7a	CELL UPDATE FDD

NOTE:	Only for FDD.

This message is used by the UE to initiate a cell update procedure.

RLC-SAP: TM

Logical channel: CCCH

Direction: UEUTRAN

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Integrity check info | CH |  | Integrity check info 10.3.3.16 |  |  |
| Message Type | MP |  | Message Type |  |  |
| UE information elements |  |  |  |  |  |
| U-RNTI | MP |  | U-RNTI 10.3.3.47 |  |  |
| START list | MP | 1 to <maxCNdomains> |  | START [40] values for all CN domains. |  |
| >CN domain identity | MP |  | CN domain identity 10.3.1.1 |  |  |
| >START | MP |  | START 10.3.3.38 | START value to be used in this CN domain. |  |
| AM_RLC error indication(RB2, RB3 or RB4) | MP |  | Boolean | TRUE indicates AM_RLC unrecoverable error [16] occurred on RB2, RB3 or RB4 in the UE |  |
| AM_RLC error indication(RB>4) | MP |  | Boolean | TRUE indicates AM_RLC unrecoverable error [16] occurred on RB>4 in the UE |  |
| Cell update cause | MP |  | Cell update cause 10.3.3.3 |  |  |
| Failure cause | OP |  | Failure cause and error information 10.3.3.14 |  |  |
| RRC transaction identifier | CV-Failure |  | RRC transaction identifier 10.3.3.36 |  |  |
| RB timer indicator | MP |  | RB timer indicator 10.3.3.28 |  |  |
| Establishment cause | OP |  | Establishment cause 10.3.3.11 |  |  |
| Traffic volume indicator | OP |  | Enumerated (TRUE) | This IE shall be set to TRUE when the criteria for event based traffic volume measurement reporting is fulfilled. Absence of this element means not fulfilled. |  |
| Reconfiguration Status Indicator | OP |  | Enumerated (TRUE) | TRUE indicates a reconfiguration procedure is ongoing within the UE or a response message has been submitted to RLC and the UE is waiting for the layer 2 ACK. |  |
| CS Call Type | CV-ConversationalCS |  | Enumerated (speech, video, other) | One spare value is needed |  |
| HS-PDSCH in CELL_FACH | OP |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support HS-PDSCH reception in CELL_FACH state. |  |
| UE Mobility State Indicator | MD |  | Enumerated (High-mobilityDetected) | Absence of this IE implies that, according to [4] either the High mobility state is not applicable or it has not been detected by the UE. |  |
| Capability change indicator | OP |  | Enumerated (TRUE) | TRUE indicates that the UE capability has changed compared to the value stored in the variable UE_CAPABILITY_TRANSFERRED. |  |
| Support of common E-DCH | OP |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support E-DCH enhanced random access in CELL_FACH state and Idle mode. |  |
| Support of HS-DSCH DRX operation | OP |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support HS-DSCH DRX operation in CELL_FACH state. |  |
| Support of MAC-i/is | OP |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support MAC-i/is operation. |  |
| Security Revert Status Indicator | OP |  | Enumerated (Reverted Back, Normal Operation) | Reverted Back indicates this UE has reverted back to the old security configuration by aborting the ongoing security mode command. Normal Operation indicates this UE has not reverted back to the old security configuration by aborting an ongoing security mode command. The absence of this IE indicates that the UE does not support reporting of security revert status. |  |
| Support of HS-SCCH DRX operation | OP |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support HS-SCCH DRX operation in CELL_FACH state. | REL-14 |
| Measurement information elements |  |  |  |  |  |
| Measured results on RACH | OP |  | Measured results on RACH FDD 10.3.7.45a |  |  |
| Other Information elements |  |  |  |  |  |
| Logged Meas Available | OP |  | Enumerated (TRUE) | Indicates the UE has logged measurements to report to the network |  |
| ANR Logging Results Available | OP |  | Enumerated (TRUE) | True indicates the UE has ANR logging results to report to the Network. |  |

| Condition | Explanation |
| --- | --- |
| Failure | This IE is mandatory present if the IE "Failure cause" is present and not needed otherwise. |
| ConversationalCS | This IE is mandatory present if the IE "Establishment cause" has the value "Originating Conversational Call" or "EmergencyCall" and a CS call is being initiated. Otherwise it is not needed. |

### 10.2.8	CELL UPDATE CONFIRM

This message confirms the cell update procedure and can be used to reallocate new RNTI information for the UE valid in the new cell.

RLC-SAP: UM

Logical channel: CCCH or DCCH

Direction: UTRANUE

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Message Type | MP |  | Message Type |  |  |
| UE Information Elements |  |  |  |  |  |
| U-RNTI | CV-CCCH |  | U-RNTI 10.3.3.47 |  |  |
| RRC transaction identifier | MP |  | RRC transaction identifier 10.3.3.36 |  |  |
| Integrity check info | CH |  | Integrity check info 10.3.3.16 |  |  |
| Integrity protection mode info | OP |  | Integrity protection mode info 10.3.3.19 | The UTRAN should not include this IE unless it is performing an SRNS relocation or a cell reselection from GERAN Iu mode |  |
| Ciphering mode info | OP |  | Ciphering mode info 10.3.3.5 | The UTRAN should not include this IE unless it is performing either an SRNS relocation or a cell reselection from GERAN Iu mode, and a change in ciphering algorithm. |  |
| Activation time | MD |  | Activation time 10.3.3.1 | Default value is "now" |  |
| New U-RNTI | OP |  | U-RNTI 10.3.3.47 |  |  |
| New C-RNTI | OP |  | C-RNTI 10.3.3.8 |  |  |
| New DSCH-RNTI | OP |  | DSCH-RNTI 10.3.3.9a | Should not be set in FDD. If received, the UE behaviour is unspecified. |  |
| New H-RNTI | OP |  | H-RNTI 10.3.3.14a |  | REL-5 |
| New Primary E-RNTI | OP |  | E-RNTI 10.3.3.10a |  | REL-6 |
| New Secondary E-RNTI | OP |  | E-RNTI 10.3.3.10a | FDD only | REL-6 |
| RNTI handling at cell re-selection | CV-URA_PCH |  | Enumerated (TRUE) | The presence of this IE indicates that the UE shall clear the variable H_RNTI, C_RNTI and E_RNTI upon cell re-selection when operating in URA_PCH with seamless transition. The absence indicates that the UE shall not clear the variable H_RNTI, C_RNTI and E_RNTI upon cell re-selection when operating in URA_PCH with seamless transition. | REL-13 |
| RRC State Indicator | MP |  | RRC State Indicator 10.3.3.35a |  |  |
| UTRAN DRX cycle length coefficient | OP |  | UTRAN DRX cycle length coefficient 10.3.3.49 |  |  |
| Wait time | OP |  | Wait time 10.3.3.50 |  |  |
| RLC re-establish indicator (RB2, RB3 and RB4) | MP |  | RLC re-establish indicator 10.3.3.35 | Should not be set to TRUE if IE "Downlink counter synchronisation info" is included in message. |  |
| RLC re-establish indicator (RB5 and upwards) | MP |  | RLC re-establish indicator 10.3.3.35 | Should not be set to TRUE if IE "Downlink counter synchronisation info" is included in message. |  |
| CN Information Elements |  |  |  |  |  |
| CN Information info | OP |  | CN Information info 10.3.1.3 |  |  |
| UTRAN Information Elements |  |  |  |  |  |
| URA identity | OP |  | URA identity 10.3.2.6 |  |  |
| RNC support for change of UE capability | OP |  | Boolean | Should be included if the message is used to perform an SRNS relocation | REL-7 |
| RB information elements |  |  |  |  |  |
| RAB information for setup | CV-CS |  | RAB information for setup 10.3.4.10 | RAB identity shall be identical to the one currently configured and UE behaviour is unspecified otherwise. |  |
| RB information to release list | OP | 1 to <maxRB> |  |  |  |
| >RB information to release | MP |  | RB information to release 10.3.4.19 |  |  |
| RB information to reconfigure list | OP | 1 to <maxRB> |  |  |  |
| >RB information to reconfigure | MP |  | RB information to reconfigure 10.3.4.18 |  |  |
| RB information to be affected list | OP | 1 to <maxRB> |  |  |  |
| >RB information to be affected | MP |  | RB information to be affected 10.3.4.17 |  |  |
| Downlink counter synchronisation info | OP |  |  |  |  |
| >RB with PDCP information list | OP | 1 to <maxRBallRABs> |  |  |  |
| >>RB with PDCP information | MP |  | RB with PDCP information 10.3.4.22 | This IE is needed for each RB having PDCP in the case of lossless SRNS relocation |  |
|  | OP |  |  |  | REL-5 |
| >>PDCP context relocation info | OP |  | PDCP context relocation info 10.3.4.1a | This IE is needed for each RB having PDCP and performing PDCP context relocation | REL-5 |
| PDCP ROHC target mode | OP |  | PDCP ROHC target mode 10.3.4.2a |  | REL-5 |
| Retrievable configuration |  |  |  |  |  |
| Retrievable configuration info | OP |  | Retrievable configuration info 10.3.8.25 |  | REL-13 |
| Other state configuration info | OP |  | Other state configuration info 10.3.8.27 |  | REL-13 |
| TrCH Information Elements |  |  |  |  |  |
| Uplink transport channels |  |  |  |  |  |
| UL Transport channel information common for all transport channels | OP |  | UL Transport channel information common for all transport channels 10.3.5.24 |  |  |
| Deleted TrCH information list | OP | 1 to <maxTrCH> |  |  |  |
| >Deleted UL TrCH information | MP |  | Deleted UL TrCH information 10.3.5.5 |  |  |
| Added or Reconfigured TrCH information list | OP | 1 to <maxTrCH> |  |  |  |
| >Added or Reconfigured UL TrCH information | MP |  | Added or Reconfigured UL TrCH information 10.3.5.2 |  |  |
| Downlink transport channels |  |  |  |  |  |
| DL Transport channel information common for all transport channels | OP |  | DL Transport channel information common for all transport channels 10.3.5.6 |  |  |
| Deleted TrCH information list | OP | 1 to <maxTrCH> |  |  |  |
| >Deleted DL TrCH information | MP |  | Deleted DL TrCH information 10.3.5.4 |  |  |
| Added or Reconfigured TrCH information list | OP | 1 to <maxTrCH> |  |  |  |
| >Added or Reconfigured DL TrCH information | MP |  | Added or Reconfigured DL TrCH information 10.3.5.1 |  |  |
| PhyCH information elements |  |  |  |  |  |
| Frequency info | OP |  | Frequency info 10.3.6.36 |  |  |
| Multi-frequency Info | OP |  | Multi-frequency Info 10.3.6.39a | This IE is used for 1.28 Mcps TDD only | REL-7 |
| DTX-DRX timing information | OP |  | DTX-DRX timing information 10.3.6.34b |  | REL-7 |
| DTX-DRX Information | OP |  | DTX-DRX Information 10.3.6.34a |  | REL-7 |
| HS-SCCH less Information | OP |  | HS-SCCH less Information 10.3.6.36ab |  | REL-7 |
| MIMO parameters | OP |  | MIMO parameters 10.3.6.41a |  | REL-7 |
| MIMO mode with four transmit antennas parameters | OP |  | MIMO mode with four transmit antennas parameters 10.3.6.142 |  | REL-11 |
| DCH Enhancements info FDD | OP |  | DCH Enhancements info FDD 10.3.6.149 |  | REL-12 |
| Control Channel DRX information | OP |  | Control Channel DRX information 1.28 Mcps TDD 10.3.6.107 | This IE is used for 1.28 Mcps TDD only | REL-8 |
| SPS Information | OP |  | SPS information 1.28 Mcps TDD 10.3.6.110 | This IE is used for 1.28 Mcps TDD only | REL-8 |
| MU-MIMO info | OP |  | MU-MIMO info 1.28 Mcps TDD 10.3.6.122 | This IE is used for 1.28 Mcps TDD only | REL-10 |
| Non-rectangular resource allocation indicator | OP |  | Enumerated(TRUE) | For 1.28 Mcps TDD only. The absence of this IE indicates that the non-rectangular resource allocation is not used. | REL-12 |
| Non-rectangular resource specific timeslots set | CV-Non-rectangularResourceAllocation |  | Bit string (7) | For 1.28 Mcps TDD only. The value 1 of a bit indicates the corresponding timeslot in which the channelization codes are assigned by physical control channel. The value 0 of a bit indicates the corresponding timeslot in which all channelization codes are assigned when the timeslot is scheduled to the UE. The first/leftmost bit of the bit string is for TS0. | REL-12 |
| Uplink radio resources |  |  |  |  |  |
| Maximum allowed UL TX power | MD |  | Maximum allowed UL TX power 10.3.6.39 | Default value is the existing maximum UL TX power |  |
| Uplink DPCH info | OP |  | Uplink DPCH info 10.3.6.88 |  |  |
| E-DCH Info | OP |  | E-DCH Info 10.3.6.97 |  | REL-6 |
| Uplink secondary cell info FDD | OP |  | Uplink secondary cell info FDD 10.3.6.115 | FDD only | REL-9 |
| Multi-carrier E-DCH Info for LCR TDD | OP |  | Multi-carrier E-DCH Info for LCR TDD 10.3.6.97a | 1.28Mcps TDD only | REL-10 |
| Uplink CLTD info FDD | OP |  | Uplink CLTD info FDD 10.3.6.125 | FDD only | REL-11 |
| Uplink OLTD info FDD | OP |  | Uplink OLTD info FDD 10.3.6.126 | FDD only | REL-11 |
| Other TTI E-DCH Configuration Information | OP |  | Other TTI E-DCH Configuration Information 10.3.6.152 | FDD only | REL-12 |
| Filtered UE power headroom reporting information | OP |  | Filtered UE power headroom reporting information 10.3.7.140 | FDD only | REL-12 |
| Downlink radio resources |  |  |  |  |  |
| Downlink HS-PDSCH Information | OP |  | Downlink HS_PDSCH Information 10.3.6.23a |  | REL-5 |
| Downlink information common for all radio links | OP |  | Downlink information common for all radio links 10.3.6.24 |  |  |
| Downlink information per radio link list | OP | 1 to <maxRL> |  | Send downlink information for each radio link to be set-up |  |
| >Downlink information for each radio link | MP |  | Downlink information for each radio link 10.3.6.27 |  |  |
| PhyCH information elements (Complete specification) |  |  |  |  |  |
| >>DCH Enhancements info FDD | OP |  | DCH Enhancements info FDD 10.3.6.149 |  | REL-12 |
| Downlink secondary cell info FDD | OP |  | Downlink secondary cell info FDD 10.3.6.31a | FDD only | REL-8 |
| Additional downlink secondary cell info list FDD | OP | 2 |  |  | REL-10 |
| >Downlink secondary cell info FDD | OP |  | Downlink secondary cell info FDD 10.3.6.31a | FDD only | REL-10 |
| Additional downlink secondary cell info list FDD 2 | OP | 4 |  |  | REL-11 |
| >Downlink secondary cell info FDD | OP |  | Downlink secondary cell info FDD 10.3.6.31a | FDD only | REL-11 |
| Common E-RGCH info FDD | OP |  | Common E-RGCH info FDD 10.3.6.138 | FDD only | REL-11 |
| MBMS PL Service Restriction Information | OP |  | Enumerated (TRUE) |  | REL-6 |
| CELL_DCH measurement occasion info LCR | OP |  | CELL_DCH measurement occasion info LCR 10.3.7.126 |  | REL-9 |
| HS-SCCH DRX in CELL_FACH Information | OP |  | HS-SCCH DRX in CELL_FACH Information 10.3.6.xx |  | REL-14 |
| WLAN Information Elements |  |  |  |  |  |
| Dedicated WLAN Offload Information | OP |  | Dedicated WLAN Offload Information 10.3.2.10 |  | REL-12 |

| Condition | Explanation |
| --- | --- |
| CCCH | This IE is mandatory present when CCCH is used and ciphering is not required and not needed otherwise. |
| CS | This IE is optionally present only if CS domain RAB mapping is reconfigured between DCH and E-DCH/HS-DSCH and shall not be present otherwise. |
| Non-rectangularResourceAllocation | This IE is optionally present if non-rectangular resource allocation is configured. Otherwise, it is not needed. |
| URA_PCH | This IE is optional present when IE "RRC State Indicator" is set to URA_PCH. Otherwise it is not needed. |

**************************************** TEXT OMITTED ***************************************

### 10.2.27	RADIO BEARER RECONFIGURATION

This message is sent from UTRAN to reconfigure parameters related to a change of QoS or to release and setup a radio bearer used for ptp transmission of MBMS services of the broadcast type. This procedure can also change the multiplexing of MAC, reconfigure transport channels and physical channels. This message is also used to perform a handover from GERAN Iu mode to UTRAN.

RLC-SAP: AM or UM or sent through GERAN Iu mode

Logical channel: DCCH or sent through GERAN Iu mode

Direction: UTRAN  UE

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Message Type | MP |  | Message Type |  |  |
| UE Information elements |  |  |  |  |  |
| RRC transaction identifier | MP |  | RRC transaction identifier 10.3.3.36 |  |  |
| Integrity check info | CH |  | Integrity check info 10.3.3.16 |  |  |
| Integrity protection mode info | OP |  | Integrity protection mode info 10.3.3.19 | The UTRAN should not include this IE unless it is performing an SRNS relocation or a handover from GERAN Iu mode |  |
| Ciphering mode info | OP |  | Ciphering mode info 10.3.3.5 | The UTRAN should not include this IE unless it is performing either an SRNS relocation or a handover from GERAN Iu mode and a change in ciphering algorithm |  |
| Activation time | MD |  | Activation time 10.3.3.1 | Default value is "now" |  |
| Dynamic activation time | OP |  | Dynamic activation time 10.3.3.1a |  | REL-13 |
| Delay restriction flag | OP |  | Enumerated (TRUE) | This IE is always set to TRUE and included if the activation time is restricted according to subclause 8.6.3.1 | REL-6 |
| New U-RNTI | OP |  | U-RNTI 10.3.3.47 |  |  |
| New C-RNTI | OP |  | C-RNTI 10.3.3.8 |  |  |
| New DSCH-RNTI | OP |  | DSCH-RNTI 10.3.3.9a | Should not be set in FDD. If received The UE behaviour is unspecified. |  |
| New H-RNTI | OP |  | H-RNTI 10.3.3.14a |  | REL-5 |
| New Primary E-RNTI | OP |  | E-RNTI 10.3.3.10a |  | REL-6 |
| New Secondary E-RNTI | OP |  | E-RNTI 10.3.3.10a | FDD only | REL-6 |
| RNTI handling at cell re-selection | CV-URA_PCH |  | Enumerated (TRUE) | The presence of this IE indicates that the UE shall clear the variable H_RNTI, C_RNTI and E_RNTI upon cell re-selection when operating in URA_PCH with seamless transition. The absence indicates that the UE shall not clear the variable H_RNTI, C_RNTI and E_RNTI upon cell re-selection when operating in URA_PCH with seamless transition. | REL-13 |
| RRC State Indicator | MP |  | RRC State Indicator 10.3.3.35a |  |  |
| UE Mobility State Indicator | CV-FACH_PCH |  | Enumerated (High-mobilityDetected) | Absence of this IE implies that, according to [4], the UE shall consider itself being in the mobility state the UE has maintained in CELL_DCH state or being not in high mobility state after the state transition, if applicable. | REL-7 |
| UTRAN DRX cycle length coefficient | OP |  | UTRAN DRX cycle length coefficient 10.3.3.49 |  |  |
| Access Group identity | OP |  | Integer (0..15) |  | REL-12 |
| CN information elements |  |  |  |  |  |
| CN Information info | OP |  | CN Information info 10.3.1.3 |  |  |
| Signalling Connection release indication | OP |  | CN domain identity 10.3.1.1 |  | REL-14 |
| UTRAN mobility information elements |  |  |  |  |  |
| RNC support for change of UE capability | OP |  | Boolean | Should be included if the message is used to perform an SRNS relocation | REL-7 |
| Reconfiguration in response to requested change of UE capability | OP |  | Enumerated (TRUE) |  | REL-7 |
| URA identity | OP |  | URA identity 10.3.2.6 |  |  |
| Specification mode information elements |  |  |  |  | REL-8 |
| Default configuration for CELL_FACH | CV-SRNS_RELOCATION |  | Default configuration for CELL_FACH 10.3.4.0a |  | REL-8 |
| CHOICE specification mode | MP |  |  |  | REL-5 |
| >Complete specification |  |  |  |  |  |
| RB information elements |  |  |  |  |  |
| >>RAB information to setup list | OP | 1 to <maxRABsetup> |  | For each RAB established | REL-14 |
| >>>RAB information for setup | MP |  | RAB information for setup 10.3.4.10 |  | REL-14 |
| >>RAB information to reconfigure list | OP | 1 to < maxRABsetup > |  |  |  |
| >>>RAB information to reconfigure | MP |  | RAB information to reconfigure 10.3.4.11 |  |  |
| >>RAB information for MBMS ptp bearer list | OP | 1 to < maxMBMSservSelect > |  |  | REL-6 |
| >>>RAB information for MBMS ptp bearer | MP |  | RAB information for MBMS ptp bearer 10.3.4.9a |  | REL-6 |
| >>RB information to reconfigure list | MP | 1to <maxRB> |  | Although this IE is not always required, need is MP to align with ASN.1 |  |
|  | OP |  |  |  | REL-4 |
| >>>RB information to reconfigure | MP |  | RB information to reconfigure 10.3.4.18 |  |  |
| >>RB information to be affected list | OP | 1 to <maxRB> |  |  |  |
| >>RB information to release list | OP | 1 to <maxRB> |  |  | REL-14 |
| >>>RB information to release | MP |  | RB information to release 10.3.4.19 |  | REL-14 |
| >>>RB information to be affected | MP |  | RB information to be affected 10.3.4.17 |  |  |
| >>RB with PDCP context relocation info list | OP | 1 to <maxRBallRABs> |  | This IE is needed for each RB having PDCP and performing PDCP context relocation | REL-5 |
| >>>PDCP context relocation info | MP |  | PDCP context relocation info 10.3.4.1a |  | REL-5 |
| >>PDCP ROHC target mode | OP |  | PDCP ROHC target mode 10.3.4.2a |  | REL-5 |
| >>UM RLC re-establishment RB List | OP | 1 to <maxRB> |  |  | REL-10 |
| >>>RB bearer to be re-established | MP |  | RB identity, 10.3.4.16 | UE behaviour is unspecified if RB identity in this IE indicates a value, which is less than or equal to 4 or indicates a radio bearer, which does not use UM RLC or is not associated with PS domain. | REL-10 |
| >>Other state configuration info | OP |  | Other state configuration info 10.3.8.27 |  | REL-13 |
| TrCH Information Elements |  |  |  |  |  |
| Uplink transport channels |  |  |  |  |  |
| >>UL Transport channel information common for all transport channels | OP |  | UL Transport channel information common for all transport channels 10.3.5.24 |  |  |
| >>Deleted TrCH information list | OP | 1 to <maxTrCH> |  |  |  |
| >>>Deleted UL TrCH information | MP |  | Deleted UL TrCH information 10.3.5.5 |  |  |
| >>Added or Reconfigured TrCH information list | OP | 1 to <maxTrCH> |  |  |  |
| >>>Added or Reconfigured UL TrCH information | MP |  | Added or Reconfigured UL TrCH information 10.3.5.2 |  |  |
| Downlink transport channels |  |  |  |  |  |
| >>DL Transport channel information common for all transport channels | OP |  | DL Transport channel information common for all transport channels 10.3.5.6 |  |  |
| >>Deleted TrCH information list | OP | 1 to <maxTrCH> |  |  |  |
| >>>Deleted DL TrCH information | MP |  | Deleted DL TrCH information 10.3.5.4 |  |  |
| >>Added or Reconfigured TrCH information list | OP | 1 to <maxTrCH> |  |  |  |
| >>>Added or Reconfigured DL TrCH information | MP |  | Added or Reconfigured DL TrCH information 10.3.5.1 |  |  |
| Retrievable configuration |  |  |  |  |  |
| >>Retrievable configuration info | OP |  | Retrievable configuration info 10.3.8.25 |  | REL-13 |
| >Preconfiguration |  |  |  |  | REL-5 |
| >>CHOICE Preconfiguration mode | MP |  |  | This value only applies in case the message is sent through GERAN Iu mode |  |
| >>>Predefined configuration identity | MP |  | Predefined configuration identity 10.3.4.5 |  |  |
| >>>Default configuration |  |  |  |  |  |
| >>>>Default configuration mode | MP |  | Enumerated (FDD, TDD) | Indicates whether the FDD or TDD version of the default configuration shall be used |  |
| >>>>Default configuration identity | MP |  | Default configuration identity 10.3.4.0 |  |  |
| PhyCH information elements |  |  |  |  |  |
| Frequency info | OP |  | Frequency info 10.3.6.36 |  |  |
| Multi-frequency Info | OP |  | Multi-frequency Info 10.3.6.39a | This IE is used for 1.28 Mcps TDD only | REL-7 |
| DTX-DRX timing information | OP |  | DTX-DRX timing information 10.3.6.34b |  | REL-7 |
| DTX-DRX Information | OP |  | DTX-DRX Information 10.3.6.34a |  | REL-7 |
| HS-SCCH less Information | OP |  | HS-SCCH less Information 10.3.6.36ab |  | REL-7 |
| MIMO parameters | OP |  | MIMO parameters 10.3.6.41a |  | REL-7 |
| MIMO mode with four transmit antennas parameters | OP |  | MIMO mode with four transmit antennas parameters 10.3.6.142 |  | REL-11 |
| DCH Enhancements info FDD | OP |  | DCH Enhancements info FDD 10.3.6.149 |  | REL-12 |
| Control Channel DRX information | OP |  | Control Channel DRX information 1.28 Mcps TDD 10.3.6.107 | This IE is used for 1.28 Mcps TDD only | REL-8 |
| SPS Information | OP |  | SPS information 1.28 Mcps TDD 10.3.6.110 | This IE is used for 1.28 Mcps TDD only | REL-8 |
| MU-MIMO info | OP |  | MU-MIMO info 1.28 Mcps TDD 10.3.6.122 | This IE is used for 1.28 Mcps TDD only | REL-10 |
| Non-rectangular resource allocation indicator | OP |  | Enumerated(TRUE) | For 1.28 Mcps TDD only. The absence of this IE indicates that the non-rectangular resource allocation is not used. | REL-12 |
| Non-rectangular resource specific timeslots set | CV-Non-rectangularResourceAllocation |  | Bit string (7) | For 1.28 Mcps TDD only. The value 1 of a bit indicates the corresponding timeslot in which the channelization codes are assigned by physical control channel. The value 0 of a bit indicates the corresponding timeslot in which all channelization codes are assigned when the timeslot is scheduled to the UE. The first/leftmost bit of the bit string is for TS0. | REL-12 |
| HS-SCCH DRX in CELL_FACH Information | OP |  | HS-SCCH DRX in CELL_FACH Information 10.3.6.xx |  | REL-14 |
| Uplink radio resources |  |  |  |  |  |
| Maximum allowed UL TX power | MD |  | Maximum allowed UL TX power 10.3.6.39 | Default value is the existing maximum UL TX power |  |
| Uplink DPCH info | OP |  | Uplink DPCH info 10.3.6.88 |  |  |
| E-DCH Info | OP |  | E-DCH Info 10.3.6.97 |  | REL-6 |
| Uplink secondary cell info FDD | OP |  | Uplink secondary cell info FDD 10.3.6.115 | FDD only | REL-9 |
| Multi-carrier E-DCH Info for LCR TDD | OP |  | Multi-carrier E-DCH Info for LCR TDD 10.3.6.97a | 1.28Mcps TDD only | REL-10 |
| Uplink CLTD info FDD | OP |  | Uplink CLTD info FDD 10.3.6.125 | FDD only | REL-11 |
| Uplink OLTD info FDD | OP |  | Uplink OLTD info FDD 10.3.6.126 | FDD only | REL-11 |
| Other TTI E-DCH Configuration Information | OP |  | Other TTI E-DCH Configuration Information 10.3.6.152 | FDD only | REL-12 |
| Filtered UE power headroom reporting information | OP |  | Filtered UE power headroom reporting information 10.3.7.140 | FDD only | REL-12 |
| Downlink radio resources |  |  |  |  |  |
| Downlink HS-PDSCH Information | OP |  | Downlink HS-PDSCH Information 10.3.6.23a |  | REL-5 |
| Downlink information common for all radio links | OP |  | Downlink information common for all radio links 10.3.6.24 |  |  |
| Downlink information per radio link list | MP | 1 to <maxRL> |  | Although this IE is not always required, need is MP to align with ASN.1 |  |
|  | OP |  |  |  | REL-4 |
| >Downlink information for each radio link | MP |  | Downlink information for each radio link 10.3.6.27 |  |  |
| Downlink secondary cell info FDD | OP |  | Downlink secondary cell info FDD 10.3.6.31a | FDD only | REL-8 |
| Additional downlink secondary cell info list FDD | OP | 2 |  |  | REL-10 |
| >Downlink secondary cell info FDD | OP |  | Downlink secondary cell info FDD 10.3.6.31a | FDD only | REL-10 |
| Additional downlink secondary cell info list FDD 2 | OP | 4 |  |  | REL-11 |
| >Downlink secondary cell info FDD | OP |  | Downlink secondary cell info FDD 10.3.6.31a | FDD only | REL-11 |
| Common E-RGCH info FDD | OP |  | Common E-RGCH info FDD 10.3.6.138 | FDD only | REL-11 |
| DPCCH2 info FDD | OP |  | DPCCH2 info FDD 10.3.6.148 | FDD only | REL-12 |
| MBMS PL Service Restriction Information | OP |  | Enumerated (TRUE) |  | REL-6 |
| CELL_DCH measurement occasion info LCR | OP |  | CELL_DCH measurement occasion info LCR 10.3.7.126 |  | REL-9 |

| Condition | Explanation |
| --- | --- |
| FACH_PCH | This IE is mandatory default when a transition from CELL_DCH to CELL_FACH, URA_PCH or CELL_PCH is requested by the message and is not needed otherwise. |
| SRNS_RELOCATION | This IE is optional if the message is used to perform an SRNS relocation and is not needed otherwise. |
| Non-rectangularResourceAllocation | This IE is optionally present if non-rectangular resource allocation is configured. Otherwise, it is not needed. |
| URA_PCH | This IE is optional present when IE "RRC State Indicator" is set to URA_PCH. Otherwise it is not needed. |

**************************************** TEXT OMITTED ***************************************

### 10.2.60	URA UPDATE

This message is used by the UE to initiate a URA update procedure.

RLC-SAP: TM

Logical channel: CCCH

Direction: UEUTRAN

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Message Type | MP |  | Message Type |  |  |
| UE information elements |  |  |  |  |  |
| U-RNTI | MP |  | U-RNTI 10.3.3.47 |  |  |
| RRC transaction identifier | CV- ProtErr |  | RRC transaction identifier 10.3.3.36 |  |  |
| Integrity check info | CH |  | Integrity check info 10.3.3.16 |  |  |
| URA update cause | MP |  | URA update cause 10.3.3.46 |  |  |
| Protocol error indicator | MD |  | Protocol error indicator 10.3.3.27 | Default value is FALSE |  |
| HS-PDSCH in CELL_PCH and URA_PCH | OP |  | Enumerated (TRUE) | This IE indicates whether the UE supports HS-PDSCH reception in CELL_PCH and URA_PCH states. The absence of this IE indicates that the UE does not support HS-PDSCH reception in CELL_PCH or URA_PCH states. | REL-7 |
| HS-PDSCH in CELL_FACH | OP |  | Enumerated (TRUE) | This IE indicates whether the UE supports HS-PDSCH reception in CELL_FACH state. The absence of this IE indicates that the UE does not support HS-PDSCH reception in CELL_FACH state. | REL-7 |
| Support of HS-DSCH DRX operation | OP |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support HS-DSCH DRX operation in CELL_FACH state. | REL-8 |
| Support of common E-DCH | OP |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support E-DCH enhanced random access in CELL_FACH state and Idle mode. | REL-8 |
| Support of MAC-i/is | OP |  | Enumerated (TRUE) | This IE indicates whether the UE supports MAC-i/is operation. | REL-8 |
| Support for Two DRX schemes in URA_PCH and CELL_PCH | OP |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support Two DRX schemes in URA_PCH and CELL_PCH | REL-7 |
| Support of HS-SCCH DRX operation | OP |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support HS-SCCH DRX operation in CELL_FACH state. | REL-14 |
| Other information elements |  |  |  |  |  |
| Protocol error information | CV-ProtErr |  | Protocol error information 10.3.8.12 |  |  |
| Other Information elements |  |  |  |  | Rel-10 |
| Logged Meas Available | OP |  | Enumerated (TRUE) | Indicates the UE has logged measurements to report to the network | Rel-10 |
| ANR Logging Results Available | OP |  | Enumerated (TRUE) | True indicates the UE has ANR logging results to report to the Network. | REL-10 |

| Condition | Explanation |
| --- | --- |
| ProtErr | The IE is mandatory present if the IE "Protocol error indicator" has the value TRUE and not needed otherwise. |

**************************************** TEXT OMITTED ***************************************

#### 10.3.3.25	Physical channel capability

| Information Element/Group name | Need | Multi | Type and Reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| Downlink physical channel capability information elements |  |  |  |  |  |
| FDD downlink physical channel capability | CH-fdd_req_sup |  |  |  |  |
| >Max no DPCH codes | MP |  | Integer (1..8) | Maximum number of DPCH codes to be simultaneously received |  |
| >Max no physical channel bits received | MP |  | Integer (1200, 2400, 3600, 4800, 7200, 9600, 14400, 19200, 28800, 38400, 48000, 57600, 67200, 76800) | Maximum number of physical channel bits received in any 10 ms interval (DPCH, S-CCPCH) |  |
| >Support for SF 512 and 80 ms TTI for DPCH | MP |  | Boolean | TRUE means supported |  |
| >Support for DCH Enhancements | CV-not_iRAT_HoInfo2 |  | Enumerated (Basic, Full) | The absence of this IE indicates that the UE does not support DCH Enhancements | REL-12 |
| >Simultaneous support for DCH Enhancements and Compressed Mode operation | CV-not_iRAT_HoInfo2 |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support simultaneous operation of DCH Enhancements and Compressed Mode. | REL-12 |
| >Simultaneous support for DCH Enhancements and DPCCH Discontinuous Transmission | CV-not_iRAT_HoInfo2 |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support simultaneous operation of DCH Enhancements and DPCCH Discontinuous Transmission. | REL-12 |
| >Support of Enhanced Serving Cell Change for Event 1c | CV-not_iRAT_HoInfo2 |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support Enhanced Serving Cell Change for Event 1c operation. | REL-12 |
| >Support of DPCCH2 | CV-not_iRAT_HoInfo2 |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support DPCCH2 transmission. | REL-12 |
| >Support of power control algorithm 3 | CV-not_iRAT_HoInfo2 |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support power control algorithm 3. | REL-13 |
| >Support of blind HARQ retransmissions for HSDPA | CV-not_iRAT_HoInfo2 |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support blind HARQ retransmissions for HSDPA. | REL-13 |
| >CHOICE Support of HS-PDSCH | CV-not_iRAT_HoInfo |  |  |  | REL-5 |
| >>Supported |  |  |  |  | REL-5 |
| >>>HS-DSCH physical layer category | MP |  | Integer (1..64) | As defined in [35]. Values 13...64 are spares. See Note 5. | REL-5 |
| >>>HS-DSCH physical layer category extension | OP |  | Integer (1..20) | As defined in [35]. See Note 6. | REL-7 |
| >>>HS-DSCH physical layer category extension 2 | OP |  | Integer (21..24) | As defined in [35]. See Note 12. Absence of this IE means that dual cell operation is not supported. | REL-8 |
| >>>HS-DSCH physical layer category extension 3 | OP |  | Integer (25..28) | As defined in [35]. See Note 13. Absence of this IE means that dual cell operation is not supported with MIMO. | REL-9 |
| >>>HS-DSCH physical layer category extension 4 | OP |  | Integer (29,30) | As defined in [35]. See Note 16. Absence of this IE means the multi-cell operation on three cells is not supported. | REL-10 |
| >>>HS-DSCH physical layer category extension 5 | OP |  | Integer (31,32) | As defined in [35]. See Note 17. Absence of this IE means the multi-cell operation on four cells is not supported. | REL-10 |
| >>>HS-DSCH physical layer category extension 6 | OP |  | Integer (33,34) | As defined in [35]. See Note 18. Absence of this IE means the multi-cell operation on five and /or six cells is not supported. | REL-11 |
| >>>HS-DSCH physical layer category extension 7 | OP |  | Integer (35,36) | As defined in [35]. See Note 19. Absence of this IE means the multi-cell operation on seven and/or eight cells is not supported. | REL-11 |
| >>>HS-DSCH physical layer category extension 8 | OP |  | Integer (37,38) | As defined in [35]. See Note 20. Absence of this IE means the MIMO mode with four transmit antennas operation is not supported. | REL-11 |
| >>>HS-SCCHless HS-DSCH operation support | OP |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support HS-SCCHless HS-DSCH operation. | REL-7 |
| >>>Enhanced F-DPCH support | OP |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support enhanced F-DPCH | REL-7 |
| >>>HS-PDSCH in CELL_FACH | OP |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support HS-PDSCH reception in CELL_FACH | REL-7 |
| >>>HS-PDSCH in CELL_PCH and URA_PCH | OP |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support HS-PDSCH in neither CELL_PCH nor URA_PCH states. | REL-7 |
| >>>Target Cell Pre-Configuration | OP |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support HS-SCCH reception in target cell | REL-8 |
| >>>Support of HS-DSCH DRX operation | OP |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support HS-DSCH DRX operation | REL-8 |
| >>>Support of MIMO only with single stream restriction | OP |  | Enumerated (TRUE) | The presence of this IE means that the UE supports MIMO only with single stream restriction. | REL-9 |
| >>> Non-contiguous multi-cell with MIMO | OP |  | Enumerated (TRUE) | The presence of this IE means that the UE supports non-contiguous multi-cell with MIMO in the frequency bands where it supports non-contiguous multi-cell operation. | REL-11 |
| >>>Support of MIMO mode with four transmit antennas only with dual stream restriction | OP |  | Enumerated (TRUE) | The presence of this IE means that the UE supports MIMO mode with four transmit antennas operation only with dual stream restriction. | REL-11 |
| >>>Support of NodeB triggered HS-DPCCH transmission | OP |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support NodeB triggered HS-DPCCH transmission | REL-11 |
| >>>Support of HS-DSCH DRX operation with second DRX cycle | OP |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support HS-DSCH DRX operation with second DRX cycle in CELL_FACH state. | REL-11 |
| >>Unsupported |  |  |  | (no data) | REL-5 |
| >DRX enhancements | CV-not_iRAT_HoInfo2 |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support DRX enhancements. | REL-12 |
| >HS-DPCCH overhead reduction | CV-not_iRAT_HoInfo2 |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support HS-DPCCH overhead reduction for multi-RAB with DCH. | REL-12 |
| 3.84 Mcps TDD downlink physical channel capability | CH-3.84_Mcps_tdd_req_sup |  |  |  | Name changed in REL-4 |
| >Maximum number of timeslots per frame | MP |  | Integer (1..14) |  |  |
| >Maximum number of physical channels per frame | MP |  | Integer (1..224) |  |  |
| >Minimum SF | MP |  | Integer (1, 16) |  |  |
| >Support of PDSCH | MP |  | Boolean | TRUE means supported |  |
| >CHOICE Support of HS-PDSCH | CV-not_iRAT_HoInfo |  |  |  | REL-5 |
| >>Supported |  |  |  |  | REL-5 |
| >>>HS-DSCH physical layer category | MP |  | Integer (1..64) | As defined in [35] | REL-5 |
| >>Unsupported |  |  |  | (no data) | REL-5 |
| >Maximum number of physical channels per timeslot | MP |  | Integer (5..16) |  |  |
| 7.68 Mcps TDD downlink physical channel capability | CH-7.68_Mcps_tdd_req_sup |  |  |  | REL-7 |
| >Maximum number of timeslots per frame | MP |  | Integer (1..14) |  | REL-7 |
| >Maximum number of physical channels per frame | MP |  | Integer (1..448) |  | REL-7 |
| >Minimum SF | MP |  | Integer (1, 32) |  | REL-7 |
| >Support of PDSCH | MP |  | Boolean | TRUE means supported | REL-7 |
| >CHOICE Support of HS-PDSCH | CV-not_iRAT_HoInfo |  |  |  | REL-7 |
| >>Supported |  |  |  |  | REL-7 |
| >>>HS-DSCH physical layer category | MP |  | Integer (1..64) | As defined in [35] | REL-7 |
| >>Unsupported |  |  |  | (no data) | REL-7 |
| >Maximum number of physical channels per timeslot | MP |  | Integer (1..32) |  | REL-7 |
| 1.28 Mcps TDD downlink physical channel capability | CH-1.28_Mcps_tdd_req_sup |  |  |  | REL-4 |
| >Maximum number of timeslots per subframe | MP |  | Integer (1..6) |  | REL-4 |
| >Maximum number of physical channels per subframe | MP |  | Integer (1..96) |  | REL-4 |
| >Minimum SF | MP |  | Integer (1, 16) |  | REL-4 |
| >Support of PDSCH | MP |  | Boolean | TRUE means supported | REL-4 |
| >CHOICE Support of HS-PDSCH | CV-not_iRAT_HoInfo |  |  |  | REL-5 |
| >>Supported |  |  |  |  | REL-5 |
| >>>HS-DSCH physical layer category | MP |  | Integer (1..64) | As defined in [35]. Values 16...64 are spares. See Note 8. | REL-5 |
| >>>HS-DSCH physical layer category extension | OP |  | Integer (1..64) | As defined in [35]. See Note 6. | REL-8 |
| >>>Multi-carrier HS-DSCH physical layer category | OP |  | Integer (1..64) | As defined in [35]. Absent if downlink multiple carrier is not supported. Values 45...64 are spares. See Note 10. | REL-7 |
| >>>Multi-carrier HS-DSCH physical layer category extension | OP |  | Integer (1..36) | As defined in [35]. Absent if downlink multiple carrier is not supported. See Note 11. | REL-8 |
| >>>Multi-carrier HS-DSCH physical layer category extension2 | OP |  | Integer (37..64) | This IE shall be used if the Multi-carrier HS-DSCH physical layer category is larger than 36. Values 45...64 are spares. See Note 11. | REL-10 |
| >>>Support of SF Mode For HS-PDSCH dual stream | OP |  | Enumerated (SF1, SF1/SF16) | For,1.28 Mcps TDD only The absence of this IE indicates that the UE does not support MIMO. | REL-8 |
| >>Unsupported |  |  |  | (no data) | REL-5 |
| >Maximum number of physical channels per timeslot | MP |  | Integer (1..16) |  | REL-4 |
| >Support of 8PSK | MP |  | Boolean | TRUE means supported | REL-4 |
| >Support of non-rectangular resource allocation | CV-not_iRAT_HoInfo2 |  | Enumerated (TRUE) | For 1.28 Mcps TDD only.The absence of this IE indicates that the UE does not support the non-rectangular resource allocation. | REL-12 |
| >Support of HS-SCCH DRX operation | OP |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support HS-SCCH DRX operation | REL-14 |
| Uplink physical channel capability information elements |  |  |  |  |  |
| FDD uplink physical channel capability | CH-fdd_req_sup |  |  |  |  |
| >Maximum number of DPDCH bits transmitted per 10 ms | MP |  | Integer (600, 1200, 2400, 4800. 9600, 19200. 28800, 38400, 48000, 57600) |  |  |
| >Support of cell reselection indication reporting | CV-not_iRAT_HoInfo2 |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support the report of Cell Reselection Indication during common E-DCH transmission. | REL-12 |
| >Support of Serving E-DCH cell decoupling | CV-not_iRAT_HoInfo2 |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support Serving E-DCH cell decoupling operation. | REL-12 |
| >Support of Radio Links without DPCH/F-DPCH | CV-not_iRAT_HoInfo2 |  | Enumerated (TRUE) | The absence of this IE indicates that the UE does not support Radio Links without DPCH/F-DPCH. | REL-12 |
| >CHOICE Support of E-DCH | CV-not_iRAT_HoInfo |  |  |  | REL-6 |
| >>Supported |  |  |  |  | REL-6 |
| >>>E-DCH physical layer category | MP |  | Integer (1..16) | As defined in [35] in Rel-6 Values 7 to 16 are spares. See Note 4. | REL-6 |
| >>>E-DCH physical layer category extension | OP |  | Integer (7) | As defined in [35]. | REL-7 |
| >>>E-DCH physical layer category extension 2 | OP |  | Integer (8..9) | As defined in [35]. See Note 14. Absence of this IE means that Dual Cell E-DCH operation is not supported. | REL-9 |
| >>>E-DCH physical layer category extension 3 | OP |  | Integer (10..12) | As defined in [35]. See Note 21. Absence of this IE means that neither 64 QAM in the uplink nor UL MIMO operation is supported. | REL-11 |

**************************************** TEXT OMITTED ***************************************

#### 10.3.6.xx	HS-SCCH DRX in CELL_FACH information

These parameters configure the UE in CELL_FACH state to discontinuously receive HS-SCCH order and HS-DSCH.

See subclause 8.6.6.xx related to usage of these IE’s.

NOTE:	Only for FDD.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| HS-SCCH DRX cycleFACH | MD |  | Enumerated (320, spare1) | Determines the actual length of the DRX Cycle during HS-SCCH DRX operation, in subframes. Default value is the value included in the IE "HS-DSCH DRX cycleFACH" of the System Information Block Type 5/5bis. | REL-14 |
| HS-SCCH RX burstFACH | MP |  | Enumerated (1, 2) | Determines the period within the DRX Cycle that the UE continuously receives HS-SCCH order, in subframes | REL-14 |

**************************************** TEXT OMITTED ***************************************

## 11.2	PDU definitions

--**************************************************************

--

-- TABULAR: The message type and integrity check info are not

-- visible in this module as they are defined in the class module.

-- Also, all FDD/TDD specific choices have the FDD option first

-- and TDD second, just for consistency.

--

--**************************************************************

PDU-definitions DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

--**************************************************************

--

-- IE parameter types from other modules

--

--**************************************************************

IMPORTS

-- Core Network IEs :

CN-DomainIdentity,

CN-InformationInfo,

CN-InformationInfo-r6,

CN-InformationInfoFull,

DomainSpecificAccessRestrictionParam-v670ext,

NAS-Message,

PagingRecordTypeID,

PagingPermissionWithAccessControlParameters,

PLMN-Identity,

**************************************** TEXT OMITTED ***************************************

CellUpdateFDD-r11	::=	SEQUENCE	{

-- User equipment IEs

u-RNTI									U-RNTI,

startList								STARTList,

am-RLC-ErrorIndicationRb2-3or4		BOOLEAN,

am-RLC-ErrorIndicationRb5orAbove		BOOLEAN,

cellUpdateCause						CellUpdateCause,

-- TABULAR: RRC transaction identifier is nested in FailureCauseWithProtErrTrId

failureCause							FailureCauseWithProtErrTrId				OPTIONAL,

rb-timer-indicator						Rb-timer-indicator,

establishmentCause						EstablishmentCause							OPTIONAL,

trafficVolumeIndicator				ENUMERATED { true }						OPTIONAL,

reconfigurationStatusIndicator		ENUMERATED { true }						OPTIONAL,

csCallType							ENUMERATED { speech, video, other, spare }	OPTIONAL,

hspdschReception-CellFach				ENUMERATED { true }						OPTIONAL,

ueMobilityStateIndicator				High-MobilityDetected						OPTIONAL,

capabilityChangeIndicator				ENUMERATED { true }						OPTIONAL,

supportOfCommonEDCH					ENUMERATED { true }						OPTIONAL,

supportOfHS-DSCHDRXOperation			ENUMERATED { true }						OPTIONAL,

supportOfMACiis						ENUMERATED { true }						OPTIONAL,

securityRevertStatusIndicator

ENUMERATED { revertedBack, normalOperation }	OPTIONAL,

supportOfHS-SCCHDRXOperation									ENUMERATED { true }						OPTIONAL,

spare3									ENUMERATED { true }						OPTIONAL,

spare2									ENUMERATED { true }						OPTIONAL,

spare1									ENUMERATED { true }						OPTIONAL,

-- Measurement IEs

measuredResultsOnRACH					MeasuredResultsOnRACHFDD-r11				OPTIONAL,

-- Other IEs

loggedMeasAvailable					ENUMERATED { true }						OPTIONAL,

loggedANRResultsAvailable				ENUMERATED { true }						OPTIONAL

}

CellUpdate-vb50ext-IEs ::= 			SEQUENCE {

-- Measurement IEs

measuredResultsOnRACHEUTRAFreq	MeasuredResultsOnRACH-EUTRAFreq			OPTIONAL

}

CellUpdate-vc50ext-IEs ::=			SEQUENCE {

-- Measurement IEs

measuredResultsOnRACH				MeasuredResultsOnRACH-vc50ext			OPTIONAL

}

-- ***************************************************

--

-- CELL UPDATE CONFIRM

--

-- ***************************************************

CellUpdateConfirm ::= CHOICE {

r3								SEQUENCE {

cellUpdateConfirm-r3			CellUpdateConfirm-r3-IEs,

v3a0NonCriticalExtensions		SEQUENCE {

cellUpdateConfirm-v3a0ext		CellUpdateConfirm-v3a0ext,

laterNonCriticalExtensions		SEQUENCE {

-- Container for additional R99 extensions

cellUpdateConfirm-r3-add-ext	BIT STRING	OPTIONAL,

v4b0NonCriticalExtensions		SEQUENCE {

cellUpdateConfirm-v4b0ext		CellUpdateConfirm-v4b0ext-IEs,

v590NonCriticalExtenstions		SEQUENCE {

cellUpdateConfirm-v590ext		CellUpdateConfirm-v590ext-IEs,

v5d0NonCriticalExtenstions		SEQUENCE {

cellUpdateConfirm-v5d0ext		CellUpdateConfirm-v5d0ext-IEs,

v690NonCriticalExtensions		SEQUENCE {

cellUpdateConfirm-v690ext		CellUpdateConfirm-v690ext-IEs,

nonCriticalExtensions			SEQUENCE {} OPTIONAL

}		OPTIONAL

}		OPTIONAL

}		OPTIONAL

}		OPTIONAL

}		OPTIONAL

}		OPTIONAL

},

later-than-r3					SEQUENCE {

rrc-TransactionIdentifier		RRC-TransactionIdentifier,

criticalExtensions				CHOICE {

r4								SEQUENCE {

cellUpdateConfirm-r4			CellUpdateConfirm-r4-IEs,

v4d0NonCriticalExtensions		SEQUENCE {

-- Container for adding non critical extensions after freezing REL-5

cellUpdateConfirm-r4-add-ext	BIT STRING		OPTIONAL,

v590NonCriticalExtenstions		SEQUENCE {

cellUpdateConfirm-v590ext		CellUpdateConfirm-v590ext-IEs,

v5d0NonCriticalExtenstions		SEQUENCE {

cellUpdateConfirm-v5d0ext		CellUpdateConfirm-v5d0ext-IEs,

v690NonCriticalExtensions		SEQUENCE {

cellUpdateConfirm-v690ext		CellUpdateConfirm-v690ext-IEs,

nonCriticalExtensions			SEQUENCE {}		OPTIONAL

}	OPTIONAL

}	OPTIONAL

}	OPTIONAL

}	OPTIONAL

},

criticalExtensions				CHOICE {

r5								SEQUENCE {

cellUpdateConfirm-r5			CellUpdateConfirm-r5-IEs,

-- Container for adding non critical extensions after freezing REL-6

cellUpdateConfirm-r5-add-ext	BIT STRING		OPTIONAL,

v5d0NonCriticalExtenstions		SEQUENCE {

cellUpdateConfirm-v5d0ext		CellUpdateConfirm-v5d0ext-IEs,

v690NonCriticalExtensions		SEQUENCE {

cellUpdateConfirm-v690ext		CellUpdateConfirm-v690ext-IEs,

nonCriticalExtensions			SEQUENCE {}		OPTIONAL

}	OPTIONAL

}	OPTIONAL

},

criticalExtensions				CHOICE {

r6								SEQUENCE {

cellUpdateConfirm-r6			CellUpdateConfirm-r6-IEs,

-- Container for adding non critical extensions after freezing REL-7

cellUpdateConfirm-r6-add-ext	BIT STRING		OPTIONAL,

v6b0NonCriticalExtensions		SEQUENCE {

cellUpdateConfirm-v6b0ext		CellUpdateConfirm-v6b0ext-IEs,

nonCriticalExtensions			SEQUENCE {}		OPTIONAL

}	OPTIONAL

},

criticalExtensions				CHOICE {

r7								SEQUENCE {

cellUpdateConfirm-r7			CellUpdateConfirm-r7-IEs,

-- Container for adding non critical extensions after freezing REL-8

cellUpdateConfirm-r7-add-ext	BIT STRING

(CONTAINING CellUpdateConfirm-r7-add-ext-IEs)	OPTIONAL,

v780NonCriticalExtensions		SEQUENCE {

cellUpdateConfirm-v780ext		CellUpdateConfirm-v780ext-IEs,

v860NonCriticalExtensions		SEQUENCE {

cellUpdateConfirm-v860ext		CellUpdateConfirm-v860ext-IEs,

nonCriticalExtensions			SEQUENCE {}		OPTIONAL

}	OPTIONAL

}	OPTIONAL

},

criticalExtensions				CHOICE {

r8								SEQUENCE {

cellUpdateConfirm-r8			CellUpdateConfirm-r8-IEs,

-- Container for adding non critical extensions after freezing REL-9

cellUpdateConfirm-r8-add-ext	BIT STRING		OPTIONAL,

v7d0NonCriticalExtensions		SEQUENCE {

cellUpdateConfirm-v7d0ext		CellUpdateConfirm-v7d0ext-IEs,

v7f0NonCriticalExtensions		SEQUENCE {

cellUpdateConfirm-v7f0ext	CellUpdateConfirm-v7f0ext-IEs,

v890NonCriticalExtensions		SEQUENCE {

cellUpdateConfirm-v890ext

CellUpdateConfirm-v890ext-IEs,

v7g0NonCriticalExtensions		SEQUENCE {

cellUpdateConfirm-v7g0ext

CellUpdateConfirm-v7g0ext-IEs,

v8a0NonCriticalExtensions		SEQUENCE {

cellUpdateConfirm-v8a0ext

CellUpdateConfirm-v8a0ext-IEs,

nonCriticalExtensions			SEQUENCE {}		OPTIONAL

}	OPTIONAL

}	OPTIONAL

}	OPTIONAL

}	OPTIONAL

}	OPTIONAL

},

criticalExtensions				CHOICE {

r9								SEQUENCE {

cellUpdateConfirm-r9			CellUpdateConfirm-r9-IEs,

-- Container for adding non critical extensions after

-- freezing REL-10

cellUpdateConfirm-r9-add-ext	BIT STRING		OPTIONAL,

v950NonCriticalExtensions		SEQUENCE {

cellUpdateConfirm-v950ext		CellUpdateConfirm-v950ext-IEs,

v9c0NonCriticalExtensions		SEQUENCE {

cellUpdateConfirm-v9c0ext		CellUpdateConfirm-v9c0ext-IEs,

nonCriticalExtensions			SEQUENCE {}		OPTIONAL

}	OPTIONAL

}	OPTIONAL

},

criticalExtensions				CHOICE {

r10								SEQUENCE {

cellUpdateConfirm-r10			CellUpdateConfirm-r10-IEs,

-- Container for adding non critical extensions after

-- freezing REL-11

cellUpdateConfirm-r10-add-ext	BIT STRING		OPTIONAL,

v9c0NonCriticalExtensions		SEQUENCE {

cellUpdateConfirm-v9c0ext		CellUpdateConfirm-v9c0ext-IEs,

vb50NonCriticalExtensions		SEQUENCE {

cellUpdateConfirm-vb50ext

CellUpdateConfirm-vb50ext-IEs,

nonCriticalExtensions		SEQUENCE {}		OPTIONAL

}	OPTIONAL

}	OPTIONAL

},

criticalExtensions				CHOICE {

r11								SEQUENCE {

cellUpdateConfirm-r11			CellUpdateConfirm-r11-IEs,

-- Container for adding non critical extensions after

-- freezing REL-12

cellUpdateConfirm-r11-add-ext

BIT STRING	OPTIONAL,

nonCriticalExtensions			SEQUENCE {}		OPTIONAL

},

criticalExtensions				CHOICE {

r12								SEQUENCE {

cellUpdateConfirm-r12	CellUpdateConfirm-r12-IEs,

-- Container for adding non critical extensions

-- after freezing REL-13

cellUpdateConfirm-r12-add-ext

BIT STRING	OPTIONAL,

nonCriticalExtensions		SEQUENCE {}		OPTIONAL

},

criticalExtensions				CHOICE {

r13								SEQUENCE {

cellUpdateConfirm-r13	CellUpdateConfirm-r13-IEs,

-- Container for adding non critical extensions

-- after freezing REL-14

cellUpdateConfirm-r13-add-ext

BIT STRING	OPTIONAL,

nonCriticalExtensions	SEQUENCE {}	OPTIONAL

},

criticalExtensions																		CellUpdateConfirm-noncriticalExtr14-IEs

}

}

}

}

}

}

}

}

}

}

}

}

CellUpdateConfirm-noncriticalExtr14-IEs ::= CHOICE {

r14					SEQUENCE {

cellUpdateConfrim-r14   CellUpdateConfirm-r14-IEs,

-- Container for adding non critical extensions

-- after freezing REL-15

cellUpdateConfirm-r14-add-ext	BIT STRING	OPTIONAL,

nonCriticalExtensions	SEQUENCE {}	OPTIONAL

},

CriticalExtensions   SEQUENCE {}

}

CellUpdateConfirm-r14-IEs ::= SEQUENCE {

-- User equipment IEs

integrityProtectionModeInfo			IntegrityProtectionModeInfo-r7		OPTIONAL,

cipheringModeInfo						CipheringModeInfo-r7					OPTIONAL,

activationTime							ActivationTime							OPTIONAL,

new-U-RNTI								U-RNTI									OPTIONAL,

new-C-RNTI								C-RNTI									OPTIONAL,

-- The IE "new-DSCH-RNTI" should not be included in FDD mode,

-- and if received the UE behaviour is unspecified

new-DSCH-RNTI							DSCH-RNTI								OPTIONAL,

new-H-RNTI								H-RNTI									OPTIONAL,

newPrimary-E-RNTI						E-RNTI									OPTIONAL,

newSecondary-E-RNTI					E-RNTI									OPTIONAL,

rntiHandlingAtCellReselection			ENUMERATED { true }					OPTIONAL,

rrc-StateIndicator						RRC-StateIndicator,

utran-DRX-CycleLengthCoeff			UTRAN-DRX-CycleLengthCoefficient-r7	OPTIONAL,

waitTime								WaitTime								OPTIONAL,

rlc-Re-establishIndicatorRb2-3or4	BOOLEAN,

rlc-Re-establishIndicatorRb5orAbove	BOOLEAN,

-- CN information elements

cn-InformationInfo						CN-InformationInfo-r6					OPTIONAL,

-- UTRAN mobility IEs

ura-Identity							URA-Identity							OPTIONAL,

supportForChangeOfUE-Capability		BOOLEAN									OPTIONAL,

-- Specification mode information

-- Radio bearer IEs

rab-InformationSetup					RAB-InformationSetup-r11				OPTIONAL,

rb-InformationReleaseList				RB-InformationReleaseList				OPTIONAL,

rb-InformationReconfigList			RB-InformationReconfigList-r11		OPTIONAL,

rb-InformationAffectedList			RB-InformationAffectedList-r8			OPTIONAL,

dl-CounterSynchronisationInfo			DL-CounterSynchronisationInfo-r5		OPTIONAL,

pdcp-ROHC-TargetMode					PDCP-ROHC-TargetMode					OPTIONAL,

retrievableConfigInfo					RetrievableConfigInfo					OPTIONAL,

otherStateConfig				OtherStateConfig					OPTIONAL,

-- Transport channel IEs

ul-CommonTransChInfo					UL-CommonTransChInfo-r12				OPTIONAL,

ul-deletedTransChInfoList				UL-DeletedTransChInfoList-r6			OPTIONAL,

ul-AddReconfTransChInfoList			UL-AddReconfTransChInfoList-r8		OPTIONAL,

dl-CommonTransChInfo					DL-CommonTransChInfo-r4				OPTIONAL,

dl-DeletedTransChInfoList				DL-DeletedTransChInfoList-r7			OPTIONAL,

dl-AddReconfTransChInfoList			DL-AddReconfTransChInfoList-r13		OPTIONAL,

-- Physical channel IEs

frequencyInfo							FrequencyInfo							OPTIONAL,

multi-frequencyInfo					Multi-frequencyInfo-LCR-r7			OPTIONAL,

dtx-drx-TimingInfo						DTX-DRX-TimingInfo-r7					OPTIONAL,

dtx-drx-Info							DTX-DRX-Info-r12						OPTIONAL,

hs-scch-LessInfo						HS-SCCH-LessInfo-r7					OPTIONAL,

mimoParameters							MIMO-Parameters-r9						OPTIONAL,

mimo4x4Parameters						MIMO4x4-Parameters						OPTIONAL,

dch-Enhancements-Info-FDD				DCH-Enhancements-Info-FDD				OPTIONAL,

maxAllowedUL-TX-Power					MaxAllowedUL-TX-Power					OPTIONAL,

ul-DPCH-Info							UL-DPCH-Info-r11						OPTIONAL,

ul-EDCH-Information					UL-EDCH-Information-r11				OPTIONAL,

ul-SecondaryCellInfoFDD				UL-SecondaryCellInfoFDD-r13			OPTIONAL,

ul-MulticarrierEDCHInfo-TDD128		UL-MulticarrierEDCHInfo-TDD128		OPTIONAL,

ul-CLTD-InfoFDD						UL-CLTD-InfoFDD						OPTIONAL,

ul-OLTD-InfoFDD						UL-OLTD-InfoFDD						OPTIONAL,

ul-OtherTTIConfiguration-Info			UL-OtherTTIConfiguration-Information	OPTIONAL,

filteredUPHReportInfo					FilteredUEPowerHeadroomReportInfo	OPTIONAL,

dl-HSPDSCH-Information				DL-HSPDSCH-Information-r12			OPTIONAL,

dl-CommonInformation					DL-CommonInformation-r12				OPTIONAL,

dl-InformationPerRL-List				DL-InformationPerRL-List-r13			OPTIONAL,

dl-SecondaryCellInfoFDD				DL-SecondaryCellInfoFDD-r11			OPTIONAL,

additionalDLSecCellInfoListFDD		AdditionalDLSecCellInfoListFDD-r11	OPTIONAL,

additionalDLSecCellInfoListFDD2		AdditionalDLSecCellInfoListFDD2		OPTIONAL,

commonERGCHInfoFDD						CommonERGCHInfoFDD						OPTIONAL,

controlChannelDRXInfo-TDD128			ControlChannelDRXInfo-TDD128-r8		OPTIONAL,

sps-Information-TDD128				SPS-Information-TDD128-r8				OPTIONAL,

mu-MIMO-Info-TDD128					MU-MIMO-Info-TDD128					OPTIONAL,

nonrectResAllocInd-TDD128				Non-rectResAllocInd-TDD128			OPTIONAL,

nonrectResSpecTSset-TDD128			Non-rectResSpecTSset-TDD128			OPTIONAL,

hs-scch-DrxCellfach-info			HS-SCCH-DrxCellfach-info				OPTIONAL,

-- MBMS IEs

mbms-PL-ServiceRestrictInfo			MBMS-PL-ServiceRestrictInfo-r6		OPTIONAL,

-- Measurement IEs for LCR

cellDCHMeasOccasionInfo-TDD128		CellDCHMeasOccasionInfo-TDD128-r9	OPTIONAL,

-- WLAN IEs

dedicatedWLANOffloadInformation		DedicatedWLANOffloadInformation		OPTIONAL

}

HS-SCCH-DrxCellfach-info ::=     SEQUENCE {

hs-scch-DrxCycleFach		 HS-SCCH-DrxCycleFach							OPTIONAL,

hs-scch-RxBurstFach                     ENUMERATED (1, 2)

}

HS-SCCH-DrxCycleFach ::=				ENUMERATED {64, spare1 }

**************************************** TEXT OMITTED ***************************************

-- ***************************************************

--

-- URA UPDATE

--

-- ***************************************************

URAUpdate ::= SEQUENCE {

-- User equipment IEs

u-RNTI							U-RNTI,

ura-UpdateCause					URA-UpdateCause,

protocolErrorIndicator			ProtocolErrorIndicatorWithMoreInfo,

laterNonCriticalExtensions			SEQUENCE {

-- Container for additional R99 extensions

uraUpdate-r3-add-ext		BIT STRING

(CONTAINING URAUpdate-r3-add-ext-IEs)		OPTIONAL,

v770NonCriticalExtensions	SEQUENCE {

uraUpdate-v770ext			UraUpdate-v770ext-IEs,

v860NonCriticalExtensions	SEQUENCE {

uraUpdate-v860ext			URAUpdate-v860ext-IEs,

va40NonCriticalExtensions		SEQUENCE {

uraUpdate-va40ext			URAUpdate-va40ext-IEs,

nonCriticalExtensions 		URAUpdate-vexyext-IEs  	OPTIONAL

}	OPTIONAL

}	OPTIONAL

}	OPTIONAL

}	OPTIONAL

}

URAUpdate-r3-add-ext-IEs ::=		SEQUENCE {

uraUpdate-v7e0ext					URAUpdate-v7e0ext-IEs,

nonCriticalExtensions				SEQUENCE {}		OPTIONAL

}

UraUpdate-v770ext-IEs ::= SEQUENCE {

-- User equipment IEs

support-hsdschReception-CellUraPch		ENUMERATED { true }				OPTIONAL,

support-hsdschReception-CellFach		ENUMERATED { true }				OPTIONAL

}

URAUpdate-v7e0ext-IEs ::=			SEQUENCE {

-- User equipment IEs

supportForTwoDRXSchemesInPCH		ENUMERATED { true }				OPTIONAL

}

URAUpdate-v860ext-IEs ::= SEQUENCE {

-- User equipment IEs

supportOfHS-DSCHDRXOperation			ENUMERATED { true }				OPTIONAL,

supportOfCommonEDCH						ENUMERATED { true }				OPTIONAL,

supportOfMACiis							ENUMERATED { true }				OPTIONAL

}

URAUpdate-va40ext-IEs ::= SEQUENCE {

loggedMeasAvailable		ENUMERATED { true }	OPTIONAL,

loggedANRResultsAvailable		ENUMERATED { true }		OPTIONAL

}

URAUpdate-vexyext-IEs ::= SEQUENCE {

supportOfHS-SCCHDRXOperation			ENUMERATED { true }				OPTIONAL

nonCriticalExtensions 					SEQUENCE {}					 	OPTIONAL

}

**************************************** TEXT OMITTED ***************************************

RadioBearerReconfiguration-r14-IEs ::= SEQUENCE {

-- User equipment IEs

integrityProtectionModeInfo		IntegrityProtectionModeInfo-r7				OPTIONAL,

cipheringModeInfo				CipheringModeInfo-r7						OPTIONAL,

activationTime					ActivationTime								OPTIONAL,

dynamicActivationTime			DynamicActivationTime						OPTIONAL,

delayRestrictionFlag			DelayRestrictionFlag						OPTIONAL,

new-U-RNTI						U-RNTI										OPTIONAL,

new-C-RNTI						C-RNTI										OPTIONAL,

-- The IE "new-DSCH-RNTI" should not be included in FDD mode,

-- and if received the UE behaviour is unspecified

new-DSCH-RNTI					DSCH-RNTI									OPTIONAL,

new-H-RNTI						H-RNTI										OPTIONAL,

newPrimary-E-RNTI				E-RNTI										OPTIONAL,

newSecondary-E-RNTI			E-RNTI										OPTIONAL,

rntiHandlingAtCellReselection			ENUMERATED { true }	OPTIONAL,

rrc-StateIndicator				RRC-StateIndicator,

ueMobilityStateIndicator		High-MobilityDetected						OPTIONAL,

utran-DRX-CycleLengthCoeff	UTRAN-DRX-CycleLengthCoefficient-r7		OPTIONAL,

accessGroupIdentity			AccessGroupIdentity						OPTIONAL,

-- Core network IEs

cn-InformationInfo				CN-InformationInfo-r6						OPTIONAL,

signallingConnectionRelIndication	CN-DomainIdentity				OPTIONAL,

-- UTRAN mobility IEs

ura-Identity					URA-Identity								OPTIONAL,

supportForChangeOfUE-Capability	BOOLEAN									OPTIONAL,

responseToChangeOfUE-Capability	ENUMERATED { true }							OPTIONAL,

-- Specification mode information

defaultConfigForCellFACH		DefaultConfigForCellFACH					OPTIONAL,

specificationMode				CHOICE {

complete						SEQUENCE {

-- Radio bearer IEs

rab-InformationSetupList		RAB-InformationSetupList-r11		OPTIONAL,

rab-InformationReconfigList	RAB-InformationReconfigList-r8	OPTIONAL,

rab-InformationMBMSPtpList	RAB-InformationMBMSPtpList		OPTIONAL,

rb-InformationReconfigList	RB-InformationReconfigList-r11	OPTIONAL,

rb-InformationAffectedList	RB-InformationAffectedList-r8		OPTIONAL,

rb-InformationReleaseList		RB-InformationReleaseList   OPTIONAL,

rb-PDCPContextRelocationList	RB-PDCPContextRelocationList		OPTIONAL,

pdcp-ROHC-TargetMode			PDCP-ROHC-TargetMode				OPTIONAL,

umRlc-ReEstablishmentRBList	RB-IdentityList					OPTIONAL,

otherStateConfig				OtherStateConfig					OPTIONAL,

-- Transport channel IEs

ul-CommonTransChInfo			UL-CommonTransChInfo-r12			OPTIONAL,

ul-deletedTransChInfoList		UL-DeletedTransChInfoList-r6		OPTIONAL,

ul-AddReconfTransChInfoList	UL-AddReconfTransChInfoList-r8	OPTIONAL,

dl-CommonTransChInfo			DL-CommonTransChInfo-r4			OPTIONAL,

dl-DeletedTransChInfoList		DL-DeletedTransChInfoList-r7		OPTIONAL,

dl-AddReconfTransChInfoList	DL-AddReconfTransChInfoList-r13	OPTIONAL,

-- Retrievable configuration

retrievableConfigInfo			RetrievableConfigInfo				OPTIONAL

},

preconfiguration				SEQUENCE {

-- All IEs that include an FDD/TDD choice are split in two IEs for this message,

-- one for the FDD only elements and one for the TDD only elements, so that one

-- FDD/TDD choice in this level is sufficient.

preConfigMode					CHOICE {

predefinedConfigIdentity		PredefinedConfigIdentity,

defaultConfig					SEQUENCE {

defaultConfigMode				DefaultConfigMode,

defaultConfigIdentity 			DefaultConfigIdentity-r6

}

}

}

},

-- Physical channel IEs

frequencyInfo					FrequencyInfo								OPTIONAL,

multi-frequencyInfo			Multi-frequencyInfo-LCR-r7				OPTIONAL,

dtx-drx-TimingInfo				DTX-DRX-TimingInfo-r7						OPTIONAL,

dtx-drx-Info					DTX-DRX-Info-r12							OPTIONAL,

hs-scch-LessInfo				HS-SCCH-LessInfo-r7						OPTIONAL,

mimoParameters					MIMO-Parameters-r9							OPTIONAL,

mimo4x4Parameters				MIMO4x4-Parameters							OPTIONAL,

dch-Enhancements-Info-FDD		DCH-Enhancements-Info-FDD					OPTIONAL,

maxAllowedUL-TX-Power			MaxAllowedUL-TX-Power						OPTIONAL,

ul-DPCH-Info					UL-DPCH-Info-r11							OPTIONAL,

ul-EDCH-Information			UL-EDCH-Information-r11					OPTIONAL,

ul-SecondaryCellInfoFDD		UL-SecondaryCellInfoFDD-r13				OPTIONAL,

ul-MulticarrierEDCHInfo-TDD128	UL-MulticarrierEDCHInfo-TDD128		OPTIONAL,

ul-CLTD-InfoFDD				UL-CLTD-InfoFDD							OPTIONAL,

ul-OLTD-InfoFDD				UL-OLTD-InfoFDD							OPTIONAL,

ul-OtherTTIConfiguration-Info	UL-OtherTTIConfiguration-Information		OPTIONAL,

filteredUPHReportInfo			FilteredUEPowerHeadroomReportInfo		OPTIONAL,

dl-HSPDSCH-Information		DL-HSPDSCH-Information-r12				OPTIONAL,

dl-CommonInformation			DL-CommonInformation-r12					OPTIONAL,

dl-InformationPerRL-List		DL-InformationPerRL-List-r13				OPTIONAL,

dl-SecondaryCellInfoFDD		DL-SecondaryCellInfoFDD-r11				OPTIONAL,

additionalDLSecCellInfoListFDD	AdditionalDLSecCellInfoListFDD-r11	OPTIONAL,

additionalDLSecCellInfoListFDD2	AdditionalDLSecCellInfoListFDD2		OPTIONAL,

commonERGCHInfoFDD				CommonERGCHInfoFDD							OPTIONAL,

dPCCH2InfoFDD					DPCCH2InfoFDD								OPTIONAL,

controlChannelDRXInfo-TDD128	ControlChannelDRXInfo-TDD128-r8			OPTIONAL,

sps-Information-TDD128		SPS-Information-TDD128-r8					OPTIONAL,

mu-MIMO-Info-TDD128			MU-MIMO-Info-TDD128						OPTIONAL,

nonrectResAllocInd-TDD128		Non-rectResAllocInd-TDD128				OPTIONAL,

nonrectResSpecTSset-TDD128	Non-rectResSpecTSset-TDD128				OPTIONAL,

hs-scch-DrxCellfach-info			HS-SCCH-DrxCellfach-info		OPTIONAL,

-- MBMS IEs

mbms-PL-ServiceRestrictInfo	MBMS-PL-ServiceRestrictInfo-r6			OPTIONAL,

-- Measurement IEs for LCR

cellDCHMeasOccasionInfo-TDD128	CellDCHMeasOccasionInfo-TDD128-r9	OPTIONAL

}

**************************************** TEXT OMITTED ***************************************

## 11.3	Information element definitions

InformationElements DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

hiPDSCHidentities,

hiPUSCHidentities,

hiRM,

maxAC,

maxAdditionalMeas,

maxAddPos,

maxMultipleFrequencyBandsEUTRA,

maxMultipleFrequencyBandsFDD,

maxASC,

maxASCmap,

maxASCpersist,

maxBeacons,

maxBTs,

maxCCTrCH,

maxCellMeas,

maxCellMeas-1,

maxCellMeas-ext,

maxCellMeas-ext2,

maxCellMeas-ext-1,

**************************************** TEXT OMITTED ***************************************

PhysicalChannelCapability-vb80ext ::=	SEQUENCE {

fddPhysChCapability						SEQUENCE {

downlinkPhysChCapability					DL-PhysChCapabilityFDD-vb80ext

}		OPTIONAL

}

PhysicalChannelCapability-vc50ext ::=	SEQUENCE {

fddPhysChCapability						SEQUENCE {

downlinkPhysChCapability					DL-PhysChCapabilityFDD-vc50ext	OPTIONAL,

uplinkPhysChCapability					UL-PhysChCapabilityFDD-vc50ext	OPTIONAL

}		OPTIONAL,

tddPhysChCapability						SEQUENCE {

downlinkPhysChCapability					DL-PhysChCapabilityInfoTDD-128-vc50ext

}		OPTIONAL

}

PhysicalChannelCapability-vd20ext ::=	SEQUENCE {

fddPhysChCapability						SEQUENCE {

downlinkPhysChCapability					DL-PhysChCapabilityFDD-vd20ext	OPTIONAL,

uplinkPhysChCapability					UL-PhysChCapabilityFDD-vd20ext	OPTIONAL

}		OPTIONAL

}

PhysicalChannelCapability-vexyext ::=	SEQUENCE {

fddPhysChCapability						SEQUENCE {

downlinkPhysChCapability					DL-PhysChCapabilityFDD-vexyext	OPTIONAL

}		OPTIONAL

}

**************************************** TEXT OMITTED ***************************************

UE-RadioAccessCapability-vexyext-IEs ::=		SEQUENCE {

measurementCapability						MeasurementCapability-vexyext		OPTIONAL,

supportofSimultaneousSetupandReleaseofRABsRBs		ENUMERATED { true }				OPTIONAL,

physicalChannelCapability				PhysicalChannelCapability-vexyext		OPTIONAL

}

**************************************** TEXT OMITTED ***************************************

# 13	Protocol timers, counters, other parameters and default configurations

The information provided in subclauses 13.1 and 13.2 shall be treated as informative. The normative text is specified in the relevant subclauses in clause 8 and clause 8 shall prevail.

## 13.1	Timers for UE

| Timer | Start | Stop | At expiry |
| --- | --- | --- | --- |
| T300 | Transmission of RRC CONNECTION REQUEST in case of connection establishment for reasons other than MBMS reception | Reception of RRC CONNECTION SETUP | Retransmit RRC CONNECTION REQUEST if V300 =< N300, else go to Idle mode |
| T302 | Transmission of CELL UPDATE/URA UPDATE | Reception of CELL UPDATE CONFIRM/URA UPDATE CONFIRM | Retransmit CELL UPDATE/URA UPDATE if V302 =< N302, else, go to Idle mode |
| T304 | Transmission of UE CAPABILITY INFORMATION | Reception of UE CAPABILITY INFORMATION CONFIRM | Retransmit UE CAPABILITY INFORMATION if V304 =< N304, else initiate a cell update procedure |
| T305 | Entering CELL_FACH or URA_PCH or CELL_PCH state. Reception of CELL UPDATE CONFIRM/URA UPDATE CONFIRM. | Entering another state. | Transmit CELL UPDATE if T307 is not activated and the UE detects "in service area". Otherwise, if T307 is not active, start T307. |
| T307 | When the timer T305 has expired and the UE detects "out of service area". | When the UE detects "in service area". | Transit to idle mode |
| T308 | Transmission of RRC CONNECTION RELEASE COMPLETE | Not stopped | Transmit RRC CONNECTION RELEASE COMPLETE if V308 <=N308, else go to idle mode. |
| T309 | Upon reception of CELL CHANGE ORDER FROM UTRAN message | Successful response to a connection establishment request in the new cell. | Resume the connection to UTRAN |
| T310 | Transmission of PUSCH CAPACITY REQUEST | Reception of PHYSICAL SHARED CHANNEL ALLOCATION | Transmit PUSCH CAPACITY REQUEST if V310 =< N310, else procedure stops. |
| T311 | Reception of PHYSICAL SHARED CHANNEL ALLOCATION message with the CHOICE "PUSCH allocation" set to "PUSCH allocation pending". | Reception of PHYSICAL SHARED CHANNEL ALLOCATION message with CHOICE "PUSCH allocation" set to "PUSCH allocation assignment". | UE may initiate a PUSCH capacity request procedure. |
| T312 | When the UE starts to establish dedicated CH. For 1.28 Mcps TDD, it can also apply for physical shared channel establishment. For FDD, timer T312 runs independently for the downlink frequency associated with the primary uplink frequency and for the downlink frequency associated with the secondary uplink frequency. | When the UE detects N312 "in sync" indication from L1. | The criteria for physical channel establishment failure is fulfilled |
| T313 | When the UE detects consecutive N313 "out of sync" indication from L1. For FDD, timer T313 runs independently for the downlink frequency associated with the primary uplink frequency and for the downlink frequency associated with the secondary uplink frequency. | When the UE detects consecutive N315 "in sync" indication from L1. | The criteria for Radio Link failure is fulfilled. |
| T314 | When the criteria for radio link failure are fulfilled. The timer is started if radio bearer(s) that are associated with T314 exist or if only RRC connection exists only to the CS domain. | When the Cell Update procedure has been completed. | See subclause 8.3.1.13. |
| T315 | When the criteria for radio link failure are fulfilled. The timer is started only if radio bearer(s) that are associated with T315 exist or if RRC connection exists to PS domain. | When the Cell Update procedure has been completed. | See subclause 8.3.1.14. |
| T316 | When the UE detects "out of service area" in URA_PCH or CELL_PCH state | When the UE detects "in service area". | Initiate cell update procedure if in service area is detected. Otherwise start timer T317, transit to CELL_FACH state and initiate cell update procedure when the UE detects "in service area". |
| T317 | When the T316 expires or when in CELL_FACH state, the UE detects "out of service area". | When the UE detects "in service area". | T317 never expires. |
| T318 | Transmission of RRC CONNECTION REQUEST in case of connection establishment for MBMS reception | Reception of RRC CONNECTION SETUP | Enter idle mode |
| T319 | When entering CELL_PCH or URA_PCH. | When leaving CELL_PCH or URA_PCH state. | The UE activates starts the DRX cycle based on “DRX cycle length coefficient”. |
| T320 | When receiving the CELL UPDATE CONFIRM message with IE “Wait time” and IE “RRC State Indicator” set to the value “CELL_PCH” or “URA_PCH”. | When initiating the cell update or URA update procedure, see subclause 8.3.1.2. | See subclause 8.3.1.16. |
| T321 | See subclause 8.5.49 and 8.5.xy. | See subclause 8.5.49 and 8.5.xy. | See subclause 8.5.49 and 8.5.xy. |

**************************************** TEXT OMITTED ***************************************

### 13.4.8oxx	HS_SCCH_DRX_CELL_FACH_STATUS

This variable indicates whether HS-SCCH DRX operation is supported in CELL_FACH. See subclause 8.5.48 & 8.6.6.xx for actions related to the setting of this variable.

NOTE:	FDD only.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description | Version |
| --- | --- | --- | --- | --- | --- |
| HS-SCCH DRX in CELL_FACH status | MP |  | Boolean | TRUE: HS-SCCH DRX operation in CELL_FACH is supported. Set to FALSE when leaving CELL_FACH or when dedicated H-RNTI is cleared. | REL-14 |

****************************************** TEXT END ******************************************