3GPP TSG-RAN WG2 Meeting #96	R2-167711

Reno, USA, 14 - 18 November 2016

Agenda item:	9.3.1.1.1

Source:	Nokia, Alcatel-Lucent Shanghai Bell

Title:	Downlink Mobility Measurements in Connected Mode

WID/SID:	WI Code - Release 14

Document for:	Discussion and Decision

# Introduction

Agreements of RAN2#95bis

Agreements for DL-based mobility in RRC_CONNECTED mode (optimized for data transmission, at least for network-controlled mobility) mobility with RRC involvement, concerning beams and the relation to the NR cell definition:

1. UE at least measures one or more individual beams and gNB should have mechanisms to consider those beams to perform HO. Note: This is necessary at least to trigger inter-gNB handovers and to optimize HO ping-pongs / HO failures.

–	FFS: whether UE report individual and/or combined quality of multiple beams

2. UE should be able to distinguish between the beams from its serving cell and beams from non-serving cells for RRM measurements in active mobility. UE should be able to determine if a beam is from its serving cell.

–	FFS whether serving/non serving cell may be termed 'serving/non serving set of beam)

–	FFS: whether the UE is informed via dedicated signalling or implicitly detected by the UE based on some broadcast signals.

-	FFS how the cell in connected relates to the cell in idle

3. 	Study how to derive a cell quality based on measurements from individual beams

4. 	In connected mode, intra-cell mobility can be handled by mobility without RRC involvement.

-FFS whether there may be cases that do require RRC involvement.

5	UE should be able to identify a beam. FFS how beams are identified (to be defined by RAN1)

Agreement

1	For connected mode, cell level signal quality for RRM measurement can be derived from multiple beams, if multiple beams can be detected (this does not preclude RRM measurement made on a single beam)

FFS how to combine beam measurements to a cell level single quality

Agreements in RAN1#86

Agreements:

- For L3 mobility based on DL measurement in CONNECTED mode UE:

  - At least non-UE-specific DL signals can be used for CONNECTED mode RRM measurement

    - FFS UE-specific DL signals for this purpose

  - Study the following DL signals for CONNECTED mode RRM measurement

    - Option 1: Cell related RS which is carrying Cell-ID (e.g. NR-PSS, NR-SSS)

    - Option 2: RS for mobility

      - FFS how to associate it with beam-ID and/or Cell-ID

    - Option 3: RS for demodulating broadcast channel

    - Option 4: A combination of option 1 and 2

    - Other options are not precluded

  - At least one of cell-level and beam-level measurement quantities is supported for RRM reporting.

    - FFS which RRM measurement quantities to define, e.g., RSRP, RSRQ

  - Study the following options for RRM measurement quantities to be reported for L3 mobility:

    - Option 1: derived per cell (e.g., if multi-beam, as a function of multi-beam measurements)

    - Option 2: derived per beam

    - Option 3: A combination of option 1 and 2

    - Other options are not precluded

FFS for other UE states (if introduced by RAN2)

In this contribution we discuss the connected mode measurements for NR and propose a measurement model for deriving beam and cell qualities in NR which applies for both intra- and inter cell measurements in single and multi-beam systems.

# 2	Single Beam and Multi Beam operation

To provide background information for the proposed measurement model for NR a single beam and multi beam operation modes are discussed briefly.

Considering a single beam based deployment from intra cell perspective, to manage UE mobility inside one cell, the network needs to have knowledge which TRP or TRPs can be used to serve UE. In one approach each TRP transmits identifiable reference signal that identifies a beam. In a single beam based operation this beam may also identify a TRP but this would not be visible to UE as the same reference signal may be transmitted using one or multiple TRPs. If two or more TRPs share the same reference signals, from UE perspective they would be seen as one; UE measures and detects reference signals transmitted by TRPs but not the TRPs. When UE reports the measurements on the reference signals the network determines the association between TRP and the reference signals.

In a multi beam based operation, a single TRP may have multiple beams (Figure 1) which need to be identifiable by UE for mobility purposes. To facilitate this, each beam is used to transmit a different reference signal a beam reference signal. From UE perspective it would detect different reference signals (transmitted using different beams) but the TRPs used to transmit would not be visible as UE measures reference signals. As a further example, same reference signal is transmitted by using multiple beams concurrently, UE would not be able to detect multiple beams as it measures the reference signals.

Figure 1 Illustration of a single beam and multi beam cells

# 3	Measurement Model for NR

Based on the discussion in the previous contribution in RAN2#95bis [R2-166166] the single beam and multi beam operation mode measurement in NR could be harmonized to a single measurement model. Regardless of the operation mode or frequency band (Low frequency/High frequency), UE would perform mobility measurements and reporting based on DL signals (without identifying a TRP or TRPs used to transmit the signals) beam level measurements and reporting in CONNECTED mode.

In NR the mobility measurements, in addition to inter-cell mobility, should provide mechanism to handle intra-cell mobility. It would be desirable to have single model for intra/inter cell mobility measurements so that it applies for both single and multi-beam operation modes.

LTE principles could also be applied as baseline (with some enhancements or simply reused) for NR. To manage measurements and reporting in a beam-based system the following high level functions are considered by the proposed model:

- Physical layer measurement samples (L1) on reference signals

- Higher layer filtering (e.g. moving average) of the measurement samples to determine beam quality/qualities

  - Layer 2 filtering

- Higher layer filtering (e.g. moving average) and mechanism to determine cell quality based on individual beam qualities

  - Layer 3 filtering

- Time To Trigger principle for intra-cell and inter-cell events

- Signal quality threshold levels for beam level and cell level reporting actions

### 3.1 Beam Quality vs. Cell Quality

In Figure 2 we provide the example measurement model to cover intra and inter cell measurements based on beam specific reference signals. The use of other signals for measurements, e.g. NR- Synchronization Signal (NR-SS) is not excluded by the model.

The model follows LTE principles: for each L1 measurement there is a corresponding higher layer filtered beam level sample and higher layer filtered cell quality sample. Measurement model provides one quality value for each beam and one quality value per cell. Two levels of filtering are considered to cover the intra-cell and inter cell mobility.

As intra cell mobility is considered to have minimum involvement of RRC signaling it can be done quicker and it allows to base the mobility decisions on shorter term evaluations to be more reactive to the changing channel conditions. In contrast, the inter cell handover is RRC based and therefore most likely heavier procedure at both UE and network side meaning the handover decisions for inter-cell would require good longer term evaluation of the cell to minimize the ping-pong effect and handover failure rates.

Figure 2. Example measurement model for beam and cell quality measurements

Figure 2 consists of two functions using the same measurement samples provided by L1 filtering entity. Here we provide a high level descriptions how the measurement model works:

Beam Quality

- L1 measurements done per physical layer signal per beam reference signal

- Higher layer filtering (L2 / MAC layer is assumed to have filtering as RRC is not involved) is applied per beam

  - L2 Filtering parameters are configured by network

  - L2 Filtering is used to determine that criteria related to beam measurements are evaluated based on stable measurements.

  - L2 filtering stabilizes the beam measurements for intra-cell beam events (e.g. to evaluate whether to trigger beam management actions such as beam reporting or beam recovery) and measurements provided to the selection function

- Filtered Beam measurements are used to evaluate criteria, e.g. for beam management actions of the serving cell and for e.g. beam quality reporting in an inter-cell measurement report (beam report)

Cell Quality

- Beam specific measurement samples are reused for determining cell quality

  - Filtered values provide further input to measurement report triggered by cell level reporting criteria

- Selection function is applied to filtered beam measurements (e.g. maximum RSRP)

- This value is further filtered to obtain long term cell quality value. A single value is derived for the cell and used to evaluate criteria for inter-cell event and potential action such as measurement reporting.

- Similar selection function is run for serving cell and neighbor cell measurements. UE derives a single cell quality value out of filtered beam measurement results

Observation 1: Same measurement samples can be used for determining individual beam quality levels and cell quality

Observation 2: Having two levels of filtering provides network the means to optimize handover performance in the network by configuration of measurement report triggering

Proposal 1: Consider L2 filtering of beam level measurement samples for intra-cell beam management (mobility without RRC involvement)

Proposal 2: Consider L3 filtering and selection function for inter-cell mobility (with RRC involvement).

Proposal 3: Selection function in the proposed model selects the highest RSRP value

### 3.2 Time-To-Trigger

Time to trigger parameter is used in LTE to evaluate measurement reporting criteria for configured time. Similar principle can be reused in NR with some enhancement or options for network to optimize the UE reporting. Separate TTT parameters should be used for intra-cell and inter cell reporting criteria/action. One way to configure the values would be to use:

  - Shorter TTT value for evaluating trigger for beam level reporting (to quickly react to beam level events such as reporting new beams or quality change on the current serving beam/set of serving beams etc..)

  - Longer TTT value for evaluating trigger for cell level events (measurement reports)

The used filtering coefficients, threshold and TTT values are configured by network by using e.g. RRC signaling.

Observation 3: Separate Time to Trigger parameters for triggering beam level reporting and cell level evaluation of reporting/action criteria could be beneficial in NR.

Proposal 4: Consider reusing the TTT concept of LTE and consider two different TTT parameters in NR for evaluation of reporting/action criteria, intra cell (TTTbeam) and inter cell (TTTcell).

### 3.3 Example Operation of the Measurement model

Figure 3 illustrates the derivation of cell quality based on the beam RSRP measurements. Two beams are illustrated in this scenario: Beam1 and Beam2. In the proposed model the selection function utilizes the highest measured RSRP to represent the cell quality: Time-to-Trigger runs based on the Cell RSRP. Beam1 is above the TH1 which initiates the TTT for the cell. After a while measurements indicate that Beam1 level falls below the TH1 but Beam2 value is above. This keeps the TTT running for cell level measurement. Once TTT timer has elapsed a criteria for cell level measurement event can be triggered e.g. measurement report. Furthermore, similar evaluation is run per cell based on beam level measurements.

Figure 3. Example of evaluating a cell quality with the proposed model

Figure 4 illustrates one example of beam level measurements and event/action triggering based on beam level threshold (TH2) and TTT for determining beam level events/actions. Three beams (1, 2, 3) are considered in this example. For the Beam 3, which is constantly above a specific TH2, no actions are determined. For Beam 2 the RSRP is observed to be above the TH2 for the duration of TTT (beam) it may trigger specific actions e.g. beam management related reporting. After a while it is observed that the Beam2 RSRP level falls below TH2 and stays there for the duration of TTT, another action may be triggered. Beam 1 quality exceeds the TH2 but does not stay above the threshold for the duration of TTT thus no actions are triggered.

Figure 4.. Example of evaluating beam qualities with the proposed model

## 3.3	UE beamforming and Mobility Measurements

One further aspect regarding the NR measurement is to consider how the UE beamforming and potential implications are handled in the measurement model and also in the RRM measurements in general.

In higher frequencies UE may have multiple potential beam directions it can use for beam measurements. It may be able to detect multiple beams per its beam direction. If we consider e.g. the derivation of cell quality, UE would only form one value for each cell regardless of how many different beam directions it used for the measurements.

As another example if UE has obtained measurements on same downlink beam using multiple different UE beams it should use one RSRP value per downlink beam (e.g. the highest measured RSRP) when considering criteria for beam reporting/beam management actions.

While the UE beamforming is not visible in the measurement model it may be beneficial to have it “visible” when actually reporting measurements. If UE is able to detect e.g. two distinct sets of downlink beams (i.e. beam groups) using two different RX directions, it may report the beams per direction.

How to manage and what information are included in intra/inter cell beam measurement reports is FFS and should be network configurable.

Observation 4: UE beamforming should not be visible in the measurements used to evaluate reporting criteria but may be visible in a beam information which is reported to network

Proposal 5: Consider NR measurement model where UE beamforming is not visible. UE beamforming may be visible when reporting measurements.

# 4 	Conclusions

Observation 1: Same measurement samples can be used for determining individual beam quality levels and cell quality

Observation 2: Having two levels of filtering provides network the means to optimize handover performance in the network by configuration of measurement report triggering

Observation 3: Separate Time to Trigger parameters for triggering beam level reporting and cell level evaluation of reporting/action criteria could be beneficial in NR.

Observation 4: UE beamforming should not be visible in the measurements used to evaluate reporting criteria but may be visible in a beam information which is reported to network

Proposal 1: Consider L2 filtering of beam level measurement samples for intra-cell beam management (mobility without RRC involvement)

Proposal 2: Consider L3 filtering and selection function for inter-cell mobility (with RRC involvement).

Proposal 3: Selection function in the proposed model selects the highest RSRP value

Proposal 4: Consider reusing the TTT concept of LTE and consider two different TTT parameters in NR for evaluation of reporting/action criteria, intra cell (TTTbeam) and inter cell (TTTcell).

Proposal 5: Consider NR measurement model where UE beamforming is not visible. UE beamforming may be visible when reporting measurements.

# References

- R2-166166, Discussion on Cell Definition in New Radio Nokia, Alcatel-Lucent Shanghai Bell