3GPP TSG-RAN WG2 Meeting #96	R2-167713

Reno, USA, 14 - 18 November 2016

Agenda item:	9.3.1.1.2

Source:	Nokia, Alcatel-Lucent Shanghai Bell

Title:	Beam Recovery In NR

WID/SID:	WI Code - Release 14

Document for:	Discussion and Decision

# 1	Introduction

RAN1#86bis made the following agreements for beam management [3]:

Agreements: - UL beam management is to be further studied in NR - Similar procedures can be defined as DL beam management with details FFS, e.g.: - U-1: is used to enable TRP measurement on different UE Tx beams to support selection of UE Tx beams/TRP Rx beam(s) - Note: this is not necessarily useful in all cases - U-2: is used to enable TRP measurement on different TRP Rx beams to possibly change/select inter/intra-TRP Rx beam(s) - U-3: is used to enable TRP measurement on the same TRP Rx beam to change UE Tx beam in the case UE uses beamforming - FFS Indication of information related to Tx/Rx beam correspondence is supported - Study UL beam management based on: - PRACH - SRS - DM-RS - Other channels and reference signals are not precluded - Study uplink beam management procedure by considering the Tx/Rx beam correspondence - For the case of TRP and UE have Tx/Rx beam correspondence - For the case of TRP has no Tx/Rx beam correspondence and/or UE has no Tx/Rx beam correspondence … Agreements: - NR supports mechanism(s) in the case of link failure and/or blockage for NR - Whether to use new procedure is FFS - Study at least the following aspects: - Whether or not an DL or UL signal transmission for this mechanism is needed - E.g., RACH preamble sequence, DL/UL reference signal, control channel, etc. - If needed, resource allocation for this mechanisms - E.g., RACH resource corresponding mechanism, etc.

In this contribution we discuss beam recovery and related procedures in downlink beam management.

# 2	Discussion

Beam, or link recovery in this contribution discusses when a beam or link loss is detected but alternative link still exist for communication between UE and network.

In NR the link may be lost / degrade due to various reasons especially in higher frequencies e.g. due to beam blockage, beam misalignment or changes in radio environment so that the current communication beams do not provide adequate quality for communication. Although the current link cannot be used, UE may have alternative links that are usable. When there exist alternative link(s) for communication the connection between UE and network should be recovered via beam/link recovery procedure.

### 2.1 Measurements on periodical beam reference signals

In downlink beam management UE measures periodical beam reference signals (BRS) during sweeping sub frames. Based on the periodical measurements UE determines potential beams for communication and reports them to network based on configuration. UE may report multiple potential beams, candidate beams, to network. UE may also have multiple distinctive beam directions and thus it may report the potential candidate beams or beam groups per beam direction.

Periodical signals also enable UE to track the serving beam or set of serving beams and indicate link quality changes to network by triggering UE initiated measurement reports. In addition, UE is able to detect potential link failures and initiate recovery actions based on the periodical measurements.

Observation 1: UE is able detect link failure / blockage on periodic beam reference signals

Proposal 1: Support the use of periodical beam reference signals on downlink for beam management in NR

Proposal 2: Support the use of periodical beam reference signals for UE initiated beam recovery in NR

#### 2.1 UE event triggered Beam Recovery Procedures

As UE measures the BRS signals periodically, it is the first entity to detect the link failure / blockage during inactivity. During the active communication network may detect the link degradation e.g. if UE does not send any data on allocated resource or if UE does not send any feedback to DL transmission. Vice versa UE may observe link failure/blockage in similar manner although it may not expect any predetermined schedule of resource allocations unless configured. Monitoring the current link outside BRS periods may be useful in case the BRS periodicity is long but defining UE based actions on DATA/ACK-NACK signalling so that it could be controlled by network is difficult.

Thus, UE should determine link failure/blockage based on the reception quality of beam reference signals associated with PDCCH reception

Observation 2: UE triggered recovery actions should be based on the BRS signals and UE actions should be controlled or configurable by network.

Proposal 3: Determine link failure from beam reference signal(s) associated to serving beam(s) of downlink control channel reception.

Proposal 4: Support UE event triggered beam recovery procedures/actions in NR that are based on BRS measurements.

When network determines the link degradation (but the link is not lost yet) it may try use alternative beams in the candidate set or request UE to provide up to date beam information e.g. by triggering a request using DCI/MAC CE. Although the link is degraded UE may be able to provide beam report indicating alternative beams (e.g. different beam group) after next BRS measurement.

#### 2.2.1 Non contention based Recovery Procedures

In one option UE could be to configured with dedicated uplink transmission resource(s) for beam recovery e.g. to request resources to send beam report indicating new serving beam or set of serving beams.

The UE would trigger transmission on the resource e.g. when it loses the current serving NR-PDCCH beam(s) or some other triggering condition is fulfilled related to beam recovery initiation. Resource(s) may be associated to some candidate beam(s) the UE is having. This approach has a drawback requiring periodic resources in practice but may be provide shorter recovery latency than random access based recovery procedure. As a fallback mechanism, random access procedure could be performed if recovery is not successful.

Observation 3: Considering non-contention based mechanisms for beam recovery may speed up the beam recovery

Proposal 5: Support UE event triggered UL signal based beam recovery procedures/actions in NR that can be configured by network

#### 2.2.2 Recovery via random access procedure

Random access procedure may be used as fallback mechanism when the link cannot be recovered by other means but UE has detected potential beams in a current serving cell. In one option using RA-procedure UE indicates the strongest/adequate beam by sending msg1 on resource corresponding to specific set of beams or potentially on resource corresponding a specific beams (re-acquire serving cell DL and UL beams, as described in R1-1610288). Network is able to detect the link recovery e.g. based on the UE identity in msg3. When the link has been re-established network may request UE to provide full beam feedback. Alternatively as UE had measured the potential beams before initiating random access procedure network may request to provide beam information already in msg3.

Use of RA-procedure has a drawback of being dependent on RACH resource periodicity and prone to collisions and thus additional delay.

Proposal 6: Consider beam recovery via random access procedure that provides initial beam acquisition, and acts as a fallback when link cannot be recovered by using other configured mechanisms.

##### 3.2 Discussion on Network Beam Recovery Actions

In network based beam recovery actions, the network is the entity triggering the beam recovery actions. Network may be able to detect link degradation e.g. on UE uplink transmission quality (data /ACK-NACK) or simply based on missing transmission when UE scheduled or missing feedback. In another case UE may have been configured with periodical transmission of uplink signal for network to track. Such periodical UL signal could be e.g. SRS or dedicated signal just to indicate connectivity to the network. When the UL signal is not received by the network on specific time instance it may determine that link needs to be recovered.

Periodical UL signal may be used to maintain the link between UE and network when there is no communication. UE would transmit periodical signal e.g. to indicate that current candidate beam set is still valid (beams are above the quality threshold) and UE’s beam is aligned. The periodicity of the signal may follow e.g. UE DRX cycle. When the network detects the link is lost it may try reach UE by scheduling on alternate candidate beams. Doing a blind scheduling on candidate beams may be costly in resource wise and periodical uplink signal may have an impact on UE power consumptions.

In case the degradation of transmission quality network may try to schedule UE on previously reported alternate candidate beams. Scheduling may recovery the link in case the current serving beam was blocked but the UE beam alignment was still valid.

Alternatively UE may have measured and detected candidate beams in multiple beam directions and reported them to network e.g. via means of beam grouping e.g. so that candidate beams of one beam direction are reported in same group. Network may try to reach UE using the candidate beams of alternate beam groups knowing that if one beam direction is blocked, alternate directions may be used. However, without at least some level of pre-agreement how the UE would monitor each reported direction, trying the alternate beams may become very costly in resource wise.

Observation 4: Network initiated beam recovery actions may have high resource cost without preconfigured recovery procedure

To alleviate the potential resource cost of network initiated beam recovery, a pre-determined time periods could be configured to UE when it would monitor the alternate beam directions (or different groups). When the network detects a link failure and determines that recovery action is needed it knows that in case UE is still reachable by the network (e.g. the current link used for DL control is blocked but the UE beam alignment is still valid). Based on the cycles network knows that UE would monitor each beam group on specific time instances which enables network to recover the link by scheduling UE to allocate UL grant or request UE to respond with uplink signal which may be either pre-configured or allocated in the DCI message.

Proposal 7: Consider a network beam recovery scheme based on network configured beam group monitoring cycles for UE

# 3	Conclusions

In this contribution beam recovery was discussed. Based on discussion the following observations and proposals are made:

Observation 1: UE is able detect link failure / blockage on periodic beam reference signals

Observation 2: UE triggered recovery actions should be based on the BRS signals and UE actions should be controlled or configurable by network.

Observation 3: Considering non-contention based mechanisms for beam recovery may speed up the beam recovery

Observation 4: Network initiated beam recovery actions may have high resource cost without preconfigured recovery procedure

Proposal 1: Support the use of periodical beam reference signals on downlink for beam management in NR

Proposal 2: Support the use of periodical beam reference signals for UE initiated beam recovery in NR

Proposal 3: Determine link failure from beam reference signal(s) associated to serving beam(s) of downlink control channel reception.

Proposal 4: Support UE event triggered beam recovery procedures/actions in NR that are based on BRS measurements.

Proposal 5: Support UE event triggered UL signal based beam recovery procedures/actions in NR that can be configured by network

Proposal 6: Consider beam recovery via random access procedure that provides initial beam acquisition, and acts as a fallback when link cannot be recovered by using other configured mechanisms.

Proposal 7: Consider a network beam recovery scheme based on network configured beam group monitoring cycles for UE

# References

- RP-160671, “Study on New Radio Access Technology”, NTT DoCoMo

- TR 38.913, “Study on Scenarios and Requirements for Next Generation Access Technologies”, 3GPP

- RAN1 #86bis Chairman Notes

- R1-1610240, “Beam management – DCI Monitoring”, Nokia

- R1-1610241, “Beam management – reciprocity issues”, Nokia.

- R1-1610288, “Beam Management in initial access”, Nokia

- R1-1610243, “On procedures for beam selection and feedback signalling”, Nokia