3GPP TSG-RAN WG2 Meeting #96	R2-167947

Reno, USA, 14th – 18th November 2016

Source:	CATT, Alcatel-Lucent Shanghai Bell, Nokia

Title:	Packet duplication across legs

Agenda Item:	9.2.1.1

Document for:	Discussion and Decision

# Introduction

In the previous RAN2 meetings, the support of packet duplication across legs in a multi-connectivity configuration was mentioned several times as an attractive mean to cope with the unsteady channel of high frequency links. However, although apparently not controversial, the support of this feature was not captured so far as part of the new features to be studied for the NR U-plane stack. This contribution aims at capturing this agreement, also extended to carrier aggregation.

# Discussion

## Packet duplication in support of URLLC service

In our companion contribution [1], we run a latency analysis showing that URLLC latency requirements can hardly be met even using shortest TTI and HARQ RTT assumptions discussed in RAN1. Hence the NR latency needs further optimization in support of URLLC service. One approach, which ensures reliability at the same time, is to support bearer duplication, which aims at transmitting the same packet through different paths thus reducing the HARQ latency.

## Packet duplication in support of HF link blockage

A number of contributions e.g. [2]-[5] highlight the unsteady behavior of the high frequency (HF) link, where simple motion of the UE can lead to abrupt variations of the channel leading to significant signal losses, also known as the “blocking” behavior of such channel. A specific model was developed and captured in Section 7.6.4 of [6]. Since standalone NR deployments in HF bands without low frequency (LF) assistance shall not be precluded, one way to address the unpredictable blockage of an HF link is to make use of multiple such links to increase the overall reliability by means of diversity. Depending on whether these multiple links are connected to the split/merge point in the network through non-ideal or ideal backhaul, the configuration is referred to as multi-connectivity or carrier aggregation. Multi-connectivity has been designed in legacy LTE as the Dual Connectivity feature, where a bearer is split across legs to enhance the throughput performance.  However packet duplication across legs should be added on top as a key feature in support of the specifics of NR, aka HF links. This feature is expected to increase the reliability and latency of transmissions of both user data and control signaling as well as contribute to the 0ms interruption time during handover [7][8]. On the other hand, it cannot be mandated that all radio bearers support this feature, given the high associated resource usage. Hence this feature should be configurable.

Proposal 1: Packet duplication across legs shall be supported in a multi-connectivity configuration in NR.

Proposal 2: Packet duplication across component carriers shall be supported in a carrier aggregation configuration in NR.

# Conclusion

This contribution proposes capturing studying packet duplication across legs in both multi-connectivity configuration and carrier aggregation for NR:

Proposal 1: Packet duplication across legs shall be supported in a multi-connectivity configuration in NR.

Proposal 2: Packet duplication across component carriers shall be supported in a carrier aggregation configuration in NR.

# References

- R2-167949, “NR latency analysis”, CATT

- R2-166100, “High Frequency NR RRC Mobility”, Samsung Electronics Co., Ltd

- R2-164755, “Implications of High Frequency Bands on Mobility”, Nokia, Alcatel-Lucent Shanghai Bell

- R2-165585, “General aspects for NR HF cell”, Huawei, HiSilicon

- R2-163515, “Implications of High Frequency Bands on Mobility”, Nokia, Alcatel-Lucent Shanghai Bell

- TR 38.900, “Study on channel model for frequency spectrum above 6 GHz, release 14”, ETSI

- R2-166784, “How to realize zero ms UP interruption in NR”, Ericsson

- R2-167004, “Discussion on mobility for URLLC”, Huawei, HiSilicon