3GPP TSG-RAN WG2 Meeting #96															    R2-168153

Reno, USA, 14th – 18th November 2016

Title: 	Service continuity for the Evolved ProSe Remote UE

Source: 	ZTE

Agenda item:	8.4.2.3

Document for:	Discussion and Approval

Introduction

In RAN#71 meeting, the SI named “Further Enhancements LTE Device to Device, UE to Network Relays for IoT and Wearables” was agreed. One of the objectives is to study path selection/switch between the cellular link and relay link and provide service continuity. During RAN2#95bis meeting, the scenarios for service continuity are briefly discussed. And it is agreed that the relay solution shall support service continuity when the Evolved ProSe Remote UE selects or switches its path between Uu and PC5. However, it is still FFS whether to support service continuity when switching from one PC5 to another PC5, and switching from one PC5 to non-3GPP access and vice versa [2].

In this paper, we discuss the remaining open issues on scenarios for supporting service continuity for evolved remote UE. And then, we discuss the example signalling procedure for the path switch of remote remote UE and present our considerations.

Discussion

Scenarios

As agreed in RAN2#95bis meeting, the relay solution shall support service continuity when the Evolved ProSe Remote UE selects or switches its path between Uu and PC5. In this section, we further discuss other possible scenarios and aspects considering the mobility of the evolved remote UE. Generally, there are the following three scenarios which need further discussion:

- Scenario 1: evolved Remote UE path switch between cellular link and relay link (including 3GPP access and non-3GPP access)

- Scenario 2: evolved Remote UE path switch from relay link to another relay link

- Scenario 3: evolved Remote UE moves together with evolved relay UE

Scenario 1: evolved Remote UE path switch between cellular link and relay link (including 3GPP access and non-3GPP access)

In the scenario of path switch from cellular link to relay link, the evolved remote UE may moves close to a evolved relay UE or a evolved relay UE moves close to the evolved remote UE, which is illustrated in Figure 1a and Figure 1b respectively. As illustrated in Figure 1a and 1b, the serving cell of the target relay UE may be the same as the previous serving cell of the evolved remote UE as illustrated in figure 1a. Alternatively, the serving cell of the target relay UE may be different from the previous serving cell of the evolved remote UE as illustrated in figure 1b. Similarly, in the scenario of path switch from relay link to cellular link, the evolved relay UE may moves away from the evolved remote UE or the evolved remote UE moves away from the evolved relay UE, which is illustrated in figure 2a and figure 2b respectively. The target eNB may be the same as the serving eNB of previous connected relay UE as illustrated in figure 2a. Alternatively, the target eNB may be different from the serving eNB of previous connected relay UE as illustrated in figure 2b.

It shall be noted that PC5 or non-3GPP access could be used in the relay link. The question is that whether the non-3GPP access case should be supported for the service continuity of the evolved remote UE. According to the objective of the feD2D SI [1], it is assumed that evolved Remote UEs can support both WAN and D2D connection, and that evolved Remote UEs have 3GPP subscription credentials. The D2D connection is realized by either LTE sidelink or non-3GPP technology. All non-3GPP technologies should be considered transparent for generic relay architecture over them. As we can see, the eNB shall maintain the UE context of the evolved Remote UE which is using non-3GPP access. Moreover, generic radio protocol including end-to-end PDCP between evolved Remote UE and the eNB shall be used for the non-3GPP access evolved Remote UEs. As a result, non-3GPP access evolved Remote UEs shall be treated as a normal evolved remote UE using LTE access when considering service continuity.

Figure 1a.  Scenario 1a - path switch from cellular link to relay link (in the same cell)

Figure 1b. Scenario 1b - path switch from cellular link to relay link (in different cells)

Figure 2a. Scenario 2a - path switch from relay link to cellular link (in the same cell)

Figure 2b. Scenario 2b - path switch from relay link to cellular link (in different cells)

Scenario 2: evolved Remote UE path switch from relay link to another relay link

In this scenario, the evolved remote UE connects to the network via relay UE1 at first. And then, relay UE1 moves away from the evolved remote UE or the evolved remote UE moves away from relay UE1, which is illustrated in figure 3a and Figure 3b respectively. In figure 3a, since the evolved relay UE (relay UE1) moves away from the evolved remote UE, the evolved remote UE reselects another evolved relay UE. Then the evolved remote UE shall switch from relay link 1 to relay link 2 and the serving cell of relay UE 1 and relay UE 2 is the same cell (scenario 3a). In figure 3b, the evolved remote UE moves away from relay UE 1 and eNB1. Then, the evolved remote UE reselects relay UE3 and relay UE 3’s serving cell is different from the previous serving cell of the evolved remote UE (scenario 3b).  As a result, the evolved remote UE shall switch from a relay link to another relay link together with cell handover. Similarly as scenario 1, PC5 or non-3GPP access could be used in the relay link. As analyzed above, the service continuity when switching from one PC5 to non-3GPP access and vice versa should also be supported.

Figure 3a. Scenario 3a - path switch from a relay link to another relay link (in the same cell)

Figure 3b. Scenario 3b - path switch from a relay link to another relay link (in different cells)

Scenario 3: evolved Remote UE moves together with evolved relay UE

As we know, the evolved remote UE may be paired with a evolved relay UE and moves together with the paired evolved relay UE. In this scenario, the evolved remote UE and evolved relay UE are always served by the same cell. As illustrated in figure 4, the evolved remote UE connects to the network via relay UE 1 and the serving eNB of evolved remote UE and relay UE1 is eNB1. Then, the evolved remote UE and relay UE1 moves to eNB2’s coverage. The evolved remote UE and relay UE1 shall handover from eNB1 to eNB2 whereas the evolved remote UE’s relay link keeps unchanged.

Figure 4. Scenario 4 - evolved remote UE moves together with evolved relay UE

Proposal 1: In addition to the scenario that evolved remote UE switches path between Uu and PC5, the following two scenarios shall also be studied: 1) evolved remote UE path switch from one PC5 link to another PC5 link; 2) evolved Remote UE moves together with evolved relay UE.

Proposal 2: It is suggested to support service continuity for non-3GPP access evolved Remote UEs when switching from one Uu/PC5 to non-3GPP access and vice versa.

Observation1: The serving cell of the evolved remote UE may change when the evolved remote UE path switch between cellular link and relay link, or between relay links.

Proposal 3: The scenarios that evolved remote UE path switch together with cell change  should be studied.

Procedures

In this section, we discuss the procedure to support mobility for the evolved remote UE. We take the scenarios that evolved remote UE paths switch from cellular link to relay link in the same cell and in different cells for example, which are analyzed in the below.

Scenario 1) path switch from cellular link to relay link in the same cell

In this scenario, the evolved remote UE communicates directly with the eNB via Uu interface, and then switches to the evolved relay UE and communicates indirectly with the eNB via the evolved relay UE. The serving cell of the evolved relay UE is the same as the serving cell of the evolved remote UE. Figure 5 illustrates an example signalling procedure for this scenario. Generally, the legacy PC5 connection establishment procedure could be reused.

On the other hand, it should be further studied that how could the evolved remote UE perform path switch decision. In R13 UE-to-Network Relay, the in coverage evolved remote UE searches and selects evolved relay UE only if the Uu link quality is below a network configured threshold. In R14, since the primary objective of feD2D SI is to address power efficiency for the wearable device, relay link is preferred for the evolved remote UE, which is different from R13 relay design. So when the evolved remote UE communicates with eNB directly, it may also keep searching for a candidate evolved relay UE. And the evolved remote UE may switch to a relay link once the evolved remote UE detects that the PC5 link quality is better than configured threshold, even though the Uu link quality is not so bad. On the other hand, when the evolved remote UE detects that the PC5 link quality is below the configured threshold, the evolved remote UE switch back to cellular link.

Observation 2：The evolved remote UE shall switch to a relay link once the evolved remote UE detects that the PC5 link quality is good enough, even though the Uu link quality is not so bad.

Proposal 4: The eNB could configure PC5 link quality threshold/hysterisis to help the evolved remote UE to perform the path switch decision.

Figure 5 path switch from cellular link to relay link in the same cell

Scenario 2: path switch from cellular link to relay link in different cells

In this scenario, the evolved remote UE communicates directly with the eNB via Uu interface, and then switches to the evolved relay UE served by the cell that is different from the previous serving cell of the evolved remote UE. Figure 6 illustrates an example signalling procedure for this scenario. The question is how to support evolved remote UE path switch in different cells.

As we know, to support the R14 layer 2 relay, the eNB has to maintain and manage the context of the evolved remote UE, no matter the evolved remote UE is using cellular link or relay link. As a result, if the serving cell of the evolved remote UE changes when performing path switch between cellular link and relay link or between relay links, the procedure of the evolved remote UE path switch shall also trigger a cell handover procedure. In detail, after the evolved remote UE discovers a suitable evolved relay UE and makes path switch decision, the evolved remote UE shall report to the eNB either via cellular link or relay link. And then, the eNB shall initiate cell handover procedure to the serving eNB of the target relay UE. During the handover preparation procedure, the evolved remote UE’s UE context is transferred from the source eNB to the target eNB.

Proposal 5: After the evolved remote UE makes path switch decision, the evolved remote UE shall report to the eNB either via cellular link or relay link. And then, the eNB shall initiate cell handover procedure if the serving eNB of the target relay UE is different from the previous serving eNB of the evolved remote UE.

Figure 6 path switch from cellular link to relay link in different cells

Conclusion

In this paper, we discussed various UE mobility scenarios for path switch between cellular link and relay link, and between relay links. And then, we discussed the example signalling procedure for the path switch of evolved remote UE and presented our considerations. We have the following observations and proposals:

Proposal 1: In addition to the scenario that evolved remote UE switches path between Uu and PC5, the following two scenarios shall also be studied: 1) evolved remote UE path switch from one PC5 link to another PC5 link; 2) evolved Remote UE moves together with evolved relay UE.

Proposal 2: It is suggested to support service continuity for non-3GPP access evolved Remote UEs when switching from one Uu/PC5 to non-3GPP access and vice versa.

Observation1: The serving cell of the evolved remote UE may change when the evolved remote UE path switch between cellular link and relay link, or between relay links.

Proposal 3: The scenarios that evolved remote UE path switch together with cell change  should be studied.

Observation 2：The evolved remote UE shall switch to a relay link once the evolved remote UE detects that the PC5 link quality is good enough, even though the Uu link quality is not so bad.

Proposal 4: The eNB could configure PC5 link quality threshold/hysterisis to help the evolved remote UE to perform the path switch decision.

Proposal 5: After the evolved remote UE makes path switch decision, the evolved remote UE shall report to the eNB either via cellular link or relay link. And then, the eNB shall initiate cell handover procedure if the serving eNB of the target relay UE is different from the previous serving eNB of the evolved remote UE.

Reference

[1] RP-161303, Revision of SI: Further Enhancements LTE Device to Device, UE to Network Relays for IoT and Wearables, RAN 72, Busan, Korea, June 13 - 16, 2016.

[2] R2-167181, Text proposal to TR 36.746 from e-mail discussion 95#34, Ericsson