3GPP TSG RAN2 WG Meeting #96	R2-168359

14-18 Nov 2016, Reno, USA

Source:	BlackBerry UK Ltd

Title:	Handling of SI requests from unauthenticated UEs in NR

Document for:	Decision

Agenda Item:	9.2.2.2

# Introduction

At the RAN2#95-bis meeting the following agreements regarding on-demand system information have been reached:

Agreements

1: 	For on demand SI, other SIs may be broadcasted at configurable periodicity (equivalent to SI period in LTE) and for a certain duration.

2	Request of the other SI by idle and “new state” UE should be performed without state transition.

3	For an SI required by the UE, the UE should know whether it is available in the cell and whether it is broadcast or not before it sends the other SI request (e.g. by checking minimum SI).

In this contribution the implications on the SI requests coming from IDLE mode UEs on the downlink radio resource load in the cell are discussed and some solutions are proposed.

# System information requests

There are three RRC states in NR (CONNECTED, INACTIVE and IDLE). It is assumed that system information may be requested from UEs in any of these RRC states. The gNB has a security context for the UE in both INACTIVE and CONNECTED states and hence such SI requests may be considered to be authentic. However, this is not the case for IDLE mode UEs. Based on the RAN2 agreements from last meeting, the UEs can read the essential (minimum) system information and can request any other SIBs available in a cell in IDLE mode and this means that SI requests may be originating from UEs that have no security context at the gNB. Since SI messages can be very large (and this is one of the main considerations for classifying certain SIBs as on-demand SIBs) and have to be transmitted over pessimistic radio bearers, responding to SI requests originating from an unauthenticated UE potentially exposes the system to denial of service style attacks. One or more unauthenticated malicious UEs may trigger repeated SI requests to significantly increase the load on the downlink radio resources triggering multiple SI transmissions from the eNB.

Observation 1: responding to SI requests from IDLE mode UEs (for which the gNB has no security context) may render the radio network open to denial of service style attacks.

In order to solve the above issue, mechanisms to ensure that SIB requests are originating from authenticated UEs are needed. It is assumed that a security context is required to authenticate these requests. Although the security architecture and NAS/AS split between the UE’s security context in NR is still not yet fully clear, it may be assumed that a gNB is unlikely to have the security context of the IDLE mode UEs. Whilst SA3 will investigate the necessary mechanisms to authenticate SIB requests, RAN2 should investigate how to ensure that the necessary air-interface signalling framework is in place to allow any SIB request authentication.

Specifically, the following are the issues that should be studied:

- What security context can be used for SIB request authentication?

- Is authentication needed for all SIB requests or can this be conditional (e.g. gNB decision)?

- What is the additional overhead involved in SIB request authentication and how to minimize this?

It is clear that the above considerations would have implications on the RAN2 work and on the SA3 work. It is hence proposed to send an LS to SA3 informing them about RAN2 agreements about the on demand SIB requests and the implications of these requests originating from IDLE mode UEs.

Proposal 1: RAN2 should investigate radio level signalling mechanisms to enable gNB to authenticate SIB requests from IDLE mode UEs

Proposal 2: Send an LS to SA3 informing them about our decisions regarding on demand SIB requests

# Conclusions and proposals

In this contribution we have highlighted some potential implications of SIB requests from IDLE mode UEs. The following are proposed:

Proposal 1: RAN2 should investigate radio level signalling mechanisms to enable gNB to authenticate SIB requests from IDLE mode UEs

Proposal 2: Send an LS to SA3 informing them about our decisions regarding on demand SIB requests

# References

- Chairman’s notes from RAN2#95-bis meeting