3GPP TSG-RAN WG2 #96	Tdoc R2-168629

# Reno, Nevada, USA, 14th – 18th November 2016

Agenda Item:	9.2.1.4

Source:	Ericsson

Title:	NR UP Design for URLLC

Document for:	Discussion, Decision

# Introduction

Transmission reliability and latency enhancement are two key aspects for URLLC [1], defined as Ultra-Reliable and Low-Latency Communications in TR 22.862. This paper mainly discusses 3 aspects: HARQ and ARQ, duplicated data transmission based on multiple-connectivity, UL access enhancement for URLLC.

# Discussion

In this paper we address three aspects relating to the user plane which are of importance to improve the reliability and reduce the latency. First we address the single connectivity case and discuss improvements to HARQ and ARQ. Then we discuss how to improve the reliability in the multi-connectivity case. Our third and last topic focuses on how to reduce latency through grant-free uplink transmissions.

## HARQ and ARQ

For any UP design, resource efficiency is important, but it is equally important to support features enabling URLLC transmissions. There is typically a trade-off between improving reliability and reducing latency versus resource efficiency. Therefore, we must carefully analyse trading resource efficiency for a reliability boosting or latency reduction. For UP design in provisioning URLLC services, RLC retransmission may not be feasible due to the latency requirement of URLLC services. To meet the packet loss rate and delay requirement, a key function MAC HARQ operation, i.e., the packet loss rate will mainly depend on the residual error of MAC HARQ and the delay in air interface will depend on the HARQ round trip time (RTT) and number of transmission/retransmission attempts for one URLLC packet. Even though RLC retransmission may be allowed for some URLLC traffic within certain delay budget, the allowed maximum number of RLC retransmission would be rather small and the residual error of HARQ is still important to meet the target packet loss rate. Hence we have the following observation.

RLC retransmissions are not feasible for URLLC due to the long RTT, therefore HARQ performance is more important for the reliability.

In LTE, to efficiently reduce residual error rate, two kinds of retransmission mechanisms were mainly introduced, i.e. hybrid-ARQ (HARQ) in MAC and ARQ in RLC. Retransmission of missed or erroneous data units are handled primarily by HARQ, complemented by the ARQ. Such two-level retransmission structure conducts efficiency-reliability trade-off with fast (HARQ) and reliable (ARQ) feedbacks. The HARQ mechanism was designed to target for very fast retransmission but with not very high ambition on minimizing BLER to reduce physical control channel overhead. Complementarily, RLC status reports are transmitted with relatively high robustness (reliability of 10^-5 or even higher robustness) to eventually minimize the residual error rate at L2.

At present, for URLLC, one of challenging demanding is ultra-low latency, a legacy ARQ retransmission (LTE-like waiting feedback triggering retransmission takes time) may render an unacceptable delay, it thus becomes almost invalid in URLLC cases. On the other hand, a legacy HARQ re-transmission relies on an error rate of feedback channel, which may result in an unacceptable residual error rate at L2. So the way forward is to enhance UP RLC ARQ and HARQ functionality to have a new tradeoff in reliability, latency and efficiency per URLLC requirements, for which some possible aspects to consider as follows:

- Improvement on HARQ: to improve reliability of HARQ feedback, either by reducing relying on HARQ feedback, e.g., by using configurable/adaptive TTI bundling (i.e. autonomous re-transmission), or enhancing HARQ feedback channel design, e.g., by adding CRC to secure the HARQ feedback;

Improvement on ARQ: to speed up ARQ RTT, e.g., by allowing configurable ARQ-level autonomous re-transmission(s) even before ARQ status PDU. Advantages and disadvantages can be further analyzed based on the given solution alternatives/features at MAC and RLC sublayers, in terms of implementation complexity, standardization effort, scalability, and achievable performance.

Enhancements to HARQ (such as TTI bundling and improved HARQ feedback channel) to satisfy URLLC requirements should be investigated.

Enhancements to ARQ (such as shorter RTT) to satisfy URLLC requirements should be investigated.

## Duplication in Multi-connectivity scenario

In addition to improving the reliability per link, transmitting a packet over multiple links in a multi-connectivity scenario can improve reliability.

Data duplication is one of tools for reliability enhancement for URLLC in a multi-connectivity scenario.

Data duplication can be implemented in several ways, e.g. on different layers in the stack. To minimize the risk of unforeseen problems and using the LTE UP stack as baseline we think data duplication can be implemented on PDCP. A PDCP PDU would be duplicated and transmitted on different legs in a MC scenario. As PDCP today features detection of duplicated PDUs, this solution already works on the receiving side and works in both uplink and downlink.

Data duplication of PDCP PDUs in multi-connectivity (MC) scenarios can be used to improve reliability for URLLC.

## UL enhancement for URLLC

The current dynamic grant request-response procedure in LTE may result in unacceptable delays for URLLC. To shorten the delay, semi-persistent scheduling (SPS) can be beneficial for UL traffic as shown in the latency reduction study for LTE [3GPP TR 36.881]. The use of SPS in UL for URLLC in NR can similarly be beneficial. The cost of using SPS can be reduced radio resource efficiency as radio resources are reserved. In here it is up to the network how to partition the radio resources.

Semi-persistent scheduling for UL is supported to reduce latency for URLLC.

Complementarily, as one way of improving radio resource efficiency, grant-free UL access and/or in combination with contention based UL access may be of interest in this URLLC context and, at least two options exist, i) UE sends contending scheduling request (SR) before getting a grant for UL transmissions. ii) UE sends contending SR plus data at contention channels. These two fashions may provide different performance in latency and reliability. Previous discussions for latency reduction could be a valid starting point for URLLC [2]. However, at URLLC context, a very high successful detection rate of contention signals is necessary so that new improving features of L1 and L2 are needed.

Contention-based uplink access for NR is supported to increase radio resource efficiency and reduce latency for URLLC.

It is noted that contention based transmissions may bring uncertainty in general. However, a contention success rate is normally determined by many factors, which could be considered and optimized to meet the performance goal on both latency and reliability. Factors that may for example influence this are the number of contending UEs for a shared radio resource, traffic arrival rate, redundancy-based protection of contention signals (e.g. Time/Frequency/code diversity employments and other), receiver algorithms, prioritizing strategy, etc. Therefore, a proper design of contention scheme needs to be designed to achieve a reliability target within a latency limit.

Note that semi-persistent UL scheduling and contention based UL access are not mutually expelling each other. They could co-exist and even associate each other to deliver a possible better balanced performance in both efficiency and latency control. In an accompanying contribution [3] we expand more on the options for UL scheduling including SPS and contention based UL scheduling.

# Conclusion

In section 2 , we made the following observations:

Observation 1	RLC retransmissions are not feasible for URLLC due to the long RTT, therefore HARQ performance is more important for the reliability.

Observation 2	Data duplication is one of tools for reliability enhancement for URLLC in a multi-connectivity scenario.

Based on the discussion in section 2 we propose the following:

Proposal 1	Enhancements to HARQ (such as TTI bundling and improved HARQ feedback channel) to satisfy URLLC requirements should be investigated.

Proposal 2	Enhancements to ARQ (such as shorter RTT and autonomous retransmissions) to satisfy URLLC requirements should be investigated.

Proposal 3	Data duplication of PDCP PDUs in multi-connectivity (MC) scenarios can be used to improve reliability for URLLC.

Proposal 4	Semi-persistent scheduling for UL is supported to reduce latency for URLLC.

Proposal 5	Contention-based uplink access for NR is supported to increase radio resource efficiency and reduce latency for URLLC.

# References

R2-166821, “UP aspects for URLLC traffics”, Ericsson, 3GPP TSG-RAN WG2 #95bis，Kaohsiung, Taiwan, 10th – 14th October 2016

R2-166800,	“Aspects for pre scheduling and contention based UL transmission”, Ericsson, 3GPP TSG-RAN WG2 #95bis，Kaohsiung, Taiwan, 10th – 14th October 2016

R2-168669- Uplink dynamic scheduling in NR, Ericsson