3GPP TSG-RAN WG2 #96	Tdoc R2-168707

# Reno, Nevada, USA, 14th – 18th November 2016

Agenda Item:	9.4

Source:	Ericsson

Title:	Radio interface implications of network slicing

Document for:	Discussion, Decision

# Introduction

This contribution is a revision of R2-166931 submitted but not presented at RAN2#95bis. The changes include removing the section on radio interface functionality for supporting slice selection which is now elaborate more in a separate paper R2-168708 entitled “Signalling aspects of network slicing”.

The support for network slicing is an important part of the vision for the Next Generation architecture [1] [2]. As stated by different fora, such as NGMN [1], network slicing consists of deploying multiple end-to-end logical networks in support of independent business operations. In contrast to deploying an independent network infrastructure, each instance of a slice (blueprint) should be possible to realize as a logical network corresponding to a shared infrastructure (including shared processing, storage, transport, radio spectrum, and hardware platforms), where it co-exists with other slices having potentially different characteristics.

Figure 1 Example of multiple slice instances deployed   in the same network infrastructure

In this way, the infrastructure and assets utilization will be much more cost- and energy-efficient while the logical separation allows for a flexible and independent configuration and management of the slices without compromising stability and security. Enabling slice realization over a common physical infrastructure would of course not prevent the realization of a slice instance by means of dedicated resources and assets.

In 3GPP, the notion of network slicing has also been introduced as a way to address the needs for the different vertical industries, translated into a wide range of use cases for the Next Generation architecture. To give a few examples, [2] explicitly states that a network slice should support the communication service requirements of particular use case(s). A set of service and operations requirements associated to network slicing has been provided. In SA2 [3], network slicing is presented as one of the high level architectural requirements and is listed as one of the key issues in order to enable the operator to create networks customised to provide optimized solutions for different market scenarios, which present diverse requirements. Example use cases for such customized logical networks could include public safety, V2X, Internet-of-Things (IoT), highlighting the importance of network slicing as a common platform to address multiple industries.

The support for network slicing is also listed as a requirement for the new RAT [4]. In order to fulfill that, one of the objectives listed in [5] is to study and identify specification impacts when enabling the realization of network slicing. Some initial studies have also been done within the research community [6].

In RAN3 the following aspects have been captured for network slicing in TR 38.801(the editor’s notes have been removed):

RAN awareness of slices

-	RAN shall support a differentiated handling of traffic for different network slices which have been pre-configured. How RAN supports the slice enabling in terms of RAN functions (i.e. the set of network functions that comprise each slice) is implementation dependent.

Selection of RAN part of the network slice

-	RAN shall support the selection of the RAN part of the network slice, by a slice ID provided by the UE which unambiguously identifies one of the pre-configured network slices in the PLMN.

Resource management between slices

-	RAN shall support policy enforcement between slices as per service level agreements. It should be possible for a single RAN node to support multiple slices. The RAN should be free to apply the best RRM policy for the SLA in place to each supported slice.

Support of QoS

-	RAN shall support QoS differentiation within a slice.

RAN selection of CN entity

-	RAN shall support initial selection of the CN entity for initial routing of uplink messages based on received slice ID and a mapping in the RAN node (CN entity, slices supported). If no slice ID is received, the RAN selects the CN entity based on NNSF like function, e.g. UE temporary ID.

Resource isolation between slices

-	RAN shall support resource isolation between slices. RAN resource isolation may be achieved by means of RRM policies and protection mechanisms that should avoid that shortage of shared resources in one slice breaks the service level agreement for another slice. It should be possible to fully dedicate RAN resources to a certain slice

This contribution further elaborates on these principles with focus on RAN2 aspects.

# Discussion

In addition to the principles adopted in RAN3, the following aspects need further consideration in RAN2:

- Efficient multiplexing of RAN level resources for different slices;

- Overload/congestion control of common resources;

Below are some assumptions and proposals in these areas.

## Efficient multiplexing of RAN level resources for different slices

According to RAN3’s assumptions a single RAN instance may support multiple network slices (this does not of course preclude implementations relying on dedicated resources, however those don’t require new standardization efforts). Given this assumption, it is important that the radio interface solution allows for efficient RAN resource multiplexing of traffic associated with different slices. The background for this is that it is very important that the RAN resources such as spectrum, power, antennas, sites, backhaul transport are efficiently managed in order to maximise utilisation.

In addition, as it may be so that different slices serve different purposes corresponding to (different) slice-specific requirements, the solution may need to be realized using different protocol configurations. In such cases it is desirable that it is still possible to dynamically multiplex traffic from different users associated with different slices.

On the other hand, aiming for minimum overhead and efficient radio resource utilization should not imply a restriction of the concept applicability itself. In [7], it is proposed that ‘The combination of L1 resource component and L2 configuration/functions, to address the needs of different services and requirements, can be defined as ‘RAN Slice’, however we think that the linkage between numerology and slicing in the RAN is too restrictive, as supporting multiple services is NOT the same as RAN slicing. This does not mean that the operator should be prevented from configuring network slices specific to one type of service, we are rather saying that the standard should not restrict operator’s choices as it is impossible to predict which needs the market will present in the future, so NW slicing should be a toolbox allowing both specific and less specific slices depending on operator’s choice.

In particular, expanding further on the above, we have the following concerns if the NW slicing concept is mapped over the radio interface to something very specific and very restrictive and if support of multiple services is equalized to NW Slicing the concern with L1 component is that:

- The same UE can be configured to use multiple numerologies, possibly shifting between numerologies (e.g. in a TDM fashion);

  - It is only if we are dealing with a certain UE class that is only capable of a certain numerology that we get this absolute linkage between “service, UE type and numerology”.

- The UE can also use multiple services within the same numerology.

The concern with the L2 configuration component is on the other hand:

- L2 configuration for common channels (e.g. initial access) should be as much as possible the same for the different slices to maximize RAN resources. This is actually already part of the agreed guidelines;

- For the L2 configuration of dedicated channels, UEs using different configuration can easily be multiplexed on the same numerology;

- There could be L2 configurations that are useful for many slices / services. There could be many more slices (e.g. 100) than configurations;

- It should be possible in later releases to add new configurations, or even new protocols if need arises.

- Based on the above reasoning, we propose the following:

- Proposal 1    Network slicing should aim for minimum overhead and allow for an efficient utilization of scarce RAN resources

- Proposal 2     A network slice shall not be directly linked to a physical or L2 configuration, i.e. it should be possible to have different slices using the same configuration and a single slice using different configurations;

## Overload/congestion control of common resources

When a pool of resources is shared among multiple slice instances, events associated with one slice that may result in overload and/or congestion on common resources will, if not managed properly, negatively impact the performance of another slice. Therefore, the RAN needs mechanisms to protect common signalling resources so that any overload or congestion caused by events associated with one network slice does not have a negative effect on the performance of other network slices. Common signalling resources in this case can refer to radio access channels (common control channels), signalling connections, and common signalling nodes in RAN, CN and service layer.

Example of protection mechanisms, which need to be slice-specific (or configurable per slice), could include access barring mechanisms, reject with back off timer, user and service based admission control.

Also in [8], the point is made that RAN should offer means to protect slices from each other, but it can also be observed that it may be so that the UEs are not aware of that, for example, ACB is per slice: such mechanism could be configured to be based on access control information not explicitly mapped to slices, but to parameters configured at the UE, based on slice knowledge (e.g. which slice the UE is attached to), making that transparent to the UE.

Slice-specific access control should also be a consistent component of a more general access control mechanism for NR.

The RAN should offer means to protect slices from each other, i.e., avoid that shortage of shared resources (e.g. common signaling resources) in one slice breaks the service level agreement for another slice. Such mechanism could be transparent to the UE, i.e. it may or may not be necessary to make the UE aware that the access control mechanism is slice-specific and the most suitable solution should be investigated.

The RAN should offer means to protect slices from each other, i.e., avoid that shortage of shared resources (e.g. common signaling resources) in one slice breaks the service level agreement for another slice. Such mechanism could be transparent to the UE, i.e.it may or may not be necessary to make the UE aware that the access control mechanism is slice-specific and the most suitable solution should be investigated.

# Conclusion

In the previous sections we made the following proposals:

- Network slicing should aim for minimum overhead and allow for an efficient utilization of scarce RAN resources

- A network slice shall not be directly linked to a physical or L2 configuration, i.e. it should be possible to have different slices using same configuration and a single slice using different configurations.

- The RAN should offer means to protect slices from each other, i.e., avoid that shortage of shared resources (e.g. common signaling resources) in one slice breaks the service level agreement for another slice. Such mechanism could be transparent to the UE, i.e.it may or may not be necessary to make the UE aware that the access control mechanism is slice-specific and the most suitable solution should be investigated.

It is proposed to capture this in relevant TR together with the RAN3 assumptions.

# Text proposal

## 5.1	Functional split

Editor’s note: intended to capture functional split between RAN and CN.

5.1.1 RAN functions related to support for network slicing

RAN awareness of slices

-	RAN shall support a differentiated handling of traffic for different network slices which have been pre-configured. How RAN supports the slice enabling in terms of RAN functions (i.e. the set of network functions that comprise each slice) is implementation dependent.

Selection of RAN part of the network slice

-	RAN shall support the selection of the RAN part of the network slice, by a slice ID provided by the UE which unambiguously identifies one of the pre-configured network slices in the PLMN.

Resource management between slices

-	RAN shall support policy enforcement between slices as per service level agreements. It should be possible for a single RAN node to support multiple slices. The RAN should be free to apply the best RRM policy for the SLA in place to each supported slice.

Support of QoS

-	RAN shall support QoS differentiation within a slice.

RAN selection of CN entity

-	RAN shall support initial selection of the CN entity for initial routing of uplink messages based on received slice ID and a mapping in the RAN node (CN entity, slices supported). If no slice ID is received, the RAN selects the CN entity based on NNSF like function, e.g. UE temporary ID.

Resource isolation between slices

-	RAN shall support resource isolation between slices. RAN resource isolation may be achieved by means of RRM policies and protection mechanisms that should avoid that shortage of shared resources in one slice breaks the service level agreement for another slice. It should be possible to fully dedicate RAN resources to a certain slice.

Efficient multiplexing of RAN level resources

- The solution for NW slicing at RAN should aim for minimum overhead and allow for an efficient utilization of scarce RAN resources, but a network slice shall not be directly linked to a physical or L2 configuration, i.e. it should be possible to have different slices using the same configuration and a single slice using different configurations. It is up to RAN implementation how the policies stemming from the SLAs are translated into radio resource management strategies and radio configurations.

Overload/congestion control of common resources

- A mechanism should be defined for protecting common resources for overload and congestion in one slice, i.e. avoid that shortage of shared resources (e.g. common signaling resources) in one slice breaks the service level agreement for another slice. Such mechanism could be transparent to the UE, i.e.it may or may not be necessary to make the UE aware that the access control mechanism is slice-specific and the most suitable solution should be investigated.

# References

NMGN 5G white paper,

3GPP TR 22.891 V14.0.0 (2016-03) Technical Specification Group Services and System Aspects; Feasibility Study on New Services and Markets Technology Enablers; Stage 1 (Release 14)

3GPP TR 23.799 V0.2.0 (2016-02) Technical Specification Group Services and System Aspects; Study on Architecture for Next Generation System (Release 14)

3GPP TR 38.913 V0.3.0 (2016-03), 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Study on Scenarios and Requirements for Next Generation Access Technologies; (Release 14)

RP-160671, New SID Proposal: Study on New Radio Access Technology

METIS-II White Paper on 5G RAN Architecture and Functional design

R2-165282, E2E network slice: concept and requirements from RAN perspective, Samsung

R2-165019,	Key Principles for Support of Network Slicing in RAN, Huawei, HiSilicon