3GPP TSG-RAN WG2 #97	Tdoc R2-1700833

# Athens, Greece, 13th – 17th February 2017

Agenda Item:	10.2.1.2

Source:	Ericsson

Title:	Data duplication in lower layers (HARQ)

Document for:	Discussion, Decision

# Introduction

Transmission reliability and latency enhancement are two key aspects for URLLC, defined as Ultra-Reliable and Low-Latency Communications in TR 22.862 and TR 38.913.

At present, 3GPP RAN2#96 has the following agreements:

- 	Multi-connectivity (e.g. with packet duplication, link selection) should be studied for achieving the reliability requirements for URLLC.

And 3GPP RAN2 NR Ad hoc meeting has made the following agreements:

1:  Packet duplication is supported for user plane and control plane in NR-PDCP (This agreement does not preclude discussion of other mechanisms to improve mobility robustness)

FFS whether packet duplication should also be supported for LTE-NR dual connectivity

2  The PDCP function in the transmitter supports packet duplication and the PDCP function in the receiver supports duplicate packet removal.

1	RLC retransmission (ARQ) is not assumed to be used for meeting the strict user plane latency requirements of URLLC .

2	RAN2 will study redundancy schemes operating below PDCP in CA scenarios for the purpose of meeting the reliability/latency requirements of URLLC. Study should consider the performance of the underlying Phy layer.

In this contribution, we analyse how redundancy schemes operating below PDCP in CA scenarios can be used to reach the reliability and latency requirements of URLLC. A TP describing data duplication in lower layers is also provided.

# Discussion

In a general sense, multi-connectivity (MC) conceptually can be defined to include all the radio schemes involving more than one radio links to serve a UE for both uplink (UL) and downlink (DL). In NR, MC can include terminologies of dual-connectivity (DC), and carrier aggregation (CA) from a system architecture perspective, which are applicable to both collocated eNBs and non-collocated eNBs.

Characteristically, MC can contribute to URLLC in at least two aspects: i) boosting reliability without sacrificing  latency-wise performance; ii) removing the interruption time otherwise caused by mobility. The philosophy to emphasize is that MC as a kind of diversity scheme can obtain a large magnitude of gains in reliability and latency that any link-level diversity scheme by its own cannot achieve [2].

For URLLC, two or more (if considering future enhancement) channels might be needed for extreme-reliability cases such as error rates of 10-5 to 10-9 within a given latency bound [1]. Both DL and UL MC may be of special interests to URLLC, though UL MC may have limiting factors, such as UE’s power.

## Duplication on PDCP

It had been agreed in the RAN2 NR Ad-hoc, January 2017, to support packet duplication based on the DC architecture with PDCP. This way, duplication can be applied flexibly to scenarios with centralized and non-centralized PDCP, scenarios with different backhaul latencies and different scheduler implementations, i.e. in principle also to LTE-NR integration.

DC-based PDCP duplication provides a flexible way to achieve diversity for different deployment and backhaul scenarios, by building on separate MAC/schedulers.

For URLLC highly reliable transmissions with a very low BLER are targeted (e.g. 10^-5 residual error). The PDCP duplication solutions helps in maintaining this low residual error rate, in cases it cannot be reached on the respective carriers e.g. due to a temporary outage/fading dip, or due to unanticipated change or wrong channel state information.

Furter aspects of PDCP duplication is discussed in [4].

## Duplication on MAC

Beside the PDCP duplication scheme, to achieve the strict latency and reliability requirements of URLLC for NR, a duplication scheme based on lowers layers, i.e. MAC, and especially for the NR-CA scenario seems beneficial. DC uses different MAC entities, thus synchronization of the duplicate transmissions is more difficult. It would require coordination among the MAC entities and their respective independently operating schedulers.

In CA however, one common MAC entity, i.e. one scheduler, controls transmissions on all carriers, so that duplicate transmissions can be timed more accurately to achieve the URLLC latency bound. The MAC layer has more timely information on instant radio channel quality measurements (such as CSI report, A/N feedback). Hence, in terms of scheduling and logical channel multiplexing, duplication / diversity at MAC layer has more advantages, both in terms of achieving time-criticality of transmissions as well as to improve resource efficiency, as compared to solutions based on duplication on PDCP layer.

CA-based duplication provides a more optimized way than PDCP duplication to achieve time-critical diversity, by using a common MAC/scheduler.

Besides CA, TTI repetitions can be configured to increase transmission reliability especially in coverage limited scenarios. The increased reliability is in this case traded for higher transmission latency. The repetitions are performed on one carrier, which could be limiting in case there is not enough spectrum available. Furthermore, frequency diversity gains are limited with repetitions in time, relative only to the used bandwidth and potential frequency hopping supported among the repeated TTIs, as further discussed in [5]. With TTI repetitions, the received soft bits of multiple consecutive transmissions within the same HARQ process are combined, this way additional soft combining gain as well as incremental redundancy gains (if supported within TTI repetitions) can be achieved. TTI repetitions are especially interesting for UL transmissions, since the maximum transmit power is available in each TTI which can be aggregated in the receiver, by e.g. soft combining.

With TTI repetitions using one carrier, time-diversity with limited frequency-diversity is achieved, however at the expense of increased latency.

Duplication based on CA can be seen as a complementary tool for the scheduler to further improve the transmission reliability without sacrifizing latency. It becomes especially interesting in scenarios where reliability on one of the carriers cannot be guaranteed by e.g. TTI repetitions or other physical layer features. In these scenarios it is thus beneficial to have a backup carrier available, for example in situations like a temporary outage of the cell/carrier, since e.g. UE moves to cell edge of one cell/carrier.

Therefore, a L2 solution based on CA seems beneficial to complement TTI repetitions and othe physical layer solutions.

Duplication solution based on CA is supported on MAC.

### Duplication solution

The duplication solution on MAC based on CA can be realized by:

A) Duplicate transmission of MAC PDUs (same transport blocks): In this case, two transport blocks of the same size would be created by the MAC multiplexing and assembly entity and filled with the same duplicated MAC PDU. The HARQ transmissions among both carriers are thus coupled, i.e. require the same TBS, which may not be possible at all times depending on carrier bandwidths and loads. Joint decoding (soft combining) of the two transmissions could be enabled in the receiver (RAN1 impact), in which case also HARQ feedback must be aligned among the carriers. Alternatively the existing RLC duplicate discard function can be used to handle the duplicates.

MAC PDU duplication (same transport block) could be considered to enable joint decoding. It requires strict alignment of the transmissions (transport block size) among the carriers.

B) Duplicate transmission of MAC SDUs, i.e. RLC PDUs or RLC PDU segments: In this case the MAC multiplexing and assembly entity would be responsible to transmit MAC SDU duplicates via the different carriers, i.e. RLC PDUs and RLC PDU segments (of different size, if needed). The HARQ transmissions among the carriers, e.g. transport block size (and required spectrum), as well as HARQ feedback transmissions would be entirely independent. This way, different carrier bandwidths (numerologies) or carriers with different traffic load can be considered. In the receiver, the existing RLC duplicate discard function would handle the duplicates. This solution is outlined in Figure 1.

MAC SDU duplication is flexible since strict alignment, i.e. the same transport block size, among carriers is not required.

Figure 2: MAC duplication of MAC SDUs among carriers (Solution B).

Comparing the two options, it becomes obvious that MAC PDU duplication comes with the burden of synchronizing transmissions among the carriers, which increases the scheduling complexity in the eNB. The soft combining gains achieved in this scenario may be helpful however only in situations where both carriers provide insufficient reliability, i.e. where both individual transmissions fail. If one carrier provides sufficient reliability, duplicate transmissions without soft combining provide the same reliability. Bearing in mind that highly reliable transmissions per carrier can be achieved by physical layer features, and considering that URLLC traffic would only be served when at least one carrier can provide the sufficient reliability, we believe that the the situations where soft combining among carriers helps are negligible.

Example: The residual transmission error is 10^-3 for each individual and independent transmission. With duplication among the independent carriers it can be decreased to 10^-6, i.e. a failure is only possible if both individual transmissions fail at the same time. Only in these 10^-6 cases soft combining could provide an additional gain in received SINR, which depending on absolute SINR, would further decrease the already low overall residual error.

When duplicate transmissions on reliable channels are employed, the additional gains of soft combining over reception with duplicate discard are negligable.

In conclusions we propose:

Duplication solution for CA builds on MAC SDU duplication as well as RLC duplicate discard (Solution B).

# Conclusion

In section 2, we made the following observations:

Observation 1	DC-based PDCP duplication provides a flexible way to achieve diversity for different deployment and backhaul scenarios, by building on separate MAC/schedulers.

Observation 2	CA-based duplication provides a more optimized way than PDCP duplication to achieve time-critical diversity, by using a common MAC/scheduler.

Observation 3	With TTI repetitions using one carrier, time-diversity with limited frequency-diversity is achieved, however to the expense of increased latency.

Observation 4	MAC PDU duplication (same transport block) could be considered to enable joint decoding. It requires synchronization strict alignment of the transmissions (transport block size) among the carriers.

Observation 5	MAC SDU duplication is flexible since strict alignment, i.e. the same transport block size, among carriers is not required.

Observation 6	When duplicate transmissions on reliable channels are employed, the additional gains of soft combining over reception with duplicate discard are negligable.

Based on the discussion in section 2 we propose the following:

Proposal 1	Duplication solution based on CA is supported on MAC.

Proposal 2	Duplication solution for CA builds on MAC SDU duplication as well as RLC duplicate discard (Solution B).

# References

R1-167061, “On URLLC design principles”, Ericsson, TSG-RAN WG1 #86，Gothenburg, Sweden, August 22–26, 2016

R2-168629, “NR UP Design for URLLC”, Ericsson, 3GPP TSG-RAN WG2 #96，Reno, Nevada, USA, 14th – 18th November 2016

3GPP TR 36.912 V13.0.0 (2015-12), Technical Report,3GPP, Technical Specification Group Radio Access Network, “Feasibility study for Further Advancements for E-UTRA (LTE-Advanced),(Release 13)”

R2-1700834, Further aspects of data duplication in PDCP layer, Ericsson, RAN2#97

R2-1700835, URLLC aspects for grant-free transmission in NR, Ericsson, RAN2#97

# Text proposal

NR MAC supports data duplication in carrier aggregation. Thereby, the MAC transmitter is configured to duplicate and transmit MAC SDUs among different carries. HARQ operation of these transmissions is independent. On the receiver side, the existing duplicate discard functionality of RLC is used to discard the duplicates.