3GPP TSG-RAN WG2 #97	Tdoc R2-1700835

Athens, Greece, 13th – 17th February 2017	Update of R2-1700436

Agenda Item:	10.2.1.4

Source:	Ericsson

Title:	URLLC aspects for grant-free UL transmission in NR

Document for:	Discussion, Decision

# Introduction

The “autonomous/grant-free/contention based” UL transmission for NR has been discussed in RAN1. Based on RAN1 discussions, this contribution discusses some RAN2 aspects concerning “autonomous/grant-free/contention based” UL transmission. A more general discussion on grant-free access is given in [3].

In RAN1#87 the following was agreed for UL URLLC scheduling:

At least an UL transmission scheme without grant is supported for URLLC - Resource may or may not be shared among one or more users - FFS: resource configuration details - FFS other details of design

In RAN1#87ah, the following was agreed for UL URLLC scheduling:

• For an UL transmission scheme without grant • at least semi-static resource (re-)configuration is supported − FFS: The resource configuration includes at least physical resource in time and frequency domain and RS parameters − Higher-layer signaling could be similar to Rel-8 LTE SPS − FFS: MCS • RS is transmitted together with data − channel structure of grant-based data transmission can be starting point

- For an UL transmission scheme with/without grant - K repetitions including initial transmission (with the same or different RV and FFS with different MCS) (K>=1) for the same transport block are supported, - FFS the way K is determined - FFS: hopping mechanisms over the transmissions

# Discussion

## Pre-configured semi-static grant-free access

In LTE, semi-persistent scheduling (SPS) has been shown to be beneficial for UL traffic as seen in the latency reduction study for LTE [1]. The use of SPS in UL for URLLC in NR can similarly be beneficial. To shorten the UL delay, the grant can instead be configured before UL data arrival. By doing so, steps of SR and UL grant are not needed, and a UE can directly transmit in a pre-configured UL grant. The use of configured grants is also called grant-free transmission.

Previous discussions for LTE latency reduction could be a valid starting point for LTE or NR URLLC [2], as agreed in RAN1 #87AH.

Grant-free access by a preconfigured scheduling scheme is supported to reduce UL latency.

In LTE, enhancements were recently introduced as an enhancement to SPS in UL [1][2]. With this technique, a UE uses the configured grant only when it has UL data to transmit to reduce unnecessary interference and battery consumption.

UE does not transmit on a grant-free resource when it has an empty buffer.

## Contention-based UL access

### Resource inefficiency in SPS

To achieve highly robust UL data transmissions for URLLC use cases, a low code rate is required. If the reliable transmissions also have strict latency requirements, the transmissions need to be short. These demands lead to a preference of large bandwidth but short time allocations for URLLC in a general sense. However, if each UE is allocated with dedicated UL resources of large bandwidth, the network will run out of UL resources soon, when attempting to serve a larger amount of URLLC requests. Thus, a reasonable improvement is to assign shared UL resources between UEs for efficiency, i.e. allowing contention-based transmissions.

Given large-bandwidth and short-transmission allocations suitable for URLLC and its infrequent traffic, shared resources with a contention-based transmission approach is important from a resource efficiency perspective.

### Longer delay in UL dynamic scheduling

Table 1. FDD latency for first data, assuming 7-symbol slots.

| [us] | Slot duration | DL/Configured UL |  | Dynamic UL |  |
| --- | --- | --- | --- | --- | --- |
|  |  | fast | slow | fast | slow |
| 15 kHz | 500 | 1000 | 1500 | 1500 | 3000 |
| 30 kHz | 250 | 500 | 750 | 750 | 1500 |
| 60 kHz | 125 | 250 | 375 | 375 | 750 |
| 120 kHz | 63 | 125 | 188 | 188 | 375 |

Table 2. TDD latency for first data, assuming 7-symbol slots and alternating UL-DL pattern.

| [us] | Slot duration | DL/Configured UL |  | Dynamic UL |  |
| --- | --- | --- | --- | --- | --- |
|  |  | fast | slow | fast | slow |
| 15 kHz | 500 | 1500 | 2000 | 2000 | 3500 |
| 30 kHz | 250 | 750 | 1000 | 1000 | 1750 |
| 60 kHz | 125 | 375 | 500 | 500 | 875 |
| 120 kHz | 63 | 188 | 250 | 250 | 438 |

Differences in end-to-end one-way latency for dynamic UL and DL scheduling are shown in Table 1 for FDD, and in Table 2 for TDD with a alternating UL-DL slot pattern. These tables cover four numerologies and two different processing delays:

- “fast” meaning reception processing within the same slot as the transmission, with 1symbol TX/RX processing

- “slow” allowing for one slot of TX/RX processing.

Additionally, a delay due to SR and UL grant has a significant impact on the total delay. In a dynamic scheduling for UL, scheduling request being followed by an UL grant results in a longer delay compared to DL. This may be unacceptable for the URLLC traffic because of the same requirement in UL as in DL.

Dynamic UL scheduling causes significantly longer delay as compared to DL.

### Grant-free access with shared resources

As mentioned above, grant-free access builds on the pre-allocation of resources to UEs. With large bandwidth allocations (for robust MCS) or a higher number of UEs to schedule, the UL spectrum becomes a limiting factor in the scheduler. If resources can be shared by multiple UEs the UL capacity increases. Even though this reduces the reliability due to risk of collisions, it can be a reasonable tool for URLLC traffic since each scheduled UE may have sporadic and sparse traffic, giving risks at manageable levels.

Grant-free access with shared resources and contention based transmission should be supported.

The grant-free resource should be of limited size to allow multiple users, have a robust MCS, and therefore be of low payload size. To avoid unnecessary contention in the case of larger packages, the network should be able to switch a UE to a grant-based resource when needed, where it can continue after a first grant-free transmission.

The framework of grant-free resources should support fast switching to grant-based resources.

## Collision resolution

For URLLC, a reliable transmission of a message within a delay bound of 1ms needs to be provided at a reliability level of 1-10-5. This mandates that only less than 10-5 of the transmissions may be allowed to either fail in delivery or lead to latencies exceeding the 1ms bound. In other words, successful in-time transmission needs to be guaranteed for a given reliability level within a required latency bound. As compared to eMBB traffic, a transmitter may not be possible to do retransmissions or many retransmissions, to same extent, for URLLC.

Figure 1 Reliability vs. latency where latencies are guaranteed up to the reliability level.

In Figure 1, a reliability is specified by the failure probability ε of packets not successfully delivered to a receiver within the latency bound, when these packets are either erroneous, lost or arrive too late. Therefore, if contention-based UL resource usage is allowed, a collision resolution scheme must be in place to guarantee a success before a required latency level.

A reliable collision resolution scheme is required for URLLC, since a high success rate must be achievable within a latency bound.

### Collision resolution

It is noted that contention based transmissions may bring uncertainty in general. However, a contention success rate is determined by many factors. The rate could be controllable to meet a performance goal on both latency and reliability, though at a certain cost of resource efficiency and processing complexity. These factors, for example, may include the number of contending UEs for a shared radio resource, expected traffic arrival rate, redundancy-based protection of contention signals (e.g. time/frequency/code/space diversity employments), type and complexity of receiver algorithms, transmission timing prioritizing strategy. By considering these factors, reliability targets within a latency limit can be achieved.

Contention resolution scheme design is important to achieve latency and reliability requirement.

For instance, in contention-based resource allocations, a gNB usually is unaware of UE identities for a particular shared resource block. However, this awareness of UE ID could trigger a dynamic UL grant, providing dedicated resource for a timely UL transmission. Thus, it is beneficial to have a gNB to get ID of contending UEs, especially in cases of collision.

In order for network to identify the contending UE,

- One way is to include UE ID in the contention based transmission directly, which can rely on the physical layer design to differentiate between contending UEs.

- Another way is to carry this information on contention-free resources instead, e.g., SR on PUCCH, together with the contention based transmission. This is beneficial considering the little resource occupancy due to SR on PUCCH and the avoidance of contention.

A collision resolution mechanism based on reliable UE identification should be available for contention based access.

### Retransmissions

Given the latency constraints of URLLC the number of transmissions and retransmissions may need to be very limited. That means, if a collision occurs on a shared resource, the retransmission should have a high reliability, which is best ensured on a contention-free resource. It should therefore be possible to schedule the retransmission on a grant-based resource.

A retransmission mechanism should include methods for reducing the number of retransmission collisions with a limited impact on their latency.

Retransmissions of contention-based transmissions should be on granted resources.

# Conclusion

In section 2 we made the following observations:

Observation 1	Given large-bandwidth and short-transmission allocations suitable for URLLC and its infrequent traffic, shared resources with a contention-based transmission approach is important from a resource efficiency perspective.

Observation 2	Dynamic UL scheduling causes significantly longer delay as compared to DL.

Observation 3	A reliable collision resolution scheme is required for URLLC, since a high success rate must be achievable within a latency bound.

Observation 4	Contention resolution scheme design is important to achieve latency and reliability requirement.

Observation 5	A retransmission mechanism should include methods for reducing the number of retransmission collisions with a limited impact on their latency.

Based on the discussion in Section 2 we propose the following:

Proposal 1	Grant-free access by a preconfigured scheduling scheme is supported to reduce UL latency.

Proposal 2	UE does not transmit on a grant-free resource when it has an empty buffer.

Proposal 3	Grant-free access with shared resources and contention based transmission should be supported.

Proposal 4	The framework of grant-free resources should support fast switching to grant-based resources.

Proposal 5	A collision resolution mechanism based on reliable UE identification should be available for contention based access.

Proposal 6	Retransmissions of contention-based transmissions should be on granted resources.

# References

TR 36.881, Study on latency reduction techniques for LTE, v0.6.0

RP-160667, WI L2 latency reduction techniques for LTE.