3GPP TSG-RAN WG2 #97	Tdoc R2-1700837

# Athens, Greece, 13th – 17th February 2017

Agenda Item:	10.2.1.1

Source:	Ericsson

Title:	NR RLC segmentation header details

Document for:	Discussion, Decision

# Introduction

In previous RAN2 meetings, segmentation and concatenation functionality had been discussed and lead to the following agreements and working assumptions:

RAN2#95bis

Agreements

1: In NR, the segmentation function is only placed in the RLC layer as in LTE.

Agreement

=>	 SO-based segmentation can be considered for both segmentation and resegmentation as a baseline in NR user plane to support high data rate. (Does not imply anything about location of concatenation). At least overhead for the low data rate case should be analysed further.

RAN2#96-AH

Agreements

1  MAC sub-headers are interleaved with MAC SDUs.

Agreements

1	As in LTE the UEs shall not send padding if there is data available and the remaining TB size is greater than X bytes (actual number can be discussed later when header sizes are known. In LTE X = 7 bytes)

2	MAC CEs are not placed in the middle of the MAC PDU but at the beginning or at the end (placement can be decided later)

3	Working assumption on no RLC concatenation taken at RAN2#96 is confirmed (i.e. concatenation of RLC PDUs is performed in MAC)

It should be observed that even this paper is continuation of the discussion in NR Ad Hoc, the topic can be equally discussed in the normative phase of NR.

# Discussion

In RAN2#AdHoc several views on how the RLC headers should be updated to allow, as much as possible, pre-processing of user plane packets by layer-2. To reduce the overhead caused by the introduction of a Segmentation Offset (SO) when segmentation of a RLC SDU is required, two alternatives have been proposed. In [1] and [2] it is proposed to use a two-bit approach like the LTE RLC FI field; meanwhile in [3] and [4] it is advocated for a one bit indication.

## RLC segmentation and header length

Figure 1 illustrates how the MAC PDU is constructed, which is also further discussed below. It is highlighted in green, which header and payload can be pre-processed and stored in memory. It is shown in red, which headers must be created or modified on the fly i.e. are dependent on TB size.

Figure 1: MAC PDU construction.

If RLC does no longer support concatenation of RLC SDUs, the variable length train of L-fields in RLC is not needed anymore. The MAC L field points to the end of the RLC PDU and thereby to the beginning of the next MAC sub-header.

The RLC header must indicate whether a complete PDU or just a PDU Segment follows. If it is a segment, the Segment Offset (SO) field should indicate the position of the first payload octet in the SDU. When the RLC transmitter pre-generates RLC headers, it will typically assume that all RLC PDUs can be sent without segmentation and will hence not include the SO field. When the RLC transmitter is unable to send the entire RLC PDU due to limited space in the MAC PDU, it must at least indicate that the last payload octet of the RLC PDU is not the last octet of the RLC SDU. But since the first octet of the RLC PDU payload contains the first octet of the RLC SDU, the segment offset is 0 and should preferably be omitted. This avoids having to update the RLC header size for the first segment of an PDU.

The header of the RLC PDU indicates with one bit whether the last octet of the RLC PDU payload is also the last octet of the RLC SDU.

Any subsequent segment (not starting with the first octet of the RLC SDU) must carry the additional SO field. To inform the RLC receiver whether the SO field follows the RLC “base header”, an additional bit is needed in the AMD PDU header:

The header of the RLC PDU indicates with one bit whether the first octet of the RLC PDU payload is also the first octet of the RLC SDU.

If the above-mentioned header field indicates that the first octet of the RLC PDU is not the first octet of the RLC SDU the SO field is present in the header.

By using two bits to indicate the presence of the SO, it is possible to further support Layer-2 pre-processing and therefore pre-generation of packets requiring segmentation. This is possible because when the transport block size is determined by the transmitter, and if segmentation is unavoidable, it is not required to interleave any additional header field in an already pre-constructed PDU in memory, but only to update the segmentation flags for the first segment. The subsequent segments can be pre-constructed including the segmentation offset and only in case of middle segments, the segmentation flags might require update (see Figure 2).

Figure 2 Two bits aproach segmentation indication working with pre-generation (example of transmission requiring three segments)

In contrast, a one-bit solution requires to interleave in real-time the SO field because of the uncertainty about the size of the MAC PDU that can be constructed. Similarly, as in the previous solution, there is need for fields update in between each segment (unless the last-one), as shown by Figure 3.

Figure 3 One bit Segmentation indication impact in pre-generation (example of transmission requiring three segments)

This clearly give and advantage to the two bits indication and would contribute to a faster processing of packet transmissions. The exact placement of the header fields discussed here can be discussed in the work item phase.

# Conclusion

Based in the previous section discussion we made the following proposals

Proposal 1	The header of the RLC PDU indicates with one bit whether the last octet of the RLC PDU payload is also the last octet of the RLC SDU.

Proposal 2	The header of the RLC PDU indicates with one bit whether the first octet of the RLC PDU payload is also the first octet of the RLC SDU.

Proposal 3	If the above-mentioned header field indicates that the first octet of the RLC PDU is not the first octet of the RLC SDU the SO field is present in the header.

# References

R2-1700253. Further details on NR segmentation. Nokia, Alcatel-Lucent Shanghai Bell

R2-1700429	. Further design aspects of NR User Plane stack. Ericsson

R2-1700281. Further considerations on SO based segmentation. LG Electronics Inc.

R2-1700561. Further details of RLC protocol without concatenation. Qualcomm Incorporated