3GPP TSG-RAN WG2 #97	Tdoc R2-1700840

Athens, Greece, 13th – 17th February 2017	Resubmission of R2-1700433

Agenda Item:	10.2.1.5

Source: 	Ericsson

Title:  	Lossless PDCP SN reconfiguration at HO

Document for:	Discussion, Decision

# Introduction

This contribution introduces some solutions to achieve lossless PDCP SN reconfigurations.

It should be observed that this paper can also be discussed in the normative phase of NR.

# Discussion

If a radio bearer is configured with a large SN value range by a node due to a high data rate service, and the UE moves to a node not supporting that SN format, then it is necessary to shorten the SN length at handover. Also, when a UE is configured with a small SN value range and moves to a target node that supports a larger SN format, then the target node may need to extend the SN length due to the higher data rates which may be supported by this node.

Hence, the following cases for handling the PDCP SN format at handover exist:

- Both the source and the target node support the same SN value.
This case is handled without any data losses assuming that the same SN range is configured.

- The target node does not support the same set of SN range values, or the target node configures a SN value higher than the configured by the source node. In this case, the SN length needs to be extended at HO.
This case can be handled without any data losses.

- The target node does not support the same set of SN values range values, or the target node configures a SN value lower than the configured by the source node. In this case, the SN length range needs to be shortened.
This case is handled with data loss.

Note that it is sufficient to change PDCP SN length only during handover when MAC layers are reset and PDCP and RLC re-established. Changing the PDCP SN length at any time would be significantly more complex to specify and may require updates of the PDCP SN field of PDCP PDU(s) that are buffered in the RLC and the MAC layers.

PDCP SN may be reconfigured only during a handover

## SN length decrease

As listed above, SN length decrease is the only case which the standard does not handle efficiently. In this cases, all data which has not been received by the UE may be lost on the radio interface and would have to be retransmitted by higher layers (i.e. above PDCP).

Different approaches may be used to address this problem depending on ambition level:

- To study RAN mechanisms which reduce the data losses

- To study RAN mechanisms with no data losses

### Mechanism to reduce data loss (alt. 1)

When a handover happens, the source node forwards PDCP SDU(s) with the initial SN length to the target node.

In general, the conversation of the number may be performed as follows, using an example where the difference in SN size is 2 bits:

- The 2 most significant bits of the SN is moved to the HFN as two new bits of a new HFN and forming the 2 least significant bits of this new HFN;

- The 2 most significant bits of the initial SN is dropped to form a new shorter SN format.

The following picture shows an example about how this conversion can be done when the source node had configured a SN length of 14 bits and the target node configures a SN length of 12 bits.

HFN

PDCP SN

COUNT

2 MSB

HFN

PDCP SN

2 MSB

12 bit PDCP SN

20 bit HFN

HFN

PDCP SN

COUNT

2 MSB

HFN

PDCP SN

2 MSB

12 bit PDCP SN

20 bit HFN

If a PDCP SDU with a long SN length is sent from the source node to the target node, then the target node will only read a subset of the least significant bits of the SN length configured by the source node.

After the reconfiguration, the network may request a PDCP status report from the UE. According to current LTE specs, the FMS (First missing PDCP SN) that the UE transmits (after the reconfiguration) only contains the number of bits configured by the network for the PDCP SN. Since the new PDCP receiving window is smaller than the previous one, the UE should only send a report for a subset of the PDCP PDUs which were in the PDCP receiving window before the reconfiguration. This is exemplified in the figure below.

As indicated above, the UE PDCP receiving window after the reconfiguration will be smaller than before the reconfiguration due to the smaller SN length. Typically, the UE shouldn’t have filled more than half of the PDCP receiving window. Otherwise, there could be HFN desync issues. For this reason, after the reconfiguration, the UE should still consider half of the PDCP receiving window. This is depicted in the figure above.

The PDCP PDUs that have been received, and are waiting in the PDCP receiver window that is outside the shorter window range, can be forwarded to higher layers. This means that there will be data losses because the PDCP PDUs that are not received within this area of the window will be lost.

While this is quite simple, the fact that the receiver side only keeps a subset of the original PDCP receiving window, there is still risks of having PDCP losses.

### Mechanism to avoid data losses

Another way which would avoid data losses is to ensure that PDCP PDUs for which acknowledgement was not received, are retransmitted by the target cell.

In the network, for example, the source target sends all packets still pending for acknowledgment to the target node. The information includes both the HFN and the SN value. The target node then prepares new PDCP PDUs with a reconfigured SN length. The HFN and SN should be reset (i.e. initialized to zero)..

In the UE, before applying the new PDCP configuration, the UE forwards the received PDCP SDUs to higher layers (even if there are holes in the sequence number range), then resets the SN and HFN (i.e. initializes the values to zero).

If nothing more is done, the target node would simply retransmit all unacknowledged data. However, to limit the retransmissions, the network could request the UE to send a report like the FMS. This FMS would not use the SN length of the target cell but it should use a different SN length. Different options are possible. One option could be to use the SN length of the source cell. Another option could be to introduce a new format which could contain the HFN and the SN.

This solution has limited complexity and avoids losing data when a HO is performed when the PDCP SN length is reduced; thus, this solution is preferred.

# Proposals

We kindly request RAN2 to discuss and agree on:

Proposal 1	PDCP SN may be reconfigured only during a handover

Proposal 2	Capture these solutions in the TR, and discuss possible recommendations.