3GPP TSG-RAN WG2 #97	Tdoc R2-1700886

Athens, Greece, 13th – 17th February 2017

Agenda Item:	10.2.2.1

Source:	Ericsson

Title:	Use of INACTIVE to IDLE state transition

Document for:	Discussion, Decision

# Introduction

At the last meeting there was some progress on the state modelling and state transitions for NR. This contribution attempts to further progress the area of state transition from RRC_INACTIVE to RRC_IDLE addressing the following topic:

- When is this transition needed;

- How should it be triggered / executed.

A text proposal is made to 38.804 capturing the proposals.

# Current agreed text on state transitions (38.304)

Figure 5.5.2-1:	UE state machine and state transitions between NR and E-UTRAN

NOTE 2:	It is FFS how the UE transits from RRC_INACTIVE to RRC_IDLE in NR.

NOTE 3:	It is FFS how the UE transits from RRC_CONNECTED to RRC_INACTIVE

Paging operation details for the NR RRC_IDLE and RRC_INACTIVE state are specified in 10.1.1.2.

The following state transitions are supported between the aforementioned RRC states (as also presented in Figure 5.5.2-1):

-	from RRC_IDLE to RRC_CONNECTED, following the "connection setup" procedure (e.g. request, setup, complete);

-	from RRC_CONNECTED to RRC_IDLE, following (at least) the "connection release" procedure;

-	from RRC_CONNECTED to RRC_INACTIVE, following the "connection inactivation" procedure;

-	from RRC_INACTIVE to RRC_CONNECTED, following the "connection activation" procedure;

-	from RRC_INACTIVE to RRC_IDLE.

NOTE 4:	Number of steps for each RRC procedure and the corresponding RRC message will be decided in the work item phase.

# Discussion on the need for transitions from RRC_INACTIVE to RRC_IDLE

In our view the transition from RRC_INACTIVE to RRC_IDLE is needed to handle special cases where the AS context in the UE should be cleared (and the UE should go to IDLE). Example cases where may be needed:

- UE is performing a RAN area updating, but the network decides it is no longer needed to have UE in RRC_INACTIVE, in this case it can send a release message ordering UE to RRC_IDLE

- UE is initiating a state transition to RRC_CONNECTED by the network is overloaded, in this case the network may decide to release the UE RAN context and send UE to IDLE.

Observation 1: There are some scenarios where transitions from RRC_INACTIVE to RRC_IDLE is useful.

Given the agreement on the last RAN2 meeting that the UE will be reachable for CN-initiated paging in RRC_INACTIVE, it is not required to actively release the UE to RRC_IDLE from RRC_INACTIVE in case the RAN decides to delete the RAN Context and request the tear down of the CN / RAN connection. The reason for this is that this can be done on the network side only, and when data arrives (UL or DL) the UE and Network state would be re-synchronized (see discussion in R2-1700885).

- If DL data arrives, the UE will be paged from CN (since CN thinks UE is in RRC_IDLE), triggering the UE to contact the network which will trigger a NAS recovery since the RAN context cannot be resumed.

- If UL data arrives, the UE will try to resume the RAN context i.e. perform a transition to RRC_CONNECTED, which will in this case involve a NAS recovery since the RAN context cannot resume.

Observation 2: There are no need for an explicit network initiated procedure for ordering UEs in RRC_INACTIVE to RRC_IDLE.

In case the network for some reason wants to order a UE in RRC_INACTIVE to RRC_IDLE it would be possible to page the UE, and then when UE respond to the page, release the UE to RRC_IDLE. This is however not required since UE state will anyway be synchronized when signalling/data arrives.

Proposal 1. Transition from RRC_INACTIVE to RRC_IDLE can be supported using an RRC release procedure when the UE contacts the network.

Proposal 2. No network triggered procedure for order UEs in RRC_INACTIVE to RRC_IDLE is needed.

Proposal 3. Consider the proposed procedure above.

# Conclusion

Observation 1: There are some scenarios where transitions from RRC_INACTIVE to RRC_IDLE is useful.

Observation 2: There are no need for an explicit network initiated procedure for ordering UEs in RRC_INACTIVE to RRC_IDLE.

Proposal 1. Transition from RRC_INACTIVE to RRC_IDLE can be supported using an RRC release procedure when the UE contacts the network.

Proposal 2. No network triggered procedure for order UEs in RRC_INACTIVE to RRC_IDLE is needed.

Proposal 3. Consider the proposed procedure above

# Text proposal to TR 38.804

RRC supports the following three states which can be characterised as follows:

Editor’s note: more characteristics are to be added for each state depending on the progress of study.

-	RRC_IDLE:

-	Cell re-selection mobility;

-	[FFS:	The UE AS context is not stored in any gNB or in the UE;]

-	Paging is initiated by CN;

-	Paging area is managed by CN.

-	RRC_INACTIVE:

-	Cell re-selection mobility;

-	CN – NR RAN connection (both C/U-planes) has been established for UE;

-	The UE AS context is stored in at least one gNB and the UE;

-	Paging is initiated by NR RAN;

-	RAN-based notification area is managed by NR RAN;

-	NR RAN knows the RAN-based notification area which the UE belongs to;

-	RRC_CONNECTED:

-	The UE has an NR RRC connection;

-	The UE has an AS context in NR;

-	NR RAN knows the cell which the UE belongs to;

-	Transfer of unicast data to/from the UE;

-	Network controlled mobility, i.e. handover within NR and to/from E-UTRAN.

NOTE 1:	How to model RRC_INACTIVE in the specification will be decided in the work item phase.

Figure 5.5.2-1 illustrates an overview of UE state machine and state transitions in NR.

Figure 5.5.2-1:	UE state machine and state transitions between NR and E-UTRAN

NOTE 2:	It is FFS how the UE transits from RRC_INACTIVE to RRC_IDLE in NR.

NOTE 3:	It is FFS how the UE transits from RRC_CONNECTED to RRC_INACTIVE

Paging operation details for the NR RRC_IDLE and RRC_INACTIVE state are specified in 10.1.1.2.

The following state transitions are supported between the aforementioned RRC states (as also presented in Figure 5.5.2-1):

-	from RRC_IDLE to RRC_CONNECTED, following the "connection setup" procedure (e.g. request, setup, complete);

-	from RRC_CONNECTED to RRC_IDLE, following (at least) the "connection release" procedure;

-	from RRC_CONNECTED to RRC_INACTIVE, following the "connection inactivation" procedure;

-	from RRC_INACTIVE to RRC_CONNECTED, following the "connection activation" procedure;

-	from RRC_INACTIVE to RRC_IDLE, triggered in response to UE signaling.

NOTE 4:	Number of steps for each RRC procedure and the corresponding RRC message will be decided in the work item phase.