3GPP TSG RAN WG2 Meeting #97	R2-1701276

Athens, Greece, 13th-17th February 2017

Agenda item:	8.10.1

Source: 	Samsung

Title: 	MCCH and SI Change Notification

Document for:	Discussion and Decision

- Introduction

This contribution discusses about MCCH and SI change notification for MBMS-dedicated Cell.

- Discussion

Following agreement is made in RAN1#87 regarding MCCH change and SI change notification.

- For a Rel-14 FeMBMS carrier, the current DCI format 1-C used to indicate MCCH change, is extended to also indicate SI change.

In view of the above RAN1 agreement and also RAN2’s earlier agreement for targeting a common design for MCCH change and SI change notification, we discuss further details on the approach and periodicity of this mechanism.

In legacy, SI change notification is derived from BCCH modification period obtained from BCCH-Config and PCCH-Config [1].  RAN2 in previous meeting made working assumption that BCCH-Config and PCCH-Config IEs should not be provided for FeMBMS SI if RAN1 decides to use L1 signalling to notify the SI modification. Now with RAN1 agreement available, RAN2 can confirm this assumption.

Proposal 1: In accordance with RAN1 agreement, RAN2 confirm that BCCH-Config and PCCH-Config IEs are not provided MBMS-dedicated Cell.

In legacy, MCCH change notification configuration is obtained from SIB13 as

MBMS-NotificationConfig-r9 ::=				SEQUENCE {

notificationRepetitionCoeff-r9		ENUMERATED {n2, n4},

notificationOffset-r9				INTEGER (0..10),

notificationSF-Index-r9				INTEGER (1..6)

}

Targeting common design, RAN2 should decide to have a common notification configuration for MCCH and SI modification notification. As on MBMS dedicated Cell, SI is primarily targeted for MBMS operation, keeping same periodicity for MCCH and SI modification notification, as far as possible, will be quite efficient (e.g. power saving etc.).  Legacy MCCH change notification configuration should be adapted to meet this common notification configuration for MCCH and SI change.

Proposal 2:  As baseline, legacy MCCH notification configuration should be adapted to meet this common notification configuration signalling for MCCH and SI change in MBMS-dedicated cell.

As RAN1 has agreed to transmit the MCCH change and SI modification in the PDCCH region of the CAS, hence

we do not need notificationSF-Index as CAS sub-frame is fixed as 0.

Further, notificationRepetitionCoeff and notificationOffset should now define selected CAS (System Frame Number(SFN) of the CAS sub-frame) to carry notification as

SFN mod notification repetition period = notificationOffset

where notification repetition period is derived as shortest MCCH modification period/ notificationRepetitionCoeff and notificationOffset should be multiple of 4 to indicate the CAS. A set of values for notificationOffset provides flexibility to network to choose particular CAS subframes for notification purpose.

In legacy, MCCH modification period used is either fixed to rf512 or rf1024. There seems no reason for this to change for MBMS-dedicated cell. However, in case of SI modification, there are three possibilities:

Option 1: It is explicitly indicated by a new parameter siModificationPeriod (in absence of BCCH-Config and PCCH-Config) which can be configured to similar values like MCCH modification period. Since SI for MBMS-dedicated cell primarily serves MBMS, siModificationPeriod should have similar values configurable by network (modification period is not applicable to ETWS, CMAS and EAB parameters)

Option 2: siModificationPeriod is implicitly derived as the shortest MCCH modification period in the MBMS-dedicated cell. In this case, however, siModificationPeriod would be subject to changes in MBMS area configurations for MBMS-dedicated cell.

Option 3: siModificationPeriod is fixed as rf512. In this case, if the shortest MCCH modification period in the cell is rf1024, there would be fewer notifications for SI change. Therefore, it implies the notification repetition period is also fixed as rf512.

We think option 1 provides more flexibility for the network in configuration.

Possible common configuration message is as follows:

MBMS-NotificationConfig-r14 ::=		SEQUENCE {

notificationRepetitionCoeff-r14		ENUMERATED {n2, n4},

notificationOffset-r14				ENUMERATED {n0, n4, n8, n16, n32},

siModificationPeriod-r14			ENUMERATED {rf512, rf1024},

}

Proposal 3:  Common notification configuration message is provided for MCCH and SI change notification.

Proposal 4: siModificationPeriod is defined and its duration is provided in the new common notification message.

As indicated earlier, unlike legacy where notification is carried in MBSFN sub-frame only, for MBMS-dedicated Cell notification is carried in non-MBSFN sub-frame. RAN2 can check with RAN1 which bit will signify SI change, apart from 8-bit MCCH change indication and whether M-RNTI is still used to decode on this non-MBSFN sub-frame. This information should be useful for configuration purpose by higher layer.

Proposal 5: RAN2 checks with RAN1 about which bit in DCI 1-C will signify SI change, apart from 8-bit MCCH change indication, and whether M-RNTI is still used to decode on this non-MBSFN sub-frame.

- Conclusion

Proposal 1: In accordance with RAN1 agreement, RAN2 confirm that BCCH-Config and PCCH-Config IEs are not provided MBMS-dedicated Cell.

Proposal 2:  As baseline, legacy MCCH notification configuration should be adapted to meet this common notification configuration signalling for MCCH and SI change in MBMS-dedicated cell.

Proposal 3:  Common notification configuration message is provided for MCCH and SI change notification.

Proposal 4: siModificationPeriod is defined and its duration is provided in the new common notification message.

Proposal 5: RAN2 checks with RAN1 about which bit in DCI 1-C will signify SI change, apart from 8-bit MCCH change indication, and whether M-RNTI is still used to decode on this non-MBSFN sub-frame.

Annex giving procedural changes also provide capturing proposed changes in specification.

- References

[1] R2-168074 SI and MCCH change notification reception.doc

Related procedural changes (Annex)

*********First change**********

- 5.2.1.3	System information validity and notification of changes

Change of system information (other than for ETWS, CMAS and EAB parameters and other than for AB parameters for NB-IoT) only occurs at specific radio frames, i.e. the concept of a modification period is used. System information may be transmitted a number of times with the same content within a modification period, as defined by its scheduling. The modification period boundaries are defined by SFN values for which SFN mod m= 0, where m is the number of radio frames comprising the modification period. The modification period is configured by system information. If H-SFN is provided in SystemInformationBlockType1-BR, modification period boundaries for BL UEs and UEs in CE are defined by SFN values for which (H-SFN * 1024 + SFN) mod m=0. For NB-IoT, H-SFN is always provided and the modification period boundaries are defined by SFN values for which (H-SFN * 1024 + SFN) mod m=0.

To enable system information update notification for RRC_IDLE UEs configured to use a DRX cycle longer than the modification period, an eDRX acquisition period is defined. The boundaries of the eDRX acquisition period are determined by H-SFN values for which H-SFN mod 256 =0. For NB-IoT, the boundaries of the eDRX acquisition period are determined by H-SFN values for which H-SFN mod 1024 =0.

When the network changes (some of the) system information, it first notifies the UEs about this change, i.e. this may be done throughout a modification period. In the next modification period, the network transmits the updated system information. These general principles are illustrated in figure 5.2.1.3-1, in which different colours indicate different system information. Upon receiving a change notification, the UE not configured to use a DRX cycle that is longer than the modification period acquires the new system information immediately from the start of the next modification period. Upon receiving a change notification applicable to eDRX, a UE in RRC_IDLE configured to use a DRX cycle that is longer than the modification period acquires the updated system information immediately from the start of the next eDRX acquisition period. The UE applies the previously acquired system information until the UE acquires the new system information. The possible boundaries of modification for SystemInformationBlockType1-BR are defined by SFN values for which SFN mod 512 = 0 except for notification of ETWS/CMAS for which the eNB may change SystemInformationBlockType1-BR content at any time. For NB-IoT, the possible boundaries of modification for SystemInformationBlockType1-NB are defined by SFN values for which (H-SFN * 1024 + SFN) mod 4096 = 0.

Figure 5.2.1.3-1: Change of system Information

The Paging message is used to inform UEs in RRC_IDLE and UEs in RRC_CONNECTED about a system information change. If the UE is in RRC_CONNECTED or is not configured to use a DRX cycle longer than the modification period in RRC_IDLE, and receives a Paging message including the systemInfoModification, it knows that the system information will change at the next modification period boundary. A UE in RRC_IDLE that is configured to use a DRX cycle longer than the modification period, and receives in an eDRX acquisition period at least one Paging message including the systemInfoModification-eDRX, shall acquire the updated system information at the next eDRX acquisition period boundary. Although the UE may be informed about changes in system information, no further details are provided e.g. regarding which system information will change, except if systemInfoValueTagSI is received by BL UEs or UEs in CE.

In MBMS-dedicated cell, the possible boundaries of modification for system information are defined by SFN values for which SFN mod siModificationPeriod = 0. System information change notification transmitted over CAS subframe is used to inform UEs about a system information change. On reception of system information change, UE knows that the system information will change at the next modification period boundary.

In RRC_CONNECTED, BL UEs or UEs in CE or NB-IoT UEs are not required to acquire system information except when T311 is running or upon handover where the UE is only required to acquire the MasterInformationBlock in the target PCell. In RRC_IDLE, E-UTRAN may notify BL UEs or UEs in CE or NB-IoT UEs about SI update, and except for NB-IoT, ETWS and CMAS notification and EAB modification, using Direct Indication information, as specified in 6.6 (or 6.7.5 in NB-IoT) and TS 36.212 [22].

NOTE:	Upon system information change essential for BL UEs, UEs in CE, or NB-IoT UEs in RRC_CONNECTED, E-UTRAN may initiate connection release.

*********Next change**********

- 5.8.1.3	MCCH information validity and notification of changes

Change of MCCH information only occurs at specific radio frames, i.e. the concept of a modification period is used. Within a modification period, the same MCCH information may be transmitted a number of times, as defined by its scheduling (which is based on a repetition period). The modification period boundaries are defined by SFN values for which SFN mod m= 0, where m is the number of radio frames comprising the modification period. The modification period is configured by means of SystemInformationBlockType13.

When the network changes (some of) the MCCH information, it notifies the UEs about the change during a first modification period. In the next modification period, the network transmits the updated MCCH information. These general principles are illustrated in figure 5.8.1.3-1, in which different colours indicate different MCCH information. Upon receiving a change notification, a UE interested to receive MBMS services acquires the new MCCH information immediately from the start of the next modification period. The UE applies the previously acquired MCCH information until the UE acquires the new MCCH information.

Figure 5.8.1.3-1: Change of MCCH Information

Indication of an MBMS specific RNTI, the M-RNTI (see TS 36.321 [6]), on PDCCH is used to inform UEs in RRC_IDLE and UEs in RRC_CONNECTED about an MCCH information change. When receiving an MCCH information change notification, the UE knows that the MCCH information will change at the next modification period boundary. The notification on PDCCH indicates which of the MCCHs will change, which is done by means of an 8-bit bitmap. Within this bitmap, the bit at the position indicated by the field notificationIndicator is used to indicate changes for that MBSFN area: if the bit is set to "1", the corresponding MCCH will change. No further details are provided e.g. regarding which MCCH information will change. The MCCH information change notification is used to inform the UE about a change of MCCH information upon session start or about the start of MBMS counting.

The MCCH information change notifications on PDCCH are transmitted periodically and are carried on MBSFN subframes only. These MCCH information change notification occasions are common for all MCCHs that are configured, and configurable by parameters included in SystemInformationBlockType13: a repetition coefficient, a radio frame offset and a subframe index. These common notification occasions are based on the MCCH with the shortest modification period.

NOTE 1:	E-UTRAN may modify the MBMS configuration information provided on MCCH at the same time as updating the MBMS configuration information carried on BCCH i.e. at a coinciding BCCH and MCCH modification period. Upon detecting that a new MCCH is configured on BCCH, a UE interested to receive one or more MBMS services should acquire the MCCH, unless it knows that the services it is interested in are not provided by the corresponding MBSFN area.

A UE that is receiving an MBMS service via MRB shall acquire the MCCH information from the start of each modification period. A UE interested to receive MBMS from a carrier on which dl-Bandwidth included in MasterInformationBlock is set to n6 shall acquire the MCCH information at least once every MCCH modification period. A UE that is not receiving an MBMS service via MRB, as well as UEs that are receiving an MBMS service via MRB but potentially interested to receive other services not started yet in another MBSFN area from a carrier on which dl-Bandwidth included in MasterInformationBlock is other than n6, shall verify that the stored MCCH information remains valid by attempting to find the MCCH information change notification at least notificationRepetitionCoeff times during the modification period of the applicable MCCH(s), if no MCCH information change notification is received.

NOTE 2:	In case the UE is aware which MCCH(s) E-UTRAN uses for the service(s) it is interested to receive, the UE may only need to monitor change notifications for a subset of the MCCHs that are configured, referred to as the 'applicable MCCH(s)' in the above.

In MBMS-dedicated cell, indication of an MBMS specific RNTI, the M-RNTI, on PDCCH is used to inform UEs about an MCCH information change or system information change. When receiving an MCCH information change notification, the UE knows that the MCCH information will change at the next modification period boundary.

The MCCH information change or system information change notifications on PDCCH are transmitted periodically on CAS subframe. These MCCH information change notification occasions are common for all MCCHs that are configured, and configurable by parameters included in SystemInformationBlockType13:a repetition coefficient, a radio frame offset and SI modification period. These common notification occasions are based on the MCCH with the shortest modification period.

*********Next change**********

- 6.3.7	MBMS information elements

- –	MBMS-NotificationConfig

The IE MBMS-NotificationConfig specifies the MBMS notification related configuration parameters, that are applicable for all MBSFN areas.

MBMS-NotificationConfig information element

-- ASN1START

MBMS-NotificationConfig-r9 ::=				SEQUENCE {

notificationRepetitionCoeff-r9		ENUMERATED {n2, n4},

notificationOffset-r9				INTEGER (0..10),

notificationSF-Index-r9				INTEGER (1..6)

}

MBMS-NotificationConfig-r14 ::=		SEQUENCE {

notificationRepetitionCoeff-r14		ENUMERATED {n2, n4},

notificationOffset-r14				ENUMERATED {n0, n4, n8, n16, n32},

siModificationPeriod-r14			ENUMERATED {rf512, rf1024},

}

-- ASN1STOP

MBMS-NotificationConfig field descriptions

notificationOffset Indicates, together with the notificationRepetitionCoeff, the radio frames in which the MCCH information change notification is scheduled i.e. the MCCH information change notification is scheduled in radio frames for which: SFN mod notification repetition period = notificationOffset.

notificationRepetitionCoeff Actual change notification repetition period common for all MCCHs that are configured= shortest modification period/ notificationRepetitionCoeff. The ‘shortest modificaton period’ corresponds with the lowest value of mcch-ModificationPeriod of all MCCHs that are configured. Value n2 corresponds to coefficient 2, and so on.

notificationSF-Index Indicates the subframe used to transmit MCCH change notifications on PDCCH. FDD: Value 1, 2, 3, 4, 5 and 6 correspond with subframe #1, #2, #3 #6, #7, and #8 respectively. TDD: Value 1, 2, 3, 4, and 5 correspond with subframe #3, #4, #7, #8, and #9 respectively.

notificationOffset-r14 Indicates, together with the notificationRepetitionCoeff, the radio frames having CAS subframe in which the MCCH information change or system information change notification is scheduled i.e. the MCCH information change or system information change notification is scheduled in radio frames for which: SFN mod notification repetition period = notificationOffset

notificationRepetitionCoeff-r14 Actual change notification repetition period for system information and common for all MCCHs that are configured= shortest modification period/ notificationRepetitionCoeff. The ‘shortest modificaton period’ corresponds with the lowest value of mcch-ModificationPeriod of all MCCHs that are configured. Value n2 corresponds to coefficient 2, and so on.

siModificationPeriod-r14 Indicates the period after which SystemInformation transmitted can change. Value rf512 corresponds to 512 radio frames, rf1024 corresponds to 1024 radio frames.