3GPP TSG-RAN WG2 NR #97 Meeting                                                      R2-1701478

Athens, Greece 13th-17th February 2017                              (Resubmission of R2-1700465)

Agenda item:	10.3.1.1.1

Source:	Samsung

Title:	Connected mobility for multiple numerologies in NR

Document for:	Discussion & Decision

# 1. Introduction

RAN#71 in March approved a 5G SID [1] on New Radio (NR) access technology, which targets a unified framework for traffic with diverse QoS requirements e.g. enhanced mobile broadband (eMBB), massive machine-type-communications (mMTC), ultra-reliable and low latency communications (URLLC).

In RAN1 [2] using different sets of numerology and frame structures for different service verticals is being considered. From RAN1#87 and Ad-hoc meetings, the followings were agreed:

RAN1 Agreements (#87)

- NR should provide support for carrier aggregation, including different carriers having same or different numerologies.

- For DL, dynamic resource sharing between URLLC and eMBB is supported by transmitting URLLC scheduled traffic

  - URLLC transmission may occur in resources scheduled for ongoing eMBB traffic

- NR strives for efficient support of dynamic resource allocation of different numerologies in FDM/TDM fashion.

  - Potential specification impact includes but is not limited to:

    - FFS:CSI-RS measurement

  - FFS: the time and frequency granularity of dynamic resource allocation

    - FFS: If spectrum confinement (filtering, windowing, …) can be dynamically varied or not

- For initial access, UE can assume a signal corresponding to a specific subcarrier spacing of NR-PSS/SSS in a given frequency band given by specification

- NR should support adaptation and network indication of the valid time and frequency resources which may be used for inter-/intra-frequency RRM measurements and reports for ‘CONNECTED’ mode UEs

  - FFS: UE-specific and/or cell-specific indication

  - FFS: dynamic adaptation/configuration

  - FFS: idle and possibly new state modes

  - FFS: whether to support aperiodic

- It is up to RAN4 for determining requirements regarding the extent the UE can restrict its measurement to a subband of the configured bandwidth

- Companies are encouraged to bring results and analysis until RAN1-NR considering at least the following aspects:

  - Cell coverage in CONNECTED and IDLE

  - Overhead of RS resources (e.g. Number of resource elements, BW used for RS mapping, Resource usage in time)

  - Accuracy of the RS measurement quantity

RAN1 Agreements (#Ad-hoc JAN.)

- The following always-on signals are used for RRM measurement for L3 mobility in IDLE mode:

  - NR synchronization signal, or

  - NR synchronization signal, and additional DM-RS for PBCH if DM-RS is supported for PBCH, or

    - Note: How to use DM-RS for RRM measurement is up to UE implementation

  - DM-RS for PBCH if DM-RS is supported for PBCH

  - Note that down selection will be needed if DM-RS for PBCH is supported

- For CONNECTED mode RRM measurement for L3 mobility, the following RS can be used if needed, in addition to IDLE mode RS:

  - FFS: CSI-RS,

  - FFS: RS separately designed from CSI-RS

  - Note that possibility of multiplexing of wideband RS in SS block is not precluded

From RAN2#94/96 meetings, the followings were agreed:

RAN2 Agreements (#94)

3: 	For typical NR inter-eNB network controlled mobility, minimise the required measurement configuration to be provided to the UE to configure measurements (e.g. avoid the need to provide detailed 'cell' level information). More detailed information may be provided to address some cases.

RAN2 Agreements (#96)

Agreements for connected active

1: The scope of the RRM measurement should mainly be to facilitate the RRC driven ‘cell’ level mobility.

2: 	RAN2 working assumption to be confirmed by RAN1:

a/ Connected active mode RRM measurement and reporting based on at least the signals used by idle mode RRM measurement should be supported in the NR.

b/  Additional RS may need to be used for RRM measurement in the connected active mode besides the signals used by idle mode RRM measurement, which is dependent on RAN1’s decision.

3: 	The RRM measurement framework (measurement object, measurement id, reporting config) in LTE as a baseline in NR.

This contribution discusses RRM measurement and handover procedure in NR when the network supports multiple numerologies.

# 2. 	Design Considerations for Connected Mobility in NR

  - Key Design Issues to Support Multiple Numerologies for HO

Resource for RRM measurement

For the convenience of discussion, how the resource for RRM measurement is configured can be categorized as following options:

Option 1 – common resource across cells

Option 2 – only numerology specific resource per cell

Option 3 – common resource part across cells + numerology specific additional resource part per cell

For design simplicity, it is nice to have a common operation as well as common resource/reference signal for RRM measurement (as option 1), regardless of the type of service/slice. The common operation/resource, however, may not be efficient from the resource usage perspective, if it must support all kinds of services as well as requirements – even including future services. For example in the case of high mobility or wide BW, more resource elements to map RS are needed to acquire accurate measurement quantity. If we decide common resource for RRM measurement, the network should provide the densest RS to meet the most stringent requirement of a service requiring high mobility or wide BW. This may incur the higher overhead to the network for supporting the multiple services/numerologies.

If we separate resources for each service/slice group which can be categorized in terms of supporting numerology (as option 2), resources in carrier could be used more efficiently. For example, URLLC service may require shorter periodicity of RS than that for eMBB service. If URLLC UE is not connected to a cell at all, it is fine for the cell to provide just the nominal periodicity of RS for RRM measurement. Furthermore, the impact from the additional RS could be confined within the part of whole resource which is assigned only for the specific service.

Since option 1 and 2 has each benefit and drawback, option 3 can be considered as a compromising way. According to option 3, common resource for RRM measurement is assigned across cells to meet nominal services/requirements. If services with more stringent requirements are needed, additional resources could be assigned to provide proper accuracy for RRM measurement.

Based on the observation, we assume option 1 as a baseline and option 3 for further optimization for RRM measurement and HO procedure.

Observation 1: Common resource across cells for RRM measurement regardless of the type of numerology/service is beneficial for design simplicity, but may not provide sufficient accuracy of measurement quantity for all possible services.

Observation 2: Different resource for RRM measurement per numerology could be considered for further optimization from the resource efficiency and measurement accuracy perspectives.

Type of RS for RRM measurement

RAN1 have studied on possible type of RS for RRM measurement for IDLE and CONNECTED. So far, three types of RS(s) are on the table for always-on signal; NR-xSS (synchronization signal), NR-xSS and additional DM-RS for PBCH, or DM-RS for PBCH. It is noted that DM-RS for PBCH is not agreed yet. Thus we can assume that at least NR-xSS is used as common and always-on signal for RRM measurement for IDLE mode. Meanwhile, in addition to the IDLE mode RS, CSI-RS or new RS different from CSI-RS are candidates for CONNECTED mode RRM measurement.

The overhead of NR-xSS should be minimized since NR-xSS is the “Always-on” signal in NR. The aspects of the overhead comprise number of resource elements, BW used for RS mapping, and resource usage in time. However there is a trade-off between minimizing overhead of RS and accuracy of RS measurement. Due to the longer period of NR-xSS compared to that (1ms) of CRS in LTE, the network may not provide the sufficient number of samples to UE only via NR-xSS, subject to detection error and tracking multiple cells. Thus additional RS may be required in specific scenarios to achieve more accurate RS measurement quality. Having said that, we assume that NR-xSS is used for both IDLE and CONNECTED, and additional RS is used for CONNECTED. Definitely NR-xSS in common resource can be contributed to RRM measurement of CONNECTED UE as well.

Observation 3: According to RAN1 discussion on RS for RRM measurement, NR-xSS or additional RS is considered as candidates for CONNECTED.

Observation 4: There is a design consideration - a trade-off between RS overhead reduction and accuracy of measurement.

Observation 5: Connected active mode RRM measurement and reporting are feasible based on signals in common resource regardless of services, at the cost of resource wastage.

Indication of measurement resource

RAN1 agreed at RAN1#87 meeting on the indication of valid time and frequency resource for inter-/intra-frequency RRM measurements and reports for CONNECTED mode UE. Especially for intra-frequency case, the agreement implies that a time and frequency portion of the carrier is indicated to CONNECTED UE. The time and frequency portion is indicated to measure neighbouring cells, since for the serving cell UE already knows the location of RS.

Since UE-specific indication to dynamic reconfiguration may cause high signalling overhead in backhaul, the indication can be minimized in size or be distributed via over-the-air signal. For the serving cell, at least UE-specific (via RRC like LTE) indication could be reused.

Proposal 1: To CONNECTED mode UE, measurement resource including time/frequency portion of the carrier should be indicated by the network for neighbouring cell RRM measurement. Minimizing overhead in network side and UE complexity should be taken into account for design of the indication.

Potential impact of numerology to measurement/HO accuracy

The single cell may separate the resource into multiple regions comprising common and dedicated numerologies for multiple services/slices and configure those multiple regions dynamically to avoid resource wastage. Under these situations, UE which operates with dedicated numerology will monitor neighbouring cells via common resource and common numerology, since the UE does not know the dedicated resource/numerology configuration at those neighbouring cells. In case that the UE measures neighbouring cells only by common numerology within common resource, there is no impact from numerology mismatch between serving and neighbouring cells.

Observation 6: There is no issue if UE measures neighbouring cells only by common numerology/RS within common RRM resource across cells.

There however may be some constraints for RRM measurement based on common resource. First, it is only applicable to synchronous cell deployment. Second, as identified in observations, it may not meet requirements for all kind of services, but meet those for typical services. Last, it may suppress the freedom of cell planning and HO strategy for different service/slices.

If further enhancements are considered to handle those constrained scenarios, the alignment of common resource/numerology across cells is not assured or UE may experience numerology mismatch between serving cell and neighbouring cells. This mismatch of numerology between serving and neighbouring cell is harmful to obtain the consistent cell quality when resource is shared dynamically. Although the numerology where a UE operates for measurement is static, the numerology as well as physical parameters which are associated to the resource of neighbouring cell could be transient (e.g. time sharing). In this case, the measured cell quality of the neighbouring cell may be varied across different resource regions with different numerologies. Without any clue, the UE just obtains filtered result based on all those measured quantity regardless of numerology type which is used for the RRM resource.

Having said that, to measure accurate cell quality in certain limited scenarios, differentiation of measured RRM resource in terms of numerology is required. In this respect, the time domain measurement resource restrictions for the UE to support LTE eICIC could be reused. Owing to the restriction, the UE can measure just a reliable resource for RRM measurement.

Observation 7: There may be some potential issues due to numerology mismatch only for limited RRM measurement scenarios.

Proposal 2:  Even though there are potential problems due to numerology mismatch, measurement resource restriction for LTE eICIC could be reused. It is open for discussion whether further enhancement for NR is needed or not.

# 3. Summary

Based on the discussion and identified observations in this contribution, we propose followings:

Observation 1: Common resource across cells for RRM measurement regardless of the type of numerology/service is beneficial for design simplicity, but may not provide sufficient accuracy of measurement quantity for all possible services.

Observation 2: Different resource for RRM measurement per numerology could be considered for further optimization from the resource efficiency and measurement accuracy perspectives.

Observation 3: According to RAN1 discussion on RS for RRM measurement, NR-xSS or additional RS is considered as candidates for CONNECTED.

Observation 4: There is a design consideration - a trade-off between RS overhead reduction and accuracy of measurement.

Observation 5: Connected active mode RRM measurement and reporting are feasible based on signals in common resource regardless of services, at the cost of resource wastage.

Observation 6: There is no issue if UE measures neighbouring cells only by common numerology/RS within common RRM resource across cells.

Observation 7: There may be some potential issues due to numerology mismatch only for limited RRM measurement scenarios.

Proposal 1: To CONNECTED mode UE, measurement resource including time/frequency portion of the carrier should be indicated by the network for neighbouring cell RRM measurement. Minimizing overhead in network side and UE complexity should be taken into account for design of the indication.

Proposal 2: Even though there are potential problems due to numerology mismatch, measurement resource restriction for LTE eICIC could be reused. It is open for discussion whether further enhancement for NR is needed or not.

References

[1] R2-160671, New SID Proposal: Study on NR New Radio Access Technology

[2] 3GPP TR 36.889 (2016-03)