3GPP TSG-RAN WG2 #97	Tdoc R2-1701652

# Athens, Greece, 13th – 17th February 2017

Agenda Item:	8.18.2

Source:	Ericsson

Title:	UE and eNB interaction for codec bit-rate selection

Document for:	Discussion, Decision

# Introduction

At RAN#73, a work item on Voice and Video enhancements for LTE has been approved [1]. The objectives of this work item are to specify the following features based on the output of the study item report [2]:

- The codec mode/rate selection and adaptation solution details specification

- VoLTE/ViLTE signalling optimization

- VoLTE quality/coverage enhancements

During RAN2#96 it was agreed [3]:

- Introduce a rate recommendation query message from the UE to the eNB with a MAC CE introduced for bit rate recommendation message from the eNB to the UE.

  - FFS on avoiding ping-pong between UE and eNB.

  - In this case UE should not go beyond the recommendation bitrate from eNB.

  - Whether to use MAC CE for this message can be revisited if necessary.

This document discusses the definition and values of the bit rate recommendation conveyed in the eNB-to-UE message and the RAN mechanisms to avoid a potential “ping-pong” effect of the rate adaptation.

# Discussion

## Recommended bitrate

The recommended bitrate is defined in [4] as the bitrate in Kbps of the physical layer at the time when the decision is made.

An example on how the codec bitrate for the EVS 7.2 Kbps translates to the bitrate at the physical layer is presented in [4]. The examples contain the effect of MAC, RLC, and PDCP headers and assumes the case of fully operating ROHC header compression. With the assumptions of long RLC and PDCP sequence numbers and PHR and long BSR for every transmission, the bitrate at the physical layer is 12.8 Kbps (an overhead of 5.6Kbps) with one RTP packet containing one speech frame and no RLC segmentation is done.

A further calculation of the transport overhead is presented in [4] with four speech frames bundled into one RTP packet by the application, and the bitrate at the physical layer is for this case 9Kbps (an overhead of 1.8 Kbps) under the same assumption of no RLC segmentation and sizes of MAC headers and MAC CEs. This application encapsulation of speech frames into single RTP packets is not supported by [6], but may serve as an example for the current exposition.

Using a MAC bundling of two RTP packets into one transmission and short RLC and PDCP sequence numbers and no PHR or BSR gives an overhead of 2.2Kbps.

Based on the discussion above, it can be noted that the actual overhead will be implementation dependent, but the range of the overhead is assumed to be within 2 to 6 Kbps due to the RAN headers and different aggregation and encapsulation schemes.

The IP and higher layer overhead for the speech codec data is 320 bits for Ipv4 and 480 for Ipv6 [7], corresponding to an addition of 16Kbps and 24Kbps if ROHC is not applied. With full ROHC header compression, it is assumed in [5] that the headers are compressed to 24 bits, corresponding to 1.2 Kbps.

The bitrate for the RTP payload at the physical layer including RAN headers and the IP and higher layer overhead for the AMR and AMR-WB codecs [7] and the EVS codec for the headerless payload format [8] for different assumptions of RAN headers, state of ROHC, and IP version are summarized in Table 1.

Table 1: Codec payload bitrate

| AMR | AMR-WB | EVS | Payload |  | Bitrate at physical layer [Kbps] |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Size [bits] | Bitrate [Kbps] | Assumed 1.8 Kbps RAN header overhead |  |  |  | Assumed 5.6 Kbps RAN header overhead |  |  |  |
|  |  |  |  |  | Excl. IP | Ipv4 | Ipv6 | Full ROHC | Excl. IP | Ipv4 | Ipv6 | Full ROHC |
| 4.75 |  |  | 112 | 5,6 | 8 | 24 | 32 | 9 | 12 | 28 | 36 | 13 |
| 5.15 |  |  | 120 | 6 | 8 | 24 | 32 | 9 | 12 | 28 | 36 | 13 |
| 5.9 |  |  | 128 | 6,4 | 9 | 25 | 33 | 10 | 12 | 28 | 36 | 14 |
|  |  | AMRWB 6.6 IO | 136 | 6,8 | 9 | 25 | 33 | 10 | 13 | 29 | 37 | 14 |
| 6.7 | 6.6 | 7.2 | 144 | 7,2 | 9 | 25 | 33 | 11 | 13 | 29 | 37 | 14 |
| 7.4 |  | 8 | 160 | 8 | 10 | 26 | 34 | 11 | 14 | 30 | 38 | 15 |
| 7.95 |  |  | 176 | 8,8 | 11 | 27 | 35 | 12 | 15 | 31 | 39 | 16 |
|  |  | AMRWB 8.85 IO | 184 | 9,2 | 11 | 27 | 35 | 13 | 15 | 31 | 39 | 16 |
|  | 8.85 | 9.6 | 192 | 9,6 | 12 | 28 | 36 | 13 | 16 | 32 | 40 | 17 |
| 10.2 |  |  | 216 | 10,8 | 13 | 29 | 37 | 14 | 17 | 33 | 41 | 18 |
| 12.2 |  | AMRWB 12.65 IO | 256 | 12,8 | 15 | 31 | 39 | 16 | 19 | 35 | 43 | 20 |
|  | 12.65 | 13.2 | 264 | 13,2 | 15 | 31 | 39 | 17 | 19 | 35 | 43 | 20 |
|  |  | AMRWB 14.25 IO | 288 | 14,4 | 17 | 33 | 41 | 18 | 20 | 36 | 44 | 22 |
|  | 14.25 |  | 296 | 14,8 | 17 | 33 | 41 | 18 | 21 | 37 | 45 | 22 |
|  |  | AMRWB 15.65 IO | 320 | 16 | 18 | 34 | 42 | 19 | 22 | 38 | 46 | 23 |
|  | 15.85 | 16.4 | 328 | 16,4 | 19 | 35 | 43 | 20 | 22 | 38 | 46 | 24 |
|  |  | AMRWB 18.25 IO | 368 | 18,4 | 21 | 37 | 45 | 22 | 24 | 40 | 48 | 26 |
|  | 18.25 |  | 376 | 18,8 | 21 | 37 | 45 | 22 | 25 | 41 | 49 | 26 |
|  |  | AMRWB 19.85 IO | 400 | 20 | 22 | 38 | 46 | 23 | 26 | 42 | 50 | 27 |
|  | 19.85 |  | 408 | 20,4 | 23 | 39 | 47 | 24 | 26 | 42 | 50 | 28 |
|  |  |  | 464 | 23,2 | 25 | 41 | 49 | 27 | 29 | 45 | 53 | 30 |
|  | 23.05 | AMRWB 23.05 IO | 472 | 23,6 | 26 | 42 | 50 | 27 | 30 | 46 | 54 | 31 |
|  |  | AMRWB 23.85 IO | 480 | 24 | 26 | 42 | 50 | 27 | 30 | 46 | 54 | 31 |
|  | 23.85 | 24.4 | 488 | 24,4 | 27 | 43 | 51 | 28 | 30 | 46 | 54 | 32 |
|  |  | 32 | 640 | 32 | 34 | 50 | 58 | 35 | 38 | 54 | 62 | 39 |
|  |  | 48 | 960 | 48 | 50 | 66 | 74 | 51 | 54 | 70 | 78 | 55 |
|  |  | 64 | 1280 | 64 | 66 | 82 | 90 | 67 | 70 | 86 | 94 | 71 |
|  |  | 96 | 1920 | 96 | 98 | 114 | 122 | 99 | 102 | 118 | 126 | 103 |
|  |  | 128 | 2560 | 128 | 130 | 146 | 154 | 131 | 134 | 150 | 158 | 135 |

The bitrate at the physical layer for the voice media varies significantly between different configurations of the VoLTE service and the RAN and a sufficiently rich granularity of the recommended physical layer bitrate is needed.

The recommended bitrate at the physical layer includes all RAN headers as well as the effect of the operation of ROHC.

A proposed range of bitrates at the physical layer covering the bitrates produced by different configurations of the system and speech codecs being used is presented in Table 2. The lower values in the table, index 2 to 16 (corresponding to bitrates 8Kbps to 104Kbps) are primarily designed for the voice media. For the higher bitrate values, a pseudo-logarithmic scaling is applied to provide a range of bitrates that is deemed suitable for different bitrates of a video stream for real-time communication.

The index 1, with value 0 Kbps, is considered for e.g. signalling from the eNB to the UE that currently no media is recommended to be transmitted on the logical channel, but that the data bearer may still be maintained and media control plane data (e.g. RTCP) may be sent for keeping the media connection alive.

The index 0 is considered for signalling to the UE that currently no recommended bitrate is available or that a previous bitrate recommendation no longer is valid but no new recommendation is given.

Table 2: Proposed bitrate values for the Recommended bitrate

| Index | Recommended Bitrate value [Kbit/s] | Index | Recommended Bitrate value [Kbit/s] |
| --- | --- | --- | --- |
| 0 | Note 1 | 32 | 700 |
| 1 | 0 | 33 | 800 |
| 2 | 8 | 34 | 900 |
| 3 | 10 | 35 | 1000 |
| 4 | 12 | 36 | 1100 |
| 5 | 16 | 37 | 1200 |
| 6 | 20 | 38 | 1300 |
| 7 | 24 | 39 | 1400 |
| 8 | 28 | 40 | 1500 |
| 9 | 32 | 41 | 1750 |
| 10 | 36 | 42 | 2000 |
| 11 | 40 | 43 | 2250 |
| 12 | 48 | 44 | 2500 |
| 13 | 56 | 45 | 2750 |
| 14 | 72 | 46 | 3000 |
| 15 | 88 | 47 | 3500 |
| 16 | 104 | 48 | 4000 |
| 17 | 120 | 49 | 4500 |
| 18 | 140 | 50 | 5000 |
| 19 | 160 | 51 | 5500 |
| 20 | 180 | 52 | 6000 |
| 21 | 200 | 53 | 6500 |
| 22 | 220 | 54 | 7000 |
| 23 | 240 | 55 | 7500 |
| 24 | 260 | 56 | 8000 |
| 25 | 280 | 57 | Reserved |
| 26 | 300 | 58 | Reserved |
| 27 | 350 | 59 | Reserved |
| 28 | 400 | 60 | Reserved |
| 29 | 450 | 61 | Reserved |
| 30 | 500 | 62 | Reserved |
| 31 | 600 | 63 | Reserved |
| Note1: This index is used for indicating that the previous bitrate recommendation is no longer valid and no new bitrate recommendation is given. |  |  |  |

Use the values presented in Table 2 for the RAN recommended bitrate.

## UE bitrate query and eNB reporting

The use cases for code bitrate adaptation during an on-going call is described in Sections 5.3.2 and 5.4.3 of [2]. This is further addressed in [9] for the operation of an MTSI client in the case of adapting the bitrate in response to a bitrate recommendation from the eNB:

When an MTSI client in terminal receives an RMAB message for the local uplink that triggers adaptation:

- For video and if TMMBR is supported in the session, a corresponding TMMBN message notifying the remote media receiver of this changed local uplink restriction shall be sent, as described in clause 10.3.2.

- For both video and voice, an adaptation resulting in a reduction of the media sender bitrate shall be initiated immediately without further signaling.

- For the case of video and if TMMBR is supported in the session:

  - An adaptation resulting in an increase of the media sender bitrate in uplink shall delay the media bitrate increase to allow sufficient time for the remote media receiver to receive and react to the TMMBN in bullet 1 above.

  - It is recommended that the bitrate in a TMMBN from bullet 1 that is increasing the media sender bitrate in uplink is set to correspond to the previously received RMAB message, to avoid unnecessary TMMBN, RMABQ, and RMAB signaling (see bullet 3 below) caused by a possible step-wise increase.

  - The remote media receiver receiving a TMMBN with increased bitrate shall, if found necessary, send a TMMBR with an appropriate lower bitrate than was received in the TMMBN, as described by section 3.5.4 of CCM [43]. When deciding whether or not to send TMMBR, the remote media receiver shall take all available adaptation triggers into account, e.g. any bitrate limit from a received RMAB message. The remote media receiver [should/may] therefore send an RMABQ message for its downlink, if supported, to trigger receiving an RMAB message with recent information.

  - After such delay in bullet 3.a) above, the media sender bitrate is increased. The bitrate increase shall take all available adaptation triggers into account, which can cause the bitrate increase to be separated into several steps (see clause C.2.5).

A UE bitrate query supports the use case for video rate adaptation described in [9].

It has been agreed to have a prohibit timer for this query message to ensure that the eNB can control how often a UE can transmit a query. Since a client may have several media flows on different logical channels and the query is separate for downlink and uplink direction, the UE should be able send separate queries to request information about the different logical channels and for the different directions. So when the UE transmits this query MAC CE, the prohibit timer should only apply to that logical channel and that direction (uplink/downlink), while the UE should be allowed to send another MAC CE for another logical channel or another direction.

The prohibit timer for the query MAC CE only prohibits the UE from retransmitting the exact same MAC CE to the eNB during the configured time.

The proposed updates to TS 36.321 are described in [10].

# Conclusion

Based on the discussion in section 2 we propose the following:

Observation 1	The bitrate at the physical layer for the voice media varies significantly between different configurations of the VoLTE service and the RAN and a sufficiently rich granularity of the recommended physical layer bitrate is needed.

Observation 2	The recommended bitrate at the physical layer includes all RAN headers as well as the effect of the operation of ROHC.

Observation 3	A UE bitrate query supports the use case for video rate adaptation described in [9].

Based on the discussion in section 2 we propose the following:

Proposal 1	Use the values presented in Table 2 for the RAN recommended bitrate.

Proposal 2	The prohibit timer for the query MAC CE only prohibits the UE from retransmitting the exact same MAC CE to the eNB during the configured time.

# References

- RP-161815, “New Work Item on Voice and Video enhancement for LTE”, CMCC, Huawei, China Telecom

- 3GPP TR 36.750, “Study on Voice and Video enhancement for LTE”

- Draft Report for RAN2#96, 3GPP RAN2, December 2016

- R2-169175, “Introduction of Voice and Video enhancement for LTE (Draft CR to 36.300)”, CMCC

- R2-164579, “VoLTE/video quality related enhancements”, Ericsson

- GSMA PRD IR.92, “IMS profile for Voice and SMS”

- 3GPP TS 26.114, IP Multimedia Subsystem (IMS); Multimedia telephony; Media handling and interaction

- 3GPP TS 26.445, Codec for Enhanced Voice Services (EVS); Detailed Algorithmic Description

- S4-170218, Draft CR 26.114 on RAN explicit bitrate recommendation, Ericsson, Intel

- R2-1701661, Updates for CR to 36.321 for VoLTE/ViLTE enhancements, Ericsson