3GPP TSG-RAN WG2 Meeting #97	R2-1701867

Athens, Greece, 13 - 17 February 2017

Agenda item:	10.2.2.4

Source:	Nokia, Alcatel-Lucent Shanghai Bell

Title:	Clarification of cell barred condition

WID/SID:	FS_ NR_newRAT - Release 14

Document for:	Discussion and Decision

# 1	Introduction

This paper provides a text proposal to TR 38.804 for clarifying minimum SI acquisition aspects and to clarify the conditions under which a UE considers a cell as barred. The conditions under which a cell is considered as barred depends on whether the UE has any valid stored SI or no stored SI at all. The TP is based on [1].

# 2	Text Proposal

<< skip unchanged part >>

### 5.5.3	System information handling

System information is divided into minimum SI and other SI. Minimum SI is periodically broadcast. The minimum SI comprises basic information required for initial access to a cell and information for acquiring any other SI broadcast periodically or provisioned via on-demand basis i.e. scheduling information. The other SI encompasses everything not broadcast in the minimum SI.

The other SI may either be broadcast, or provisioned in a dedicated manner, either triggered by the network or upon request from the UE as illustrated in Figure 5.5.3.1.2-1.  For the other SI required by the UE, before the UE sends the other SI request the UE needs to know whether it is available in the cell and whether it is broadcast or not. The UE in RRC_IDLE or RRC_INACTIVE should be able to request the other SI without requiring a state transition. For the UE in RRC_CONNECTED, dedicated RRC signaling can be used for the request and delivery of the other SI. The other SI may be broadcast at configurable periodicity and for certain duration. It is network decision whether the other SI is broadcast or delivered through dedicated UE specific RRC signaling.

Each cell on which the UE is allowed to camp broadcasts at least some contents of the minimum SI, while there may be cells in the system on which the UE cannot camp and do not broadcast the minimum SI. For a cell/frequency that is considered for camping by the UE, the UE should not be required to acquire the contents of the minimum SI of that cell/frequency from another cell/frequency layer. This does not preclude the case that the UE applies stored SI from previously visited cell(s). If the UE cannot determine the full contents of the minimum SI of a cell the UE shall consider that cell as barred. The UE makes this determination by either receiving the full contents of minimum SI from the cell or by receiving some contents of minimum SI from the cell and the rest of contents of minimum SI from valid stored SI from previously visited cells),. It is desirable for the UE to learn very quickly that this cell cannot be camped on.

NOTE 1:	Reception of the minimum SI via SFN is not precluded and pending the outcome of RAN1 study.

NOTE 2:	It is FFS whether Msg.1 and/or Msg.3 are/is used to carry the other SI request.

NOTE 3: It is FFS whether there is an additional indication that an on-demand SI is actually being broadcast at this

# References

- R2-1700660, “Text Proposal to TR 38.804 capturing the latest agreements on system information” LG Electronics Inc.