3GPP TSG-RAN WG2 #97                                                                                R2-1701928

Athens, Greece, 13th -17th Feb. 2017

Agenda item:	10.2.2.2

Source: 	ZTE, ZTE Microelectronics

Title: 	Consideration on context maintenance

Document for:	Discussion and Approval

Introduction

According to last RAN2 meeting agreement, this contribution discussed RLC context, PDCP context maintenance and whether and when UE context is fetched to a new serving gNB and how UE’s UP protocol entity is located if UE context is not fully fetched to new serving gNB. Proposals are given based on the discussion.

Discussion

RLC context maintenance

In last RAN2 Ad hoc meeting, it is agreed that UE maintains the same PDCP entity like in RRC_CONNECTED and maintains PDCP COUNT and SN of PDCP. The possibility to maintain the RLC entity and SN is FFS. We assume this agreement is suitable for both solution A and B.

1: Agree to the context description as a baseline, with additional enhancements FFS, and with the RLC information as FFS: The UE context in RRC_RRC_INACTIVE includes the configuration of radio bearers, logical channels and security. The UE maintains the same PDCP entity like in RRC_CONNECTED and maintains PDCP COUNT and SN of PDCP. The possibility to maintain the RLC entity and SN is FFS. Additional information can be considered for the context if a need is identified.

In legacy LTE, when UE is transited to RRC_RRC_INACTIVE state, UE and network side will re-establish RLC entities for all DRBs and SRBs. This means all variables including RLC SN are reset. All the timers are stop and reset. The pending RLC SDU and AMD PDUs are discarded. In other words, the RLC context is not maintained when UE is in RRC_INACTIVE state. This processing in legacy LTE is reasonable because the context is stored for fast RRC restoration, but not for data transmission in RRC_INACTIVE state.

While for UL data transmission in RRC_INACTIVE state for NR, it needs to be considered differently.

When UE is to be suspended to RRC_INACTIVE state in NR, it can be assumed that there is no more data to be transmitted, and the re-ordering window is empty. Thus there will be no difference whether to maintain the RLC SN and variables. Thus, the RLC context can be maintained or not when UE is suspended to RRC_INACTIVE. On the other side, RLC entities and its corresponding configurations should be maintained as that in RRC_INACTIVE state for later UL data transmission in RRC_INACTIVE state, to avoid additional configuration overhead when UE is in RRC_INACTIVE.

Proposal 1: When UE is transited to RRC_INACTIVE state, RLC entities and corresponding configuration for at least those DRBs which are allowed for UL transmission in RRC_INACTIVE state should be maintained. The RLC SN, timers and variables are reset to their initial values.

For transmitting UL data in RRC_INACTIVE, depending on the grant size, we cannot assume a RLC SDU can always be fit into a MAC PDU. The RLC segmentation function should be supported for UL transmission in RRC_INACTIVE state for both solution A and solution B. If segmentation is supported, the RLC context should at least be maintained for that RLC PDU in the same way as that in RRC_CONNECTED state, i.e. the RLC SN maintenance, reassembling and reordering functions is applied.

Further, it is agreed that subsequent UL transmission is allowed without move UE into RRC_CONNECTED at least for solution B and FFS for solution A. when UE is in RRC_INACTIVE state; there are probably multiple bursts of UL data transmission with multiple data packets for each burst. In this case, there are multiple choices on how to maintain the RLC context. The straight forward way is to maintain RLC context as that in RRC_CONNECTED state, i.e. the RLC SN, variables and timers are maintained continuously. Other options include resetting the RLC SN and times on certain conditions to help RLC SN alignment between UE and network, e.g. RLC reset for each PDCP PDU.  But how to make UE and network synchronize the reset action brings more complexity and difficulty.

As to whether to maintain the RLC entities and context on cell reselection, we think it depends on at least 2 factors:

- Factor 1:Whether the security is updated on cell reselection

- Factor 2: Whether the context/RLC entities are transferred to new serving gNB.

If the security is to be updated according to SA3 conclusion, one option is to re-establish RLC entity just like the processing for handover in LTE. Another option is to design a mechanism for distinguish which security key is used for a received PDCP SDU. When the security key is updated on cell reselection, the encryption keys before and after cell reselection differs. And a PDCP SDU encrypted using old key may still on air when cell selection occurs. If the RLC entity is not re-established to discard the pending RLC PDUs, the said mechanism is needed for receiver to distinguish the key used for this PDCP SDU, i.e. to aware this PDCP SDU is encrypted with old key. This kind of mechanism will be too complicated to implement and the benefit to have it is limited. Thus it is preferred to re-establish the RLC entity at any time the security is updated.

Another factor is about the complexity to transfer RLC context. To move the RLC entity and context from anchor gNB to serving gNB causes complexity. It is not simple to move the RLC variables, timers and pending RLC PDUs of a RLC entity.

The last case is when UE is transited to RRC_CONNECTED state from RRC_INACTIVE. Whether to maintain RLC entity and context in this case is also related to security issue similar to the case of cell reselection.

After all, for all cases, if allowed, it is preferred to maintain the RLC context as much as possible, to support continuous data transmission and to avoid additional UE power consumption for retransmission due to RLC re-establishment.

Proposal 2: When UE is in RRC_INACTIVE or when UE is resuming to RRC_CONNECTED state, if security key is not updated and RLC context is not relocated to a new serving gNB, the RLC context/RLC SN should be maintained. Otherwise, the RLC entity is re-established.

Following table summarizes the discussion on the RLC context maintenance under different security updating and context transferring conditions.

Table 1. Principle of RLC context maintenance

|  | RLC is relocated Yes | RLC is relocated No |
| --- | --- | --- |
| Security is updated Yes | RLC re-establishment | RLC re-establishment |
| Security is updated No | RLC re-establishment | Maintain RLC context/SN |

PDCP context maintenance

In legacy LTE, when UE is suspended into RRC_INACTIVE, network side can configure the UE to keep ROHC context which can be continued after UE is resumed to RRC_CONNECTED.

For UL data transmission in RRC_INACTIVE state in NR, the ROHC context can also be maintained as before suspension. The main benefit to maintain the ROHC context is to save IP protocol layer overhead. Because the data packets transmitted in RRC_INACTIVE state are mainly small ones, the IP layer overhead is significant without ROHC.

In legacy LTE, ROHC context is stored when UE is suspended to RRC_INACTIVE.  And ROHC is reset when PDCP entity is re-established except for the case when UE is resumed to RRC_CONECTED and drb-ContinueROHC is configured. The same principle can also be applied for UL transmission in RRC_INACTIVE. Therefore, header compression is used for UL data transmission in RRC_INACTIVE and ROHC context is maintained as in RRC_CONNECTED.

Proposal 3: The ROHC context should be maintained when UE is transited from RRC_CONNECTED to RRC_INACTIVE, and be maintained as in RRC_CONNECTED when UE is in RRC_INACTIVE.

Another issue about PDCP is the PDCP SN/COUNT maintenance on cell reselection and resuming to RRC_CONNECTED state.

In legacy LTE, when UE is resumed back to RRC_CONNECTED, the PDCP entity is re-established with PDCP SN and COUNT reset. There is no problem for legacy LTE since there is no UL or DL data transmission in RRC_INACTIVE state.

For UL data transmission in RRC_INACTIVE state in NR, the gNB may decide to move UE into RRC_CONNECTED state because of there is more data to be transmitted. In other words, the data transmission before and after UE state transition should be continuous. If the PDCP COUNT/SN is reset, the re-ordering function of PDCP will not work. For the same reason, when cell reselection occurs, it is also preferred to maintain the PDCP SN/COUNT.

Further, the PDCP SN/COUNT could be maintained regardless of whether PDCP entity is re-established or not. This is like handover in legacy LTE. When PDCP is re-established for handover in LTE, the DPCP SN and COUNT is still kept.

Proposal 4: The PDCP SN/COUNT is maintained on cell reselection and on resuming to RRC_CONNECTED state, regardless PDCP entity is re-established or not.

Context fetching and protocol entity location

In last RAN2 Ad hoc meeting, it is agreed that “the UE context is maintained in an anchor gNB” for solution A.

1: Agree to the context description as a baseline, with additional enhancements FFS, and with the RLC information as FFS: The UE context in RRC_RRC_INACTIVE includes the configuration of radio bearers, logical channels and security. The UE maintains the same PDCP entity like in RRC_CONNECTED and maintains PDCP COUNT and SN of PDCP. The possibility to maintain the RLC entity and SN is FFS. Additional information can be considered for the context if a need is identified. 6 The UE context is maintained in an anchor gNB.

When UE camps on the cell which belongs to a new serving gNB, there are basically two options to handle the context and UP protocol entities.

The first option is to fetch the UE context to new serving gNB, and relocating UE’s UP layer protocol entity including PDCP entity by a “path switch” like procedure. This processing will impact the CN-RAN connection for the UE. And fetching from one gNB to another may be too complex for RLC context. Thus RLC context have to be re-established in the new gNB. Another issue is the additional delay caused by context fetching and relocation of protocol entities. The total delay depends on the Xn interfacing transporting delay and the context fetching/protocol entity relocation procedure. After context fetching and protocol entity relocation, there is no such delay for data transmission.

The second option is to keep the context/protocol entity in the anchor gNB.  Compared to first choice, this is anticipated with no initial context fetching and protocol entity relocation delay. The UL data received by serving gNB are forwarded to anchor gNB to finish UP processing, and DL data to transmitted are forwarded from anchor gNB to serving gNB to complete the procedure.

Thus these 2 options are suit for different cases respectively.

For infrequent and small UL data transmission, the second one is more preferable since there is no additional initial delay caused by context fetching and protocol entity relocation. While for the frequent or big volume of data transmission, to relocate the protocol entity may be more beneficial because the initial delay caused by context fetching and protocol entity relocation become marginal with the increase of the number of UL/DL transmissions.

Whether to fetch context to new serving gNB and relocating the PDCP and RLC entity should be decided by network, possibly according to BSR indicated by UE, the pending DL data to be transmitted and UE history behavior.

Proposal 5: It is up to network to decide whether to transfer UE context and relocating UE UP protocol entities when UE camps to cell under a new serving gNB.

In the second option, i.e. the UE context is kept on original anchor gNB, there are several alternatives according to different UP protocol entity locations. When evaluating the alternatives, both DL and UL transmission should be considered to get a uniform solution.

For PDCP entity, it is for sure located on anchor gNB. If PDCP entity is relocated to new serving gNB, it is the option 1. Then RRC context and configuration is also located on anchor gNB along with PDCP entity.

For both UL and DL transmission, the MAC and physical layer entity should be located in the same gNB for the tight interaction and fast responding.

While for RLC entity, there are three alternatives to be considered.

Alt1: Full RLC function/context located on original anchor gNB.

Alt2: The ARQ function locates is located on original anchor gNB, while the segmentation function is located on serving gNB.

Alt3: Full RLC function and context is located on serving gNB.

The Alt1 is the simplest one. When UE transmit UL data in the cell under new serving gNB, the MAC and PHY entity is established by default configuration. The PDCP and RLC located on the original anchor gNB. Thus the UL data is forwarded from serving gNB to anchor gNB in form of MAC SDU.

The time needed to establish connection between serving gNB and anchor gNB is also minimized, e.g. a common Xn tunnel can be used for data forwarding. And since there is no context fetching, there is no delay caused by context fetching and entity relocation.

Further, because PDCP and RLC entity is kept on original anchor gNB, they need not to be re-established if security needs not updated.

One drawback of this alternative is the segmentation efficiency for DL transmission because the RLC and MAC are not on the same gNB. But this is not a big problem because during the procedure, DL transmission is mainly for RLC ARQ response which is very small. Even if there are pending application data to be transmitted to UE during the procedure, the segmentation is still not a big issue since the big data can always be segmented to small piece in advance in anchor gNB.

Figure 1.Alternative 1 for UP protocol location.

In alternative 2, RLC functions are partitioned into ARQ and segmentation and located on anchor gNB and serving gNB respectively. It seems the segmentation efficiency and initial delay overhead is balanced in this alternative. But it also bring the complexity of interfacing between the two part of RLC functions. In alternative 3, RLC function is fully moved to serving gNB. The segmentation efficiency is high with the cost of relocating RLC entity.

In summary, as proposed above, to keep PDCP entity on anchor gNB is more suitable for small and infrequent UL data transmission, the simplest alternative is more preferable.

Proposal 6: UE’s context, i.e. PDCP, RLC and RRC context and PDCP/RLC entity are located on anchor gNB, while MAC and PHY entity are located on serving gNB, until UE context is fetched to a new serving gNB.

Conclusion

Proposal 1: When UE is transited to RRC_INACTIVE state, RLC entities and corresponding configuration for at least those DRBs which are allowed for UL transmission in RRC_INACTIVE state should be maintained. The RLC SN, timers and variables are reset to their initial values.

Proposal 2: When UE is in RRC_INACTIVE or when UE is resuming to RRC_CONNECTED state, if security key is not updated and RLC context is not relocated to a new serving gNB, the RLC context/RLC SN should be maintained. Otherwise, the RLC entity is re-established.

Proposal 3: The ROHC context should be maintained when UE is transited from RRC_CONNECTED to RRC_INACTIVE, and be maintained as in RRC_CONNECTED when UE is in RRC_INACTIVE.

Proposal 3: The ROHC context should be maintained when UE is transited from RRC_CONNECTED to RRC_INACTIVE, and be maintained as in RRC_CONNECTED when UE is in RRC_INACTIVE.

Proposal 4: The PDCP SN/COUNT is maintained on cell reselection and on resuming to RRC_CONNECTED state, regardless PDCP entity is re-established or not.

Proposal 5: It is up to network to decide whether to transfer UE context and relocating UE UP protocol entities when UE camps to cell under a new serving gNB.

Proposal 6: UE’s context, i.e. PDCP, RLC and RRC context and PDCP/RLC entity are located on anchor gNB, while MAC and PHY entity are located on serving gNB, until UE context is fetched to a new serving gNB.

Reference

[1] RAN2-NR-AH-Spokane-Chairman-Notes

[2] 3GPP TS 36.331, Radio Resource Control (RRC), V14.1.0

[3] 3GPP TS36.323, Packet Data Convergence Protocol (PDCP) specification, V14.1.0

[4] 3GPP TS36.322, Radio Link Control (RLC) protocol specification, V13.2.0