3GPP TSG-RAN WG2 #97bis	R2-1702750

Spokane, USA, 3rd – 7th April 2017

Agenda Item:	10.3.3.2

Source:	Ericsson

Title:	Duplication in UL in Dual connectivity

Document for:	Discussion, Decision

# Introduction

This contribution discusses several aspects of UL data (DRB) duplication for Dual Connectivity such as configuration, activation/deactivation, and data duplication operation.

# Discussion

Data duplication may be more useful for ultra-reliable devices and services. However, UL user plane data duplication might also be an interesting option to cope with situations in which radio links are not stable e.g. coverage limited situations, while trying to sustain the certain UL bit rate. It could also be an option to help to achieve the 0 ms interruption time requirement.

UL data duplication is per DRBs

## Configuration and activation

UL data duplication configuration should be made via RRC. This configuration may consist, for example, of which RBs may transmit duplicated packets.

UL (DRB) data duplication is configured per DRB via RRC message

Another aspect is when UL data duplication should be enabled or disabled. There are different options to achieve this:

-RRC message: When the feature is configured, duplication is also started. When the duplication is to be stopped, the NW sends another RRC message to “remove” the configuration.

- Event trigger: This mechanism could be similar to a measurement event. Upon a certain event (configured by the network), the UE would activate/deactivate the PDCP data duplication.

- PDCP: PDCP layer activate and deactivate duplication by means of a PDCP control command. In this case, the NW could send a PDCP control command to the UE when it wants to activate or deactivate duplication.

- MAC: MAC CE could be used to activate or deactivate PDPC duplication; however, using MAC to control PDCP functions is not beneficial.

Activation/deactivation of the duplication of UL data may be dependent on aspects such as, for example, link quality or network resources. In any case, duplication of UL PDCP PDUs may take a considerable amount of resources in the network and, therefore, we think that activation/deactivation should be in control of the network. From this angle, we think that the best alternatives to control the activation and deactivation of this feature are RRC or PDCP.

Using a PDCP command to enable and disable the UL data duplication is straightforward. It gives flexibility to the network to activate and deactivate the feature in a more dynamic manner than what RRC would do. It also minimizes the exchange of signalling messages, which is desirable in the situations (e.g. limited coverage) in which data duplication may be needed and possible inter-layer interaction.

UL (DRB) data duplication is activated/deactivated by the network

PDCP Control Command activate/deactivate UL (DRB) data duplication

After PDCP duplication has been activated, the UE should deliver the same PDCP PDUs to both RLC entities. As in LTE, PDCP delivers data to RLC only upon request from lower layers, i.e. when an uplink grant becomes available.

## Failure in one leg

There may be situations in which, after duplication is activated, data in one of the legs does not go through due to e.g. bad radio conditions. The other leg, however, may perform adequately. That results in that data is received in the NW through one of the legs.

In the leg which had bad radio, the RLC might be performing retransmissions which might be no longer needed (as it has been received through the other leg). If the leg recovers, the RLC may still transmit the data pending in the RLC/MAC (i.e. the retransmissions). All this data, however, will be discarded by the NW. Thus, it is preferable if the UE does not transmit it. 
In the worst cases, if a leg does not recover and the maximum number of RLC retransmissions is be reached, this would possibly trigger an RLF failure, which is not needed in this case.

This could be solved by that the UE keep data in PDCP, and wait until it is “pulled down” to an RLC when a new grant is received. This way, duplicate data would not pile up in a RLC for a bad link. When the UE receives the RLC-ACK via the working leg, PDUS from the PDCP transmit window is removed and then, later when the link is back up, these PDUs would not have been considered for duplicate transmission.

Discuss whether a mechanism is needed to prevent wasting radio resources and unnecessary RLF triggering due failures in one link

# Conclusion

Here there are two lists, one listing observations, and one listing proposals. Listing observations are not always needed, but listing proposals are always useful. To update a list, place the cursor inside the list and press F9.

Based on the discussion in section 2 we propose the following:

Proposal 1	UL data duplication is per DRBs

Proposal 2	UL (DRB) data duplication is configured per DRB via RRC message

Proposal 3	UL (DRB) data duplication is activated/deactivated by the network

Proposal 4	PDCP Control Command activate/deactivate UL (DRB) data duplication

Proposal 5	Discuss whether a mechanism is needed to prevent wasting radio resources and unnecessary RLF triggering due failures in one link