3GPP TSG-RAN WG2 Meeting #97bis	R2-1702758

Spokane, USA, 3rd – 7th April 2016

Source:	Spreadtrum Communications

Title:	On demand SI request

Agenda Item:	10.4.1.4

Document for:	Discussion and Decision

# Introduction

After RAN2 #97 there is still no agreement whether Other System Information via on-demand basis (Other SI) should be requested through MSG1 or MSG3. Both approaches have their advantages and disadvantages. An LS was sent to RAN1 [1] to ask details about the NR PRACH preamble and a LS response was received in [2]. In this contribution we continue the discussion.

# Discussion

Two competing schemes are being discussed when it comes to requesting Other System Information via on-demand (Other SI), namely a) requesting Other SI through MSG1 or b) through MSG3. The two options are shown in Figure 1 and Figure 2 respectively:

Figure 1 MSG1-based Other SI Request

Figure 2 MSG3-based Other SI Request

Next, we give our comparison on the two solutions with respect to various aspects:

- Preamble collision probability

The problem of preamble collision can occur in both MSG1-based approach as well as in the legacy RACH procedure, i.e., the MSG3-based approach. The key to minimizing the preamble collision probability is the space of preambles and their design. According to the LS response to RAN2 [2] “The design of the PRACH preambles formats and resource allocations is ongoing in RAN1 and the number of PRACH preambles/resources is still open. In general, RAN1 aims at a flexible design that can handle a range in random access load.” Furthermore, the LS response [2] stated that “If multiple UEs transmit the same PRACH preamble resulting in a collision, the gNB might be able to detect the preamble from the combined transmission from multiple UEs, but may not be able to distinguish the UEs.  The probability of successful detection depends on the channel conditions.” Since in MSG1-based approach a preamble is mapped to an requested SI even if the gNB is not able to distinguish the UEs it can infer the requested SI.

Observation 1: Requesting Other System Information via on-demand through MSG1 may have better detection capability upon collision depending on preamble design.

The Other SI can be obtained from the gNB in Broadcast or Unicast fashion. We distinguish two scenarios of collision when the same Other System Information is requested on-demand by multiple UEs:

- Collision of Other System Information via On-Demand (Broadcast case): If the preambles of two or more UEs collide in an MSG1-based approach when the network is supposed to send Other SI in the broadcast the gNB might be able to infer the requested Other SI based on the detected preamble. Even if the gNB cannot detect which UE sent the request it can indicate scheduling parameters to the UEs regarding the Other SI and start the broadcast according to the scheduling information. This is not possible in MSG3-based approach where there is no SI indicated in the preamble.

Observation 2: In the event of collision in On Demand Broadcast, the network can recover from the collision if the requested System Information is detected even if the network could not decode which UE sent the request.

- Collision of Other System Information via On-Demand (Unicast case): If the preambles of two or more UEs collide in MSG1-based approach when the network is supposed to send Other SI in a unicast the latter might not be able to decode which UE requests the SI, according to LS response [2], Therefore, it cannot deliver the Other SI in a dedicated fashion to the UE. However, in such a scenario the network can decide to provide the required scheduling parameters and deliver the requested System Information in the Broadcast.

Observation 3: In the event of collision in On Demand Unicast, if the Other SI requested is detected but the UE that sent the request is not detected, then the network can decide to start sending the requested System Information through Broadcast.

- Number of preambles

In MSG1 based approach a number of preambles needs to be reserved to indicate different SI. This is not the case in MSG3 approach where a common preamble space is used. Even though preamble reservation complicates the design, RAN1 is currently working on designing the PRACH preamble formats and resource allocations [2].

Observation 4: Preamble design and reservation is currently ongoing in RAN1.

- Number of messages exchanged

MSG3 based approach involves more message exchanges which actually increases the probability that one of those messages is erroneously received. Furthermore, it consumes more resources including the battery of the UEs.

Observation 5: MSG1-based approach involves a smaller number of transmissions which is more resource efficient and decreases the power consumption of UEs.

- RACH load

In MSG1-based approach the RACH load does not increase with increasing number of SI requests unlike the MSG3-based approach.

Observation 6: MSG1-based approach has a better behaviour with respect to RACH load with increasing number of SI requests.

Hence based on the above observations we propose the following:

Proposal: RAN2 should adopt a MSG1-based approach for requesting Other System Information via on-demand.

# Conclusion

Based on the above we observe and propose the following:

Observation 1: Requesting Other System Information via on-demand through MSG1 may have better detection capability upon collision depending on preamble design.

Observation 2: In the event of collision in On Demand Broadcast, the network can recover from the collision if the requested System Information is detected even if the network could not decode which UE sent the request.

Observation 3: In the event of collision in On Demand Unicast, if the Other SI requested is detected but the UE that sent the request is not detected, then the network can decide to start sending the requested System Information through Broadcast.

Observation 4: Preamble design and reservation is currently ongoing in RAN1.

Observation 5: MSG1-based approach involves a smaller number of transmissions which is more resource efficient and decreases the power consumption of UEs.

Observation 6: MSG1-based approach has a better behaviour with respect to RACH load with increasing number of SI requests.

Proposal: RAN2 should adopt a MSG1-based approach for requesting Other System Information via on-demand.

# References

- R2-1700653 LS on PRACH preambles for on demand SI requests

- R2-1700701 Reply LS on PRACH preambles for on demand SI requests