3GPP TSG-RAN WG2 Meeting #97bis	Tdoc R2-1702853

# Spokane, USA, April 3-7, 2017

Agenda Item:	10.2.4.4

Source:	Ericsson

Title:	System information structure and contents

Document for:	Discussion, Decision

# Introduction

RAN2 has agreed (and captured in 38.804) that “The minimum SI comprises basic information required for initial access to a cell and information for acquiring any other SI broadcast periodically or provisioned via on-demand basis, i.e. scheduling information. The other SI encompasses everything not broadcast in the minimum SI.” and captured in the Annex A.4 of 38.804 additional details of system information

With regards to system information provisioning on stage-3 level:

-	The minimum SI includes at least SFN, list of PLMN, Cell ID, cell camping parameters, RACH parameters.

- A unique global cell ID is broadcast for an NR cell.

-	If network allows on demand mechanism, parameters required for requesting other SI-block(s) (if any needed, e.g. RACH preambles for request) shall be included in minimum SI.

-	Cell-reselection neighbouring cell information is considered as other SI.

-	PWS information can be classified into the other SI.

-	The scheduling information for the other SI includes SIB type, validity information, SI periodicity and SI-window information and is provided irrespective of whether the other SI is periodically broadcast or not.

-	For other SI, UE can request one or more SI-block(s) or all SI-blocks in a single request.

-	For the other SI required by the UE, before the UE sends the other SI request the UE needs to know whether it is available in the cell and whether it is broadcast or not. This can be done by checking the minimum SI which provides the scheduling information for the other SI including SIB type, validity information, SI periodicity and SI-window information based on LTE.

-	The scheduling information in minimum SI includes an indicator whether the concerned SI-block is periodically broadcasted or provided on demand. If minimum SI indicates that a SIB is not broadcasted, then UE does not assume that this SIB is a periodically broadcasted in its SI-window at every SI periodicity. Therefore the UE may send an SI request to receive this SIB.

-	After sending the SI request, for receiving the requested SIB, UE monitors the SI window of the requested SIB in one or more SI periodicities of that SIB.

-	Broadcasting some kind of index/identifier in minimum SI to enable the UE to avoid re-acquisition of already stored SI-block(s)/SI message(s). The index/identifier and associated system information can be applicable in more than one cell. System information valid in one cell may be valid also in other cells.

-	It is FFS what the index/identifier is, e.g. single index or area plus value tag, etc.

In this contribution, we propose a structure for the system information and propose a list of the contents of the minimum system information.

In section 2, we first propose to adopt the LTE system information framework and terminology also for NR, and then review the RAN2 agreements and contents of the LTE system information, and a list of information elements for minimum system information (i.e. information needed for initial access and SI acquisition for the rest of the system information).

# Discussion

## Structure of the system information

It can be noted that the naming of the LTE messages is somewhat confusing, as MIB, SIB1 and SI are defined as messages. SIB1 is thus defined as a message but the rest of the SIBs are defined as information elements. Since information elements and messages are conceptually different things, it would be desirable to avoid mixing up their naming in this manner.

One possible solution would be to call both SIB1 and SI as System Information messages and use numbering such as SI1 message (for SIB1) and SI2 message (for the other SIBs). MIB should then also have “message” in its name because it is a message and not an information element, perhaps named Master Information Message (MIM) instead of MIB. In that way, the System Information Block (SIB) could be solely used for information elements that are contained in the SI2 message.

However, this level of detail can probably be discussed during the specification drafting, and for easy understanding, we assume the LTE naming convention in the rest of the paper.

The implicit assumption in the RAN2 discussion on system information has been that the LTE structure with SI blocks and messages is on some level reflected in NR (most obvious from the agreements in Annex A.4 of TR38.804, which explicitly uses terms “SIB” and “SI-block”). We propose to adopt the LTE concept directly to NR

- System information is divided to master information block and system information blocks. Note that in order to avoid confusion, we will call the NR master information block NR-MIB and NR system information blocks NR-SIBs. This is since the contents of the NR MIB and SIBs may differ from the contents of the LTE MIB and SIBs.

- NR-MIB and NR-SIB1 are separate RRC messages.

- All other NR-SIBs are transmitted in SI messages scheduled in NR-SIB1.

System information in NR consists of MasterInformationBlock (NR-MIB) and System Information Blocks (NR-SIBs).

NR-MIB and NR-SIB1 are separate RRC messages, while all other NR-SIBs are transmitted in SI messages scheduled in NR-SIB1.

In addition to master and system information blocks, the NR system information is divided to “minimum” and “other” system information with the understanding that “The minimum SI comprises basic information required for initial access to a cell and information for acquiring any other SI broadcast periodically or provisioned via on-demand basis, i.e. scheduling information. The other SI encompasses everything not broadcast in the minimum SI”.

Note that the current agreements in RAN2 imply that minimum system information will contain (parts of) LTE SIB1 and (parts of) LTE SIB2. The main benefit of this split is that it allows different scheduling of LTE SIB1 (containing primarily cell camping parameters) and LTE SIB2 (containing cell access parameters). In theory, the NR minimum system information could also be split between NR-SIB1 and NR-SIB2, but since the full minimum system information is needed to access the cell, it would seem strange to provide parts of it only on-demand. In our opinion, it is preferable to provide the full minimum system information in NR-SIB1 (i.e. move information needed for initial access from LTE SIB2 to NR-SIB1). Using a staggered approach with UE having to first read the NR-MIB and NR-SIB1 to obtain the scheduling information of NR-SIB2, and then waiting for the NR-SIB2 to be scheduled may create an additional delay for the connection set-up.

Minimum system information consists of NR-MIB and NR-SIB1 (which contains information from LTE SIB1 and SIB2), while all other NR-SIBs are part of other system information.

## Contents of the minimum system information

In this section, we first review the already agreed minimum system information contents (based on 38.804) and then evaluate which LTE SIB1 and SIB2 contents are needed for initial access to a cell and for acquiring any other SI.

Based on current RAN2 agreements and proposal 3, the following information will need to be distributed between NR-MIB and NR-SIB1

- SFN, List of PLMNs, Cell ID, Cell camping parameters, RACH parameters.

- Parameters for requesting other SI-block(s).

- Scheduling information for other SI.

  - SIB type

  - Validity information

  - SI periodicity

  - SI-window information

- Indication whether SI block is periodically broadcasted or provided on demand.

- Index/identifier in minimum SI to enable the UE to avoid re-acquisition of already stored SI-block(s)/SI message(s).

In LTE, the MasterInformationBlock contains only information needed to acquire LTE SIB1 (especially DL bandwidth and PHICH duration are needed to locate the PDCCH) and the 8 most significant bits of SFN

MasterInformationBlock ::=			SEQUENCE {

dl-Bandwidth						ENUMERATED {

n6, n15, n25, n50, n75, n100},

phich-Config						PHICH-Config,

systemFrameNumber					BIT STRING (SIZE (8)),

schedulingInfoSIB1-BR-r13			INTEGER (0..31),

spare								BIT STRING (SIZE (5))

}

Adopting a similar approach for NR, we propose that NR-MIB contains the SFN and information needed to acquire NR-SIB1. However, it’s not clear that the NR DL bandwidth information is needed or sufficient to identify the NR-PDCCH/PDSCH location and it is not obvious that the physical channel providing the NR minimum SI (NR-SIB1) shall be derived from the PCI of the cell. We therefore propose to replace it with more general physical channel configuration for NR-SIB1

NR-MasterInformationBlock ::=			SEQUENCE {

systemFrameNumber					BIT STRING (SIZE (FFS)),

physicalChannelConfigSIB1			PhysicalChannelConfigSIB1,

spare								BIT STRING (SIZE (FFS))

}

NR-MIB contains at least (part of) the system frame number and the physical channel configuration needed to receive NR-SIB1

Note that in addition to the contents in Proposal 4, the MIB may also contain additional information elements, such as a system information value tag. The size of the spare bits should be selected based on the final contents of the MIB as well as the physical channel transmission rate.

Given the earlier RAN2 agreements, this means that NR-SIB1 should contain at least the information elements listed in Table 1.

Table 1: List of RAN2 agreed minimum system information contents (excluding contents of MIB). Note that this is not the full list of NR-SIB1 contents (see Table 3)

| Information element | LTE location | Proposed NR location |
| --- | --- | --- |
| List of PLMNs | SIB1 | NR-SIB1 |
| Cell ID | SIB1 | NR-SIB1 |
| Camping parameters | SIB1 (cell barred, intra-freq reselection allowed, QRxLevMin & offset) | NR-SIB1 |
| Parameters for requesting other SI | N/A | NR-SIB1 |
| RACH configuration | SIB2 (as part of Radio Resource Config Common SIB). Note that the RACH configuration is not sufficient to access the cell, and is only singled out here due to specific RAN2 agreement. | NR-SIB1 |
| Scheduling information for other SI o SIB type o Validity information o SI periodicity o SI-window information | SIB1 SIB1 (valueTag/list of valueTags for eMTC) SIB1 SIB1 | NR-SIB1 NR-SIB1 (list of valueTags) NR-SIB1 NR-SIB1 |
| Periodically broadcasted indication | N/A | NR-SIB1 |

Note that the Index/identifier to avoid re-acquisition is not included in the table since it is addressed by the system information value tag, which is proposed in [2] to be included in the NR-MIB.

We have also listed the LTE SIB1 and SIB2 contents in Table 2, and proposed a suitable NR SIB for them. Basically we propose that the NR-SIB1 contains information from LTE SIB1 and SIB2 needed for initial access. See text below the table for justification of the placement.

Base-line NR-SIB1 contains information from LTE SIB1 and SIB2 needed for initial access.

Table 2: LTE SIB1 and SIB2 contents.

| SIB1 | Comment | NR location |
| --- | --- | --- |
| plmn-IdentityList | Including a per PLMN indication of a cell reserved for operator use. | NR-SIB1 |
| trackingAreaCode |  | NR-SIB1 |
| cellIdentity |  | NR-SIB1 (already agreed to be part of minimum SI) |
| cellBarred |  | NR-SIB1 (already agreed to be part of minimum SI) |
| intraFreqReselection | Indicating whether intra-frequency reselection is allowed in barred cells. | NR-SIB1 (already agreed to be part of minimum SI) |
| CSG related information csg-Indication csg-Identity |  | NR-SIB2+ |
| Cell Selection Information q-RxLevMin q-RxLevMinOffset |  | NR-SIB1 (already agreed to be part of minimum SI) |
| p-Max |  | NR-SIB1 (to be verified by RAN1) |
| freqBandIndicator |  | NR-SIB1 |
| schedulingInfoList | List of SI periodicity and mapping of SIBs to SI messages | NR-SIB1 |
| tdd-Config |  | Not needed due to dynamic TDD? |
| WindowLength |  | NR-SIB1 |
| systemInfoValueTag |  | NR-MIB |
| SIB2 | Comment | NR location |
| Access barring information |  | NR-SIB1 |
| Radio Resource Config Common |  | NR-SIB1 (at least for parameters needed for initial access). |
| UE Timers and Constants |  | NR-SIB2+ |
| Frequency information | UL carrier frequency, ARFCN, bandwidth and additional spectrum mask | NR-SIB1 |
| MBSFN configuration |  | NR-SIB2+ (if needed) |
| timeAlignmentTimerCommon |  | NR-SIB2+ |

Some of the information elements agreed to be included in NR minimum system information have direct LTE counterparts. This is especially true for the cell ID and list of PLMNs (though the new use cases envisioned for NR might require further extensions to enhance the support for network sharing, see [1]). The inclusion of such parameters in NR-SIB1 is straightforward.

In addition to the PLMN ID list and cell ID, it seems natural to assume that tracking area code is also needed, but since this depends on the 5G core network design, the need for tracking area code should be verified with SA2. However, we have assumed for the sake of progress that tracking area code is needed.

The cell camping parameters are assumed to refer to cell barred, intrafrequency reselection allowed, QRxLevMin and QRxLevMinOffset IEs in LTE SIB1. The actual information for NR might depend on technical solutions chosen for idle mode mobility (due to e.g. beam based mobility), but for baseline operation it is assumed that the LTE information elements are a good starting point.

The LTE SIB1 also includes CSG indicator, and SIB2 MBSFN subframe configuration. These are examples of “services” supported by the cell and there is a RAN2 agreement to broadcast these “services” in system information. On one hand, the UE should probably avoid camping on a cell not supporting the service UE is interested in. This would speak in the favour of including supported “services” in NR-SIB1. On the other hand, this information is not really required to access the system, and it might be useful for the UE to be able to request and receive on-demand system information from all cells, including e.g. CSG cells UE is not a member of. For the time being, we have assumed that NR-SIB1 does not contain information about supported service, but this assumption might require further analysis in subsequent meetings.

Note that MBSFN subframe configuration is also used to provide forward compatibility in LTE design. We assume that such mechanisms are built into the basic NR frame structure, and do not need special configuration. However, if RAN1 indicates that subframe configuration is needed, this information should probably be provided in NR-SIB1.

Assume that the base-line NR-SIB1 does not contain information about services supported by a cell.

The LTE SIB1 also contains an indication of the frequency band, including multi-band information, while LTE SIB2 contains information on UL frequency bands. For NR, the UE will also need the UL frequency band information in order to be able to transmit the request for on-demand system information (at least for msg3 based schemes).

The need for TDD configuration is not clear at this point of time, but the current assumption in RAN1 has been a rather flexible control channel design. This includes support for dynamic TDD, in which case the UE may be able to determine the UL and DL allocations using physical control channels. For the time being, we have assumed that TDD configuration does not need to be signalled to the UE, but this obviously needs to be verified with RAN1.

Assume that the base-line NR-SIB1 does not contain information on TDD UL/DL configuration (subject to RAN1 verification).

The network should have mechanisms to control the access load by barring selected UEs. This barring should be done as part of initial access, which implies that the access barring parameters should be part of the NR-SIB1.

The minimum system information needs to provide all required physical channel and higher layer information needed for initial access. The explicitly mentioned RAN2 agreements on RACH configuration does not seem to be sufficient, and it seems natural to provide a full configuration (similar to radio configuration common in LTE SIB2) in minimum system information even though the exact physical parameters are not known at this point of time. For simplicity, we have named this information “initial access configuration”.

The need for UE timers and constants seems less clear, and one could envision using default values for the timers and constants until network has a chance to reconfigure those with dedicated signalling. For this reason, we propose that the UE timers and constants are not part of the NR-SIB1. The only exception to this may be the TA timer, which should be configurable based on the cell size.

Assume that the base-line NR-SIB1 does not contain UE timers and constants.

Table 3: Baseline NR-SIB1 contents. Note that additional information elements are likely to be included based on further RAN2 decisions.

| Information element | Comment |
| --- | --- |
| List of PLMNs | Including indication of a cell reserved for operator use. May need to be extended to support new use cases, see [1] |
| Tracking Area Code | Need to be verified with SA2. |
| Cell Identity |  |
| Cell camping parameters | See table 4 |
| p-Max | Need to be verified with RAN1 |
| Frequency Band Information | Including multi-band information Including UL band information from LTE SIB2. |
| Access Barring parameters | May aim for a unified approach instead of reusing LTE IEs. |
| Initial access configuration | Including RACH configuration, but also other physical channel and higher layer configurations needed at least to be able to receive messages 2 and 4 and to transmit msg3. |
| Scheduling Info List | See table 5 for details |
| Additional parameters for requesting other SI | E.g. special RA preamble values, dedicated RACH opportunities, … |

Table 4: Cell camping parameters

| Cell camping parameters | Comment |
| --- | --- |
| Cell Barred |  |
| Intra Frequency Reselection allowed |  |
| Q_RxLevMin |  |
| Q_RxLevMinOffset |  |

Table 5: SI scheduling information parameters

| Scheduling Info List | Comment |
| --- | --- |
| SI Periodicity |  |
| SIB Mapping info | Sequence of SIBs |
| Value Tag / Index / Identifier | Value tag / Index / Identifier per SI message |
| SI Window Length | SI Window per SI message |
| On-demand indication | Indication whether the SI is broadcast or available on-demand |

The contents of the NR-SIB are summarized in Table 3, and we propose to adopt this as the baseline NR-SIB1 contents. Note that additional information elements are likely to be included based on further RAN2 decisions.

Adopt the list of information elements in Table 3 as the baseline NR-SIB1 contents.

# Conclusion

Based on the discussion in section 2 we propose the following:

Proposal 1	System information in NR consists of MasterInformationBlock (NR-MIB) and System Information Blocks (NR-SIBs).

Proposal 2	NR-MIB and NR-SIB1 are separate RRC messages, while all other NR-SIBs are transmitted in SI messages scheduled in NR-SIB1.

Proposal 3	Minimum system information consists of NR-MIB and NR-SIB1 (which contains information from LTE SIB1 and SIB2), while all other NR-SIBs are part of other system information.

Proposal 4	NR-MIB contains at least (part of) the system frame number and the physical channel configuration needed to receive NR-SIB1

Proposal 5	Base-line NR-SIB1 contains information from LTE SIB1 and SIB2 needed for initial access.

Proposal 6	Assume that the base-line NR-SIB1 does not contain information about services supported by a cell.

Proposal 7	Assume that the base-line NR-SIB1 does not contain information on TDD UL/DL configuration (subject to RAN1 verification).

Proposal 8	Assume that the base-line NR-SIB1 does not contain UE timers and constants.

Proposal 9	Adopt the list of information elements in Table 3 as the baseline NR-SIB1 contents.

# References

R2-1702860, “NR minimum SI at network sharing”, Ericsson, 3GPP RAN2#97bis, Spokane, USA, 3rd – 7th April 2017

R2-1702858, “Stored system information”, Ericsson, 3GPP RAN2#97bis, Spokane, USA, 3rd – 7th April 2017

# Appendix: LTE SIB1 and SIB2 contents.

SystemInformationBlockType1 ::=		SEQUENCE {

cellAccessRelatedInfo				SEQUENCE {

plmn-IdentityList					PLMN-IdentityList,

trackingAreaCode					TrackingAreaCode,

cellIdentity						CellIdentity,

cellBarred							ENUMERATED {barred, notBarred},

intraFreqReselection				ENUMERATED {allowed, notAllowed},

csg-Indication						BOOLEAN,

csg-Identity						CSG-Identity			OPTIONAL	-- Need OR

},

cellSelectionInfo					SEQUENCE {

q-RxLevMin							Q-RxLevMin,

q-RxLevMinOffset					INTEGER (1..8)			OPTIONAL	-- Need OP

},

p-Max								P-Max						OPTIONAL,			-- Need OP

freqBandIndicator					FreqBandIndicator,

schedulingInfoList					SchedulingInfoList,

tdd-Config							TDD-Config					OPTIONAL,	-- Cond TDD

si-WindowLength						ENUMERATED {

ms1, ms2, ms5, ms10, ms15, ms20,

ms40},

systemInfoValueTag					INTEGER (0..31),

nonCriticalExtension				SystemInformationBlockType1-v890-IEs	OPTIONAL

}

SchedulingInfoList ::= SEQUENCE (SIZE (1..maxSI-Message)) OF SchedulingInfo

SchedulingInfo ::=	SEQUENCE {

si-Periodicity						ENUMERATED {

rf8, rf16, rf32, rf64, rf128, rf256, rf512},

sib-MappingInfo						SIB-MappingInfo

}

SIB-MappingInfo ::= SEQUENCE (SIZE (0..maxSIB-1)) OF SIB-Type

SIB-Type ::=						ENUMERATED {

sibType3, sibType4, sibType5, sibType6,

sibType7, sibType8, sibType9, sibType10,

sibType11, sibType12-v920, sibType13-v920,

sibType14-v1130, sibType15-v1130,

sibType16-v1130, sibType17-v1250, sibType18-v1250,

..., sibType19-v1250, sibType20-v1310}

SystemInformationBlockType2 ::=		SEQUENCE {

ac-BarringInfo						SEQUENCE {

ac-BarringForEmergency				BOOLEAN,

ac-BarringForMO-Signalling			AC-BarringConfig				OPTIONAL,	-- Need OP

ac-BarringForMO-Data				AC-BarringConfig				OPTIONAL	-- Need OP

}																		OPTIONAL,	-- Need OP

radioResourceConfigCommon			RadioResourceConfigCommonSIB,

ue-TimersAndConstants				UE-TimersAndConstants,

freqInfo							SEQUENCE {

ul-CarrierFreq						ARFCN-ValueEUTRA				OPTIONAL,	-- Need OP

ul-Bandwidth						ENUMERATED {n6, n15, n25, n50, n75, n100}

OPTIONAL,	-- Need OP

additionalSpectrumEmission			AdditionalSpectrumEmission

},

mbsfn-SubframeConfigList			MBSFN-SubframeConfigList			OPTIONAL, 	-- Need OR

timeAlignmentTimerCommon			TimeAlignmentTimer,

...,

lateNonCriticalExtension		OCTET STRING (CONTAINING SystemInformationBlockType2-v8h0-IEs)						OPTIONAL,

[[	ssac-BarringForMMTEL-Voice-r9		AC-BarringConfig				OPTIONAL,	-- Need OP

ssac-BarringForMMTEL-Video-r9		AC-BarringConfig				OPTIONAL	-- Need OP

]],

[[	ac-BarringForCSFB-r10				AC-BarringConfig			OPTIONAL	-- Need OP

]],

[[	ac-BarringSkipForMMTELVoice-r12		ENUMERATED {true}			OPTIONAL,	 -- Need OP

ac-BarringSkipForMMTELVideo-r12		ENUMERATED {true}			OPTIONAL,	 -- Need OP

ac-BarringSkipForSMS-r12			ENUMERATED {true}			OPTIONAL,	 -- Need OP

ac-BarringPerPLMN-List-r12			AC-BarringPerPLMN-List-r12	OPTIONAL	-- Need OP

]],

[[	voiceServiceCauseIndication-r12		ENUMERATED {true}			OPTIONAL	-- Need OP

]],

[[	acdc-BarringForCommon-r13			ACDC-BarringForCommon-r13		OPTIONAL,	-- Need OP

acdc-BarringPerPLMN-List-r13		ACDC-BarringPerPLMN-List-r13	OPTIONAL	-- Need OP

]],

[[

udt-RestrictingForCommon-r13			UDT-Restricting-r13				OPTIONAL,	-- Need OR

udt-RestrictingPerPLMN-List-r13		UDT-RestrictingPerPLMN-List-r13	OPTIONAL,	-- Need OR

cIoT-EPS-OptimisationInfo-r13		CIOT-EPS-OptimisationInfo-r13 	OPTIONAL,	-- Need OP

useFullResumeID-r13					ENUMERATED {true}				OPTIONAL	-- Need OP

]]

}

SystemInformationBlockType2-v8h0-IEs ::=	SEQUENCE {

multiBandInfoList				SEQUENCE (SIZE (1..maxMultiBands)) OF AdditionalSpectrumEmission	OPTIONAL,	-- Need OR

nonCriticalExtension			SystemInformationBlockType2-v9e0-IEs	OPTIONAL

}

# Appendix: RAN1 system information agreements

RAN1 has agreed

- For the minimum system information delivery,

  - Part of minimum system information is transmitted in NR-PBCH

  - The remaining minimum system information is transmitted in shared downlink channel via NR-PDSCH

  - FFS  how the configuration information for the remaining minimum system information is provided, e.g.:

    - NR-PBCH provides the control channel search space

    - NR-PBCH provides the scheduling assignment

    - Part of the control channel search space/scheduling assignment could be derived by the specification

  - FFS numerology for NR-PDSCH for the remaining minimum system information