3GPP TSG-RAN WG2 Meeting #97bis														     R2-1703066

Spokane, USA, 3rd – 7th April 2017

Title: 	Remote UE SRB forwarding at Relay UE in feD2D

Source: 	ZTE

Agenda item:	9.1.2.2

Document for:	Discussion and Approval

Introduction

In feD2D SI, one essential issue is how to forward the RRC signalling of remote UE SRB to and from eNB at relay UE. This paper analyzes possible approaches to support remote UE RRC signalling forwarding.

Discussion on Remote UE SRB relaying at Relay UE

How to transfer remote UE SRBs at relay UE is illustrated in three ways as following.

Option 1: The remote UE SRBs are carried in relay UE SRB.

In this option, the Remote UE’s RRC signalling is delivered through relay UE SRB. And different SRBs are handled differently. It’s straightforward to map remote UE’s SRB1 and SRB2 onto relay UE’s SRB1 and SRB2 respectively. But for SRB0 of Remote UE, it’s suggested to carry it in relay UE’s SRB1 since relay UE should be already in RRC connected state when remote UE transmits RRC signalling on SRB0.

Fig. 1. Using relay UE SRB to carry remote UE SRB

Option 2: The remote UE SRBs are carried in relay UE Uu DRB.

Another option is to aggregate the SRBs of remote UE into relay UE’s Uu DRB. In this option, the remote UE’s SRB are handled with the same priority as the relay UE Uu DRB, which might impact the performance of RRC signalling for remote UE. It requires careful considerations to determine which Uu DRB of relay UE is selected to transmit the RRC signalling from remote UE SRB.

Fig. 2. Using relay UE Uu DRB to carry remote UE SRB

Option 3: The remote UE SRBs are carried in a Tunnel SRB.

Similar to the Tunnel DRB concept in [1], Tunnel SRB is proposed to carry the remote UE SRB. As shown in Fig. 3, the remote UE SRBs are carried in Tunnel SRB. The Tunnel SRB is established between relay UE and eNB and is managed by eNB. Regarding the issue whether relay UE SRB should be multiplexed into Tunnel SRB, careful analysis is required. For example, Tunnel SRB can be only setup after the relay UE is in RRC connected. That is to say, the SRB0 of relay UE is not possible to be carried by Tunnel SRB. And some special SRB1 of relay UE, e.g., the RRCConnectionSetupComplete message is also unlikely to be carried by Tunnel SRB.

Fig. 3. Using Tunnel SRB to carry remote UE SRB

To sum up, in Option 1, the SRB0 of remote UE cannot be handled differently on aspect of QoS handling, e.g. latency requirement, due to the fact that it is carried in relay UE SRB1 or SRB2. For Option 2, the QoS control of remote UE SRB is dependent to the QoS handling of the relay UE DRB. Option 3 eliminates these issues since different Tunnel SRBs could be established for remote UE SRB0/SRB1/SRB2, respectively. That is to say, the more subtle QoS control for remote UE SRBs could be achieved by Option 3. But Option 3 requires new signallings in order to setup and maintain the Tunnel SRB.

Proposal 1: It is proposed to discuss the three solutions on how to handle the remote UE SRB at relay UE in RAN2.

Option 1: The remote UE SRBs are carried in relay UE SRB.

Option 2: The remote UE SRBs are carried in relay UE DRB.

Option 3: The remote UE SRBs are carried in a Tunnel SRB.

Conclusion

In this contribution, we discuss the possible solutions for relay UE to transfer remote UE SRB. And we have the following proposals.

Proposal 1: It is proposed to discuss the three solutions on how to handle the remote UE SRB at relay UE in RAN2.

Option 1: The remote UE SRBs are carried in relay UE SRB.

Option 2: The remote UE SRBs are carried in relay UE DRB.

Option 3: The remote UE SRBs are carried in a Tunnel SRB.

Conclusion

[1] R2-1703063  Bearer Configuration at Relay UE in feD2D  ZTE