3GPP TSG-RAN WG2 #97bis	Tdoc R2-1703299

# Spokane, WA, USA, 3rd – 7th April 2017

Agenda Item:		8.26

Souce:					Ericsson, Samsung

Title:					Review issue list for ASN.1 freeze

Version:				0.5

Document for:		Discussion and decision

# Introduction

This document provides an overview of list of issues resulting from the review of the PDU specification including their status. For some issues the proposed solution is indicated as well as the company & Tdoc introducing this in the standard. For some of the issues this document includes further considerations. The following companies volunteered for the review.

| ID | Companies | Contact persons | Email |
| --- | --- | --- | --- |
| C | CATT | Yuan Zhang | zhangyuan1@catt.cn |
| E | Ericsson | Kai-Erik Sunell Mattias Bergström | kai-erik.sunell@ericsson.com mattias.a.bergstrom@ericsson.com |
| T | HTC | Frank Wu | frank_wu@htc.com |
| H | Huawei | Yi Guo Simone Provvedi Lei Liu | yi.guo@huawei.com simone.provvedi@huawei.com emmy.liulei@huawei.com |
| I | Intel | Hyung-Nam Choi Youn Hyoung Heo | hyung-nam.choi@intel.com youn.hyoung.heo@intel.com |
| L | LG Electronics Inc | Oanyong Lee | aidoy.lee@lge.com |
| M | MediaTek | Alex Hsu Johan Johansson | Alex.hsu@mediatek.com Johan.johansson@mediatek.com |
| N | Nokia | Tero Henttonen Jarkko Koskela Tomala Malgorzata | tero.henttonen@nokia.com jarkko.t.koskela@nokia.com malgorzata.tomala@nokia.com |
| D | NTT DOCOMO | Hideaki Takahashi Wuri A Hapsari | hideaki.takahashi.vx@nttdocomo.com wuri@nttdocomo.com |
| Q | Qualcomm | Kitazoe Masato Özcan Özturk Mungal Dhanda | mkitazoe@qti.qualcomm.com oozturk@qti.qualcomm.com mdhanda@qti.qualcomm.com |
| R | Rohde & Schwarz | Juergen Schlienz | juergen.schlienz@rohde-schwarz.com |
| S | Samsung | Himke van der Velde | himke.vandervelde@samsung.com |
| U | Sequans | Olivier Marco | omarco@sequans.com |
| W | Sierra Wireless | Steven Bennett | sbennett@sierrawireless.com |
| Y | Sony | Brian Martin | brian.martin@sony.com |
| Z | ZTE | Huang He | huang.he4@zte.com.cn |

# Discussion

<Moved to separate discussion paper>

# Conclusion & recommendation

This paper includes an overview of list of issues resulting from the review of the PDU specification. RAN2 is requested to endorse the status including the solutions proposed.

# References

[1] 	TS 36.331 E-UTRA RRC specification

# Review issue list (Annex)

Classification: 1: straigthforward clarification/ correction that can be included in next rapporteurs update, 2: small issue i.e. solution expected to be concluded easily e.g. by e-mail, 3: more significant issue i.e. requiring further discussion/ contributions, 4: more significant issue i.e. requiring further discussion/ contributions during next RAN2 working group meeting. Abbreviations used: TBD (to be done), TBC (to be confirmed/ concluded)

Companies are encouraged to report issues in the same manner as they correct issues in CRs, i.e. propose changes that are shown in the same manner as change marks. In that way, it is easier to progress the review if the solution proposals indicate how the changes in the same manner as they should be captured in the final CR. Note that also the issue list itself needs to be maintained and updated whenever new issues are added. It is easy to add new issues by accepting change marks but the downside is that the proposed changes disappear. It is therefore suggested to use “simulated change marks” for the issue reporting, i.e.

- Added parts are marked with underlined red coloured text, e.g. new text

- Deleted parts are marked with strikethrough red coloured text, e.g. delated text

- If there is a need to high-light something by marking text with a colour, e.g. to high-light small changes, it is recommended that yellow colour is used, e.g. spelling error

Some examples are shown below for issues in (a non-existent) subclause 11.4 for corrections of field descriptions, procedures and ASN.1 code.

All issues should be numbered in a format X.yyy where

- X is a n unique ID (letter) assigned to each company, see the table in clause 1

- yyy is a running number, i.e. 001, 002, …. 999

Upon submission of the review issue list it is a good idea and highly recommended to update the Next Issue list in clause 7, i.e. add the number of the next issue in the table. Otherwise there is a risk of e.g. duplicated issues. Note that it is possible to add new issues in the list by using “ real” change marks without changing the proposed solutions.

| No | Clause(s) | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- | --- |
| General |  |  |  |  |  |
| S.001 | IE used in lists | Should by default have extension marker | 2 | Add extension marker unless explicitly stated otherwise for individual case | CR |
| S.002 | General | Introduction of new IEs/ messages (e.g. MBMS specific version), resulted in several cases of fields using same content. In cases of multiple fields using same content, in principle an IE should be defined | 1/ 2 | Can consider (not urgent anyhow) | CR |
| S.003 | General | Many fields (in particular UE capabilities) are not according to naming convetions e.g. ce-pusch-nb-maxTbs should be ce-PUSCH-NB-MaxTBS | 1 | Update | CR |
| E.006 | 5.3.10.3a3 LWIP specific DRB addition or reconfiguration Many other sub-clauses | The radio interface standard is referenced as LTE in many places but usually RAN2 uses UTRA and EUTRA which are the names that are spelled out on (most of) the 3GPP specifications cover page. | 2 | It could be discussed whether all instances of LTE should be changed to EUTRA. It can be challenging to make such a change because e.g. LWA work item has already used LTE for a long time. On the other hand, why do we need two names for the standard? Is it acceptable that some work items use LTE and others EUTRA in the same specification? Or should all EUTRA changed to LTE even though that is not the name of the standard? | CR |
| E.007 | 5.6.18 Delay Budget Report 6.2.2 Message definitions | The wording “air interface” is used in some sub-clauses (probably from the same CR) but usually RAN2 specifications use the wording “radio interface”. There are no reasons to have two names for the same thing. | 2 | Use consistently the wording “radio interface” since the wording is already used in the specification (and “air interface” may create confusions with pneumatics). In sub-clause 5.6.18.1, change “air” to “radio” The purpose of this procedure is to inform E-UTRAN about the desired increment/decrement in the Uu radioair interface delay. In sub-clause 6.2.2, change “air” to “radio” in the followin information element definition – DelayBudgetReport The DelayBudgetReport message is used to inform E-UTRAN about the desired increment/decrement in the Uu radioair interface delay. In sub-clause 6.2.2, change “air” to “radio” in the following field description ueReportCause Indicates the UE preferred configuration change. Value type1 indicates that the UE prefers adjusting the CDRX cycle length by delayBudgetAdjustment. Value type2 indicates that the UE prefers adjusting the coverage enhancement configuration so that the Uu radioair interface delay changes by delayBudgetAdjustment. | CR |
| I.001 | Several | Changes to legacy MBMS and SC-PTM related IEs were introduced by FeMBMS, V2X, FeMTC and eNB-IoT. In some cases it is not clear for which features the changes are applicable, e.g. SIB13: The IE MBMS-NotificationConfig-v14xy was introduced in the context of FeMBMS. However, from the field description of notificationSF-Index it is not clear whether it is applicable for FeMBMS capable UEs only or also for V2X capable UEs. MBMS-NotificationConfig-v14xy ::= SEQUENCE { notificationSF-Index-v14xy INTEGER (7..8) OPTIONAL } SIB20: the fields sc-mcch-RepetitionPeriod-v14x0 and sc-mcch-ModificationPeriod-v14x0 were introduced in the context of V2X. However, from the concerned field descriptions it is not clear whether it is applicable for V2X capable UEs only or any Rel-14 UE supporting SC-PTM. sc-mcch-RepetitionPeriod-v14x0 ENUMERATED {rf1, spare1} OPTIONAL, -- Need OR sc-mcch-ModificationPeriod-v14x0 ENUMERATED {rf1, spare1} OPTIONAL -- Need OR | 3 | Discuss whether the concerned changes should be made applicable for the feature which introduced them only or any feature. |  |
| N.002 | All over the specifications (e.g. N.119) | Many fields cannot be fully released to avoid fullConfig during handover, and there are differing practices on how to avoid this | 4 | Discuss how we should handle release of fields to avoid full configuration. For example the following from CSI-Process: [[ eMIMO-Type-v14xy CSI-RS-ConfigEMIMO-v14xy OPTIONAL, -- Need ON eMIMO-Type2-r14 CSI-RS-ConfigEMIMO2-r14 OPTIONAL, -- Need ON eMIMO-Hybrid-r14 CSI-RS-ConfigEMIMO-Hybrid-r14 OPTIONAL, -- Need ON advancedCodebookEnabled-r14 BOOLEAN OPTIONAL -- Need ON ]] All of the fields allow release of contents within the upper level field, but once the upper level fields are configured, they can never be released and will always trigger a fullConfig handover even if the features have been turned off. | TDoc (Nokia) |
| N.003 | Several places (e.g. N.101, N.105, N132, N.133, N.148) | Should we unwrap single-field setup parts of release setup constructs? | 2 | We have several cases of release-setup structs where the setup SEQUENCE has only a single field and is not extendible. This works fine, but takes more space in specification. I.e. we could replace thes kinds of structures field1-r14 CHOICE { release NULL, setup SEQUENCE { containedField-r14 ENUMERATED {value1, value2} } } With these constructs: field1-r14 CHOICE { release NULL, containedField-r14 ENUMERATED {value1, value2} } There is no difference in the encoding parts, so this should mainly be discussed to ensure we have unified approach to all such cases. |  |
| N.004 | Several places with Need OP | Inconsistent wording on field absence conditions for Need OP fields | 2 | At least two different ways are used: “If absent, …” and “If the field is absent, the UE shall …” It should be decided if we want to unfiy the wordins everywhere (which requires quite some changes). |  |
| N.196 | CE mode A/B | Definition of CE modes is rather hidden | 3 | For someone not involved in MTC/NB-IoT before it is a tremendous effort to go figure out what is CE mode and what is CE mode A and CE mode B. Hunting for a definition in RAN1 specs is very challenging. Also, RAN1 spec uses “CEmode” while RAN2 specs spell it out as “CE mode”. It would be nice to have a simple definition added to glossary in 36.300 or 36.331 |  |
| 3.1 Definitions |  |  |  |  |  |
| 3.2 Abbreviations |  |  |  |  |  |
| M.001 | 3.2 | MUST abbreviation | 1 | MUST MultiUser Superposition Transmission | CR |
| E.060 | 3.2 | PRB abbreviation is missing | 1 | Add the abbreviation in subclause 3.2 PRB Physical Resource Block | CR |
| 4 General |  |  |  |  |  |
| H.001 | 4.2.1 UE states and state transitions including inter RAT | The current text indicates that for NB-IoT, mobility between E-UTRA and UTRAN, GERAN is not supported and hence only the E-UTRA states depicted in Figure 4.2.1-1 are applicable In rel-14, this is not completely TRUE as SA2 has introduced support for inter-RAT idle mode mobility, however without support at AS level. | 2 | Clarify that mobility is not supported at AS level: For NB-IoT, mobility between E-UTRA and UTRAN, GERAN and between E-UTRA and CDMA2000 1xRTT and CDMA2000 HRPD is not supported at AS level and hence only the E-UTRA states depicted in Figure 4.2.1-1 are applicable. | CR |
| 5.1 General |  |  |  |  |  |
| H.002 | 5.1.1 Introduction | Currently, the description of sub-clause 5.8 and 5.8a is not accurate. It is necessary to indicate the difference between 5.8 and 5.8a. | 2 | The procedural requirements are structured according to the main functional areas: system information (5.2), connection control (5.3), inter-RAT mobility (5.4) and measurements (5.5). In addition sub-clause 5.6 covers other aspects e.g. NAS dedicated information transfer, UE capability transfer, sub-clause 5.7 specifies the generic error handling, sub-clause 5.8 covers MBMS (i.e. MBMS service reception via MRB), clause 5.8a covers SC-PTM (i.e. MBMS service reception via SC-MRB), sub-clause 5.9 covers RN-specific procedures and sub-clause 5.10 covers sidelink. For NB-IoT, only a subset of the above procedural requirements applies: system information (5.2), connection control (5.3), some part of other aspects (5.6), and general error handling (5.7), and SC-PTM (5.8a). Subclauses inter-RAT mobility (5.4), measurements (5.5), MBMS (5.8), RN procedures (5.9) and Sidelink (5.10) are not applicable in NB-IoT. | CR |
| H.003 | 5.1.2 General requirements | SC-MRB is introduced in SC-PTM, therefore, not only MRBs but also SC-MRBs should be considered. | 2 | 1> consider the term 'radio bearer' (RB) to cover SRBs and DRBs but not MRBs or SC-MRBs unless explicitly stated otherwise; | CR |
| 5.2 System information |  |  |  |  |  |
| E.002 | 5.2.1.1 General 5.2.1.2 Scheduling 5.2.5bx Acquisition of an SI message from MBMS-dedicated cell 5.2.1.3 System information validity and notification of changes 5.8.1.3 MCCH information validity and notification of changes 6.2.1 General message structure 6.2.2 Message definitions 6.6a Direct Indication FeMBMS | Subclause 5.2.1.1 introduces the concept of MBMS dedicated cell and it is subsequently used in multiple subclauses. The concept is not defined in this specification but the name however gives some idea what it is supposed to mean. | 1 | The concept is defined in 36.300 and a reference to the definition in 36.300 could be added, e.g. In MBMS-dedicated cell (see TS 36.300 [9]), non-MBSFN subframes are used for providing MasterInformationBlock-MBMS (MIB-MBMS) and SystemInformationBlockType1-MBMS. SIBs other than SystemInformationBlockType1-MBMS are carried in SystemInformation-MBMS message which is also provided on non-MBSFN subframes. | CR |
| I.002 | 5.2.1.2 Scheduling | In the FeMBMS MAC CR1024 the value “FFF4” has been specified as additional SI-RNTI value. Therefore, the value “FFF9” needs to be corrected accordingly. For UEs other than BL UE or UEs in CE SI-RNTI is used to address SystemInformationBlockType1 as well as all SI messages. On MBMS-dedicated cell and on FeMBMS/Unicast-mixed cell, SI-RNTI with value FFFF is used to address all SI messages whereas SI-RNTI with value FFF9 is used to address SystemInformationBlockType1-MBMS. | 2 | Replace value “FFF9” with “FFF4”. Ericsson: It should not be changed to FFF4 as we have agreed to use FFF9 and change MAC. Though this could use similar approach as according to E.100 and only refer to MAC spec. | CR |
| L.003 | 5.2.2.28 | For transmission and CBR measurement, p2x-CommTxPoolNormalCommon, v2x-CommTxPoolNormal, p2x-CommTxPoolNormal is also used. However, it is missing in this senction. | 2 | 2> if configured to transmit V2X sidelink communication: 3> use the resource pool indicated by v2x-CommTxPoolNormalCommon, p2x-CommTxPoolNormalCommon, v2x-CommTxPoolNormal, p2x-CommTxPoolNormal or by v2x-CommTxPoolExceptional for V2X sidelink communication transmission, as specified in 5.10.13; 3> perform CBR measurement on the transmission resource pool(s) indicated by v2x-CommTxPoolNormalCommon, p2x-CommTxPoolNormalCommon, v2x-CommTxPoolNormal, p2x-CommTxPoolNormal and v2x-CommTxPoolExceptional for V2X sidelink communication transmission, as specified in 5.5.3; Coordinator (Ericsson): Same as Z.001, N.009. | CR |
| Z.001 | 5.2.2.28 Actions upon reception of SystemInformationBlockType21 | V2X sidelink communication includes P2X related and non-P2X related V2X sidelink communication. So, if the UE is configured to transmit V2X sidelink communication, the UE may use P2X related or non-P2X related resource pool. However, P2X related resource pool is missing in the description. | 2 | Correct the text as follows: 2> if configured to transmit V2X sidelink communication: 3> use the resource pool indicated by v2x-CommTxPoolNormalCommon or p2x-CommTxPoolNormalCommon or by v2x-CommTxPoolExceptional for V2X sidelink communication transmission, as specified in 5.10.13; Coordinator (Ericsson): Same as L.003, N.009 | CR |
| N.009 | 5.2.2.28 Actions upon reception of SystemInformationBlockType21 | P-UEs are not considered by the current text | 2 | Add the following condition: 2> if configured to transmit P2X related V2X sidelink communication 3> use the resource pool indicated by p2x-CommTxPoolNormalCommon or by v2x-CommTxPoolExceptional for P2X related V2X sidelink communication transmission, as specified in 5.10.13; Coordinator (Ericsson): Same L.003, Z.001. | CR |
| M.002 | 5.2.2.3 | The “depending on” in “and for NB-IoT SystemInformationBlockTypeX-NB (depending on support of multi-carrier paging/NPRACH)” may cause confusion | 2 | Don’t use “depending on”. Change to “and for NB-IoT, SystemInformationBlockTypeX-NB, if paging or PRACH on non-anchor carrier is supported” Coordinator (Ericsson): Same as I.003. See also M.003. | CR |
| I.004 | 5.2.2.3 System information acquisition by the UE | The condition “depending on support of multi-carrier paging/NPRACH” is only needed for real optional features, but support of multi-carrier paging/NPRACH is mandatory with IOT bits. Therefore, the condition can be removed. Furthermore, a redundant space can be removed. 1> neither initiate the RRC connection establishment/resume procedure nor initiate transmission of the RRCConnectionReestablishmentRequest message until the UE has a valid version of the MasterInformationBlock (MasterInformationBlock-NB in NB-IoT) and SystemInformationBlockType1 (SystemInformationBlockType1-NB in NB-IoT) messages as well as SystemInformationBlockType2 (SystemInformationBlockType2-NB in NB-IoT) , and for NB-IoT, SystemInformationBlockType22-NB (depending on support of multi-carrier paging/NPRACH); | 2 | Remove the condition “depending on support of multi-carrier paging/NPRACH” and the redundant space. Coordinator (Ericsson): Same as M.002 | CR |
| L.001 | 5.2.2.3 System information required by the UE | The procedure text has missing comma which is putting in similar sentences of 5.2.2.4. | 2 | Correct the text as follows; 3> the MasterInformationBlock (or MasterInformationBlock-NB in NB-IoT), SystemInformationBlockType1-BR (or SystemInformationBlockType1-NB in NB-IoT) and SystemInformationBlockType2 (or SystemInformationBlockType2-NB in NB-IoT), and for NB-IoT, SystemInformationBlockTypeX-NB (depending on support of multi-carrier paging/NPRACH); | CR NB_IOTenh-Core CR2625R2 |
| Z.002 | 5.2.2.3 System information required by the UE | The reason for first change: The R14 UE should receive SIB-22 only when the network schedules such SIB (considering the R13 network would not schedule SIB-22). The reason for second change: The meaning of “/” isn’t so clear. It’s suggested to use “and/or” | 3 | 2> if in RRC_IDLE: 3> if the UE is a NB-IoT UE: 4> the MasterInformationBlock-NB and SystemInformationBlockType1-NB as well as SystemInformationBlockType2-NB through SystemInformationBlockType5-NB, SystemInformationBlockType22-NB if scheduled (depending on support of multi-carrier paging and/or NPRACH); | CR |
| Z.003 | 5.2.2.3 System information required by the UE | The reason for first change: The R14 UE should receive SIB-22 only when the network schedules such SIB (considering the R13 network would not schedule SIB-22). The reason for second change: The meaning of “/” isn’t so clear. It’s suggested to use “and/or” | 3 | 2> if in RRC_CONNECTED and T311 is running; and 2> the UE is a BL UE or the UE is in CE or the UE is a NB-IoT UE; 3> the MasterInformationBlock (or MasterInformationBlock-NB in NB-IoT), SystemInformationBlockType1-BR (or SystemInformationBlockType1-NB in NB-IoT) and SystemInformationBlockType2 (or SystemInformationBlockType2-NB in NB-IoT) , and for NB-IoT SystemInformationBlockType22-NB if scheduled (depending on support of multi-carrier paging and/or NPRACH); | CR |
| E.120 | 5.2.2.4 | UE should check both schedulinginfolist and SIB1-MBMS; | 2 | Currently added clause is if and should be elseif: 2> if the UE is capable of MBMS reception as specified in 5.8: 3> if schedulingInfoList indicates that SystemInformationBlockType13 is present and the UE does not have stored a valid version of this system information block: 4> acquire SystemInformationBlockType13; 3> elseif SystemInformationBlockType13 is present in SystemInformationBlockType1-MBMS and the UE does not have stored a valid version of this system information block: 4> acquire SystemInformationBlockType13 | CR |
| L.002 | 5.2.2.4 | Similar to inter-frequency discovery, PCell could only provide the frequency information for receiving V2X sidelink communciation. Then, the UE is required to read Sib21 for the concerned freuqency. | 2 | Suggest to add the following paragraph 1> if the UE is capable of V2X sidelink communication and, for each of the one or more frequencies included in v2x-InterFreqInfoList, if included in SystemInformationBlockType21 and for which the UE is configured by upper layers to receive V2X sidelink communication on: 2> if SysemInformationBlockType21 of the serving cell/ PCell includes v2x-InterFreqInfoList which does not provide Rx resource pool for V2X sidelink communication for the concerned frequency; and 2> if schedulingInfoList of the cell on the concerned frequency indicates that SystemInformationBlockType21 is present and the UE does not have stored a valid version of this system information block: 3> acquire SystemInformationBlockType21; | CR |
| M.003 | 5.2.2.4 | Similar to the previous issue, the “depending on” in “and for NB-IoT SystemInformationBlockTypeX-NB (depending on support of multi-carrier paging/NPRACH)” may cause confusion | 2 | Change to “and for NB-IoT, SystemInformationBlockTypeX-NB, if paging or PRACH on non-anchor carrier is supported” Coordinator (Ericsson): See also M.002. | CR |
| Q.001 | 5.2.2.4 System information acquisition by the UE | 3> if SystemInformationBlockType13 is present in SystemInformationBlockType1-MBMS and the UE does not have stored a valid version of this system information block: 4> acquire SystemInformationBlockType13; | 1 | “Acquire” usually relates to the UE action to additionally read the system information. But in this case SIB13 is included in SIB1-MBMS. Suggest to at least distingush with the case schedulingInfoList–MBMS indicates that SystemInformationBlockType13 is present. Ericsson: How about: “acquire SIB13 from SIB1-MBMS”? |  |
| Z.004 | 5.2.2.4 System information acquisition by the UE | The reason for first change: The R14 UE should receive SIB-22 only when the network schedules such SIB (considering the R13 network would not schedule SIB-22). The reason for second change: The meaning of “/” isn’t so clear. It’s suggested to use “and/or” | 3 | 1> neither initiate the RRC connection establishment/resume procedure nor initiate transmission of the RRCConnectionReestablishmentRequest message until the UE has a valid version of the MasterInformationBlock (MasterInformationBlock-NB in NB-IoT) and SystemInformationBlockType1 (SystemInformationBlockType1-NB in NB-IoT) messages as well as SystemInformationBlockType2 (SystemInformationBlockType2-NB in NB-IoT) , and for NB-IoT, SystemInformationBlockType22-NB if scheduled (depending on support of multi-carrier paging and/or NPRACH); | CR |
| M.004 | 5.2.2.5 | Similar to the previous issue, the “depending on” in “and for NB-IoT, SystemInformationBlockTypeX-NB if scheduled (depending on support of multi-carrier paging/NPRACH)” may cause confuaion | 2 | Change to “or for NB-IoT, SystemInformationBlockTypeX-NB if scheduled” | CR |
| I.005 | 5.2.2.5 Essential system information missing | The condition “depending on support of multi-carrier paging/NPRACH” is only needed for real optional features, but support of multi-carrier paging/NPRACH is mandatory with IOT bits. Therefore, the condition can be removed. 2> else if the UE is unable to acquire the SystemInformationBlockType2 (or SystemInformationBlockType2-NB in NB-IoT) and for NB-IoT, SystemInformationBlockType22-NB if scheduled (depending on support of multi-carrier paging/NPRACH): | 2 | Remove the condition “depending on support of multi-carrier paging/NPRACH”. | CR |
| Z.005 | 5.2.2.5 Essential system information missing | The meaning of “/” isn’t so clear. It’s suggested to use “and/or” | 3 | 2> else if the UE is unable to acquire the SystemInformationBlockType2 (or SystemInformationBlockType2-NB in NB-IoT) and for NB-IoT, SystemInformationBlockType22-NB if scheduled (depending on support of multi-carrier paging and/or NPRACH): 3> treat the cell as barred in accordance with TS 36.304 [4]; | CR |
| N.006 | 5.2.2.7 ims-EmergencySupport | Procedure for handling this field could be clarified | 2 | Make the following clarification visible in procedures: CR2485: NOTE: Forwarding of release-9 field, ims-EmergencySupport, to upper layers is only necessary for the purpose of eCall feature in release-14. The change does not affect UE behaviour of earlier releases. Coordinator (Ericsson): Is your proposal to add a note? | CR |
| N.011 | 5.2.2.9 Actions upon reception of SystemInformationBlockType2 | 1> if cp-CIoT-EPS-Optimisation is received for the selected PLMN: 2> forward cp-CIoT-EPS-Optimisation to upper layers, if received for the selected PLMN; | 2 | Remove redundancy in conditions “if received for the selected PLMN” Coordinator (Ericsson): Which part of the text is redundant? |  |
| Q.002 | 5.2.2.9 Action upon reception of systemInformationBlockType2 | Provides a very generic statement about what to do upon recpetion of SystemInformationBlockType22-NB while more detailed actions to take are defined in section 5.2.2.x | 1 | Add reference to section 5.2.2.x to aid the reader as follows: 2> read and act on information sent in SystemInformationBlockType22-NB as defined in 5.2.2.x; | CR |
| N.012 | 5.2.2.9 Actions upon reception of SystemInformationBlockType2 | 1> if SystemInformationBlockType22-NB is scheduled: | 2 | Is the action needed at all under SIB2? There is a separate sub-section 5.2.2.X Actions upon reception of SystemInformationBlockType22-NB, instructing the UE how and what information to acquire | CR |
| Z.006 | 5.2.2.9 Actions upon reception of SystemInformationBlockType2 | There alreary has the section “5.2.2.x Actions upon reception of SystemInformationBlockType22-NB”, such text is redundant and should be deleted. | 3 | 1> if SystemInformationBlockType22-NB is scheduled: 2> read and act on information sent in SystemInformationBlockType22-NB; Coordinator (Ericsson): Same as N.007, I.006 | CR |
| N.007 | 5.2.2.x SIB22-NB reception | Why is there field specific handling for this SIB? There seems to be nothing special => there should not be anything special |  | Change the procedural to “No UE requirements related to the contents of this SystemInformationBlock22-NIB apply other than those specified elsewhere e.g. within procedures using the concerned system information, and/ or within the corresponding field descriptions.” Coordinator (Ericsson): Same as I.006, Z.006. | CR |
| I.006 | 5.2.2.x Actions upon reception of SystemInformationBlockType22-NB | Section number 5.2.2.x to be corrected to 5.2.2.29. Furthermore, the current description does not provide further information than the field description: Upon receiving SystemInformationBlockType22-NB, the UE shall: 1> apply the configuration included in the dl-CarrierConfigList; 1> apply the configuration included in the ul-CarrierConfigList; 1> apply the configuration included in the pcch-MultiCarrierConfig; 1> apply the configuration included in the nprach-MultiCarrierConfig; | 2 | Correct section number 5.2.2.x to 5.2.2.29. Replace the current description with the standard text as shown below. No UE requirements related to the contents of this SystemInformationBlock apply other than those specified elsewhere e.g. within procedures using the concerned system information, and/ or within the corresponding field descriptions. Coordinator (Ericsson): Same as N.007, Z.006. | CR |
| E.100 | 5.2.5bx Acquisition of an SI message from MBMS-dedicated cell | This subclause refers to a specific RNTI value which is defined in TS 36.321. If this value were to change changes to two specs need to be made. | 2 | 1> receive DL-SCH using SI-RNTI with value FFFFin accordance with TS 36.321 [6] from the start of the SI-window and continue until the end of the SI-window whose absolute length in time is given by si-WindowLength, or until the SI message was received, excluding the following subframes: | CR |
| N.008 | 5.2.5bx Acquisition of an SI message from MBMS-dedicated cell | This section seems to be inserted in the wrong place and not according to the CR in R2-1702419 | 2 | Create section 5.2.3b and move this part there. | CR |
| L.004 | 5.5.2.5 | tx-ResourcePoolToRemoveList, tx-ResourcePoolToAddList is missing in the first 3> bullet. | 2 | 2> if an entry with the matching measObjectId exists in the measObjectList within the VarMeasConfig, for this entry: 3> reconfigure the entry with the value received for this measObject, except for the fields cellsToAddModList, blackCellsToAddModList, whiteCellsToAddModList, altTTT-CellsToAddModList, cellsToRemoveList, blackCellsToRemoveList, whiteCellsToRemoveList, altTTT-CellsToRemoveList, measSubframePatternConfigNeigh, measDS-Config, wlan-ToAddModList, and wlan-ToRemoveList, tx-ResourcePoolToRemoveList and tx-ResourcePoolToAddList; | CR |
| L.005 | 5.5.2.5 | Wrong field name. v2x-poolIdentity is not present in ASN.1. | 2 | 3> if the received measObject includes the tx-ResourcePoolToRemoveList: 4> for each v2x-poolIdentity SL-V2X-TxPoolReportIdentity in tx-ResourcePoolToRemoveList: 5> remove the entry with the matching v2x-poolIdentity SL-V2X-TxPoolReportIdentity from the tx-ResourcePoolToAddList; 3> if the received measObject includes the tx-ResourcePoolToAddList: 4> for each v2x-poolIdentity SL-V2X-TxPoolReportIdentity in tx-ResourcePoolToAddList: 5> add a new entry for the received v2x-poolIdentity SL-V2X-TxPoolReportIdentity to the tx-ResourcePoolToAddList; | CR |
| 5.3 Connection control |  |  |  |  |  |
| M.011 | 5.3.1.3 | The agreement ‘UL grant can be provided by RRC signalling. If UE doesn’t get the UL grant from RRC signalling from source eNB, UE should monitor the PDCCH of target eNB for getting UL grant.’ Is not captured. ’ | 2 | Correct the text as follows; After receiving the handover message, the UE attempts to access the target PCell at the first available RACH occasion according to Random Access resource selection defined in TS 36.321 [6], i.e. the handover is asynchronous, or at the first available PUSCH occasion if rach-Skip is configured. Consequently, when allocating a dedicated preamble for the random access in the target PCell, E-UTRA shall ensure it is available from the first RACH occasion the UE may use. The first available PUSCH occasion is provided by ul-ConfigInfo if configured, otherwise, UE shall monitor the PDCCH of target eNB. Upon successful completion of the handover, the UE sends a message used to confirm the handover. | CR |
| N.013 | 5.3.1.3 “initial PUSCH” | What is initial PUSCH? Some definition is needed | 2 | Define somewhere what is initial PUSCH or write more explicitly what is UE behaviour Coordinator (Ericsson): Are you going to provide a definition or text proposal? Otherwise it is difficult to correct this issue. | CR |
| M.013 | 5.3.10.10 | According to agreement ‘For the MBB solution, the UE shall reset layer-2 only after stopping the connection to the source cell(s).’ For SCG change, the source cells should be the source SCG cells, but not source MCG cells. | 1 | 1> if makeBeforeBreakSCG is configured: 2> stop timer T313, if running; 2> start timer T307 with the timer value set to t307, as included in the mobilityControlInfoSCG; 2> start synchronising to the DL of the target PSCell, if needed; 2> perform the remainder of this procedure including and following resetting MAC after the UE stops the uplink transmission/downlink reception with the source SCG cell(s); NOTE x: It is up to UE implementation when to stop the uplink transmission/ downlink reception with the source SCG cell(s) to initiate re-tuning for the connection to the target cell [16], if makeBeforeBreakSCG is configured. | CR |
| I.012 | 5.3.10.10 SCG reconfiguration | Due to the new note x all notes in this section should be renumbered, i.e. x->1, 0->1, 1->2 and 2->3. NOTE x: It is up to UE implementation when to stop the uplink transmission/ downlink reception with the source cell(s) to initiate re-tuning for the connection to the target cell [16], if makeBeforeBreakSCG is configured. | 2 | Renumber all notes from 1 to 3. Coordinator (Ericsson): According to TR 21.801 every attempt shall be made to use continuous numbering. However, if continuous numbering cannot be maintained, a new element may be inserted in existing text using an appropriate alphanumeric designation that does not disturb the existing numbering scheme. This applies to all elements (e.g. clause, subclause, annex, figure, table, note, list). So perhaps we should try to maintain the numbering and use alphanumeric designations instead. | CR |
| I.013 | 5.3.10.10 SCG reconfiguration | In the highlight part the condition “if rach-skipSCG is configured” is needed. 3> the procedure ends, except that the following actions are performed when MAC successfully completes the random access procedure on the PSCell or when MAC indicates the successful reception of a PDCCH transmission addressed to C-RNTI: | 2 | Add condition “if rach-skipSCG is configured” as shown below. 3> the procedure ends, except that the following actions are performed when MAC successfully completes the random access procedure on the PSCell or when MAC indicates the successful reception of a PDCCH transmission addressed to C-RNTI if rach-skipSCG is configured: | CR |
| C.001 | 5.3.10.15a | When V2X UE receives a RRCConnectionReconfiguration, the dedicated V2X resource might indicated by commTxResources or v2x-InterFreqInfoList, the latter one is missing in current CR | 2 | The UE shall: 1> if the RRCConnectionReconfiguration message includes the sl-V2X-ConfigDedicated or sl-P2X-ConfigDedicated: 2> if commTxResources is included and set to setup;: or 2> if v2x-InterFreqInfoList is included: 3> use the resources indicated by commTxResources or v2x-InterFreqInfoList for V2X sidelink communication transmission, as specified in 5.10.13; 3> perform CBR measurement on the transmission resource pool indicated in sl-V2X-ConfigDedicated for V2X sidelink communication transmission, as specified in 5.5.3; | CR |
| E.103 | 5.3.10.15a V2X sidelink Communication dedicated configuration | If sl-V2X-ConfigDedicated is not included (i.e. the text is performed for P2X) the highlighted text becomes ambiguous. It is not clear which resources the UE would use for CBR measurements. 1> if the RRCConnectionReconfiguration message includes the sl-V2X-ConfigDedicated or sl-P2X-ConfigDedicated: 2> if commTxResources is included and set to setup: 3> use the resources indicated by commTxResources for V2X sidelink communication transmission, as specified in 5.10.13; 3> perform CBR measurement on the transmission resource pool indicated in sl-V2X-ConfigDedicated for V2X sidelink communication transmission, as specified in 5.5.3; | 2 | Add “or sl-P2X-ConfigDedicated”. 1> if the RRCConnectionReconfiguration message includes the sl-V2X-ConfigDedicated or sl-P2X-ConfigDedicated: 2> if commTxResources is included and set to setup: 3> use the resources indicated by commTxResources for V2X sidelink communication transmission, as specified in 5.10.13; 3> perform CBR measurement on the transmission resource pool indicated in sl-V2X-ConfigDedicated or sl-P2X-ConfigDedicated for V2X sidelink communication transmission, as specified in 5.5.3; | CR |
| N.014 | 5.3.10.15a V2X sidelink Communication dedicated configuration | Usage of sl-P2X-ConfigDedicated needs discussions | 2 | If N.054 is agreed, then the procedure in section 5.3.10.15a will have to be slightly changed as well: “The UE shall: 1> if the RRCConnectionReconfiguration message includes the sl-V2X-ConfigDedicated or sl-P2X-ConfigDedicated:” The strikeout part should be removed if we don’t have two fields. | CR |
| Z.007 | 5.3.10.15a V2X sidelink Communication dedicated configuration | The resource pools indicated in sl-V2X-ConfigDedicated include mode 3 or mode 4 resource pools and inter-frequency resource pools. If the frequency used to transmit V2X sidelink communication is included in v2x-InterFreqInfoList, the IE commTxResources may be absent. In this situation, the UE shall perform CBR measurement on the transmission resource pool indicated in v2x-InterFreqInfoList. However, this case is not supported in the current specification. As a result, the description of the performing CBR measurement on the transmission resource pool indicated in v2x-InterFreqInfoList should be not removed from the first 2>bullet to the third 2>bullet. | 3 | Correct the text as follows: 2> if commTxResources is included and set to setup: 3> use the resources indicated by commTxResources for V2X sidelink communication transmission, as specified in 5.10.13; 3> perform CBR measurement on the transmission resource pool indicated in commTxResources for V2X sidelink communication transmission, as specified in 5.5.3; 2> else if commTxResources is included and set to release: 3> release the resources allocated for V2X sidelink communication transmission previously configured by commTxResources; 2> if v2x-InterFreqInfoList is included: 3> use the synchronization configuration and resource configuration parameters for V2X sidelink communication on frequencies included in v2x-InterFreqInfoList; 3> perform CBR measurement on the transmission resource pool indicated in v2x-InterFreqInfoList for V2X sidelink communication transmission, as specified in 5.5.3; | CR |
| Z.008 | 5.3.10.15a V2X sidelink Communication dedicated configuration | It was agreed in RAN1 that UE also measures CBR on the exceptional pool. UE reports the CBR measurement on the exceptional pool if requested by eNB. UE adaptation of transmission parameters is supported on the exceptional pool. However, it is not clear whether radio parameters adaptation based on CBR on the exceptional pool configured in mobilityControlInfoV2X should be supported, which should be further discussed. If yes, the description of UE performing CBR measurement on the transmission resource pool indicated in v2x-CommTxPoolExceptional in the HO scenario should be added in section 5.3.10.15a. | 3 | Coordinator (Ericsson): Are you going to provide a solution or text proposal? Otherwise it is difficult to correct this issue. |  |
| H.005 | 5.3.10.6 | if the pusch-EnhancementsConfiguration is included in the received physicalConfigDedicated, for the associated serving cell: Only pusch enhanced mode can be configured for PCell, here we mentioned associated serving cell, shall we make clear that it is for PCell? | 2 | Change the “the associated serving cell:” to “PCell” | CR |
| Q.004 | 5.3.10.6 Physical channel configuration | pusch-EnhancementsConfiguration is called pusch-EnhancmentsConfig in physicalConfigDedicated | 1 | Correct Ie name as follows: pusch-EnhancementsConfiguration | CR |
| L.007 | 5.3.10.6 Physical channel reconfiguration | There is a capital letter which is not needed in the procedure text. | 1 | Correct the text as follows; 2> else: 3> use the carrier configuration received in the system information for the uplink carrier and downlink carrier used during the Random random access procedure; | CR NB_IOTenh-Core CR2625R2 |
| M.014 | 5.3.3.3 | No indication to MAC | 3 | 1> else if the UE supports mo-VoiceCall establishment cause for mobile originating MMTEL video and UE is establishing the RRC connection for mobile originating MMTEL video and SystemInformationBlockType2 includes videoServiceCauseIndication: 2> set the establishmentCause to mo-VoiceCall; 2> indicate MAC entity to include mo-VideoCall to MSG3 Coordinator (Ericsson): Same as M.015? | CR |
| M.015 | 5.3.3.3a | No indication to MAC | 3 | 1> else if the UE supports mo-VoiceCall establishment cause for mobile originating MMTEL video and UE is resuming the RRC connection for mobile originating MMTEL video and SystemInformationBlockType2 includes videoServiceCauseIndication: 2> set the resumeCause to mo-VoiceCall; 2> indicate MAC entity to include mo-VideoCall to MSG3 Coordinator (Ericsson): Same as M.014? | CR |
| M.006 | 5.3.3.4 | The term “frequency offset” has a different meaning and may cause confusion | 2 | Remove “frequency” in “discard the dedicated frequency offset provided by the redirectedCarrierOffsetDedicated” | CR |
| E.004 | 5.3.3.4 Reception of the RRCConnectionSetup by the UE | The procedure references dcn-id field but the field is defined with an identifier name dcn-ID in the ASN.1, dcn-ID INTEGER (0..65535) OPTIONAL, According naming convention, ID should be spelled with capital letters in this case. | 2 | Change the field name from dcn-id to dcn-ID in subclause 5.3.3.4 2> include dcn-idID if a DCN value (see TS 23.401 [41]) is received from upper layers; Coordinator (Ericsson): See also I.008. | CR |
| I.008 | 5.3.3.4 Reception of the RRCConnectionSetup by the UE | To be consistent the “dcn-id” should be corrected to “dcn-Id”. Furthermore, a line break should be added for the subsequent condition. 2> include dcn-id if a DCN value (see TS 23.401 [41]) is received from upper layers;2> submit the RRCConnectionSetupComplete message to lower layers for transmission, upon which the procedure ends; | 2 | Replace “dcn-id” with “dcn-Id” and add a line break for the subsequent condition. Coordinator (Ericsson): See also E.004. | CR |
| N.021 | 5.3.3.4 Reception of the RRCConnectionSetup by the UE | “DCN value” as such does not exist in the referenced TS: 2> include dcn-id if a DCN value (see TS 23.401 [41]) is received from upper layers; | 2 | Change “DCN value” to “DCN-ID value” | CR |
| N.016 | 5.3.3 RRC connection estalishment | Procedure description captured below messeges flows should cover “Establishment” and “Resume”: The purpose of this procedure is to establish or resume an RRC connection from a suspended RRC connection or resume an RRC connection from a light RRC connection. | 2 | Replace by simplified sentence: “The purpose of this procedure is to establish or resume an RRC connection” Applicability to light RRC connection state isn’t missed as still clarified below. | CR |
| M.007 | 5.3.3.4a | Similar to previous issue, the term “frequency offset” may cause confusion | 2 | Remove “frequency” in “discard the dedicated frequency offset provided by the redirectedCarrierOffsetDedicated” | CR |
| H.008 | 5.3.3.4a Reception of the RRCConnectionResume by the UE | It is clear that redirectedCarrierOffsetDedicated only apply to NB-IoT so there is no need to clarify that applicability of the bullet. This will aslo align with the text related to timer T322 later in the section | 2 | Change as follows: 1> for NB-IoT, if stored, discard the dedicated frequency offset provided by the redirectedCarrierOffsetDedicated; | CR |
| M.008 | 5.3.3.x | According to the agreement in RAN2#97, the purpose of such an indication is to inform upper layer the delivery failure of NAS information carried by RRCConnectionSetupComplete. However, the title “Failure to deliver RRCConnectionSetupComplete message” does not reflect this point. | 2 | Change the title to “Failure to deliver NAS information in RRCConnectionSetupComplete message” | CR |
| M.009 | 5.3.3.x | Similar to previous issue, “NAS information” should be included in the text. | 2 | Change “information” to “NAS information” | CR |
| N.019 | 5.3.3.x Failure to deliver RRCConnectionSetupComplete message | Procedure: - if the UE is a NB-IoT UE and radio link failure occurs before the successful delivery of RRCConnectionSetupComplete; - inform upper layers about the possible failure to deliver the information contained in the RRCConnectionSetupComplete message; indicates that upper layers receives information about possible failure to deliver the message, which seems to suggest a new cause for RLF notofocation to upper layers. | 3 | Clarify the failure difference from regular RLF | CR/ TDoc WI (Nokia?) |
| Q.003 | 5.3.3.x Failure to deliver RRCConnectionsetupComplete message | Handling of failure to deliver RRCConnectionSetupComplete message is defined before defining when/how to submit RRCConnectionSetupComplete message. | 1 | Suggest moving this new subclause to between subclause 5.3.3.4 and subclause 5.3.3.5. As it is an abnormal case handling, perhaps it is even better to move this to after subclause 5.3.3.9. |  |
| E.126 | 5.3.5.3 and 5.3.5.4 | Clarify that the UE indicates gap indications for each serving cell | 2 | Currently it is captured in the field-description of servCellId that the UE indicates a gap indication per serving cell. This is better captured in the procedural text. Suggestion: 1> set the content of RRCConnectionReconfigurationComplete message as follows: 2> if the RRCConnectionReconfiguration message includes perCC-GapIndicationRequest: 3> include perCC-ListGapIndication and numFreqEffective for each serving cell; | CR |
| M.005 | 5.3.5.4 | The procedure text is correct but the style differs from rest of the specification | 1 | Change “if neither handoverWithoutWT-Change nor keep-LWA-Config is configured” to: if neither either handoverWithoutWT-Change nor or keep-LWA-Config is not configured | CR |
| M.012 | 5.3.5.4 | The description maybe misleading. No matter makebeforebreak is configured or not, UE should perform DL sync to target eNB. In DC, I am not clear whether UE can perform DL sync simultaneously with both target MCG and target SCG. | 2 | 1> if makeBeforeBreak is configured: 2> start synchronising to the DL of the target PSCell, if mobilityControlInfoSCG is included; 2> perform the remainder of this procedure including and following resetting MAC after the UE stops the uplink transmission/downlink reception with the source cell(s); | CR |
| I.045 | 5.3.5.4 Reception of an RRCConnectionReconfiguration including the mobilityControlInfo by the UE (handover) | The condition “if rach-skip is configured” is missing for th condition “if MAC indicates the successful reception of a PDCCH transmission addressed to C-RNTI”. | 2 | Add the missing condition “if rach-skip is configured” as shown below. 1> if MAC successfully completes the random access procedure; or 1> if MAC indicates the successful reception of a PDCCH transmission addressed to C-RNTI if rach-skip is configured: 2> stop timer T304; 2> release ul-ConfigInfo, if configured; | CR |
| T.003 | 5.3.5.4 Reception of an RRCConnectionReconfiguration including the mobilityControlInfo by the UE (handover) | The “makeBeforeBreak” should only be applicable to handover and not applicable to SCG change. | 2 | 1> if makeBeforeBreak is configured: 2> start synchronising to the DL of the target PSCell, if mobilityControlInfoSCG is included; 2> perform the remainder of this procedure including and following resetting MAC after the UE stops the uplink transmission/downlink reception with the source cell(s); Coordinator (Ericsson): Same as H.004. | CR LTE_eMob-Core CR2689R3 |
| H.004 | 5.3.5.4 Reception of an RRCConnectionReconfiguration including the mobilityControlInfo by the UE (handover) | Since the UE delays L2 reset after it stops connection with the source cell, then how can the UE start synchronising to the DL of the target PSCell? In addition, if some change is needed for makebeforebreak, it should be added in Sec 5.3.10.10 SCG reconfiguration. | 2 | Remove the following sentence: 1> if makeBeforeBreak is configured: 2> start synchronising to the DL of the target PSCell, if mobilityControlInfoSCG is included; 2> perform the remainder of this procedure including and following resetting MAC after the UE stops the uplink transmission/downlink reception with the source cell(s); Coordinator (Ericsson): Same as T.003. | CR |
| E.932 | 5.3.5.4 Reception of an RRCConnectionReconfiguration including the mobilityControlInfo by the UE (handover) | Strange procedure text. Should the highlighted text be executed also in case makeBeforeBreakSCG-r14 is not included in mobilityControlInfoSCG? If so, there is contradiction with field description 1> if makeBeforeBreak is configured: 2> start synchronising to the DL of the target PSCell, if mobilityControlInfoSCG is included; 2> perform the remainder of this procedure including and following resetting MAC after the UE stops the uplink transmission/downlink reception with the source cell(s); NOTE 1b: It is up to UE implementation when to stop the uplink transmission/ downlink reception with the source cell(s) to initiate re-tuning for connection to the target cell [16], if makeBeforeBreak is configured. | 3 | Further discussion needed. | CR / TDoc WI (Ericsson) |
| E.933 | 5.3.5.4 Reception of an RRCConnectionReconfiguration including the mobilityControlInfo by the UE (handover) | Not clear from the procedure text on whether inclusion of makeBeforeBreak adds means any additional/new requirements on UE to continue ul/dl with source cell. With the Note 1b, it seems it will not be possible to get any improved performance by setting the makeBeforeBreak. Is this intentional? Existing text: 1> if makeBeforeBreak is configured: 2> start synchronising to the DL of the target PSCell, if mobilityControlInfoSCG is included; 2> perform the remainder of this procedure including and following resetting MAC after the UE stops the uplink transmission/downlink reception with the source cell(s); NOTE 1b: It is up to UE implementation when to stop the uplink transmission/ downlink reception with the source cell(s) to initiate re-tuning for connection to the target cell [16], if makeBeforeBreak is configured. | 4 | Further discussion needed. | CR/ TDoc WI (Ericsson) |
| I.009 | 5.3.5.4 Reception of an RRCConnectionReconfiguration including the mobilityControlInfo by the UE (handover) | To be consistent, the word “received” is not needed in the condition below and thus can be removed. 1> if the received RRCConnectionReconfiguration message includes the rach-Skip: 2> apply the NTA value for the target MCG PTAG, see TS 36.213 [23], as indicated by targetTA in rach-Skip; | 2 | Remove “received” from the concerned condition. | CR |
| S.004 | 5.3.5.5, delay budget, BW preference, SPS assistance | For BW preference and SPS assistance the following is specified for the case of HO: a) The delay reporting configuration is forwarded to target (as part of otherConfig) b) The UE does not release the delay reporting configuration c) The information reported by the UE is forwarded to target (as part of UEAssistanceInformation) d) UE does repeat the information reported when reported too late to be forwarded (as part of UEAssistanceInformation) For delay budget, c and d are not covered due to the use of a specific message. It is unclear if this difference is intentional Furthermore, upon re-establishment, otherConfig is released including BW preference and delay budget configuration but SPS assistance is not | 2 | Release SPS assistance configuration upon re-establisment. Discuss handling of delay buget reporting info upon HO i.e. whether to align with other cases | CR/ TDoc XX |
| S.005 | 5.3.5.x or 5.3.8.x? | A section for T323 expiry seems missing | 2 | Introduce (straigtforward correction) | CR |
| I.011 | 5.3.8.3 Reception of the RRCConnectionRelease by the UE | In the conditions below, some editorial corrections can be made: 1> for NB-IoT, if the RRCConnectionRelease message includes the redirectedCarrierInfo: 2> if the redirectedCarrierOffsetDedicated is included in the redirectedCarrierInfo: 3> store the redirectedCarrierOffsetDedicated for the frequency in redirectedCarrierInfo; 3> start timer T322, with the timer value set according to the value of t322 in redirectedCarrierInfo; | 2 | Correct the conditions as shown below: i) remove blank in the first condition of level “2>”; ii) in the first condition of level “3>” replace “redirectedCarrierOffsetDedicated” with “dedicated frequency offset”: 1> for NB-IoT, if the RRCConnectionRelease message includes the redirectedCarrierInfo: 2> if the redirectedCarrierOffsetDedicated is included in the redirectedCarrierInfo: 3> store the dedicated frequency offset redirectedCarrierOffsetDedicated for the frequency in redirectedCarrierInfo; 3> start timer T322, with the timer value set according to the value of t322 in redirectedCarrierInfo; | CR |
| E.005 | 5.3.8.5 UE actions upon receiving the expiry of dataInactivityTimer | The procedure references DataInactivityTimer information element whereas the procedure is termed as UE actions upon expiry of dataInactivityTimer according to the corresponding field name. A problem arises, since neither an abstract data type definition (information element) nor the reference to its value (field) can “expire”. The intention of the procedure is not to reference the value where the timer expires but instead reference an event or indication of the timer expiry from MAC regardless what value is used and how the possible values are defined. The timer will run from 0 to the value where it expires and the timer is defined in 36.321 and termed as DataInactityTimer as shown below, 5.xx Data inactivity monitoring The MAC entity may be configured by RRC with a Data inactivity monitoring functionlality, when in RRC_CONNECTED. RRC controls Data inactivity operation by configuring the timers DataInactivityTimer. It is therefore proposed to align the procedure name with the timer name in 36.321 and remove the word “Timer” from the identifier names in 36.331 because they are not timers even though they define and carry the value where the timer expires. Otherwise the name DataInacitivityTimer is used in two different purposes. | 3 | Change the sub-clause title as 5.3.8.5 UE actions upon receiving the expiry of DdataInactivityTimer Remove the word “Timer” from the identifier names in subclause 6.3.2 – DataInactivityTimer The IE DataInactivityTimer is used to control Data inactivity operation. Corresponds to the timer for data inactivity monitoring in TS 36.321 [6]. Value s1 corresponds to 1 second, s2 corresponds to 2 seconds and so on. DataInactivityTimer information element -- ASN1START DataInactivityTimer-r14 ::= ENUMERATED { s1, s2, s3, s5, s7, s10, s15, s20, s40, s50, s60, s80, s100, s120, s150, s180} Remove the word “Timer” from the identifier names in MAC-MainConfig in subclayse 6.3.2 [[ dataInactivityTimerConfig-r14 CHOICE { release NULL, setup SEQUENCE { dataInactivityTimer-r14 DataInactivityTimer-r14 } } OPTIONAL -- Need ON ]] Remove the word “Timer” from the NB-IoT imports in subclause 6.7.1 TMGI-r9, TrackingAreaCode, DataInactivityTimer-r14 FROM EUTRA-RRC-Definitions; -- ASN1STOP Remove the word “Timer” from the identifier names in MAC-MainConfig-NB in subclause 6.7.3 [[ dataInactivityTimerConfig-r14 CHOICE { release NULL, setup SEQUENCE { dataInactivityTimer-r14 DataInactivityTimer-r14 } } OPTIONAL -- Need ON ]] | CR |
| N.023 | 5.3.8.5 UE actions upon receiving the expiry of dataInactivityTimer | The section title refers to a field “dataInactivityTimer”, while such field does not exist | 2 | To align with the IE description use capital letter for “Data” or call it “timer for data inactivity monitoring”: UE actions upon receiving the expiry of DataInactivityTimer UE actions upon receiving the expiry of timer for data inactivity monitoring | CR |
| S.006 | 5.3.8.7 | Actions upon selecting inter-RAT cell seem misplaced | 2 | Move to 5.4.6 | CR |
| M.010 | 5.3.8.x Txxx expiry | Txxx seems to apply to NB-IOT UE only. Should we mention this? | 2 | Mention “for NB-IOT UE” somewhere in the text | CR |
| L.006 | 5.3.8.x Txxx expiry | The procedure text is needed to be correct for more understanding. | 1 | Correct the text as follows; The UE shall: 1> if Txxx expires: 2> discard the redirectedCarrierOffsetDedicated provided in by RRCConnectionRelease message; | CR NB_IOTenh-Core CR2625R2 |
| 5.4 Inter-RAT mobility |  |  |  |  |  |
| 5.5 Measurements |  |  |  |  |  |
| T.004 | 5.5.1 Introduction | Since reportAnyWLAN In ReportConfigInterRAT is configurable now, the introduction in 5.5.1 should be revised accordingly. | 3 | Detected cells - these are cells that are not listed within the measurement object(s) but are detected by the UE on the carrier frequency(ies) indicated by the measurement object(s) or, for inter-RAT WLAN, the WLANs not included in the MeasObjectWLAN but meeting the triggering requirements if reportAnyWLAN is configured. | CR LTE_WLAN_aggr-Core CR2676R3 |
| C.003 | 5.5.2.5 | “tx-ResourcePoolToAddList” should be “tx-ResourcePoolToRemoveList” | 1 | 5> remove the entry with the matching v2x-poolIdentity from the tx- ResourcePoolToRemoveListResourcePoolToAddList; | CR |
| E.904 | 5.5.2.9 Measurement gap configuration | Procedure text is not updated to cover measGapConfig-r14 See also E.902 5.5.2.9 Measurement gap configuration The UE shall: 1> if measGapConfig is set to setup: 2> if a measurement gap configuration is already setup, release the measurement gap configuration; 2> setup the measurement gap configuration indicated by the measGapConfig in accordance with the received gapOffset, i.e., the first subframe of each gap occurs at an SFN and subframe meeting the following condition (SFN and subframe of MCG cells): SFN mod T = FLOOR(gapOffset/10); subframe = gapOffset mod 10; with T = MGRP/10 as defined in TS 36.133 [16]; NOTE: The UE applies a single gap, which timing is relative to the MCG cells, even when configured with DC. 1> else: 2> release the measurement gap configuration; | 4 | Text proposal should be provided in separate CR. Coordinator (Ericsson): See also E.902 | CR/ TDoc WI (Ericsson) |
| L.008 | 5.5.3.1 | P-UE does not perform CBR measurement. However, it is possible for the P-UE to perform CBR measurement if P-UE is capable of CBR measurement. | 2 | The UE capable of CBR measurement when configured to transmit non-P2X related V2X sidelink communication shall: if in coverage on the frequency used for V2X sidelink communication transmission as defined in TS 36.304 [4, 11.4]; or | CR |
| L.009 | 5.5.3.1 | The UE performs CBR measurement on the ‘configured’ pool. | 2 | 2> if the UE is in RRC_IDLE: 3> if the concerned frequency is the camped frequency: 4> perform CBR measurement on the pools in v2x-CommTxPoolNormalCommon, p2x-CommTxPoolNormalCommon and v2x-CommTxPoolExceptional, if configured, in SystemInformationBlockType21; 3> else if the concerned frequency is included in v2x-InterFreqInfoList within SystemInformationBlockType21: 4> perform CBR measurement on pools in v2x-CommTxPoolNormal, p2x-CommTxPoolNormal and v2x-CommTxPoolExceptional, if configured, in v2x-InterFreqInfoList for the concerned frequency in SystemInformationBlockType21; 3> else if the concerned frequency broadcasts SystemInformationBlockType21: 4> perform CBR measurement on pools in v2x-CommTxPoolNormalCommon, p2x-CommTxPoolNormalCommon and v2x-CommTxPoolExceptional, if configured, in SystemInformationBlockType21; 2> if the UE is in RRC_CONNECTED: 3> if the concerned frequency is the PCell’s frequency: 4> perform CBR measurement on the pools in v2x-CommTxPoolNormalDedicated in RRCConnectionReconfiguration and v2x-CommTxPoolExceptional, if configured, in SystemInformationBlockType21; 3> else if the concerned frequency is included in v2x-InterFreqInfoList within RRCConnectionReconfiguration: 4> perform CBR measurement on pools in v2x-CommTxPoolNormal, p2x-CommTxPoolNormal and v2x-CommTxPoolExceptional, if configured, in v2x-InterFreqInfoList for the concerned frequency in RRCConnectionReconfiguration; 3> else if the concerned frequency broadcasts SystemInformationBlockType21: 4> perform CBR measurement on pools in v2x-CommTxPoolNormalCommon, p2x-CommTxPoolNormalCommon and v2x-CommTxPoolExceptional, if configured, in SystemInformationBlockType21; | CR |
| E.102 | 5.5.3.1 General | The first paragraph lists a large number of exceptions to when layer 3 filtering is used. It may be confusing which measurements use layer 3 filtering, i.e. there might be more exceptions to the rule than cases when the rule is used. There is also a related note in section 5.5.3.2: NOTE 1: This does not include quantities configured solely for UE Rx-Tx time difference, SSTD measurements and RSSI, channel occupancy measurements, WLAN measurements of Band, Carrier Info, Available Admission Capacity, Backhaul Bandwidth, Channel Utilization, and Station Count, CBR measurement, and UL PDCP Packet Delay per QCI measurement i.e. for those types of measurements the UE ignores the triggerQuantity and reportQuantity. | 3 | Consider rewriting the paragraph making it clear which measurements use and which do not use layer 3 filtering. | CR/ TDoc WI (Ericsson) |
| H. 009 | 5.5.3.1 General | 1. For V2X sidelink communication operated on a carrier other than the serving carrier, UE should perform CBR measurement as per serving cell configuration only if the concerned frequency and the associated resource pool configuration are both included in v2x-InterFreqInfoList; 2.CBR measurement should apply to also mode-3 pools, but this is now missing in the current CR. | 2 | Add the following texts in subclause 5.5.3.1 if in coverage on the frequency used for V2X sidelink communication transmission as defined in TS 36.304 [4, 11.4]; or if the concerned frequency is included in v2x-InterFreqInfoList in RRCConnectionReconfiguration or in v2x-InterFreqInfoList within SystemInformationBlockType21: 2> if the UE is in RRC_IDLE: 3> if the concerned frequency is the camped frequency: 4> perform CBR measurement on the pools in v2x-CommTxPoolNormalCommon, p2x-CommTxPoolNormalCommon and v2x-CommTxPoolExceptional in SystemInformationBlockType21; 3> else if the concerned frequency as well as the resource pools of the concerned freqeuncy are is included in v2x-InterFreqInfoList within SystemInformationBlockType21: 4> perform CBR measurement on pools in v2x-CommTxPoolNormal, p2x-CommTxPoolNormal and v2x-CommTxPoolExceptional in v2x-InterFreqInfoList for the concerned frequency in SystemInformationBlockType21; 3> else if the concerned frequency broadcasts SystemInformationBlockType21: 4> perform CBR measurement on pools in v2x-CommTxPoolNormalCommon, p2x-CommTxPoolNormalCommon and v2x-CommTxPoolExceptional in SystemInformationBlockType21broadcasted on the concerned freqeuncy; 2> if the UE is in RRC_CONNECTED: 3> if the concerned frequency is the PCell’s frequency: 4> perform CBR measurement on the pools in v2x-CommTxPoolNormalDedicated or v2x-SchedulingPool in RRCConnectionReconfiguration and v2x-CommTxPoolExceptional in SystemInformationBlockType21; 3> else if the concerned frequency as well as the resource pools of the concerned freqeuncy are is included in v2x-InterFreqInfoList within RRCConnectionReconfiguration: 4> perform CBR measurement on pools in v2x-CommTxPoolNormal, p2x-CommTxPoolNormal, v2x-SchedulingPool and v2x-CommTxPoolExceptional in v2x-InterFreqInfoList for the concerned frequency in RRCConnectionReconfiguration; 3> else if the concerned frequency broadcasts SystemInformationBlockType21: 4> perform CBR measurement on pools in v2x-CommTxPoolNormalCommon, p2x-CommTxPoolNormalCommon and v2x-CommTxPoolExceptional in SystemInformationBlockType21 broadcasted on the concerned freqeuncy; 1> else: 2> perform CBR measurement on the transmission pools in SL-V2X-Preconfiguration for the concerned frequency; | CR |
| N.025 | 5.5.3.1 General | P-UEs do not perform CBR measurements while the text mandates Idle UEs to perform CBR measurments on p2x-CommTxPoolNormalCommon | 2 | p2x-CommTxPoolNormalCommon should be removed from the measured pools for UEs in RRC_IDLE (both intra- and inter-frequency) | CR |
| N.026 | 5.5.3.1 General | Procedure for CBR measurements for RRC_IDLE UEs does not acount for the case when only frequency without resources configuration is provided or only reception resources are provided. | 2 | Replace this condition: 3> else if the concerned frequency is included in v2x-InterFreqInfoList within SystemInformationBlockType21: With something like: 3> else if v2x-UE-SelectionConfigList containing v2x-CommTxPoolNormal or v2x-CommTxPoolExceptional is included in v2x-InterFreqInfoList for the concerned frequency within SystemInformationBlockType21: Or like: 3> else if v2x-CommTxPoolExceptional is included in v2x-InterFreqInfoList for the concerned frequency in SystemInformationBlockType21 The latter approach is aligned with some other paragraphs (e.g. 5.10.13) | CR |
| N.027 | 5.5.3.1 General | Similar issue to N.025 but the issue is more cmplicated for RRC_CONNECTED UEs. p2x-CommTxPoolNormal may be measured but only in case UE is configured to report this measurement by the eNB (as normally CBR measurement capable UE wil not transmit on this resource pool) | 3/4 | A separate condition should be defined for measuring CBR on p2x-CommTxPoolNormal only in case it is configured by the eNB for reporting. Or this may be handled in even another way. UE might not be necessarily aware that this is p2x pool, so a condition should be added that measurements should be performed also on any v2x pool configured for reporting by the eNB. This might require some more discussions | TDoc WI (Nokia) |
| N.028 | 5.5.3.1 General | For RRC_CONNECTED the eNB configures TX resource pools. UE does not have to perform CBR measurements for Tx pools on other frequencies if they are not configured as Tx resource pools | 2/3 | Remove: 3> else if the concerned frequency broadcasts SystemInformationBlockType21: 4> perform CBR measurement on pools in v2x-CommTxPoolNormalCommon, p2x-CommTxPoolNormalCommon and v2x-CommTxPoolExceptional in SystemInformationBlockType21; From procedure for RRC_CONNECTED UEs | CR |
| Z.010 | 5.5.3.1 General | It was agreed in RAN2 that P-UEs do not perform CBR measurement and IDLE UEs do not report CBR measurements. So RRC_IDLE UE do not perform CBR measurement on the pools in p2x-CommTxPoolNormalCommon in SIB21. | 2 | Correct the text as follows: 2> if the UE is in RRC_IDLE: 3> if the concerned frequency is the camped frequency: 4> perform CBR measurement on the pools in v2x-CommTxPoolNormalCommon, p2x-CommTxPoolNormalCommon and v2x-CommTxPoolExceptional in SystemInformationBlockType21; | CR |
| Z.011 | 5.5.3.1 General | It was agreed in RAN2 that the UE will measure all configured tx pools (i.e. dedicate configured pools for RRC Connected UEs and the SIB21 configured pools for IDLE UEs). In our understanding, mode 3 UE perform CBR measruement only on the pools in v2x-CommTxPoolExceptional in SIB21. And it is not necessary for mode 4 UE to perform CBR measruement on the pools in CommTxPoolExceptional since exceptional pool would not be used for V2X transmission by mode 4 UEs. However, it is not specified in the current specification. |  | Is it common understanding that mode 3 UE perform CBR measruement only on the pools in v2x-CommTxPoolExceptional in SIB21 and mode 4 UE do not perform CBR measruement on the pools in CommTxPoolExceptional? If yes, is it necessary to clarify it in the text procedure in the below? 2> if the UE is in RRC_CONNECTED: 3> if the concerned frequency is the PCell’s frequency: 4> perform CBR measurement on the pools in v2x-CommTxPoolNormalDedicated in RRCConnectionReconfiguration and v2x-CommTxPoolExceptional in SystemInformationBlockType21; Coordinator (Ericsson): A clarification proposal would make it easier to capture this in the CR. |  |
| Z.012 | 5.5.3.1 General | We assume that it is common understanding that RRC_CONNECTED UE could perform CBR measurement and reporting on the normal respurce pools configured in SIB21 in the last meeting. However, it is not supported in the current specification. We would like to know other companies’ view first. | 3 | We assume that it is common understanding that RRC_CONNECTED UE could perform CBR measurement and reporting on the normal respurce pools configured in SIB21 in the last meeting. Is it correct? If yes, the following text procedure should be modified to support the above case. 2> if the UE is in RRC_CONNECTED: 3> if the concerned frequency is the PCell’s frequency: 4> perform CBR measurement on the pools in v2x-CommTxPoolNormalDedicated in RRCConnectionReconfiguration and v2x-CommTxPoolExceptional in SystemInformationBlockType21; Coordinator (Ericsson): A clarification proposal would make it easier to capture this in the CR. |  |
| L.010 | 5.5.4.1 | The measurement result for PCell is only reported together. | 2 | 2> if the triggerType is set to event and if the entry condition applicable for this event, i.e. the event corresponding with the eventId of the corresponding reportConfig within VarMeasConfig, is fulfilled for one or more applicable transmission resource pools for all measurements taken during timeToTrigger defined for this event within the VarMeasConfig, while the VarMeasReportList does not include an measurement reporting entry for this measId (a first transmission resource pool triggers the event): 3> include a measurement reporting entry within the VarMeasReportList for this measId; 3> set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0; 3> include the PCellconcerned cell(s) andor the concerned transmission resource pool(s) in the poolsTriggeredList defined within the VarMeasReportList for this measId; 3> initiate the measurement reporting procedure, as specified in 5.5.5; | CR |
| Z.009 | 5.5.4.1 General | For CBR measurement report on Tx transmission pools, there is no need to include “the concerned cells” in the poolsTriggeredList defined within the VarMeasReportList for this measId. | 2 | Correct the following text as modified: 2> if the triggerType is set to event and if the entry condition applicable for this event, i.e. the event corresponding with the eventId of the corresponding reportConfig within VarMeasConfig, is fulfilled for one or more applicable transmission resource pools for all measurements taken during timeToTrigger defined for this event within the VarMeasConfig, while the VarMeasReportList does not include an measurement reporting entry for this measId (a first transmission resource pool triggers the event): 3> include a measurement reporting entry within the VarMeasReportList for this measId; 3> set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0; 3> include the concerned cell(s) or the concerned transmission resource pool(s) in the poolsTriggeredList defined within the VarMeasReportList for this measId; Coordinator (Ericsson): Same as C.002, E.105, N.031. | CR |
| C.002 | 5.5.4.1 General | The following words in current CR is ambigous. “include the concerned cell(s) or the concerned transmission resource pool(s) in the poolsTriggeredList defined within the VarMeasReportList for this measId;” | 2 | Disucussion the reason why cell(s) need to be reported when the event defined for CBR measurement is fulfilled, since the measurement object is resource pools. The “or” in the sentence would lead to a misunderstanding that it can only report cell(s) but not report resources pools. 5.5.4.1 General <text removed here> 2> if the triggerType is set to event and if the entry condition applicable for this event, i.e. the event corresponding with the eventId of the corresponding reportConfig within VarMeasConfig, is fulfilled for one or more applicable transmission resource pools for all measurements taken during timeToTrigger defined for this event within the VarMeasConfig, while the VarMeasReportList does not include an measurement reporting entry for this measId (a first transmission resource pool triggers the event): 3> include a measurement reporting entry within the VarMeasReportList for this measId; 3> set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0; 3> include the concerned cell(s) or the concerned transmission resource pool(s) in the poolsTriggeredList defined within the VarMeasReportList for this measId; 3> initiate the measurement reporting procedure, as specified in 5.5.5; Coordinator (Ericsson): Same as Z.009, E.105, N.031. | CR/ TDoc XX |
| E.105 | 5.5.4.1 General | The inclusion of “concerned cells” in the below text is incorrect as this text deals with transmission pools and V2X. 3> include the concerned cell(s) or the concerned transmission resource pool(s) in the poolsTriggeredList defined within the VarMeasReportList for this measId; | 2 | Remove the “concerned cells” 3> include the concerned cell(s) or the concerned transmission resource pool(s) in the poolsTriggeredList defined within the VarMeasReportList for this measId; Coordinator (Ericsson): Same as Z.009, C.002, N.031. | CR |
| N.031 | 5.5.4.1 General | Why are the cells mentioned for transmission pool measurements reporting? include the concerned cell(s) or the concerned transmission resource pool(s) in the poolsTriggeredList defined within the VarMeasReportList for this measId; | 2 | Remove the highlighted part Coordinator (Ericsson): Same as Z.009, C.002, E.105. | CR |
| E.104 | 5.5.4.1 General | While the highlighted text is technically correct, it fits the overall structure better to be added at another place in the procedure. 1> for each measId included in the measIdList within VarMeasConfig: 2> if the corresponding measObject concerns SL: 3> consider the transmission resource pools indicated by the tx-ResourcePoolToAddList defined within the VarMeasConfig for this measId to be applicable; 2> else if the corresponding reportConfig includes a purpose set to reportStrongestCellsForSON: | 2 | Consider the following changes: 1> for each measId included in the measIdList within VarMeasConfig: 2> if the corresponding measObject concerns SL: 3> consider the transmission resource pools indicated by the tx-ResourcePoolToAddList defined within the VarMeasConfig for this measId to be applicable; 2> else if the corresponding reportConfig includes a purpose set to reportStrongestCellsForSON: <SNIP to further down in the same procedure> 3> else if the corresponding measObject concerns WLAN: 4> consider a WLAN on the associated set of frequencies, as indicated by carrierFreq or on all WLAN frequencies when carrierFreq is not present, to be applicable if the WLAN matches all WLAN identifiers of at least one entry within wlan-Id-List for this measId; 3> else if the corresponding measObject concerns SL: 4> consider the transmission resource pools indicated by the tx-ResourcePoolToAddList defined within the VarMeasConfig for this measId to be applicable; | CR |
| E.106 | 5.5.4.1 General | While the highlighted text is correct, it fits the overall structure better if it is added in a different place. 4> initiate the measurement reporting procedure, as specified in 5.5.5, when it has determined the strongest cells on the associated frequency; 2> else if the purpose is set to sidelink and if a (first) CBR measurement result is available: 3> include a measurement reporting entry within the VarMeasReportList for this measId; 3> set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0; 3> initiate the measurement reporting procedure as specified in 5.5.5 immediately; 2> upon expiry of the periodical reporting timer for this measId: | 2 | Consider the following changes: 2> else if the purpose is included and set to reportStrongestCells or to reportStrongestCellsForSON or to reportLocation or to sidelink and if a (first) measurement result is available: 3> include a measurement reporting entry within the VarMeasReportList for this measId; 3> set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0; 3> if the purpose is set to reportStrongestCells and reportStrongestCSI-RSs is not included: 4> if the triggerType is set to periodical and the corresponding reportConfig includes the ul-DelayConfig: 5> initiate the measurement reporting procedure, as specified in 5.5.5, immediately after a first measurement result is provided by lower layers; 4> else if the corresponding measurement object concerns WLAN: 5> initiate the measurement reporting procedure, as specified in 5.5.5, immediately after the quantity to be reported becomes available for the PCell and for the applicable WLAN(s); 4> else if the reportAmount exceeds 1: 5> initiate the measurement reporting procedure, as specified in 5.5.5, immediately after the quantity to be reported becomes available for the PCell; 4> else (i.e. the reportAmount is equal to 1): 5> initiate the measurement reporting procedure, as specified in 5.5.5, immediately after the quantity to be reported becomes available for the PCell and for the strongest cell among the applicable cells, or becomes available for the pair of PCell and the PSCell in case of SSTD measurements; 3> else if the purpose is set to reportLocation: 4> initiate the measurement reporting procedure, as specified in 5.5.5, immediately after both the quantity to be reported for the PCell and the location information become available; 3> else if the purpose is set to sidelink: 4> initiate the measurement reporting procedure as specified in 5.5.5 immediately; 3> else: 4> initiate the measurement reporting procedure, as specified in 5.5.5, when it has determined the strongest cells on the associated frequency; 2> else if the purpose is set to sidelink and if a (first) CBR measurement result is available: 3> include a measurement reporting entry within the VarMeasReportList for this measId; 3> set the numberOfReportsSent defined within the VarMeasReportList for this measId to 0; 3> initiate the measurement reporting procedure as specified in 5.5.5 immediately; | CR |
| N.033 | 5.5.4.14 and 5.5.4.15 (V1/V2 events) | “Ms is expressed in percent.” while SL-CBR values are defined in 0.01. Probably should be aligned | 2 | Change values of SL-CBR field (in ASN.1) from 0.01 multiples to percents. | CR |
| L.011 | 5.5.5 | For CBR reporting, only results for PCell is included. So, it is necessary to add ‘ except if purpose for the reportConfig associated with the measId that triggered the measurement reporting is set to sidelink’ | 2 | 1> set the measResultPCell to include the quantities of the PCell; 1> set the measResultServFreqList to include for each SCell that is configured, if any, within measResultSCell the quantities of the concerned SCell, if available according to performance requirements in [16] except if purpose for the reportConfig associated with the measId that triggered the measurement reporting is set to sidelink; 1> if the reportConfig associated with the measId that triggered the measurement reporting includes reportAddNeighMeas: | CR |
| 5.6 Other |  |  |  |  |  |
| E.061 | 5.6.3.3 Reception of the UECapabilityEnquiry by the UE | Category M2 is missing. | 1 | 3> if the UE is a category 0, M1 or M1M2 UE, or supports any UE capability information in ue-RadioPagingInfo, according to TS 36.306 [5]: 4> include ue-RadioPagingInfo and set the fields according to TS 36.306 [5]; | CR |
| E.127 | 5.6.10 UE Assistance Information | Collision in 5.6.10.3 | 3 | There seem to be a CR-collision in 5.6.10.3. One CR adds " for power preference indications " in the first line. And another CR adds V2X-indications in this section which. But the result is that the V2X indications are " for power preference indications " which is not the intention. This needs to be sorted out. Further, in this section it is also added that the UE indicates its preferred bandwidth but it seems not possible to not have any preference w.r.t. e.g. UL while having a preference w.r.t. DL. This probably is not the intention. | CR/ TDoc WI (Ericsson) |
| L.012 | 5.6.10.3 | This reporting is only related to V2X. So, it is necessary to add ‘for V2X’ | 2 | 1> if configured to provide SPS assistance information: 2> if there is any traffic for V2X sidelink communication which needs to report SPS assistance information: 3> include trafficPatternInfoListSL in the UEAssistanceInformation message; 2> if there is any traffic for uplink communication for V2X which needs to report SPS assistance information: 3> include trafficPatternInfoListUL in the UEAssistanceInformation message; | CR |
| N.037 | 5.6.10.3 Actions related to transmission of UEAssistanceInformation message | Undefined Txyz is used in the procedure, in addition the procedure is defined incorrectly: 1> start timer Txyz with the timer value set to the bw-PreferenceIndicationTimer; Starting timer is defined as nested procedure for setting content of UEAssistanceInformation Related to N.036 | 2 | Start timer related procedure shoudn’t be used as nested instruction for setting content of UEAssistanceInformation message |  |
| N.037 | 5.6.18 Delay Budget Report | The purpose of this procedure is to inform E-UTRAN about the desired increment/decrement in the Uu air interface delay. “desired increment/decrement” isn’t aligned with intenton, stage 2 description and field decriptions Related to N.044 | 2 | Change “desired” to “preferred” to align with intended behaviour description in field descriptions (delayBudgetAdjustment) | CR |
| N.034 | 5.6.18.2 Initiation | “upon initiating the procedure, the UE shall”, problem seems to be that initiation is defined in subbullets. i.e. UE would never initiate procedure | 2 | Remove “upon initiating the procedure, “ | CR |
| 5.8 MBMS |  |  |  |  |  |
| 5.8a SC-PTM |  |  |  |  |  |
| H.010 | 5.8a.1.1 General | For BL UE or UE in CE, BR-BCCH is introduced which is missing in the general description. | 2 | A limited amount of SC-PTM control information is provided on the BCCH or BR-BCCH. This primarily concerns the information needed to acquire the SC-MCCH. | CR |
| L.027 | 5.8a.1.3 | For the third paragrah, it is shown as : Upon receiving a change notification, a BL UE, UE in CE or NB-IoT UE interested to receive MBMS services transmitted using SC-PTM acquires the new SC-MCCH information starting from the same subframe Dissimilarly with the second paragraph, it seems to be not clarified that what “the same subframe” in the third paragraph means. | 2 | Upon receiving a change notification, a BL UE, UE in CE or NB-IoT UE interested to receive MBMS services transmitted using SC-PTM acquires the new SC-MCCH information starting from the same first subframe of the change notification | CR |
| C.005 | 5.8a.1.3 SC-MCCH information validity and notification of changes | Similar as R13 SC-PTM mechanism, it is better to restrict that SC-MCCH change notification can only be present in subframes which schedules the first SC-MCCH in a repetition period for UE power saving. | 3 | Correct the text as follows: When the network changes (some of) the SC-MCCH information for start of new MBMS service(s) transmitted using SC-PTM, it notifies BL UEs, UEs in CE or NB-IoT UEs about the change in every PDCCH which schedules the first SC-MCCH in a repetition period in the current modification period. | CR/ TDoc XX |
| H.012 | 5.8a.1.3 SC-MCCH information validity and notification of changes | In NB-IoT and FeMTC, the UE does not acquire acquire the new SC-MCCH information in the same subframe | 3 | Can be handled via contribution at next meeting. Alternatively, change as follows: When the network changes (some of) the SC-MCCH information for start of new MBMS service(s) transmitted using SC-PTM, it notifies BL UEs, UEs in CE or NB-IoT UEs about the change in every PDCCH which schedules the SC-MCCH in the current modification period. Upon receiving a change notification, a BL UE, UE in CE or NB-IoT UE interested to receive MBMS services transmitted using SC-PTM acquires the new SC-MCCH information starting from the same subframescheduled by the PDCCH. The BL UE, UE in CE or NB-IoT UEs applies the previously acquired SC-MCCH information until the BL UE, UE in CE or NB-IoT UE acquires the new SC-MCCH information. | Tdoc WI (Huawei) |
| C.006 | 5.8a.1.3 SC-MCCH information validity and notification of changes | For a NB-IoT UE, it cannot receive SC-MCCH notification and SC-MCCH information in the same subframe. | 2 | Correct the text as follows: Upon receiving a change notification, a BL UE, or UE in CE or NB-IoT UE interested to receive MBMS services transmitted using SC-PTM acquires the new SC-MCCH information starting from the same subframe , and a NB-IoT UE interested to receive MBMS services transmitted using SC-PTM acquires the new SC-MCCH information according to the schedule information in PDCCH. | CR |
| H.011 | 5.8a.1.3 SC-MCCH information validity and notification of changes | For BL UE, UE in CE and NB-IoT UE, the ), the modification period boundaries are defined by (H-SFN * 1024 + SFN ) mod sc-mcch-ModificationPeriod = 0 | 2/3 | Change as follows (similar toSysten information in 5.2.1.3) Change of SC-MCCH information only occurs at specific radio frames, i.e. the concept of a modification period is used. Within a modification period, the same SC-MCCH information may be transmitted a number of times, as defined by its scheduling (which is based on a repetition period). The modification period boundaries are defined by SFN values for which SFN mod m= 0, where m is the number of radio frames comprising the modification period. The modification period is configured by means of SystemInformationBlockType20 (SystemInformationBlockType20-NB in NB-IoT). If H-SFN is provided in SystemInformationBlockType1-BR, modification period boundaries for BL UEs and UEs in CE are defined by SFN values for which (H-SFN * 1024 + SFN) mod m=0. The modification period boundaries for NB-IoT UEs are defined by SFN values for which (H-SFN * 1024 + SFN) mod m=0. | CR |
| H.013 | 5.8a.1.3 SC-MCCH information validity and notification of changes | The text describes that the network informs UE about change of SC-MCCH information using PDCCH . However, which information is used in the PDCCH to notify the change is not specified, | 3 | It is proposed to specify the information used in the PDCCH in a similar way to Direct Indication information in the paging case. This can be handled via contribution at the next meeting | Tdoc WI (Huawei) |
| Q.005 | 5.8a.2.1 General | This sentence could be mis-interpreted to mean procedure applies to UEs in CE or NB-IoT UE. The procedure applies to SC-PTM capable UEs that are in RRC_IDLE and to the SC-PTM capable UEs that are not BL UEs, UEs in CE or NB-IoT UEs in RRC_CONNECTED. | 1 | Propose to change the sentence as follows: The procedure applies to SC-PTM capable UEs that are in RRC_IDLE and to the SC-PTM capable UEs that are not BL UEs, UEs in CE or NB-IoT UEs in RRC_CONNECTED. This procedure also applies to SC-PTM capable UEs that are in RRC_CONNECTED except for BL UEs, UEs in CE or NB-IoT UEs. | CR |
| H.014 | 5.8a.2.3 SC-MCCH information acquisition by the UE | In NB-IoT and FeMTC, the UE does not acquire the new SC-MCCH information in the same subframe | 3 | Can be handled via contribution at next meeting. Alternatively, change as follows: A SC-PTM capable UE shall: 1> if the procedure is triggered by an SC-MCCH information change notification and the UE has no ongoing MBMS service: 2> except for BL UE, UE in CE or NB-IoT UE, start acquiring the SCPTMConfiguration message from the subframe where the change notification was received; 2> for a BL UE, UE in CE or NB-IoT UE, acquires the SCPTMConfiguration message scheduled by the PDCCH where the change notification was received; | Tdoc WI (Huawei) |
| H.015 | 5.8a.3.3 SC-MRB establishment | We normally don’t distinguish the message names in NB-ioT | 2 | Change as follows: 1> configure the physical layer in accordance with the sc-mtch-InfoList, applicable for the SC-MRB, as included in the SCPTMConfiguration message (SCPTMConfiguration-NB in NB-IoT); | CR |
| 5.9 RN procedures |  |  |  |  |  |
| 5.10 ProSe |  |  |  |  |  |
| L.017 | 5.10.12 | PCell could provide inter-frequency Rx resource pool for V2X dedicated carrier. Hence, even in the case the UE is OOC on the frequency for V2X, the UE could monitor eNB provided resources. | 2 | 1> if the conditions for sidelink operation as defined in 5.10.1d are met: 2> if in coverage on the frequency used for V2X sidelink communication, as defined in TS 36.304 [4, 11.4]:; or 2> if the frequency used to receive V2X sidelink communication is included in v2x-InterFreqInfoList in RRCConnectionReconfiguration or in v2x-InterFreqInfoList within SystemInformationBlockType21: 3> if the cell chosen for V2X sidelink communication reception broadcasts SystemInformationBlockType21 including v2x-CommRxPool included in sl-V2X-ConfigCommon; or, 3> if the UE is configured with v2x-CommRxPool in v2x-InterFreqInfoList in RRCConnectionReconfiguration or SystemInformationBlockType21 for the concerned frequency; or, 3> if the UE is configured with v2x-CommRxPool included in mobilityControlInfoV2X in RRCConnectionReconfiguration: 4> configure lower layers to monitor sidelink control information and the corresponding data using the pool of resources indicated; 2> else (i.e. out of coverage on the carrier for V2X sidelink communication): 3> configure lower layers to monitor sidelink control information and the corresponding data using the pool of resources that were preconfigured (i.e. v2x-CommRxPoolList in SL-V2X-Preconfiguration defined in 9.3); | CR |
| C.008 | 5.10.12 V2X sidelink communication monitoring | In RAN2#97, RAN2 agreed: Support inter-carrier configuration of RX resource pool for V2x sidelink communication This agreement is not involved in current spec. or CR | 2 | 5.10.12 V2X sidelink communication monitoring A UE capable of V2X sidelink communication that is configured by upper layers to receive V2X sidelink communication shall: 1> if the conditions for sidelink operation as defined in 5.10.1d are met: 2> if in coverage on the frequency used for V2X sidelink communication, as defined in TS 36.304 [4, 11.4]: 3> if the cell chosen for V2X sidelink communication reception broadcasts SystemInformationBlockType21 including v2x-CommRxPool included in sl-V2X-ConfigCommon or included in SL-InterFreqInfoV2X; or 3> if the UE is configured with v2x-CommRxPool included in mobilityControlInfoV2X or included in SL-InterFreqInfoV2X in RRCConnectionReconfiguration: 4> configure lower layers to monitor sidelink control information and the corresponding data using the pool of resources indicated on the concerned cell; 2> else (i.e. out of coverage on the carrier for V2X sidelink communication): 3> configure lower layers to monitor sidelink control information and the corresponding data using the pool of resources that were preconfigured on the concerned cell (i.e. v2x-CommRxPoolList in SL-V2X-Preconfiguration defined in 9.3); | CR |
| C.007 | 5.10.13.1 5.10.13.1a | Exceptional pool can be used when there is not a sensing result for the configured TX resource pool. In current spec and CR, this agreement is only applicable for V-UE but not P-UE performing partial sensing. | 2 | RAN2 to discuss whether this agreement is also applicable for P-UE performing partial sensing. | CR |
| L.018 | 5.10.13.1 | The transmission resources for the concerned frequency is used. | 2 | 4> else if the UE is configured with v2x-commTxPoolNormalDedicated in sl-P2X-ConfigDedicated or p2x-CommTxPoolNormal in the entry of v2x-InterFreqInfoList for the concerned frequency in RRCConnectionReconfiguration and the UE is configured to transmit P2X related V2X sidelink communication: | CR |
| L.019 | 5.10.13.1 | The marked condition is missing. | 2 | 5> else if SystemInformationBlockType21 includes v2x-CommTxPoolExceptional in sl-V2X-ConfigCommon: 6> from the moment the UE initiates connection establishment until receiving an RRCConnectionReconfiguration including sl-V2X-ConfigDedicated or sl-P2X-ConfigDedicated, until receiving an RRCConnectionRelease or an RRCConnectionReject, or until a result of sensing on the resources configured in v2x-CommTxPoolNormalCommon or v2x-CommTxPoolNormal in v2x-InterFreqInfoList for the concerned frequency, if configured, becomes available in accordance with TS 36.213 [23]: | CR |
| L.020 | 5.10.13.1 | For the transmission on inter-frequency resource, the UE is required to perform sensing on the resources. | 2 | The UE capable of non-P2X related V2X sidelink communication that is configured by upper layers to transmit V2X sidelink communication shall perform sensing on all pools of resources which may be used for transmission of the sidelink control information and the corresponding data. The pools of resources are indicated by SL-V2X-Preconfiguration, v2x-CommTxPoolNormalCommon, or v2x-commTxPoolNormalDedicated in sl-V2X-ConfigDedicated, v2x-CommTxPoolNormal in v2x-InterFreqInfoList for the concerned frequency as configured above. | CR |
| N.040 | 5.10.13.1 | 6> select a resource pool from p2x-CommTxPoolNormalCommon or p2x-CommTxPoolNormal in v2x-InterFreqInfoList for the concerned frequency according to 5.10.13.2 by ignoring zoneConfig in SystemInformationBlockType21; | 2 | There may be zone configs directly in SIB21 and in v2x-InterFreqInfoList, so it might be good to clarify that. Also word “by” does not seem to be approapriate here. Possible change: “,but ignoring any zoneConfig provided in SIB21” | CR |
| E.009 | 5.10.13.1 Transmission of V2X sidelink communication | The procedure has an informative note with a formulation “normal or exceptional cases” as high-lighted below. What is the purpose of that formulation? Shouldn’t that always be true because exceptional sounds like the opposite of normal? NOTE 1: If there are multiple frequencies for which normal or exceptional pools are configured, it is up to UE implementation which frequency is selected for V2X sidelink communication transmission in normal or exceptional cases. | 2 | It needs to be confirmed whether there are other cases than normal and exceptional cases, e.g. unexceptional abnormal cases. If not, the formulation can be removed. NOTE 1: If there are multiple frequencies for which normal or exceptional pools are configured, it is up to UE implementation which frequency is selected for V2X sidelink communication transmission in normal or exceptional cases. | CR/ TBC |
| H.016 | 5.10.13.1 Transmission of V2X sidelink communication | The conditions for the UE to use of exceptional pool and those for the UE to use P2X common pools above should not hold an “if…else if …” relation. Instead, they should be independent with each other and thus be put in two parallel “if”. | 1 | Remove the word “else” in the following text. 5> else if SystemInformationBlockType21 includes v2x-CommTxPoolExceptional in sl-V2X-ConfigCommon: 6> from the moment the UE initiates connection establishment until receiving an RRCConnectionReconfiguration including sl-V2X-ConfigDedicated or sl-P2X-ConfigDedicated, until receiving an RRCConnectionRelease or an RRCConnectionReject or until a result of sensing on the resources configured in v2x-CommTxPoolNormalCommon, if configured, becomes available in accordance with TS 36.213 [23]; | CR |
| L.021 | 5.10.13.1a | Editorial correction | 1 | 1> if partialSensing is not included and randomSelection is included in resourceSelectionConfigP2X of the pool selected:; or 1> if both partialSensing and randomSelection is included in resourceSelectionConfigP2X of the pool selected, and the UE selects to use random selection: | CR |
| L.022 | 5.10.13.2 | Some pool information is missing. | 2 | The UE shall select a pool of resources which includes a zoneID equals to the Zone_id calculated according to above mentioned formulae and indicated by v2x-CommTxPoolNormalDedicated, v2x-CommTxPoolNormalCommon, v2x-CommTxPoolNormal in v2x-InterFreqInfoList or p2x-CommTxPoolNormal in v2x-InterFreqInfoList in RRCConnectionReconfiguration, or v2x-CommTxPoolList according to 5.10.13.1. | CR |
| H.064 | 5.10.2.1 General | As specified in the message definition and the procedure of SidelinkUEInformation, one purpose to transmit SidelinkUEincformation is to report the synchronization reference used by the UE for V2X sidelink communication. This purpose is currently missing in subclause 5.10.2.1. | 2 | Add the following texts in subclause 5.10.2.1. The purpose of this procedure is to inform E-UTRAN that the UE is interested or no longer interested to receive sidelink communication or discovery, to receive V2X sidelink communication, as well as to request assignment or release of transmission resources for sidelink communication or discovery announcements or V2X sidelink communication or sidelink discovery gaps and, to report parameters related to sidelink discovery from system information of inter-frequency/PLMN cells. and to report the synchronization reference used by the UE for V2X sidelink communication. | CR |
| L.013 | 5.10.7.2 | P-UE does not perform sync transmission. | 2 | A UE capable of V2X sidelink communication and SLSS/PSBCH transmission shall, when transmitting non-P2X related V2X sidelink communication in accordance with 5.10.13, and if the conditions for V2X sidelink communication operation as defined in 5.10.1d are met and when the following conditions are met: | CR |
| L.014 | 5.10.7.3 | Wrong field name. Added ‘else’. Otherwise, for V2X sidelink communication, the UE selects twice. | 2 | 2> if the UE has a selected SyncRef UE and inCoverage in the MasterInformationBlock-SL message received from this UE is set to FALSE while the SLSS from this UE is part of the set defined for out of coverage, see TS 36.211 [21]: 3> select the same SLSSID as the SLSSID of the selected SyncRef UE; 3> select the subframe in which to transmit the SLSS according to the syncOffsetIndicator1 or syncOffsetIndicator2 included in the preconfigured sidelink parameters (i.e. preconfigSync in SL-Preconfiguration or v2x-CommPreconfigSync in SL-V2X-Preconfiguration defined in 9.3) corresponding to the frequency used for V2X sidelink communication, such that the subframe timing is different from the SLSS of the selected SyncRef UE; 2> else if the UE has a selected SyncRef UE and the SLSS from this UE was transmitted on the subframe indicated by syncOffsetIndicator3 that is included in the syncOffsetIndicators in SL-V2X-Preconfiguration, and is corresponding to the frequency used for V2X sidelink communicaton: 3> select SLSSID 169; 3> select the subframe indicated by syncOffsetIndicator2; 2> else if the UE has a selected SyncRef UE: 3> select the SLSSID from the set defined for out of coverage having an index that is 168 more than the index of the SLSSID of the selected SyncRef UE, see TS 36.211 [21]; 3> select the subframe in which to transmit the SLSS according to syncOffsetIndicator1 or syncOffsetIndicator2 included in the preconfigured sidelink parameters (i.e. preconfigSync in SL-Preconfiguration or v2x-CommPreconfigSync in SL-V2X-Preconfiguration defined in 9.3), such that the subframe timing is different from the SLSS of the selected SyncRef UE; 2> else (i.e. no SyncRef UE selected): 3> if triggered by V2X sidelink communication, randomly select, using a uniform distribution, an SLSSID from the set of sequences defined for out of coverage except SLSSID 168 and 169, see TS 36.211 [21]; 3> else, randomly select, using a uniform distribution, an SLSSID from the set of sequences defined for out of coverage, see TS 36.211 [21]; 3> select the subframe in which to transmit the SLSS according to the syncOffsetIndicator1 or syncOffsetIndicator2 (arbitrary selection between these) included in the preconfigured sidelink parameters (i.e. preconfigSync in SL-Preconfiguration or v2x-CommPreconfigSync in SL-V2X-Preconfiguration defined in 9.3); | CR |
| z.014 | 5.10.7.3 | 1) “preconfigSync” is only included in “SL-Preconfiguration”, while it is “v2x-CommPreconfigSync” in “SL-V2X-Preconfiguration”. 2) For the bullet “1> else:”, it includes sidelink communication and V2X sidelink communication, while the highlighted “corresponding to the frequency for V2X sidelink communication” in the second bullet 3> limits to V2X sidelink communication. The highlighted words should be removed. 1> else: 2> select the synchronisation reference UE (i.e. SyncRef UE) as defined in 5.10.8; 2> if the UE has a selected SyncRef UE and inCoverage in the MasterInformationBlock-SL message received from this UE is set to TRUE; or 2> if the UE has a selected SyncRef UE and inCoverage in the MasterInformationBlock-SL message received from this UE is set to FALSE while the SLSS from this UE is part of the set defined for out of coverage, see TS 36.211 [21]: 3> select the same SLSSID as the SLSSID of the selected SyncRef UE; 3> select the subframe in which to transmit the SLSS according to the syncOffsetIndicator1 or syncOffsetIndicator2 included in the preconfigured sidelink parameters (i.e. preconfigSync in SL-Preconfiguration or in SL-V2X-Preconfiguration defined in 9.3) corresponding to the frequency used for V2X sidelink communication, such that the subframe timing is different from the SLSS of the selected SyncRef UE; 2> else if the UE has a selected SyncRef UE and the SLSS from this UE was transmitted on the subframe indicated by syncOffsetIndicator3 that is included in the syncOffsetIndicators in SL-V2X-Preconfiguration, and is corresponding to the frequency used for V2X sidelink communicaton: 3> select SLSSID 169; 3> select the subframe indicated by syncOffsetIndicator2; 2> else if the UE has a selected SyncRef UE: 3> if triggered by V2X sidelink communication, randomly select, using a uniform distribution, an SLSSID from the set of sequences defined for out of coverage except SLSSID 168 and 169, see TS 36.211 [21]; 3> select the SLSSID from the set defined for out of coverage having an index that is 168 more than the index of the SLSSID of the selected SyncRef UE, see TS 36.211 [21]; 3> select the subframe in which to transmit the SLSS according to syncOffsetIndicator1 or syncOffsetIndicator2 included in the preconfigured sidelink parameters (i.e. preconfigSync in SL-Preconfiguration or in SL-V2X-Preconfiguration defined in 9.3), such that the subframe timing is different from the SLSS of the selected SyncRef UE; 2> else (i.e. no SyncRef UE selected): 3> randomly select, using a uniform distribution, an SLSSID from the set of sequences defined for out of coverage, see TS 36.211 [21]; 3> select the subframe in which to transmit the SLSS according to the syncOffsetIndicator1 or syncOffsetIndicator2 (arbitrary selection between these) included in the preconfigured sidelink parameters (i.e. preconfigSync in SL-Preconfiguration or in SL-V2X-Preconfiguration defined in 9.3); | 2 | Corrections as below: 1> else: 2> select the synchronisation reference UE (i.e. SyncRef UE) as defined in 5.10.8; 2> if the UE has a selected SyncRef UE and inCoverage in the MasterInformationBlock-SL message received from this UE is set to TRUE; or 2> if the UE has a selected SyncRef UE and inCoverage in the MasterInformationBlock-SL message received from this UE is set to FALSE while the SLSS from this UE is part of the set defined for out of coverage, see TS 36.211 [21]: 3> select the same SLSSID as the SLSSID of the selected SyncRef UE; 3> select the subframe in which to transmit the SLSS according to the syncOffsetIndicator1 or syncOffsetIndicator2 included in the preconfigured sidelink parameters (i.e. preconfigSync in SL-Preconfiguration or v2x-CommPreconfigSync in SL-V2X-Preconfiguration defined in 9.3) corresponding to the frequency used for V2X sidelink communication, such that the subframe timing is different from the SLSS of the selected SyncRef UE; 2> else if the UE has a selected SyncRef UE and the SLSS from this UE was transmitted on the subframe indicated by syncOffsetIndicator3 that is included in the syncOffsetIndicators in SL-V2X-Preconfiguration, and is corresponding to the frequency used for V2X sidelink communicaton: 3> select SLSSID 169; 3> select the subframe indicated by syncOffsetIndicator2; 2> else if the UE has a selected SyncRef UE: 3> if triggered by V2X sidelink communication, randomly select, using a uniform distribution, an SLSSID from the set of sequences defined for out of coverage except SLSSID 168 and 169, see TS 36.211 [21]; 3> select the SLSSID from the set defined for out of coverage having an index that is 168 more than the index of the SLSSID of the selected SyncRef UE, see TS 36.211 [21]; 3> select the subframe in which to transmit the SLSS according to syncOffsetIndicator1 or syncOffsetIndicator2 included in the preconfigured sidelink parameters (i.e. preconfigSync in SL-Preconfiguration or v2x-CommPreconfigSync in SL-V2X-Preconfiguration defined in 9.3), such that the subframe timing is different from the SLSS of the selected SyncRef UE; 2> else (i.e. no SyncRef UE selected): 3> randomly select, using a uniform distribution, an SLSSID from the set of sequences defined for out of coverage, see TS 36.211 [21]; 3> select the subframe in which to transmit the SLSS according to the syncOffsetIndicator1 or syncOffsetIndicator2 (arbitrary selection between these) included in the preconfigured sidelink parameters (i.e. preconfigSync in SL-Preconfiguration or v2x-CommPreconfigSync in SL-V2X-Preconfiguration defined in 9.3); | CR |
| z.015 | 5.10.7.4 | “preconfigGeneral” is only included in “SL-Preconfiguration”, while it is “v2x-CommPreconfigGeneral” in “SL-V2X-Preconfiguration”. 1> else if out of coverage on the frequency used for V2X sidelink communication as defined in TS 36.304 [4, 11.4]; and the UE selects GNSS timing as the synchronization reference source and syncOffsetIndicator3 is not included in SL-V2X-Preconfiguration: 3> set inCoverage to TRUE; 3> set sl-Bandwidth, subframeAssignmentSL and reserved to the value of the corresponding field included in the preconfigured sidelink parameters (i.e. preconfigGeneral in SL-V2X-Preconfiguration defined in 9.3); 1> else if the UE has a selected SyncRef UE (as defined in 5.10.8): 2> set inCoverage to FALSE; 2> set sl-Bandwidth, subframeAssignmentSL and reserved to the value of the corresponding field included in the received MasterInformationBlock-SL; 1> else: 2> set inCoverage to FALSE; 2> set sl-Bandwidth, subframeAssignmentSL and reserved to the value of the corresponding field included in the preconfigured sidelink parameters (i.e. preconfigGeneral in SL-Preconfiguration or in SL-V2X-Preconfiguration defined in 9.3); | 2 | Corrections as below: 1> else if out of coverage on the frequency used for V2X sidelink communication as defined in TS 36.304 [4, 11.4]; and the UE selects GNSS timing as the synchronization reference source and syncOffsetIndicator3 is not included in SL-V2X-Preconfiguration: 3> set inCoverage to TRUE; 3> set sl-Bandwidth, subframeAssignmentSL and reserved to the value of the corresponding field included in the preconfigured sidelink parameters (i.e. v2x-CommPreconfigGeneralpreconfigGeneral in SL-V2X-Preconfiguration defined in 9.3); 1> else if the UE has a selected SyncRef UE (as defined in 5.10.8): 2> set inCoverage to FALSE; 2> set sl-Bandwidth, subframeAssignmentSL and reserved to the value of the corresponding field included in the received MasterInformationBlock-SL; 1> else: 2> set inCoverage to FALSE; 2> set sl-Bandwidth, subframeAssignmentSL and reserved to the value of the corresponding field included in the preconfigured sidelink parameters (i.e. preconfigGeneral in SL-Preconfiguration or v2x-CommPreconfigGeneral in SL-V2X-Preconfiguration defined in 9.3); | CR |
| E.008 | 5.10.7.4 Transmission of MasterInformationBlock-SL message | The procedure has an “elseif” condition with a straight jump from level 1 to level 3. Is there something from level 2 missing or is this only an indentation (or formatting) errors? | 2 | It needs to be confirmed whether something is missing here or if the indentation level should be corrected from 3 to 2 as shown below; 1> else if out of coverage on the frequency used for V2X sidelink communication as defined in TS 36.304 [4, 11.4]; and the UE selects GNSS timing as the synchronization reference source and syncOffsetIndicator3 is not included in SL-V2X-Preconfiguration: 2>3> set inCoverage to TRUE; 2>3> set sl-Bandwidth, subframeAssignmentSL and reserved to the value of the corresponding field included in the preconfigured sidelink parameters (i.e. preconfigGeneral in SL-V2X-Preconfiguration defined in 9.3); | CR/ TBC |
| Z.013 | 5.10.7.4 Transmission of MasterInformationBlock-SL message | In the following procedure, for the first 1> bullet, shouldn’t UE use the sidelink parameters in SIB21 of the serving cell/Pcell? Specifically, for sl-Bandwidth, which is carried in v2x-InterFreqInfoList in SIB21, it’s natural to use it in SIB21 instead of SL-V2X-Preconfiguration. For subframeAssignmentSL and reserved, which are not carried in v2x-InterFreqInfoList in SIB21, two ways to tackle it as suggested in the right column. By the way, the bullet number 3 should be 2. The following text is copied from specification. 1> else if out of coverage on the frequency used for V2X sidelink communication as defined in TS 36.304 [4, 11.4]; and the concerned frequency is included in v2x-InterFreqInfoList in RRCConnectionReconfiguration or in v2x-InterFreqInfoList within SystemInformationBlockType21 of the serving cell/ PCell; or 1> else if out of coverage on the frequency used for V2X sidelink communication as defined in TS 36.304 [4, 11.4]; and the UE selects GNSS timing as the synchronization reference source and syncOffsetIndicator3 is not included in SL-V2X-Preconfiguration: 3> set inCoverage to TRUE; 3> set sl-Bandwidth, subframeAssignmentSL and reserved to the value of the corresponding field included in the preconfigured sidelink parameters (i.e. preconfigGeneral in SL-V2X-Preconfiguration defined in 9.3); | 3 | For sl-Bandwidth, set it the the value in v2x-InterFreqInfoList in SIB21. For subframeAssignmentSL and reserved, one way to set them to the values in SL-V2X-Preconfiguration. The drawback is that the preconfigured parameters may be different from the values configured by the neighbor carrier. The alternative way preferred by ZTE is to have v2x-InterFreqInfoList in SIB21also carry subframeAssignmentSL and reserved. The bullets number 3 should be 2. |  |
| L.015 | 5.10.8.2 | Editorial correction. | 1 | 2> if typeTxSync is configured for the concerned frequency and set to enb: | CR |
| L.016 | 5.10.8.2 | For clarifying the condition, added condition. | 2 | - else, if triggered by V2X sidelink communication, and out of coverage on the frequeny for V2X sidelink communication, and for the frequency used for V2X sidelink communication, if syncPriority in SL-V2X-Preconfiguration is set to gnss and GNSS is reliable in accordance with TS 36.101 [42] and TS 36.133 [16]: | CR |
| E.109 | 5.10.9.1 General | In the text below it is clarified that for sidelink discovery and sidelink communication syncOffsetIndicator-r12 (i.e. with suffix r12) is used. No such clarification is done for V2X sidelink communication. The MIB-SL for sidelink discovery and sidelink communication uses a fixed schedule with a periodicity of 40 ms without repetitions. In particular, the MIB-SL is scheduled in subframes indicated by syncOffsetIndicator-r12 i.e. for which (10*DFN + subframe number) mod 40 = syncOffsetIndicator-r12. The MIB-SL for V2X sidelink communication uses a fixed schedule with a periodicity of 160 ms without repetitions. In particular, the MIB-SL is scheduled in subframes indicated by syncOffsetIndicator i.e. for which (10*DFN + subframe number) mod 160 = syncOffsetIndicator. | 3 | Determine what syncOffsetIndicator to use for V2X (e.g.v14xy syncOffsetIndicator-v14xy?) | CR/ TDoc WI (Ericsson) |
| 6.2.1 General message structure |  |  |  |  |  |
| H.017 | 6.2.1 SC-MCCH-Message | There is no spare in the new MessageClassExtension | 2 | Discuss and conclude whether we should allow for one spare | TBC |
| 6.2.2 Message definitions |  |  |  |  |  |
| S.008 | 6.2.2 SystemInformationBlockType1-MBMS | There are still quite some fields/ IE names missing –r14 suffix e.g. PLMN-IdentityList-MBMS, SchedulingInfoList-MBMS, PLMN-IdentityInfo/List-MBMS, SchedulingInfo(List)-MBMS, SIB-Type-MBMS, NonMBSFN-SubframeConfig | 1 | Update according to conventions | CR |
| N.045 | 6.2.2 delayBudgetAdjustment-r14 | The parameter can take values for two differen reasons (type 1 -DRX adjustments, type 2 – physical coverage enhancemenst), hence the split in values. The arrangements of values isn’t in consequitive sequence. On the other hand, the values defined for one type won’t be preculded to use for type 2. | 2 | Try to make ASN.1 design more clear or clarify why values from one subset can not be re-used for other purpose. Alteratively, define common set in order? |  |
| S.010 | 6.2.2 DelayBudgetReport | Was there a particular motivation for introducing a separate message rather than re-using UEAssistanceInformation i.e. what was the fundamental difference compared to BW preference and SPS assistance (that are handled by UEAssistanceInformation). Note that configuration is also not separate (but added to otherConfig) | 2/ 3 | Would require discussion (related to S.004) | TDoc WI |
| N.047 | 6.2.2 DelayBudgetReport:: delayBudgetAdjustment-r14 | Comments inside message are confusing | 2 | Remove comments; If comments are needed, they would be better as NOTE or as guidelines inside field description. | CR |
| E.010 | 6.2.2 DelayBudgetReport | delayBudgetAdjustment-r14 field has comments in the middle of a list of enumerated values. Comment lines have two issues; - Depending on where they are placed they may create issues in the long-term if a new line character is accidentally dropped out. - It has always been unclear whether comment lines in ASN.1 are normative or informative text. | 2 | Consider removing the comment lines in ASN.1 and moving the comments text in the field description, e.g. delayBudgetAdjustment-r14 ENUMERATED { --values mainly for CDRX cycle length msMinus1280, msMinus640, msMinus320, msMinus160, msMinus80, msMinus60, msMinus40, msMinus20, ms20, ms40, ms60, ms80, ms160, ms320, ms640, ms1280, ms0, --value for both CDRX cycle length and coverage enh. --values mainly for PHY coverage enhancement msMinus192, msMinus168,msMinus144, msMinus120, msMinus96, msMinus72, msMinus48 , msMinus24, delayBudgetAdjustment Indicates the preferred amount of increment/decrement with respect to the current configuration. Value in number of milliseconds. Value ms40 corresponds to 40 milliseconds, msMinus40 corresponds to -40 milliseconds and so on. Values from msMinus1280 to 0ms are mainly for CDRX cycle length. Values from msMinus192 to msMinus24 are for both CDRX cycle length and physical layer coverage enhancements. | CR |
| M.016 | 6.2.2 DelayBudgetReport | Value name and field description | 2 | DelayBudgetReport-r14-IEs ::= SEQUENCE { ueReportCause-r14 ENUMERATED {type1, type2}, delayBudgetAdjustment-r14 ENUMERATED { --values mainly for CDRX cycle length msMinus1280, msMinus640, msMinus320, msMinus160, msMinus80, msMinus60, msMinus40, msMinus20, ms20, msPlus40, msPlus60, msPlus80, msPlus160, msPlus320, msPlus640, msPlus1280, ms0, --value for both CDRX cycle length and coverage enh. --values mainly for PHY coverage enhancement msMinus192, msMinus168,msMinus144, msMinus120, msMinus96, msMinus72, msMinus48 , msMinus24, ms24, ms48, ms72, msPlus96, msPlus120, msPlus144, msPlus168, msPlus192 }, delayBudgetAdjustment Indicates the preferred amount of increment/decrement with respect to the current configuration. Value in number of milliseconds. Value msPlus40 corresponds to +40 milliseconds, msMinus40 corresponds to -40 milliseconds and so on. | CR |
| N.044 | 6.2.2 DelayBudgetReport | The DelayBudgetReport message is used to inform E-UTRAN about the desired increment/decrement in the Uu air interface delay. “desired increment/decrement” isn’t aligned with intenton, stage 2 description and field decriptions Related to N.037 | 2 | Change “desired” to “preferred” to align with intended behaviour description in field descriptions (delayBudgetAdjustment) | CR |
| N.050 | 6.2.2 DelayBudgetReport | Separate to two report types | 3 | Discuss whether it makes sense to have two totally separate reports under a CHOICE, to make it clearer what the UE is reporting. |  |
| N.049 | 6.2.2 DelayBudgetReport:: delayBudgetAdjustment-r14 | The values are given in a strange order | 2 | Discuss whether to order the values more appropriately. |  |
| N.048 | 6.2.2 DelayBudgetReport::ueReportCause-r14 | Type names are confusing | 2 | Use “cDRX” for type1 and “coverageEnh” for type2. | CR |
| S.011 | 6.2.2 DelayBudgetReport-r14-IEs | Comments about for which case the values apply seem misplaced | 1/ 2 | Move following comment to before ms0? --value for both CDRX cycle length and coverage enh | CR |
| E.128 | 6.2.2 MasterInformationBlock-MBMS | Missing description in field description of what value 0 means for additionalNonMBSFNSubframes. | 2 | Should describe that value 0 means that no additional MBSFN subframes are configured. additionalNonMBSFNSubframes Configures additional non-MBSFN subframes where SystemInformationBlockType1-MBMS and SystemInformation-MBMS may be transmitted. Value 0, 1, 2, 3 mean zero, one, two, three additional non-MBSFN subframes are configured after each subframe which has PBCH. | CR |
| N.198 | 6.2.2 MasterInformationBlock-MBMS | A new dl-Bandwidth-r14 is introduced in Rel-14 in MIB-MBMS. There are some references to dl-Bandwidth in the spec that is unclear whether it refers to the newly introduced parameter or the existing parameter in existing MasterInformationBlock IE. | 2 | Clarify which BW is used in each instance Ericsson: it migth be best to use another parameter name in MIB-MBMS and in MBMS related places mention both. |  |
| N.052 | 6.2.2 MasterInformationBlock-MBMS::systemFrameNumber | The SFN length for MBMS dedicated carrier is unclear | 2 | The MBMS-carrier SFN still has 10 bits, and the current field only indicates how to derive 8 of them. Since the PBCH TTI is 160ms, the UE should derive 4 LSBs from PBCH decoding instead of 2 LSBs only. This is as per RAN1 LS R1-1613758: “The MIB is transmitted in SFN mod 4 = 0 and can change only in SFN mod 16 = 0 and contains systemFrameNumber equal to the 6 most significant bits of the SFN” Therefore, it is proposed to adopt the following text, which also clarifies that this is the SFN of an MBMS-dedicated cell: “systemFrameNumber Defines the 6 most significant bits of the SFN of the MBMS-dedicated cell. As indicated in TS 36.211 [21, 6.6.1], the 4 least significant bits of the SFN are acquired implicitly in the P-BCH decoding, i.e. timing of 160ms P-BCH TTI indicates 4 least significant bits (within 40ms P-BCH TTI, the first radio frame: 00, the fourth radio frame: 01, the eigth radio frame: 10, the last radio frame: 11).” See also TH16 | CR |
| N.057 | 6.2.2 PerCC-GapIndication | Field description for gapIndication is not clear and could be improved for better readability. Field description of servCellId in the same IE is also terrible and needs improvement. | 2 | As example, change… From: “Value gapIndication of gap cooresponding to gap is needed for that servCellId” To: “Value of gap means gap is needed for that servCellId” From: “value gapIndication of nogap-noNcsg cooresponding to no gap and no ncsg is needed for that servCellId” To: “value of nogap-noNcsg means no gap and no ncsg is needed for that servCellId” | CR |
| E.130 | 6.2.2 RRCConnectionReconfiguration | Relation between fields sl-V2X-ConfigDedicated-r14 and sl-P2X-ConfigDedicated-r14. | 3 | These fields provide the exact same information but it is unclear how they relate to each other. Can the eNB configure only one at a time? If so, why do we have two seperate fields instead of one field with a more generic name? Or if the eNB can configure both, how does the UE behave (e.g. if periodic-BSR-TimerSL has one value in the V2X-field but another value in the P2X-field)? | CR/ TDocs WI (Ericsson) |
| E.131 | 6.2.2 RRCConnectionReconfiguration | Ambigious field description for srs-SwitchFromServCellIndex-r14. It is unclear if the UL transmissions will (for sure) be interrupted or may be interrupted in the following: srs-SwitchFromServCellIndex-r14 Indicates the serving cell whose UL transmission is interrupted during SRS transmission on a PUSCH-less cell. During SRS transmission on a PUSCH-less cell, the UE may temporarily suspend the UL transmission on a serving cell with PUSCH in the same CG to allow the PUSCH-less cell to transmit SRS. The PUSCH-less cell is always a TDD cell but the serving cell with PUSCH may be either a FDD or TDD cell. | 2 | We assume the UL transmissions "may" be interrupted and hence we should clarify as follows: srs-SwitchFromServCellIndex-r14 Indicates the serving cell whose UL transmission may beis interrupted during SRS transmission on a PUSCH-less cell. During SRS transmission on a PUSCH-less cell, the UE may temporarily suspend the UL transmission on a serving cell with PUSCH in the same CG to allow the PUSCH-less cell to transmit SRS. The PUSCH-less cell is always a TDD cell but the serving cell with PUSCH may be either a FDD or TDD cell. | CR |
| L.033 | 6.2.2 RRCConnectionReconfiguration | Rearrange the fields in field description in alphabetical order | 1 | Switch between sl-V2X-ConfigDedicated and sl-P2X-ConfigDedicated | CR |
| N.200 | 6.2.2 RRCConnectionReconfiguration | Field description for perCC-GapIndicationRequest says only perCC-ListGapIndication in included but the procedure text in 5.3.5.3 and 5.3.5.4 says numFreqEffective is also included | 2 | Clarify procedural text Coordinator (Ericsson): A clarification proposal would make it easier to capture this in the CR. |  |
| N.201 | 6.2.2 RRCConnectionReconfiguration | Field description for systemInformationBlockType1Dedicated says it can also transfer SystemInformationBlockType1-BR but the ASN.1 defines the field as OCTET STRING CONTAINING SystemInformationBlockType1 only | 3 | Clarify which should be the correct one to include! Coordinator (Ericsson): A clarification proposal would make it easier to capture this in the CR. |  |
| N.055 | 6.2.2 RRCConnectionReconfiguration::perCC-GapIndicationRequest | Not clear this is one-shot field | 2 | Indicate in field description this is a one-shot field. Coordinator (Ericsson): A text proposal would make it easier to capture this in the CR. |  |
| N.054 | 6.2.2 RRCConnectionReconfiguration::sl-P2X-ConfigDedicated-r14, :: sl-P2X-ConfigDedicated-r14 | Can both of these be configured at the same time for the same UE? If not, why are both fields needed? | 2 | If both can not be configured at the same time, for one UE, define only one field. To differentiate V2X and P2X, a “type”-field can be added inside SL-V2X-ConfigDedicated-r14 if there is a need to do so. Coordinator (Ericsson): A text proposal would make it easier to capture this in the CR. |  |
| E.012 | 6.2.2 RRCConnectionReconfigurationComplete | The following reference (high-lighted below) should include the referenced subclause of the specification numFreqEffective This field is used to indicate the number of effective frequencies that a UE measures in series according to [TS 36.133]. Simultaneous measurement in parallel on multiple frequencies can be equivalent to a single effective frequency. | 2 | The exact reference needs to be checked with RAN1 or RAN4. | CR/ TBD (Ericsson) |
| N.056 | 6.2.2 RRCConnectionReconfigurationComplete::numFreqEffective | Field description is not clear | 2 | What do “effective frequencies” and “in series” mean? I couldn’t find this in 36.133. A clearer wording would have to be found somehow. Coordinator (Ericsson): A clarification proposal would make it easier to capture this in the CR. |  |
| S.013 | 6.2.2 RRCConnectionReconfiguration-v14x0, SCellToAddModExt-v14xy | The additional field is signalled differently for cells in the original and extended list. This is because only the original list has an extension marker. Seems desirable to prevent similar misalignment for future extensions | 2 | Add extension marker to SCellToAddModExt-v14xy (so that at least the same extension options exist) | CR |
| S.015 | 6.2.2 RRCConnectionRelease-v14xy-IEs | Is really the intention that paging configuration is optional in case of moving to light connected? No particular UE behaviour is defined (but UE would simply not be reachable, nor do RAN area updates) | 2 | If not intended, consider restructuring the ASN.1 or at least to introduce an E-UTRAN constraint | TDoc WI |
| L.028 | 6.2.2 SCPTMConfiguration | In SCPTMConfiguration field descriptions, description of ‘scptm-NeighbourCellList’ seems unclear | 2 | List of neighbour cells providing MBMS services via SC-MRB. If When absent, the UE shall assume that MBMS services listed in the SCPTMConfiguration message are not provided via SC-MRB in any neighbour cell. | CR |
| H.022 | 6.2.2 SCPTMConfiguration-BR | Parameter p-b-r14 is missing | 2 | Add parameter as follows SCPTMConfiguration-BR-r14 ::= SEQUENCE { sc-mtch-InfoList-r14 SC-MTCH-InfoList-BR-r14, scptm-NeighbourCellList-r14 SCPTM-NeighbourCellList-r13 OPTIONAL, -- Need OP p-b-r14 INTEGER (0..3) OPTIONAL, -- Need ON lateNonCriticalExtension OCTET STRING OPTIONAL, nonCriticalExtension SEQUENCE {} OPTIONAL } p-b Parameter: for the PDSCH scrambled by G-RNTI, see TS 36.213 [23, Table 5.2-1]. | CR |
| L.029 | 6.2.2 SCPTMConfiguration-BR | In SCPTMConfiguration-BR field descriptions, description of ‘scptm-NeighbourCellList’ seems unclear | 2 | scptm-NeighbourCellList List of neighbour cells providing MBMS services via SC-MRB. If When absent, the BL UE or UE in CE shall assume that MBMS services listed in the SCPTMConfiguration-BR message are not provided via SC-MRB in any neighbour cell. | CR |
| E.019 | 6.2.2 SidelinkUEInformation | V2X-CommTxResourceReq-r14 information element is only referenced once and it is not imported to any other module. There is no reason to define this new data type. | 2 | To be confirmed why this new data type is needed. If there are no reasons, remove the information element V2X-CommTxResourceReq-r14 V2X-CommTxResourceReq-r14 ::= SEQUENCE { v2x-CommTxFreqList-r14 SL-V2X-CommFreqList-r14 OPTIONAL, v2x-TypeTxSyncList-r14 SL-TypeTxSyncList-r14 OPTIONAL, v2x-DestinationInfoList-r14 SL-DestinationInfoList-r12 OPTIONAL } and add the removed fields to the ordinary non-critical extension of this message SidelinkUEInformation-v14x0-IEs ::= SEQUENCE { v2x-CommRxInterestedFreqList-r14 SL-V2X-CommFreqList-r14 OPTIONAL, v2x-CommTxResourceReq-r14 V2X-CommTxResourceReq-r14 OPTIONAL, v2x-CommTxResourceReq-r14 SEQUENCE { v2x-CommTxFreqList-r14 SL-V2X-CommFreqList-r14 OPTIONAL, v2x-TypeTxSyncList-r14 SL-TypeTxSyncList-r14 OPTIONAL, v2x-DestinationInfoList-r14 SL-DestinationInfoList-r12 OPTIONAL } OPTIONAL, nonCriticalExtension SEQUENCE {} OPTIONAL } | CR/ TBC |
| C.004 | 6.2.2 SidelinkUEInformation | SL-V2X-CommFreqList-r14 shall start with “0”, since 0 indicates PCell | 2 | SL-V2X-CommFreqList-r14 ::= SEQUENCE (SIZE (1..maxFreqV2X-r14)) OF INTEGER (10..maxFreq) Coordinator (Ericsson): Same as Z.016. | CR |
| Z.016 | 6.2.2 SidelinkUEInformation | The value range for SL-V2X-CommFreqList-r14 should be (0…maxFreq-1) instead of (1…maxFreq), since the description clearly says value 0 corresponds to Pcell. | 2 | SL-V2X-CommFreqList-r14 ::= SEQUENCE (SIZE (1..maxFreqV2X-r14)) OF INTEGER (10..maxFreq-1) v2x-CommRxInterestedFreqList Indicates the index(es) of the frequency(ies) on which the UE is interested to receive V2X sidelink communication. The value 1 corresponds to the frequency of first entry in v2x-InterFreqInfoList broadcast in SIB21, while the value 2 corresponds to the frequency of second entry in v2x-InterFreqInfoList broadcast in SIB21 and so on. The value 0 corresponds the PCell’s frequency. Coordinator (Ericsson): Same as C.004. The proposal to add “-1” after the maxFreq identifier name probably aims to indicate substraction of one from the maxFreq constant value which, however, cannot happen because ASN.1 does not support any arithmetics. I.e. maxFreq-1 is a new identifier name and it is different identifier than maxFreq. If it is necessary to redefine the upper range value, a new constant, e.g. maxFreqMinus1-r14, should be defined in subclause 6.4. | CR |
| S.019a | 6.2.2 SidelinkUEInformation, v2x-DestinationInfoList | It seems destinations are not listed per frequency. Is this really intended | 2 | Discuss (WI) | TDoc WI |
| N.066 | 6.2.2 SidelinkUEInformation:: v2x-CommRxInterestedFreqList | Editorials for the field description | 2 | See highlighted parts below: v2x-CommRxInterestedFreqList Indicates the indexes of the frequencies on which the UE is interested to receive V2X sidelink communication. The value 1 corresponds to the frequency of first entry in v2x-InterFreqInfoList broadcast in SIB21, while the value 2 corresponds to the frequency of second entry in v2x-InterFreqInfoList broadcast in SIB21 and so on. The value 0 corresponds the PCell’s frequency. | CR |
| N.067 | 6.2.2 SidelinkUEInformation:: v2x-CommTxFreqList | Editorials for the field description | 2 | There’s no need to differentiate between one or multiple frequencies, it just makes the text harder to read: v2x-CommTxFreqList Indicates the indexes of the frequencies on which the UE is interested to transmit V2X sidelink communication. The value 1 corresponds to the frequency of first entry in v2x-InterFreqInfoList broadcast in SIB21, while the value 2 corresponds to the frequency of second entry in v2x-InterFreqInfoList broadcast in SIB21 and so on. The value 0 corresponds the PCell’s frequency. | CR |
| N.069 | 6.2.2 SidelinkUEInformation:: v2x-CommTxFreqList, :: v2x-TypeTxSyncList | Not mentioning that v2x-CommTxFreqList and v2x-TypeTxSyncList should always have the same number of entries in the field description, only hinting strongly towards that? | 2 | If there should always be the same number of entries in both, then the following is proposed: v2x-TypeTxSyncList Indicates the synchronization reference used by the UE, listed in the same order and with equal number of entries as indicated by v2X-CommTxFreqList. v2x-TypeTxSyncList Indicates the synchronization reference used by the UE for the frequency(ies) indicated by v2x-CommTxFreqList in the same order. | CR |
| N.065 | 6.2.2 SidelinkUEInformation::SL-V2X-CommFreqList-r14 | Better to use named types for SEQUENCE OF | 2 | Instead of SL-V2X-CommFreqList-r14 ::= SEQUENCE (SIZE (1..maxFreqV2X-r14)) OF INTEGER (1..maxFreq) Use SL-V2X-CommFreqList-r14 ::= SEQUENCE (SIZE (1..maxFreqV2X-r14)) OF SL-V2X-CommFreq-r14 SL-V2X-CommFreq-r14 ::= INTEGER (1..maxFreq) | CR |
| Z.017 | 6.2.2 SPS-Config | For the SPS-Config-v14x0， the SPS-ConfigUL IE includes two semiPersistSchedInterval related parameters which are semiPersistSchedInterval and semiPersistSchedIntervalUL-v14x0。This may bring confusion for the UE. In fact, only semiPersistSchedIntervalUL-v14x0 is useful in the SPS-Config-v14x0. So we suggest to add some explanations to it. SPS-Config-v14x0 ::= SEQUENCE { ul-V-SPS-RNTI-r14 C-RNTI OPTIONAL, -- Need OR sl-V-SPS-RNTI-r14 C-RNTI OPTIONAL, -- Need OR sps-ConfigUL-ToAddModList-r14 SPS-ConfigUL-ToAddModList-r14 OPTIONAL, -- Need ON sps-ConfigUL-ToReleaseList-r14 SPS-ConfigUL-ToReleaseList-r14 OPTIONAL, -- Need ON sps-ConfigSL-ToAddModList-r14 SPS-ConfigSL-ToAddModList-r14 OPTIONAL, -- Need ON sps-ConfigSL-ToReleaseList-r14 SPS-ConfigSL-ToReleaseList-r14 OPTIONAL -- Need ON } SPS-ConfigUL-ToAddModList-r14 ::= SEQUENCE (SIZE (1..maxConfigSPS-r14)) OF SPS-ConfigUL SPS-ConfigUL-ToReleaseList-r14 ::= SEQUENCE (SIZE (1..maxConfigSPS-r14)) OF SPS-ConfigIndex-r14 SPS-ConfigSL-ToAddModList-r14 ::= SEQUENCE (SIZE (1..maxConfigSPS-r14)) OF SPS-ConfigSL-r14 SPS-ConfigSL-ToReleaseList-r14 ::= SEQUENCE (SIZE (1..maxConfigSPS-r14)) OF SPS-ConfigIndex-r14 SPS-ConfigUL ::= CHOICE { release NULL, setup SEQUENCE { semiPersistSchedIntervalUL ENUMERATED { sf10, sf20, sf32, sf40, sf64, sf80, sf128, sf160, sf320, sf640, sf1-v14xy, sf2-v14xy, sf3-v14xy, sf4-v14xy, sf5-v14xy, spare1}, implicitReleaseAfter ENUMERATED {e2, e3, e4, e8}, p0-Persistent SEQUENCE { p0-NominalPUSCH-Persistent INTEGER (-126..24), p0-UE-PUSCH-Persistent INTEGER (-8..7) } OPTIONAL, -- Need OP twoIntervalsConfig ENUMERATED {true} OPTIONAL, -- Cond TDD ..., [[ p0-PersistentSubframeSet2-r12 CHOICE { release NULL, setup SEQUENCE { p0-NominalPUSCH-PersistentSubframeSet2-r12 INTEGER (-126..24), p0-UE-PUSCH-PersistentSubframeSet2-r12 INTEGER (-8..7) } } OPTIONAL -- Need ON ]], [[ numberOfConfUlSPS-Processes-r13 INTEGER (1..8) OPTIONAL -- Need OR ]], [[ fixedRV-NonAdaptive-r14 ENUMERATED {true} OPTIONAL, -- Need OR sps-ConfigIndex-r14 SPS-ConfigIndex-r14 OPTIONAL, -- Need OR semiPersistSchedIntervalUL-v14x0 ENUMERATED { sf50, sf100, sf200, sf300, sf400, sf500, sf600, sf700, sf800, sf900, sf1000, spare5, spare4, spare3, spare2, spare1} ]] } } | 3 | Add some explanations in the semiPersistSchedIntervalUL field description. semiPersistSchedIntervalUL Semi-persistent scheduling interval in uplink, see TS 36.321 [6]. Value in number of sub-frames. Value sf10 corresponds to 10 sub-frames, sf20 corresponds to 20 sub-frames and so on. For TDD, when the configured Semi-persistent scheduling interval is greater than or equal to 10 sub-frames, the UE shall round this parameter down to the nearest integer (of 10 sub-frames), e.g. sf10 corresponds to 10 sub-frames, sf32 corresponds to 30 sub-frames, sf128 corresponds to 120 sub-frames. If semiPersistSchedIntervalUL and semiPersistSchedIntervalUL-v14x0 coexist in the SPS-ConfigUL, the semiPersistSchedIntervalUL is invalid. | CR |
| E.020 | 6.2.2 SystemInformationBlockType1 | The presence of the field eCallOverIMS-Support indicates whether the cell supports eCall over IMS services. Usually the wording “support” is used for UE features whereas network indicates whether the feature is allowed or not. | 2 | Reformulate the field description in subclause 6.2.2 as eCallOverIMS-AllowedSupport Indicates whether the cell allowssupports eCall over IMS services for UEs as defined in TS 23.401 [41]. If absent, eCall over IMS is not allowedsupported by the network in the cell. NOTE 2. Rename the field identifier in subclause 6.2.2 as eCallOverIMS-AllowedSupport-r14 ENUMERATED {true} OPTIONAL, -- Need OR Change the field reference in procedures in subclause 5.2.2.7 3> forward the eCallOverIMS-AllowedSupport to upper layers, if present; | CR |
| Q.006 | 6.2.2 SystemInformationBlockType1 | SystemInformationBlockType1-v14xy-IEs ::= SEQUENCE { eCallOverIMS-Support-r14 ENUMERATED {true} OPTIONAL, -- Need OR tdd-Config-v14xy TDD-Config-v14xy OPTIONAL, -- Cond TDD-OR nonCriticalExtension SEQUENCE {} OPTIONAL } | 1 | The choice of the name of Conditional presence “TDD-OR” for the field tdd-Config-v14xy is not normal. We usually do not mention ‘ON’ or ‘OR’ in the name. Suggest to use TDD2 or so. | CR |
| Q.007 | 6.2.2 SystemInformationBlockType13 | mbsfn-AreaInfoListV2X | 2 | Handling of the IE is not clear either in ASN.1 or in procedural text. Coordinator (Ericsson): A text proposal would make it easier to capture this in the CR. |  |
| S.028 | 6.3.1 SystemInformationBlockType15, MBMS-CarrierType-r14 | It would seem more natural to model the additional functionality by an optional field that is configured for carriers having such additional feMBMS functionality (rather than by a choice) | 1/ 2 | Change to MBMS-CarrierType-r14 ::= SEQUENCE { fembms-CarrierFreq-r14 FEMBMS-CarrierFreq-r14 OPTIONAL -- Need OR } Ericsson: Same as Q.008. | CR |
| Q.008 | 6.2.2 SystemInformationBlockType15 | mbms-IntraFreqCarrierType-r14 | 2 | The need of the choice ‘mbms-CarrierFreq-r14’ is not entirely clear for intra-frequency since the legacy carrier can be implicitly signalled by the omission of MBMS-CarrierType-r14. One solution is to make mbms-IntraFreqCarrierType-r14 be FEMBMS-CarrierFreq-r14 OPTIONAL. Ericsson: so instead of choice, have the parameter optional. Does that then need field description update? Same as S.029. | CR |
| S.029 | 6.3.1 SystemInformationBlockType15, MBMS-CarrierType-r14 | Normally period and offset are configured by a joint encoding i.e. choice | 2 | Change to something like: MBMS-PeriodOffset-r14 ::= CHOICE { oneFrame-r14 NULL, fourFrame-r14 INTEGER (0..3) } Ericsson: a problem arises since the full info is not always know thus configuring frameOffset only and having that as 0 works as an indication only. If we adopt Samsung way, indication migth be needed separately? | CR |
| E.129 | 6.2.2 SystemInformationBlockType1-MBMS | Field description for subframeAllocation could be clarified. | 2 | The field description can be clarified to: subframeAllocation Defines the subframes that are allocated for non-MBSFN within the radio frame allocation period defined by the radioFrameAllocationPeriod and the radioFrameAllocationOffset. “0” “1” denotes that the corresponding subframe is a non-MBSFN subframe. “1” denotes that the corresponding subframe is a non-MBSFN subframe. If E-UTRAN configures a value other than ”0” for additionalNonMBSFNSubframes, the value “1” is configured for those subframes in subframeAllocation that map to subframes pointed out by additionalNonMBSFNSubframes, | CR |
| N.073 | 6.2.2 SystemInformationBlockType1-MBMS | Missing suffixes | 2 | Where missing, add -r14 to all IEs and fields in this message (since the content is fully defined within Rel-14). Coordinator (Ericsson): Corrected during v14.2.0 CR implementation. | CR |
| N.041 | 6.2.2 SystemInformationBlockType1-MBMS message | NonMBSFN-SubframeConfig ::= SEQUENCE { radioFrameAllocationPeriod-r14 ENUMERATED {rf4, rf8, rf16, rf32, rf64, rf128, rf512}, radioFrameAllocationOffset-r14 INTEGER (0..7), subframeAllocation-r14 BIT STRING (SIZE(9)) } “1” denotes that the corresponding subframe is non-MBSFN subframe. “1” denotes that the corresponding subframe is non-MBSFN subframe. If EUTRAN configures value other than ”0” for additionalNonMBSFNSubframes, value “1” is configured for those subframes in subframeAllocation that map to subframes pointed by additionalNonMBSFNSubframes, | 2 | Description needs to be changed but not sure whether this will work ok. Should so called CAS SFs also be indicated by this configuration? Probably yes, so should be captured in field description. If allocation period is up to 512 rf then how a bitmap of size (9) be enough? Coordinator (Ericsson): A text proposal would make it easier to capture this in the CR. Ericsson: Our thinking was that the so called CAS are not configurable but are fixed by MBMS-dedicated cell definition. Assuming this can be confirmed with possible needed clarification: This configuration is for additional MBSFN subframes to CAS and to those indicated by MIB-MBMS. We provided clarification suggestion in E.129. The bitmap (9) was assumed to be enough to enable 9 consequtive nonMBSFN subframes to be enough for SI broadcasting with what ever periodicity. Exact value can be discussed. Other option is to have a fixed “CAS” configuration using NonMBSFN-SubframeConfig. | CR |
| S.020 | 6.2.2 SystemInformationBlockType1-MBMS, systemInformationBlockType13 | A need code is missing. It seems there are two ways to transfer SIB13 i.e. either in an SI message, or by inclusion in SIB1 directly. This means UE should not simply release stored SIB13 upon absence in SIB1 i.e. need OR is not appropriate, but OP with some text? | 2 | Use need OP and clarify the handling in field description i.e. upon absence UE releases the stored SIB13 only if it was acquired from SIB1 Ericsson: Sounds like to a good solution. | CR |
| N.074 | 6.2.2 SystemInformationBlockType1-MBMS:: subframeAllocation | Field description is rather unclear | 2 | The intent seems to be that MIB-MBMS and SIB1-MBMS are consistent, but the description is really complicated to understand. Hence, we propose the following: “subframeAllocation “1” denotes that the corresponding subframe is non-MBSFN subframe. E-UTRAN configures this field consistently with values in additionalNonMBSFNSubframes within MasterInformationBlock-MBMS” Alternatively, the following could also be used: “subframeAllocation “1” denotes that the corresponding subframe is non-MBSFN subframe. E-UTRAN configures this field consistently with values in additionalNonMBSFNSubframes within MasterInformationBlock-MBMS” We would like to discuss what exactly is needed here to ensure consistency in network configuration. Coordinator (Ericsson): It is not clear from this description what it reads in the specification now and how you would like to change it. Ericsson: Related to E.129 and N.041. |  |
| N.071 | 6.2.2 SystemInformationBlockType1-MBMS::PLMN-IdentityList-MBMS | No point to define non-extendible SEQUENCE just for wrapping an individual field | 2 | Remove PLMN-IdentityInfo-MBMS and change definition to PLMN-IdentityList-MBMS ::= SEQUENCE (SIZE (1..maxPLMN-r11)) OF PLMN-Identity PLMN-IdentityList-MBMS ::= SEQUENCE (SIZE (1..maxPLMN-r11)) OF PLMN-IdentityInfo-MBMS PLMN-IdentityInfo-MBMS ::= SEQUENCE { plmn-Identity PLMN-Identity } | CR |
| N.070 | 6.2.2 SystemInformationBlockType1-MBMS::SIB-Type-MBMS | The IE SIB-Type-MBMS is not extendible, so it only allows defining up to 2 new SIB types for the MBMS carrier (since it has 2 spare values) | 2 | The IE SIB-Type-MBMS is not extendible, but has two spare values. This means it can accommodate maximum two new SIBs – it should be discussed if this is reasonable or whether ellipsis should be added. |  |
| N.075 | 6.2.2 SystemInformationBlockType1-MBMS::systemInformationBlockType13 | Missing need code and field description could be clearer | 2 | Add Need OR and change “parameter” to “field” in the field description, i.e. systemInformationBlockType13 E-UTRAN does not configure this field if schedulingInfoList–MBMS indicates that SystemInformationBlockType13 is present. | CR |
| L.030 | 6.2.2 SystemInformationBlockType20 |  | 2 | mpdcch-StartSF-SC-MCCH Configuration of the sStarting subframes configuration of the MPDCCH search space for SC-MCCH, see TS 36.213 [23]. | CR |
| L.031 | 6.2.2 SystemInformationBlockType20 |  | 2 | pdsch-maxNumRepetitionCEmodeA-SC-MTCH Maximum value to indicate the set of PDSCH repetition numbers for SC-MTCH to the UEs in CE mode A, see TS 36.213 [23]. | CR |
| L.032 | 6.2.2 SystemInformationBlockType20 |  | 2 | pdsch-maxNumRepetitionCEmodeB-SC-MTCH Maximum value to indicate the set of PDSCH repetition numbers for SC-MTCH to the UEs in CE mode B, see TS 36.213 [23]. | CR |
| E.024 | 6.2.2 UEAssistanceInformation | BW-Preference-r14 and SPS-AssistanceInformation-r14 information elements are referenced only once and they not imported to any other module. There are no reasons to define these new data types. See also E.110. | 2 | To be confirmed why this new data type is needed. If there are no reasons, remove the information elements in subclause 6.2.2. BW-Preference-r14 ::= SEQUENCE { dl-Preference-r14 ENUMERATED {mhz1dot4, mhz5, mhz20, spare1} OPTIONAL, ul-Preference-r14 ENUMERATED {mhz1dot4, mhz5, spare2, spare1} OPTIONAL } SPS-AssistanceInformation-r14 ::= SEQUENCE { trafficPatternInfoListSL-r14 TrafficPatternInfoListSL-r14 OPTIONAL, trafficPatternInfoListUL-r14 TrafficPatternInfoListUL-r14 OPTIONAL } and add the extension fields to the ordinary non-critical extension of the message in subclause 6.2.2 as UEAssistanceInformation-v14x0-IEs ::= SEQUENCE { bw-Preference-r14 BW-Preference-r14 OPTIONAL, sps-AssistanceInformation-r14 SPS-AssistanceInformation-r14 OPTIONAL, bw-Preference-r14 SEQUENCE { dl-Preference-r14 ENUMERATED {mhz1dot4, mhz5, mhz20, spare1} OPTIONAL, ul-Preference-r14 ENUMERATED {mhz1dot4, mhz5, spare2, spare1} OPTIONAL }, sps-AssistanceInformation-r14 SEQUENCE { trafficPatternInfoListSL-r14 TrafficPatternInfoListSL-r14 OPTIONAL, trafficPatternInfoListUL-r14 TrafficPatternInfoListUL-r14 OPTIONAL }, nonCriticalExtension SEQUENCE {} OPTIONAL } | CR/ TBC |
| E.110 | 6.2.2 UEAssistanceInformation | The Information Elements TrafficPatternInfoListSL and TrafficPatternInfoListUL both contain the same information. It is more efficient to create one information element TrafficPatternInfoList instead. SPS-AssistanceInformation-r14 ::= SEQUENCE { trafficPatternInfoListSL-r14 TrafficPatternInfoListSL-r14 OPTIONAL, trafficPatternInfoListUL-r14 TrafficPatternInfoListUL-r14 OPTIONAL } TrafficPatternInfoListSL-r14 ::= SEQUENCE (SIZE (1..maxTrafficPattern-r14)) OF TrafficPatternInfo-r14 TrafficPatternInfoListUL-r14 ::= SEQUENCE (SIZE (1..maxTrafficPattern-r14)) OF TrafficPatternInfo-r14 See also E.024. | 2 | Replace the two IEs with one. SPS-AssistanceInformation-r14 ::= SEQUENCE { trafficPatternInfoListSL-r14 TrafficPatternInfoListSL-r14 OPTIONAL, trafficPatternInfoListUL-r14 TrafficPatternInfoListUL-r14 OPTIONAL } TrafficPatternInfoListSL-r14 ::= SEQUENCE (SIZE (1..maxTrafficPattern-r14)) OF TrafficPatternInfo-r14 TrafficPatternInfoListUL-r14 ::= SEQUENCE (SIZE (1..maxTrafficPattern-r14)) OF TrafficPatternInfo-r14 | CR |
| E.111 | 6.2.2 UEAssistanceInformation | The values cemhz1dot4 and cemhz5 are not in the ASN.1 code. ul-Preference Indicates UE’s preference on configuration of maximum PUSCH bandwidth. The value cemhz1dot4 corresponds to CE mode usage in 1.4MHz bandwidth, and cemhz5 corresponds to CE mode usage in 5MHz bandwidth. | 1 | Change the field description to: ul-Preference Indicates UE’s preference on configuration of maximum PUSCH bandwidth. The value cemhz1dot4 corresponds to CE mode usage in 1.4MHz bandwidth, and cemhz5 corresponds to CE mode usage in 5MHz bandwidth. | CR |
| E.00? | 6.2.2 UEAssistanceInformation | In the field description of o dl-Preference “or normal coverage” seems misplaced and should be removed. Also space before the units are missing. | 2 | dl-Preference Indicates UE’s preference on configuration of maximum PDSCH bandwidth. The value mhz1dot4 corresponds to CE mode usage in 1.4 MHz bandwidth, mhz5 corresponds to CE mode usage in 5 MHz bandwidth, and mhz20 corresponds to CE mode usage in 20 MHz bandwidth or normal coverage. | CR |
| L.034 | 6.2.2 UEAssistanceInformation | Rearrange the fields in field description in alphabetical order | 1 | logicalChannelIdentityUL should be placed before messageSize TrafficPatternInfoSL/ TrafficPatternInfoUL should be placed before trafficPeriodicity | CR |
| I.017 | 6.2.2 UEAssistanceInformation field descriptions | A 6-bitstring value is for the field messageSize.However, further information of e.g. unit (in bits?) and format to be used is missing. messageSize-r14 BIT STRING (SIZE (6)) messageSize Indicates the maximum TB size based on the observed traffic pattern. | 2 | Clarify further details of field messageSize. Coordinator (Ericsson): It is easier to capture this in the CR if there is a text proposal. | CR |
| S.021 | 6.2.2 UEAssistanceInformation, messageSize | The field concerns a bit string, but the actual setting is not clarified. If covered elsewhere, a reference should be added | 2 | Introduce clarification | Flag |
| N.076 | 6.2.2 UEAssistanceInformation::BW-Preference-r14 | UL spare values defined, which is against Rel-13 agreement not to do that unless there is clear reason and error handling defined. | 2 | Remove all spares, i.e. use BW-Preference-r14 ::= SEQUENCE { dl-Preference-r14 ENUMERATED {mhz1dot4, mhz5, mhz20, spare1} OPTIONAL, ul-Preference-r14 ENUMERATED {mhz1dot4, mhz5, spare1} OPTIONAL } | CR |
| N.076b | 6.2.2 UEAssistanceInformation::TrafficPatternInfo-r14 | UL spare values defined, which is against Rel-13 agreement not to do that unless there is clear reason and error handling defined. | 2 | Remove spare values, i.e. use TrafficPatternInfo-r14 ::= SEQUENCE { trafficPeriodicity-r14 ENUMERATED { sf20, sf50, sf100, sf200, sf300, sf400, sf500, sf600, sf700, sf800, sf900, sf1000, spare4, spare3, spare2, spare1} | CR |
| S.022 | 6.2.2 UECapabilityEnquiry, requestDiffFallbackCombList | Second sentence of field description seems procedural detail, better covered there (probably already clear). E-UTRAN constraint seems missing | 2 | Remove 2nd sentence and clarify that EUTRAN does not include togethr with skipFallback | TDoc WI |
| E.121 | 6.2.2 WLANConnectionStatusReport | Field description tells EUTRAN what to do with the report: If the wlan-Status-v14x0 is included, E-UTRAN ignores the wlan-Status-r13. | 2 | Sentence is not needed If the wlan-Status-v14x0 is included, E-UTRAN ignores the wlan-Status-r13. | CR |
| N.078 | 6.2.2 WLANConnectionStatusReport::wlan-Status-v14x0 | E-UTRAN actions should be re-worded to be more appropriate | 2 | Normally we don’t indicate E_UTRAN is mandated to do something – in this case, likely it would do so, but we shouldn’t mandate the action. Therefore, we would propose to change the field to use “may ignore” instead, i.e. wlan-Status Indicates the connection status to WLAN and the cause of failures. If the wlan-Status-v14x0 is included, E-UTRAN may ignores the wlan-Status-r13. | CR |
| S.023 | 6.2.2 WLANConnectionStatusReport-v14x0-IEs | Field wlan-Status-v14x0 should probably be optional, as otherwise it is not possible to signal an extension other than wlan-Status values in combination with legacy status values | 2 | Add optional | CR |
| 6.3.1 System information blocks |  |  |  |  |  |
| E.025 | 6.3.1 SystemInformationBlockType2 | mbsfn-SubframeConfigList field description has a formulation “EUTRAN always includes” which imposes a certain type of network behaviour. | 2 | Remove the word “always” and reformulate as, mbsfn-SubframeConfigList Defines the subframes that are reserved for MBSFN in downlink. NOTE 1. If the cell is a FeMBMS/Unicast mixed cell, EUTRAN always includes mbsfn-SubframeConfigList-v14xy on FeMBMS/Unicast mixed cell. | CR |
| N.081 | 6.3.1 SystemInformationBlockType2 | mbsfn-SubframeConfigList Defines the subframes that are reserved for MBSFN in downlink. NOTE 1. EUTRAN always includes mbsfn-SubframeConfigList-v14xy on FeMBMS/Unicast mixed cell. According to current definition of feMBMS Unicast/mixed carrier from 36.300 the highlighted part is not always relevant. | 2 | Remove NOTE1. Ericsson: it needs to be there to indicate feMBMS unicast carrier. | CR |
| S.024 | 6.3.1 SystemInformationBlockType2, mbsfn-SubframeConfigList-v14xy | Need code is missing. Moreover, relation to existing field is not entirely clear. Seems both fields can be configured together (but is original field always configured when extension is included?) | 1/ 2 | Add need code. Consider clarification of relation Coordinator (Ericsson): See also E.122, I.018, N.083. Ericsson: Original field does not need to be always included, these are independent. Though, fembms mixed carrier is indicated by the Rel-14 field. | CR, TDoc feMBMS |
| E.122 | 6.3.1 SystemInformationBlockType2 | There is no need code for: MBSFN-SubframeConfigList-v14xy | 1 | Add need code: [[ mbsfn-SubframeConfigList-v14xy MBSFN-SubframeConfigList-v14xy OPTIONAL --Need OP, videoServiceCauseIndication-r14 ENUMERATED {true} OPTIONAL, -- Need OP rrc-LightConnectionSupport-r14 ENUMERATED {true} OPTIONAL -- Need OP ]] Coordinator (Ericsson): Same as S.024, I.018, N.083. | CR |
| I.018 | 6.3.1 SystemInformationBlockType2 | Need code is missing for mbsfn-SubframeConfigList-v14xy. To be consistent with the legacy field NEED OR looks ok. | 2 | Add missing NEED OR for mbsfn-SubframeConfigList-v14xy. Coordinator (Ericsson): Same as S.024, E.122, N.083. | CR |
| N.083 | 6.3.1 SystemInformationBlockType2::mbsfn-SubframeConfigList-v14xy | Need code missing | 2 | Add Need OR Coordinator (Ericsson): Same as S.024, I.018, E.122. | CR |
| L.035 | 6.3.1 SystemInformationBlockType13 | It does not seem necessary to introduce new field ‘mbsfn-AreaInfoListV2X-r14’ | 2 | Remove ‘mbsfn-AreaInfoListV2X-r14’ | CR |
| N.085 | 6.3.1 SystemInformationBlockType13::mbsfn-AreaInfoListV2X-r14 | The field is not referenced in the specification | 2 | Discuss if the field should have some associated procedural text |  |
| E.040 | 6.3.1 SystemInformationBlockType13 | notificationConfig-v14xy and mbsfn-AreaInfoListV2X-r14 are missing Need codes. | 1 | Add Need codes OR [[ notificationConfig-v14xy MBMS-NotificationConfig-v14xy OPTIONAL, -- Need OR mbsfn-AreaInfoListV2X-r14 MBSFN-AreaInfoList-r9 OPTIONAL -- Need OR ]] Coordinator (Ericsson): Same as I.019, S.026. | CR |
| I.019 | 6.3.1 SystemInformationBlockType13 | Need code for notificationConfig-v14xy and mbsfn-AreaInfoListV2X-r14 is missing. [[ notificationConfig-v14xy MBMS-NotificationConfig-v14xy OPTIONAL, mbsfn-AreaInfoListV2X-r14 MBSFN-AreaInfoList-r9 OPTIONAL ]] | 2 | Add missing Need code for notificationConfig-v14xy and mbsfn-AreaInfoListV2X-r14. Need OR looks ok. Coordinator (Ericsson): Same as E.040, S.026. | CR |
| S.026 | 6.3.1 SystemInformationBlockType13 | Need codes are missing for new extensions | 1 | Add need OR Coordinator (Ericsson): Same as E.040, I.019. | CR |
| N.086 | 6.3.1 SystemInformationBlockType13:: notificationConfig-v14xy, ::mbsfn-AreaInfoListV2X-r14 | Need codes are missing | 2 | Add needs codes (presumably Need OR) to the fields Coordinator (Ericsson): Same as E.040. | CR |
| S.027 | 6.3.1 SystemInformationBlockType15 | Need codes are missing for new extensions | 1 | Add need OR Coordinator (Ericsson): Same as I.020. | CR |
| I.020 | 6.3.1 SystemInformationBlockType15 | Need code for mbms-IntraFreqCarrierType-r14 and mbms-InterFreqCarrierTypeList-r14 is missing. To be consistent with legacy fields NEED OR looks ok. | 2 | Add missing NEED OR for mbms-IntraFreqCarrierType-r14 and mbms-InterFreqCarrierTypeList-r14. Coordinator (Ericsson): Same as S.027. | CR |
| E.123 | 6.3.1 SystemInformationBlockType15 | All new field descriptions refer to “FeMBMS carrier” which means frequency where either FeMBMS/Unicast mixed cells or MBMS-dedicated cells are operating. | 2 | Could be discussed if the “FeMBMS carrier” needs to be refined with more explicit wording mentioning both FeMBMS/Unicast and MBMS dedicated cells. | CR/ TBC |
| E.026 | 6.3.1 SystemInformationBlockType15 | FEMBMS-CarrierFreq-r14 information element is referenced only once and it is not imported to any other module. There is no reason to define this new data type. | 2 | To be confirmed why this new data type is needed. If there are no reasons, remove the information element FEMBMS-CarrierFreq-r14 ::= SEQUENCE { frameOffset-r14 INTEGER (0..3), framePeriod-r14 ENUMERATED {n1, n4} OPTIONAL -- Need OR } and add the extension fields to MBMS-CarrierType-r14 ::= CHOICE { mbms-CarrierFreq-r14 NULL, fembms-CarrierFreq-r14 SEQUENCE { frameOffset-r14 INTEGER (0..3), framePeriod-r14 ENUMERATED {n1, n4} OPTIONAL -- Need OR }, fembms-CarrierFreq-r14 FEMBMS-CarrierFreq-r14 } | CR/ TBC |
| N.087 | 6.3.1 SystemInformationBlockType15 | mbms-InterFreqCarrierTypeList Contains a list of indications whether a neighboring frequency is pre-Rel-14 MBMS carrier or FeMBMS carrier | 2 | Description should indicate that this field is included only if mbms-SAI-InterFreqList-r11 is included. The number of entries is the same in both fields and carrier type relates to the frequency indicated in mbms-SAI-InterFreqList-r11 in the corresponding entry index. Ericsson: Seem OK. Same as S.030 but better? | CR |
| S.030 | 6.3.1 SystemInformationBlockType15, InterFreqCarrierTypeList | As the carrierFreq is not included, it seems this list is parallel to mbms-SAI-InterFreqList i.e. nth entry concerns the same frequency of nth entry of that parallel list | 2 | Add the statement that the lists concern the same number of frequencies and listed in the same order as mbms-SAI-InterFreqList Ericsson: Same as N.087. | CR |
| H.025 | 6.3.1 SystemInformationBlockType20 | Parameters mpdcch-SC-MCCH-Config-r14, sc-mcch-RepetionPeriod-BR-r14, sc-mcch-ModificationPeriod-BR-r14 are defined as OPTIONAL. However they should be mandatory present when sib20 apply to BL UEs and UEs in CE | 3 | Make the fields mandatory present in the top level IE proposed above. Alternatively, make the IEs individually OPTIONAL – Cond BR-BCCH With condition BR-BCCH: the field is mandatory present if SystemInformationBlockType20 is sent on BR-BCCH . Otherwise the field is not present. Linked to H.024 | TBC |
| H.026 | 6.3.1 SystemInformationBlockType20 | Parameters sc-mcch-CarrierFreq-r14, sc-mcch-Offset-BR-r14 and sc-mcch-RepetionPeriod-BR-r14, are defined as MANDATORY. However they are not needed when sib20 apply to legacy UEs | 3 | Keep the fields mandatory present in the top level IE proposed above. Alternatively, make the IEs OPTIONAL – Cond BR-BCCH Linked to H.024 | TBC |
| H.024 | 6.3.1 SystemInformationBlockType20 | The parameters required for FeMTC have been added individually as non critical extensions However, all these parameters are needed together and applicable only when SIB-20 is provided for FeMTC. They replace the legacy parameters (i.e. they are not extension but replacement of the legacy parameters) | 3 | It would be simpler to create a new SIB SystemInformationBlockType20-BR-r14 only used when carried within SystemInformation-BR Alternatively, group all the parameters under one top level IE, e.g systemInformationBlockType20-BR-r14, Optional – Cond BR-BCCH , which is only applicable when SystemInformationBlockType20 is sent on BR-BCCH and clarify that, if present, the BL UE or a UE in CE ignores the legacy IEs. Need to discuss which approach for signalling the parameters specific to FeMTC. This has impact on comments H.025, H.026, | TBC |
| H.028 | 6.3.1 SystemInformationBlockType20 | IE MPDCCH-SC-MCCH-Config-r14. : Parameters mpdcch-Narrowband-SC-MCCH-r14 and mpdcch-NumRepetition-SC-MCCH-r14 are defined as OPTIONAL w/o need code. However they should be mandatory | 2/3 | Make all fields mandatory present in IE MPDCCH-SC-MCCH-Config-r14. | CR |
| Q.009 | 6.3.1 SystemInformationBlockType20 message | sc-mcch-RepetitionPeriod-BR-r14 and sc-mcch-ModificationPeriod-BR-r14 are defined as optional but UE behaviour is not defined if either of these two IEs are not included in SIB20. | 2 | Perhaps clarification needed to say use –r13 versions of the parameter if r14 version is omitted. | CR |
| Q.010 | 6.3.1 SystemInformationBlockType20 message | pdsch-maxnumRepetitionCEmodeA-SC-MTCH-r14 and pdsch-maxnumRepetitionCEmodeB-SC-MTCH-r14 are optional and UE behaviour undefined if either of these IEs are omitted. | 2 | Define UE behaviour if either of these IE is missing. |  |
| Q.011 | 6.3.1 SystemInformationBlockType20 message | mpdcch-Narrowband-SC-MCCH-r14 is optional and UE behaviour undefined if this IEs is omitted. | 2 | Define UE behaviour if this IE is missing, e.g. use same narrowband as that used for SIB receptiion. | CR |
| Q.012 | 6.3.1 SystemInformationBlockType20 message | mpdcch-NumRepetition-SC-MCCH-r14 is optional and UE behaviour undefined if this IEs is omitted. | 2 | Define UE behaviour if this IE is missing, e.g. use same values as defined for MPDCCH common search space.for the corresponding CE level.. | CR |
| Q.013 | 6.3.1 SystemInformationBlockType20 message | sc-mcch-onDurationTimerSCPTM-r14 is optional and UE behaviour undefined if this IEs is omitted. | 2 | Define UE behaviour if this IE is missing. |  |
| C.010 | 6.3.1 SystemInformationBlockType20/SC-MTCH-InfoList-BR | Narrowband has already been defined for MPDCCH transmission for SC-PTM transmission, i.e. mpdcch-Narrowband-SC-MCCH-r14 and mpdcch-Narrowband-SC-MTCH-r14. It is not necessary to define carrier frequency for SC-MCCH and SC-MTCH transmission. | 3 | sc-mcch-CarrierFreq-r14 in IE SystemInformationBlockType20 and sc-mtch-CarrierFreq-r14 in SC-MTCH-InfoList-BR are deleted. | CR/ TDoc WI |
| E.027 | 6.3.1 SystemInformationBlockType20 | Optional fields mpdcch-SC-MCCH-Config-r14, sc-mcch-RepetionPeriod-BR-r14, sc-mcch-ModificationPeriod-BR-r14, mpdcch-Narrowband-SC-MCCH-r14, mpdcch-NumRepetition-SC-MCCH-r14, sc-mcch-onDurationTimerSCPTM-r14 are missing Need code. | 1 | All these fields are broadcasted and therefore Need OR should be used mpdcch-SC-MCCH-Config-r14 MPDCCH-SC-MCCH-Config-r14 OPTIONAL, -- Need OR sc-mcch-RepetionPeriod-BR-r14 ENUMERATED {rf32, rf128, rf512, rf1024, rf2048, rf4096, rf8192, rf16384} OPTIONAL, -- Need OR sc-mcch-ModificationPeriod-BR-r14 ENUMERATED { rf32, rf128, rf256, rf512, rf1024, rf2048, rf4096, rf8192, rf16384, rf32768, rf65536, rf131072, rf262144, rf524288, rf1048576, spare1} OPTIONAL -- Need OR mpdcch-Narrowband-SC-MCCH-r14 INTEGER (1.. maxAvailNarrowBands-r13) OPTIONAL, -- Need OR mpdcch-NumRepetition-SC-MCCH-r14 ENUMERATED {r1, r2, r4, r8, r16, r32, r64, r128, r256} OPTIONAL, -- Need OR sc-mcch-onDurationTimerSCPTM-r14 ENUMERATED {psf10, psf20, psf100, psf300, psf500, psf1000, psf1200, psf1600} OPTIONAL, -- Need OR Coordinator (Ericsson): Same as N.090, S.031. | CR |
| N.090 | 6.3.1 SystemInformationBlockType20 | NEED codes missing in the new fields | 2 | Add NEED codes Coordinator (Ericsson): Same as E.027, S.031. |  |
| S.031 | 6.3.1 SystemInformationBlockType20 (also for sub-fields of MPDCCH-SC-MCCH-Config) | Need codes are missing for new extensions. Also, for sc-mcch-SchedulingInfo need code OP is stated but there is no procedural text/ statement in the fiel description | 1 | Add need OR (by default) Coordinator (Ericsson): Same as N.090, E.027. | CR |
| H.027 | 6.3.1 SystemInformationBlockType20 | Parameter sc-mcch-SchedulingInfo-r14 is defined as OPTIONAL Need OP. However, there is no field description. | 2 | Add field description as follows: sc-mcch-schedulingInfo DRX information for the SC-MCCH. If this field is absent, DRX is not used for SC-MCCH reception. | CR |
| H.029 | 6.3.1 SystemInformationBlockType20 | IE SC-MCCH-SchedulingInfo-r14: The DRX mechanism is identical for SC-MCCH and SC-MTCH. Hower different parameter names have been used which leads to text duplication in MAC specification. It would better to use the same parameter names | 2/3 | Rename the parameters as follows: - sc-mcch-onDurationTimerSCPTM-r14 - sc-mcch-InactivityTimerSCPTM-r14 | CR |
| H.030 | 6.3.1 SystemInformationBlockType20 | The field description of sc-mcch-ModificationPeriod-BR defines the boundaries of the modification period using (H-FN*1024 + SFN). However , HFN may not be present in SIB1-BR | 2/3 | Revise the description as follows sc-mcch-ModificationPeriod-BR Defines periodically appearing boundaries for BL UE or UE in CE, i.e. radio frames for which (H-SFN*1024 + SFN) mod sc-mcch-ModificationPeriod-BR = 0 if hyperSFN is present in SIB1-BR or radio frames for which SFN mod sc-mcch-ModificationPeriod-BR = 0 otherwise. The contents of different transmissions of SC-MCCH information can only be different if there is at least one such boundary in-between them. Value rf32 corresponds to 32 radio frames, value rf128 corresponds to 128 radio frames and so on. | CR |
| H.031 | 6.3.1 SystemInformationBlockType20 | The field description of sc-mcch-Offset-BR defines the boundaries of the repetition period using (H-FN*1024 + SFN). However, HFN may not be present in SIB1-BR | 2 | Revise the description as follows sc-mcch-Offset-BR Indicates, together with the sc-mcch-RepetitionPeriod-BR, the boundary of the repetition period for BL UE or UE in CE: (H-SFN*1024 + SFN) mod sc-mcch-RepetitionPeriod-BR = sc-mcch-Offset-BR if hyperSFN is present in SIB1-BR or radio frames for which (SFN mod mod sc-mcch-RepetitionPeriod-BR) = sc-mcch-Offset-BR otherwise. | CR |
| H.032 | 6.3.1 SystemInformationBlockType20 | The field description of sc-mcch-onDurationTimerSCPTM is not correct | 2 | Revise the description as follows sc-mcch-onDurationTimerSCPTM Indicates the duration in subframes during which SC-MCCH may be scheduled in MPDCCH sub-frames, see TS 36.321 [6].Timer for SC-MCCH reception in TS 36.321 [6]. Value in number of PDCCH sub-frames. Value psf10 corresponds to 10 PDCCH sub-frames, psf20 corresponds to 20 PDCCH sub-frames and so on. | CR |
| N.089 | 6.3.1 SystemInformationBlockType20 | Remove spares from enums | 2 | It’s not clear we need the spares, and legacy UEs (i.e. after Rel-14) will not comprehend them. It might be better to just remove them to avoid issue later on. | CR |
| E.112 | 6.3.1 SystemInformationBlockType21 9.3.2 SL-V2X-Preconfiguration | The field description contains procedural matters better captured elsewhere. offsetDFN Indicates the timing offset for the UE to determine DFN timing. If configured, the UE determines “DFN timing=GNSS timing+offsetDFN” if gnss is configured in typeTxSync. | 2 | Consider updating the field description and section 5.10.14. offsetDFN Indicates the timing offset for the UE to determine DFN timing. If configured, the UE determines “DFN timing=GNSS timing+offsetDFN” if gnss is configured in typeTxSync. 5.10.14 DFN derivation from GNSS When the UE selects GNSS as the synchronization reference source, the DFN used for V2X sidelink communication is derived from the current UTC time, by the following formulae: DFN= Floor (0.1*(Tcurrent –Tref + offsetDFN)) mod 1024 SubframeNumber= Floor (Tcurrent –Tref + offsetDFN) mod 10 Where: Tcurrent is the current UTC time that obtained from GNSS. This value is expressed in milliseconds; Tref is the reference UTC time 00:00:00 on Gregorian calendar date 1 January, 1900 (midnight between Thursday, December 31, 1899 and Friday, January 1, 1900). This value is expressed in milliseconds; OffsetDFN is the value offsetDFN if configured otherwise zero. | CR |
| E.113 | 6.3.1 SystemInformationBlockType21 9.3.2 SL-V2X-Preconfiguration | The field description does not reveal which unit is used. offsetDFN Indicates the timing offset for the UE to determine DFN timing. If configured, the UE determines “DFN timing=GNSS timing+offsetDFN” if gnss is configured in typeTxSync. | 2 | Determine which time unit is used (milliseconds?) and add that to the field description. | CR/ TBC |
| E.114 | 6.3.1 SystemInformationBlockType21 6.3.8 SL-V2X-ConfigDedicated 9.3.2 SL-V2X-Preconfiguration | The field description contains procedural matters already captured in MAC. Additionally, the description can be improved. thresSL-TxPrioritization Indicates the threshold used to determine whether SL TX is prioritized over UL TX overlapping in time. If the SL data to be transmitted has a PPPP value lower than than this threshold, then the SL data can be prioritized over UL transmission in the same subframe. This value shall overwrite thresSL-TxPrioritization configured in SL-V2X-Preconfiguration if any. | 2 | Update the field description accordingly. thresSL-TxPrioritization Indicates the threshold used to determine whether SL V2X transmission is prioritized over uplink transmission if they collide in time (see TS 36.321 [6]).SL TX is prioritized over UL TX overlapping in time. If the SL data to be transmitted has a PPPP value lower than than this threshold, then the SL data can be prioritized over UL transmission in the same subframe. This value shall overwriteIf this field is present, the UE shall ignore thresSL-TxPrioritization configured in SL-V2X-Preconfiguration if any. | CR |
| L.036 | 6.3.1 SystemInformationBlockType21 | ‘typeTxSync-r14’ does not need to be mandatory since procedure text already describes the case that the field is missing. | 2 | Make ‘typeTxSync-r14’ optional. typeTxSync-r14 SL-TypeTxSync-r14 OPTIONAL, --Need OR | CR |
| N.091 | 6.3.1 SystemInformationBlockType21 | offsetDFN allows for values INTEGER (0..1000) but in the field description it is not clear what the values mean | 2 | Refine the field description NOTE: same problem with this field description is in pre-configuration IE as well. |  |
| C.009 | 6.3.1 SystemInformationBlockType5 | If the cell does not support SC-PTM, scptm-FreqOffset may not be broadcasted in SIB5. According to the field description of scptm-FreqOffset and text description in TS 36.304, the UE would definitely reselect to a SC-PTM cell if the UE is interested to receive an MBMS service and the current frequency is included in the USD of the service. Similar as default vaules of cell offset and frequency offset, We suggest that if scptm-FreqOffset is not present, the UE use 0 dB. If it is agreed, the filed description of scptm-FreqOffset in IE SystemInformationBlockType5-NB also needs to be updated. | 3 | Correct the text as follows: scptm-FreqOffset Parameter QoffsetSCPTM in TS 36.304 [4]. Actual value QoffsetSCPTM = field value * 2 [dB]. If the field is not present, the UE uses infinite 0 dBs for the SC-PTM frequency offset with cell ranking as specified in TS 36.304 [4]. | CR/ TDoc XX |
| N.084 | 6.3.1 SystemInformationBlockType5::scptm-FreqOffset | Field description could be clearer | 2 | Change field descfription to: scptm-FreqOffset Parameter QoffsetSCPTM in TS 36.304 [4]. Actual value QoffsetSCPTM = field value * 2 [dB]. If the field is not present, the UE uses infinite dBs for the SC-PTM frequency offset with cell ranking considers the SC-PTM frequency offset with cell ranking to have value “infinite”, as specified in TS 36.304 [4]. | CR |
| H.023 | 6.3.1 SystemInformationBlockType5 | The field description of scptm-FreqOffset indicates to use infinite dBs in absence of the IE. The usual terminology is value of infinity | 2 | Change as follows: scptm-FreqOffset Parameter QoffsetSCPTM in TS 36.304 [4]. Actual value QoffsetSCPTM = field value * 2 [dB]. If the field is not present, the value of infinity shall be used for QoffsetSCPTM. the UE uses infinite dBs for the SC-PTM frequency offset with cell ranking as specified in TS 36.304 [4]. | CR |
| S.025 | 6.3.1 SystemInformationBlockType5 field, cellSelectionInfoCE1 | 2nd sentence of field description seems unclear i.e. is present | 1/ 2 | Remove ‘is present’ | CR |
| 6.3.2 Radio resource control information elements |  |  |  |  |  |
| N.139 | 6.3.2 | AntennaInfoDedicated-v14xy ::= SEQUENCE { ce-ue-TxAntennaSelection-config-r14 ENUMERATED {on} OPTIONAL -- Need OR } SEQUENCE with ENUMERATED with one choice | 2 | Similar problem as in comment N.141. Could be simplified |  |
| M.020 | 6.3.2 bitRateQueryProhibitTimer-r14 | Should be cond DBR or move to DelayBudgetReportingConfig-r14 | 3 | Move to under DelayBudgetReportingConfig-r14 | CR |
| N.136 | 6.3.2 ce-pdsch-maxBandwidth-config-r14 | ce-pdsch-maxBandwidth-config-r14 ENUMERATED {bw5, bw20} OPTIONAL, -- Need OP This IE cannot be released (e.g. by target eNB during handover) | 2 | Change OP to OR as for the other fields. | CR |
| N.137 | 6.3.2 ce-pusch-maxBandwidth-config-r14 | ce-pusch-maxBandwidth-config-r14 ENUMERATED {bw5} OPTIONAL, -- Need OP This IE cannot be released (e.g. by target eNB during handover) | 2 | Change OP to OR as for the other fields | CR |
| S.032 | 6.3.2 CSI-Process | Fields eMIMO-Type2 and eMIMO-Hybrid are related i.e. always need to be configured together | 2 | Introduce E-UTRAN constraint in field description | TDoc eFD-MIMO (Sam) |
| N.094 | 6.3.2 CSI-Process eMIMO-fields added in Rel-14 | Same structure that is used in CSI-RS-Config – use the IE created there | 2 | The following could be used: [[ eMIMO-Config-r14 CSI-RS-Config-v14xy OPTIONAL, -- Need ON ]] Ericsson: This comment seems a mix of CSI-RS process(TM10) and CSI-RS config, original config for TM9. Clarify before introducing suggested correction. | CR |
| I.047 | 6.3.2 CSI-Process field descriptions | Field description of advancedCodebookEnabled can be improved as it is of type BOOLEAN. | 2 | Update field description of advancedCodebookEnabled as shown below. advancedCodebookEnabled Value TRUE indicates thatwhether the UE should use the advanced code book defined in TS 36.213 [23]. EUTRAN does not configure the field when the UE is configured with eMIMO-Type is set to beamformed, when the UE is configured with eMIMO-Hybrid or when the UE is configured with semiOpenLoop. | CR |
| N.099 | 6.3.2 CSI-RS-ConfigBeamformed-r14, CSI-RS-ConfigNonPrecoded | These could be separated into their own IE descriptions since for the most part, these parameters are only referred to and used in RAN1 specifications. | 4 | Discuss how to group the CSI-RS IEs differently to make the specification more readable. | CR/ TDoc WI (Nokia) |
| N.095 | 6.3.2 CSI-RS-ConfigEMIMO:: CSI-RS-ConfigEMIMO-Hybrid-r14 | The comments in ASN.1 seem more relevant as field descriptions since they relate to how the fields should be configured | 3 | Put the following commetns to field description of type1Info-r14 (which doesn’t seem to even exist yet): CSI-RS-ConfigEMIMO-Hybrid-r14 ::= CHOICE { release NULL, setup SEQUENCE { type1Info-r14 SEQUENCE { -- Additonal parameters associated with the 1st EMIMO configuration applicable, -- configured only when a 2nd EMIMO configuration is assigned The field description could/should state the same things, e.g: “Type1Info Indicates additonal parameters associated with the 1st EMIMO configuration applicable. E-UTRAN only configures this field when a 2nd EMIMO configuration is assigned” However, it is not clear what the “1st” and “2nd” EMIMO configurations mean, so that should also be clarified. Ericsson: it is OK to have a field description and move these comments but how it is unclear what is 1sta dn 2nd configuration? It cannot become a concept document or tutorial for FD-MIMO. | CR |
| N.098 | 6.3.2 CSI-RS-ConfigEMIMO::CSI-RS-ConfigBeamformed-r14::ace-For4Tx-PerResourceConfigList-r14 | Field description missing | 2 | Add field description since the name tells pretty much nothing Coordinator (Ericsson): Please propose some field descriptions that can be captured in the CR. |  |
| N.096 | 6.3.2 CSI-RS-ConfigNonPrecoded-v14xy::nzp-ResourceConfigTM9-original-v14xy, | Unnamed SEQUENCE that corresponds to structure existing in another place as well | 2 | Since there is duplication, a acommon IE should be defined: CSI-RS-ConfigNonPrecoded-v14xy::= SEQUENCE { csi-RS-ConfigNZP-EMIMO-v14xy CSI-RS-ConfigNZP-EMIMO-v14xy OPTIONAL, -- Need ON codebookConfigN1-v14xy ENUMERATED {n5, n6, n7, n10, n12, n14, n16}, codebookConfigN2-r14xy ENUMERATED {n5, n6, n7 }, nzp-ResourceConfigTM9-original-r14 NZP-ResourceConfig-r14 OPTIONAL -- Need ON } NZP-ResourceConfig-r14 ::= SEQUENCE { transmissionComb-r14 NZP-TransmissionComb-r14 OPTIONAL, -- Need OR frequencyDensity-r14 NZP-FrequencyDensity-r14 OPTIONAL -- Need OR } NZP-ResourceConfig-r13 ::= SEQUENCE { resourceConfig-r13 ResourceConfig-r13, ..., [[ resourceConfig-v14xy NZP-ResourceConfig-r14 OPTIONAL -- Need ON ]] } Ericsson: ok that there is duplication and something could be done. However, the suggestion is to put the addition for CSI-RS config into NZP CSI-RS config which is bit weird. This is due to Rel-13 choice of using CSI-RS original config for TM9. | CR |
| N.100 | 6.3.2 CSI-RS-ConfigNZP-EMIMO-v14xy | Comments inside ASN.1 suggest using setup-release | 3 | Discuss where to have the setup-release clause, and how that is placed inside the other fields. |  |
| H.033 | 6.3.2 CSI-RS-ConfigNZP-r11 | In Rel-14 FeMBMS carrier, subframe 4 and subframe 9 can be used for MBMS. So the corresponding mbsfn subframe configuration should be extended. | 2 | Add the following change: CSI-RS-ConfigNZP-r11 ::= SEQUENCE { csi-RS-ConfigNZPId-r11 CSI-RS-ConfigNZPId-r11, antennaPortsCount-r11 ENUMERATED {an1, an2, an4, an8}, resourceConfig-r11 INTEGER (0..31), subframeConfig-r11 INTEGER (0..154), scramblingIdentity-r11 INTEGER (0..503), qcl-CRS-Info-r11 SEQUENCE { qcl-ScramblingIdentity-r11 INTEGER (0..503), crs-PortsCount-r11 ENUMERATED {n1, n2, n4, spare1}, mbsfn-SubframeConfigList-r11 CHOICE { release NULL, setup SEQUENCE { subframeConfigList MBSFN-SubframeConfigList } } OPTIONAL -- Need ON } OPTIONAL, -- Need OR ..., [[ csi-RS-ConfigNZPId-v1310 CSI-RS-ConfigNZPId-v1310 OPTIONAL -- Need ON ]], [[ transmissionComb-r14 NZP-TransmissionComb-r14 OPTIONAL, -- Need OR frequencyDensity-r14 NZP-FrequencyDensity-r14 OPTIONAL -- Need OR ]], [[ qcl-CRS-Info-v14xy SEQUENCE { mbsfn-SubframeConfigList-v14xy MBSFN-SubframeConfigList-v14xy OPTIIONAL -- Need OR } ]] } | CR |
| S.033 | 6.3.2 CSI-RS-Config-v14xy | Fields eMIMO-Type2 and eMIMO-Hybrid are related i.e. always need to be configured together | 2 | Introduce E-UTRAN constraint in field description | TDoc eFD-MIMO (Sam) |
| N.131 | 6.3.2 DRB-ToAddMod::lwa-WLAN-AC-r14 | The field should be conditional to the presence of lwa-UL-Config-r14, and UE should delete it in case that field is deleted | 2 | Correct as follows: ASN.1 lwa-WLAN-AC-r14 ENUMERATED {ac-bk, ac-be, ac-vi, ac-vo} OPTIONAL -- Need OPCond UL-LWA Field description: “lwa-WLAN-AC For LWA bearers, indicates the corresponding WLAN access category for uplink. AC-BK (value ac-bk) corresponds to Background access category, AC-BE (value ac-be) corresponds to Best Effort access category, AC-VI (value ac-vi) corresponds to Video access category and AC-VO (value ac-vo) corresponds to Voice access category as defined by IEEE 802.11-2012 [67]. This field is included only when ul-LWA-DRB-ViaWLAN is set to TRUE or ul-LWA-DataSplitThreshold is configured. If lwa-WLAN-AC is not configured, it is left up to UE to decide which IEEE 802.11 AC value to use when performing transmissions of packets for this DRB over WLAN in the uplink.” New condition Cond UL-LWA: The field is optional, Need ON in case lwa-UL-Config-r14 is present. Otherwise the field is not present and the UE shall delete any existing value for this field. | CR |
| N.135 | 6.3.2 enable256QAM | Field description does not list UL UE category 20, which was approved by RAN#75 | 2 | Change UE Categories range from 16-19 to 16-20 | CR |
| N.138 | 6.3.2 enable256QAM-r14 | This field has condition OR and a release/setup choice /two types of release) | 3 | Discuss if both release mechanisms are needed Coordinator (Ericsson): If you have any preference, it would be good to express it. |  |
| N.124 | 6.3.2 LAA config: Format4A and Format4B | Can we have configuration of only format 4A/B without either 0A or 0B | 3 | Currently, any combination of configuration is possible (including having nothing). If we would require conditions, RAN1 should be consulted. Nothing is broken but field descriptions could state that if E-UTRAN should always configured either 0A/B when 4A/B is used. |  |
| H.043 | 6.3.2 LogicalChannelConfig | laa-Allowed-r14 is used to indicate whether UL transmission via LAA SCell is allowed, however, the name of this IE is not that clear | 2 | [[ laa-UL-Allowed-r14 BOOLEAN OPTIONAL, -- Need ON laa-UL-Allowed Indicates whether the data of a logical channel is allowed to be transmitted via UL of LAA SCells. Value TRUE indicates that the logical channel is allowed to be sent via UL of LAA SCells. Value FALSE indicates that the logical channel is not allowed to be sent via UL of LAA SCells. | CR |
| D.002 | 6.3.2 MAC-MainConf | Based on the ASN.1 rule, the IEs which are defined in the same version (i.e., v14.2.0 in this case) should be included in a same brackets ([[ ]]). | 1 | If our understanding is correct, change the definition as follows: [[ skipUplinkTx-r14 CHOICE { release NULL, setup SEQUENCE { skipUplinkTxSPS-r14 ENUMERATED {true} OPTIONAL, -- Need OR skipUplinkTxDynamic-r14 ENUMERATED {true} OPTIONAL -- Need OR } } OPTIONAL -- Need ON ]], [[ dataInactivityTimerConfig-r14 CHOICE { release NULL, setup SEQUENCE { dataInactivityTimer-r14 DataInactivityTimer-r14 } } OPTIONAL -- Need ON ]] Coordinator (Ericsson): Same as N.103. | CR |
| N.103 | 6.3.2 MAC-MainConfig::dataInactivityTimerConfig-r14 | Unnecessary extension addition group – there is already one for Rel-14 extensions | 2 | Remove unnecessary extension addition group Coordinator (Ericsson): Same as D.002. | CR |
| E.029 | 6.3.2 MAC-MainConfig | skipUplinkTxSPS field description has a formulation “E-UTRAN always..” which imposes a network requirement. There should not be any difference if the word “always” is removed. | 2 | Remove the word “always” and reformulate as skipUplinkTxSPS If configured, the UE skips UL transmissions for a configured uplink grant if no data is available for transmission in the UE buffer as described in TS 36.321 [6]. If semiPersistSchedIntervalUL is shorter than sf10, E-UTRAN always configures skipUplinkTxSPS when semiPersistSchedIntervalUL is shorter than sf10. | CR |
| E.132 | 6.3.2 MAC-MainConfig | Unclear relation between skipUplinkTxSPS-r14 and skipUplinkTxDynamic-r14 The following field description seem to suggest that, when configured, the UE will transmit padding on configured UL grants. But the intention of this field is likely that it only relates to whether or not UE can transmit paddin in UL for dynamic grants. skipUplinkTxDynamic If configured, the UE skips UL transmissions for an uplink grant other than a configured uplink grant if no data is available for transmission in the UE buffer as described in TS 36.321 [6] Note that the capability for this reads: Indicates whether the UE supports skipping of UL transmission for an uplink grant indicated on PDCCH if no data is available for transmission as described in TS 36.321 [6]. | 2 | A possible correction can be to do the following or in some other way try to refer to a dynamic grant: skipUplinkTxDynamic If configured, the UE skips UL transmissions for non-configured an uplink grant other than a configured uplink grant if no data is available for transmission in the UE buffer as described in TS 36.321 [6]. | CR |
| N.102 | 6.3.2 MAC-MainConfig::dataInactivityTimerConfig-r14 | Potentially unnecessary SEQUENCE wrapping | 2 | See N.003 for generic discussion on this |  |
| N.104 | 6.3.2 MAC-MainConfig::drx-Config | The role of drx-Config-r13 is unclear (with regards to UL LAA specifically, but also in general) | 3 | It should be clarified whether only the UL retransmission timer (which relates to asynchronous HARQ) applies for UL LAA, or also the OnDuration and ReTx timers Further, there is a potential Rel-13 issue: The drx-Config-r13 is a critical extension, but it seemsit doesn’t contain all the content of DRX-Config (without suffix), and requires presence of that should this have been a non-critical extension anyway originally?? Coordinator (Ericsson): The first question should be clarified or confirmed. The Rel-13 issue needs to be handled in a separate maintenance CR. | TBC |
| N.097 | 6.3.2 Multiple places within CSI-RS configuration | Magic numbers used a lot when defining CSI-RS process etc. sizes | 4 | Magic numbers (e.g. SIZE (1..8)) are used a lot with the CSI-RS-related fields. We should discuss if named constants should be defined for these to make the ASN.1 more readable. Ericsson: it follows the Rel-13 Magic… |  |
| S.034 | 6.3.2 PDCCH-ConfigLAA-r14 | General principle was to use delta signalling restrictively at lower levels i.e. when the field size is not marginal (say 10b can be saved). Here it is used for fields of 2b | 2 | Change enableMonitoring fields to simply need OR (rather than choice setup/ release with optional, need OR) | CR |
| N.105 | 6.3.2 PDCCH-ConfigSCell::enableMonitoringDCI-Format0B-r14, PDCCH-ConfigSCell::enableMonitoringDCI-Format4B-r14 | Potentially unnecessary SEQUENCE wrapping | 2 | See N.003 for generic discussion on this |  |
| N.106 | 6.3.2 PDCCH-ConfigSCell::enableMonitoringDCI-Format0B-r14, PDCCH-ConfigSCell::enableMonitoringDCI-Format4B-r14 | Same value range used – could be a named IE | 2 | Could be replaced to use e.g. type ScheduledSubframes-r14::= ENUMERATED {sf2, sf3, sf4} | CR |
| E.030 | 6.3.2 PDCP-Config | ul-LWA-DRB-ViaWLAN-r14 is defined as a single code-point enumeration with Optional Release. It means that the field is released if it is absent. It would be better to follow the convention to define these simple indicators as boolean Optional No action (ON). | 2 | Change the data type to boolean and the Need code to ON ul-LWA-DRB-ViaWLAN-r14 ENUMERATED {true}BOOLEAN OPTIONAL, -- Need ORON | CR |
| N.107 | 6.3.2 PDCP-Config:: ul-LWA-DRB-ViaWLAN-r14 | The field seems unnecessary | 2 | The presence of the upper level field (UL-LWA-Config) could also be used to indicate that UL over WLAN is used. Hence, this field could be removed entirely. | CR |
| T.005 | 6.3.2 PDCP-Config | For all LWA bearers, there is one LWAAP entity in the eNB and one LWAAP entity in the UE. Therefore, unlike RLC entity, in the description in ul-LWA-DRB-ViaWLAN the “associated” before LWAAP entitly should be removed. | 2 | ul-LWA-DRB-ViaWLAN Indicates whether the UE shall send PDCP PDUs via the associated LWAAP entity as specified in TS 36.323 [8]. E-UTRAN only configures this field (i.e. indicates value TRUE) for LWA DRBs. | CR |
| H.044 | 6.3.2 PDCP-Config | The condition of ul-LWA-DRB-ViaWLAN-r14 is “Need OR”, it should be “Need ON” | 2 | [[ ul-LWA-Config-r14 CHOICE { release NULL, setup SEQUENCE { ul-LWA-DRB-ViaWLAN-r14 ENUMERATED {true} OPTIONAL, -- Need ORON Coordinator (Ericsson): Same as M.017. | CR |
| M.017 | 6.3.2 PDCP-Config | Need code for ul-LWA-DRB-ViaWLAN-r14 is OR, but should be ON to be consistent with uplink DuCo specification | 2 | Change need code for ul-LWA-DRB-ViaWLAN-r14 to ON from OR Coordinator (Ericsson): Same as H.044. | CR |
| M.018 | 6.3.2 PDCP-Config | WT MAC address is specified as part of PDCP config but shoud be common for all LWA bearers | 3 | There are several options: Option 1: Clarify in the field description of wt-MAC-Address-r14 that this value applies on a per UE basis; e.g., something like: This value should be the same for all LWA bearers Option 2: Could be made part of LWAAP configuration Coordinator (Ericsson): If you have any preference, it would be good to express it. | CR/ TDoc WI? |
| N.108 | 6.3.2 PDCP-Config:: ul-LWA-Config-r14:: ul-LWA-DataSplitThreshold-r14 | Shouldn’t this field be mandatory to provide in case UL over WLAN is configured? | 2 | Since the threshold is used to determine when to split, this field should be mandatory present when the UL over WLAN configuration is provided. | CR |
| N.109 | 6.3.2 PDCP-Config:: ul-LWA-Config-r14:: ul-LWA-DataSplitThreshold-r14 | Infinity value is missing, so there is no possibility to force UE to use WLAN link only for UL | 2 | Add “infinity” as the last value in the value range (i.e. replace “spare1” with “infinity”). | CR |
| C.011 | 6.3.2 PDSCH-Config | The name of the IEs are incorrect, including ce-pdsch-maxBandwidth-config-r14, ce-pdsch-tenProcesses-config-r14, ce-HarqAckBundling-config-r14 and ce-schedulingEnhancement-config-r14. ce-pdsch-tenProcesses-config-r14 is not clear to configure ten HARQ processes. | 1 | PDSCH-ConfigDedicated-v14xy ::= SEQUENCE { ce-PDSCH-MaxBandwidth-Config-r14ce-pdsch-maxBandwidth-config-r14 ENUMERATED {bw5, bw20} OPTIONAL, -- Need OP ce-PDSCH-TenHARQProcesses-Config-r14ce-pdsch-tenProcesses-config-r14 ENUMERATED {on} OPTIONAL, -- Need OR ce-HARQACKBundling-Config-r14ce-HarqAckBundling-config-r14 ENUMERATED {on} OPTIONAL, -- Need OR ce-SchedulingEnhancement-Config-r14ce-schedulingEnhancement-config-r14 ENUMERATED {range1, range2} OPTIONAL -- Need OR } | CR |
| C.012 | 6.3.2 PDSCH-Config | The max bandwidth can by configured to 5MHz for BL UEs and 5MHz or 20MHz for UEs in CE. | 2 | Add the clarification in the field description of ce-PDSCH-MaxBandwidth-Config. The max bandwidth can by configured to 5MHz for BL UEs and 5MHz or 20MHz for UEs in CE. | CR |
| H.034 | 6.3.2 PDSCH-Config | Same issue as H.033 (CSI-RS-ConfigNZP-r11) | 2 | Add the following change: PDSCH-RE-MappingQCL-Config-r11 ::= SEQUENCE { pdsch-RE-MappingQCL-ConfigId-r11 PDSCH-RE-MappingQCL-ConfigId-r11, optionalSetOfFields-r11 SEQUENCE { crs-PortsCount-r11 ENUMERATED {n1, n2, n4, spare1}, crs-FreqShift-r11 INTEGER (0..5), mbsfn-SubframeConfigList-r11 CHOICE { release NULL, setup SEQUENCE { subframeConfigList MBSFN-SubframeConfigList } } OPTIONAL, -- Need ON pdsch-Start-r11 ENUMERATED {reserved, n1, n2, n3, n4, assigned} } OPTIONAL, -- Need OP csi-RS-ConfigZPId-r11 CSI-RS-ConfigZPId-r11, qcl-CSI-RS-ConfigNZPId-r11 CSI-RS-ConfigNZPId-r11 OPTIONAL, -- Need OR ..., [[ optionalSetOfFields-v14xySEQUENCE { mbsfn-SubframeConfigList-v14xy MBSFN-SubframeConfigList-v14xy OPTIONAL -- Need OR } ]] } | CR |
| L.023 | 6.3.2 PDSCH-Config | The ce-pdsch-maxBandwidth-config-r14 is using for maximum PDSCH channel bandwidth in CE mode A and B but this bandwidth information is not aligned with previous bandwidth information. | 2 | Change the value as follows; ce-pdsch-maxBandwidth-config-r14 ENUMERATED {bw5 n25, bw20 n100} OPTIONAL, -- Need OP and change the corresponding field desciption as follows; ce-pdsch-maxBandwidth-config Maximum PDSCH channel bandwidth in CE mode A and B, see TS 36.212 [22] and TS 36.213 [23]. Value bw5 corresponds to 5 MHz, and value bw20 corresponds to 20 MHz. If this field is not configured, the maximum PDSCH channel bandwidth in CE mode A and B is set to 1.4 MHz. Parameter: transmission bandwidth configuration, see TS 36.101 [42, table 5.6-1]. n25 corresponds to 25 resource blocks, n100 to 100 resource blocks. | CR |
| N.110 | 6.3.2 PDSCH-Config::PDSCH-ConfigDedicated-v14xy | The fields are not following naming rules, and are quite long | 2 | Several proposals: Remove the suffix “-config” from all fields “pdsch” should be capitalized Starting letter of word should be capitalized Proposal shown below: PDSCH-ConfigDedicated-v14xy ::= SEQUENCE { ce-PDSCH-MaxBandwidth-r14 ENUMERATED {bw5, bw20} OPTIONAL, -- Need OP ce-PDSCH-TenProcesses-r14 ENUMERATED {on} OPTIONAL, -- Need OR ce-HARQ-AckBundling-r14 ENUMERATED {on} OPTIONAL, -- Need OR ce-SchedulingEnhancement-r14 ENUMERATED {range1, range2} OPTIONAL -- Need OR } Coordinator (Ericsson): It is difficult to see how the specification text reads now and what kind of changes are proposed. | CR |
| N.111 | 6.3.2 PDSCH-Config::PDSCH-ConfigDedicated-v14xy:: ce-PDSCH-MaxBandwidth-r14 | Field description should state how the value can be released since Need OP is used | 2 | Proposal: ce-pdsch-maxBandwidth-config Maximum PDSCH channel bandwidth in CE mode A and B, see TS 36.212 [22] and TS 36.213 [23]. Value bw5 corresponds to 5 MHz, and value bw20 corresponds to 20 MHz. If this field is not configured, the maximum PDSCH channel bandwidth in CE mode A and B is set to 1.4 MHz. If this field is not signalled, UE shall release any existing value and set the maximum PDSCH channel bandwidth in CE mode A and B to 1.4 MHz. | CR |
| H.037 | 6.3.2 PerCC-GapIndication | The Note for PerCC-GapIndication is unclear to show what is ‘the same configuration’. | 2 | Add the meaning of same configuration: Note: Within one RRC connection, the UE should maintain the consistent per CC measurement gap preference for the same CA and measurement configuration. | CR |
| E.906 | 6.3.2 PerCC-ListGapIndication | The IE is now defined in section 6.3.2 Radio resource control information elements. Should be 6.3.5, same as MeasGapConfig The field is only used in one place, in RRCConnectionReconfigurationComplete, but a separate field desciption is still moitvated and nice. | 2 | Move IE to 6.3.5 Measurement information elements | CR |
| E.907 | 6.3.2 PerCC-ListGapIndication | Field name not according to naming convention. Should end with -list. Should include “pref”. | 2 | Use PerCC-GapPrefList PerCC-GapPrefListListGapIndication-r14 ::= SEQUENCE (SIZE (1..maxServCell-r13)) OF PerCC-GapIndication-r14 In section 5.3.5.3: 1> set the content of RRCConnectionReconfigurationComplete message as follows: 2> if the RRCConnectionReconfiguration message includes perCC-GapIndicationRequest: 3> include perCC-GapPrefListListGapIndication and numFreqEffective; In section 5.3.5.4: 2> if the RRCConnectionReconfiguration message includes perCC-GapIndicationRequest: 3> include perCC-GapPrefListListGapIndication and numFreqEffective; In section 6.2.2 RRCConnectionReconfiguration, field description: perCC-GapIndicationRequest Indicates that UE shall include perCC-GapPrefListListGapIndication in the RRCConnectionReconfigurationComplete message. In section 6.2.2 RRCConnectionReconfigurationComplete: RRCConnectionReconfigurationComplete-v14xy-IEs ::= SEQUENCE { perCC-GapPrefListListGapIndication-r14 PerCC-GapPrefListListGapIndication-r14 OPTIONAL, numFreqEffective-r14 INTEGER (1..12) OPTIONAL, nonCriticalExtension SEQUENCE {} OPTIONAL } perCC-GapPrefListListGapIndication This field is used to indicate per CC measurement gap preference by the UE. | CR |
| E.908 | 6.3.2 PerCC-ListGapIndication | Reword text The IE PerCC-ListGapIndication is used to specify the UE measurement gap preference. | 2 | The IE PerCC-ListGapIndication is used to indicatespecify the UE measurement gap preference per component carrier (serving cell). | CR |
| E.909 | 6.3.2 PerCC-ListGapIndication | Improve field description text Existing: servCellId This field is used to refer to the cooresponding serving cell ID that the UE indicate the gapIndication. The UE shall indicate all servCellId along with gapIndication for all the serving cells in the configured CA. | 3 | Proposal: servCellId This field identifies the serving cell for which the measurement gap preference is provided. This field is used to refer to the cooresponding serving cell ID that the UE indicate the gapIndication. The UE shall indicate all servCellId along with gapIndication for all the serving cells in the configured CA. | CR |
| E.910 | 6.3.2 PerCC-ListGapIndication | Improve field description text. Existing: gapIndication This field is used to indicate the gap preference by the UE. Value gapIndication of gap cooresponding to gap is needed for that servCellId, value gapIndication of nogap-noNcsg cooresponding to no gap and no ncsg is needed for that servCellId, value gapIndication of ncsg cooresponding to ncsg is needed for that servCellId. | 3 | Proposal: gapIndication This field is used to indicate the measurement gap preference for the component carrier (serving cell). Value gap indicates UE preference to be configured with a measurement gap, value ncsg indicates UE preference to be configured with a ncsg gap pattern, and value nogap-noNcsg indicates UE preference to be configured with no gap pattern. This field is used to indicate the gap preference by the UE. Value gapIndication of gap cooresponding to gap is needed for that servCellId, value gapIndication of nogap-noNcsg cooresponding to no gap and no ncsg is needed for that servCellId, value gapIndication of ncsg cooresponding to ncsg is needed for that servCellId. | CR |
| E.911 | 6.3.2 PerCC-ListGapIndication | Order of the enumerated values should be in more logical order. gapIndication-r14 ENUMERATED {gap, nogap-noNcsg, ncsg, spare}, | 2 | Proposal: gapIndication-r14 ENUMERATED {gap, ncsg, nogap-noNcsg, ncsg, spare}, | CR |
| E.912 | 6.3.2 PerCC-ListGapIndication | Move and reformulate the marked text to initial text for PerCC-ListGapIndication Existing: servCellId This field is used to refer to the cooresponding serving cell ID that the UE indicate the gapIndication. The UE shall indicate all servCellId along with gapIndication for all the serving cells in the configured CA. | 2 | Proposal: – PerCC-ListGapIndication The IE PerCC-ListGapIndication is used to specify the UE measurement gap preference. The UE shall indicate measurement gap preference for all serving cells in the configuration. servCellId This field is used to refer to the cooresponding serving cell ID that the UE indicate the gapIndication. The UE shall indicate all servCellId along with gapIndication for all the serving cells in the configured CA. | CR |
| E.133 | 6.3.2 PerCC-ListGapIndication | Unclear note. A note says the following which is not clear: Note: Within one RRC connection, the UE should maintain the consistent per CC measurement gap preference for the same configuration | 2 | We assume it intends to say: Note: Within one RRC connection for a UE, the gap preference does not change for a certain configuration. Within one RRC connection, the UE should maintain the consistent per CC measurement gap preference for the same configuration | CR |
| I.046 | 6.3.2 PerCC-ListGapIndication | The field description of servCellId and gapIndication can be improved. | 2 | Update the field description of servCellId and gapIndication as shown below. servCellId This field is used to refer to the corresponding serving cell ID that the UE indicates the gapIndication. The UE shall indicate allthe servCellId along with gapIndication for all the serving cells in the configured CA. gapIndication This field is used to indicate the measurement gap preference by the UE. Value gapIndication of gap cooresponding to gap indicates that a measurement gap is needed for thatthe associated servCellId, value gapIndication of nogap-noNcsg cooresponding to no indicates that neither a measurement gap and no nor a ncsg is needed for thatthe associated servCellId, value gapIndication of ncsg cooresponding to indicates that ncsg is needed for thatthe associated servCellId. | CR |
| N.115 | 6.3.2 PerCC-ListGapIndication | Is extendibility needed for the IE? | 2 | Discuss if the field should be extendible |  |
| N.116 | 6.3.2 PerCC-ListGapIndication | NOTE is not properly worded and seems more like normative behaviour that should be captured in procedural text | 3 | If kept, the NOTE should be in caps, with some wording clarifications. Alternatively, this could be put as normative text to field description, e.g. as follows: gapIndication This field is used to indicate the gap preference by the UE. Value gapIndication of gap cooresponding to gap is needed for that servCellId, value gapIndication of nogap-noNcsg cooresponding to no gap and no ncsg is needed for that servCellId, value gapIndication of ncsg cooresponding to ncsg is needed for that servCellId. The UE shall indicate the per CC measurement gap preference consistenly for the same configuration during the same RRC connection. Alternatively, the NOTE could read as follows. NOTE: Within one RRC connection, Tthe UE should maintain the consistent per CC measurement gap preference for the same configuration during the same RRC connection. |  |
| N.112 | 6.3.2 PerCC-ListGapIndication::gapIndication-r14 | Enum names could be more descriptive | 2 | Use ENUMERATED {none, measGap, ncsg, spare}, Instead of ENUMERATED {gap, nogap-noNcsg, ncsg, spare}, | CR |
| N.113 | 6.3.2 PerCC-ListGapIndication::gapIndication-r14 | Is the spare value needed? What should eNB do if it receives a (future UE’s) signalling with spare value? | 2 | Remove possibility to signal spare value (unless a specific reason for having it is identified) | CR |
| N.114 | 6.3.2 PerCC-ListGapIndication::gapIndication-r14 | The field description is long and difficult to understand. | 2 | Use the following field description: gapIndication This field is used to indicate the gap preference by the UE for a serving carrier indicated by servCellId. Value gap corresponds normal measurement gap, value none corresponds to no gap or ncsg, and value ncsg corresponds to ncsg. gapIndication This field is used to indicate the gap preference by the UE. Value gapIndication of gap cooresponding to gap is needed for that servCellId, value gapIndication of nogap-noNcsg cooresponding to no gap and no ncsg is needed for that servCellId, value gapIndication of ncsg cooresponding to ncsg is needed for that servCellId. | CR |
| C.013 | 6.3.2 PhysicalConfigDedicated | Name of IE ce-ue-TxAntennaSelection-config is incorrect. | 1 | AntennaInfoDedicated-v14xy ::= SEQUENCE { ce-UE-TxAntennaSelection-Config-r14ce-ue-TxAntennaSelection-config-r14 ENUMERATED {on} OPTIONAL -- Need OR } | CR |
| C.014 | 6.3.2 PhysicalConfigDedicated | The name of the IEs including ce-pusch-nb-maxTbs-config-r14 and ce-pusch-maxBandwidth-config-r14 are incorrect. | 1 | Update the name of IEs and the field descriptions in field descriptions. PUSCH-ConfigDedicated-v14xy ::= SEQUENCE { ce-PUSCH-NB-MaxTBS-Config-r14ce-pusch-nb-maxTbs-config-r14 ENUMERATED {on} OPTIONAL, -- Need OR ce-PUSCH-MaxBandwidth-Config-r14ce-pusch-maxBandwidth-config-r14 ENUMERATED {bw5} OPTIONAL -- Need OP } | CR |
| E.124 | 6.3.2 PhysicalConfigDedicated | Parameter semiopenloop has following field description: semiOpenLoop Indicates whether semi-open-loop transmission is used for deriving CSI reporting and corresponding PDSCH transmission (DMRS). It should be clarified which all possible prior configurations UE needs to ignore when this is given | 3 | Would need some discussions. Samsung said they would provide rel-14 CR for remaining MIMO. This migth be in it as this has been on table also earlier. | CR/ TBC |
| E.125 | 6.3.2 PhysicalConfigDedicated | CSI-RS-Config-v14xy is not given to UE in PhysicalConfigDedicated or PhysicalConfigDedicatedSCell | 2 | Add CSI-RS-Config-v14xy to PhysicalConfigDedicated and PhysicalConfigDedicatedSCell csi-RS-ConfigZP-Ap-r14 CSI-RS-ConfigZP-Ap-r14 OPTIONAL, -- Need ON semiOpenLoop-r14 BOOLEAN OPTIONAL -- Need ON csi-rs-config-v14xy CSI-RS-Config-v14xy OPTIONAL, -- Need ON Additional fix required to both Rel-14 and rel-13 for supporting TM9 and Class B: that TM9 can be configured with NZP config and that the additional configurations that are not present in CSI-RS config are ignored, like QCL and scramblingID. Further the the Pc parameter is given twice for the original resource as it is given both in CSI-RS config and by P-C-AndCBSR. (handled by separate CRs?) | CR/ TBC |
| N.092 | 6.3.2 CSI-RS-Config-v14xy | This IE is nowhere used (referenced). | 3 | Correct the field usage or remove it entirely Ericsson: it needs to be added in physical config. See E.125. |  |
| E.033 | 6.3.2 PhysicalConfigDedicated | SoundingRS-AperiodicSet-r14 and SoundingRS-AperiodicSetUpPTsExt-r14 contain legacy fields. The whole content of these information elements should be -r14. Otherwise the information elements are not needed. | 1 | Change the release suffix to -r14 soundingRS-UL-ConfigDedicatedAperiodic-r10-r14 SoundingRS-UL-ConfigDedicatedAperiodic-r10 } and soundingRS-UL-ConfigDedicatedAperiodicUpPTsExt-r13-r14 SoundingRS-UL-ConfigDedicatedAperiodicUpPTsExt-r13 } Coordinator (Ericsson): soundingRS-UL-ConfigDedicatedAperiodic is corrected during v14.2.0 CR implementation. ConfigDedicatedAperiodicUpPTsExt still needs to be fixed. Same as N.121 and N.122. | CR |
| N.121 | 6.3.2 PhysicalConfigDedicated:: SoundingRS-AperiodicSet-r14:: soundingRS-UL-ConfigDedicatedAperiodic-r10 | Wrong suffix | 2 | Use -r14 instead of -r10 Coordinator (Ericsson): Same as E.033. | CR |
| N.122 | 6.3.2 PhysicalConfigDedicated:: SoundingRS-AperiodicSetUpPTsExt-r14:: soundingRS-UL-ConfigDedicatedAperiodicUpPTsExt-r13 | Wrong suffix | 2 | Use -r14 instead of -r13 Coordinator (Ericsson): Same as E.033. | CR |
| H.038 | 6.3.2 PhysicalConfigDedicated | Some fields have the same name but different definition for PCell and SCell, e.g. soundingRS-UL-AperiodicConfigDedicatedList-r14 and soundingRS-UL-ConfigDedicatedApUpPTsExtList-r14 (added by rapporteur). Although they are in different structures, they may cause confusion | 2 | Change soundingRS-UL-AperiodicConfigDedicatedList-r14 to soundingRS-UL-AperiodicConfigDedicatedListSCell-r14, and soundingRS-UL-ConfigDedicatedApUpPTsExtListSCell-r14, in PhysicalConfigDedicatedSCell. And add the corresponding field description as follows: soundingRS-UL-AperiodicConfigDedicatedListSCell Indicates aperiodic soundingRS configuration except for the extension sounding symbols of the UpPTs subframe for this serving cell. soundingRS-UL-ConfigDedicatedApUpPTsExtListSCell Indicates aperiodic soundingRS configuration in extension sounding symbols of the UpPTs subframe for this serving cell. | CR |
| H.039 | 6.3.2 PhysicalConfigDedicated | The field description of srsCcSetIndexList is not correct, because it is included in soundingRS-UL-AperiodicConfigDedicatedList and soundingRS-UL-ConfigDedicatedApUpPTsExtList, not in soundingRS-UL-ConfigDedicated. | 2 | srsCcSetIndexList Indicate the srsCcSetIndex list which the soundingRS-UL-ConfigDedicatedsoundingRS-UL-AperiodicConfigDedicatedList and soundingRS-UL-ConfigDedicatedApUpPTsExtList belongs to. | CR |
| H.040 | 6.3.2 PhysicalConfigDedicated | When the number of SCells is small than or equal to 5, Type B trigger included in PhysicalConfigDedicatedSCell-r10 is configured; else Type A trigger included in PhysicalConfigDedicated is configured and typeB configuration should be ignored. And TypeA configuration and Type B configuration will not be configured simultaneously. So some clarification should be added. | 2 | Add the following field description for typeA-SRS-TPC-PDCCH-Group-r14: typeA-SRS-TPC-PDCCH-Group-r14 Indicates Type A trigger configuration for SRS transmission on a PUSCH-less SCell. E-UTRAN configures the UE with either typeA-SRS-TPC-PDCCH-Group-r14 or typeB-SRS-TPC-PDCCH-Group-r14, if any. If typeA-SRS-TPC-PDCCH-Group-r14 is present, the UE ignores typeB-SRS-TPC-PDCCH-Group-r14. | CR |
| L.024 | 6.3.2 PhysicalConfigDedicated | The ce-pdsch-puschEnhancement-config-r14 has missing comma. | 1 | Correct the IE as follows; ce-pdsch-puschEnhancement-config-r14 ENUMERATED {on} OPTIONAL, -- Need OR | CR |
| M.019 | 6.3.2 PhysicalConfigDedicated | Editorial correction of field descriptions of energyDetectionThresholdOffs and maxEnergyDetectionThreshold | 1 | Change “Unit in dB” to Units in dB | CR |
| I.048 | 6.3.2 PhysicalConfigDedicated field descriptions | Field description of semiOpenLoop can be improved as it is of type BOOLEAN. | 2 | Update field description of semiOpenLoop as shown below. semiOpenLoop Value TRUE indicates thatwhether semi-open-loop transmission is used for deriving CSI reporting and corresponding PDSCH transmission (DMRS). | CR |
| S.035 | 6.3.2 PhysicalConfigDedicated(SCell) | Some conditions (e.g. Cross-Carrier-ConfigUL) are inapproriate as related to the setting of other configuration parameters, regardless of whether those fields are included in same message | 3? | Specify as E-UTRAN constraint in the fiel description Coordinator (Ericsson): Classification is missing. It should probably be 3. A proposal would be useful to have. Otherwise it is more difficult to capture the constraint in the CR. | CR |
| N.119 | 6.3.2 PhysicalConfigDedicated: antennaInfo-v14xy, pucch-ConfigDedicated-v14xy, pusch-ConfigDedicated-v14xy, pdsch-ConfigDedicated-v14xy, pusch-EnhancementsConfig-r14, csi-RS-ConfigZP-Ap-r14, typeB-SRS-TPC-PDCCH-Config-r14, uplinkPUSCH-LessPowerControlDedicated-v14xy | Fields cannot be released once configured | 3 | Use Need OR or create release-setup-wrapper for the fields (may require grouping; See also N.002) |  |
| N.118 | 6.3.2 PhysicalConfigDedicated: Cond Srs-Trigger-TypeA | Condition is unclearly defined | 2 | Use the name “SRS-Trigger-TypeA” and the following text: The field is mandatory present if typeA-SRS-TPC-PDCCH-Group-r14 is configuredpresent. Otherwise any existing value for this field is deleted the field is not present and UE shall delete any existing value for this field. | CR |
| N.125 | 6.3.2 PhysicalConfigDedicated:: absenceOfAnyOtherTechnology | Parameter names should normally indicate presence, not absence. | 2 | Use “presenceOfAnyOtherTechnology” and change field description to presenceabsenceOfAnyOtherTechnology Presence of this field indicates the potential presence of any other technology sharing the carrier, as specified in TS 36.213 [23]; absence of this field indicates absence on a long term basis (e.g. by level of regulation) of any other technology sharing the carrier. Presence of this field indicates absence on a long term basis (e.g. by level of regulation) of any other technology sharing the carrier; absence of this field indicates the potential presence of any other technology sharing the carrier, as specified in TS 36.213 [23]. | CR |
| N.120 | 6.3.2 PhysicalConfigDedicated:: crossCarrierSchedulingConfig-UL-r14 | Potentially unnecessary SEQUENCE wrapping | 2 | See N.003 for generic discussion on this |  |
| N.117 | 6.3.2 PhysicalConfigDedicated::typeA-SRS-TPC-PDCCH-Group-r14 | Field cannot be released once configured | 2 | Use Need OR or create release-setup-wrapper for the field | CR |
| H.035 | 6.3.2 PRACH-Config | The terminology in the field description for rootSequenceIndexHighSpeed and zeroCorrelationZoneConfigHighSpeed is not aligned with RAN1 spec. RAN1 uses ‘restricted set type B’ in TS 36.211 to indicate performance enhancement restricted set. | 2 | Align the terminology with RAN1 spec: rootSequenceIndexHighSpeed The field indicates starting logical root sequence index used to derive the 64 random access preambles based on performance enhancement restricted set type B in high speed scenario, see TS 36.211 [21, 5.7.2]. If this field is present, the UE shall generate random access preambles based on performance enhancement restricted set type B. zeroCorrelationZoneConfigHighSpeed The field indicates NCS configuration for the performance enhancement restricted set type B in high speed scenario, see TS 36.211 [21, 5.7.2]. If this field is present, the UE shall generate random access preambles based on performance enhancement restricted set type B. | CR |
| H.036 | 6.3.2 PRACH-Config | If rootSequenceIndexHighSpeed and zeroCorrelationZoneConfigHighSpeed are present, the legacy parameters i.e. rootSequenceIndex and zeroCorrelationZoneConfig should be ignored. | 2 | Clarify the UE behaviour with legacy parameters: rootSequenceIndexHighSpeed The field indicates starting logical root sequence index used to derive the 64 random access preambles based on performance enhancement restricted set in high speed scenario, see TS 36.211 [21, 5.7.2]. If this field is present, the UE shall generate random access preambles based on performance enhancement restricted set and ignore rootSequenceIndex. zeroCorrelationZoneConfigHighSpeed The field indicates NCS configuration for the performance enhancement restricted set in high speed scenario, see TS 36.211 [21, 5.7.2]. If this field is present, the UE shall generate random access preambles based on performance enhancement restricted set and ignore zeroCorrelationZoneConfig. | CR |
| L.025 | 6.3.2 PUCCH-Config | The pucch-NumRepetitionCE description is not familiar with previous description in TS-36.331. Some words are needed to be change others which are already using in the specification. | 1 | change the field desciption as follows; pucch-NumRepetitionCE Number of PUCCH repetitions for PUCCH format 1/1a and for PUCCH format 2/2a/2b for CE modes A and B, see TS 36.211 [21] and TS 36.213 [23]. EUTRAN does not configure pucch-NumRepetitionCE-format2-r13 for CE mode B in this release of specification. For UE UEs in CE mode B supporting extended PUCCH repetition, if pucch-NumRepetitionCE-format1-r14 is included signalled then the UE shall ignore it overrides pucch-NumRepetitionCE-format1-r13. | CR |
| I.021 | 6.3.2 PUCCH-Config field descriptions | In the field description of pucch-NumRepetitionCE the highlighted part can be improved: references to field names should be set to italics and using term “overrides” is not consistent with current spec. pucch-NumRepetitionCE Number of PUCCH repetitions for PUCCH format 1/1a and for PUCCH format 2/2a/2b for CE modes A and B, see TS 36.211 [21] and TS 36.213 [23]. The UE shall ignore pucch-NumRepetitionCE-format2-r13, if received, for CE mode B in this release of specification. For UEs in CE mode B supporting extended PUCCH repetition, if pucch-NumRepetitionCE-format1-r14 is signalled then it overrides pucch-NumRepetitionCE-format1-r13. | 2 | Correct the field description of pucch-NumRepetitionCE as shown below. pucch-NumRepetitionCE Number of PUCCH repetitions for PUCCH format 1/1a and for PUCCH format 2/2a/2b for CE modes A and B, see TS 36.211 [21] and TS 36.213 [23]. The UE shall ignore pucch-NumRepetitionCE-format2-r13, if received, for CE mode B in this release of specification. ForThe UEUEs in CE mode B supporting extended PUCCH repetition shall use, if pucch-NumRepetitionCE-format1-r14 is, if signalled, and ignorethen it overrides pucch-NumRepetitionCE-format1-r13. | CR |
| I.022 | 6.3.2 PUCCH-Config field descriptions | In the field description below the highlighted part can be improved: references to field names should be set to italics, using term “overrides” is not consistent with current spec, and presence of pucch-NumRepetitionCE-Msg4-Level3-r13 is optional. pucch-NumRepetitionCE-Msg4-Level0, pucch-NumRepetitionCE-Msg4-Level1, pucch-NumRepetitionCE-Msg4-Level2, pucch-NumRepetitionCE-Msg4-Level3 Number of repetitions for PUCCH carrying HARQ response to PDSCH containing Msg4 for PRACH CE levels 0, 1, 2 and 3, see TS 36.211 [21] and TS 36.213 [23]. Value n1 corresponds to 1 repetition, value n2 corresponds to 2 repetitions, and so on. For BL UEs or non-BL UEs in enhanced coverage supporting extended PUCCH repetition, if pucch-NumRepetitionCE-Msg4-Level3-r14 is signalled then it overrides pucch-NumRepetitionCE-Msg4-Level3-r13. | 2 | Correct the field description as shown below. pucch-NumRepetitionCE-Msg4-Level0, pucch-NumRepetitionCE-Msg4-Level1, pucch-NumRepetitionCE-Msg4-Level2, pucch-NumRepetitionCE-Msg4-Level3 Number of repetitions for PUCCH carrying HARQ response to PDSCH containing Msg4 for PRACH CE levels 0, 1, 2 and 3, see TS 36.211 [21] and TS 36.213 [23]. Value n1 corresponds to 1 repetition, value n2 corresponds to 2 repetitions, and so on. For BL UEs or non-BL UEs in enhanced coverage supporting extended PUCCH repetition, if pucch-NumRepetitionCE-Msg4-Level3-r14 is signalled then it overrides pucch-NumRepetitionCE-Msg4-Level3-r13. The BL UE or non-BL UE in enhanced coverage supporting extended PUCCH repetition shall use pucch-NumRepetitionCE-Msg4-Level3-r14, if signalled, and ignore pucch-NumRepetitionCE-Msg4-Level3-r13, if signalled. | CR |
| N.127 | 6.3.2 PUCCH-Config::pucch-NumRepetitionCE, pucch-NumRepetitionCE-Msg4-Level0, pucch-NumRepetitionCE-Msg4-Level1, pucch-NumRepetitionCE-Msg4-Level2, pucch-NumRepetitionCE-Msg4-Level3 | Field descriptions doesn’t follow usual conventions to indicate override actions on presence of original and extended field | 2 | Correct to “pucch-NumRepetitionCE Number of PUCCH repetitions for PUCCH format 1/1a and for PUCCH format 2/2a/2b for CE modes A and B, see TS 36.211 [21] and TS 36.213 [23]. The UE shall ignore pucch-NumRepetitionCE-format2-r13, if received, for CE mode B in this release of specification. For UEs in CE mode B supporting extended PUCCH repetition, if pucch-NumRepetitionCE-format1-r14 is signalled, the UE shall ignore pucch-NumRepetitionCE-format1-r13.” And “pucch-NumRepetitionCE-Msg4-Level0, pucch-NumRepetitionCE-Msg4-Level1, pucch-NumRepetitionCE-Msg4-Level2, pucch-NumRepetitionCE-Msg4-Level3 Number of repetitions for PUCCH carrying HARQ response to PDSCH containing Msg4 for PRACH CE levels 0, 1, 2 and 3, see TS 36.211 [21] and TS 36.213 [23]. Value n1 corresponds to 1 repetition, value n2 corresponds to 2 repetitions, and so on. For BL UEs or non-BL UEs in enhanced coverage supporting extended PUCCH repetition, if pucch-NumRepetitionCE-Msg4-Level3-r14 is signalled, the UE shall ignore pucch-NumRepetitionCE-Msg4-Level3-r13.” | CR |
| I.049 | 6.3.2 PUSCH-Config | Field description of ul-DMRS-IFDMA can be improved as it is of type BOOLEAN. | 2 | Update field description of ul-DMRS-IFDMA as shown below. ul-DMRS-IFDMA Value TRUE indicates thatwhether the UE is configured with enhanced UL DMRS. | CR |
| I.050 | 6.3.2 PUSCH-Config | Field name dmrs-LessUpPTS-r14 is used for PUSCH configuration in UpPTS in IE TDD-PUSCH-UpPTS-r14 and as capability name as well. But this should be avoided to avoid ambiguity. | 2 | Rename dmrs-LessUpPTS-r14 to dmrs-LessUpPTS-Config-r14 in ASN.1 and field description in IE PUSCH-Config. | CR |
| L.026 | 6.3.2 PUSCH-Config | The ce-pusch-maxBandwidth-config is using for maximum PUSCH channel bandwidth in CE mode A but this bandwidth information is not aligned with previous bandwidth information. | 2 | Change the value as follows; ce-pusch-maxBandwidth-config-r14 ENUMERATED {n25 bw5} OPTIONAL -- Need OP and change the corresponding field desciption as follows; ce-pusch-maxBandwidth-config Maximum PUSCH channel bandwidth in CE mode A, see TS 36.212 [22] and TS 36.213 [23]. Value bw5 corresponds to 5 MHz. If this field is not configured, the maximum PUSCH channel bandwidth in CE mode A set to 1.4 MHz. The maximum PUSCH channel bandwidth in CE mode B is 1.4 MHz regardless of the setting of this parameter. Parameter: transmission bandwidth configuration, see TS 36.101 [42, table 5.6-1]. n25 corresponds to 25 resource blocks. | CR |
| N.129 | 6.3.2 PUSCH-ConfigDedicated-v14xy:: ce-pusch-nb-maxTbs-config-r14 | The field description could be improved and the field name doesn’t follow ASN.1 naming conventions | 2 | Change to: ce-pusch-nb-maxTbsPUSCH-NB-MaxTBS-r14 ENUMERATED {on} OPTIONAL, -- Need OR and ce-pusch-nb-maxTbsPUSHC-NB-MaxTBS Indicates use Activation of 2984 bits maximum PUSCH TBS in 1.4 MHz in CE mode A, see TS 36.212 [22] and TS 36.213 [23]. | CR |
| N.128 | 6.3.2 PUSCH-ConfigDedicated-v14xy::ce-pusch-maxBandwidth-config-r14 | The field could also explicitly indicate both 1.4 and 5 MHz, and should follow ASN.1 naming conventions | 2 | Several points: 1) The suffix “-config” seems unnecessary 2) The field is OP but indicates UE assumes 1.4 MHz on absence – if the field was mandatory, it would consume same amount of bits and it would be easier to understand the contents. E.g. like this: ce-pusch-mPUSCH-MaxBandwidth-r14 ENUMERATED {bw1dot4, bw5} OPTIONAL, -- Need OP 3) With 2), the field description could be simp-lified as follows: “ce-pusch-mPUSCH-MaxBandwidth Maximum PUSCH channel bandwidth in CE mode A, see TS 36.212 [22] and TS 36.213 [23]. Value bw5 corresponds to 5 MHz and value bw1dot4 correspond to 1.4 MHz. The maximum PUSCH channel bandwidth in CE mode B is 1.4 MHz regardless of the setting of this parameter.” | CR |
| H.097 | 6.3.2 RadioResourceConfigCommonSCell-r10 | Same issue as H.033 | 2 | Add the following field: RadioResourceConfigCommonSCell-r10 ::= SEQUENCE { -- DL configuration as well as configuration applicable for DL and UL nonUL-Configuration-r10 SEQUENCE { -- 1: Cell characteristics dl-Bandwidth-r10 ENUMERATED {n6, n15, n25, n50, n75, n100}, -- 2: Physical configuration, general antennaInfoCommon-r10 AntennaInfoCommon, mbsfn-SubframeConfigList-r10 MBSFN-SubframeConfigList OPTIONAL, -- Need OR -- 3: Physical configuration, control phich-Config-r10 PHICH-Config, -- 4: Physical configuration, physical channels pdsch-ConfigCommon-r10 PDSCH-ConfigCommon, tdd-Config-r10 TDD-Config OPTIONAL -- Cond TDDSCell }, -- UL configuration ul-Configuration-r10 SEQUENCE { ul-FreqInfo-r10 SEQUENCE { ul-CarrierFreq-r10 ARFCN-ValueEUTRA OPTIONAL, -- Need OP ul-Bandwidth-r10 ENUMERATED {n6, n15, n25, n50, n75, n100} OPTIONAL, -- Need OP additionalSpectrumEmissionSCell-r10 AdditionalSpectrumEmission }, p-Max-r10 P-Max OPTIONAL, -- Need OP uplinkPowerControlCommonSCell-r10 UplinkPowerControlCommonSCell-r10, -- A special version of IE UplinkPowerControlCommon may be introduced -- 3: Physical configuration, control soundingRS-UL-ConfigCommon-r10 SoundingRS-UL-ConfigCommon, ul-CyclicPrefixLength-r10 UL-CyclicPrefixLength, -- 4: Physical configuration, physical channels prach-ConfigSCell-r10 PRACH-ConfigSCell-r10 OPTIONAL, -- Cond TDD-OR-NoR11 pusch-ConfigCommon-r10 PUSCH-ConfigCommon } OPTIONAL, -- Need OR ..., [[ ul-CarrierFreq-v1090 ARFCN-ValueEUTRA-v9e0 OPTIONAL -- Need OP ]], [[ rach-ConfigCommonSCell-r11 RACH-ConfigCommonSCell-r11 OPTIONAL, -- Cond ULSCell prach-ConfigSCell-r11 PRACH-Config OPTIONAL, -- Cond UL tdd-Config-v1130 TDD-Config-v1130 OPTIONAL, -- Cond TDD2 uplinkPowerControlCommonSCell-v1130 UplinkPowerControlCommonSCell-v1130 OPTIONAL -- Cond UL ]], [[ pusch-ConfigCommon-v1270 PUSCH-ConfigCommon-v1270 OPTIONAL -- Need OR ]], [[ pucch-ConfigCommon-r13 PUCCH-ConfigCommon OPTIONAL, -- Cond UL uplinkPowerControlCommonSCell-v1310 UplinkPowerControlCommonSCell-v1310 OPTIONAL -- Cond UL ]], [[mbsfn-SubframeConfigList-v14xy MBSFN-SubframeConfigList-v14xy OPTIONAL-- Need OR ]] } | CR |
| E.134 | 6.3.2 RadioResourceConfigDedicated | Unclear field description. Currently the field description for lwip-DL-Aggregation, lwip-UL-Aggregation says: Indicates whether LWIP is configured to utilize LWIP aggregation in DL or UL. | 2 | It should be Indicates whether LWIP is configured to utilize LWIP aggregation in DL and UL, respectively. | CR |
| N.132 | 6.3.2 RLC-Config-v14xy::pollByte-r14 | Potentially unnecessary SEQUENCE wrapping | 2 | See N.003 for generic discussion on this |  |
| N.133 | 6.3.2 SoundingRS-UL-ConfigDedicatedAperiodic-v14xy:: srs-SubframeIndication-r14 | Potentially unnecessary SEQUENCE wrapping | 2 | See N.003 for generic discussion on this |  |
| L.038 | 6.3.2 SPS-Config | The description for the case semiPersistSchedIntervalUL and semiPersistSchedIntervalUL-v14x0 are configured is necessary. | 2 | semiPersistSchedIntervalUL Semi-persistent scheduling interval in uplink, see TS 36.321 [6]. Value in number of sub-frames. Value sf10 corresponds to 10 sub-frames, sf20 corresponds to 20 sub-frames and so on. For TDD, when the configured Semi-persistent scheduling interval is greater than or equal to 10 sub-frames, the UE shall round this parameter down to the nearest integer (of 10 sub-frames), e.g. sf10 corresponds to 10 sub-frames, sf32 corresponds to 30 sub-frames, sf128 corresponds to 120 sub-frames. If semiPersistSchedIntervalUL-v14x0 is configured, the UE only considers this extension (and ignores semiPersistSchedIntervalUL i.e. without suffix). | CR |
| L.039 | 6.3.2 SPS-Config | Rearrange the fields in field description in alphabetical order | 1 | semiPersistSchedIntervalSL should be placed before semiPersistSchedIntervalUL | CR |
| H.041 | 6.3.2 SRS-TPC-PDCCH-Config | According to the agreements in RAN1#86bis, the SRS trigger field is ‘Per UE configured size’ and TPC is also ‘UE specifically configured 1 bit or 2 bits’. But in current spec. fieldTypeFormat3B-r14 is configured per SCell in current spec, then eNB may configure different value for different SCell, that is not aligned with RAN1 agreements. | 3 | Option 1: Add clarification in field description: fieldTypeFormat3B The type of a field within the group DCI with SRS request fields (optional) and TPC commands for a PUSCH-Less SCell, which indicates how many bits in the field are for SRS request (0 or 1/2) and how many bits in the field are for TPC (1 or 2). Note that for Type A, there is a common SRS request field for all SCells in the set, but each SCell has its own TPC command bits. See TS 36.212 [22, 5.3.3.1.7A]. It should be configured with the same value for all PUSCH-less SCells. Option 2: move fieldTypeFormat3B-r14 into PhysicalConfigDedicated, and move the corresponding field description to PhysicalConfigDedicated: [[ typeA-SRS-TPC-PDCCH-Group-r14 SEQUENCE (SIZE (1..32)) OF SRS-TPC-PDCCH-Config-r14 OPTIONAL, -- Need ON fieldTypeFormat3B-r14 INTEGER (1..4) OPTIONAL, -- Need ON SRS-TPC-PDCCH-Config-r14 ::= CHOICE { release NULL, setup SEQUENCE { srs-TPC-RNTI-r14 BIT STRING (SIZE (16)), startingBitOfFormat3B-r14 INTEGER (0..31), fieldTypeFormat3B-r14 INTEGER (1..4), srsCcSetGroupIndexlist SEQUENCE (SIZE(1..4)) OF SrsCcSetGroupIndex OPTIONAL -- Cond Srs-Trigger-TypeA } } We prefer Option2, and we would like provide CR on it. | CR |
| E.033b | 6.3.2 TDD-Config | The field description of specialSubframePatterns has a sentence that starts with “And”. This is not really proper use of specification. | 1 | Remove the word “And” specialSubframePatterns Indicates Configuration as in TS 36.211 [21, table 4.2-1] where ssp0 points to Configuration 0, ssp1 to Configuration 1 etc. Value ssp7 points to Configuration 7 for extended cyclic prefix and, value ssp9 points to Configuration 9 for normal cyclic prefix and value ssp10 points to Configration 10 for normal cyclic prefix. E-UTRAN signals ssp7 only when setting specialSubframePatterns (without suffix i.e. the version defined in REL-8) to ssp4. E-UTRAN signals value ssp9 only when setting specialSubframePatterns (without suffix) to ssp5. E-UTRAN signals value ssp10 only when setting specialSubframePatterns (without suffix) to ssp0 or ssp5. If specialSubframePatterns-v1130 or specialSubframePatterns-v14xy is present, the UE shall ignore specialSubframePatterns (without suffix).) . And ifIf both specialSubframePatterns-v1130 and specialSubframePatterns-v14xy are present, the UE shall ignore specialSubframePatterns-v1130. | CR |
| H.042 | 6.3.2 UplinkPowerControl | The implementation of R2-1702211 is not correct. | 1 | The following field description should be updated according to R2-1702211 alpha Parameter: αSRS. See TS 36.213 [23, 5.1.31.1] where al0 corresponds to 0, al04 corresponds to value 0.4, al05 to 0.5, al06 to 0.6, al07 to 0.7, al08 to 0.8, al09 to 0.9 and al1 corresponds to 1. This field applies for uplink power control subframe set 1 if uplink power control subframe sets are configured by tpc-SubframeSet. alpha-SRS Parameter: αSRS See TS 36.213 [23, 5.1.13.1] where al0 corresponds to 0, al04 corresponds to value 0.4, al05 to 0.5, al06 to 0.6, al07 to 0.7, al08 to 0.8, al09 to 0.9 and al1 corresponds to 1. This field applies for SRS power control on a PUSCH-less SCell. | CR |
| 6.3.3 Security control information elements |  |  |  |  |  |
| 6.3.4 Mobility control information elements |  |  |  |  |  |
| S.036 | 6.3.4 MobilityControlInfo | Some changes/ alignment of need codes seem desirable and some condtions seem appropriate | 2 | Need ON should be used for mobilityControlInfoV2X For mobility enhancements, adopt the need that we agreed for one-shot cases Mobility enhancements seem appropriate only when there is actual mobility within E-UTRAN (and not to E-UTRAN), co condition should be added | CR |
| E.931 | 6.3.4 MobilityControlInfo | Field descriptions do accurately capture the timing of actions. Not use “whether”. Existing: makeBeforeBreak Indicates whether the UE shall continue uplink transmission/ downlink reception with the source cell(s) before performing the first transmission through PRACH to the target PCell, or through PUSCH to the target PCell while rach-Skip is configured. makeBeforeBreakSCG Indicates whether the UE shall continue uplink transmission/ downlink reception with the source cell(s) before performing the first transmission through PRACH to the target PSCell, or through PUSCH to the target PSCell while rach-SkipSCG is configured. | 2 | Proposal makeBeforeBreak Indicates whetherthat the UE shall continue uplink transmission/ downlink reception with the source cell(s) beforeuntil performing the first transmission through PRACH to the target PCell, or (if rach-Skip is configured) until performing the first transmission through PUSCH to the target PCell while rach-Skip is configured. makeBeforeBreakSCG Indicates whetherthat the UE shall continue uplink transmission/ downlink reception with the source cell(s) beforeuntil performing the first transmission through PRACH to the target PSCell, or (if rach-Skip is configured) until performing the first transmission through PUSCH to the target PSCell while rach-Skip is configured. | CR |
| I.051 | 6.3.4 MobilityControlInfo | The field descriptions of the new fields handoverWithoutWT-Change-r14 and keep-LWA-Config-r14 can be improved due to type BOOLEAN. | 2 | Update field descriptions of handoverWithoutWT-Change-r14 and keep-LWA-Config-r14 as shown below. handoverWithoutWT-Change Value TRUE indicates that thewhether UE shall performs handover where LWA configuration is retained with the same WT and uses LWA end-marker for PDCP key change indication as defined in TS 36.323 [8]. keep-LWA-Config Value TRUE indicates that thewhether UE shall retains the LWA configuration during handover. | CR |
| N.140 | 6.3.4 MobilityControlInfo (all new fields) | All of the new fields should be “one-shot” configurations, but this is not really clear, as some are even Need OR. We should discuss better how to handle the one-shot fields and how to mark them in specification. | 4 | Discuss whether we should simply use ON for all fields (since they are one-shot), and how to ensure the rules are followed | CR/ TDoc (Nokia) |
| S.037 | 6.3.4 MobilityControlInfo-eLWA | For both subfields, considering use of boolean, there seems no need to also have optionality | 2 | Remove optionality or change to Enum (as for mobility enhancement subfields) | CR |
| S.038 | 6.3.4 MobilityControlInfo-eLWA | Extension marker seems not needed Need to group 2 subfields can be debated (typically done if savings are larger) | 2 | Remove extension marker | CR |
| M.021 | 6.3.4 MobilityControlInfo-eLWA-r14 | Need code is ON but it seems more appropriate to make it cond HO. Also the IE is defined as a sequence (HO without WT change, Keep LWA config) but it should be a choice | 3 | Change need code of mobilityControlInfo-eLWA-r14 to ON. Change MobilityControlInfoSCG-r12 to a CHOICE structure. | CR |
| S.039 | 6.3.4 RACH-Skip | Are there any constraints regading the values when used in MCI/ MCI-SCG? E.g. can field in MCI be set to PSTAG? | 2/3 | Discuss, and include constraints if appropriate | Flag |
| 6.3.5 Measurement information elements |  |  |  |  |  |
| S.050 | 6.3.5 | Spares don’t seem to have any particular value i.e. no real gain compared to another future extension | 2 | Remove | CR |
| S.048 | 6.3.5 cbr-PSCCH | Statement about when to include field should be moved from field description | 2 | Remove and introduce procedural text | TDoc V2X |
| E.902 | 6.3.5 MeasConfig | Not clear if field description intends to describe how E-UTRAN includes field in the message, and how UE behaves with respect to earier configured measGapConfig/ measGapConfig-r14. Both measGapConfig and measGapConfig-r14 are optional fields in MeasConfig. We raised this comment during CR agreement email discussion. It was commented that eNB is expected to (when including measGapConfig-r14) explicitly release (by including the field measGapConfig with choice set to “release”. From the existing field desciption, this is not clear. “The UE shall ignore…” seems to focus more on the signalling aspect, not the storage and usage of the information. Furthermore, since the Choice releaase/setup is per serving cell on measGapConfig-r14, eNB would (in case measGapConfig is included) signal “release” in measGapConfig-r14 per each serving cell. Since UE will apply gap configuration of EITHER measGapConfig OR measGapConfig-r14, we propose to simplyfy the handling by clarify that E-UTRAN includes either measGapConfig or measGapConfig -r14 in measConfig in field descr state that if measGapConfig (without suffix) is included, UE (implicitly) discards information in measGapConfig-14 (if provided earlier) in field descr state that if measGapConfig-r14 is included, UE (implicitly) discards information in measGapConfig (without suffix) (if provided earlier) See also E.904. Existing field descr: measGapConfig Used to setup and release measurement gaps. E-UTRAN configures the UE with either measGapConfig-r14 or measGapConfig, if any. In case measGapConfig-r14 is signalled, the UE shall ignore measGapConfig (i.e. without suffix) and visa versa. MeasGapConfig ::= CHOICE { release NULL, setup SEQUENCE { gapOffset CHOICE { gp0 INTEGER (0..39), gp1 INTEGER (0..79), ..., [[ gp2-r14 INTEGER (0..39), gp3-r14 INTEGER (0..79), gp-ncsg1-r1 INTEGER (0..39), gp-ncsg2-r14 INTEGER (0..79), gp-ncsg3-r14 INTEGER (0..39), gp-ncsg4-r14 INTEGER (0..79), gp-nonUniform1-r14 INTEGER (0..1279), gp-nonUniform2-r14 INTEGER (0..2559), gp-nonUniform3-r14 INTEGER (0..5119), gp-nonUniform4-r14 INTEGER (0..10239) ]] } } } MeasGapConfig-r14 ::= SEQUENCE (SIZE (1..maxServCell-r13)) OF PerCC-MeasGapConfig-r14 PerCC-MeasGapConfig-r14 ::= CHOICE { release SEQUENCE { servCellId ServCellIndex-r13 }, setup SEQUENCE { servCellId ServCellIndex-r13, measGapConfig MeasGapConfig } } | 2 | Modified field description text: measGapConfig Used to setup and release measurement gaps. E-UTRAN includes either measGapConfig (without suffix) or measGapConfig-r14, if any. If E-UTRAN includes measGapConfig (without suffix), the UE shall discard measurement gap configuration earlier provided with measGapConfig-r14, if any. If E-UTRAN includes measGapConfig-r14, the UE shall discard measurement gap configuration earlier provided with measGapConfig (without suffix), if any. E-UTRAN configures the UE with either measGapConfig-r14 or measGapConfig, if any. In case measGapConfig-r14 is signalled, the UE shall ignore measGapConfig (i.e. without suffix) and visa versa. Some text could also be moved to procedure in 5.5.2.9 Measurement gap configuration | CR |
| N.144 | 6.3.5 MeasConfig::measGapConfig-r14 | Field cannot be released | 3 | Since only one gap config should be present, the R14 one has to be releasable. For that, Need OR could be the simplest, but we are open to also release-setup structure. Also the field description is strange, and should be changed as follows: measGapConfig Used to setup and release measurement gaps. E-UTRAN configures the UE with either measGapConfig-r14 or measGapConfig, if any. In case measGapConfig-r14 is signalled, the UE shall ignore measGapConfig (i.e. without suffix) and visa versa. |  |
| S.040 | 6.3.5 measGapConfig | FD states that UE shall ignore legacy field/ -r14 version, but we normally don’t impose such requirements on UE i.e. it is networks responsibility to configure and release versions properly | 2 | State that network configures the UE either with measConfig (without suffix) or with measConfig-r14 | CR |
| S.041 | 6.3.5 measGapConfig | It is unclear how per CC gap is handled when SCell is released. According to convention network is resonsible to initiate release i.e. no UE autonomous action | 2 | Clarify that E-UTRAN initiates release (note there are cases in which UE autonomously releases SCells: re-establishment, connection resumption). Upon re-establishment UE already releases measGap, but similar UE autonomous action seems required upon connection resumption (it may actually be preferrable to specify per CC gap as part of SCell configuration) | TDoc |
| S.042 | 6.3.5 MeasGapConfig | Brackets are not used for extension of choices | 1 | Remove | CR |
| E.903 | 6.3.5 MeasGapConfig | Reword the field description servCellId serving cell ID corresponding to serving cell specific measGapConfig. | 2 | Proposal: servCellId Identifies the serving cell for which measurement gap configuration is provided (setup) or deleted (release). serving cell ID corresponding to serving cell specific measGapConfig. | CR |
| H.046 | 6.3.5 MeasGapConfig | The field description for gapOffset is not clear. The description for gp-ncsg1 and gp-ncsg3 has no difference, and the description for gp-ncsg2 and gp-ncsg4 has no difference. | 2 | Correct the field description according to R2-1700708 gapOffset ……gapOffset of gp-ncsg1 corresponds to gap offset of NCSG Pattern Id “1” with VIRP = 40ms and ML = 4ms, gapOffset of gp-ncsg2 corresponds to gap offset of of NCSG Pattern Id “2” with VIRP = 80ms and ML = 4ms, gapOffset of gp-ncsg3 corresponds to gap offset of NCSG Pattern Id “3” with VIRP = 40ms and ML = 3ms, gapOffset of gp-ncsg4 corresponds to gap offset of of NCSG Pattern Id “4” with VIRP = 80ms and ML = 3ms. …... | CR |
| N.145 | 6.3.5 MeasGapConfig-r14 | Strange construct for a release-setup | 3 | The intent seems to be to have release-setup, but this is quite strangely captured. Better would be the normal structure, i.e. PerCC-MeasGapConfig-r14 ::= CHOICE { release NULL, setup SEQUENCE { servCellId ServCellIndex-r13, measGapConfig MeasGapConfig } } PerCC-MeasGapConfig-r14 ::= CHOICE { release SEQUENCE { servCellId ServCellIndex-r13 }, setup SEQUENCE { servCellId ServCellIndex-r13, measGapConfig MeasGapConfig } } The field are also missing the -r14 suffixes, which should be added. | CR |
| S.045 | 6.3.5 MeasObject/ MeasObjectSL | Normally an object concerns a particular frequency. In this case it concerns a list of resource pools. It is however not clear which frequency the pools to be measured are on Object should preferrably be limited to a single frequency (Pool identities seem to be unique on a frequency only i.e. not across frequencies) | 3 | Further discussion seems required Note that if SL and regular RRM measurements are configured on a particular frequency, preferrably this should be covered by a single measurement object (i.e. so far we try to have one per frequency). Such approach would mean that the pools are added to measObjectEUTRA | TDoc V2X |
| N.143 | 6.3.5 MeasObjectSL | tx-ResourcePoolToAddList List of transmission resource pools to be added to the list of pools. | 2 | Add highlighted part to the description | CR |
| N.146 | 6.3.5 MeasObjectSL | IE description shouldn’t contain usage instructions for individual fields | 2 | Move the “E-UTRAN configures at least one resource pool in the MeasObjectSL” to field description of tx-ResourcePoolToAddList: tx-ResourcePoolToAddList List of transmission pools to be added to the list of pools. E-UTRAN configures at least one resource pool when MeasObjectSL is configured. | CR |
| H.047 | 6.3.5 MeasObjectWLAN | In the field description of bandIndicatorListWLAN, it should be “value band60-v14xy indicates the 60GHz” | 2 | bandIndicatorListWLAN Includes the list of WLAN bands. Value band2dot4 indicates the 2.4Ghz band, value band5 indicates the 5Ghz band and value band60-v14xy indicates the 60GHz band. | CR |
| S.047 | 6.3.5 measResultListWLAN-r14 | It seems this field merely facilitates reporting for more WLANs. Normally this is done by creating a 2nd list that is an extension of the first i.e. just containing the additional entries | 2 | Change name to measResultListExtWLAN and change list size to only include the additional number of entries i.e. 24 additional to the 8 from REL-13 | CR |
| S.046 | 6.3.5 MeasResults/ MeasResultListCBR | It is not clear for which frequency the results are reported. Seem remark about MeasObjectSL | 3 | If measObject is added as an extension of measObjectEUTRA, the same approach should be used for the corresponding results | TDoc V2X |
| S.043 | 6.3.5 PerCC-MeasGapConfig | It seems there is no delta signalling i.e.E-UTRAN always has to signal the configuration for all SCells Typically we use separate fields/ IEs for release and add so we only need to add the config of the SCell that is added. Would also enable release and add of SCell while re-using the same identity, similar to SCell configuration | 2 | Alignment to normal practice should be considered | Flag |
| S.044 | 6.3.5 PerCC-MeasGapConfig | Normally specifc versions are reflected at tail of name | 1 | Rename to MeasGapSCellList | CR |
| M.022 | 6.3.5 reportAnyWLAN | Editorial correction. Current text uses an apostrophe that can be avoided. | 1 | Change field description of reportAnyWLAN to: Indicates UE to report any WLAN AP meeting the triggering requirements, even if it’s it is not included in the corresponding MeasObjectWLAN. | CR |
| I.024 | 6.3.5 ReportConfigEUTRA | New triggers for periodic measurement reporting have been introduced: “reportLocation” for V2V and “sidelink” for V2X. Both can be merged and added on the same level. Furthermore, there is no need for adding spare values. [[ purpose-v14xy ENUMERATED {reportLocation, spare3, spare2, spare1} OPTIONAL, -- Need ON periodical-r14 SEQUENCE { purpose ENUMERATED {sidelink} } ]] | 2 | Change ASN.1 structure for adding the new triggers for periodic measurement reporting as shown below. [[ periodical-v14xy SEQUENCE { purpose-v14xy ENUMERATED {reportLocation, sidelink} } OPTIONAL -- Need ON ]] Coordinator (Ericsson): Same as E.115, N.141, S.049. | CR |
| E.115 | 6.3.5 ReportConfigEUTRA | Two purposes for peridocial reporting have been added independently of each other and can be merged. Additionally spare values can be removed. [[ purpose-v14xy ENUMERATED {reportLocation, spare3, spare2, spare1} OPTIONAL, -- Need ON periodical-r14 SEQUENCE { purpose ENUMERATED {sidelink} } ]] | 2 | Update the ASN.1 code accordingly: [[ purpose-v14xy ENUMERATED {reportLocation, sidelinkspare3, spare2, spare1} OPTIONAL, -- Need ON periodical-r14 SEQUENCE { purpose ENUMERATED {sidelink} } ]] Coordinator (Ericsson): Same as I.024, N.141, S.049. | CR |
| N.141 | 6.3.5 ReportConfigEUTRA | periodical-r14 SEQUENCE { purpose ENUMERATED {sidelink} } The SEQUENCE is not encoded in ASN.1 because there is only 1 choice in the ENUMERATED |  | Remove the periodical-r14 entry. Could be handled like this (same IE): purpose-v14xy ENUMERATED {reportLocation, spare3, spare2, spare1} OPTIONAL, -- Need ON “sidelink” value can replace spare3. Coordinator (Ericsson): Same as I.024, E.115, S.049. | CR |
| S.049 | 6.3.5 ReportConfigEUTRA, purpose | Value sidelink should be merged with other extension (used for location) i.e. concerns non-critical extension of purpose | 2 | Merge Coordinator (Ericsson): Same as I.024, E.115, N.141. | CR |
| I.023 | 6.3.5 ReportConfigEUTRA | For the new events V1 and V2 the statement below is missing. “The E-UTRA measurement reporting events concerning CBR are labelled VN with N equal to 1 and 2.” | 2 | Add the missing statement wrt to the new events V1 and V2 as shown below. The E-UTRA measurement reporting events concerning CSI-RS are labelled CN with N equal to 1 and 2. Event C1: CSI-RS resource becomes better than absolute threshold; Event C2: CSI-RS resource becomes amount of offset better than reference CSI-RS resource. The E-UTRA measurement reporting events concerning CBR are labelled VN with N equal to 1 and 2. Event V1: CBR becomes larger than absolute threshold; Event V2: CBR becomes smaller than absolute thereshold. | CR |
| N.147 | 6.3.5 ReportConfigEUTRA::purpose-v14xy, periodical-r14 | Unclear structure and it’s not clear how the CE/NCE work in this case (no field descriptions provided, either!) | 3 | Several points: 1. Critical extension to TriggerType seems to be intended – should do that instead of the current way. 2. In purpose-v14xy, remove the spare values if not needed 3. periodical-r14 is missing OPTIONAL, and single-value ENUM is not encoded. Proposal: Use simple BOOLEANs instead with Need ON [[ purpose-v14xy ENUMERATED {reportLocation, spare3, spare2, spare1} OPTIONAL, -- Need ON periodical-r14 SEQUENCE { purpose ENUMERATED {sidelink} } reportLocation-r14 BOOLEAN -- Need ON periodicalSideLink-r14 BOOLEAN -- Need ON ]] | CR |
| S.051 | 6.3.5 WLAN-SuspendConfig | Extension marker seems not needed Need to group 2 subfields can be debated (typically done if savings are larger) | 2 | Remove extension marker | CR |
| S.052 | 6.3.5 WLAN-SuspendConfig | For both subfields, considering use of boolean, there seems no need to also have optionality | 2 | Remove optionality or change to Enum (as for mobility enhancement subfields) | CR |
| I.052 | 6.3.5 WLAN-SuspendConfig | The field descriptions of the new fields wlan-SuspendResumeAllowed-r14 and wlan-SuspendTriggersStatusReport-r14 can be improved due to type BOOLEAN. | 2 | Update field descriptions of wlan-SuspendResumeAllowed-r14 and wlan-SuspendTriggersStatusReport-r14 as shown below. Furthermore, the redundant space in the heading “WLAN- SuspendConfig field descriptions” needs to be removed. WLAN- SuspendConfig field descriptions wlan-SuspendResumeAllowed Value TRUE indicates thatwhether the UE is allowed to use suspend-resume mechanism, i.e., to indicate WLAN being temporarily unavailable and WLAN being available again after temporary unavailability. wlan-SuspendTriggersStatusReport Value TRUE indicates thatwhether the UE shall trigger PDCP status report as defined in TS 36.323 [8] when WLAN is temporarily unavailable and UE reports this status. | CR |
| S.053 | 6.3.5 wlan-SuspendTriggersStatusReport | Name is somewhat confusing i.e. does not affect triggering of WLAN connection status report but PDCP status report | 1 | Change to wlan-SuspendPDCP-StatusReport | CR |
| 6.3.6 Other information elements |  |  |  |  |  |
| H.051 | 6.3.6 | VoLTE capability is only contained in general capability, we need to decide whether FDD/TDD diff for “pusch-Enhancements-r14” is same or not. If it is different, we should introduce “MMTEL-Parameters-r14 “ in “UE-EUTRA-CapabilityAddXDD-Mode”; | 2 | Need to know common understanding. In RAN1 LS, there is double bracket for it, as [yes]. We think we could remove double bracket, and introduce “MMTEL-Parameters-r14 “ in “UE-EUTRA-CapabilityAddXDD-Mode”; As Rapporteur, we would like to provide CR on it. | CR/ TDoc WI (Huawei) |
| H.052 | 6.3.6 | So far rel-14, eLAA capabilities only contains LAA-Parameters-v14xy ::= SEQUENCE { crossCarrierSchedulingLAA-UL-r14 ENUMERATED {supported} OPTIONAL, uplinkLAA-r14 ENUMERATED {supported} OPTIONAL, twoStepSchedulingTimingInfo-r14 ENUMERATED {nPlus1, nPlus2, nPlus3} OPTIONAL } Based on RAN1 LS, additional capabilities need to be introduced, 1-5 USS blind decoding adjustment (candidate adjustment) 1-6 USS blind decoding reduction (not monitoring DCI format 0A/0B/4A/4B) 1-7 Out of sequence grant handling | 2 | Introduce new LAA capabilities based on RAN1 LS. As Rapporteur, we would like to provide 331 and 306 CRs on it. | CR/ TDoc WI (Huawei) |
| E.135 | 6.3.6 – UE-EUTRA-Capability | Relation between recommendedBitRate and recommendedBitRateQuery. | 2 | The UE must support the recommended bit rate to support the query of the recommended bit rate. We should do the following addtiion. recommendedBitRateQuery Indicates whether the UE supports the bit rate recommendation query message from the UE to the eNB as specified in TS 36.321 [6, 6.1.3.X]. If this field is included, the UE shall also include the recommendedBitRate field. | CR |
| S.059 | 6.3.6 BandCombinationParameters-v14xy, retuningTimeBandPairList | Isn’t it possible to merge this information with the other band combination parameter extensions? I.e. Isn’t the intention to provide for each band X of a BC, the retuning time of bands affected when X is reconfigured? BTW: use of FBI to indicate bandB (rather than based on order of bands in BC) seems efficient unless there are parameters (retuning delays) for most band pairs | 2/ 3 | Discuss and conclude whether to move to BandParameters-v14xy the following retuningTimeParameters-r14 SEQUENCE (SIZE (1..maxSimultaneousBands-r10)) OF RetuningTimeParameters-r14 OPTIONAL | TDoc |
| S.055 | 6.3.6 BW-Config / DelayBudgetReportingConfig | Optional, Need OR seems simpler than setup/ release choice Need for separate IE could also be discussed | 1 | Matter of taste (both are used) Coordinator (Ericsson): See also E.038. | CR |
| E.038 | 6.3.6 OtherConfig | BW-Config-r14 and DelayBudgetReportingConfig-r14 are referenced only once and they are not imported to any other module. Are these new data types needed? | 2 | To be confirmed why this new data type is needed. If there are no reasons, remove the information elements DelayBudgetReportingConfig-r14 ::= CHOICE{ release NULL, setup SEQUENCE{ delayBudgetReportingProhibitTimer-r14 ENUMERATED { s0, s0dot4, s0dot8, s1dot6, s3, s6, s12, s30} } } BW-Config-r14 ::= CHOICE { release NULL, setup SEQUENCE{ bw-PreferenceIndicationTimer-r14 ENUMERATED {s0, s0dot5, s1, s2, s5, s10, s20, s30, s60, s90, s120, s300, s600, spare3, spare2, spare1} } } and move the extension fields to the extension addition group (of the OtherConfig information element) [[ bw-Config-r14 BW-Config-r14 OPTIONAL, -- Need ON delayBudgetReportingConfig-r14 DelayBudgetReportingConfig-r14 OPTIONAL, -- Need ON bw-Config-r14 CHOICE{ release NULL, setup SEQUENCE { bw-PreferenceIndicationTimer-r14 ENUMERATED {s0, s0dot5, s1, s2, s5, s10, s20, s30, s60, s90, s120, s300, s600, spare3, spare2, spare1} } } OPTIONAL, -- Need ON delayBudgetReportingConfig-r14 CHOICE{ release NULL, setup SEQUENCE{ delayBudgetReportingProhibitTimer-r14 ENUMERATED {s0, s0dot4, s0dot8, s1dot6, s3, s6, s12, s30} } } OPTIONAL, -- Need ON sps-AssistanceInfoReport-r14 ENUMERATED {allowed} OPTIONAL -- Need OR ]] Coordinator (Ericsson): See also S.055. | CR/ TBC |
| M.023 | 6.3.6 Lwa-UL | Editorial correction. Current text is gramatically incorrect. | 1 | Change field desrciption of LWA-UL to: Indicates whether the UE supports the LWA bearerfeature in the UL | CR |
| N.151 | 6.3.6 MIMO-UE-ParametersPerTM-r14xy::nzp-CSI-RS-AperiodicInfo-r14, nzp-CSI-RS-PeriodicInfo-r14 | Parameter names are confusing and lacking field description | 2 | Remove the “Info” from the names and create (at least placeholder) field descriptions based on current RAN1 information. |  |
| M.026 | 6.3.6 mmtel-Parameters-r14 | Missing suffix | 1 | MMTEL-Parameters-r14xy Coordinator (Ericsson): It is not clear what is corrected here. Normally -r14 suffix is not appended with xy suffix. | CR |
| M.025 | 6.3.6 MUST capability | MUST capability is missing | 4 |  | CR/ TDoc WI (MediaTek?) |
| M.024 | 6.3.6 naics-Capability-List | Wrong IE name | 2 | naics-Capability-List Indicates that UE supports NAICS, i.e. receiving assistance information from serving cell and using it to cancel or suppress interference of neighbouring cell(s) for at least one band combination. If not present, UE does not support NAICS for any band combination. The field numberOfNAICS-CapableCC indicates the number of component carriers where the NAICS processing is supported and the field numberOfAggregatedPRB indicates the maximum aggregated bandwidth across these of component carriers (expressed as a number of PRBs) with the restriction that NAICS is only supported over the full carrier bandwidth. The UE shall indicate the combination of {numberOfNAICS-CapableCC, numberOfNAICS-CapableCC numberOfAggregatedPRB} for every supported numberOfNAICS-CapableCC, e.g. if a UE supports {x CC, y PRBs} and {x-n CC, y-m PRBs} where n>=1 and m>=0, the UE shall indicate both. | CR |
| E.039 | 6.3.6 OtherConfig | The information element has a field description for bWPreferenceIndicationTime but there is no such field in ASN.1. | 2 | It needs to be confirmed whether the correct name for the field is the same as the following identifier bw-PreferenceIndicationTimer-r14 ENUMERATED {s0, s0dot5, s1, s2, s5, s10, s20, s30, s60, s90, s120, s300, s600, spare3, spare2, spare1} If so, align the field description with ASN.1 bWPreferenceIndicationTimerbw-PreferenceIndicationTimer Prohibit timer for bandwidth preference indication reporting. Value in seconds. Value s0 means prohibit timer is set to 0 second, value s0dot5 means prohibit timer is set to 0.5 second, value s1 means prohibit timer is set to 1 second and so on. | CR/ TBC |
| I.025 | 6.3.6 OtherConfig field descriptions | Field description of sps-AssistanceInfoReport can be improved to be consistent with the other field descriptions. sps-AssistanceInfoReport if configured, the UE is allowed to report SPS-AssistanceInformation. | 2 | Improve field description of sps-AssistanceInfoReport as shown below. sps-AssistanceInfoReport if configured, the UE is allowed to report SPS-AssistanceInformation.The field is used to indicate whether the UE is allowed to report SPS-AssistanceInformation. | CR |
| S.054 | 6.3.6 OtherConfig, sps-AssistanceInfoReport | Normally delta signalling (need ON) is provided for fields after extension marker | 2 | Change to boolean with need ON | CR |
| N.148 | 6.3.6 OtherConfig::delayBudgetReportingProhibitTimer-r14 | Potentially unnecessary SEQUENCE wrapping | 2 | See N.003 for generic discussion on this |  |
| S.061 | 6.3.6 PhyLayerParameters-v14xy, CE related capabilities | Several capabilities concern a particular CE mode only, but this is not reflected in name nor in information structure (e.g. use separate groups for common, mode A and mode B?) | 2 | Change may be considered | TDoc MTC |
| S.060 | 6.3.6 PhyLayerParameters-v14xy, ce-pdsch-tenProcesses | Seems more extensible if actual number is indicated by value of he field i.e. that name just indicates it concerns the number | 1 | Somewhat matter of taste | CR |
| N.150 | 6.3.6 PhyLayerParameters-v14xy::ce-* | Several fields don’t follow ASN.1 naming conventions (dash after acronym, capitalize acronyms and first letter of words) | 2 | Fix the field names |  |
| S.058 | 6.3.6 SupportedBandCombination-v14xy, SupportedBandCombinationAdd-v14xy | Rather than two separate parallel lists, could consider to (from now on) use one parallel list with first entries corresponding to SupportedBandCombination while any additional entries (~CombinationAdd) are placed afterwards | 2 | Discuss and conclude | Flag |
| C.016 | 6.3.6 UE-EUTRA-Capability | Incorrect format of IE names in PhyLayerParameters-v14xy. | 1 | Update the name of IEs and the field descriptions in field descriptions. PhyLayerParameters-v14xy ::= SEQUENCE { ce-PUSCH-NB-MaxTbs-r14ce-pusch-nb-maxTbs-r14 ENUMERATED {supported} OPTIONAL, ce-PDSCH-PUSCH-MaxBandwidth-r14 ce-pdsch-pusch-maxBandwidth-r14 ENUMERATED {bw5, bw20} OPTIONAL, ce-HARQAckBundling-r14 ENUMERATED {supported} OPTIONAL, ce-PDSCH-TenHARQProcesses-r14ce-pdsch-tenProcesses-r14 ENUMERATED {supported} OPTIONAL, ce-RetuningSymbols-r14ce-RetuningSymbols-r14 ENUMERATED {n0, n1} OPTIONAL, ce-PDSCH-PUSCHEnhancement-r14ce-pdsch-puschEnhancement-r14 ENUMERATED {supported} OPTIONAL, ce-SchedulingEnhancement-r14ce-schedulingEnhancement-r14 ENUMERATED {supported} OPTIONAL, ce-SRSEnhancement-r14ce-srsEnhancement-r14 ENUMERATED {supported} OPTIONAL, ce-PUCCHEnhancement-r14ce-pucchEnhancement-r14 ENUMERATED {supported} OPTIONAL, ce-ClosedLoopTxAntennaSelection-r14ce-ClosedLoopTxAntennaSelection-r14 ENUMERATED {supported} OPTIONAL } | CR |
| H.049 | 6.3.6 UE-EUTRA-Capability | The field description for perServingCellMeasurementGap is not clear. | 2 | perServingCellMeasurementGap Indicates whether the UE supports per serving cell measurement gap indication, as specified in TS 36.133 [16]. | CR |
| I.027 | 6.3.6 UE-EUTRA-Capability | In UE-EUTRA-Capability-v14xy-IEs the capability tdd-TTI-Bundling-r14 has been added in the MAC parameters group but this is not aligned with the 36.306 CR R2-1700950 where this capability is added in the PHY parameters group. MAC-Parameters-v14xy ::= SEQUENCE { shortSPS-IntervalFDD-r14 ENUMERATED {supported} OPTIONAL, shortSPS-IntervalTDD-r14 ENUMERATED {supported} OPTIONAL, skipUplinkDynamic-r14 ENUMERATED {supported} OPTIONAL, skipUplinkSPS-r14 ENUMERATED {supported} OPTIONAL, tdd-TTI-Bundling-r14 ENUMERATED {supported} OPTIONAL, dataInactMon-r14 ENUMERATED {supported} OPTIONAL } | 2 | Discuss in which group to add the capability tdd-TTI-Bundling-r14 in RRC. Proposal is to add it in the phyLayerParameters-v14xy. | CR/ TDoc WI |
| I.030 | 6.3.6 UE-EUTRA-Capability | In UE-EUTRA-Capability-v1350-IEs: the OPTIONAL for ce-Parameters-v1350 can be removed as CE-Parameters-v1350 contains just one entry with optional presence. Furthermore, for the capability unicastFrequencyHopping in CE-Parameters-v1350 the column “FDD/TDD diff” has been set to “No”. However, as there is no intention to define UE-EUTRA-CapabilityAddXDD-Mode-v14xy incl. ce-Parameters-v1350 it is better to change the setting to “-“. | 2 | Remove OPTIONAL for ce-Parameters-v1350. Set column “FDD/TDD diff” of unicastFrequencyHopping to “-“. | CR |
| L.041 | 6.3.6 UE-EUTRA-Capability | The ce-pdsch-pusch-maxBandwidth-r14 is using for maximum supported PDSCH/PUSCH channel bandwidth in CE mode A and B but this bandwidth information is not aligned with previous bandwidth information. | 2 | Change the value as follows; ce-pdsch-pusch-maxBandwidth-r14 ENUMERATED {n25 bw5, n100 bw20} OPTIONAL, and change the corresponding field desciption as follows; ce-pdsch-pusch-maxBandwidth Indicates the maximum supported PDSCH/PUSCH channel bandwidth in CE mode A and B, as specified in TS 36.212 [22] and TS 36.213 [23]. Value bw5 corresponds to 5 MHz and value bw20 corresponds to 20 MHz. If the field is absent the maximum PDSCH/PUSCH channel bandwidth in CE mode A and B is 1.4 MHz. If the setting of this parameter is 20 MHz, the max supported PUSCH channel bandwidth in CE mode A is 5 MHz. The maximum PUSCH channel bandwidth in CE mode B is 1.4 MHz regardless of the setting of this parameter. . Parameter: transmission bandwidth configuration, see TS 36.101 [42, table 5.6-1]. n25 corresponds to 25 resource blocks, n100 to 100 resource blocks. | CR |
| I.026 | 6.3.6 UE-EUTRA-Capability field descriptions | CR2667 on Introduction of SRS switching capability for LTE was not fully implemented: the field descriptions of retuningTimeBandPairList-r14, rf-RetuningTimeDL-r14, rf-RetuningTime and UL-r14 switchingBandA, switchingBandB are missing. | 2 | Add the missing field descriptions related to SRS switching. | CR/ TDoc WI |
| I.028 | 6.3.6 UE-EUTRA-Capability field descriptions | In the field description of capability dataInactMon, column "FDD/TDD diff" an "-" is missing. | 2 | Add missing "-" in the column "FDD/TDD diff". | CR |
| I.029 | 6.3.6 UE-EUTRA-Capability field descriptions | In the context of FeMTC multiple capabilities have been introduced in PhyLayerParameters-v14xy which can be set differently for “FDD/TDD diff”: Yes: ce-pusch-nb-maxTbs-r14, ce-pdsch-pusch-maxBandwidth-r14, ce-ClosedLoopTxAntennaSelection-r14 No: ce-RetuningSymbols-r14, ce-pdsch-puschEnhancement-r14, ce-schedulingEnhancement-r14, ce-pucchEnhancement-r14 Not possible to signal different values: ce-HarqAckBundling-r14, ce-pdsch-tenProcesses-r14, ce-srsEnhancement-r14, ceMeasurements-r14 For the capabilities which have been defined as “Yes” and “No” new UE-EUTRA-CapabilityAddXDD-Mode-v14xy needs to be introduced. | 2 | Introduce new UE-EUTRA-CapabilityAddXDD-Mode-v14xy incl. PhyLayerParameters-v14xy to allow different setting for FDD and TDD. | CR |
| I.031 | 6.3.6 UE-EUTRA-Capability field descriptions | For the measurement gap capabilities ncsg, nonUniformGap, perServingCellMeasurementGap and shortMeasurementGap a “No” has been set in the column “FDD/TDD diff” for the concerned field descriptions. However, as there is no intention to define UE-EUTRA-CapabilityAddXDD-Mode-v14xy incl. these capabilities it is better to change the setting to “-“. | 2 | Discuss if there is an use-case for setting the measurement gap capabilities differently for FDD and TDD. If there is no use-case then the setting should be changed to “-“. | CR |
| I.032 | 6.3.6 UE-EUTRA-Capability field descriptions | Field description of ul-256QAM-perCC-InfoList: “-“ in column “FDD/TDD diff” to be set. | 2 | Set “-“ in column “FDD/TDD diff”. | CR |
| I.054 | 6.3.6 UE-EUTRA-Capability field descriptions | In IE AccessStratumRelease, “rel14” needs to be added, and in the field description of accessStratumRelease the release indicator to be updated to rel14. AccessStratumRelease ::= ENUMERATED { rel8, rel9, rel10, rel11, rel12, rel13, spare2, spare1, ...} | 2 | Update IE and field description of AccessStratumRelease as shown below. AccessStratumRelease ::= ENUMERATED { rel8, rel9, rel10, rel11, rel12, rel13, rel14spare2, spare1, ...} accessStratumRelease Set to rel14 in this version of the specification. | CR |
| I.055 | 6.3.6 UE-EUTRA-Capability field descriptions | Field description of tdd-SpecialSubframe to be updated to reflect tdd-SpecialSubframe-r11 and tdd-SpecialSubframe-r14. | 2 | Update field description of tdd-SpecialSubframe as shown below. tdd-SpecialSubframe Indicates whether the UE supports TDD special subframe defined in TS 36.211 [21]. A UE shall indicate tdd-SpecialSubframe-r11 if it supports the TDD special subframes ssp7 and ssp9. A UE shall indicate tdd-SpecialSubframe-r14 if it supports the TDD special subframe ssp10. | CR |
| S.057 | 6.3.6 UE-EUTRA-Capability-v14xy-IEs, General | Many UE capability fields (in particular CE related) are not according to naming convetions e.g. ce-pusch-nb-maxTbs should be ce-PUSCH-NB-MaxTBS | 1 | Align with naming conventions | CR |
| S.056 | 6.3.6 UE-EUTRA-Capability-v14xy-IEs, xdd-Add-UE-EUTRA-Capabilities | So far FDD/ TDD differentiation is not supported for any of the UE capabilties. Normally this is supported for physical configuration fields. Measurement and MBMS are also typical candidates Note also that for several CE capabilities column in FD indicates that differences are allowed | 2/ 3 | Introduce FDD/ TDD differentiation at least for physical configuration capabilities. Further discussion is required for other features e.g.measurements, mobility, MBMS | Flag |
| N.149 | 6.3.6 UE-EUTRA-Capability-v14xy-IEs::ue-CategoryUL-v14xya, ue-CategoryUL-v14xyb | Combine the Rel-14 UE categories to one entry ue-CategoryUL-v14xya ENUMERATED {m2} OPTIONAL, ue-CategoryUL-v14xyb INTEGER (16..20) OPTIONAL, | 2 | Use the following (similar as used in ue-CategoryUL-v1310: ue-CategoryUL-v14xy ENUMERATED {n16, n17, n18, n19, n20, m2) OPTIONAL, | CR |
| E.062 | 6.3.6 UE-EUTRA-Capability | That 10 DL HARQ processes is only supported for full-duplex UEs is missing from the UE-EUTRA-Capability field description. | 2 | ce-pdsch-tenProcesses Indicates whether the UE supports 10 DL HARQ processes in FD FDD in CE mode A. | CR |
| I.053 | 6.3.6 UE-RadioPagingInfo | During RRC_IDLE, a cat.M2 UE has to behave same as cat.M1. To reflect this fact, it should be clarified in the field description of ue-Category, ue-CategoryDL that the cat.M2 UE shall always include the indication of M1 in the UE-RadioPagingInfo. UE-RadioPagingInfo-r12 ::= SEQUENCE { ue-Category-v1250 INTEGER (0) OPTIONAL, ..., [[ ue-CategoryDL-v1310 ENUMERATED {m1} OPTIONAL, ce-ModeA-r13 ENUMERATED {true} OPTIONAL, ce-ModeB-r13 ENUMERATED {true} OPTIONAL ]] } | 2 | Update the field description of ue-Category, ue-CategoryDL as shown below. ue-Category, ue-CategoryDL UE category as defined in TS 36.306 [5]. A category M2 UE shall always include the field ue-CategoryDL-v1310 in this version of the specification. | CR |
| N.152 | 6.3.6 ul-256QAM-perCC-InfoList-r14 | Capability is supposed to be per CC. Currently it seems to indicate the UL 256QAM support in general (per UE) + per CC (ul-256QAM + ul-256QAM-perCC-InfoList). | 2 | Clarify if both indications are necessary and whether they need to be mutually eclusive (The UE includes ul-256QAM-perCC_InfoList only if the field ul-256QAM is not included.) |  |
| N.153 | 6.3.6 Ul-64QAM | Filed description refers to n14 value | 2 | Change n14 to 14. Alternatively, align with the field “enable64QAM” and make the description generic by reference to Ts36.306 (by listing UE Categories that support is defined in TS 36.306 [5, Table 4.1A-2 and Table 4.1A-6]) |  |
| 6.3.7 MBMS information elements |  |  |  |  |  |
| S.062 | 6.3.7 MBMS-NotificationConfig | Optionality not required i.e. already present one level above (SIB13). Note that need code is missing also | 2 | Remove Coordinator (Ericsson): Same as E.041, I.033. | CR |
| I.033 | 6.3.7 MBMS-NotificationConfig | IE MBMS-NotificationConfig-v14xy the OPTIONAL can be removed as it contains only one field. Otherwise, NEED code is missing. MBMS-NotificationConfig-v14xy ::= SEQUENCE { notificationSF-Index-v14xy INTEGER (7..8) OPTIONAL } | 2 | Remove OPTIONAL in IE MBMS-NotificationConfig-v14xy. Coordinator (Ericsson): Same as S.062, E.041. | CR |
| E.041 | 6.3.7 MBMS-NotificationConfig | notificationSF-Index-v14xy field is defined as optional in MBMS-NotificationConfig-v14xy information element but there is already an optionality bit in SystemInformationBlockType13 where MBMS-NotificationConfig-v14xy is referenced. | 2 | Remove the optional keyword in MBMS-NotificationConfig-v14xy ::= SEQUENCE { notificationSF-Index-v14xy INTEGER (7..8) OPTIONAL } Coordinator (Ericsson): Same as S.062, I.033.. | CR |
| H.053 | 6.3.7 MBMS-NotificationConfig | On FeMBMS/Unicast-mixed cell, MCCH change notification can be transmitted in subframes #4 and #9 when either is configured as unicast subframe or legacy MBSFN subframe with control region. Current fields don’t reflect this. | 2 | Add the following changes: MBMS-NotificationConfig-v14xy ::= SEQUENCE { notificationSF-Index-v14xy INTEGER (7..810) OPTIONAL } notificationSF-Index Indicates the subframe used to transmit MCCH change notifications on PDCCH. FDD: Value 1, 2, 3, 4, 5 and 6 correspond with subframe #1, #2, #3 #6, #7, and #8 respectively. Value 7, 8, 9 and 810 correspond with subframe #0, #4, #5 and #59 respectively. If notificationSF-Index-v14xy is included, UE ignores notificationSF-Index-r9. TDD: Value 1, 2, 3, 4, and 5 correspond with subframe #3, #4, #7, #8, and #9 respectively. | CR |
| S.063 | 6.3.7 MBSFN-AreaInfo | All extension fields should be optional and have need code OR | 1/ 2 | Add Coordinator (Ericsson): Same as E.042, N.154. | CR |
| E.042 | 6.3.7 MBSFN-AreaInfoList | Optional field subcarrierSpacingMBMS-r14 is missing Need code. | 1 | Add Need code OR subcarrierSpacingMBMS-r14 ENUMERATED {khz-7dot5, khz-1dot25} OPTIONAL, -- Need OR Coordinator (Ericsson): Same as N.154, S.063. See also I.034. | CR |
| N.154 | 6.3.7 MBSFN-AreaInfoList | subcarrierSpacingMBMS-r14 ENUMERATED {khz-7dot5, khz-1dot25} OPTIONAL, NEED code is missing | 2 | NEED OR should be added Coordinator (Ericsson): Same as E.042, S.063. See also I.034. | CR |
| I.034 | 6.3.7 MBSFN-AreaInfoList | The ASN.1 structure of the Rel-14 extensions needs to be corrected: -Need code for subcarrierSpacingMBMS-r14 is missing. -To keep the logical context mcch-RepetitionPeriod-v14x0 and mcch-ModificationPeriod-v14x0 should be introduced as NCE of mcch-Config-r9. [[ subcarrierSpacingMBMS-r14 ENUMERATED {khz-7dot5, khz-1dot25} OPTIONAL, mcch-RepetitionPeriod-v14x0 ENUMERATED {rf1, rf2, rf4, rf8, rf16, spare3, spare2, spare1} OPTIONAL, -- Need OR mcch-ModificationPeriod-v14x0 ENUMERATED {rf1, rf2, rf4, rf8, rf16, rf32, rf64, rf128, rf256, spare7, spare6, spare5, spare4, spare3, spare2, spare1} OPTIONAL -- Need OR ]] | 2 | Correct the ASN.1 structure of the Rel-14 extensions as shown below. [[ subcarrierSpacingMBMS-r14 ENUMERATED {khz-7dot5, khz-1dot25} OPTIONAL, -- Need OR mcch-Config-v14xy SEQUENCE { mcch-RepetitionPeriod-v14x0 ENUMERATED {rf1, rf2, rf4, rf8, rf16, spare3, spare2, spare1} OPTIONAL, -- Need OR mcch-ModificationPeriod-v14x0 ENUMERATED {rf1, rf2, rf4, rf8, rf16, rf32, rf64, rf128, rf256, spare7, spare6, spare5, spare4, spare3, spare2, spare1} OPTIONAL -- Need OR } OPTIONAL -- Need OR ]] Coordinator (Ericsson): See also S.063, N.154, E.042. | CR |
| N.156 | 6.3.7 MBSFN-AreaInfoList:: mcch-ModificationPeriod-v14x0, mcch-RepetitionPeriod-v14x0 | Are the spares needed – since these are always used in broadcast, some UEs will never comprehend these, causing some issues. | 2 | Remove spares | CR |
| N.158 | 6.3.7 MBSFN-AreaInfoList::mcchModificationPeriod-v14x0 | Unify wording of “UE shall ignore” with other instances in specification | 2 | Use the following: “In case mcch-ModificationPeriod-v14x0 is configured, the UE shall ignore the configuration of mcch-ModificationPeriod-r9.” | CR |
| N.159 | 6.3.7 PMCH-InfoList::mch-SchedulingPeriod-v14x0 | Unify wording of “UE shall ignore” with other instances in specification | 2 | Use the following: “In case mch-SchedulingPeriod-v14x0 is configured, the UE shall ignore the configuration of mch-SchedulingPeriod-r12.” | CR |
| S.064 | 6.3.7 SC-MTCH-InfoList-BR | Several optional fields don’t have a need code (mpdcch-Narrowband-SC-MTCH, mpdcch-NumRepetition-SC-MTCH) If UE behaviour is specified for the case the field is absent, OP should be used. Otherwise, need OR seems approriate | 1/ 2 | Add (by default use OR) | CR |
| N.155 | 6.3.7 SC-MTCH-InfoList-BR | The description of the IE is identical t o SC-MTCH-InfoList. | 2 | It should be indicated in the description that this IE is applicable to BR/CE UEs |  |
| 6.3.7a SC_PTM information elements |  |  |  |  |  |
| H.054 | 6.3.7a SC-MTCH-InfoList-BR | Parameter mpdcch-Narrowband-SC-MTCH-r14 is defined as OPTIONAL w/o any need code | 2/3 | Change the parameter to mandatory Coordinator (Ericsson): See also Q.014. | CR |
| H.055 | 6.3.7a SC-MTCH-InfoList-BR | Parameter mpdcch-NumRepetition-SC-MTCH-r14 is defined as OPTIONAL w/o any need code | 2/3 | Change the parameter to Mandatory Coordinator (Ericsson): See also Q.015. | CR |
| Q.014 | 6.3.7a SC-MTCH-InfoList-BR IE | mpdcch-Narrowband-SC-MTCH-r14 is optional and UE behaviour undefined if this IEs is omitted. | 2 | Define UE behaviour if this IE is missing, e.g. use same values as defined for previous SC-MTCH in the list, or use same narrow band as used for reception of this SIB. Coordinator (Ericsson): See also H.054. | CR |
| Q.015 | 6.3.7a SC-MTCH-InfoList-BR IE | mpdcch-NumRepetition-SC-MTCH-r14 is optional and UE behaviour undefined if this IEs is omitted. | 2 | Define UE behaviour if this IE is missing, e.g. use same values as defined for previous SC-MTCH in the list. Coordinator (Ericsson): See also H.055. | CR |
| H.056 | 6.3.7a SC-MTCH-InfoList-BR | There is no need for ‘BR’ in the parameter name sc-mtch-SchedulingInfo-BR r14 The field description is not correct | 2 | Rename parameter to name sc-mtch-SchedulingInfo-BR- r14 Correct the field description as follows: sc-mtch-schedulingInfo-BR DRX information for the SC-MTCH. If this field is absent, DRX is not used for SC-MTCH reception. the SC-MTCH may be scheduled in any subframe. | CR |
| H.057 | 6.3.7a SC-MTCH-InfoList-BR | The field description of onDurationTimerSCPTM is not correct | 2 | Revise the description as follows onDurationTimerSCPTM Timer for SC-MTCH reception in TS 36.321 [6]. Value in number of PDCCH sub-frames. Value psf1300 corresponds to 1300 PDCCH sub-frame, psf2400 corresponds to 2400 PDCCH sub-frames and so on. | CR |
| H.058 | 6.3.7a SC-MTCH-InfoList-BR | The field description of mpdcch-PDSCH- MaxBandwidth-SC-MTCH does not specify the meaning of the value | 2 | Revise the description as follows mpdcch-PDSCH- MaxBandwidth-SC-MTCH Maximum PDSCH channel bandwidth for SC-MTCH, see TS 36.213 [23]. Value bw6 corresponds to 6 PRBs and value bw24 corresponds to 24 PRBs. | CR |
| H.059 | 6.3.7a SC-MTCH-InfoList-BR | Parameter p-a-r14 is missing | 2 | Add parameter p-a-r14 in IE SC-MTCH-Info-BR-r14 as follows: p-a-r14 ENUMERATED { dB-6, dB-4dot77, dB-3, dB-1dot77, dB0, dB1, dB2, dB3} OPTIONAL, -- Need ON p-a Parameter : , for the SC-MTCH per G-RNTI, see TS 36.213 [23, 5.2]. Value dB-6 corresponds to -6 dB, dB-4dot77 corresponds to -4.77 dB etc. | CR |
| 6.3.8 ProSe information elements |  |  |  |  |  |
| N.166 | 6.3.8 Several sidelink fields | Does it make sense to use spare values? | 3 | Many sidelink fields define multiple spare values. As per Rel-13 ASN.1 guidelines, we avoid defining spares for dedicated signalling, but can do so for SI (where error handling is defined). Since SL is a bit of mix, we should consider in which cases the spares are really needed. |  |
| N.167 | 6.3.8 SL-CBR-MeasConfig::SL-ThresS-RSSI-CBR, | The field description should be algined with other measurement quantieis. Also, shouldn’t this mapping be handled in RAN4 specification instead of the field description here? | 3 | Clarify if the mapping should be rather have a reference to RAN4 specifications. If not, the following would be more in line with other parts of RRC: The actual value is (-112 + field value*2) dBm. | CR |
| Z.019 | 6.3.8 SL-CBR-PSSCH-TxConfigList | The definition for cr-Limit does not accurately reflect the RAN1 agreement, which is copied as following. The RAN2 description for cr-Limit is also copied. It could be seen that RAN2 regards the cr-Limit as a limit on channel occupancy for one specific PPPP. However from RAN1 agreement, cr-Limit is a limit on channel occupancy for a group of PPPP. In another word, RAN1 only limits the total channel occupancy for all PPPP but RAN2 put a stricter limit for every single PPPP. cr-Limit Indicates the maximum limit on the occupancy ratio for a specific CBR range and a specific PPPP. Value 0 corresponds to 0, value 1 to 0.0001, value 2 to 0.0002, and so on (i.e. in steps of 0.0001) until value 10000, which corresponds to 1. RAN1 agreement: Relation between CR and CRlimit The UE shall ensure the following limit is met per PPPP, UE shall ensure Suffix i and k denote the PPPP of a packet in increasing priority order | 3 | To align with RAN1 agreement, we need to do: The priorityList could be changed to one single PPPP value, this requires other companies’ comments. The field description for cr-limit should be modified according to RAN1 agreement. The following change is only an example. cr-Limit Indicates the maximum limit on the occupancy ratio for a specific CBR range and all the PPPP(s) lower than athe specific PPPP. Value 0 corresponds to 0, value 1 to 0.0001, value 2 to 0.0002, and so on (i.e. in steps of 0.0001) until value 10000, which corresponds to 1. | CR |
| N.162 | 6.3.8 SL-CBR-Range | IE description is difficult to understand. | 3 | Remove the pasted sentence in the IE description includes: – SL-CBR-Range The IE SL-CBR-Range defines the upper bound (i.e. cbr-ThreshHigh) and lower bound (i.e. cbr-ThreshLow) of the CBR range. The CBR range includes the value of the upper bound but excludes the value of the lower bound except 0. Value 0 is included in the range if set to the lower bound. or improve the wording. | CR |
| E.043 | 6.3.8 SL-CommResourcePool | The field description of restrictResourceReservationPeriod has a (should) recommendation to overwrite another field if restrictResourceReservationPeriod is configured restrictResourceReservationPeriod If configured, the field should overwrite restrictResourceReservationPeriod configured in v2x-ResourceSelectionConfig for transmission on this pool. | 2 | To be confrmed whether a requirement or recommendation is needed. | TBC |
| E.136 | 6.3.8 SL-CommResourcePool | Unclear field description for cbr-MeasConfig. The field description just says the following. It does not explain how this parameter is used what the purpose is or anything. Some elaboration is needed. cbr-MeasConfig Indicates parameter for CBR measurement. | 3 | Regarding the use of this parameter, it seems unclear what the relation is to the normal measConfig. We assume we dont introduce a new framework for measurement reporting and hence this field seems unnecessary. It was agreed for CBR-measurements that the existing measurement framework should be used so we don’t understand why for CBR measurements it seems to be suggested that a new kind of measconfig is used. It seems unnecessary and it should be ensured that the the existing measconfig is used. It refers to the IE SL-CBR-MeasConfig which only contains one field. This field could be directly included in SL-CommResourcePool. The use of this fields need to be confirmed. | TBC |
| L.042 | 6.3.8 SL-CommResourcePool | V2X related description is missing. | 2 | The IE SL-CommResourcePool and SL-CommResourcePoolV2X specifies the configuration information for an individual pool of resources for sidelink communication and V2X sidelink communication respectively. The IE covers the configuration of both the sidelink control information and the data. | CR |
| N.169 | 6.3.8 SL-CommResourcePool::Cond Tx, Cond P2X | Is it possible to release fields tagged with these conditions? | 3 | Currently, the condition only says “Otherwise the field is not present.”, so it’s not clear what UE should do if that happens: Does it delete the previous value, or retain it? | TBD/ TDoc? |
| E.044 | 6.3.8 SL-CommTxPoolSensingConfig | The field description of pssch-TxConfigList has a (can) verbal form for possibility and capability which is something that is usually not used in 36.331. pssch-TxConfigList Indicates PSSCH TX parameters such as MCS, PRB number, retransmission number, which can be associated with the condition of the UE absolute speed and synchronization reference type in UE autonomous resource selection (see TS 36.213 [23]). | 2 | To be confrmed whether the verbal form for possiblity and capability is needed in the field description at all or if something else e.g. a requirement is needed. | TBC |
| L.043 | 6.3.8 SL-CommTxPoolSensingConfig | Align the wording. | 1 | Add ‘related V2X’ p2x-SensingConfig Indicates the sensing configuration for P2X related V2X sidelink communication transmission only. | CR |
| N.161 | 6.3.8 SL-InterFreqInfoListV2X-r14) | This IE is used times in RRCConnectionReconfiguration (in sl-V2X-ConfigDedicated-r14 and sl-P2X-ConfigDedicated-r14), but there is no mechanism for a target eNB to release it at incoming handover. | 2 | Add a release/setup choice or Need OR. Need OR may be safer as the target eNB may not support V2X. |  |
| C.017 | 6.3.8 SL-InterFreqInfoV2X | Both common and dedicated configuration of TX resource pool for mode 4 are indicated by IE ” SL-InterFreqInfoV2X” in SIB21 and RRCConnectionReconfiguration. Therefore, eNB cannot indicate the dedicated TX pool in an addition/release manner, unlike what eNB can do in non-inter-frequency configuration case. This may increase the signalling overload. | 3 | A proposed modification is given. In case of inter-frequency configuration, V2X resource is indicated in v2x-UE-SelectionConfigList included in SL-InterFreqInfoV2X for RRC_IDLE UE. V2X resource is indicated in SL-V2X-ConfigDedicated included in SL-InterFreqInfoV2X for RRC_CONNECTED. SL-InterFreqInfoListV2X-r14 ::= SEQUENCE (SIZE (1..maxFreq)) OF SL-InterFreqInfoV2X-r14 SL-InterFreqInfoV2X-r14::= SEQUENCE { plmn-IdentityList-r14 PLMN-IdentityList OPTIONAL, v2x-CommCarrierFreq-r14 ARFCN-ValueEUTRA-r9, sl-MaxTxPower-r14 P-Max, sl-Bandwidth-r14 ENUMERATED {n6, n15, n25, n50, n75, n100}, v2x-SchedulingPool-r14 SL-CommResourcePoolV2X-r14 OPTIONAL, -- Need OR v2x-UE-SelectionConfigList-r14 SL-V2X-UE-SelectionConfigList-r14 OPTIONAL, -- Need OR SL-V2X-ConfigDedicated-r14 ::= SEQUENCE { commTxResources-r14 CHOICE { release NULL, setup CHOICE { scheduled-r14 SEQUENCE { sl-D-RNTI-r14 C-RNTI, mac-MainConfig-r14 MAC-MainConfigSL-r12 OPTIONAL, -- Need OP v2x-SchedulingPool-r14 SL-CommResourcePoolV2X-r14 OPTIONAL, -- Need OP mcs-r14 INTEGER (0..31) OPTIONAL, -- Need OP logicalChGroupInfoList-r14 LogicalChGroupInfoList-r13 }, ue-Selected-r14 SEQUENCE { v2x-CommTxPoolNormalDedicated-r14 SEQUENCE { poolToReleaseList-r14 SL-TxPoolToReleaseListV2X-r14 OPTIONAL, -- Need ON poolToAddModList-r14 SL-TxPoolToAddModListV2X-r14 OPTIONAL, -- Need ON v2x-CommTxPoolSensingConfig-r14 SL-CommTxPoolSensingConfig-r14 OPTIONAL -- Need ON } } } } OPTIONAL, -- Need ON SL-TxPoolToAddModListV2X-r14 ::= SEQUENCE { poolIdentity-r14 SL-V2X-TxPoolIdentity-r14, pool-r14 SL-CommResourcePoolV2X-r14 } SL-TxPoolToReleaseListV2X-r14 ::= SEQUENCE (SIZE (1.. maxSL-V2X-TxPool-r14)) OF SL-V2X-TxPoolIdentity-r14 ... } | CR/ TDoc XX |
| H.062 | 6.3.8 SL-InterFreqInfoV2X | A space after “SL-InterFreqInfoV2X-r14” is missing. | 1 | SL-InterFreqInfoV2X-r14 ::= SEQUENCE { plmn-IdentityList-r14 PLMN-IdentityList OPTIONAL, v2x-CommCarrierFreq-r14 ARFCN-ValueEUTRA-r9, sl-MaxTxPower-r14 P-Max, sl-Bandwidth-r14 ENUMERATED {n6, n15, n25, n50, n75, n100}, v2x-SchedulingPool-r14 SL-CommResourcePoolV2X-r14 OPTIONAL, -- Need OR v2x-UE-SelectionConfigList-r14 SL-V2X-UE-SelectionConfigList-r14 OPTIONAL, -- Need OR ... } | CR |
| I.035 | 6.3.8 SL-InterFreqInfoV2X | Need code for field plmn-IdentityList-r14 is missing. | 2 | Add missing need code for field plmn-IdentityList-r14. Need OR looks ok. | CR |
| L.044 | 6.3.8 SL-InterFreqInfoV2X | The network may only provide the frequency information. Hence, it seems that sl-MaxTxPower-r14, sl-Bandwidth-r14 are optional fields | 2 | Make fields optional sl-MaxTxPower-r14 P-Max OPTIONAL, --Need OR sl-Bandwidth-r14 ENUMERATED {n6, n15, n25, n50, n75, n100} OPTIONAL, --Need OR | CR |
| L.045 | 6.3.8 SL-InterFreqInfoV2X | RAN2 agreement in RAN2#97 ‘10. The serving carrier indicates to the UE the frequency carrier on which the UE may acquire the inter-carrier sidelink resource configuration.’ is not captured. | 2 | Suggest to add anchorCarrierFreqList-r14 SL-V2X-AnchorCarrierFreqList-r14 OPTIONAL, --Need OR | CR |
| N.164 | 6.3.8 SL-InterFreqInfoV2X | plmn-IdentityList-r14 PLMN-IdentityList OPTIONAL, lacking NEED code | 2 | Add NEED OP and add in the description than if lacking then the config concerns serving PLMN (first entry as indicated in SIB1?) | CR |
| Z.018 | 6.3.8 SL-InterFreqInfoV2X | The size for SL-InterFreqInfoListV2X should be (0…maxFreq-1) instead of (1…maxFreq). It is because maxFreq comprises Pcell and SL-InterFreqInfoListV2X. | 2 | SL-InterFreqInfoListV2X-r14 ::= SEQUENCE (SIZE (10..maxFreq-1)) OF SL-InterFreqInfoV2X-r14 | CR |
| E.137 | 6.3.8 SL-OffsetIndicator | Unclear motivation. Currently there are these two IEs and it is unclear why we need both since one is a subset of the other. Maybe it is useful to have both but RAN2 should confirm this if so. SL-OffsetIndicatorSync-v14xy ::= INTEGER (40..159) SL-OffsetIndicatorSync-r14 ::= INTEGER (0..159) | 3 | The use of these fields should be confirmed. | TBC |
| N.169 | 6.3.8 SL-OffsetIndicatorSync-v14xy, SL-OffsetIndicatorSync-r14 | Two overapping value ranges defined with non-critical extension | 2 | Why are these fields related? Wouldn't it be better to have separate names, e.g. |  |
| L.049 | 6.3.8 SL-P2X-ResourceSelectionConfig | Clarify the meaning of the IE. | 2 | The IE SL-P2X-ResourceSelectionConfig includes the configuration of resource selection mechanism for P2X related V2X sidelink communication. E-UTRAN configures at least one resource selection mechanism. | CR |
| L.050 | 6.3.8 SL-P2X-SensingConfig | Align the wording. | 1 | The IE SL-P2X-SensingConfig includes the configuration of resource sensing for P2X related V2X sidelink communication. |  |
| E.046 | 6.3.8 SL-P2X-SensingConfig-r14 | The field description of minNumCandidateSF has a network recommendation. Usually RAN2 spefications do not impose requirements or recommendations for deployments. minNumCandidateSF Indicates the minimum number of subframes that should be included in the possible candidate resources. | 2 | To be confirmed why a recommendation is needed. If a recommendation is not needed, the field description could be reformulated by using a statement of fact instead, minNumCandidateSF Indicates the minimum number of subframes that should be are included in the possible candidate resources. | CR/ TBC |
| E.046 | 6.3.8 SL-P2X-SensingConfig-r14 | The information element SL-P2X-SensingConfig-r14 is referenced only once and it is not imported to any other module. | 2 | Why is it necessary to define this new data type? If there are no reasons, the information element should be removed and the fields should be moved to SL-CommTxPoolSensingConfig-r14 where the information element is referenced. | CR/ TBC |
| E.116 | 6.3.8 SL-PSSCH-TxConfig | There may be a logical error in the configuration of the parameters associated with a certain UE speed threshold. The UE can be configured with a list of SL-PSCCH-TxConfig, and which configuration is used depends on the speed of the UE as captured in parameter thresUE-speed. If the speed is below this threshold the parameters in parametersAboveThres are used and if the speed is below the threshold the parameters in parametersBelowThres are used. The problem is that the speed of the UE can be both below one threshold and and above another threshold at the same time. Which parameters should the UE use in that case? | 3 | Discuss in contribution. A possible solution is that the eNB ensures that the parameters “between” thresholds always are the same. | CR/ WI (Ericsson) |
| E.047 | 6.3.8 SL-PSSCH-TxConfig | It is not necessary to outline examples of the information element content because the exact content can be seen from ASN.1 and field descriptions. – SL-PSSCH-TxConfig The IE SL-PSSCH-TxConfig indicates PSSCH TX parameters such as MCS, PRB number, retransmission number. | 2 | Remove the “such as “ part – SL-PSSCH-TxConfig The IE SL-PSSCH-TxConfig indicates PSSCH TX parameters such as MCS, PRB number, retransmission number. | CR |
| E.048 | 6.3.8 SL-PSSCH-TxConfig | A shall requirement for “considerations” is too vague to be e.g. tested. Why is this requirement needed? – SL-PSSCH-TxConfig The IE SL-PSSCH-TxConfig indicates PSSCH TX parameters such as MCS, PRB number, retransmission number. When lower layers select parameters from the range indicated in IE SL-PSSCH-TxConfig, the UE shall consider both configurations in IE SL-PSSCH-TxConfigList and IE SL-CBR-PSSCH-TxConfigList | 2 | It needs to be confirmed exactly what is required from the UE. If there are no reasons for a requirement, the sentence could be reformulated as (if kept at all) – SL-PSSCH-TxConfig The IE SL-PSSCH-TxConfig indicates PSSCH TX parameters such as MCS, PRB number, retransmission number. When lower layers select parameters from the range indicated in IE SL-PSSCH-TxConfig, the UE shall considers both configurations in IE SL-PSSCH-TxConfigList and IE SL-CBR-PSSCH-TxConfigList | CR/ TBC |
| E.049 | 6.3.8 SL-PSSCH-TxConfig | The verbal form “can” indicates a capability or possibility. It does not seem correct to use this verbal form in the following fields because the fields are not sent by the UE. The intention is probably to specify that the UE is permitted to do something. allowedRetxNumberPSSCH Indicates the allowed retransmission number for transmissions on PSSCH (see TS 36.213 [23]). The value n0 indicates no retransmission for a transport block allowed; the value n1 indicates that the UE shall perform one retransmission for a transport block; and the value both indicates that the UE can autonomously select no retransmission or one retransmission for a transport block. minRB-NumberPSSCH, maxRB-NumberPSSCH Indicates the minimum and maximum number of PRBs which can be used for transmissions on PSSCH (see TS 36.213 [23]). | 2 | Change the verbal forms “can” to “may” allowedRetxNumberPSSCH Indicates the allowed retransmission number for transmissions on PSSCH (see TS 36.213 [23]). The value n0 indicates no retransmission for a transport block allowed; the value n1 indicates that the UE shall perform one retransmission for a transport block; and the value both indicates that the UE maycan autonomously select no retransmission or one retransmission for a transport block. minRB-NumberPSSCH, maxRB-NumberPSSCH Indicates the minimum and maximum number of PRBs which maycan be used for transmissions on PSSCH (see TS 36.213 [23]). | CR |
| E.050 | 6.3.8 SL-PSSCH-TxConfig | The information element SL-PSSCH-TxConfig is referenced only once and it is not imported to any other module. Why is this new data type needed? | 2 | To be confirmed why this new data type is needed. If it is not needed, it should be removed and its fields should be included in the information element (or message) where SL-PSSCH-TxConfig is now referenced. | TBC |
| N.170 | 6.3.8 SL-PSSCH-TxConfig::Cond CBR | The condition makes no sense | 2 | Does the condition imply the field using it is 1) mandatory in the mentioned IE and never present elsewhere? 2) optional (Need OR) in the mentioned IE and never present elsewhere? 3) something else? Change to: “The field is optionally present, Need OR within IE SL-CBR-PSSCH-TxConfigList-r14. Otherwise the field is not present.” | CR |
| C.018 | 6.3.8 SL-RestrictResourceReservationPeriod | The title “SL-CommResourcePool field descriptions” should be “SL-RestrictResourceReservationPeriod field descriptions” | 1 | SL-RestrictResourceReservationPeriodSL-CommResourcePool field descriptions SL-RestrictResourceReservationPeriod Indicates the allowed value for the signalling of the resource reservation period in PSCCH for V2X sidelink communication. Value v0dot2 means SL-RestrictResourceReservationPeriod is set to 0.2, value v0dot5 means SL-RestrictResourceReservationPeriod is set to 0.5, value v1 means SL-RestrictResourceReservationPeriod is set to 1, and so on. Value v0dot2 and value v0dot5 are configured in a pool-specific manner only. | CR |
| L.048 | 6.3.8 SL-SyncAllowed | Clarify the meaning of the IE. | 2 | The IE SL-SyncAllowed indicates the allowed the synchronization references for transmission resource pool. | CR |
| N.165 | 6.3.8 SL-SyncAllowed | enb-Sync If configured, the (pre-) configured resources can be used if the UE is directly or indirectly synchronized to eNB (i.e., synchronized to a reference UE which is directly synchronized to eNB). | 2 | Change description to enb-Sync If configured, the (pre-) configured resources can be used if the UE is directly or indirectly synchronized to eNB (i.e., synchronized to a reference UE which is directly synchronized to eNB) synchronized to eNB. Bracket was move right after “indirectly” which it refers to. Comma after “i.e.” is removed. Same should be applied to gnss-Sync field description for this IE. | CR |
| H.061 | 6.3.8 SL-SyncConfig | The following agreement achieved in RAN1 #87 meeting is currently absent and should be captured into TS 36.331: When two resources are included, the following behavior is used. The same sync resource is used for UEs directly synchronized with GNSS. | 2 | Add the following texts into the field description of SL-SyncConfig syncOffsetIndicator E-UTRAN should ensure syncOffsetIndicator is set to the same value as syncOffsetIndicator1 or syncOffsetIndicator2 in preconfigSync within SL-Preconfiguration, if configured. If syncOffsetIndicator-v14xy is configured, the UE shall ignore the field syncOffsetIndicator-r12. E-UTRAN should ensure syncOffsetIndicator is set to syncOffsetIndicator1 in preconfigSync within SL-V2X-Preconfiguration, if syncOffsetIndicator3 is pre-configured in SL-V2X-Preconfiguration. or if gnss-Sync is included in the corresponding entry and syncOffsetIndicator3 is not pre-configured in SL-V2X-Preconfiguration. | CR |
| L.047 | 6.3.8 SL-SyncConfig | Wrong field name. | 2 | syncOffsetIndicator E-UTRAN should ensure syncOffsetIndicator is set to the same value as syncOffsetIndicator1 or syncOffsetIndicator2 in preconfigSync within SL-Preconfiguration, if configured. If syncOffsetIndicator-v14xy is configured, the UE shall ignore the field syncOffsetIndicator-r12. E-UTRAN should ensure syncOffsetIndicators is set to syncOffsetIndicator1 in preconfigSyncv2x-CommPreconfigSync within SL-V2X-Preconfiguration, if syncOffsetIndicator3 is pre-configured in SL-V2X-Preconfiguration for a concerned frequency. | CR |
| Z.021 | 6.3.8 SL-SyncConfig | “preconfigSync” is only included in “SL-Preconfiguration”, while it is “v2x-CommPreconfigSync” in “SL-V2X-Preconfiguration”. syncOffsetIndicator E-UTRAN should ensure syncOffsetIndicator is set to the same value as syncOffsetIndicator1 or syncOffsetIndicator2 in preconfigSync within SL-Preconfiguration, if configured. If syncOffsetIndicator-v14xy is configured, the UE shall ignore the field syncOffsetIndicator-r12. E-UTRAN should ensure syncOffsetIndicator is set to syncOffsetIndicator1 in preconfigSync within SL-V2X-Preconfiguration, if syncOffsetIndicator3 is pre-configured in SL-V2X-Preconfiguration. | 2 | Correction as below: syncOffsetIndicator E-UTRAN should ensure syncOffsetIndicator is set to the same value as syncOffsetIndicator1 or syncOffsetIndicator2 in preconfigSync within SL-Preconfiguration, if configured. If syncOffsetIndicator-v14xy is configured, the UE shall ignore the field syncOffsetIndicator-r12. E-UTRAN should ensure syncOffsetIndicator is set to syncOffsetIndicator1 in v2x-CommPreconfigSyncpreconfigSync within SL-V2X-Preconfiguration, if syncOffsetIndicator3 is pre-configured in SL-V2X-Preconfiguration. | CR |
| E.051 | 6.3.8 SL-TF-ResourceConfig | RAN capabilities are not within the scope of the standard. subframeBitmap Indicates the subframe bitmap indicating resources used for sidelink. For sidelink communication, E-UTRAN configures value bs40 for FDD and the following values for TDD: value bs42 for configuration0, value bs16 for configuration1, value bs8 for configuration2, value bs12 for configuration3, value bs8 for configuration4, value bs4 for configuration5 and value bs30 for configuration6. For V2X sidelink communication, E-UTRAN can configure value bs16, bs20 or bs100 for FDD and the following values for TDD: value bs60 for configuration0, value bs40 for configuration1, value bs20 for configuration2, value bs30 for configuration3, value bs20 for configuration4, value bs10 for configuration5 and value bs50 for configuration6. | 2 | Remove the verbal form “can” and use statement of fact instead subframeBitmap Indicates the subframe bitmap indicating resources used for sidelink. For sidelink communication, E-UTRAN configures value bs40 for FDD and the following values for TDD: value bs42 for configuration0, value bs16 for configuration1, value bs8 for configuration2, value bs12 for configuration3, value bs8 for configuration4, value bs4 for configuration5 and value bs30 for configuration6. For V2X sidelink communication, E-UTRAN can configures value bs16, bs20 or bs100 for FDD and the following values for TDD: value bs60 for configuration0, value bs40 for configuration1, value bs20 for configuration2, value bs30 for configuration3, value bs20 for configuration4, value bs10 for configuration5 and value bs50 for configuration6. | CR |
| N.171 | 6.3.8 SL-ThresPSSCH-RSRP | The field description should be algined with other measurement quantieis. Also, shouldn’t this mapping be handled in RAN4 specification instead of the field description here? | 3 | Clarify if the mapping should be rather have a reference to RAN4 specifications. If not, the following would be more in line with other parts of RRC: The actual value is (-128 + (field value -1)*2) dBm. | CR |
| E.052 | 6.3.8 SL-ThresPSSCH-RSRP-List-r14 | The information element is referenced only once and it is not imported to any other module. Why is this new data type needed? | 2 | To be confirmed why this new data type is needed. If it is not needed, it should be removed and its fields should be included in the information element (or message) where SL-ThresPSSCH-RSRP-List-r14 is now referenced. | CR/ TBC |
| E.053 | 6.3.8 SL-TxPower | The information element is referenced only once and it is not imported to any other module. Why is this new data type needed? | 2 | To be confirmed why this new data type is needed. If it is not needed, it should be removed and its fields should be included in the information element (or message) where SL-TxPower is now referenced. | CR/ TBC |
| N.172 | 6.3.8 SL-TxPower::power-r14 | Could be better as txPower-r14 | 2 | Use “txPower-r14” instead | CR |
| E.054 | 6.3.8 SL-TypeTxSync | How is the spare value supposed to be used? The information element is referenced by both common and dedicated signaling. It has been agreed during previous ASN.1 reviews that spare values are not used in dedicated signaling. SL-TypeTxSync-r14 ::= ENUMERATED {gnss, enb, ue, spare1} | 2 | Remove the spare value SL-TypeTxSync-r14 ::= ENUMERATED {gnss, enb, ue, spare1} | CR |
| L.051 | 6.3.8 SL-V2X-ConfigDedicated | The range of thresSL-TxPrioritization-r14 does not need to include ‘0’ and ‘1’ since the current range of PPPP is 1 to 8 and 1 means that UL is always prioritized. | 2 | Suggested value range is 2~8 or use SL-Priority-r13 | CR |
| L.052 | 6.3.8 SL-V2X-ConfigDedicated | Priorization is done in MAC layer so that it would be good to add reference. No italic for SIB21. | 1 | thresSL-TxPrioritization Indicates the threshold used to determine whether SL TX is prioritized over UL TX overlapping in time, used as specified in TS 36.321 [6]. If the SL data to be transmitted has a PPPP value lower than this threshold, then the SL data can be prioritized over UL transmission in the same subframe. This value shall overwrite thresSL-TxPrioritization configured in SIB21 or SL-V2X-Preconfiguration if any. | CR |
| L.053 | 6.3.8 SL-V2X-ConfigDedicated | Rearrange the fields in field description in alphabetical order | 1 | Switch between sl-DV-RNTI and v2x-InterFreqInfoList. | CR |
| N.173 | 6.3.8 SL-V2X-ConfigDedicated-r14 | Lot of unnamed SEQUENCES, making the structure harder to read | 3 | Consider using IE definitions for SEQUENCES used in scheduled-r14, ue-Selected-r14, |  |
| N.175 | 6.3.8 SL-V2X-ConfigDedicated-r14::mac-MainConfig-r14, v2x-SchedulingPool-r14, mcs-r14 | Need OP used but no field descriptions – is there a reason for OP, or should these be OR? | 3 | Discuss if these fields should be releasable |  |
| N.174 | 6.3.8 SL-V2X-ConfigDedicated-r14::v2x-CommTxPoolNormalDedicated-r14 | Is this SEQUENCE needed? | 3 | This looks like an unnecessary wrapper SEQUENCE, could be easier to just remove it. Alternatively, define it as named IE. |  |
| E.045 | 6.3.8 SL-V2X-InterFreqUE-SelectionConfig | The field descriptions of p2x-CommTxPoolNormal, v2x-CommRxPool, v2x-CommTxPoolExceptional, v2x-CommTxPoolNormal use verbal form “is allowed to” instead of “may” to express permission. The verbal form for permission is “may” according to specification drafting rules, TR 20.801, Annex E table E.3. The equivalent expression “is allowed to” shall be used only in exceptional cases when the form given in the first column cannot be used for linguistic reasons as stated in TR 21.801 subclause 6.6.1. | 2 | The linguistic reason to use “is allowed to” instead of “may” should be confirmed. If there are no such linguisitic reasons, reformulate the field descriptions as p2x-CommTxPoolNormal Indicates the resources on a carrier frequency by which the UE is allowed tomay transmit P2X related V2X sidelink communication. and v2x-CommRxPool Indicates the resources on a carrier frequency by which the UE is allowed tomay receive V2X sidelink communication. and v2x-CommTxPoolExceptional Indicates the resources on a carrier frequency by which the UE is allowed tomay transmit V2X sidelink communication in exceptional conditions, as specified in 5.10.13. and v2x-CommTxPoolNormal Indicates the resources on a carrier frequency by which the UE is allowed tomay transmit V2X sidelink communication. | CR/ TBC |
| H.063 | 6.3.8 SL-V2X-InterFreqUE-SelectionConfig | A tab before “SEQUENCE” is missing. | 1 | SL-V2X-UE-SelectionConfigList-r14 ::= SEQUENCE (SIZE (1.. maxCellIntra)) OF SL-V2X-InterFreqUE-SelectionConfig-r14 | CR |
| Z.020 | 6.3.8 SL-ZoneConfig | If the IE SL-ZoneConfig is included in SystemInformationBlockType21 or in SL-V2X-Preconfiguration, the UE shall determine an identity of the zone (i.e. Zone_id) in which it is located using the following formulae: x1= Floor (x / L) Mod Nx; y1= Floor (y / W) Mod Ny; Zone_id = y1 * Nx + x1. Which means all the parametes of zoneLength, zoneWidth, zoneIdLingiMod and zoneIdLatiMod in the IE SL-ZoneConfig are necessary, not optional. SL-ZoneConfig-r14 ::= SEQUENCE { zoneLength-r14 ENUMERATED { m5, m10, m20, m50, m100, m200, m500, spare1} OPTIONAL, -- Need OR zoneWidth-r14 ENUMERATED { m5, m10, m20, m50, m100, m200, m500, spare1} OPTIONAL, -- Need OR zoneIdLongiMod-r14 INTEGER (1..4) OPTIONAL, -- Need OR zoneIdLatiMod-r14 INTEGER (1..4) OPTIONAL -- Need OR } | 2 | Delete the description of ‘OPTIONAL, -- Need OR’ for zoneLength-r14, zoneWidth-r14, zoneIdLingiMod-r14 and zoneIdLatiMod -r14 in the IE SL-ZoneConfig. SL-ZoneConfig-r14 ::= SEQUENCE { zoneLength-r14 ENUMERATED { m5, m10, m20, m50, m100, m200, m500, spare1} OPTIONAL, -- Need OR zoneWidth-r14 ENUMERATED { m5, m10, m20, m50, m100, m200, m500, spare1} OPTIONAL, -- Need OR zoneIdLongiMod-r14 INTEGER (1..4) OPTIONAL, -- Need OR zoneIdLatiMod-r14 INTEGER (1..4) OPTIONAL, -- Need OR } | CR |
| H.060 | 6.3.8 SPS-Config | The field description for the fields ul-V-SPS-RNTI and sl-V-SPS-RNTI are missing, which makes it unclear which specific parameters in other Specs (i.e. TS 36.321) these two fields correspond to. Besides, it seems that the field description of fixedRV-NonAdaptive is not placed in an improper place, not following the alphabet order. | 2 | Add the following two field descriptions for SPS-Config: sl-V-SPS -RNTI SL Semi-Persistent Scheduling V-RNTI, see TS 36.321 [6]. ul-V-SPS -RNTI UL Semi-Persistent Scheduling V-RNTI, see TS 36.321 [6]. Move the field description of fixedRV-NonAdaptive to the right place according to the alphabet order. | CR |
| N.168 | 6.3.8 tx-Parameters-ListPPPP-r14 | Add dash between List and PPPP | 2 | Use tx-Parameters-List-PPPP-r14 | CR |
| 6.4 RRC multiplicity and type constraints values |  |  |  |  |  |
| N.177 | 6.4 maxCBR-Report-r14 | Slightly long comment, could be simplified maxCBR-Report-r14 INTEGER ::= 72 -- Maximum number of CBR results reported in a report | 2 | Proposal for the comment: maxCBR-Report-r14 INTEGER ::= 72 -- Maximum number of CBR results reported in a report | CR |
| 6.5 PC5 RRC messages |  |  |  |  |  |
| 6.6 Direct Indication Information |  |  |  |  |  |
| 6.7.1 General NB-IoT message structure |  |  |  |  |  |
| L.055 | 6.7.1 General NB-IoT message structure | The new NBIOT-RRC-Definitions is not in alphabetical order. | 1 | Sort the definitions as follows; T-Reordering, TimeAlignmentTimer, TrackingAreaCode, T-Reordering | CR |
| Q.016 | 6.7.1 SystemInformationBlockType1-NB message | An extended list of SIB Types for NB is defined (SIB-TypeExt-NB-r14) but it is unclear how the enteries in SIB-MappingInfo-NB-r13 set to sibTypeExt-NB-v14xy map to the entries in SIB-MAppingInfo-NB-v14xy? The specification currently only has one SIB-TypeExt-NB-r14 defined hence it may not be a problem but in future there could be more entries and legacy UEs need to be able to handle these gracefully. | 2 | Desribe the mapping of sibTypeExt-NB-v14xy enteries in SIB-MappingInfo-NB-r13 map to enteries in SIB-MappingInfo-NB-v14xy. That is, the first sibTypeExt-NB-v14xy entery in SIB-MappingInfo-NB-r13 maps to first entery in SIB-MappingInfo-NB-v14xy, the second sibTypeExt-NB-v14xy entery in SIB-MappingInfo-NB-r13 maps to second entery in SIB-MappingInfo-NB-v14xy, and so on. sib-MappingInfo List of the SIBs mapped to this SystemInformation message.There is no mapping information of SIB2-NB; it is always present in the first SystemInformation message listed in the schedulingInfoList list. In case sib-TypeExt-r14 is included, the corresponding entry of sib-MappingInfo-r13 is set to sibTypeExt. If at least on occurance of sib-MappingInfo-r13 is set to sibTypeExt then sib-TypeExt-r14 shall be incuded. The first sibTypeExt included in schedulingInfoList corresponds to the first entry in sib-TypeExt-r14, the second sibTypeExt included in schedulingInfoList corresponds to the second entry in sib-TypeExt-r14, and so on. | CR |
| Q.017 | 6.7.1 SystemInformationBlockType1-NB message | Why is the entries in SystemInformationBlockType1-NB-v14xy-IEs marked as ‘Need OR’ but the fields within the coresponding sub-structure are marked as Need OP’ . For example see cellSelectionInfo-v14xy & CellSelectionInfo-NB-v14xy. Should it not be the other way round? | 2 | Suggest change ‘Need OR’ to ‘Need OP’ for cellSelectionInfo-v14xy in SystemInformationBlockType-NB-v14xy-IEs as the UE behaviour is clearly defined when any of the two sub-field is not present. | CR |
| 6.7.2 NB-IoT Message definitions |  |  |  |  |  |
| H.084 | 6.7.2 DL-CarrierConfigCommon-NB | Suffix of eutraControlRegionSize-r13 is not correct , should be –r14 | 2 | Change as follows: eutraControlRegionSize-r143 ENUMERATED {n1, n2, n3} | CR |
| H.085 | 6.7.2 NPRACH-ConfigSIB-NB | The word order sounds strange in the IE description | 2 | Change as follows: The IE NPRACH-ConfigSIB-NB is used to specify the NPRACH configuration in the system information for the anchor carrier. | CR |
| H.086 | 6.7.2 RLC-Config-NB | RLC-Config-NB is extended to include RLC UM configuration. However, RLC UM is only used for SC-PTM for which RLC configuration is not signallled | 2 | Change as follows: - Remove SN-FieldLength from the IMPORT - Change the ASN.1 as follows: RLC-Config-NB-r13 ::= CHOICE { am SEQUENCE { ul-AM-RLC-r13 UL-AM-RLC-NB-r13, dl-AM-RLC-r13 DL-AM-RLC-NB-r13 }, ..., um-Uni-Directional-DL-r14 SEQUENCE { dl-UM-RLC-r14 DL-UM-RLC-NB-r14 } } DL-UM-RLC-NB-r14 ::= SEQUENCE { sn-FieldLength-r14 SN-FieldLength } - remove the field description sn-FieldLength Indicates the UM RLC SN field size, see TS 36.322 [7], in bits. Value size5 means 5 bits, size10 means 10 bits. | CR |
| H.087 | 6.7.2 RLC-Config-NB | t-reordering is introduced as OPTIONAL Need OR. However, it is only needed if twoHARQProcesses is configured so it could be made Conditional instead. | 2 | Change as follows: RLC-Config-NB-v14xy ::= SEQUENCE { t-Reordering-r14 T-Reordering OPTIONAL -- Cond twoHARQNeed OR } And add the condition twoHARQ: The field is mandatory present if twoHARQ processes is configured. Otherwise, the field is not needed. | CR |
| M.027 | 6.7.2 RRCConnectionRelease-NB message | The condition “Redirection” should be in lower case, also it’s more succinct to call it “redirect” | 1 | Change the condition name “Redirection” to “redirect” | CR |
| N.180 | 6.7.2 RRCConnectionRelease-NB::Cond Redirection | Need OR used – why is that? | 3 | Discuss whether to use ON since this a one-shot field. |  |
| H.065 | 6.7.2 RRCConnectionSetupComplete-NB | Field gummei-Type-r14 is defined as ENUMERATED {native, mapped} by analogy with LTE. However, the filed does not exist in Rel-13 and the default value is ‘native’. It would be better to define the parameter as ENUMERATED {mapped} to avoid impacting UE that does not support IRAT | 2/3 | Change the field and its description as follows gummei-Type-r14 ENUMERATED {native, mapped} OPTIONAL, gummei-Type This field is used to indicate whether that the GUMMEI included is native (assigned by EPC) or mapped (from 2G/3G identifiers) as indicated by the upper layers, TS 24.301 [35]. | CR |
| H.088 | 6.7.2 SC-MTCH-InfoList | Parameter sc-mtch-CarrierConfig-r14 is defined as mandatory present. However, it could be defined as OPTIONAL need OP, with absence meaning that the anchor carrier is used. Also the field description is not correct | 2/3 | Change as follows: sc-mtch-CarrierConfig-r14 DL-CarrierConfigCommon-NB-r14 OPTIONAL, -- Need OP sc-mtch-CarrierConfig Downlink carriers that is be used for SC-MTCH. If the field is absent, the anchor carrier is used for SC-MTCH. | CR |
| H.089 | 6.7.2 SC-MTCH-InfoList | The field description of sc-mtch-SchedulingInfo-r14 is not correct. | 2 | sc-mtch-SchedulingInfo DRX information for the SC-MTCH. If this field is absent, DRX is ot used for the SC-MTCH may be scheduled in any subframe. | CR |
| H.090 | 6.7.2 SC-MTCH-InfoList | A word is missing in the field description of parameter drx-InactivityTimerSCPTM | 2 | Change as follows: drx-InactivityTimerSCPTM Timer for SC-MTCH reception in TS 36.321 [6]. Value in number of NPDCCH periods. Value pp1 corresponds to 1 NPDCCH period, pp2 corresponds to 2 NPDCCH periods and so on. | CR |
| H.066 | 6.7.2 SCPTMConfiguration-NB | There is no need to refer to NB-IoT UE in the filed description as the ASN.1 only applies to NB-IoT | 2 | Change as follows: scptm-NeighbourCellList List of neighbour cells providing MBMS services via SC-MRB. When absent, the NB-IoT UE shall assume that MBMS services listed in the SCPTMConfiguration-NB message are not provided via SC-MRB in any neighbour cell. | CR |
| H.091 | 6.7.2 SCPTM-NeighbourCellList-NB | carrierFreq-r14 in PCI-ARFCN-NB-r14 is defined as OPTIONAL w/o Need code there is no field description table | 2 | Add the need code carrierFreq-r14 CarrierFreq-NB-r13 OPTIONAL -- Need OP Add the field description table carrierFreq Indicates the frequency of the neighbour cell indicated by physCellId. Absence of the IE means that the neighbour cell is on the same frequency as the current cell physCellI Indicates the physical cell id of the neighbour cell. | CR |
| N.187 | 6.7.2 sibx-v14xy | This IE is for SIB22 | 2 | Change x to 22 | CR |
| H.070 | 6.7.2 SystemInformationBlockType15-NB | A new type MBMS-SAI-List-NB-r14 is introduced. However its definition is identical to MBMS-SAI-List-r11, so the legacy type could be reused | 2 | Change as follows: - IMPORT MBMS-SAI-List-r11 - change the ASN.1: SystemInformationBlockType15-NB-r14 ::= SEQUENCE { mbms-SAI-IntraFreq-r14 MBMS-SAI-List-NB-r114 OPTIONAL, -- Need OR mbms-SAI-InterFreqList-r14 MBMS-SAI-InterFreqList-NB-r14 OPTIONAL, -- Need OR lateNonCriticalExtension OCTET STRING OPTIONAL, ... } MBMS-SAI-List-NB-r14 ::= SEQUENCE (SIZE (1..maxSAI-MBMS-r11)) OF MBMS-SAI-r11 MBMS-SAI-InterFreqList-NB-r14 ::= SEQUENCE (SIZE (1..maxFreq)) OF MBMS-SAI-InterFreq-NB-r14 MBMS-SAI-InterFreq-NB-r14 ::= SEQUENCE { dl-CarrierFreq-r14 CarrierFreq-NB-r13, mbms-SAI-List-r14 MBMS-SAI-List-NB-r114, multiBandInfoList-r14 AdditionalBandInfoList-NB-r14 OPTIONAL -- Need OR } | CR |
| H.068 | 6.7.2 SystemInformationBlockType1-NB | schedulingInfoList-v14xy introduces a mechanism for SIB type extension as there is no spare value left. However, there may be simpler way to extend | 3 | To discuss whether this is the best way to extend IE Sib-MappingInfo-NB-r13 to allow signalling of new sib-Type without impacting legacy UEs Discuss and conclude | TBC |
| I.036 | 6.7.2 SystemInformationBlockType1-NB | Description of condition sibTypeExt: The condition is used for the optional field sib-TypeExt-r14. Therefore, saying “The field is mandatory present” is not correct. Furthermore, to improve readability the word “for” can be removed. | 2 | Correct the description of condition sibTypeExt as shown below. sibTypeExt The field is mandatoryoptionally present, need OR, if, for the corresponding entry in SIB-MappingInfo-r13 is set to sibTypeExt. Otherwise the field is not present. | CR |
| N.181 | 6.7.2 SystemInformationBlockType1-NB::ce-authorisationOffset | Clarify field description ce-AuthorisationOffset Parameter “Qoffsetauthorization“ in TS 36.304 [4]. If absent, the UE applies the value of ce-authorisationOffset in SystemInformationBlockType1-NB, if present. Otherwise, the UE applies the (default) value of 0 dB for “Qoffsetauthorization” in TS 36.304 [4]. | 2 | Change latter sentence to: If the field is absent, UE shall use value of 0 dB for “Qoffsetauthorization“. | CR |
| N.182 | 6.7.2 SystemInformationBlockType1-NB::powerClass14dBm-Offset, SystemInformationBlockType3-NB::powerClass14dBm-Offset | Clarify field description | 2 | The text is quite long and could be simplified, e.g. as: powerClass14dBm-Offset Parameter “Poffset” in TS 36.304 [4]. powerClass14dBm-Offset is only applicable for UE supporting powerClassNB-14dBm. If the field is absent, the UE applies the (default) value of 0 dB for “Poffset” in TS 36.304 [4] | CR |
| H.071 | 6.7.2 SystemInformationBlockType20-NB | Parameter sc-mcch-CarrierConfig-r14 is defined as mandory present. However, it would be better to define as OPTIONAL need OP, with absence meaning that the anchor carrier is used. | 2 | Change as follows: sc-mcch-CarrierConfig-r14 DL-CarrierConfigCommon-NB-r14 OPTIONAL, -- Need OP sc-mcch-CarrierConfig Downlink carrier that can be is used for SC-MCCH. If the field is absent, the anchor carrier is used for SC-MCCH. | CR |
| H.072 | 6.7.2 SystemInformationBlockType20-NB | Parameter sc-mcch-SchedulingInfo-r14 is defined as mandory present. However, it should be to define as OPTIONAL need OP, with absence meaning that DRX is not used. Also, the field description is missing | 2 | Change as follows: sc-mcch-SchedulingInfo-r14 SC-MCCH-SchedulingInfo-NB-r14 OPTIONAL, -- Need OP sc-mcch-SchedulingInfo DRX information for the SC-MCCH. If this field is absent, DRX is not used for SC-MCCH reception. | CR |
| H.073 | 6.7.2 SystemInformationBlockType20-NB | Field description of parameter npdcch-NumRepetitions-SC-MCCH is missing | 2 | Add the field description as follows: npdcch-NumRepetitions-SC-MCCH The maximum number of NPDCCH repetitions the UE needs to monitor for SC-MCCH multicast search space, see TS 36.213 [23]. | CR |
| H.074 | 6.7.2 SystemInformationBlockType20-NB | IE SC-MCCH-SchedulingInfo-NB-r14: The DRX mechanism is identical for SC-MCCH and SC-MTCH. Hower different parameter names have been used which leads to text duplication in MAC specification. It would better to use the same parameter names | 2/3 | Rename the parameters as follows: - sc-mcch-onDurationTimerSCPTM-r14 - sc-mcch-InactivityTimerSCPTM-r14 | CR |
| H.075 | 6.7.2 SystemInformationBlockType20-NB | A word is missing in the field description of parameter sc-mcch-InactivityTimerSCPTM | 2 | sc-mcch-InactivityTimerSCPTM Timer for SC-MCCH reception in TS 36.321 [6]. Value in number of NPDCCH periods. Value pp1 corresponds to 1 NPDCCH period, pp2 corresponds to 2 NPDCCH periods and so on. | CR |
| H.076 | 6.7.2 SystemInformationBlockType22-NB | A word is missing in the field description of parameter nprach-MultiCarrierConfig | 2 | Change as follows nprach-MultiCarrierConfig Provide the configuration for random access on non-anchor carriers. | CR |
| H.077 | 6.7.2 SystemInformationBlockType22-NB | Field description of pagingWeight and pagingWeightAnchor are not in alphabetical order | 2 | Move the description after pcch-ConfigList | CR |
| H.078 | 6.7.2 SystemInformationBlockType22-NB | It could be clarifed that the number of entries in pcch-ConfigList is the same as the number of entries in dl-CarrierConfigList and that if an entry is empty in pcch-ConfigList then the corresponding carrier in dl-CarrierConfigList is not used for paging | 2 | Change as follows pcch-ConfigList Configure the PCCH parameters for each non-anchor DL carrier . E-UTRAN includes the same number of entries, and listed in the same order, as in dl-CarrierConfigList . If entry is empty in pcch-ConfigList, then the DL non anchor carrier in the corresponding entry in dl-CarrierConfigList is not used for paging. | CR |
| H.079 | 6.7.2 SystemInformationBlockType22-NB | nprach-ConfigList : It could be clarifed that the number of entries in nprach-ConfigList is the same as the number of entries in ul-CarrierConfigList | 2 | Change as follows nprach-ConfigList Configure the NPRACH parameters for each non-anchor UL carrier. E-UTRAN includes the same number of entries, and listed in the same order, as in ul-CarrierConfigList . | CR |
| H.080 | 6.7.2 SystemInformationBlockType22-NB | NPRACH-ParametersList-NB-r14 : It could be clarifed that the number of entries in NPRACH-ParametersList-NB-r14 is the same as the number of entries in NPRACH-ParametersList-NB-r13 in SIB2-NB or 0 if the UL carrier is not used for PRACH | 2 | Change as follows nprach-ParametersList Configure NPRACH parameters for each NPRACH resource on one non-anchor UL carrier . Up to three NPRACH resources can be configured on one non-anchor UL carrier. Each NPRACH resource is associated with a different number of NPRACH repetitions. E-UTRAN includes the same number of entries, and listed in the same order, as in NPRACH-ParametersList-NB-r13 in SystemInformationBlockType2-NB or 0 entry, in which case the corresponding UL non anchor carrier is not used for PRACH. | CR |
| H.081 | 6.7.2 SystemInformationBlockType22-NB | nprach-ProbabilityAnchorList-r14 : It could be clarifed that the number of entries in nprach-ProbabilityAnchorList-r14 is the same as the number of entries in NPRACH-ParametersList-NB-r13 in SIB2-NB. The description of the selection probability for a non-anchor carrier NPRACH resource is not fully correct | 2 | Change as follows nprach-ProbabilityAnchorList Configure the NPRACH selection probability for each NPRACH resource on the anchor carrier. p0 corresponds to a probability of 0., px2 FFS …. E-UTRAN includes the same number of entries, and listed in the same order, as in NPRACH-ParametersList-NB-r13 in SystemInformationBlockType2-NB All non-anchor carriers PRACH resources have equal probability between them and the probability of selecting a given non-anchor carrier NPRACH resource is (p1001- nprach-ProbabilityAnchor)/(number of non-anchor NPRACH resources) | CR |
| H.082 | 6.7.2 SystemInformationBlockType22-NB | NPRACH-ProbabilityAnchor-NB-r14: the meaning of the different enumerated values is not defined | 4 | To be handled via contribution at next meeting. | TDoc WI (Huawei) |
| H.083 | 6.7.2 SystemInformationBlockType22-NB | The field description for the default values is not very clear | 2 | Change as follows: npdcch-NumRepetitions-RA Maximum number of repetitions for NPDCCH common search space (CSS) for RAR, Msg3 retransmission and Msg4, see TS 36.213 [23, 16.6]. If absent, the value of npdcch-NumRepetitions-RA configured in SystemInformationBlockType2-NB for the NPRACH resource in the corresponding entry of NPRACH-ParametersList nprach-ParametersList in SystemInformationBlockType2-NB applies. npdcch-Offset -RA Fractional period offset of starting subframe for NPDCCH common search space (CSS Type 2), see TS 36.213 [23, 16.6]. If absent, the value of npdcch-Offset-RA configured in SystemInformationBlockType2-NB for the NPRACH resource in the corresponding entry of NPRACH-ParametersList nprach-ParametersList in SystemInformationBlockType2-NB applies. npdcch-StartSF-CSS-RA Starting subframe configuration for NPDCCH common search space (CSS), including RAR, Msg3 retransmission, and Msg4, see TS 36.213 [23, 16.6]. If absent, the value of npdcch-StartSF-CSS-RA configured in SystemInformationBlockType2-NB for the NPRACH resource in the corresponding entry of NPRACH-ParametersList nprach-ParametersList in SystemInformationBlockType2-NB applies. nprach-NumCBRA-StartSubcarriers The number of start subcarriers for contention based random access. The UE shall select one of these start subcarriers when randomly selecting a start subcarrier in the preamble selection in 36.321 [6]. The start subcarrier indexes that the UE is allowed to randomly select from are according to the following: nprach-SubcarrierOffset + [0, nprach-NumCBRA-StartSubcarriers - 1] If absent, the value of nprach-NumCBRA-StartSubcarriers configured in SystemInformationBlockType2-NB for the NPRACH resource in the corresponding entry of NPRACH-ParametersList nprach-ParametersList in SystemInformationBlockType2-NB applies. nprach-NumSubcarriers Number of sub-carriers in a NPRACH resource, see TS 36.211 [21, 10.1.6]. In number of subcarriers. If absent, the value of nprach-NumSubcarriers configured in SystemInformationBlockType2-NB for the PRACH resource in the corresponding entry of NPRACH-ParametersList nprach-ParametersList in SystemInformationBlockType2-NB applies. nprach-Periodicity Periodicity of a NPRACH resource, see TS 36.211 [21, 10.1.6]. Unit in millisecond. If absent, the value of nprach-Periodicityconfigured in SystemInformationBlockType2-NB for the NPRACH resource in the corresponding entry in NPRACH-ParametersList nprach-ParametersList in SystemInformationBlockType2-NB applies. nprach-StartTime Start time of the NPRACH resource in one period, see TS 36.211 [21, 10.1.6]. Unit in millisecond. If absent, the value of nprach-StartTime configured in SystemInformationBlockType2-NB for the NPRACH resource in the corresponding entry of NPRACH-ParametersList nprach-ParametersList in SystemInformationBlockType2-NB applies. nprach-SubcarrierOffset Frequency location of the NPRACH resource, see TS 36.211 [21, 10.1.6]. In number of subcarriers, offset from sub-carrier 0. If absent, the value of nprach-SubcarrierOffset configured in SystemInformationBlockType2-NB for the NPRACH resource in the corresponding entry of in NPRACH-ParametersListnprach-ParametersList in SystemInformationBlockType2-NB applies. nprach-SubcarrierMSG3-RangeStart Fraction for calculating the starting subcarrier index of the range reserved for indication of UE support for multi-tone Msg3 transmission, within the NPRACH resource, see TS 36.211 [21, 10.1.6]. Multi-tone Msg3 transmission is not supported for {32, 64, 128} repetitions of NPRACH. For at least one of the NPRACH resources with the number of NPRACH repetitions other than {32, 64, 128}, the value of nprach-SubcarrierMSG3-RangeStart should not be 0. If nprach-SubcarrierMSG3-RangeStart is equal to {oneThird} or {twoThird} the start subcarrier indexes for the two partitions are given by: nprach-SubcarrierOffset + [0, floor(nprach-NumCBRA-StartSubcarriers * nprach-SubcarrierMSG3-RangeStart) -1] for the single-tone Msg3 NPRACH partition; nprach-SubcarrierOffset + [floor(nprach-NumCBRA-StartSubcarriers * nprach-SubcarrierMSG3-RangeStart), nprach-NumCBRA-StartSubcarriers - 1] for the multi-tone Msg3 NPRACH partition; If absent, the value of nprach-SubcarrierMSG3-RangeStart configured in SystemInformationBlockType2-NB for the NPRACH resource in the corresponding entry of NPRACH-ParametersList nprach-ParametersList in SystemInformationBlockType2-NB applies. | CR |
| N.186 | 6.7.2 SystemInformationBlockType22-NB:: nprach-Parameters-r14 | Fields that are Need OP have a bit imprecise wording on field absence condition | 2 | See N.004 |  |
| N.183 | 6.7.2 SystemInformationBlockType3-NB::ce-authorisationOffset | Clarify field description | 2 | Based on N.181, change field description to: ce-AuthorisationOffset Parameter “Qoffsetauthorization“ in TS 36.304 [4]. If the field is absent, the UE applies the value of ce-authorisationOffset according to presence of ce-authorisationOffset in SystemInformationBlockType1-NB if present. Otherwise, the UE applies the (default) value of 0 dB for “Qoffsetauthorization” in TS 36.304 [4]. | CR |
| H.069 | 6.7.2 SystemInformationBlockType5-NB | The field description of scptm-FreqOffset indicates to use infinite dBs in absence of the IE. The usual terminology is value of infinity | 2 | Change as follows: scptm-FreqOffset Parameter QoffsetSCPTM in TS 36.304 [4]. Actual value QoffsetSCPTM = field value * 2 [dB]. If the field is not present, the value of infinity shall be used for QoffsetSCPTM. the UE uses infinite dBs for the SC-PTM frequency offset with cell ranking as specified in TS 36.304 [4]. | CR |
| N.185 | 6.7.2 SystemInformationBlockType5-NB:: scptm-FreqOffset | Clarify field description | 2 | Change to: scptm-FreqOffset Parameter QoffsetSCPTM in TS 36.304 [4]. Actual value QoffsetSCPTM = field value * 2 [dB]. If the field is not present, the UE uses infinite dBs for the SC-PTM frequency offset with cell ranking considers the SC-PTM frequency offset with cell ranking to have value “infinite”, as specified in TS 36.304 [4]. | CR |
| N.184 | 6.7.2 SystemInformationBlockType5-NB::powerClass14dBm-Offset | Clarify field description | 2 | Based on N.182, change field description to: powerClass14dBm-Offset Parameter “Poffset” in TS 36.304 [4], only applicable for UE supporting powerClassNB-14dBm. If the field is absent, the UE applies the value according to presence of ce-authorisationOffset in SystemInformationBlockType1-NB.” |  |
| H.092 | 6.7.2 UE-Capability-NB | UE-Capability-NB-v14xy-IEs : rf-Parameters-v14xy is defined as Mandatory present. It could made OPTIONAL like the other parameters | 2 | Change as follows: UE-Capability-NB-v14xy-IEs ::= SEQUENCE { ue-Category-NB-r14 ENUMERATED {nb2} OPTIONAL, mac-Parameters-r14 MAC-Parameters-NB-r14 OPTIONAL, phyLayerParameters-v14xy PhyLayerParameters-NB-v14xy OPTIONAL, rf-Parameters-v14xy RF-Parameters-NB-v14xy OPTIONAL, nonCriticalExtension SEQUENCE {} OPTIONAL } | CR |
| H.093 | 6.7.2 UE-Capability-NB | multicarrier-NPRACH: there is no reference of where the parameter is used. | 2 | Change as follows: multicarrier-NPRACH Defines whether the UE supports NPRACH on non anchor carrier as defined in TS 36.321 [6] | CR |
| H.094 | 6.7.2 UE-Capability-NB | twoHARQ-Processes: there is no reference of where the parameter is used. | 2 | Change as follows: twoHARQ-Processes Defines whether the UE supports two HARQ processes operation in DL and UL as defined in TS 36.212 [22] and TS 36.213 [23]. | CR |
| H.095 | 6.7.2 UE-RadioPagingInfo-NB | multiCarrierPaging: there is no reference of where the parameter is used. | 2 | Change as follows: multiCarrierPaging Indicates whether the UE supports paging on non anchor carriers. as defined in TS 36.304 [4] | CR |
| L.057 | 6.7.2 NB-IoT Message definitions | The order of system information block type in NB-IoT is not clear. | 2 | Correct the definition as follows; sib16-r13 SystemInformationBlockType16-NB-r13, sib15-v14xy SystemInformationBlockType15-NB-r14, sib20-v14xy SystemInformationBlockType20-NB-r14, sibx-v14xy SystemInformationBlockTypeX-NB-r14, ..., sib15-v14xy SystemInformationBlockType15-NB-r14, sib20-v14xy SystemInformationBlockType20-NB-r14, sibx-v14xy SystemInformationBlockTypeX-NB-r14 | CR |
| 6.7.3 NB-IoT information elements |  |  |  |  |  |
| M.031 | 6.7.3 NPRACH-ConfigSIB-NB | In the description of rsrp-ThresholdsPrachInfoList, the p-Max in the equation should be “p-max” (lower case). | 1 | modify the “p-Max” in the equation as “p-max” | CR |
| Q.026 | 6.7.3 RLC-COnfig-NB IE | Shouldn’t the presence of T-Reordering in RLC-Config-NB-14xy be conditional on presence of twoHARQ-ProcessingConfig-r14 | 2 | Make T-Reordering in RLC-Config-NB-14xy conditional on presence of twoHARQ-ProcessingConfig-r14 | CR |
| N.188 | 6.7.3 RLC-Config-NB-r13 | Extension addition group is missing RLC-Config-NB-r13 ::= CHOICE { am SEQUENCE { ul-AM-RLC-r13 UL-AM-RLC-NB-r13, dl-AM-RLC-r13 DL-AM-RLC-NB-r13 }, ......, } um-Uni-Directional-DL-r14 SEQUENCE { dl-UM-RLC-r14 DL-UM-RLC-NB-r14 } } | 2 | Add [[ ]] to enclose parts after ellipsis Ericsson: Brackets are used for SEQUENCE types but they are not used for CHOICE types. RLC-Config-NB-r13 is a CHOICE type. Therefore, brackets are not needed here after ellipsis. | CR |
| Q.018 | 6.7.3 SystemInformationBlockType1-NB message | Couldn’t the R14 extensions to SIB3-NB and SIB5-NB be done in the same way, i,e, add both IEs directly as extension to intraFreqCellReselectionInfo-R13 like it is done for SIB5-NB. | 2 | Align R14 extension to SIB3-NB and SIB5-NB. | CR |
| Q.019 | 6.7.3 SystemInformationBlockType22-NB message | PCCH-MultiCarrierConfig-NB-r14 would reference one or more non-anchor downlink carriers defined in DL-CarrierConfigCommonList-NB-r14 therefore presense of PCCH-MultiCarrierConfig-NB-r14 is conditional on presense of DL-CarrierConfigCommonList-NB-r14 | 2 | Add condition that if PCCH-MultiCarrierConfig-NB-r14 is present then DL-CarrierConfigCommonList-NB-r14 also needs to be present. | CR |
| Q.020 | 6.7.3 SystemInformationBlockType22-NB message | NPRACH-MultiCarrierConfig-NB-r14 would reference one or more non-anchor downlink carriers defined in DL-CarrierConfigCommonList-NB-r14 therefore presense of NPRACH-MultiCarrierConfig-NB-r14 is conditional on presense of DL-CarrierConfigCommonList-NB-r14 | 2 | Add condition that if NPRACH-MultiCarrierConfig-NB-r14 is present then DL-CarrierConfigCommonList-NB-r14 also needs to be present. | CR |
| Q.021 | 6.7.3 SystemInformationBlockType22-NB message | NPRACH-MultiCarrierConfig-NB-r14 would reference one or more non-anchor uplink carriers defined in UL-CarrierConfigCommonList-NB-r14 therefore presense of NPRACH-MultiCarrierConfig-NB-r14 is conditional on presense of UL-CarrierConfigCommonList-NB-r14 | 2 | Add condition that If NPRACH-MultiCarrierConfig-NB-r14 is present then UL-CarrierConfigCommonList-NB-r14 also needs to be present.. | CR |
| Q.022 | 6.7.3 SystemInformationBlockType22-NB message | PCCH-Config-NB-r14 does no explicitly indicate which carrier defined in DL-CarrierConfigCommonList-NB-r14 does npdcch-NumRepetitionsPaging-r14 or pagingWeight-r14 applies to. Clarification needed how enteries in PCCH-ConfigList-NB-r14 relates to enteries in DL-CarrierConfigCommonList-NB-r14. | 2 | Define if there is one to one relationship between the enteries in the two lists and also how to handle the case if one list is shorter than than the other. |  |
| Q.023 | 6.7.3 SystemInformationBlockType22-NB message | NPRACH-ParametersList-NB-r14 does no explicitly indicate which carrier defined in UL-CarrierConfigCommonList-NB-r14 do the parameters relate to. Clarification needed how enteries in NPRACH-ConfigList-NB-r14 relates to enteries in UL-CarrierConfigCommonList-NB-r14. | 2 | Define if there is one to one relationship between the enteries in the two lists and also how to handle the case if one list is shorter than than the other. |  |
| Q.024 | 6.7.3 SystemInformationBlockType22-NB message | Npdcch-CarrierIndex-r14 may refer to downlink non-anchor carrier that is not in the DL-CarrierConfigCommonList-NB-r13 | 2 | Define how UE should handle the case if npdcch-CarirerIndex-r14 is not in DL-CarriuerConfigCommonList-NB-r14. That is, if npdcch-CarrierIndex-r14 does not exist then that NPRACH parameters for that carrier is invalid and not be used by the UE. |  |
| M.028 | 6.7.3 SystemInformationBlockTypeX-NB | For PagingWeight-NB-r14, do we need that many choices of weight (w1~w16)? Consider a scheme with 3 carriers with weight = w1~w4, we already have 55 different weight combinations. | 3 | Give fewer choices of relative weight, e.g., PagingWeight-NB-r14 ::= ENUMERATED {w1, w2, w3, w4} | CR |
| M.029 | 6.7.3 SystemInformationBlockTypeX-NB | In the description of pagingWeight, the text “The paging load for a carrier ’i’ is equal to wi/sum(w)” is confusing. Is “w1” (1) the weight for carrier #1, or (2) a relative weight of 1? Here it means (1), but in the first sentence of the description of pagingWeight, it means (2)? | 2 | Use w(i) in the descprion text. | CR |
| M.030 | 6.7.3 SystemInformationBlockTypeX-NB | In the description of nprach-ProbabilityAnchorList, the text “px2 FFS .…” is confusing. We believe that px2~px16 means some kind of probabilities, but the values are FFS. | 2 | Change “px2 FFS ….” to “ px2 to px16 are choices for probability of NPRACH resources on anchor carrier. The values are FFS.” | CR |
| I.056 | 6.7.3 UE-Capability-NB | Field description of accessStratumRelease, ue-Category-NB-r13, ue-Category-NB-r14 can be improved by replacing “rel13” with “rel14”, and by listing the explicit categories nb1 and nb2. | 2 | Update field description of accessStratumRelease, ue-Category-NB-r13, ue-Category-NB-r14 as shown below. accessStratumRelease Set to rel14rel13 in this version of the specification. ue-Category-NB-r13, ue-Category-NB-r14 UE category as defined in TS 36.306 [5]. Value nb1 corresponds to UE category NB1, value nb2 corresponds to UE category NB2. A UE shall always include the field ue-Category-NB-r13 is always included in this version of the specification. | CR |
| I.057 | 6.7.3 UE-RadioPagingInfo-NB | In UE-Capability-NB field descriptions it has been specified that the field ue-Category-NB-r13 is always included in this version of the specification. This should be also reflected in the field description of ue-Category-NB. UE-RadioPagingInfo-NB-r13 ::= SEQUENCE { ue-Category-NB-r13 ENUMERATED {nb1} OPTIONAL, ..., [[ multiCarrierPaging-r14 ENUMERATED {true} OPTIONAL ]] } | 2 | Update the field description of ue-Category-NB as shown below. ue-Category-NB UE NB-IoT category as defined in TS 36.306 [5]. A category NB2 UE shall always include the field ue-Category-NB-r13 in this version of the specification. Coordinator (Ericsson): Sounds good but is this a Rel-13 or Rel-14 issue? I.e. what does “in this version of the specification” refer to? Rel-13 or Rel-14 specifcation? | CR |
| N.190 | 6.7.3 UL-CarrierConfigCommon-NB | Why is this IE needed? UL-CarrierConfigCommon-NB-r14 ::= SEQUENCE { ul-CarrierFreq-r14 CarrierFreq-NB-r13, ... } | 2 | UL-CarrierConfigCommon-NB-r14 is defined as a sequence containing only ul-CarrierFreq-r14 (ul-CarrierFreq-r14 CarrierFreq-NB-r13,) and ellipsis. Do we know this needs to be extended? If not, no need to define a new type, but rather directly use the CarrierFreq-NB-r13 wherever required and create extension later on as seen necessary. Coordinator (Ericsson): Same as E.058, Q.025. | CR |
| E.058 | 6.7.3.2 UL-CarrierConfigCommon-NB-r14 | This information elements is referenced only once and it is not imported to any other module. Why is this new data type needed? UL-CarrierConfigCommon-NB-r14 ::= SEQUENCE { ul-CarrierFreq-r14 CarrierFreq-NB-r13, ... } | 2 | To be confirmed why this new data type is needed. If there are no reasons, include the ul-CarrierFreq-r14 field in the information element (or message) where UL-CarrierConfigCommon-NB-r14 is now referenced. Coordinator (Ericsson): Same as N.190, Q.025. | CR/ TBC |
| E.00?? | 6.7.3.2 NPRACH-ConfigSIB-NB | Incorrect reference to P-Max in rsrp-ThresholdsPrachInfoList. | 1 | RSRP threshold = Signalled RSRP threshold - min{0, (14-min(23, p-MaxP-Max)) where p-maxP-Max: is the value of p-Max field in SystemInformationBlockType1-NB. | CR |
| Q.025 | 6.7.3 UL-CarrierConfigCommon-NB | Is there need to define new IE that only consists of CarrierFreq-NB-r13 IE? | 2 | Propose to replace UL-CarrierConfigCommon-NB with CarrierFreq-Nb-r13 and remove definition of new UL-CarrierConfigCommon-NB IE. The new IE is only used in one place (definiiton od UL-CarrierConfigCommonList-NB-r14. Coordinator (Ericsson): Same as E.058, N.190. | CR |
| N.189 | 6.7.3 UL-CarrierConfigCommon-NB:: ul-CarrierFreq | Field description should be improved | 2 | Proposal: ul-CarrierFreq UL carrier frequency of non-anchor carrier. | TBD |
| I.039 | 6.7.3.1 SystemInformationBlockType22-NB | As explained in R2-1701699, the current structure separates the DL and UL carrier configuration from the paging and NPRACH configuration for the non-anchor carrier. This result in having to index the UL carrier for every NPRACH configuration or replicating the list of UL carrier for every NPRACH configuration. Likewise for the paging configuration. SystemInformationBlockType22-NB-r14 ::= SEQUENCE { dl-CarrierConfigList-r14 DL-CarrierConfigCommonList-NB-r14 OPTIONAL, -- Need OR ul-CarrierConfigList-r14 UL-CarrierConfigCommonList-NB-r14 OPTIONAL, -- Need OR pcch-MultiCarrierConfig-r14 PCCH-MultiCarrierConfig-NB-r14 OPTIONAL, -- Need OR nprach-MultiCarrierConfig-r14 NPRACH-MultiCarrierConfig-NB-r14 OPTIONAL, -- Need OR lateNonCriticalExtension OCTET STRING OPTIONAL, ... } | 3 or 4 | Our proposal is to include the NPRACH configuration and the paging configuration directly into the list containing the UL or DL carrier information as shown follow. We also plan to bring a contribution on this to the RAN2#97bis meeting in April. SystemInformationBlockTypeX-NB-r14 ::= SEQUENCE { dl-Config-nonAnchorList-r14 DL-Config-nonAnchorList-NB-r14 OPTIONAL, -- Need OR ul-Config-nonAnchorList-r14 UL-Config-nonAnchorList-NB-r14 OPTIONAL, -- Need OR ... } DL-Config-nonAnchorList-NB-r14 ::= SEQUENCE (SIZE (1.. maxNonAnchorCarriers-NB-r14)) OF DL-ConfigSIB-nonAnchor-NB-r14 UL-Config-nonAnchorList-NB-r14 ::= SEQUENCE (SIZE (1.. maxNonAnchorCarriers-NB-r14)) OF UL-ConfigSIB-nonAnchor-NB-r14 DL-ConfigSIB-nonAnchor-NB-r14 ::= SEQUENCE { dl-CarrierConfig-r14 DL-CarrierConfigCommon-NB-r14, pcch-nonAnchorConfig-r14 PCCH-NonAnchorConfig-NB-r14 OPTIONAL -- Need OR ... } UL-ConfigSIB-nonAnchor-NB-r14 ::= SEQUENCE { ul-CarrierFreq-r14 CarrierFreq-NB-r13, nprach-NonAnchorParametersList-r14 NPRACH-NonAnchorParametersList-NB-r14 OPTIONAL -- Need OR ... } | CR/ TDoc WI (Intel) |
| I.040 | 6.7.3.1 SystemInformationBlockType22-NB | As explained in R2-1701699, RAN2 has also agreed the below: =>Use different carrier selection probability for anchor and non-anchor carriers. Detail signalling is FFS. To adopt the above agreement nprach-NumSubcarriers-r14 and nprach-Periodicity-r14 should be common among the non-anchor carrier to reduce overhead. NPRACH-Parameters-NB-r14 ::= SEQUENCE { nprach-Parameters-r14 SEQUENCE { nprach-Periodicity-r14 ENUMERATED {ms40, ms80,ms160, ms240, ms320, ms640, ms1280, ms2560} OPTIONAL, -- NEED OP nprach-StartTime-r14 ENUMERATED {ms8, ms16, ms32, ms64, ms128, ms256, ms512, ms1024} OPTIONAL, -- NEED OP nprach-SubcarrierOffset-r14 ENUMERATED {n0, n12, n24, n36, n2, n18, n34, spare1} OPTIONAL, -- NEED OP nprach-NumSubcarriers-r14 ENUMERATED {n12, n24, n36, n48} OPTIONAL, -- NEED OP nprach-SubcarrierMSG3-RangeStart-r14 ENUMERATED {zero, oneThird, twoThird, one} OPTIONAL, -- NEED OP npdcch-NumRepetitions-RA-r14 ENUMERATED {r1, r2, r4, r8, r16, r32, r64, r128, r256, r512, r1024, r2048, spare4, spare3, spare2, spare1} OPTIONAL, -- NEED OP npdcch-StartSF-CSS-RA-r14 ENUMERATED {v1dot5, v2, v4, v8, v16, v32, v48, v64} OPTIONAL, -- NEED OP npdcch-Offset-RA-r14 ENUMERATED {zero, oneEighth, oneFourth, threeEighth} OPTIONAL, -- NEED OP nprach-NumCBRA-StartSubcarriers-r14 ENUMERATED {n8, n10, n11, n12, n20, n22, n23, n24,n32, n34, n35, n36, n40, n44, n46, n48} OPTIONAL, -- NEED OP npdcch-CarrierIndex-r14 INTEGER (1..maxNonAnchorCarriers-NB-r14) OPTIONAL, -- Need OP ... } OPTIONAL -- Need OR } | 3 or 4 | Our proposal is to remove nprach-NumSubcarriers-r14 and nprach-Periodicity-r14 from IE NPRACH-Parameters-NB-r14, and to create a new structure as shown below. We also plan to bring a contribution on this to the RAN2#97bis meeting in April. SystemInformationBlockTypeX-NB-r14 ::= SEQUENCE { dl-NonAnchorCarrierConfigList-NB-r14 SEQUENCE (SIZE (1.. maxNonAnchorCarriers-NB-r14)) OF DL-ConfigSIB-nonAnchor-NB-r14 OPTIONAL, -- Need OR ul-NonAnchorCarrierConfigList-NB-r14 SEQUENCE (SIZE (1.. maxNonAnchorCarriers-NB-r14)) OF UL-ConfigSIB-nonAnchor-NB-r14 OPTIONAL, --Need OR nprach-NonAnchorNumSubcarrierList-NB-r14 NPRACH-NonAnchorNumSubcarrierList-NB-r14 OPTIONAL, -- Need OR ... } UL-ConfigSIB-nonAnchor-NB-r14 ::= SEQUENCE { ul-CarrierFreq-r14 CarrierFreq-NB-r13, nprach-NonAnchorParametersList-NB-r14 NPRACH-NonAnchorParametersList-NB-r14 OPTIONAL, -- Need OR … } NPRACH-NonAnchorParametersList-NB-r14 ::= SEQUENCE (SIZE (1.. maxNPRACH-Resources-NB-r13)) OF NPRACH-Parameters-NB-r14 NPRACH-NonAnchorNumSubcarrierList-NB-r14 ::= SEQUENCE (SIZE (1.. maxNPRACH-Resources-NB-r13)) OF NPRACH-SubcarriersParameters-NB-r14 NPRACH-Parameters-NB-r14 ::= SEQUENCE { nprach-Parameters-r14 SEQUENCE { nprach-Periodicity-r14 ENUMERATED {ms40, ms80, ms160, ms240, ms320, ms640, ms1280, ms2560} OPTIONAL, -- NEED OP nprach-StartTime-r14 ENUMERATED {ms8, ms16, ms32, ms64, ms128, ms256, ms512, ms1024} OPTIONAL, -- NEED OP nprach-SubcarrierOffset-r14 ENUMERATED {n0, n12, n24, n36, n2, n18, n34, spare1} OPTIONAL, -- NEED OP nprach-NumCBRA-StartSubcarriers-r14 ENUMERATED {n8, n10, n11, n12, n20, n22, n23, n24, n32, n34, n35, n36, n40, n44, n46, n48} OPTIONAL, -- NEED OP npdcch-CarrierIndex-r14 INTEGER (1..maxNonAnchorCarriers-NB-r14) OPTIONAL -- Need OP } OPTIONAL -- Need OR } NPRACH-SubcarriersParameters-NB-r14 ::= SEQUENCE { nprach-SubcarriersParameters-r14 SEQUENCE { nprach-NumSubcarriers-r14 ENUMERATED {n12, n24, n36, n48} OPTIONAL, -- NEED OP nprach-SubcarrierMSG3-RangeStart-r14 ENUMERATED {zero, oneThird, twoThird, one} OPTIONAL -- NEED OP } OPTIONAL -- Need OR } | CR/ TDoc WI (Intel) |
| I.037 | 6.7.3.1 SystemInformationBlockType3-NB field descriptions | The field descriptions of ce-AuthorisationOffset and powerClass14dBm-Offset can be improved by adding definition of the values. Furthermore, a redundant “is” in the description of powerClass14dBm-Offset needs to be removed. | 2 | Update the field descriptions of ce-AuthorisationOffset and powerClass14dBm-Offset as shown below. ce-AuthorisationOffset Parameter “Qoffsetauthorization“ in TS 36.304 [4]. Value in dB. Value dB6 corresponds to 6 dB, dB12 corresponds to 12 dB and so on. If absent, the UE applies the value of ce-authorisationOffset in SystemInformationBlockType1-NB, if present. Otherwise, the UE applies the (default) value of 0 dB for “Qoffsetauthorization” in TS 36.304 [4]. powerClass14dBm-Offset Parameter “Poffset” in TS 36.304 [4]. Value in dB. Value dB-6 corresponds to -6 dB, dB-3 corresponds to -3 dB and so on. powerClass14dBm-Offset is is only applicable for UE supporting powerClassNB-14dBm. If absent, the UE applies the (default) value of 0 dB for “Poffset” in TS 36.304 [4]. | CR |
| I.038 | 6.7.3.1 SystemInformationBlockType5-NB field descriptions | The field descriptions of ce-AuthorisationOffset and powerClass14dBm-Offset can be improved by adding definition of the values. Furthermore, a redundant “is” in the description of powerClass14dBm-Offset needs to be removed. | 2 | Update the field descriptions of ce-AuthorisationOffset and powerClass14dBm-Offset as shown below. ce-AuthorisationOffset Parameter “Qoffsetauthorization“ in TS 36.304 [4]. Value in dB. Value dB6 corresponds to 6 dB, dB12 corresponds to 12 dB and so on. If absent, the UE applies the value of ce-authorisationOffset in SystemInformationBlockType1-NB, if present. Otherwise, the UE applies the (default) value of 0 dB for “Qoffsetauthorization” in TS 36.304 [4]. powerClass14dBm-Offset Parameter “Poffset” in TS 36.304 [4]. Value in dB. Value dB-6 corresponds to -6 dB, dB-3 corresponds to -3 dB and so on. powerClass14dBm-Offset is is only applicable for UE supporting powerClassNB-14dBm. If absent, the UE applies the (default) value of 0 dB for “Poffset” in TS 36.304 [4] | CR |
| E.057 | 6.7.3.2 RadioResourceConfigDedicated-NB | rlc-Config-v14xy field is defined as optional Need ON. How is the field supposed to be released? | 2 | To be confirmed whether the field ever needs to be released. If so, add setup/release construct as follows [[ rlc-Config-v14xy CHOICE { release NULL, setup RLC-Config-NB-v14xy } OPTIONAL -- Need ON ]] | CR/ TBC |
| Z.022 | 6.7.3.6 UE-Capability-NB | After introduction of powerClassNB-14dBm, only when neither powerClassNB-14dBm nor powerClassNB-20dBm is reported by UE, it can be considered that the UE supportes power class 23 dBm. | 3 | powerClassNB-20dBm Defines whether the UE supports power class 20dBm in NB-IoT for the band, as specified in TS 36.101 [42]. If neither powerClassNB-14dBm nor powerClassNB-20dBm is not included, UE supports power class 23 dBm in the NB-IoT band. | CR |
| I.041 | 6.7.3.6 UE-Capability-NB | rai-Support-r14 is added in MAC parameters group, but this is not aligned with the 36.306 CR R2-1702402 where it is added as general parameters; capability multiCarrierPaging-r14 is missing (in the 36.306 CR R2-1702402 added in the NB-IoT enhancements group); multiCarrier-NPRACH-r14 is added in PHY parameters group, but in the 36.306 CR R2-1702402 it is added in the NB-IoT enhancements group. MAC-Parameters-NB-r14 ::= SEQUENCE { dataInactMon-r14 ENUMERATED {supported} OPTIONAL, rai-Support-r14 ENUMERATED {supported} OPTIONAL } PhyLayerParameters-NB-v14xy ::= SEQUENCE { multiCarrier-NPRACH-r14 ENUMERATED {supported} OPTIONAL, twoHARQ-Processes-r14 ENUMERATED {supported} OPTIONAL } | 2 | Discuss in which group to add rai-Support-r14, multiCarrierPaging-r14 and multiCarrier-NPRACH-r14 in RRC. Proposal is to add rai-Support-r14 on highest level in UE-Capability-NB-v14xy-IEs, and to add multiCarrierPaging-r14 and multiCarrier-NPRACH-r14 in PhyLayerParameters-NB-v14xy. | CR |
| 6.7.3.7a SC-PTM information elements (new section) |  |  |  |  |  |
| Q.027 | 6.7.3.7 MBMS information element | Has same title as section 6.3.7 and this could be confusing. | 2 | Propose to change title to ‘NB-IoT MBMS information elements’ to be consistend with naming convention in section 6.7.3. | CR |
| Q.028 | 6.7.3.7a SC-MTCH-Info-List-NB IE | The number of cells in SCPTM-NeighbourCellList-NB can be less than than size of sc-mtch-NeighbourCell-r14 and for this case UE behaviour is not defined. | 2 | DefineUE behaviour something like ‘Only the first n bits in sc-mtch-NeighbourCell bitmap shall be considered, where n is the number of cells in scptmNeighbourCellList. | CR |
| 6.7.4 NB-IoT RRC multiplicity and type constraint values |  |  |  |  |  |
| 6.7.5 Direct Indication Information |  |  |  |  |  |
| 7.1 UE variables |  |  |  |  |  |
| 7.1a NB-IoT UE variables |  |  |  |  |  |
| 7.2 Counters |  |  |  |  |  |
| 7.3 Timers |  |  |  |  |  |
| I.042 | 7.3.1 Timers (Informative) | T341 is set to the bw-PreferenceIndicationTimer and a value of s0 (0 seconds) can be configured for this timer. Therefore, “NOTE2” referring to the handling of zero value for a timer should be added in the table. 7.3.2 Timer handling When the UE applies zero value for a timer, the timer shall be started and immediately expire unless explicitly stated otherwise. | 2 | Add “NOTE2” to the timer T341. T341 NOTE2; Upon transmitting UEAssistanceInformation message with bw-Preference.. | CR |
| M.032 | 7.3.1 Txxx | The starting condition of the newly introduced timer Txxx, “Upon receiving txxx”, seems to simple. It should contains some information about the purpose of this timer. | 2 | Change the starting condition to “Upon receiving redirectedCarrierOffsetDedicated included in RedirectedCarrierInfo” Coordinator (Ericsson): Same as E.063. | CR |
| E.063 | 7.3.1 T322 | Timer start condition is not descriptive and compliant with the start condition description for other timers. | 2 | Change the condition to; Upon receiving redirectedCarrierOffsetDedicated at RRCConnectionReleaseUpon receiving t322. Coordinator (Ericsson): Same as M.032. | CR |
| N.191 | 7.3.1 T341: Upon transmitting UEAssistanceInformation message with bw-Preference. | The description of what timeout value to use is not specified. | 2 | Locate the procedure where this timer value needs to be set. |  |
| N.192 | 5.3.7.2, 5.6.10.2, 5.6.10.3 Txyz found in 3 places | The timer description is missing | 2 | Substitute the right timer value here |  |
| 7.4 Constants |  |  |  |  |  |
| 8 Protocol data unit abstract syntax |  |  |  |  |  |
| 9.1 Specified configurations |  |  |  |  |  |
| 9.2 Default radio configurations |  |  |  |  |  |
| 9.3 ProSe pre-defined configurations |  |  |  |  |  |
| L.058 | 9.3.2 | Wrong suffix. | 2 | Change from SL-OffsetIndicatorSync-v14xy to SL-OffsetIndicatorSync-r14 | CR |
| Z.023 | 9.3.2 SL-Preconfiguration | Why the two IE for V2X “SL-PreconfigV2X-Sync-r14” and “SL-V2X-SyncOffsetIndicators-r14” were included in SL-Preconfiguration? | 2 | It is suggested to move the two IE for V2X “SL-PreconfigV2X-Sync-r14” and “SL-V2X-SyncOffsetIndicators-r14” to be included in “SL-V2X-Preconfiguration”. | CR |
| E.059 | 9.3.2 SL-V2X-Preconfiguration | There is a recommendation that “the field should overwrite” but what does that mean? This should be written from the UE perspective. restrictResourceReservationPeriod If configured, the field should overwrite restrictResourceReservationPeriod configured in v2x-ResourceSelectionConfig for transmission on this pool. | 2 | It should be confirmed whether this should be a recommendation or a requirement. It should be then reformulated such that the text is written for the UE and not for the field or the field’s value. | CR/ TBC |
| L.059 | 9.3.2 SL-V2X-Preconfiguration | The range of thresSL-TxPrioritization-r14 does not need to include ‘0’ and ‘1’ since the current range of PPPP is 1 to 8 and 1 means that UL is always prioritized. | 2 | Suggested value range is 2~8 or use SL-Priority-r13 | CR |
| L.060 | 9.3.2 SL-V2X-Preconfiguration | Zone based resource pool selection is not applied to pre-configured P2X related V2X sidelink communication pool | 2 | zoneID Indicates the zone ID for which the UE shall use this resource pool as described in 5.10.13.2. The field is absent in v2x-CommRxPoolList and p2x-CommTxPoolList-r14 in SL-V2X-PreconfigFreqInfo. | CR |
| 10.1 General |  |  |  |  |  |
| 10.2 Inter-node RRC messages |  |  |  |  |  |
| E.913 | 10.2.2 SCG-ConfigInfo | measGapConfig-r14 is missing | 2 | Include measGapConfig-r14 SCG-ConfigInfo-v14xy-IEs ::= SEQUENCE { makeBeforeBreakSCG-Req-r14 ENUMERATED {true} OPTIONAL, measGapConfig-r14 MeasGapConfig-r14 OPTIONAL, nonCriticalExtension SEQUENCE {} OPTIONAL } And in section 10.2.1: EUTRA-InterNodeDefinitions DEFINITIONS AUTOMATIC TAGS ::= BEGIN IMPORTS AntennaInfoCommon, AntennaInfoDedicated-v10i0, ARFCN-ValueEUTRA, ARFCN-ValueEUTRA-v9e0, ARFCN-ValueEUTRA-r9, CellIdentity, C-RNTI, DL-DCCH-Message, DRB-Identity, DRB-ToReleaseList, FreqBandIndicator-r11, InDeviceCoexIndication-r11, LWA-Config-r13, MasterInformationBlock, maxBands, maxFreq, maxDRB, maxSCell-r10, maxSCell-r13, maxServCell-r10, maxServCell-r13, MBMSInterestIndication-r11, MeasConfig, MeasGapConfig, MeasGapConfig-r14, MeasResultForRSSI-r13, MeasResultListWLAN-r13, OtherConfig-r9, PhysCellId, P-Max, PowerCoordinationInfo-r12, SidelinkUEInformation-r12, SL-CommConfig-r12, SL-DiscConfig-r12, RadioResourceConfigDedicated, RCLWI-Configuration-r13, RSRP-Range, RSRQ-Range, RSRQ-Range-v1250, RS-SINR-Range-r13, SCellToAddModList-r10, SCellToAddModListExt-r13, SCG-ConfigPartSCG-r12, SecurityAlgorithmConfig, SCellIndex-r10, SCellIndex-r13, SCellToReleaseList-r10, SCellToReleaseListExt-r13, ServCellIndex-r10, ServCellIndex-r13, ShortMAC-I, MeasResultSSTD-r13, SL-V2X-ConfigDedicated-r14, SystemInformationBlockType1, SystemInformationBlockType1-v890-IEs, SystemInformationBlockType2, UEAssistanceInformation-r11, UECapabilityInformation, UE-CapabilityRAT-ContainerList, UE-RadioPagingInfo-r12, WLANConnectionStatusReport-r13, WLAN-OffloadConfig-r12 FROM EUTRA-RRC-Definitions; | CR |
| N.193 | 10.2.2 Inter node definitions | Should we add measGapConfig-r14 to inter node definitions? | 3 | Needs discussion |  |
| 10.3 Inter-node RRC information element definitions |  |  |  |  |  |
| N.194 | 10.3 sourceSL-V2X-CommConfig-r14 | The message RRCConnectionReconfiguration contains both sl-V2X-ConfigDedicated-r14 and sl-P2X-ConfigDedicated-r14. But only the first IE is provided in AS-Config. | 3 | Discuss if both are needed in AS-Config (depends also on resolution to N.014, N.054 ) |  |
| 10.4 Inter-node RRC multiplicity and type constraint values |  |  |  |  |  |
| 10.5 Mandatory information in AS-Configuration |  |  |  |  |  |
| 10.6 Inter-node NB-IoT messages |  |  |  |  |  |
| 10.7 Inter-node NB-IoT RRC information element definitions |  |  |  |  |  |
| 10.8 Inter-node RRC multiplicity and type constraint values |  |  |  |  |  |
| 11 UE capability related constraints and performance requirements |  |  |  |  |  |
| Annex A (informative): Guidelines, mainly on use of ASN.1 |  |  |  |  |  |
| I.043 | B.1 Feature group indicators | In Table B.1-1, “Notes” column for FGI 18, 23, 42 the reference “or M2” is missing. | 2 | Add the missing “or M2” in the “Notes” column for FGI 18, 23, 42. 18; - can only be set to 1 if the UE has set bit number 5 and bit number 25 to 1. - If a category M1 or M2 UE does not support this feature group, this bit shall be set to 0.. 23; - If a category M1 or M2 UE does not support this feature group, this bit shall be set to 0.. 42; - If a category M1 or M2 UE supports this feature group, this bit shall be set to 1. For a UE of all other categories, this bit shall be set to 0.. | CR |
| I.044 | C.1 Feature group indicators | In Table C.1-1, “Notes” column for FGI 115 the reference “or M2” is missing | 2 | Add the missing “or M2” in the “Notes” column for FGI 115. 115; - If a category M1 or M2 UE does not support this feature group, this bit shall be set to 0.. | CR |
| Annex B (normative): Release 8 and 9 AS feature handling |  |  |  |  |  |
| N.195 | FGI4 | For FGIs that need to be set to “Yes”, FGI4 setting to “Yes” (which has been there since quite some time) has been deleted for unclear reasons | 2 | Keep the “Yes” for FGI4 (at least for normal UEs) | CR |
| D.003 | FGI 25 | There is an inconsistency between TS 36.306 and TS36.331. ceMeasurements-r14 was introduced as a UE capability bit, which is defined in TS 36.306 as follows: 4.3.6.23 ceMeasurements-r14 This field defines whether the UE supports intra-frequency RSRQ measurements and inter-frequency RSRP and RSRQ measurements in RRC_CONNECTED, as specified in TS 36.133 [16], TS 36.304 [14] and TS 36.331 [5]. In this release of specification, it is mandatory for UEs of Category M1 and M2 to support ceMeasurements-r14. Since Category M2 was introduced in Rel-14, UEs of Category M2 shall support inter-frequency measurement. However, it is described in TS 36.331 that FGI 25, which indicates the support of inter-frequency measurement, shall be set to 0 “if a category M1 or M2 UE does not support this feature group”. | 2 | Remove the words “or M2” in the Notes column for FGI 25. 25; - If a category M1 or M2 UE does not support this feature group, this bit shall be set to 0.. | CR |

# Sections not part of the review (for information)

| No | Clause(s) | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- | --- |
| 5.7 Generic error handling |  |  |  |  |  |
| 6.1 General |  |  |  |  |  |
| 8.1 General |  |  |  |  |  |
| 8.2 Structure of encoded RRC messages |  |  |  |  |  |
| 8.3 Basic production |  |  |  |  |  |
| 8.4 Extension |  |  |  |  |  |
| 11.1 UE capability related constraints |  |  |  |  |  |
| 11.2 Processing delay requirements for RRC procedures |  |  |  |  |  |

# Next issue list (Annex)

| Company | Next issue |
| --- | --- |
| CATT | C.019 |
| Ericsson | E.064, E.138, E.934 |
| HTC | T.006 |
| Huawei | H.098 |
| Intel | I.058 |
| LG Electronics Inc | L.061 |
| MediaTek | M.033 |
| Nokia | N.196 |
| NTT DOCOMO | D.004 |
| Qualcomm | Q.001 |
| Rohde & Schwarz | R.001 |
| Samsung | S.065 |
| Sequans | U.001 |
| Sierra Wireless | W.001 |
| Sony | Y.001 |
| ZTE | Z.024 |

Typical use of class

| Description | Class | Remarks |
| --- | --- | --- |
| Use of need codes/ conditions | 2 | Class 1 may be considered for some rather trivial cases i.e. not really requireing any discussion e.g. system information |

# Closed issues (Annex)

| No | Clause(s) | Description | Class | Details (proposed solution/ discussion) | Status/ ref |
| --- | --- | --- | --- | --- | --- |
| General |  |  |  |  |  |
| N.001 | Void |  |  |  | Closed |
| 3.1 Definitions |  |  |  |  |  |
| 3.2 Abbreviations |  |  |  |  |  |
| 4 General |  |  |  |  |  |
| 5.1 General |  |  |  |  |  |
| 5.2 System information |  |  |  |  |  |
| I.003 | 5.2.2.3 System information required by the UE | The condition “depending on support of multi-carrier paging/NPRACH” is only needed for real optional features, but support of multi-carrier paging/NPRACH is mandatory with IOT bits. Therefore, the condition can be removed. Furthermore, a redundant space can be removed. 3> the MasterInformationBlock (or MasterInformationBlock-NB in NB-IoT), SystemInformationBlockType1-BR (or SystemInformationBlockType1-NB in NB-IoT) and SystemInformationBlockType2 (or SystemInformationBlockType2-NB in NB-IoT) , and for NB-IoT SystemInformationBlockType22-NB (depending on support of multi-carrier paging/NPRACH); | 2 | Remove the condition “depending on support of multi-carrier paging/NPRACH” and the redundant space. Coordinator (Ericsson): Same as I.004. Can be closed. | Closed |
| 5.3 Connection control |  |  |  |  |  |
| N.015 | 5.3.12 UE actions upon leaving RRC_CONNECTED | NOTE indicates “RRCs”, but it’s not clear how there could be multiple RRC instances | 2 | Use “RRC” instead of “RRCs” Coordinator (Ericsson): Corrected during v14.2.0 CR implementation. | Closed |
| D.001 | 5.3.3.4 Reception of the RRCConnectionSetup by the UE | Editorial correction (need to insert a line feed) | 1 | 2> include dcn-id if a DCN value (see TS 23.401 [41]) is received from upper layers; 2> submit the RRCConnectionSetupComplete message to lower layers for transmission, upon which the procedure ends; The above sentence should be changed as follows: 2> include dcn-id if a DCN value (see TS 23.401 [41]) is received from upper layers; 2> submit the RRCConnectionSetupComplete message to lower layers for transmission, upon which the procedure ends; Coordinator (Ericsson): Corrected during v14.2.0 CR implementation | Closed |
| I.007 | 5.3.3.x Failure to deliver RRCConnectionSetupComplete message | Section number 5.3.3.x could be corrected to 5.3.3.3b. | 2 | Correct section number 5.3.3.x to 5.3.3.3b. Coordinator (Ericsson): Corrected during v14.2.0. CR implementation | Closed |
| E.107 | 5.3.7.2 Initiation 5.6.10.2 Initiation, 5.6.10.3 Actions related to transmission of UEAssistanceInformation message | The timer “Txyz” should get a proper value. Section 5.3.7.2: 1> release bw-Config, if configured and stop timer Txyz, if running; Section 5.6.10.2: 2> if the current maximum PDSCH/PUSCH bandwidth preference is different from the one indicated in the last transmission of the UEAssistanceInformation message and timer Txyz is not running; Section 5.6.10.3: 1> start timer Txyz with the timer value set to the bw-PreferenceIndicationTimer; | 1 | Replace “Txyz” with a suitable value. Coordinator (Ericsson): Corrected during v14.2.0 CR implementation | Closed |
| I.010 | 5.3.7.2 Initiation | In the timer table in section 7.3.1 the timer related to PDSCH/PUSCH bandwidth preference has been defined as T341. 1> release bw-Config, if configured and stop timer Txyz, if running; | 2 | Replace “Txyz” with “T341. Coordinator (Ericsson): Corrected during v14.2.0 CR implementation | Closed |
| N.022 | 5.3.7.2 Inititation | 1> release bw-Config, if configured and stop timer Txyz, if running; | 2 | Identify the Timer. Isn’t this T341? Coordinator (Ericsson): Corrected during v14.2.0 CR implementation. | Closed |
| 5.4 Inter-RAT mobility |  |  |  |  |  |
| 5.5 Measurements |  |  |  |  |  |
| 5.6 Other |  |  |  |  |  |
| E.108 | 5.6.18.2 Initiation | Reference “5.6.X.3” is incorrect. 3> initiate transmission of the DelayBudgetReport message in accordance with 5.6.X.3; | 2 | Replace with “5.6.18.3”. 3> initiate transmission of the DelayBudgetReport message in accordance with 5.6.X18.3; Coordinator (Ericsson): Corrected during v14.2.0 CR implementation. | Closed |
| S.007 | 5.6.10.2 | Bandwidth preference reporting timer should be named (also affects other specification parts) | 1 | Change Txyz to T341 Coordinator (Ericsson): Corrected during v14.2.0 CR implementation. | Closed |
| I.014 | 5.6.10.2 Initiation | In the timer table in section 7.3.1 the timer related to PDSCH/PUSCH bandwidth preference has been defined as T341. 2> if the current maximum PDSCH/PUSCH bandwidth preference is different from the one indicated in the last transmission of the UEAssistanceInformation message and timer Txyz is not running; | 2 | Replace “Txyz” with “T341. Coordinator (Ericsson): Corrected during v14.2.0 CR implementation. | Closed |
| N.036 | 5.6.10.2 Initiation (of UE Assistance Information) | Undefined Txyz is used in the procedure. Related to N.022 | 2 | Identify the Timer. Isn’t it T341? Coordinator (Ericsson): Corrected during v14.2.0 CR implementation. | Closed |
| I.015 | 5.6.10.3 Actions related to transmission of UEAssistanceInformation message | In the timer table in section 7.3.1 the timer related to PDSCH/PUSCH bandwidth preference has been defined as T341. 1> start timer Txyz with the timer value set to the bw-PreferenceIndicationTimer; | 2 | Replace “Txyz” with “T341. Coordinator (Ericsson): Corrected during v14.2.0 CR implementation. | Closed |
| 5.8 MBMS |  |  |  |  |  |
| 5.8a SC-PTM |  |  |  |  |  |
| 5.9 RN procedures |  |  |  |  |  |
| 5.10 ProSe |  |  |  |  |  |
| 6.2.1 General message structure |  |  |  |  |  |
| 6.2.2 Message definitions |  |  |  |  |  |
| S.009 | Void |  |  |  | Closed |
| E.018 | 6.2.2 RRCConnectionSetupComplete | dcn-ID field is missing release suffix. | 1 | Add release suffix dcn-ID-r14 INTEGER (0..65535) OPTIONAL, Coordinator (Ericsson): Corrected during v14.2.0 CR implementation. | Closed |
| I.016 | 6.2.2 RRCConnectionSetupComplete | To be consistent the field name in ASN.1 to be corrected to "dcn-Id-r14" and to “dcn-Id” in the field description. RRCConnectionSetupComplete-v14xy-IEs ::= SEQUENCE { dcn-ID INTEGER (0..65535) OPTIONAL, nonCriticalExtension SEQUENCE {} OPTIONAL } dcn-ID The Dedicated Core Network Identity, see TS 23.401 [41]. | 2 | Correct field name “dcn-ID” to "dcn-Id-r14" in ASN.1 and to “dcn-Id” in the field description. Coordinator (Ericsson): Corrected during v14.2.0 CR implementation. | Closed |
| E.021 | 6.2.2 SystemInformationBlockType1 | NonMBSFN-SubframeConfig information element is missing release suffix. | 1 | Add the missing release suffix, nonMBSFN-SubframeConfig-r14 NonMBSFN-SubframeConfig-r14 OPTIONAL, --Need OR and NonMBSFN-SubframeConfig-r14 ::= SEQUENCE { Coordinator (Ericsson): Corrected during v14.2.0 CR implementation. | Closed |
| E.022 | 6.2.2 SystemInformationBlockType1 | PLMN-IdentityInfo-MBMS information element is missing release suffix. | 1 | Add the missing release suffix, cellAccessRelatedInfo-r14 SEQUENCE { plmn-IdentityList-r14 PLMN-IdentityList-MBMS-r14, and PLMN-IdentityList-MBMS-r14 ::= SEQUENCE (SIZE (1..maxPLMN-r11)) OF PLMN-IdentityInfo-MBMS-r14 and PLMN-IdentityInfo-MBMS-r14 ::= SEQUENCE { Coordinator (Ericsson): Corrected during v14.2.0 CR implementation. | Closed |
| E.023 | 6.2.2 SystemInformationBlockType1 | SchedulingInfoList-MBMS and SchedulingInfo-MBMS information elements are missing release suffix. SIB-MappingInfo-MBMS and SIB-Type-MBMS information element are missing release suffix. | 1 | Add the missing release suffix, schedulingInfoList-MBMS-r14 SchedulingInfoList-MBMS-r14, si-WindowLength-r14 ENUMERATED { and SchedulingInfoList-MBMS-r14 ::= SEQUENCE (SIZE (1..maxSI-Message)) OF SchedulingInfo-MBMS-r14 and SchedulingInfo-MBMS-r14 ::= SEQUENCE { si-Periodicity-r14 ENUMERATED {rf16, rf32, rf64, rf128, rf256, rf512}, sib-MappingInfo-r14 SIB-MappingInfo-MBMS-r14 } and SIB-MappingInfo-MBMS-r14 ::= SEQUENCE (SIZE (0..maxSIB-1)) OF SIB-Type-MBMS-r14 and SIB-Type-MBMS-r14 ::= ENUMERATED { Coordinator (Ericsson): Corrected during v14.2.0 CR implementation. | Closed |
| 6.3.1 System information blocks |  |  |  |  |  |
| 6.3.2 Radio resource control information elements |  |  |  |  |  |
| E.031 | 6.3.2 PhysicalConfigDedicated | srsCcSetIndexList field and reference to SrsCcSetIndex information element are missing release suffix. | 1 | Add the missing release suffix in SoundingRS-AperiodicSet-r14 ::= SEQUENCE{ srsCcSetIndexList-r14 SEQUENCE (SIZE (1..4)) OF SrsCcSetIndex-r14 OPTIONAL, -- Cond Srs-Trigger-TypeA and SoundingRS-AperiodicSetUpPTsExt-r14 ::= SEQUENCE{ srsCcSetIndexList-r14 SEQUENCE (SIZE (1..4)) OF SrsCcSetIndex-r14 OPTIONAL, -- Cond Srs-Trigger-TypeA Coordinator (Ericsson): Corrected during v14.2.0 CR implementation. | Closed |
| H.045 | 6.3.2 RadioResourceConfigDecicated | Same issue as H.033 | 2 | Add the following change CRS-AssistanceInfo-r11 ::= SEQUENCE { physCellId-r11 PhysCellId, antennaPortsCount-r11 ENUMERATED {an1, an2, an4, spare1}, mbsfn-SubframeConfigList-r11 MBSFN-SubframeConfigList, ..., [[ mbsfn-SubframeConfigList-v14xy MBSFN-SubframeConfigList-v14xy OPTIONAL ]] } Coordinator (Ericsson): Same as H.033. Can be closed | Closed |
| L.037 | 6.3.2 SPS-Config | It is not mandatory to configure semiPersistSchedIntervalUL-v14x0. | 2 | Make the field optional semiPersistSchedIntervalUL-v14x0 ENUMERATED { sf50, sf100, sf200, sf300, sf400, sf500, sf600, sf700, sf800, sf900, sf1000, spare5, spare4, spare3, spare2, spare1} OPTIONAL -- Need OR ]] Coordinator (Ericsson): Corrected during v14.2.0 CR implementation. The field must be defined as optional present because otherwise the whole extension addition group becomes mandatory present which is not allowed since all extensions should be optional. | Closed |
| E.032 | 6.3.2 SRS-TPC-PDCCH-Config | srsCcSetIndexList field and (both definition and reference to) SrsCcSetIndex information element and its fields are missing release suffix. | 1 | Add the missing release suffix in SRS-TPC-PDCCH-Config-r14 ::= CHOICE { release NULL, setup SEQUENCE { srs-TPC-RNTI-r14 BIT STRING (SIZE (16)), startingBitOfFormat3B-r14 INTEGER (0..31), fieldTypeFormat3B-r14 INTEGER (1..4), srsCcSetIndexlist-r14 SEQUENCE (SIZE(1..4)) OF SrsCcSetIndex-r14 OPTIONAL -- Cond Srs-Trigger-TypeA } } SrsCcSetIndex-r14 ::= SEQUENCE { ccSetIndex-r14 INTEGER (0..3), ccIndexInOneCcSet-r14 INTEGER (0..7) } Coordinator (Ericsson): Corrected during v14.2.0 CR implementation. | Closed |
| E.034 | 6.3.2 UplinkPowerControlCommonPUSCH-LessCell-v14xy | The information element UplinkPowerControlCommonPUSCH-LessCell-v14xy contain fields that are missing -r14 release suffix. | 1 | Add the missing release suffix UplinkPowerControlCommonPUSCH-LessCell-v14xy ::= SEQUENCE { p0-Nominal-PeriodicSRS-r14 INTEGER (-126..24) OPTIONAL, -- Need OR p0-Nominal-AperiodicSRS-r14 INTEGER (-126..24) OPTIONAL, -- Need OR alpha-SRS-r14 Alpha-r12 OPTIONAL -- Need OR } Coordinator (Ericsson): Corrected during v14.2.0 CR implementation. | Closed |
| E.035 | 6.3.2 UplinkPUSCH-LessPowerControlDedicated-v14xy | The information element UplinkPUSCH-LessPowerControlDedicated-v14xy contain fields that are missing -r14 release suffix. | 1 | Add the missing release suffix UplinkPUSCH-LessPowerControlDedicated-v14xy ::= SEQUENCE { p0-UE-PeriodicSRS-r14 INTEGER (-8..7) OPTIONAL, -- Need OR p0-UE-AperiodicSRS-r14 INTEGER (-8..7) OPTIONAL, -- Need OR accumulationEnabled-r14 BOOLEAN } Coordinator (Ericsson): Corrected during v14.2.0 CR implementation. | Closed |
| 6.3.3 Security control information elements |  |  |  |  |  |
| 6.3.4 Mobility control information elements |  |  |  |  |  |
| 6.3.5 Measurement information elements |  |  |  |  |  |
| E.036 | 6.3.5 MeasGapConfig | PerCC-MeasGapConfig-r14 information element contains fields that are missing release suffix. | 1 | Add the missing release suffix PerCC-MeasGapConfig-r14 ::= CHOICE { release SEQUENCE { servCellId-r14 ServCellIndex-r13 }, setup SEQUENCE { servCellId-r14 ServCellIndex-r13, measGapConfig MeasGapConfig } } Ericsson:Corrected during v14.2.0 CR implementation | Closed |
| L.040 | 6.3.5 MeasResults | Editorial correction. | 1 | Make it italic cbr-PSCCH Indicates the CBR measurement results on the PSCCH of the pool indicated by poolIdentity. This field is only included for the non-adjacent PSCCH-PSSCH pool. Coordinator (Ericsson): Corrected during v14.2.0 CR implementation. | Closed |
| H.048 | 6.3.5 MeasResults | For MeasResultListCBR-r14 included, the “tab” in front of the word “OF” should be replaced by a “space”. | 1 | MeasResultListCBR-r14 ::= SEQUENCE (SIZE (1..maxCBR-Report-r14)) OF MeasResultCBR-r14 Coordinator (Ericsson): Corrected during v14.2.0 CR implementation | Closed |
| 6.3.6 Other information elements |  |  |  |  |  |
| 6.3.7 MBMS information elements |  |  |  |  |  |
| N.157 | 6.3.7 MBSFN-AreaInfoList::subcarrierSpacingMBMS-r14 | Missing need code | 2 | Add Need code – probably OR or OP (since without this field, UE assumes 15 kHz SC spacing, OP could be justified) Coordinator (Ericsson): Same as N.154. Can be closed. | Closed |
| 6.3.7a SC_PTM information elements |  |  |  |  |  |
| 6.3.8 ProSe information elements |  |  |  |  |  |
| N.176 | 6.3.8 zoneIdLongiMod-14, zoneIdLatiMod-r14 | Confusing names | 2 | Use e.g. the following: Coordinator (Ericsson): Can be closed. | Closed |
| 6.4 RRC multiplicity and type constraints values |  |  |  |  |  |
| 6.5 PC5 RRC messages |  |  |  |  |  |
| 6.6 Direct Indication Information |  |  |  |  |  |
| 6.7.1 General NB-IoT message structure |  |  |  |  |  |
| 6.7.2 NB-IoT Message definitions |  |  |  |  |  |
| H.067 | 6.7.2 SystemInformation-NB | The new sib type is defined as ‘sibx-v14xy’. Should be sib22-r14 | 2 | Change as follows: sibx-v14xysib22-r14 SystemInformationBlockType22-NB-r14 Coordinator (Ericsson): Corrected during v14.2.0 CR implementation. | Closed |
| 6.7.3 NB-IoT information elements |  |  |  |  |  |
| 6.7.3.7a SC-PTM information elements (new section) |  |  |  |  |  |
| 6.7.4 NB-IoT RRC multiplicity and type constraint values |  |  |  |  |  |
| 6.7.5 Direct Indication Information |  |  |  |  |  |
| 7.1 UE variables |  |  |  |  |  |
| 7.1a NB-IoT UE variables |  |  |  |  |  |
| 7.2 Counters |  |  |  |  |  |
| 7.3 Timers |  |  |  |  |  |
| 7.4 Constants |  |  |  |  |  |
| 8 Protocol data unit abstract syntax |  |  |  |  |  |
| 9.1 Specified configurations |  |  |  |  |  |
| 9.2 Default radio configurations |  |  |  |  |  |
| 9.3 ProSe pre-defined configurations |  |  |  |  |  |
| 10.1 General |  |  |  |  |  |
| 10.2 Inter-node RRC messages |  |  |  |  |  |
| 10.3 Inter-node RRC information element definitions |  |  |  |  |  |
| 10.4 Inter-node RRC multiplicity and type constraint values |  |  |  |  |  |
| 10.5 Mandatory information in AS-Configuration |  |  |  |  |  |
| 10.6 Inter-node NB-IoT messages |  |  |  |  |  |
| 10.7 Inter-node NB-IoT RRC information element definitions |  |  |  |  |  |
| 10.8 Inter-node RRC multiplicity and type constraint values |  |  |  |  |  |
| 11 UE capability related constraints and performance requirements |  |  |  |  |  |
| Annex A (informative): Guidelines, mainly on use of ASN.1 |  |  |  |  |  |
| Annex B (normative): Release 8 and 9 AS feature handling |  |  |  |  |  |