3GPP TSG-RAN WG2 Meeting #98	R2-1704347

Hangzhou, China, 15 - 19 May 2017

Agenda item:	10.2.3

Source:	Nokia, Alcatel-Lucent Shanghai Bell

Title:	Scenarios for MN and SN coordination involving SCG

WID/SID:	NR_newRAT (Release 15)

Document for:	Discussion and Decision

# 1	Introduction

During the discussion of [1], the following agreements were made regarding the SN addition and release procedures and retaining the SN upon MN handover.

Agreements

1: Secondary Node Addition is used when there is no SN configured and is only initiated by the MN.

2: The recipient node of SN Release cannot reject the request.

3: Intra-SN mobility can trigger a SN modification request by the SN to MN.

FFS which scenarios require MN involvement and which don't.

4: For LTE-NR DC, MN handover can happen without SN node change.

# 2	Background

The following intra-SgNB scenarios trigger SgNB modification procedure (also referring to the section in [2]):

- Change of SCG configuration (e.g. SCell addition, SCell release, release of SCG bearer and SCG split bearer, release of SCG part of MCG split bearer)

- PSCell change

- NR SI update

- Changes to measurement coordination

- S-KeNB refresh

In the following table, we discuss if MN involvement is needed or not.

| Scenario | MN involvement | Comment |
| --- | --- | --- |
| SCell addition | Yes, conditional | SN coordinates with MN based on UE capability. |
| SCell release | Yes, conditional | SN uses direct SCG SRB and informs MN later. |
| PSCell change | Yes, conditional | SN uses direct SCG SRB and informs MN later. |
| NR SI update | Yes, conditional | SN uses direct SCG SRB and informs MN later. |
| Measurement reconfiguration | Yes, conditional | SN coordinates with MN if and only if it impacts already coordinated measurements. |
| S-KeNB refresh | Yes | MN needs to provide SN new S-KeNB |
| Release of SCG bearer | Yes | MN interaction with CN. |
| Release of SCG split bearer | Yes | MN interaction with CN. |
| Release of SCG part of MCG split bearer | Yes | MN interaction with CN. |

# 3	Conclusion

In this contribution, the intra-SgNB scenarios triggering the SgNB modification procedure were discussed. The following is proposed:

Observation 1: SN needs to involve MN for a procedure that needs a renegotiation of the UE shared capabilities and measurements to avoid UE capability being exceeded.

Proposal 1: SN may inform MN after initiating the signalling of the procedure towards the UE, if the procedure required no renegotiation for the already coordinated capability sharing and measurements.

Proposal 2: SN involves MN for all procedures involving release of SCG part of bearers.

# References

- R2-1703620: Procedures between MN and SN for NSA operation, Qualcomm Incorporated

- 3GPP TS 36.300: Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2