3GPP TSG-RAN WG2 #98	Tdoc R2-1704402

Hangzhou, P.R. of China, 15th – 19th May 2017

Agenda Item:	10.3.1.9

Source:	Ericsson

Title:	Wake-up signaling in C-DRX

Document for:	Discussion, Decision

# Introduction

There is a constant need for improving the battery lifetime of mobile devices. Thus, reducing the time a device needs to monitor the DL will help the UE save battery and extending the battery lifetime.

Active UEs in connected mode must continuously monitor the downlink for Downlink Control Information, DCI, to see if there is a downlink assignment or uplink grant to which they should respond. The power saving feature connected mode DRX, C-DRX, allows the UE to enter a sleep mode after a time of inactivity in which the UE only wakes up to monitor PDCCH according to a configured On-Duration time per C-DRX Cycle, see Figure 1.

C-DRX Cycle

On-Duration

On-Duration

C-DRX Cycle

On-Duration

Figure 1: C-DRX cycles with On-Duration periods in LTE

When transmitting a DCI to a UE, a CRC is attached to each DCI message payload. The identity of the device or devices, i.e. the Radio Network Temporary identifier, RNTI, is included in the CRC calculation and not explicitly transmitted. Depending on the purpose of the DCI, different RNTIs are used. When the UE monitors and tries to decode the PDCCH, the UE will check the CRC using a set of assigned RNTIs. If the CRC check passes, the message is declared to be correctly received and intended for the UE. Thus, the identity of the device that message is intendend for is implicitly encoded in the CRC and not explicitly transmitted.

When the UE wakes up in each C-DRX cycle, it needs to monitor the PDCCH during the On-Duration period. A typical setting for LTE C-DRX On-Duration is observed to be in the order of tens of milliseconds and thus, the UE must stay awake for the entire On-Duration even if there is no data scheduled for the UE. This could be viewed as a power waste for the UE.

Reducing monitoring time in C-DRX will reduce energy consumption

The power consumption is further increased by the fact that the UE is required to be prepared to both receive and transmit data during and immediately following reception of a valid DCI during the On-Duration, i.e.,  the UE is expected to decode the data received and provide HARQ feedback shortly after reception of a valid DCI implying that the UE in order to perform the delay sensitive processing, (HARQ RTT etc.), in C-DRX need to have more processing hardware activated which consumes more power... In most cases, no DCI will be received and energy used to prepare for transmitting will be of waste.

The more processing hardware activated, the more power is consumed.

# Wake-up signaling

The use of a wake-up signal for UEs in connected mode DRX has earlier been discussed in 3GPP. The wake-up signaling in this paper proposes an alternative solution that would avoid the use of a specific wake-up receiver and thereby potentially reducing both cost and complexity but at the same time give similar energy savings.

The length of the On-Duration timer affects the reachability of the UE and the scheduling flexibility for the network but the longer On-Duration time, the more energy is used in the UE.

Since most of the scheduling occasions during the On-Duration time will not include a DCI for the UE, it would be beneficial if the monitoring time cold be kept to a minimum without affecting the reachability and/or scheduling flexibility.

In an energy perspective, it is beneficial to keep the onDuration as short as possible.

To allow the UE to not monitor the PDCCH for the entire On-Duration every C-DRX cycle, the UE could be notified when it should prepare to monitor for the PDCCH for a DCI including a scheduling assignment. This could be done by assigning UEs configured for C-DRX with a “DRX” RNTI. Then, a very short wake-up duration could be allowed for each UE when monitoring the common search space on the PDCCH to see if any DCI has a CRC attached with the assigned DRX-RNTI included in the calculation. When the UE monitors and tries to decode the PDCCH, the UE will check the CRC using the assigned DRX-RNTI. If the CRC checks, the message is declared to be correctly received and intended for the UE and the UE would know that it e.g. should start monitoring the PDCCH according to the on-Duration time, at the next C-DRX cycle, see Figure 2.

Wake-up
Monitoring with DRX-RNTI received

On-Duration monitoring

C-DRX Cycle

C-DRX Cycle

Wake-up
Monitoring

Figure 2 Wake-up monitoring

If the CRC check fails, the message is not intended as a wake-up message for the UE and it would be able to go back to sleep until the next wake-up occasion according to the C-DRX cycle, see Figure 2.

This wake-up scheme comes with a cost of an increased delay. However, design choices could be made to limit the delay by having the UEs waking up a predefined time after the wake-up signal has been received.

NR will to a large extent rely on beamforming when communicating with the UEs. This implies that the DL beams will be steered in different directions at different point in time. Hence, allowing the network to reach a group of UEs in C-DRX, in one or few TTIs, could be beneficial and could be accomplished by using a DRX-RNTI. If several UEs are assigned the same DRX-RNTI all of them would wake-up on decoding the DRX-RNTI. This could be prevented by further using a DCI format allowing to separate UEs that needs to wake up from the ones that don’t. E.g. the DRX-RNTI encoded DCI sent in the common search space could indicate the UE IDs that should wake up.

Add a new DRX-RNTI to be used in NR.

This wake-up scheme should be regarded as an add-on to the existing C-DRX framework and used when energy savings are the highest concern.

UEs in C-DRX may be optionally configured to use wake-up signaling using the DRX-RNTI.

# Conclusion

We propose:

Proposal 1	Add a new DRX-RNTI to be used in NR.

Proposal 2	UEs in C-DRX may be optionally configured to use wake-up signaling using the DRX-RNTI.