3GPP TSG-RAN WG2 #98	Tdoc R2-1704422

Hangzhou, P.R. of China, 15th – 19th May 2017

Agenda Item:	10.2.5

Source:	Ericsson

Title:	SCG SRB setup procedures

Document for:	Discussion, Decision

# Introduction

In RAN2#97, it was agreed to support the following for LTE-NR tight interworking where LTE is the master (EN-DC)

- the splitting of SRBs

- a direct SRB from the SN.

Also, in RAN2#97bis, it was further agreed that the

- for LTE-NR tight interworking where LTE is the MN with SCG SRB configured, only one SRB is required on the SN side, and only for messages corresponding to SRB1

- the following RRC messages can be sent via the SRB in the SCG: RRCConnectionReconfiguration,  RRCConnectionReconfigurationComplete, MeasurementReport

- direct SRB from the SN will not support splitting in Rel-15

- SCG SRB is of higher scheduling priority than all DRBs.

- SCG SRB can be configured based on network decision.

- Addition of SCG SRB is decided by SN.

  - FFS Whether the MN can request establishment of SCG SRB

- SCG SRB configuration is provided by NR RRC from SN.

- FFS Whether explicit configuration is also supported for measurement reports.

- EN-DC can only be configured after security activation on LTE.

In this contribution, we discuss the procedure for the configuration of the direct SRB (referred to as SCG SRB from here onwards).

# Discussion

As agreed in RAN2#97bis, the addition of the SCG SRB is decided by the SN, while it is FFS if the MN can request it. From the MN’s point of view, if RRC diversity is needed, it can be achieved through the configuration of split SRB1/2. Thus, the setup of the SCG SRB is solely to facilitate the NR RRC configuration from the SN and transfer of NR RRC measurement reports from the UE to the SN. Thus, we propose:

- The SCG SRB addition is decided by the SN and its configuration and usage is completely transparent to the MN.

Figure 1 shows the SN addition procedure in LTE DC. In the SeNB Addition Request message, the MeNB requests the SeNB to allocate radio resources for a specific E-RAB, indicating E-RAB characteristics and also indicates within SCG-ConfigInfo the MCG configuration and the entire UE capabilities for UE capability coordination to be used as basis for the reconfiguration by the SeNB. The SCG configuration is not included in the SCG-ConfigInfo. The SeNB includes the SCG configuration to be used in the SCG-Config information element when it responds to the addition request (assuming the SeNB’s RRM have decided to accept the addition request). This is the configuration that the MeNB passes to the UE in the RRCConnectionReconfiguration message and used by the UE to configure the SCG. The configuration of the SCG SRB can thus also be included in the SCG-Config and passed transparently to the UE in the RRCConnectionReconfiguration message.

Figure 1: SN addition procedure in LTE DC (taken from TS 36.300)

- The Rel-12 SN addition procedure can be applied for the setup of the SCG SRB by including the SRB configuration in the SCG-Config that is sent from the SN to the MN. The details are FFS.

Figure 2 shows the SN modification procedure in LTE DC. Regarding the SCG SRB configuration, the situation is like the SN addition procedure, except here that the SCG SRB can either be added (if it was not in SN addition) or released (if it was configured in the SN addition).

Figure 2: SN Modification procedure (taken from TS 36.300)

- The Rel-12 SN modification procedure can be applied for the addition or release of the SCG SRB by including the SRB configuration in the SCG-Config that is sent from the SN to the MN. The details are FFS.

# Conclusion

In this contribution, we have discussed the setup of the SCG SRB and we propose:

- The SCG SRB addition is decided by the SN and its configuration and usage is completely transparent to the MN.

- The Rel-12 SN addition procedure can be applied for the setup of the SCG SRB by including the SRB configuration in the SCG-Config that is sent from the SN to the MN. The details are FFS.

- The Rel-12 SN modification procedure can be applied for the addition or release of the SCG SRB by including the SRB configuration in the SCG-Config that is sent from the SN to the MN. The details are FFS.