3GPP TSG-RAN WG2 Meeting #98		R2-1704596

Hangzhou, China, 15th – 19th May 2017

Source:	vivo

Title:	Discussion on the SCG RRC feedback after SCG SRB release

Agenda Item:	10.2.5

Document for:	Discussion and Decision

# Introduction

According to the discussion on NR, RAN2 achieved the following consensus [1] for the SCG SRB:

Agreements:

1	SCG SRB can be configured based on network decision.

2	Addition of SCG SRB is decided by SN.

FFS Whether the MN can request establishment of SCG SRB

3	SCG SRB configuration is provided by NR RRC from SN.

4	NR RRC complete messages and measurement reports are mapped to the same SRB as the message initiating the procedure.

FFS Whether there are any exceptional cases for the complete messages

FFS Whether explicit configuration is also supported for measurement reports.

5	All LTE RRC messages are mapped to MCG SRB.

6	EN-DC can only be configured after security activation on LTE.

In this contribution, we discuss the potential options on how the UE sends the feedback RRC messages back to the network after releasing SCG SRB, and how to establish/release SCG SRB.

# Discussion

## Establishment of SCG SRB

From our understanding the SCG SRB is used for the SN to directly send the SCG configuration to the UE without the forwarding of the MN. However the MN anyway needs to be able to forward the SCG configuration at least for the addition/release of the SCG. And most likely the MCG has better coverage than the SCG. To ensure the transmission reliability of the SCG configuration, the SN may still choose to use the MCG SRB to forward the SCG configuration instead of sending the configuration via SCG. Thus even if the MN wants to request the establishment of the SCG SRB to avoid the forwarding signaling via MCG SRB, the SN may still send the SCG configuration to the MN even after establishing the SCG SRB. On the other hand the SN may anyway reject the request from the MN. As a result we consider that there is no clear use case for the MN to request the establishment of SCG SRB.

Proposal 1: The MN cannot request the establishment of SCG SRB.

## Release of SCG SRB

After the SCG SRB is established, we think that only the SN can release the SCG SRB without releasing SCG. Regarding the MN, there is no use case for the MN to release the SCG SRB without releasing SCG.

Proposal 2: The SN can trigger the release of the SCG SRB without releasing SCG.

Proposal 3: The MN cannot trigger the release of the SCG SRB without releasing SCG.

Based on Proposal 2 and 3, there are two ways for the SN to send the SCG SRB release configuration to the UE.

- Option 1: via MCG SRB

- Option 2: via SCG SRB

For the SN change, the target SN may not want to establish the SCG SRB. This means that the SCG SRB release configuration can only be sent via MCG SRB at the SN change. Moreover after the SCG SRB is established, the SN may need to release the SCG SRB in some cases (e.g. deteriorated signal strength or limited SCG coverage for fast moving UE). Then the SN can send the SCG SRB release configuration either via MCG SRB or via SCG SRB, based on the SN implementation (e.g. to ensure fast releasing of the SCG SRB).

Proposal 4: The SCG SRB release configuration can be sent via MCG SRB or SCG SRB.

## Complete/ MeasurementReport

Option 1: The SCG SRB release configuration via MCG SRB

Figure 1: SCG SRB release configuration via MCG SRB

If the SCG SRB release configuration (i.e. included in the RRCConnectionReconfiguration message of SCG) is sent via MCG SRB (i.e. in the container of the RRCConnectionReconfiguration message of MCG), the UE is able to send the complete message (i.e. the RRCConnectionReconfigurationComplete message of SCG) back to the SN via the MCG SRB (i.e. in the container of the RRCConnectionReconfigurationComplete message of MCG). The signaling flow is given in Figure 1.

Option 2: The SCG SRB release configuration via SCG SRB

Figure 2: SCG SRB release configuration via MCG SRB

If the SCG SRB release configuration (i.e. included in the RRCConnectionReconfiguration message of SCG) is sent via SCG SRB, the UE can send the complete message (i.e. the RRCConnectionReconfigurationComplete message of SCG) back to the SN via the MCG SRB (i.e. in the container of the UEAssistanceInformation message of MCG).  Then the MN can forward the complete message back to the SN.

Regarding the MeasurementReport message of SCG, the MeasurementReport corresponding to the SCG measurement configuration can be sent to the MN via MCG SRB after the release of SCG SRB, alike the signaling procedure given in Figure 2.

Thus after the release of the SCG SRB, it is possible that the UE sends the feedback RRC message of SCG via the MCG SRB, and the SCG RRC message can be included in the RRC container of  a MCG RRC message.

Proposal 5: After the release of the SCG SRB, the SCG RRC message (e.g. RRCConnectionReconfigurationComplete or MeasurementReport) is sent in the container of the MCG RRC message via MCG SRB and forwarded to the SN.

# Conclusion

According to the analysis given above, we have the following proposals:

Proposal 1: The MN cannot request the establishment of SCG SRB.

Proposal 2: The SN can trigger the release of the SCG SRB without releasing SCG.

Proposal 3: The MN cannot trigger the release of the SCG SRB without releasing SCG.

Proposal 4: The SCG SRB release configuration can be sent via MCG SRB or SCG SRB.

Proposal 5: After the release of the SCG SRB, the SCG RRC message (e.g. RRCConnectionReconfigurationComplete or MeasurementReport) is sent in the container of the MCG RRC message via MCG SRB and forwarded to the SN.

# Reference

- Chairman's Notes RAN2#97bis