3GPP TSG-RAN WG2 Meeting #98															     R2-1704634

Hangzhou, China, 15th – 19th May 2017

Title: 	Discussion on PC5 DRX

Source: 	ZTE

Agenda item:	9.1.3

Document for:	Discussion and Approval

Introduction

The primary objective of feD2D is to address power efficiency for evolved Remote UEs (e.g. wearable devices). At RAN2#97bis, it was agreed that Discontinuous RX will be studied for PC5. Details and options are FFS [1]. In this contribution, we will discuss the details and solutions for support of PC5 DRX, and give our considerations and proposals.

Discussion

Discontinuous reception over sidelink

LTE supports Discontinuous reception (DRX) for UE in order to conserve UE power. UE/eNB may initiate the DRX mode when there is no packet to be sent or received. During this mode, the UE does not listen to the DL all the time, but wakes up periodically to listen to the DL transmissions from eNB. When UE is not listening to the DL transmission, most of its circuitry is turned off. In this way, lower power consumption is achieved and the battery life of UE is extended.

With regard to sidelink communication, suppose the evolved remote UE’s DL packets forwarded by linked evolved relay UE only arrive occasionally, it is not necessary for the evolved remote UE to monitor the PC5 direct link all the time. Instead, PC5 can be enhanced with DRX behaviour for evolved remote UE to reduce the power consumption. The legacy DRX design applies for both RRC_IDLE and RRC_CONNECTED state. That is, the DRX takes effect no matter the UE sets up the RRC connection with eNB or not. When it comes to the PC5 DRX, only DRX for linked evolved remote UE should be considered. As agreed in RAN#97bis, the remote UE should be in linked state with a relay in order to receive paging from a relay UE. Thus, the support of PC5 DRX for evolved remote UE not linked with evolved relay UE is not as necessary as Uu DRX for RRC_IDLE UE. In this section, we will mainly discuss the PC5 DRX for evolved remote UE linked with evolved relay UE.

After the PC5 connection between evolved remote UE and evolved relay UE is set up, the evolved remote UE can receive or send data packet to the evolved relay UE. However, if the evolved remote UE receives nothing or has no data to transmit within the configured inactivity timer, it may enter sleep/inactivity state. Later the evolved remote UE wakes up periodically to monitor the PC5 link. PC5 DRX activity/inactivity state transition of evolved remote UE is shown in Figure 1. During this procedure, the evolved remote UE maintains PC5 connection with the evolved relay UE and evolved relay UE stores the evolved remote UE’s context.

Figure 1 PC5 DRX activity/inactivity state transition of evolved remote UE

As we can see, with PC5 DRX, it is not necessary for evolved remote UE to monitor PC5 link all the time, therefore the power consumption of evolved remote UE can be reduced. Furthermore, if evolved remote UE has no data to transmit for a long time and no MT traffic is expected, the PC5 connection between evolved remote UE and evolved relay UE can be released.

Proposal 1: Only PC5 DRX for linked evolved remote UE should be considered.

Proposal 2: The support of PC5 DRX for evolved remote UE not linked with evolved relay UE is not as necessary as Uu DRX for RRC_IDLE UE.

On the other hand, in order to support the PC5 DRX for evolved remote UE, the evolved relay UE should have knowledge of the PC5 DRX configuration of evolved remote UE so that the evolved relay UE could perform the Discontinuous transmission (DTX) over PC5. That is, when the MT packet for evolved remote UE arrives, the evolved relay UE should transmit it over the on duration of PC5 DRX for the evolved remote UE.

Proposal 3: In order to support the PC5 DRX for evolved remote UE, the evolved relay UE should have knowledge of the PC5 DRX configuration of evolved remote UE and then perform the PC5 DTX correspondingly.

PC5 DRX configuration

To support PC5 DRX of evolved remote UE, one essential issue is how to configure PC5 DRX configuration for evolved remote UE. There are mainly three ways as illustrated below.

- Option 1: evolved remote UE acquires PC5 DRX configuration directly from eNB.

Upon receiving power saving indication/PC5 DRX request from evolved remote UE, eNB may perform PC5 DRX configuration to evolved remote UE, which includes inactivity timer, on duration timer, DRX cycle and so on. For evolved remote UE with single Rx chain, eNB can configure non-overlapping on duration time between PC5 DRX and Uu DRX for evolved remote UE. After obtaining PC5 DRX configuration, evolved remote UE may perform discontinuous reception over PC5. Specifically, evolved remote UE may enter sleep when inactivity timer expires. It only wakes up when on duration time arrives periodically or it has UL data to transmit. In order to support the PC5 DTX of evolved relay UE, eNB should keep the evolved relay UE informed of the PC5 DRX configuration of evolved remote UE. Figure 2 shows PC5 DRX configuration of evolved remote UE with Option 1. This option is only applicable for in coverage evolved remote UE, and evolved remote UE should maintain both PC5 link and Uu link, which brings about additional power consumption.

Figure 2 Procedure of PC5 DRX configuration for evolved remote UE (Option1)

- Option 2: evolved relay UE configures PC5 DRX for linked evolved remote UE.

In this option, evolved remote UE may negotiate with the evolved relay UE for DRX configuration on the PC5 link. The evolved remote UE may send power saving indication/PC5 DRX request to evolved relay UE during or after PC5 connection establishment. Upon receiving the indication, evolved relay UE configures PC5 DRX parameters (such as inactivity timer, on duration timer, DRX cycle, subframe offset) for evolved remote UE. Then evolved remote UE may perform PC5 discontinuous reception according to the PC5 DRX configuration. Figure 3 shows the PC5 DRX configuration procedure for Option 2. This option is applicable to both in coverage and out of coverage evolved remote UE. PC5 AS signalling needs to be designed to support PC5 DRX configuration of evolved remote UE.

Figure 3 Procedure of PC5 DRX configuration for evolved remote UE (Option2)

- Option 3: evolved relay UE forwards PC5 DRX configuration assigned by eNB to linked evolved remote UE.

In the third option, evolved relay UE can inform eNB of power efficiency indication of the linked evolved remote UE and forward the eNB configured PC5 DRX configuration to evolved remote UE. Similar to Option 2, this option is applicable to both in coverage and out of coverage evolved remote UE. However, both PC5 AS signalling and RRC signalling needs to be designed to support PC5 DRX configuration of evolved remote UE.

Figure 4 Procedure of PC5 DRX configuration for evolved remote UE (Option3)

Among the three options for PC5 DRX configuration of evolved remote UE, Option 2 is advantageous over Option 1 and Option 3 in applicable scenarios and signalling impact. It is suggested to use Option 2 for PC5 DRX configuration for evolved remote UE.

Proposal 4: It is suggested that evolved relay UE configures PC5 DRX for linked evolved remote UE.

Conclusion

In this paper, we discussed the details for support of discontinuous reception over PC5, and presented three ways for PC5 DRX configuration of evolved remote UE. And we have the following observations and proposals:

Proposal 1: Only PC5 DRX for linked evolved remote UE should be considered.

Proposal 2: The support of PC5 DRX for evolved remote UE not linked with evolved relay UE is not as necessary as Uu DRX for RRC_IDLE UE.

Proposal 3: In order to support the PC5 DRX for evolved remote UE, the evolved relay UE should have knowledge of the PC5 DRX configuration of evolved remote UE and then perform the PC5 DTX correspondingly.

Proposal 4: It is suggested that evolved relay UE configures PC5 DRX for linked evolved remote UE.

Reference

[1] Report of RAN2#97bis.