3GPP TSG-RAN WG2 #98	Tdoc R2-1705116

Hangzhou, P.R. of China, 15th – 19th May 2017

Agenda Item:	10.3.4.2

Source:	Ericsson

Title:	QoS and SDAP layer aspects in NR

Document for:	Discussion, Decision

# Introduction

RAN2-97bis Spokane (April 2017)

Agreements: - New AS layer PDU is PDCP SDU - AS layer header is byte-aligned - DL packets over Uu are not marked with “Flow ID” at least for cases where UL AS reflective mapping and NAS reflective QoS is not configured for DRB. - AS layer header include the UL “Flow ID” depending on network configuration

# Discussion

In RAN2 97-bis meeting, decision was made to include Flow ID in SDAP header and to have the header byte aligned. Yet, the question of Flow Id length remains open. Possible sizes of flow ID are from 7 bits up to 16 bits when SDAP header is assumed to be one or two bytes and Maximum defined QFI value in QFI table defined by SA is 79 [1]. Flow ID value range with 7-bits should be already sufficient as it allows 128 flows to exist in one PDU session. Having larger Flow ID range requires UE to allocate more resources for Flow to DRB mapping.

Table 1: 3GPP TS23.501 System Architecture for 5G System, Stage2, V0.4.0 (2017-04)

5.7.5.4.2 Reflective QoS Activation via User Plane When the 5GC determines to activate reflective QoS via U-plane, the SMF shall include a QoS rule including an indication to the UPF via N4 interface to activate User Plane with user plane reflective. When the UPF receives a DL packet matching the QoS rule that contains an indication to activate reflective QoS, the UPF shall include the RQI in the encapsulation header on N3 reference point. The UE creates a UE derived QoS rule when the UE receives a DL packet with a RQI. (Reflective QoS Indication).

Based on the input from SA2 that UEs shall be told whether a DL packet requires an update of the NAS level SDF-to-Flow mapping. We propose that there is one-bit indication in the DL header. When the bit is set to 1, the UE indicates to NAS that it shall determine and possibly update the SDF-to-Flow mapping based on the Flow-ID present in the SDAP header [2].

The DL SDAP header includes 1-bit NAS-RQI indication that indicates whether the UE shall create (or update) a UE derived QoS rule.

Similarly, to NAS RQI, there could be AS RQI bit indication in SDAP header indicating whether the UE shall create or update a QoS Flow to DRB mapping. Having both, NAS and AS RQI indication would require the SDAP header to be 2-bytes assuming that a 6-bit flow id length is not sufficient.

Figure 1 shows the header when both, AS and NAS RQI exists in the header. Length of the Flow ID is between 7 and 16.

Figure 1: DL SDAP header with 8-bit Flow ID, NAS-RQI, AS-RQI and 6-bit Reserved field.

In DL, when NAS-RQI and AS-RQI fields are present and Flow ID is greater than 7-bits, SDAP header grows to 2 bytes

Having NAS-RQI and AS-RQI allows gNB transmitter to omit the AS Flow ID in downlink header when neither the NAS RQI bit nor the AS RQI bit is set. This would allow reducing the header size to one octet in all DL packets that should not trigger any filter-update. But of course, it would also result in a variable SDAP header size. Such header is presented in the figure 1.

Figure 2: DL SDAP header with 1-bit NAS-RQI, 1-bit AS-RQI and Flow ID omitted (AS RQI = 0).

The header length variation in the SDAP header and adds complexity when e.g. ROHC needs to identify the starting point of the IP packet in the PDCP layer.

Considering that updating the Flow-ID to DRB mapping is significantly easier than the update of the NAS filters, we don’t think that such an explicit indication should be added. Without that indication a header length of one byte provides space for a 7-bit Flow ID length.

From DRB to Flow ID mapping perspective in UE, it is not desirable to have longer Flow ID range than required as UE is required to maintain Flow to DRB mapping table. The range of 128 Flow IDs is sufficient for the use cases that currently exists [1].

The resulting downlink SDAP header is depicted in Figure 3.

Figure 3: DL SDAP header with 7-bit Flow ID and NAS-RQI

The DL and UL SDAP header contains a 7-bit Flow ID

For UL, Flow ID provides information to the gNB from which gNB is able to observe the QoS marking carried in the NG3 UL header. The NAS-RQI is not required. Hence the resulting UL header has one spare bit (R) for later use as shown in Figure 4.

Figure 4: UL SDAP header when 7-bit Flow ID is used

The UL SDAP header has one spare bit (R) for later use.

## Transparent mode for SDAP

As agreed in the last meeting there are cases when the SDAP header is not needed (e.g. when operating in LTE+DC mode towards EPC or when the network does not intend to use any reflective mapping). When the network does not configure the SDAP header one could model this in a way that the SDAP layer is absent. However, this makes the protocol look different depending on an RRC configuration. Hence, a cleaner solution is to model the absence of the SDAP header as “SDAP transparent mode” as done already in several other 3GPP protocols. In this way, SDAP can always be drawn on top of PDCP. Also a PDCP SDU is always an SDAP PDU.

When RRC de-configures the SDAP header, this is modelled as SDAP transparent mode.

# Conclusion

References in the text [1].

In section 2 we made the following observations:

Observation 1	In DL, when NAS-RQI and AS-RQI fields are present and Flow ID is greater than 7-bits, SDAP header grows to 2 bytes

Based on the discussion in section 2 we propose the following:

Proposal 1	The DL SDAP header includes 1-bit NAS-RQI indication that indicates whether the UE shall create (or update) a UE derived QoS rule.

Proposal 2	The DL and UL SDAP header contains a 7-bit Flow ID

Proposal 3	The UL SDAP header has one spare bit (R) for later use.

Proposal 4	When RRC de-configures the SDAP header, this is modelled as SDAP transparent mode.

# References

3GPP TS 23.501 V0.4.0 (2017-04)

R2-1704379 Reflective QoS and Flow-ID, Ericsson

R2-1703702. Further discussion on Reflective QoS. Ericsson