3GPP TSG-RAN WG2 #99	R2-1707821

Berlin, Germany, 21 - 25 August 2017

Agenda item:	9.9.3

Source:	Nokia, Nokia Shanghai Bell

Title:	Analysis for reducing signalling overhead for SCells

WID/SID:	LTE_euCA-Core - Release 15

Document for:	Discussion and Decision

# 1	Introduction

The euCA WID lists the following as its objectives:

The objective of this work item is to investigate and develop enhancements to - Reduced delays in Scell set-up, including shorter Scell configuration delay after UE moves from idle to connected by addressing the following aspects: - Measurements on Scell candidates (e.g. network assistance in identifying Scell candidate carriers, and best effort UE measurements for Scell candidates), - Measurement reporting (e.g. utilize UE’s earlier idle mode measurements for configuring Scell), - Scell configurations and activations (e.g. the network could immediately configure Scell for CA without additional measurements when UE’s earlier idle mode measurements are available for setting up Scell) - Reduced Scell configuration and activation signaling overhead - Consider optimizations related to UE mobility and carrier switching in licensed and unlicensed deployment using LAA and configurations of multiple Scells. The focus of the enhancements is especially on small cell deployment scenarios and when practical Carrier Aggregation (CA) support is extended beyond 5 carriers (up to 32 carriers). The CA enhancements developed in this work item aim to be applicable for traditional CA deployments and CA including LAA and DC deployment scenarios.

In this contribution, we analyse the signalling overhead for SCell configuration (highlighted above) to find out where it would be possible to have improvements.

# 2	Signalling overhead for SCell configuration

Currently, the configuration for each SCell is signalled separately, since it has been assumed that the configuration of each cell is very different, even if all cells reside in the same eNB. The RRC structure is such that the SCells are added one at a time, and all the radio parameters for one SCelll are separated into their own structures, as shown below:

SCellToAddMod-r10 ::=			SEQUENCE {

sCellIndex-r10						SCellIndex-r10,

cellIdentification-r10				SEQUENCE {

physCellId-r10						PhysCellId,

dl-CarrierFreq-r10					ARFCN-ValueEUTRA

}																OPTIONAL,	-- Cond SCellAdd

radioResourceConfigCommonSCell-r10		RadioResourceConfigCommonSCell-r10	OPTIONAL,	-- Cond SCellAdd

radioResourceConfigDedicatedSCell-r10	RadioResourceConfigDedicatedSCell-r10	OPTIONAL,	-- Cond SCellAdd2

...,

[[	dl-CarrierFreq-v1090				ARFCN-ValueEUTRA-v9e0	OPTIONAL	-- Cond EARFCN-max

]],

[[	antennaInfoDedicatedSCell-v10i0		AntennaInfoDedicated-v10i0	OPTIONAL	-- Need ON

]],

[[	srs-SwitchFromServCellIndex-r14		INTEGER (0.. 31) OPTIONAL	-- Need ON

]]

}

Note that the fields that typically end up being the largest are highlighted - They contain the entire dedicated radio configuration for each SCell. Even though the configurations are divided into common and dedicated parts, each SCell configuration is stand-alone: There is no one “common Scell configuration” that applies for all SCells, and a “delta configuration” that would be applied in case the individual SCells parameters differ from the common configuration. Annex A also shows some further ASN.1 examples.

Observation 1: SCell configuration is always done one SCell at a time, with no common parts for any of the SCells. The RRC signalling load of SCell configuration is N*SCellToAddMod, where N is the number of Scells.

To estimate the size required by, we make a rough estimation by simply counting the possible optional/mandatory fields inside each SCell configuration IE. The estimations are shown in Table 1 below, and are made as follows:

- The columns Min and Max correspond to theoretical ASN1 minimums and maximums.

  - The minimum (resp. maximum) is calculated considering all optional parameters are absent (resp. present), and that all strings/integers/lists have minimum (resp. maximum) size.

  - Neither minimum nor maximum usually make much sense, because they do not factor in the protocol aspects that may limit what can actually be signaled in a message.

- It is expected that the typical use case lies somewhere in between, and more likely closer to the minimum than to the maximum.

  - However, it is still clear that the signalling overhead has the possibility to be quite large – even assuming that in typical case one Scell config would be ~5% of the maximum 7.5 Ko(ctet) size, that would still take up 375 octets just for signalling one SCell configuration.

  - Considering that the maximum RRC message size is 8188 octets, this means ~21 such SCell configurations could be fitted into the maximum possible RRC message, which means that the SCell signalling for 32 SCells (current maximum in Rel-14) cannot be done with a single RRC message.

| Message | Min (bits) | Max (bits) | Min (octets) | Max (octets) |
| --- | --- | --- | --- | --- |
| SCellToAddMod-r10 [IE] | 7 | 61742 | 1 | 7.5 Ko |
| SCellToAddModExt-r13 [IE] | 9 | 61686 | 2 | 7.5 Ko |
| SCellToAddModExt-v1430 [IE] | 2 | 7 | 1 | 1 |
| PSCellToAddMod-r12 [IE] | 7 | 64900 | 1 | 7.9 Ko |

Table 1. Estimating minimum and maximum SCell IE sizes

Observation 2: The signalling load for adding even a single Scell can be rather large.

Considering e.g. an LAA use case, at unlicensed 5GHz band there can be e.g. 160 MHz bandwidth available for Scells, and the configuration of each SCell might be (almost exactly) the same, with just the center frequency and some PHY configurations being different. In such cases, the signalling overhead for each SCell is multiplied.

Observation 3: Almost the same SCell configurations may occur e.g. in LAA deployments or with multiple SCells occupying large contiguous carrier bandwidth (i.e. N*20 MHz).

Especially in dual connectivity, the entire initial SCell configuration must be transferred over X2, which also increases the X2 signalling load by the number of SCells in the MCG (from MeNB to SeNB) and SCG (from SeNB to MeNB).

Observation 4: The DC configuration requires transferring MCG and SCG SCell configurations over X2.

Based on these, it can be seen that the signalling load could be quite large even when it’s not necessary, requiring use of multiple RRC messages just to configure required SCells. Therefore, we propose to consider further a division of SCell configuration into common and dedicated parts so that the common parts could apply to all SCells, and dedicated could provide a delta on top of that.

Proposal 1: RAN2 to consider how to reduce the SCell signalling overhead for cases where the configurations of multiple SCells are almost the same (i.e. several parameters have the exact same values).

Proposal 2: RAN2 to consider introducing a common SCell configuration that can apply for multiple SCells to help with the signalling

# 3	SCell changes and handover with SCell configuration

SCell management also incurs some signalling overhead over both Uu and X2. Especially in small cell scenarios, e.g. CA scenario #4 inter-site CA, it is well-known that the number of SCell management actions (SCell add, remove, change) for the small cells (remote radio heads) is dominant over PCell handover for the macro-layer. Similarly, for Rel-12 small cell scenario 2 with DC, the number of PSCell management actions for small cells is found to be dominant, while PCell management actions (and related overhead) for the macro-layer are less significant.

To demonstrate the effects of the SCell management actions for CA scenario #4, examples of system level performance results in line with assumptions for Rel-12 DC studies are presented in the following.

Figure 1. Mobility events for a UE with CA / DC support

Figure 1 shows an example of the various PCell and SCell management actions that a UE may experience when following a certain trajectory. Traditionally, both PCell and (P)SCell management actions are network controlled, and UE assisted. This means that PCell mobility is controlled by the network based on UE RRM measurements, and similarly (P)SCell configuration/de-configuration is also network controlled (and based on UE measurement reports).

We consider first the case with macro-layer operating at 2 GHz carrier and small cells at 3.5 GHz carrier. We assume that UEs have PCell on the macro layer, while (P)SCell can be configured on the small cell (i.e. pico cell) layer. The following assumptions (same as also used in the results included in the 3GPP TR 36.842) are used:

- PCell handover at the macro-layer is assumed to be based on eNB receiving report of RSRP-based event A3.

- (P)SCell addition (configuration) and removal (de-configuration) are triggered by reception of eNB receiving report of RSRQ-based events A4 (neighbour becomes better than configured threshold) and A2 (serving cell becomes worse than configured threshold), respectively.

- Intra-frequency (P)SCell change on the pico-layer is triggered by eNB receiving report of RSRP-based event A6 (signal level from another SCell candidate becomes a threshold better than current (P)SCell).

Thus, whenever a handover, or (P)SCell addition/release, takes place, it also involves sending a RRC reconfiguration command to the UE.

Observation 5: (P)SCell management actions require both Uu and X2 signalling.

Figure 2 shows results for the number of mobility events per UE, per hour, for the case with Dual Connectivity (these same results as also included in 3GPP TR 36.842). Results are presented for the cases with either 2 or 10 randomly placed pico cells per macro cell area, and different UE speeds. UEs are free moving, meaning that each UE follows a trajectory with constant movement and direction through each simulation run. The number of events in Figure 2 is clearly dominated by (P)SCell related events (roughly 60%-80%). This is because a UE will naturally move across higher number of small cells (as compared to macro cells), and therefore experience more (P)SCell modifications than PCell changes. (Note that the “SCell Change” in the figure means “SCell Add + SCell removal” at the same time, i.e. SCell replacement. The numbers for those are not counted to the “SCell Add” or the “SCell removal” parts at all.)

Figure 2. Statistics for number of mobility events per UE per hour for case with free moving UEs.

Figure 3 shows similar statistics as in Figure 2, but these results are obtained for a scenario where picos placed in clusters, assuming either 4 or 10 picos placed randomly within a circular cluster with radius of 50 meters. Hotspot UEs are moving with 3 km/h in the cluster for this case. These assumptions for simulating cases with outdoor clustered small deployment cases are in line Rel-12 Small Cell Scenario 2a as defined in 3GPP TR 36.872. For such cluster cases, it is clearly observed from Figure 3 that the (P)SCell management operations are even more dominant – more than 90% of operations are (P)SCell related. This can be expected since UEs that are moving in a cluster of densely deployed picos naturally are much more likely to experience (P)SCell changes as compared to PCell handovers.

Figure 3. Statistics for number of mobility events per UE per hour for case with clustered pico cell deployment.

From the results in Figure 2 and Figure 3 it is clear that the (P)SCell management actions are most frequently occurring events. Each (P)SCell management action typically involves:

- A UE sending measurement report of RRM measurement events in the uplink to indicate need for (P)SCell management action (causing RRC signalling overhead in UL).

- The eNB (MeNB) processing the received RRM measurement event and decides on the appropriate (P)SCell management action (involving eNB processing and inter-eNB signalling).

- The eNB (MeNB) sending RRC reconfiguration message to the UE to signal the (P)SCell management action (involving RRC signalling overhead in DL; e.g. configuration of new (P)SCell, (P)SCell change, or (P)SCell remove).

Observation 6: (P)SCell management actions may cause significant signalling load over both Uu and X2.

Note that the same discussion happened in RAN3 during the SI “Further enhancements of small cell higher layer aspects for LTE” (see RP-142283 and TR36.876), with the discussion on the signalling load being visible in e.g. the R3-150596 and R3-150983. The conclusion from the RAN3 study was that there could be some X2 signalling benefits, RAN2 impacts prevented standardizing anything. However, with the RAN2 WID, these aspects could also be considered in this WID.

Proposal 3: RAN2 to investigate signalling overhead reduction for (P)SCell management actions.

# 4	Conclusions

We have discussed the signalling overhead reduction for SCell configuration over Uu and X2, and observed the following:

Observation 1: SCell configuration is always done one SCell at a time, with no common parts for any of the SCells. The RRC signalling load of SCell configuration is N*SCellToAddMod, where N is the number of Scells.

Observation 2: The signalling load for adding even a single Scell can be rather large.

Observation 3: Almost the same SCell configurations may occur e.g. in LAA deployments or with multiple SCells occupying large contiguous carrier bandwidth (i.e. N*20 MHz).

Observation 4: The DC configuration requires transferring MCG and SCG SCell configurations over X2.

Observation 5: (P)SCell management actions require both Uu and X2 signalling.

Observation 6: (P)SCell management actions may cause significant signalling load over both Uu and X2.

Based on these, we have proposed:

Proposal 1: RAN2 to consider how to reduce the SCell signalling overhead for cases where the configurations of multiple SCells are almost the same (i.e. several parameters have the exact same values).

Proposal 2: RAN2 to consider introducing a common SCell configuration that can apply for multiple SCells to help with the signalling

Proposal 3: RAN2 to investigate signalling overhead reduction for (P)SCell management actions.

# Annex A: Examples of SCell ASN.1 structures

The ASN.1 snippets below show some of the ASN.1 code blocks that may be configured for each SCell, highlighting the ones that are typically the largest ones.

SCellToAddMod:

SCellToAddMod-r10 ::=			SEQUENCE {

sCellIndex-r10						SCellIndex-r10,

cellIdentification-r10				SEQUENCE {

physCellId-r10						PhysCellId,

dl-CarrierFreq-r10					ARFCN-ValueEUTRA

}																OPTIONAL,	-- Cond SCellAdd

radioResourceConfigCommonSCell-r10		RadioResourceConfigCommonSCell-r10	OPTIONAL,	-- Cond SCellAdd

radioResourceConfigDedicatedSCell-r10	RadioResourceConfigDedicatedSCell-r10	OPTIONAL,	-- Cond SCellAdd2

...,

[[	dl-CarrierFreq-v1090				ARFCN-ValueEUTRA-v9e0	OPTIONAL	-- Cond EARFCN-max

]],

[[	antennaInfoDedicatedSCell-v10i0		AntennaInfoDedicated-v10i0	OPTIONAL	-- Need ON

]],

[[	srs-SwitchFromServCellIndex-r14		INTEGER (0.. 31) OPTIONAL	-- Need ON

]]

}

RadioResourceConfigCommonSCell

RadioResourceConfigCommonSCell-r10 ::=	SEQUENCE {

-- DL configuration as well as configuration applicable for DL and UL

nonUL-Configuration-r10					SEQUENCE {

-- 1: Cell characteristics

dl-Bandwidth-r10						ENUMERATED {n6, n15, n25, n50, n75, n100},

-- 2: Physical configuration, general

antennaInfoCommon-r10					AntennaInfoCommon,

mbsfn-SubframeConfigList-r10			MBSFN-SubframeConfigList	OPTIONAL,	-- Need OR

-- 3: Physical configuration, control

phich-Config-r10						PHICH-Config,

-- 4: Physical configuration, physical channels

pdsch-ConfigCommon-r10					PDSCH-ConfigCommon,

tdd-Config-r10							TDD-Config					OPTIONAL	-- Cond TDDSCell

},

-- UL configuration

ul-Configuration-r10				SEQUENCE {

ul-FreqInfo-r10						SEQUENCE {

ul-CarrierFreq-r10					ARFCN-ValueEUTRA			OPTIONAL,	-- Need OP

ul-Bandwidth-r10					ENUMERATED {n6, n15,

n25, n50, n75, n100}	OPTIONAL,	-- Need OP

additionalSpectrumEmissionSCell-r10		AdditionalSpectrumEmission

},

p-Max-r10							P-Max							OPTIONAL,	-- Need OP

uplinkPowerControlCommonSCell-r10		UplinkPowerControlCommonSCell-r10,

-- A special version of IE UplinkPowerControlCommon may be introduced

-- 3: Physical configuration, control

soundingRS-UL-ConfigCommon-r10		SoundingRS-UL-ConfigCommon,

ul-CyclicPrefixLength-r10			UL-CyclicPrefixLength,

-- 4: Physical configuration, physical channels

prach-ConfigSCell-r10					PRACH-ConfigSCell-r10		OPTIONAL,	-- Cond TDD-OR-NoR11

pusch-ConfigCommon-r10				PUSCH-ConfigCommon

}																		OPTIONAL,	-- Need OR

...,

[[	ul-CarrierFreq-v1090				ARFCN-ValueEUTRA-v9e0			OPTIONAL	-- Need OP

]],

[[	rach-ConfigCommonSCell-r11			RACH-ConfigCommonSCell-r11		OPTIONAL,	-- Cond ULSCell

prach-ConfigSCell-r11				PRACH-Config					OPTIONAL,	-- Cond UL

tdd-Config-v1130					TDD-Config-v1130				OPTIONAL,	-- Cond TDD2

uplinkPowerControlCommonSCell-v1130

UplinkPowerControlCommonSCell-v1130			OPTIONAL	-- Cond UL

]],

[[	pusch-ConfigCommon-v1270		PUSCH-ConfigCommon-v1270			OPTIONAL	-- Need OR

]],

[[	pucch-ConfigCommon-r13				PUCCH-ConfigCommon		OPTIONAL,	-- Cond UL

uplinkPowerControlCommonSCell-v1310

UplinkPowerControlCommonSCell-v1310	OPTIONAL	-- Cond UL

]],

[[	highSpeedConfigSCell-r14		HighSpeedConfigSCell-r14			OPTIONAL,	-- Need OR

prach-Config-v1430				PRACH-Config-v1430					OPTIONAL,	-- Cond UL

ul-Configuration-r14				SEQUENCE {

ul-FreqInfo-r14						SEQUENCE {

ul-CarrierFreq-r14					ARFCN-ValueEUTRA-r9			OPTIONAL,	-- Need OP

ul-Bandwidth-r14					ENUMERATED {n6, n15,

n25, n50, n75, n100}	OPTIONAL,	-- Need OP

additionalSpectrumEmissionSCell-r14		AdditionalSpectrumEmission

},

p-Max-r14							P-Max							OPTIONAL,	-- Need OP

soundingRS-UL-ConfigCommon-r14		SoundingRS-UL-ConfigCommon,

ul-CyclicPrefixLength-r14			UL-CyclicPrefixLength,

prach-ConfigSCell-r14					PRACH-ConfigSCell-r10		OPTIONAL,	-- Cond TDD-OR-NoR11

uplinkPowerControlCommonPUSCH-LessCell-v1430												UplinkPowerControlCommonPUSCH-LessCell-v1430	OPTIONAL	-- Need OR

}																	OPTIONAL,	-- Cond ULSRS

harq-ReferenceConfig-r14					ENUMERATED {sa2,sa4,sa5} 	OPTIONAL,		-- Need OR

soundingRS-FlexibleTiming-r14    			ENUMERATED {true}			OPTIONAL		-- Need OR

]],

[[	mbsfn-SubframeConfigList-v1430		MBSFN-SubframeConfigList-v1430		OPTIONAL-- Need ON

]]

}

RadioResourceConfigDedicatedSCell:

RadioResourceConfigDedicatedSCell-r10 ::=	SEQUENCE {

-- UE specific configuration extensions applicable for an SCell

physicalConfigDedicatedSCell-r10		PhysicalConfigDedicatedSCell-r10	OPTIONAL,	-- Need ON

...,

[[	mac-MainConfigSCell-r11			MAC-MainConfigSCell-r11			OPTIONAL	-- Cond SCellAdd

]],

[[	naics-Info-r12				NAICS-AssistanceInfo-r12		OPTIONAL	-- Need ON

]],

[[	neighCellsCRS-InfoSCell-r13			NeighCellsCRS-Info-r13		OPTIONAL	-- Need ON

]]

}

PhysicalConfigDedicatedScell:

PhysicalConfigDedicatedSCell-r10 ::=		SEQUENCE {

-- DL configuration as well as configuration applicable for DL and UL

nonUL-Configuration-r10					SEQUENCE {

antennaInfo-r10

AntennaInfoDedicated-r10	OPTIONAL,		-- Need ON

crossCarrierSchedulingConfig-r10

CrossCarrierSchedulingConfig-r10	OPTIONAL,		-- Need ON

csi-RS-Config-r10						CSI-RS-Config-r10		OPTIONAL,		-- Need ON

pdsch-ConfigDedicated-r10				PDSCH-ConfigDedicated	OPTIONAL		-- Need ON

}																	OPTIONAL,	-- Cond SCellAdd

-- UL configuration

ul-Configuration-r10					SEQUENCE {

antennaInfoUL-r10						AntennaInfoUL-r10		OPTIONAL,		-- Need ON

pusch-ConfigDedicatedSCell-r10

PUSCH-ConfigDedicatedSCell-r10		OPTIONAL,	-- Cond PUSCH-SCell1

uplinkPowerControlDedicatedSCell-r10

UplinkPowerControlDedicatedSCell-r10	OPTIONAL,		-- Need ON

cqi-ReportConfigSCell-r10			CQI-ReportConfigSCell-r10	OPTIONAL,		-- Need ON

soundingRS-UL-ConfigDedicated-r10

SoundingRS-UL-ConfigDedicated	OPTIONAL,		-- Need ON

soundingRS-UL-ConfigDedicated-v1020

SoundingRS-UL-ConfigDedicated-v1020	OPTIONAL,		-- Need ON

soundingRS-UL-ConfigDedicatedAperiodic-r10

SoundingRS-UL-ConfigDedicatedAperiodic-r10	OPTIONAL	-- Need ON

}																	OPTIONAL,	-- Cond CommonUL

...,

[[	-- DL configuration as well as configuration applicable for DL and UL

csi-RS-ConfigNZPToReleaseList-r11

CSI-RS-ConfigNZPToReleaseList-r11	OPTIONAL,		-- Need ON

csi-RS-ConfigNZPToAddModList-r11

CSI-RS-ConfigNZPToAddModList-r11	OPTIONAL,		-- Need ON

csi-RS-ConfigZPToReleaseList-r11

CSI-RS-ConfigZPToReleaseList-r11	OPTIONAL,		-- Need ON

csi-RS-ConfigZPToAddModList-r11

CSI-RS-ConfigZPToAddModList-r11	OPTIONAL,		-- Need ON

epdcch-Config-r11					EPDCCH-Config-r11			OPTIONAL,		-- Need ON

pdsch-ConfigDedicated-v1130			PDSCH-ConfigDedicated-v1130	OPTIONAL,		-- Need ON

-- UL configuration

cqi-ReportConfig-v1130				CQI-ReportConfig-v1130		OPTIONAL,		-- Need ON

pusch-ConfigDedicated-v1130

PUSCH-ConfigDedicated-v1130		OPTIONAL,	-- Cond PUSCH-SCell1

uplinkPowerControlDedicatedSCell-v1130

UplinkPowerControlDedicated-v1130	OPTIONAL		-- Need ON

]],

[[	antennaInfo-v1250					AntennaInfoDedicated-v1250	OPTIONAL,		-- Need ON

eimta-MainConfigSCell-r12

EIMTA-MainConfigServCell-r12	OPTIONAL,		-- Need ON

cqi-ReportConfigSCell-v1250			CQI-ReportConfig-v1250		OPTIONAL,		-- Need ON

uplinkPowerControlDedicatedSCell-v1250

UplinkPowerControlDedicated-v1250	OPTIONAL,		-- Need ON

csi-RS-Config-v1250					CSI-RS-Config-v1250			OPTIONAL		-- Need ON

]],

[[	pdsch-ConfigDedicated-v1280			PDSCH-ConfigDedicated-v1280	OPTIONAL		-- Need ON

]],

[[	pucch-Cell-r13						ENUMERATED {true}		OPTIONAL,	-- Cond PUCCH-SCell1

pucch-SCell							CHOICE{

release								NULL,

setup								SEQUENCE {

pucch-ConfigDedicated-r13

PUCCH-ConfigDedicated-r13	OPTIONAL,		-- Need ON

schedulingRequestConfig-r13

SchedulingRequestConfigSCell-r13	OPTIONAL,		-- Need ON

tpc-PDCCH-ConfigPUCCH-SCell-r13

TPC-PDCCH-ConfigSCell-r13	OPTIONAL,		-- Need ON

pusch-ConfigDedicated-r13

PUSCH-ConfigDedicated-r13	OPTIONAL,	-- Cond PUSCH-SCell

uplinkPowerControlDedicated-r13

UplinkPowerControlDedicatedSCell-v1310	OPTIONAL	-- Need ON

}

}																OPTIONAL,	-- Need ON

crossCarrierSchedulingConfig-r13

CrossCarrierSchedulingConfig-r13	 OPTIONAL,	-- Cond Cross-Carrier-Config

pdcch-ConfigSCell-r13				PDCCH-ConfigSCell-r13		OPTIONAL,		-- Need ON

cqi-ReportConfig-v1310				CQI-ReportConfig-v1310		OPTIONAL,		-- Need ON

pdsch-ConfigDedicated-v1310			PDSCH-ConfigDedicated-v1310	OPTIONAL,		-- Need ON

soundingRS-UL-ConfigDedicated-v1310

SoundingRS-UL-ConfigDedicated-v1310		OPTIONAL,		-- Need ON

soundingRS-UL-ConfigDedicatedUpPTsExt-r13

SoundingRS-UL-ConfigDedicatedUpPTsExt-r13	OPTIONAL,		-- Need ON

soundingRS-UL-ConfigDedicatedAperiodic-v1310

SoundingRS-UL-ConfigDedicatedAperiodic-v1310	OPTIONAL,		-- Need ON

soundingRS-UL-ConfigDedicatedAperiodicUpPTsExt-r13

SoundingRS-UL-ConfigDedicatedAperiodicUpPTsExt-r13	OPTIONAL,		-- Need ON

csi-RS-Config-v1310					CSI-RS-Config-v1310			OPTIONAL,		-- Need ON

laa-SCellConfiguration-r13			LAA-SCellConfiguration-r13	OPTIONAL,		-- Need ON

csi-RS-ConfigNZPToAddModListExt-r13	CSI-RS-ConfigNZPToAddModListExt-r13	OPTIONAL,	-- Need ON

csi-RS-ConfigNZPToReleaseListExt-r13	CSI-RS-ConfigNZPToReleaseListExt-r13	OPTIONAL	-- Need ON

]],

[[	cqi-ReportConfig-v1320				CQI-ReportConfig-v1320	OPTIONAL		-- Need ON

]],

[[	laa-SCellConfiguration-v1430		LAA-SCellConfiguration-v1430

OPTIONAL,	-- Need ON

typeB-SRS-TPC-PDCCH-Config-r14 		SRS-TPC-PDCCH-Config-r14	OPTIONAL,	-- Need ON

uplinkPUSCH-LessPowerControlDedicated-v1430 		UplinkPUSCH-LessPowerControlDedicated-v1430 OPTIONAL,		-- Need ON

soundingRS-UL-PeriodicConfigDedicatedList-r14					SEQUENCE (SIZE (1..2)) OF SoundingRS-UL-ConfigDedicated						 OPTIONAL,		-- Cond PeriodicSRS

soundingRS-UL-PeriodicConfigDedicatedUpPTsExtList-r14					SEQUENCE (SIZE (1..4)) OF SoundingRS-UL-ConfigDedicatedUpPTsExt-r13						 OPTIONAL,		-- Cond PeriodicSRSExt

soundingRS-UL-AperiodicConfigDedicatedList-r14					SEQUENCE (SIZE (1..2)) OF SoundingRS-AperiodicSet-r14						 OPTIONAL,		-- Cond AperiodicSRS

soundingRS-UL-ConfigDedicatedApUpPTsExtList-r14					SEQUENCE (SIZE (1..4)) OF SoundingRS-AperiodicSetUpPTsExt-r14   			 OPTIONAL,		-- Cond AperiodicSRSExt

must-Config-r14 						CHOICE{

release								NULL,

setup								SEQUENCE {

k-max-r14						ENUMERATED {l1, l3},

p-a-must-r14					ENUMERATED {

dB-6, dB-4dot77, dB-3, dB-1dot77,

dB0, dB1, dB2, dB3}	OPTIONAL		-- Need ON

}

}															OPTIONAL,		-- Need ON

pusch-ConfigDedicated-v1430 			PUSCH-ConfigDedicatedSCell-v1430	OPTIONAL,	-- Need ON

csi-RS-Config-v1430						CSI-RS-Config-v1430			OPTIONAL,	-- Need ON

csi-RS-ConfigZP-ApList-r14				CSI-RS-ConfigZP-ApList-r14		OPTIONAL,	-- Need ON

cqi-ReportConfig-v1430					CQI-ReportConfig-v1430	OPTIONAL,	-- Need ON

semiOpenLoop-r14						BOOLEAN						OPTIONAL,	-- Need ON

pdsch-ConfigDedicatedSCell-v1430		PDSCH-ConfigDedicatedSCell-v1430		OPTIONAL		-- Need ON

]]

}