3GPP TSG-RAN WG2 #99	Tdoc R2-1708339

Berlin, Germany, 21st – 25th August 2017

Agenda Item:	10.3.2.3

Source:	Ericsson

Title:	RLC Status Report Format

Document for:	Discussion, Decision

# Introduction

RAN2-AH in June agreed:

1.	The NR RLC STATUS PDU retains the D/C, CPT, ACK_SN, NACK_SN, SOstart, and SOend fields with the similar inerpretation as in the LTE RLC STATUS PDU.

2.	The NACK range field can be associated with a pair SOstart and SOend fields to indicate the segment offsets for the “first” and “last” RLC SDU segments in a set of consecutively lost RLC PDUs.

RAN2#98 Hangzhou agreed:

1.	NACK range field indicating the number of consequitively lost RLC SNs starting from and including NACK_SN.

2.	FFS the number of bits required

3.	E3 indicates the presence of NACK range

4.	Several NACK range fields can be included in the RLC Status PDU

5. 	The rest of the fields can follow LTE baseline

Status report is sent by RLC AM to provide positive or negative acknowledgement of the PDUs. It consists of control PDU header and Status PDU payload.  Stage-3 aspects of the RLC status report format are discussed in this contribution.

# Discussion

## Behaviour of Extension bits

RAN2-98 agreed that RLC status report includes three extension bits, or 3 bit NACK tuples. Purpose of the E-bits is to provide indication to the status report parser that which type of NACK field will follow. Figure 1 presents example of status report format with each field type. Setting E1-bit to ‘1’ indicates that NACK SN will follow. Additionally, when E2 and E3 are used. E2-bit indicates that after NACK_SN, SO_START and SO_END will follow. Finally, the E3-bit indicates that after NACK_SN, NACK_SN_RANGE will follow. This approach allows low complexity implementation as 1-bit is corresponding to a field in status report. Section 3 provides Text Proposal, which describes detailed behavior of fields in status report.

Figure 1: Example Status PDU format for 18-bit (1) and 12-bit SNs (2) with 10-bit NACK_SN_RANGE (1.3 & 2.4).

Status report format in figure 1 is adopted for 12-bit and 18-bit.

## NACK_SN_RANGE size

The NACK_SN_RANGE indicates the number of consecutive SNs lost, starting from SN indicated in the header. With 10-bit SN range, it is possible to indicate 1024 missing SNs, which should be sufficient number as the receiver window size is configured to be half of the SN space. Additionally, RAN2 agreed that multiple NACK_SN_RANGEs can be placed at the same status report. Consequently, status report is able to indicate several sequentially missing SN range gaps. Text proposal in section 3 specifies NACK_SN_RANGE behaviour.

10-bit NACK_SN_RANGE is used in status report format

## Indicating multiple missing segments

All segments, which are associated with the same SN will have same sequence number. In order to indicate multiple missing segments with status report format, multiple SO fields can be set under the same SO field.  This does not conflict with the description of the E2 and E3 bit because the NACK_SN_RANGE field may indicate the number of missing segments that will follow. This is low-complexity solution that removes overhead from the status report format. When NACK_SN_RANGE indicates the count, there is no need to create new parser implementation for the E2=1 and E3=1 field.

Figure 2: NACK_SN_RANGE is indicating number of missing segments.

Status report format supports a format where multiple missing SDU segments having the same SN are indicated.

NACK_SN_RANGE indicates the number of SO_START and SO_END pairs that follow the NACK_SN_RANGE like in figure 2.

# Conclusion

Based on the discussion in section 2 we propose the following:

Proposal 1	Status report format in figure 1 is adopted for 12-bit and 18-bit.

Proposal 2	10-bit NACK_SN_RANGE is used in status report format

Proposal 3	Status report format supports a format where multiple missing SDU segments having the same SN are indicated.

Proposal 4	NACK_SN_RANGE indicates the number of SO_START and SO_END pairs that follow the NACK_SN_RANGE like in figure 2.

# References