3GPP TSG-RAN WG2 Meeting #99		R2-1708417

Berlin, Germany, 21st – 25th August 2017

Source:	vivo

Title:	RLM / RLF in NR

Agenda Item:	10.2.9

Document for:	Discussion and Decision

- Introduction

Part I: Background for RLM/RLF considering beam failure recovery

In RAN2#97bis and #2 Adhoc meeting, RLM/RLF in NR has been widely discussed. It was agreed that:

Agreements from #97bis:

1:	For connected mode, UE declares RLF upon timer expiry due to DL OOS detection, random access procedure failure detection, and RLC failure detection.

FFS whether maximum ARQ retransmission is only criteria for RLC failure (needs to be discussed in common UP/CP session).

2	In NR RLM procedure, physical layer performs out of sync / in sync indication and RRC declares RLF.

3	For RLF purposes, RAN2 preference is that the in sync / out of sync indication should be a per cell indication, and we aim for a single procedure for both multi-beam and single beam operation.

Agreements from #98:

1:	RAN2 understand that beam failure recovery (L1 or MAC) and RLF (RRC) are performed in different layers.

Accordingly, in RAN1 #2ad hoc meeting, the following agreements were made:

Agreements: - The RS used for RLM should have following properties - Periodic transmission with short enough periodicity - Wideband transmission relative to bandwidth of active bandwidth part - Supporting both single beam and multi-beam operations - Representing control channel quality - Both CSI-RS based RLM and SS block based RLM are supported - FFS: whether or not only a single type of RS is configured to UE for RLM at a time Agreements: - NR should strive to provide aperiodic indication(s) based on beam failure recovery procedure to assist radio link failure (RLF) procedure, if same RS is used for beam failure recovery and RLM procedures. - Example 1: aperiodic indication(s) based on beam failure recovery procedure can reset/stop T310 - RAN2 can decide specific procedure - Example 2: aperiodic indication(s) based on failure of beam recovery procedure - How to use aperiodic indication can be decided in RAN2 - FFS: aperiodic indication(s) based on beam failure recovery procedure to assist RLF procedure if different RS is used

Thus, in Part I, we will discuss the interaction between RLM/RLF and beam failure recovery from RAN2 point of view.

Part II: Background for RLM/RLF considering change of CSI-RS and SS block

In RAN1 NR #2Adhoc meeting, RLM topic was discussed and made the following agreement [1]:

Agreements on radio link monitoring at RAN1#NR-AH2 meeting: Agreements: - The RS used for RLM should have following properties - Periodic transmission with short enough periodicity - Wideband transmission relative to bandwidth of active bandwidth part - Supporting both single beam and multi-beam operations - Representing control channel quality - Both CSI-RS based RLM and SS block based RLM are supported - FFS: whether or not only a single type of RS is configured to UE for RLM at a time

The new agreements will also impact on RLM procedure. Thus, in Part II, we will discuss how the RLM procedure perform if both CSI-RS based RLM and SS block based RLM are supported.

- Discussion

  - RLM/RLF considering Beam Failure Recovery

In legacy LTE, current RLM/RLF is a L3 procedure. It is controlled by physical layer problem indication out-of-sync and recovery of problem in-sync. If a problem in physical layer is detected, e.g. SINR of PDCCH is lower than a threshold, an out-of sync is indicated to high layer. After N310 consecutive out-of-sync are indicated, a timer T310 is triggered. If the problem can be solved in physical layer, an in-sync indication will be indicated to high layer. During T310 running, if N311 consecutive in-sync are indicated, the timer should be stopped. Otherwise, RLF is declared after T310 expires.

In RAN2 #97bis meeting, it was agreed that for connected mode, UE declares RLF upon timer expiry due to DL OOS detection, random access procedure failure detection, and RLC failure detection. It means that the legacy RLM/RLF procedure should be reused in NR.

Proposal 1: In NR, T310 for RLM is started if receiving N310 consecutive out-of-sync. During T310 running, if N311 consecutive in-sync are indicated, the timer should be stopped. Otherwise, RLF is declared after T310 expires.

After RAN1 discussion, beam failure recovery procedure has been introduced, which also focuses on the purpose to solve the physical layer problem. In case of channel blockage, almost or all DL beams can’t work, beam failure recovery will be triggered. During this procedure, trigger condition, recovery request, and recovery response are totally L1/MAC behaviors. It has no impact on RRC layer.

Either beam failure recovery in physical layer or RLM/RLF in RRC layer are used to detect the physical layer problem. However, the former one is trying to detect the channel blockage, which reflects the signaling strength. While the later one is trying to monitor the signaling quality. Thus, they may have different purposes and application scenarios. Thus, there is no need to inform or controlled by RRC layer for beam failure recovery procedure.

Proposal 2: Beam failure recovery procedure, including detection, requesting, and response, has no impact to RLM/RLF procedure in RRC layer.

In RAN1 agreement, it is stated two examples:

-  Example 1: aperiodic indication(s) based on beam failure recovery procedure can reset/stop T310

In our understanding, aperiodic indication based on beam failure recovery procedure in this example may be one or more of problem detection, beam failure recovery triggering, recovery requesting, recovery response, recovery success.

-  Example 2: aperiodic indication(s) based on failure of beam recovery procedure

In our understanding, aperiodic indication based on beam failure recovery procedure in this example is beam failure recovery failure.

In example 1, based on the above analysis, beam failure recovery procedure (including detection, requesting, and response) has no impact to RLM/RLF procedure in RRC layer. For recovery success, it means that physical layer problem causing beam failure is solved. If T310 is running at that time, it means the link quality problem is detected. Whether this problem is also solved by beam failure recovery:

- Case 1: if the link quality problem is caused by channel blockage, it is also solved by beam failure recovery. In this case, in-sync of regular RLM procedure should be also indicated based on regular RLM procedure. Thus, T310 will be naturally stopped by in-sync indication.

- Case 2: if the link quality problem is caused by interference, it is not also solved by beam failure recovery. In this case, it is reasonable to keep T310 running.

Thus, based on the above analysis, it is not reasonable to stop/reset T310 if beam failure recovery succeeds.

Proposal 3: RLM/RLF procedure should not be impacted by success indication for beam failure recovery procedure.

In example 2, if no response is detected after a certain number of requesting, it means that there is no available DL beam to recover the failure link. It means that there is no available DL beam to recovery the failure link. Beam failure recovery is considered as failure, which will be indicated to high layer.

In this case, although regular out-of-sync will also indicate such bad link quality or T310 will continue, there is no need for the UE to wait until T310 expires. Beam failure recovery failure means there is no proper link from physical layer point of view. There is no opportunity to receive in-sync indication. If the configuration of RLM is not proper, the UE has to wait for a very long time. It is a waste of UE power consumption and will cause long latency. Thus, an early RLF should be declared based on the indication of beam failure recovery failure, instead of waiting for T310 expiration.

Proposal 4: RLF is declared based on the indication of beam failure recovery failure from lower layer.

If RLF is declared by other causes in high layer, the following RRC re-establishment will be performed based on the current mechanism anyway. Any link recovery procedure in lower layer is not needed, including L1/MAC beam failure recovery procedure. Thus, it is reasonable to indicate RLF to L1/MAC layer, so that PHY layer will stop the beam failure recovery procedure and related operations.

Proposal 5: Once RLF is declared in high layer, it should be indicated to L1/MAC layer.

  - RLM/RLF considering change of CSI-RS and SS block

In RAN1, it was agreed that both CSI-RS based RLM and SS block based RLM were supported. Whether or not only a single type of RS is configured to UE for RLM at a time is still FFS. Currently, the reference signaling of RLM can be configured. Thus, the reference signaling of RLM can be changed in below two cases.

Case 1: Reconfigure the reference signaling of RLM

The network may reconfigure the reference signaling of RLM due to beam change, e.g. change CSI-RS1 to CSI-RS2 or change CSI-RS1 to SS block1.

Case 2: The reference signaling of RLM is deactivated

Based on RAN1 agreement, CSI-RS transmission can be deactivated in NR. CSI-RS and SS block transmission can be also deactivated because the bandwidth part (BWP) of wide band cell is deactivated. After the old CSI-RS or SS block transmission of RLM is deactivated, the reference signaling of RLM has to be changed to the new one.

- Wideband transmission relative to bandwidth of active bandwidth part

When the reference signaling of RLM is changed in case that the counter of “in sync” and “out of sync” or T310 has been running, we need to discuss whether the counter of “in sync” / “out of sync” or T310 should be reset or continued.

In our understanding, different reference signaling for RLM is related to different beams, while different beams may have huge different radio link condition. Thus, it is better to reset the counter of “in sync” and “out of sync” or T310 if the reference signaling for RLM is changed.

Proposal 6: The counter of “in sync” and “out of sync” or T310 which has been running are reset if the reference signaling for RLM is changed.

- Conclusion

In this contribution, we discuss the RLM/RLF in NR by considering the beam failure recovery and changing of SS block / CSI-RS. Based on the discussion, we have the following proposals:

Proposal 1: In NR, T310 for RLM is started if receiving N310 consecutive out-of-sync. During T310 running, if N311 consecutive in-sync are indicated, the timer should be stopped. Otherwise, RLF is declared after T310 expires.

Proposal 2: Beam failure recovery procedure, including detection, requesting, and response, has no impact to RLM/RLF procedure in RRC layer.

Proposal 3: RLM/RLF procedure should not be impacted by success indication for beam failure recovery procedure.

Proposal 4: RLF is declared based on the indication of beam failure recovery failure from lower layer. Proposal 5: Once RLF is declared in high layer, it should be indicated to L1/MAC layer.

Proposal 6: The counter of “in sync” and “out of sync” or T310 which has been running are reset if the reference signaling for RLM is changed.

- References

- R2-1703796	 Report from LTE Break-Out Session, Vice-Chair (InterDigital)