3GPP TSG-RAN WG2 Meeting#99		R2-1708427

Berlin, Germany, 21st – 25th August 2017

Source:	vivo

Title:	Considerations on RRC SI Request/Response Messages

Agenda Item:	10.4.1.5.6

Document for:	Discussion and Decision

- Introduction

In the RAN2#96 meeting, it was agreed that for UEs in connected, dedicated RRC signalling can be used for the request and delivery of other SI. In the last RAN2 Qingdao adhoc meeting, it was agreed that RRC signalling is used for SI request in Msg3, and the RRC signalling how to indicate the requested SI/SIB details left to ASN.1 work.

In this contribution, we discuss the design of RRC SI request/response messages in details.

- Discussion

- RRC message class

In LTE, each RRC message is mapped to a single message class according to type of the logical channel which the message is sent on. In the agreed guideline for 38.331[1], the logical channel type based message class is reused. Obviously idle and inactive UEs can only send the MSG3 based RRC SI Request on UL CCCH, while connected UEs shall send RRC SI Request on UL DCCH. Therefore, we propose :

Proposal1 : two RRC SI request messages need to be defined : MSG3 based SI request(RRCSiRequestCCCH for short) is transfered via UL CCCH,  SI request for connected UE(RRCSiRequestDCCH for short) is transfered via UL DCCH.

- Minimum granularity of the requested SI

It has been agreed that the SI requested via RRCSiRequestCCCH are always broadcasted. Since one SI-Message is the smallest transmission entity in the NR SI broadcast mechanism, it is natural to take one SI-Message as the minimum granularity of RRCSiRequestCCCH.

Thing is different for the SI requested via RRCSiRequestDCCH, for the requested SI is unicast delivered to UE. If one UE is only interested in partial SIBs in one SI message, sending only the SIBs interested instead of the whole SI message to the UE is more resource-efficient. Hence, one SIB can be taken as the minimum granularity of RRCSiRequestDCCH.

Proposal2: for RRCSiRequestCCCH, the minimum granularity of requested SI is one SI message; while for RRCSiRequestDCCH, the minimum granularity of requested SI is one SIB.

Proposal3: in the RRC SI delivering message responding to RRCSiRequestDCCH, only the SIBs requested are included.

- Content of SI request

In the previous email discussion [2], the content of RRCSiRequestCCCH was discussed. And most companies prefer to apply bitmap, for bitmap leads to small size of SI request.  In our understanding, bitmap is also applicable for RRCSiRequestDCCH.

Proposal4: bitmap is used to indicate the requested SI messages/SIBs in both RRCSiRequestCCCH and RRCSiRequestDCCH.

- Combination of RRCConnectionRequest and SI request

It has been proposed that MSG3 based SI request can be carried by RRCConnectionRequest message, for UEs transitioning from RRC Idle to Connected state [3]. We do see some benefits of this proposal in shortening the latency of SI acquisition. If not exceeds allowed size for MSG3, we propose to support it. To carry it out, the bitmap indicating the requested SI can be included in the RRCConnectionRequest, as an optional IE

Proposal5: support the combination of RRCConnectionRequest and SI request for UEs transitioning from RRC Idle to Connected state, if not exceeds the allowed size for MSG3. The bitmap indicating the requested SI can be included in the RRCConnectionRequest, as an optional IE.

- Conclusion

In this contribution, we discussed the design of RRC SI request/response messages. Base on the discussion, we have the following proposals:

Proposal1 : two RRC SI request messages need to be defined : MSG3 based SI request(RRCSiRequestCCCH for short) is transfered via UL CCCH,  SI request for connected UE(RRCSiRequestDCCH for short) is transfered via UL DCCH.

Proposal2: for RRCSiRequestCCCH, the minimum granularity of requested SI is one SI message; while for RRCSiRequestDCCH, the minimum granularity of requested SI is one SIB.

Proposal3: in the RRC SI delivering message responding to RRCSiRequestDCCH, only the SIBs requested are included.

Proposal4: bitmap is used to indicate the requested SI messages/SIBs in both RRCSiRequestCCCH and RRCSiRequestDCCH.

Proposal5: support the combination of RRCConnectionRequest and SI request for UEs transitioning from RRC Idle to Connected state, if not exceeds the allowed size for MSG3. The bitmap indicating the requested SI can be included in the RRCConnectionRequest, as an optional IE.

- References

- R2-1707470	 Text proposal on TS 38.331 guidelines	Ericsson, Nokia, Alcatel-Lucent Shanghai Bell

- R2-1707090	 Summary of [98#34][NR] On demand SI (Lenovo)	Lenovo, Motorola Mobility

- R2-1705511 SI Request and Delivery using Msg 3 Approach Nokia, Alcatel-Lucent Shanghai Bell