3GPP TSG-RAN WG2 Meeting #99	R2-1708447

Berlin, Germany, 21 – 25 Aug 2017

Agenda item:	9.13.1

Source: 	VEOLIA

Title: 	UL small data transmission use cases and requirements

Document for:     Discussion/Decision

# Introduction

The WI description for Further NB-IoT enhancements is given in [1]. One of the objectives of the WI is:

Further latency and power consumption reduction: - Evaluate power consumption/latency gain and specify necessary support for DL/UL data transmission on a dedicated resource during the Random Access procedure after NPRACH transmission and before the RRC connection setup is completed. [RAN2, RAN1, RAN3]

In previous RAN2 meetings, it has been discussed that early data transmission may be beneficial for small data transmission; however, it was not clear what amount of data should be considered “small”.

In this contribution, we discuss some practical use cases and more concrete values for the payload of those use cases. We believe this information will be useful to design support for UL data transmission during the RA procedure for UEs in IDLE mode.

# Water metering use case

Metering and sensor data has been frequently described as potential uses cases involving relatively infrequent exchange of small amount of data.

## Transmission periodicity

In commercial water meters currently being used, the meters transmit a packet of payload twice every day at about 12 hours interval. Depending on Service Level Agreement (SLA), some customers may require a more frequent data transmission scheme.

Remarks:

- Upon some events detection (Leaks, Tampering, …) alarms messages are transmitted. These messages could be repeated up to 4 times with a few minutes interval. If a network acknowledgement of UL TX is received (see ‎‎‎2.3), these repetitions could be avoided.

The payload transmission by water meters is typically every 12 hours.

## Payload size

The payload is about 12 Bytes when SIGFOX® [Annex ‎1] is used. For LoRaWAN® [Annex ‎2] based transmission, the size of packets is about 50 Bytes.  This is inherent to the technology used.

We would also note that it is possible to adapt the payload size based on technology being used. The bigger packet sizes can accommodate more redundancy for error detection/correction and packet loss recovery.

The optimal application payload size for smart water metering use case, corresponding to most of customer’s needs and SLA, is around 100 bytes. These 100 bytes does not include meta data (ID, …) nor checksums. Proprietary technologies (based on LoRa) can achieved this optimal payload.

Remarks:

- Wireless MBUS standard is the reference protocol for the metering industry. Application payload using this protocol is up to 256 Bytes.

- In other water use cases (Water networks leaks geolocation, Water Quality monitoring, …) an application payload up to 3000 Bytes should be transmitted. This kind of solution could afford a high battery capacity and/or lower battery life time and hence do not require an Early Data Transmission Scheme.

- When using high bit rates, a LoRaWAN® application payload of up to 242 Bytes could be transmitted in one single radio frame [Annex ‎2]. At any moment, the sensor’s application layer knows the bit rate assigned by the network and therefore adapt its payload size accordingly.

The payload size for UL transmission by water meters is in the range of 12 to 100 Bytes for regular transmission.

The optimal payload size for UL transmission by water meters is 100 Bytes for regular transmission.

## ACK for UL data

In current deployments, application level ACK is not expected for the UL transmissions from the water meters. ACK using SIGFOX® is abandoned because of its high energy consumption during downlink procedure (25 seconds reception slot) [Annex ‎3]. Using LoRaWAN®, a “confirmed frame” mode is used only when the network does not answer MAC layer ACK requests (ADRAckReq) [Annex 4].

So, the current meters operate, mainly, as “fire-and-forget” transmissions. It should be noted that missing a few packets is not critical for operation of these meters (as far as these missing packets does not prevent the solution from fulfilling SLA commitments such as monthly billing), and human intervention is expected if multiple of the packets are missing (as observed from the network side), however it should be limited as its cost is prohibitive.

It is understood that one of the prime objectives/concern for the actual meters is their battery life. Possibility of having a single battery last for 10-15 years is crucial to commercial success. Given that application level ACK involves further downlink communications after an unpredictable delay (due to unpredictable backhaul and Internet delay) results in increased power consumption, it is preferable to avoid such downlink transmissions.

It is preferred to avoid application level ACKs for the UL packets as they result in more DL communication, and hence more power consumption.

We also note that radio level “quick” ACK may be beneficial (but is not mandatory), as this can reduce in number of human interventions required. This ACK could be provided by the eNB for more efficiency.

An ACK provided by the eNB for the UL transmission would be beneficial.

# Concern with using existing LTE solutions

As described in various contributions from different companies in previous meetings, our understanding is that, full RRC Connection establishment procedure for Idle UEs involving RA procedure has a large overhead involving 5 or more messages for UL data before actual small payload transmission. This is a large overhead for applications that transmit uplink data that fits into one TBS. Even for the case of RRC suspend/resume (i.e. UP CIoT EPS Optimization), there is substantial overhead for applications that require very small amount of data to be transferred (e.g. 1 MAC block). Furthermore, there is also considerable overhead to release RRC connection.

Existing LTE procedures entail significant overhead to setup/resume and release RRC connection for transferring a small payload.

Therefore, we encourage RAN2 to discuss and specify early data transmission solution which has lower overhead and power consumption taking into account the actual payload sizes and requirements described above.

Discuss and specify early data transmission solution which has lower overhead and power consumption considering the actual payload sizes and requirements described above.

# Conclusion

In this contribution, we discussed some practical payload sizes and requirements for water meter use case. Based on the above discussion, we made the following observations:

Observation 1.	The payload transmission by water meters is typically every 12 hours.

Observation 2.	The payload size for UL transmission by water meters is in the range of 12 to 100 Bytes for regular transmission.

Observation 3.	The optimal payload size for UL transmission by water meters is 100 Bytes for regular transmission.

Observation 4.	It is preferred to avoid application level ACKs for the UL packets as they result in more DL communication, and hence more power consumption.

Observation 5.	An ACK provided by the eNB for the UL transmission would be beneficial.

Observation 6.	Existing LTE procedures entail significant overhead to setup/resume and release RRC connection for transferring a small payload.

Based on the discussion and observations, we propose:

Proposal 1.	Discuss and specify early data transmission solution which has lower overhead and power consumption considering the actual payload sizes and requirements described above.

# References

- RP-171428, New WID on Further NB-IoT enhancements, Huawei, Hi-Silicon

# Annexes

[Annex 1]: SIGFOX® System Description, https://tools.ietf.org/html/draft-zuniga-lpwan-sigfox-system-description-03

[Annex 2]: LoRaWAN® Overview, https://tools.ietf.org/html/draft-farrell-lpwan-lora-overview-01

[Annex 3]: SIGFOX® Downlink procedure, https://ask.sigfox.com/questions/1049/how-to-minimise-battery-drain-during-downlinks.html

[Annex 4]: LoRaWAN® confirmed frames mode as described in “LoRaWAN™ 101 – A Technical Introduction”, https://www.lora-alliance.org/technology