3GPP TSG-RAN WG2 #99	R2-1709252

Berlin, Germany, 21 - 25 August 2017

Agenda item:	8.13.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Operating Bands for V2X Services via Uu

WID/SID:	LTE_V2X-Core - Release 14

Document for:	Discussion and Decision

# 1	Discussion

In TS 36.101, section 5.5G, table 5.5G-1 [1], designated operating bands for V2X Communication have been defined, both for reference point Uu and PC5.

Table 5.5G-1 V2X operating band

| E-UTRA Operating Band | E-UTRA V2X Operating Band | V2X UE transmit |  |  | V2X UE receive |  |  | Duplex Mode | Interface |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |  |
| 47 | 47 | 5855 MHz |  | 5925 MHz | 5855 MHz |  | 5925 MHz | TDD | PC5 |
| 3 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD | Uu |
| 7 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD | Uu |
| 8 | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD | Uu |
| 39 | 39 | 1880 MHz | – | 1920 MHz | 1880 MHz | – | 1920 MHz | TDD | Uu |
| 41 | 41 | 2496 MHz |  | 2690 MHz | 2496 MHz |  | 2690 MHz | TDD | Uu |

From our perspective, this table is unnecessary. Messages from V2X Services are transparently sent over Uu, similar to other end-to-end services (ftp, http…). The use of the Uu reference point for V2X Services does not impact RF characteristics and minimum performance requirements for E-UTRA User Equipment (UE), i.e. there are no band specific requirements. There is therefore no technical reason why V2X Communication via Uu cannot be offered via all E-UTRA operating bands listed in table 5.5-1 of TS 36.101.
We were wondering whether table 5.5G shown above would imply for a network implementation, that V2X messages exchanged between an application running on a UE and a V2X Control Function/V2X Application Server in the EPC (reference points V3/V5) are restricted to operating bands 3, 7, 8, 39, and 41, and whether this restriction is intended and desirable.

We kindly ask RAN2 to confirm that from RAN2 point of view:

- V2X Communication via Uu can and shall be supported by every E-UTRA operating band; This also means that any RRC signaling with parameters for V2X communication can be provided via all E-UTRA operating bands.

- If the first bullet point has been confirmed, we kindly ask RAN2 to send an LS to RAN4 informing them that V2X Communication via Uu is supported by every LTE operating band from RAN2 point of view and asking whether there are any limitations on the supported bands implied by RAN4 requirements.

# 3	Summary

In this paper, we shortly discussed the applicable bands for V2X services running over Uu bands. Based on the provided argumentation we kindly request RAN2 to agree on the following:

Proposal 1: RAN2 confirms that there are no limitations on Uu bands applicable to V2X from RAN2 perspective.

Proposal 2: Send an LS to RAN4 informing this point of view and asking whether there is any RAN4 requirement, which implies limitation of Uu bands applicable to V2X.

# References

- 3GPP TS 36.101, Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception