3GPP TSG-RAN WG2 Meeting #99	Tdoc R2-1709533

Berlin, Germany, 21-25 August 2017

Agenda Item:	6

Source:	Ericsson

Title:	Secured redirect to GERAN

Document for:	Discussion, Decision

# Introduction

In this document, we discuss the issue on redirect to GERAN, and present our view on how to proceed.

# Discussion

Here is a summary of the RAN2 discussions on the redirection to GERAN issue.

- RAN2 first discussed problems with unsecured redirect to GERAN at RAN2#95bis [1], but the discussion was postponed to the next meeting.

- At RAN2#96, RAN2 agreed to attempt to address the problem, and an LS [4] was sent to SA3 to inform them of this.

- At RAN2#97, RAN2 sent LS to CT1 asking if a solution with NAS signalling to indicate whether the UE is allowed to accept unsecured redirection could be technically feasible, and at the same time asked SA3 for feedback.

- There was no discussion on the issue at RAN2#97bis.

- At RAN2#98, RAN2 received reply LS from CT1 [5], indicating that such solution is technically feasible, but RAN2 identified that the default UE behaviour assumed by CT1 (reject unsecured redirections) would lead to problems with existing networks using unsecured redirections, and asked CT1 for clarification [7].

- In SA3, two proposals on alternatives to AS protected redirect has been presented [8, 9], based on different flavours of a new “NAS-protected token” in the rrcConnectionRelease message. No conclusion is yet made by SA3.

This is our understanding of the current situation:

- Reason to study alternatives to AS-protected redirection to GERAN was claimed to be the additional delay caused by activating AS security before redirect, in particular when used for CS fallback [3]. But no delay analysis was provided.

- In TS36.331, the RRC procedure delay for initial security activation is 10ms. Even a delay of 5 or 10 times 10ms would not result in any noticeable additional delay for CS fallback and voice call establishment in GERAN. Given this, performance concern does not motivate introduction of alternative protection mechanisms.

- Alternative solution based on “NAS-protected token” impacts at least RRC and S1AP, requires standardization work in RAN2, RAN3, CT1, SA3 and SA2, and impacts UE, eNB and MME.

- AS security is already in place.

- It is technically feasible to configure the UE via NAS signalling to accept or ignore unprotected redirect message, and impacts UE and MME.

Based on the above, we recommend RAN2 to agree on the following:

The security issue with unprotected redirect to GERAN is solved by

    - activating AS security prior to redirect, and

    - configure UE to not accept unprotected redirect messages via NAS signalling.

# Conclusion

In this document, we have presented our view on how to solve the security issue with redirect to GERAN.

We ask RAN2 to agree on the following:

- The security issue with unprotected redirect to GERAN is solved by

    - activating AS security prior to redirect, and

    - configure UE to not accept unprotected redirect messages via NAS signalling.

# References

R2-166154,	CR to correct RRC Connection Release with GERAN redirection to be sent only after security activation,	Apple Europe Limited

R2-168757,	Discussion on redirection to GERAN, Source CMCC

R2-168356, LTE call redirection to GERAN, Source Nokia, Alcatel-Lucent Shanghai Bell

R2-169124, LS to SA3 on LTE call redirection to GERAN

R2-1704006, Reply LS on LTE call redirection to GERAN, Source CT1

R2-1704195, Handling of GERAN redirection without AS security, Source Nokia, Alcatel-Lucent Shanghai Bell

R2-1706149, Response LS on Handling of GERAN redirection without AS security

S3-171408, Discussion on R2-1702388 LS to SA3 on LTE Call redirection to GERAN, Source Nokia

S3-171245, Enhanced NAS token solution for LTE redirection attack, Source Huawei, Hisilicon,