3GPP TSG-RARN WG2 Meeting #99			Tdoc R2-1709788

Berlin, Germany, 21st – 25th August 2017

Title:	[DRAFT] LS on RACH agreements

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	Ericsson (RAN2)

To:	RAN1

Contact Person:

#### Name:	Mats Folke

Tel. Number:	+46761271385

E-mail Address:	mats.folke@ericsson.com

1. Overall Description:

RAN2 has discussed Random Access procedure and reached the following agreements:

Agreements:

For multiple msg1 transmissions for contention free RACH

-	A single RAR window is applied for multiple msg1 transmission.

-	The RAR window is started after transmission of the first preamble after a “offset”.

-	The UE monitors multiple RA-RNTIs.  The RA-RNTI is associated to the RACH transmission occasion in which the preamble was transmitted.

-	Once a RAR is received, the RAR reception is considered successful, as in LTE.  The UE stops multiple preamble transmission.

-	Details of RA-RNTI calculations are FFS

Agreements:

-     The following fields are always included in the RAR random access procedures:  RAPID, UL grant, TA, Temporary C-RNTI for all cases other than msg. 1 SI request.

-	BI can be included in msg. 2 as in LTE. UE behaviour with respect to backoff is the same as in LTE

In order to finalize the design of the RAR, RAN2 has the following questions for RAN1:

Q1: What is the size of the RAPID field (Random Access Preamble Identifier)?

Q2: What is the size of the Uplink grant?

Q3: What is the size of the Timing advance command (field TA above)?

Q4: What is the size of the Temporary C-RNTI?

Q5: Does the Uplink grant in the RAR contain a HARQ process ID? If not, RAN2 will decide on a HARQ process ID for msg3 transmission.

2. Actions:

To RAN1 group.

ACTION: 	RAN2 asks RAN1 to take the above agreements into account and reply to the questions above.

3. Date of Next RAN2 Meetings:

RAN2#99bis	9th – 13th October	Prague, Czech Republic

RAN2#100	27th November – 1st December	Reno, Nevada, USA