3GPP TSG-RAN WG2 #99	Tdoc R2-1709913

Berlin, Germany, 21st – 25th August 2017

Agenda Item:	10.2.6.2 – NR UE categories

Source:	Ericsson

Title:	Result of offline discussion #36 on need for UE Categories

Document for:	Discussion, Decision

# Introduction

Based on the online and offline discussions, the following is supposed to be captured.

# Discussion

In NR, the number of supported MIMO layers is signalled as explicit UE capability and not part of a UE category.

In NR, the modulation schemes are signalled as explicit UE capability and not part of a UE category.

RAN2 assumes that the UE’s band combinations together with the baseband capabilities (modulation scheme, MIMO layers, …) comprise all information necessary to calculate the maximum data rate achievable on each serving cell, in each cell group and per UE. Providing the data rate (category) explicitly is therefore unnecessary and undesirable in terms of signalling overhead.

A per-UE category is required for UEs supporting a peak data rate that is lower than the data rate achievable according to the above-mentioned parameters. However, UEs supporting dual connectivity (MR-DC, NR-NR DC) shall not advertise such a lower data rate (category) to avoid the need for inter-node negotiation of the data rate split.