3GPP TSG-RAN WG2 #99	Tdoc R2-1709964

Berlin, Germany, 21st – 25th August 2017

Agenda Item:	10.2.6.2 – NR UE categories

Source:	Ericsson

Title:	Result of offline discussion #36 on need for UE Categories

Document for:	Discussion, Decision

# Introduction

Based on the online and offline discussions, the following is supposed to be captured.

# Discussion

To be agreed In NR, the number of supported MIMO layers is signalled as explicit UE capability and not part of a UE category. In NR, the modulation schemes are signalled as explicit UE capability and not part of a UE category. RAN2 assumes that the UE’s band combinations together with the baseband capabilities (modulation scheme, MIMO layers, …) comprise all information necessary to calculate the maximum data rate achievable on each serving cell, in each cell group and per UE. If RAN1 and RAN4 agree with that assumption, RAN2 intends to apply the following: “A non-DC UE supporting a peak data rate (bits per “TTI”) that is lower than the data rate achievable according to the above-mentioned parameters indicates this by a per-UE category (data rate). However, a UE supporting dual connectivity (MR-DC, NR-NR DC) shall not advertise a category (data rate) that is lower than the highest data rate achievable according to any of the DC band combinations (to avoid the need for inter-node negotiation of the data rate split).”