3GPP TSG RAN WG2 Meeting #99	R2-1709972

Berlin, Germany, 21st – 25th August 2017

Title:	LS on SPS and SR for sTTI

Response to:

Release:	Rel-15

Study Item:	LTE_STTIandPT

Source:	RAN2

To:	RAN1

Contact Person:

Name:	Li Chai

E-mail Address:	chaili@huawei.com

1. Overall Description:

In RAN2#99 meeting, the following agreements for Scheduling Requests for short TTI were made:

Agreements: - A restriction similar to LCP restriction can be used to determine SR configuration to logical channel mapping - In the case of overlapping occasions of sPUCCH and PUCCH, it is left up to UE implementation which of sPUCCH or PUCCH SR resources to send SR on when SR can be sent on both PUCCH and sPUCCH. In case of non-overlapping SR occasions, the UE can transmit on the earliest SR occasion. The UE doesn’t transmit on both sPUCCH and PUCCH simultaneously. - Like in legacy LTE, inclusion of a BSR in a MAC PDU for transmission cancel all pending SRs, no specification change needed for this.

Based on RAN2 agreement, only one SR will be delivered to L1 from MAC when overlapping occasions of sPUCCH and PUCCH.

In addition, the following working assumptions for SR were made:

- An SR transmitted on sPUCCH starts an ssr-ProhibitTimer to prohibit SRs on sPUCCH until it times out;

- Different SR_COUNTERs are used.

Furthermore, In RAN2#99 meeting, the following agreements for SPS for short TTI were made:

Agreements: - RAN2 sees benefits to support SPS for short TTI - Same LCP restriction as dynamic grant will apply for SPS grant. - sTTI SPS intervals are configured by RRC. New sTTI-based interval values are introduced. The minimum value of 1 sTTI is supported. Other values are FFS. - In DL/UL SPS with asynchronous HARQ, the HARQ process ID is derived from the sTTI-number (index of sTTI within subframe). Details are FFS - Send LS to RAN1 to indicate RAN2 desire to support SPS and agreements. Ask the feasibility of SPS activation/deactivation for sTTI

Finally, RAN2 would like to inform RAN1that RAN2 desire to support SPS and check the feasibility of SPS activation/deactivation for sTTI.

2. Actions:

To RAN1 group.

ACTION: RAN2 would like to request RAN1 to take the above information into account in the discussions of SR and SPS.

In addition, RAN2 would like to inform RAN1that RAN2 desire to support SPS and check the feasibility of SPS activation/deactivation for sTTI.

3. Date of Next RAN2 Meetings:

TSG RAN2 Meeting #99bis	09 - 13 October 2017		Prague, Czechia

TSG RAN2 Meeting #100		27 Nov - 1 Dec 2017		Reno, USA