3GPP TSG-RAN WG2#99bis	Tdoc R2-1710383

Prague, Czech Republic, October 9-13, 2017	(Update of R2-1708162)

Agenda Item:	10.4.1.6.2

Source:	Ericsson

Title:	System information structure and contents

Document for:	Discussion, Decision

# Introduction

In this contribution, we propose a structure for the system information and propose a list of the contents of the minimum system information excluding the MIB (as the contents of the MIB are to a great extent already agreed in RAN1 and RAN2).

In section 2, we first propose to adopt the LTE system information framework and terminology also for NR, and then review the RAN2 agreements and contents of the LTE system information, and a list of information elements for minimum system information (i.e. information needed for initial access and SI acquisition for the rest of the system information).

# Discussion

## Structure of the system information

It can be noted that the naming of the LTE messages is somewhat confusing, as MIB, SIB1 and SI are defined as messages. SIB1 is thus defined as a message but the rest of the SIBs are defined as information elements. Since information elements and messages are conceptually different things, it would be desirable to avoid mixing up their naming in this manner.

One possible solution would be to call both SIB1 and SI as System Information messages and use numbering such as SI1 message (for SIB1) and SI2 message (for the other SIBs). MIB should then also have “message” in its name because it is a message and not an information element, perhaps named Master Information Message (MIM) instead of MIB. In that way, the System Information Block (SIB) could be solely used for information elements that are contained in the SI2 message.

However, this level of detail can probably be discussed during the specification drafting, and for easy understanding, we assume the LTE naming convention in the rest of the paper.

The implicit assumption in the RAN2 discussion on system information has been that the LTE structure with SI blocks and messages is on some level reflected in NR (most obvious from the agreements in Annex A.4 of TR38.804, which explicitly uses terms “SIB” and “SI-block”). We propose to adopt the LTE concept directly to NR

- System information is divided to master information block and system information blocks. Note that in order to avoid confusion, we will call the NR master information block NR-MIB and NR system information blocks NR-SIBs. This is since the contents of the NR MIB and SIBs may differ from the contents of the LTE MIB and SIBs.

- NR-MIB and NR-SIB1 are separate RRC messages.

- All other NR-SIBs are transmitted in SI messages scheduled in NR-SIB1.

System information in NR consists of MasterInformationBlock (NR-MIB) and System Information Blocks (NR-SIBs).

NR-MIB and NR-SIB1 are separate RRC messages, while all other NR-SIBs are transmitted in SI messages scheduled in NR-SIB1.

In addition to master and system information blocks, the NR system information is divided to “minimum” and “other” system information with the understanding that “The minimum SI comprises basic information required for initial access to a cell and information for acquiring any other SI broadcast periodically or provisioned via on-demand basis, i.e. scheduling information. The other SI encompasses everything not broadcast in the minimum SI”.

Note that the current agreements in RAN2 imply that minimum system information will contain (parts of) LTE SIB1 and (parts of) LTE SIB2. The main benefit of this split is that it allows different scheduling of LTE SIB1 (containing primarily cell camping parameters) and LTE SIB2 (containing cell access parameters). In theory, the NR minimum system information could also be split between NR-SIB1 and NR-SIB2, but since the full minimum system information is needed to access the cell, it would seem strange to provide parts of it only on-demand. In our opinion, it is preferable to provide the full minimum system information in NR-SIB1 (i.e. move information needed for initial access from LTE SIB2 to NR-SIB1). Using a staggered approach with UE having to first read the NR-MIB and NR-SIB1 to obtain the scheduling information of NR-SIB2, and then waiting for the NR-SIB2 to be scheduled may create an additional delay for the connection set-up.

Minimum system information consists of NR-MIB and NR-SIB1 (which contains information from LTE SIB1 and LTE SIB2), while all other NR-SIBs are part of other system information.

## Contents of the minimum system information

In this section, we evaluate which LTE SIB1 and LTE SIB2 contents are needed for initial access to a cell and for acquiring any other SI. We also observe a number of additional RAN2 agreements for system information and discuss the impact of those on RRC specification.

We have listed the LTE SIB1 and LTE SIB2 contents in Table 2, and proposed a suitable NR SIB for them. Basically we propose that the NR-SIB1 contains information from LTE SIB1 and LTE SIB2 needed for initial access. See text below the table for justification of the placement.

Base-line NR-SIB1 contains information from LTE SIB1 and SIB2 needed for initial access.

Table 2: LTE SIB1 and SIB2 contents.

| SIB1 | Comment | NR location |
| --- | --- | --- |
| plmn-IdentityList | Including a per PLMN indication of a cell reserved for operator use. | NR-SIB1. Note that the information related to multi-operator support might be extended in NR compared to what is supported in LTE, see [1]. |
| trackingAreaCode |  | NR-SIB1 |
| cellIdentity |  | NR-SIB1 (already agreed to be part of minimum SI) |
| cellBarred |  | NR-MIB or NR-SIB1 (already agreed to be part of minimum SI), depending on the discussion on the indication in MIB that cell is not campable. |
| intraFreqReselection | Indicating whether intra-frequency reselection is allowed in barred cells. | NR-SIB1 (already agreed to be part of minimum SI) |
| CSG related information csg-Indication csg-Identity |  | NR-SIB2+ |
| Cell Selection Information q-RxLevMin q-RxLevMinOffset |  | NR-SIB1 (already agreed to be part of minimum SI) |
| p-Max |  | NR-SIB1 (to be verified by RAN1) |
| freqBandIndicator |  | NR-SIB1 |
| schedulingInfoList | List of SI periodicity and mapping of SIBs to SI messages | NR-SIB1 |
| tdd-Config |  | NR-SIB1 |
| WindowLength |  | NR-SIB1 |
| systemInfoValueTag |  | NR-SIB1 |
| SIB2 | Comment | NR location |
| Access barring information |  | NR-SIB1 |
| Radio Resource Config Common |  | NR-SIB1 (at least for parameters needed for initial access). |
| UE Timers and Constants |  | NR-SIB2+ |
| Frequency information | UL carrier frequency, ARFCN, bandwidth and additional spectrum mask | NR-SIB1 |
| MBSFN configuration | MBMS not part of Rel-15, and even though this was introduced in LTE prior to MBMS introduction, due NR frame structure being more flexible, this is probably not needed even for forward compatibility. | Likely not needed (NR-SIB2+ if needed) |
| timeAlignmentTimerCommon |  | NR-SIB2+ |

Some of the LTE system information elements have already been agreed to be included in NR minimum system information, e.g. cell ID and list of PLMNs (though the new use cases envisioned for NR might require further extensions to enhance the support for network sharing, see [1]). The inclusion of such parameters in NR-SIB1 is straightforward.

In addition to the PLMN ID list and cell ID, it seems natural to assume that tracking area code is also needed in NR, but since this depends on the 5G core network design, the need for tracking area code should be verified with SA2. However, we have assumed for the sake of progress that tracking area code is needed.

The LTE cell camping parameters (intrafrequency reselection allowed, QRxLevMin and QRxLevMinOffset) are included in LTE SIB1). The actual information for NR might depend on technical solutions chosen for idle mode mobility (due to e.g. beam based mobility), but for baseline operation it is assumed that the LTE information elements are a good starting point.

The LTE SIB1 also includes CSG indicator, and SIB2 MBSFN subframe configuration. These are examples of “services” supported by the cell and there is a RAN2 agreement to broadcast these “services” in system information. On one hand, the UE should probably avoid camping on a cell not supporting the service UE is interested in. This would speak in the favour of including supported “services” in NR-SIB1. On the other hand, this information is not really required to access the system, and it might be useful for the UE to be able to request and receive on-demand system information from all cells, including e.g. CSG cells UE is not a member of. For the time being, we have assumed that NR-SIB1 does not contain information about supported service, but think that a reserved cell indication in system information should be introduced for forward compatibility purposes, e.g., to later be able to define cells with various restrictions for certain UEs.

Note that MBSFN subframe configuration is also used to provide forward compatibility in LTE design. We assume that such mechanisms are built into the basic NR frame structure, and do not need special configuration. However, if RAN1 indicates that subframe configuration is needed, this information should probably be provided in NR-SIB1.

Assume that the base-line NR-SIB1 does not contain information about services supported by a cell, but rather a reserved cell indication is introduced for forward compatibility.

The LTE SIB1 also contains an indication of the frequency band, including multi-band information, while LTE SIB2 contains information on UL frequency bands. For NR, the UE will also need the UL frequency band information in order to be able to transmit the request for on-demand system information (at least for msg3 based request).

RAN1 has indicated that TDD configuration needs to be signalled to the UE during initial access and hand-over.

Assume that the base-line NR-SIB1 contains DL frequency band, including multi-band information, UL frequency band and TDD UL/DL configuration.

The network should have mechanisms to control the access load by barring selected UEs. This barring should be done as part of initial access, which implies that the access barring parameters should be part of the NR-SIB1.

The minimum system information needs to provide all required physical channel and higher layer information needed for initial access. The explicitly mentioned RAN2 agreements on RACH configuration does not seem to be sufficient, and it seems natural to provide a full configuration (similar to radio configuration common in LTE SIB2) in minimum system information even though the exact physical parameters are not known at this point of time. For simplicity, we have named this information “initial access configuration”.

The need for UE timers and constants seems less clear, and one could envision using default values for the timers and constants until network has a chance to reconfigure those with dedicated signalling. For this reason, we propose that the UE timers and constants are not part of the NR-SIB1. The only exception to this may be the TA timer, which should be configurable based on the cell size.

Assume that the base-line NR-SIB1 does not contain UE timers and constants.

In RAN2#99, it was also agreed that synchronisation information can be provided via both dedicated RRC signalling and broadcast (but not in NR MIB). The synchronization information is an indication related to the synchronization information for the carrier. When the indication is enabled, UE can utilize serving cell timing to derive the index of SS block transmitted by neighbour cell. In our understanding this information is primarily needed for intra-frequency measurements and therefore should not be included in the minimum system information.

Assume that synchronization information is not included in base-line NR-SIB1.

Table 3: Baseline NR-SIB1 contents. Note that additional information elements are likely to be included based on further RAN2 decisions.

| Information element | Comment |
| --- | --- |
| List of PLMNs | Including indication of a cell reserved for operator use. May need to be extended to support new use cases, see [1] |
| Tracking Area Code | Need to be verified with SA2. May need to be extended to support new use cases, see [1] |
| Cell Identity | May need to be extended to support new use cases, see [1] |
| Cell camping parameters | Cell Barred Intra Frequency Reselection allowed Q_RxLevMin Q_RxLevMinOffset |
| p-Max | Needed in order to limit the UE's uplink transmission power in the NR cell |
| Frequency Band Information | Including multi-band information Including UL band information from LTE SIB2. |
| TDD UL/DL configuration | According to RAN1 agreement |
| Access Barring parameters | May aim for a unified approach instead of reusing LTE IEs. |
| Initial access configuration | Including RACH configuration, but also other physical channel and higher layer configurations needed at least to be able to receive messages 2 and 4 and to transmit msg3. |
| Scheduling Info List | SI Periodicity SIB Mapping info (Sequence of SIBs) Value Tag / Index / Identifier (per SI message) SI Window Length SI Window (per SI message) On-demand indication, i.e. an indication whether the SI message is broadcast or available on-demand and an indication if msg1 or msg3 based mechanism should be used to request on-demand system information. |
| Additional parameters for requesting other SI | E.g. special RA preamble values, dedicated RACH opportunities, … |
| systemInfoValueTag | Index/identifier to avoid re-acquisition of system information. Indicates the current valid version of the system information in the cell. Note that several valuetags may be needed, e.g. one for NR-SIB1 and per SIB or per SI message valuetags for other SI. |
| Area ID |  |

The contents of the NR-SIB are summarized in Table 3, and we propose to adopt this as the baseline NR-SIB1 contents. Note that additional information elements are likely to be included based on further RAN2 decisions.

Adopt the list of information elements in Table 3 as the baseline NR-SIB1 contents.

# Conclusion

Based on the discussion in section 2 we propose the following:

Proposal 1	System information in NR consists of MasterInformationBlock (NR-MIB) and System Information Blocks (NR-SIBs).

Proposal 2	NR-MIB and NR-SIB1 are separate RRC messages, while all other NR-SIBs are transmitted in SI messages scheduled in NR-SIB1.

Proposal 3	Minimum system information consists of NR-MIB and NR-SIB1 (which contains information from LTE SIB1 and LTE SIB2), while all other NR-SIBs are part of other system information.

Proposal 4	Base-line NR-SIB1 contains information from LTE SIB1 and SIB2 needed for initial access.

Proposal 5	Assume that the base-line NR-SIB1 does not contain information about services supported by a cell, but rather a reserved cell indication is introduced for forward compatibility.

Proposal 6	Assume that the base-line NR-SIB1 contains DL frequency band, including multi-band information, UL frequency band and TDD UL/DL configuration.

Proposal 7	Assume that the base-line NR-SIB1 does not contain UE timers and constants.

Proposal 8	Assume that synchronization information is not included in base-line NR-SIB1.

Proposal 9	Adopt the list of information elements in Table 3 as the baseline NR-SIB1 contents.

# References

R2-1708169, “System information content at network sharing”, Ericsson

R2-1708160, “Indication in NR system information that cell is not campable”, Ericsson

# Appendix: LTE SIB1 and SIB2 contents.

SystemInformationBlockType1 ::=		SEQUENCE {

cellAccessRelatedInfo				SEQUENCE {

plmn-IdentityList					PLMN-IdentityList,

trackingAreaCode					TrackingAreaCode,

cellIdentity						CellIdentity,

cellBarred							ENUMERATED {barred, notBarred},

intraFreqReselection				ENUMERATED {allowed, notAllowed},

csg-Indication						BOOLEAN,

csg-Identity						CSG-Identity			OPTIONAL	-- Need OR

},

cellSelectionInfo					SEQUENCE {

q-RxLevMin							Q-RxLevMin,

q-RxLevMinOffset					INTEGER (1..8)			OPTIONAL	-- Need OP

},

p-Max								P-Max						OPTIONAL,			-- Need OP

freqBandIndicator					FreqBandIndicator,

schedulingInfoList					SchedulingInfoList,

tdd-Config							TDD-Config					OPTIONAL,	-- Cond TDD

si-WindowLength						ENUMERATED {

ms1, ms2, ms5, ms10, ms15, ms20,

ms40},

systemInfoValueTag					INTEGER (0..31),

nonCriticalExtension				SystemInformationBlockType1-v890-IEs	OPTIONAL

}

SchedulingInfoList ::= SEQUENCE (SIZE (1..maxSI-Message)) OF SchedulingInfo

SchedulingInfo ::=	SEQUENCE {

si-Periodicity						ENUMERATED {

rf8, rf16, rf32, rf64, rf128, rf256, rf512},

sib-MappingInfo						SIB-MappingInfo

}

SIB-MappingInfo ::= SEQUENCE (SIZE (0..maxSIB-1)) OF SIB-Type

SIB-Type ::=						ENUMERATED {

sibType3, sibType4, sibType5, sibType6,

sibType7, sibType8, sibType9, sibType10,

sibType11, sibType12-v920, sibType13-v920,

sibType14-v1130, sibType15-v1130,

sibType16-v1130, sibType17-v1250, sibType18-v1250,

..., sibType19-v1250, sibType20-v1310}

SystemInformationBlockType2 ::=		SEQUENCE {

ac-BarringInfo						SEQUENCE {

ac-BarringForEmergency				BOOLEAN,

ac-BarringForMO-Signalling			AC-BarringConfig				OPTIONAL,	-- Need OP

ac-BarringForMO-Data				AC-BarringConfig				OPTIONAL	-- Need OP

}																		OPTIONAL,	-- Need OP

radioResourceConfigCommon			RadioResourceConfigCommonSIB,

ue-TimersAndConstants				UE-TimersAndConstants,

freqInfo							SEQUENCE {

ul-CarrierFreq						ARFCN-ValueEUTRA				OPTIONAL,	-- Need OP

ul-Bandwidth						ENUMERATED {n6, n15, n25, n50, n75, n100}

OPTIONAL,	-- Need OP

additionalSpectrumEmission			AdditionalSpectrumEmission

},

mbsfn-SubframeConfigList			MBSFN-SubframeConfigList			OPTIONAL, 	-- Need OR

timeAlignmentTimerCommon			TimeAlignmentTimer,

...,

lateNonCriticalExtension		OCTET STRING (CONTAINING SystemInformationBlockType2-v8h0-IEs)						OPTIONAL,

[[	ssac-BarringForMMTEL-Voice-r9		AC-BarringConfig				OPTIONAL,	-- Need OP

ssac-BarringForMMTEL-Video-r9		AC-BarringConfig				OPTIONAL	-- Need OP

]],

[[	ac-BarringForCSFB-r10				AC-BarringConfig			OPTIONAL	-- Need OP

]],

[[	ac-BarringSkipForMMTELVoice-r12		ENUMERATED {true}			OPTIONAL,	 -- Need OP

ac-BarringSkipForMMTELVideo-r12		ENUMERATED {true}			OPTIONAL,	 -- Need OP

ac-BarringSkipForSMS-r12			ENUMERATED {true}			OPTIONAL,	 -- Need OP

ac-BarringPerPLMN-List-r12			AC-BarringPerPLMN-List-r12	OPTIONAL	-- Need OP

]],

[[	voiceServiceCauseIndication-r12		ENUMERATED {true}			OPTIONAL	-- Need OP

]],

[[	acdc-BarringForCommon-r13			ACDC-BarringForCommon-r13		OPTIONAL,	-- Need OP

acdc-BarringPerPLMN-List-r13		ACDC-BarringPerPLMN-List-r13	OPTIONAL	-- Need OP

]],

[[

udt-RestrictingForCommon-r13			UDT-Restricting-r13				OPTIONAL,	-- Need OR

udt-RestrictingPerPLMN-List-r13		UDT-RestrictingPerPLMN-List-r13	OPTIONAL,	-- Need OR

cIoT-EPS-OptimisationInfo-r13		CIOT-EPS-OptimisationInfo-r13 	OPTIONAL,	-- Need OP

useFullResumeID-r13					ENUMERATED {true}				OPTIONAL	-- Need OP

]]

}

SystemInformationBlockType2-v8h0-IEs ::=	SEQUENCE {

multiBandInfoList				SEQUENCE (SIZE (1..maxMultiBands)) OF AdditionalSpectrumEmission	OPTIONAL,	-- Need OR

nonCriticalExtension			SystemInformationBlockType2-v9e0-IEs	OPTIONAL

}