3GPP TSG-RAN2 Meeting #99bis	R2-1710458

Prague, Czech Republic, 9th – 13th October, 2017	Revision of: R2-1708068

Agenda item:	10.4.1.6.2

Source: 	Huawei, HiSilicon

Title: 		Detailed design of the contents of System Information

Document for:	Discussion and Decision

# Introduction

Compared with the previous paper, the following changes have been made:

- Added Proposal 7 and related text, according to the agreement on system information in the last meeting.

The contents of system information have been discussed for several meetings and some agreements have been achieved and captured in [1].

With regards to system information provisioning on stage-3 level: - The minimum SI includes at least SFN, list of PLMN, Cell ID, cell camping parameters, RACH parameters. - A unique global cell ID is broadcast for an NR cell. - If network allows on demand mechanism, parameters required for requesting other SI-block(s) (if any needed, e.g. RACH preambles for request) shall be included in minimum SI. - Cell-reselection neighbouring cell information is considered as other SI. - PWS information can be classified into the other SI. - The scheduling information for the other SI includes SIB type, validity information, SI periodicity and SI-window information and is provided irrespective of whether the other SI is periodically broadcast or not. - The scheduling information in minimum SI includes an indicator whether the concerned SI-block is periodically broadcasted or provided on demand. If minimum SI indicates that a SIB is not broadcasted, then UE does not assume that this SIB is a periodically broadcasted in its SI-window at every SI periodicity. Therefore the UE may send an SI request to receive this SIB. - Broadcasting some kind of index/identifier in minimum SI to enable the UE to avoid re-acquisition of already stored SI-block(s)/SI message(s). The index/identifier and associated system information can be applicable in more than one cell. System information valid in one cell may be valid also in other cells. - It is FFS what the index/identifier is, e.g. single index or area plus value tag, etc

For the MIB Content, the following agreements have been made in RAN1#89 and RAN1 Adhoc, indicated in the LS from RAN1 [2]:

- RAN1 has agreed to include the following information in the NR-PBCH: - (Part of) SFN: [7 - 10] bits - Information for remaining minimum system information scheduling: [x] bits - Bits reserved for future use: [x] bits - CRC: [16+y] bits - Additionally, RAN1 is discussing whether the following L1 related information needs to be included: - Information regarding bandwidth part: [x] bits - Information for quick identification that there is no corresponding RMSI to the PBCH: [0-1] bits - SS burst set periodicity: [0-3] bits - Information on actual transmitted SS block(s): [0-x] bits - Information on tracking RS: [x] bits - Timing information within radio frame: [0 - 7] bits RAN1 also ask RAN2 to consider the necessity including the following higher layer information in the NR-PBCH in the LS [2]: - Hyper-SFN - Information for quick identification that UE cannot camp on the cell - Area ID - Value tag - Cell ID extension Agreements: - Working assumption: 3 bits of SS block index are carried by changing the DMRS sequence within each 5ms period - It can be further considered to limit the number of bits carried in this way to 2 if carrying 3 bits is shown to cause problems - FFS: details of scrambling of the PBCH which may or may not carry a part of timing information FFS: 5 ms half radio frame interval indicationRemaining bits of the timing information are carried explicitly in the NR-PBCH payload

Then in RAN2 # Ad hoc, the following agreement had been achieved based on the RAN1 LS:

Agreements 1 For cell ID extension we can indicate to RAN1 that RAN2 understand this to be referring to physical cell ID extension (i.e. not related to GCI) and RAN2 has not identified a RAN2 need for Cell ID extension and leave the discussion and final decision whether this is needed to RAN1. Can further indicate that there will be a GCI in SIB1. Can also indicate that adding such an extension in future releases would be possible but it would not be understood by legacy UEs. 2 There is some indication in MIB that a cell is not campable (at least to address the NSA cell case). If additional information is needed then at most this information would be 2 bits. FFS whether the SIB1 presence flag (understood to be RMSI in RAN1's terminology) or omission of SIB1 scheduling information could be used for this purpose or an additional indicator (could be today's cellbarred bit) is needed. FFS whether an intra-freq Reselection indicator would be useful in MIB. 3 RAN2 will let RAN1 conclude how much of SFN to include in MIB and RAN2 can further discuss how much additional SFN should be carried in a SIB. Can discuss more offline whether RAN2 have a preference for the minimum number of SFN bits that can be determined by reading MIB RAN2 will indicate to RAN1 that no value tag/area related info will be included in MIB

In last RAN2 meeting, the following agreement had been achieved

Working assumption 1: 1 bit (final name FFS, but same UE behaviour as cellbarred in LTE) is included in NR MIB to indicate that a cell cannot be camped on. intraFreqReselection is 'not allowed' (not signalled). BarredTimer is specified, value FFS. cellBarred and intraFreqReselection signalling in SIB1 (as in LTE) Agreements 2: Synchronisation information (1 bit) is provided to inform that the UE can utilise serving cell timing to derive the index of SS-block transmitted by neighbour cell (e.g. radio frame or SFN or symbol level synchronisation) 3: The synchronisation information can be provided via both dedicated RRC signalling and broadcast, but not via NR MIB

There are still FFS and open issues of contents of system information. This contribution provides some discussion on the organization and contents of system information in NR.

# Discussion

## Minimum System Information

### Content of Minimum SI

Based on RAN2 agreements, the minimum SI can be regarded as roughly equivalent to the information contained in MIB/SIB1/SIB2 in LTE, and Minimum SI shall enable the UE to camp on the cell and access the cell. Then such minimum SI should comprise at least:

- PLMN selection and cell selection; - Information necessary to access the cell for either requesting the Other SI or initiating transition to RRC connected mode - Information of other SIs available in the cell, including the SIB type and validity information - Scheduling information for periodically broadcasted SI-block(s), the scheduling information for other SI should include SIB type, validity information, periodicity, SI-window information - Configuration for requesting other SIB(s) if on demand mechanism is allowed

Although some specific IEs might not be same, e.g. numerology related IEs and slice related IE, the most of the functions and information are also supported in LTE. Based on this, we provides the initial content of MIB and RMSI in the following sections:

#### Parameters in MIB

MIB will be sent on the NR-PBCH channel, while the other minimum SI will be sent on the NR PDSCH channel. As agreed in RAN1 and RAN2 and discussed in [3], we prefer to only include the necessary information in the MIB, as shown in Table1.

Table 1: Parameter and size of NR-PBCH

| Parameter | Comment | Need for NR |
| --- | --- | --- |
| Duration | PHICH-structure | NO NEED |
| Resource | PHICH-structure | NO NEED |
| SFN | 7…10 | YES |
|  Information on actual transmitted SS block(s) | 0…x | New |
| RMSI scheduling information | FFS | New |
| [Information regarding bandwidth part] | FFS | New |
| Information for quick identification that there is no corresponding RMSI to the PBCH | 1 | FFS |
| [Information for quick identification that UE can not camp on the cell] | 1 | New |
| SS burst set periodicity | 0…3 | New |
| Timing information within radio frame | 0…7 | New |
| Information on tracking RS | x | New |
| - Reserved bits | >=10 | Update |
| CRC bits | 19 | Update (Not in the content of MIB) |
| Total | 40+[11]+[x] | Total |

Proposal 1: The basic information of PBCH in NR includes SFN, scheduling information, information regarding bandwidth and SS burst set periodicity and timing information within radio frame.

#### Parameters in RMSI

Table 2: Parameters of RMSI

| Parameter | Comment | Need for NR |
| --- | --- | --- |
| CellAccessRelatedInfo | - Cell Access related Info: Plmn/Tac/Cell Id/Cell Barred and so on. | YES |
| CellSelectionInfo |  | YES |
| TDD specifics |  | YES |
| schedulingInfoList |  | Update |
| - SIB type - Validity information - SI periodicity - SI-window information |  | YES |
| - Indication whether SI block is periodically broadcasted or provided on demand |  | New |
| - Area ID |  | New |
| - Value tag |  | Update(TBD) |
| Other Parameters |  | YES |
| Extended Information |  | YES |
| AC Barring Info |  | YES |
| UL frequency Info |  | YES |
| RadioResourceConfigCommon |  | YES |
| UE Timers and Constants |  | YES |
| timeAlignmentTimerCommon |  | YES |
| mbsfn-SubframeConfigList |  | Not for R15 |
| voiceServiceCauseIndication-r12 |  | May not, depends on the cause value design. |
| udt-estrictingForCommon-r13 |  | FFS |
| udt-RestrictingPerPLMN-List-r13 |  | FFS |
| cIoT-EPS-OptimisationInfo-r13 |  | Not for R15 |
| useFullResumeID-r13 |  | Not for R15 |

Proposal 2: The basic information of RMSI in NR includes CellAccessRelatedInfo, CellSelectionInfo, TDD specifics, schedulingInfo with the indication of the broadcast status for other SI, Area ID and Value tag, ACBarringInfo, UL frequency Info, RadioResourceConfigCommon, UE Timers and Constants, and timeAlignmentTimerCommon.

### Structure of minimum SI

To key point of structure design of minimum SI is to consider the capacity of one scheduling unit and the size of the RMSI. As agreed in RAN1, PBCH will indicate the information for remaining minimum system information scheduling. This means that there is no limitation of usage of default numerology/ TTI length of RMSI, which is a kind of dynamic scheduling mode. Hence, it is possible that one scheduling unit is enough to transmit all the remaining minimum SI, based on the above analysis. Although splitting the RMSI into SIB1 and SIB2 can provide the flexibility of processing different priority information in different transmission cycles, considering the time limitation of transmission in high frequency scenarios, the benefit is weakened. Then, it is preferred to name RMSI as “SIB1” to facilitate the number of SIBs of other SI.

Observation 1: Since there is no limitation of usage of default numerology/ TTI length of RMSI and it is a kind of dynamic scheduling mode for the RMSI, it is possible that one scheduling unit is enough to transmit all the remaining minimum SI.

Observation 2: Although splitting the RMSI into SIB1 and SIB2 can provide the flexibility of processing different priority information in different transmission cycles, considering the time limitation of transmission in high frequency scenarios, the benefit is weakened.

Therefore, it is proposed:

Proposal 3: Minimum SI in NR includes MIB and RMSI, without splitting into SIB1 and SIB2.

Proposal 4: it is preferred to name RMSI as “SIB1” to facilitate the number of SIBs of other SI.

## Other SI

As specified in [1], the other SI encompasses everything not broadcast in the minimum SI. And Other SI in NR shall include the rest of the SIBs (i.e. SIB3, SIB4, etc). The detailed content of the system information blocks depends on further study. Reselection parameters are already agreed to be classified into other SI, then LTE SIB3~5 can be taken as baseline. The functions of Relay, heNB, WLAN, D2D, SCPTM, NB-IoT and V2X are not in the scope of rel15 NR. Hence, the corresponding SIB9, SIB17-SIB21 are not needed in other SI. Similarly, SIB 6-8 are not needed as well since we do not support interworking with UMTS/GERAN/CDMA in rel-15.

Proposal 5: Reselection parameters can take LTE SIB3~5 as baseline and are classified into Other SI.

Proposal 6: Since the functions of Relay, heNB, WLAN, D2D, SCPTM and V2X are not in Rel15 NR, the corresponding SIB9, SIB17-SIB21 are not needed in other SI. Similarly, SIB 6-8 are not needed as well since no interworking with UMTS/GERAN/CDMA in Rel-15.

In last meeting, it has been agreed that synchronisation information (1 bit) is provided to inform that the UE can utilise serving cell timing to derive the index of SS-block transmitted by neighbour cell (e.g. radio frame or SFN or symbol level synchronisation). And it is concluded that the synchronisation information can be provided not via NR MIB. Considering that synchronisation information (1 bit) is provided to inform that the UE can utilise serving cell timing to derive the index of SS-block transmitted by neighbour cell (e.g. radio frame or SFN or symbol level synchronisation), it is better to locate the information in the cell reselection information. Currently, the cell reselection information in LTE is per carrier, e.g., SIB3 (intra-freq reselection)/ SIB5 (inter-freq reselection). It is desirable that similar configuration will be considered in NR.

Besides, according to RAN1 AH2 agreements, it was said that “When there is the indication for a carrier, UE can utilize serving cell timing to derive the index of SS block transmitted by neighbour cell (e.g., radio frame or SFN or symbol level synchronization)”. It is reasonable that the synchronization information should be provided per carrier.

Proposal 7: The synchronisation information is provided per carrier and will be located in cell reselection information.

# Conclusion

The contribution addressed the detailed design on the contents of System Information for NR. The resulting observations and proposals are as follows:

Observation 1: Since there is no limitation of usage of default numerology/ TTI length of RMSI and it is a kind of dynamic scheduling mode for the RMSI, it is possible that one scheduling unit is enough to transmit all the remaining minimum SI.

Observation 2: Although splitting the RMSI into SIB1 and SIB2 can provide the flexibility of processing different priority information in different transmission cycles, considering the time limitation of transmission in high frequency scenarios, the benefit is weakened.

Proposal 1: The basic information of PBCH in NR includes SFN, scheduling information, information regarding bandwidth and SS burst set periodicity and timing information within radio frame.

Proposal 2: The basic information of RMSI in NR includes CellAccessRelatedInfo, CellSelectionInfo, TDD specifics, schedulingInfo with the indication of the broadcast status for other SI, Area ID and Value tag, ACBarringInfo, UL frequency Info, RadioResourceConfigCommon, UE Timers and Constants, and timeAlignmentTimerCommon.

Proposal 3: Minimum SI in NR includes MIB and RMSI, without splitting into SIB1 and SIB2.

Proposal 4: it is preferred to name RMSI as “SIB1” to facilitate the number of SIBs of other SI.

Proposal 5: Reselection parameters can take LTE SIB3~5 as baseline and are classified into Other SI.

Proposal 6: Since the functions of Relay, heNB, WLAN, D2D, SCPTM and V2X are not in Rel15 NR, the corresponding SIB9, SIB17-SIB21 are not needed in other SI. Similarly, SIB 6-8 are not needed as well since no interworking with UMTS/GERAN/CDMA is in Rel-15.

Proposal 7: The synchronisation information is provided per carrier and will be located in cell reselection information.

# Reference

- TS 38.304 v003

- R1-1709789, LS on NR-PBCH content, Ericsson, RAN1#89