3GPP TSG-RAN WG2 #99bis	Tdoc R2-1711240

Prague, Czech Republic, 9th – 13th October 2017

Agenda Item:	10.2.2.3

Source:	Ericsson

Title:	Number of supported DRBs in EN-DC and LTE

Document for:	Discussion, Decision

# Introduction

RAN2 received LS from SA2 [1] regarding to the number of supported DRBs. This contribution discusses multi-DRB use cases as well as UE and network limitations related to the number of DRBs supported. The number of DRBs is related to LTE, EN-DC and NR radio interfaces and therefore, this contribution discusses all of the cases.

# Discussion

## LTE baseline

LTE supports a maximum of 8 DRBs.

The DRB-ToAddModList has up to maxDRB entries…

DRB-ToAddModList ::=				SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod

where maxDRB is defined to be 11.

maxDRB						INTEGER ::= 11	-- Maximum number of Data Radio Bearers

The DRB configuration contains among others the eps-BearerIdentity, the drb-Identity and the logicalChannelIdentity.

DRB-ToAddMod ::=	SEQUENCE {

eps-BearerIdentity					INTEGER (0..15)			OPTIONAL,		-- Cond DRB-Setup

drb-Identity						DRB-Identity,

pdcp-Config							PDCP-Config				OPTIONAL,		-- Cond PDCP

rlc-Config							RLC-Config				OPTIONAL,		-- Cond SetupM

logicalChannelIdentity				INTEGER (3..10)			OPTIONAL,		-- Cond DRB-SetupM

logicalChannelConfig				LogicalChannelConfig	OPTIONAL,		-- Cond SetupM

...,

[[	drb-TypeChange-r12					ENUMERATED {toMCG}		OPTIONAL,		-- Need OP

rlc-Config-v1250					RLC-Config-v1250		OPTIONAL		-- Need ON

]],

[[	rlc-Config-v1310					RLC-Config-v1310		OPTIONAL,		-- Need ON

drb-TypeLWA-r13						BOOLEAN					OPTIONAL,		-- Need ON

drb-TypeLWIP-r13					ENUMERATED {lwip, lwip-DL-only,

lwip-UL-only, eutran}		OPTIONAL		-- Need ON

]]

}

DRB-Identity ::=					INTEGER (1..32)

The eps-BearerIdentity can take values from 0 to 15 and the DRB-Identity values between 1 and 32. Having a larger value range for DRB ID than for EPS bearer IDs allows the eNB to fulfil the security requirements (see 36.331, section 5.3.1.2): “The eNB is responsible for avoiding reuse of the COUNT with the same RB identity and with the same KeNB, e.g. due to the transfer of large volumes of data, release and establishment of new RBs. In order to avoid such re-use, the eNB may e.g. use different RB identities for successive RB establishments, trigger an intra cell handover or an RRC_CONNECTED to RRC_IDLE to RRC_CONNECTED transition.”

Overall, the number of simultaneous DRBs that may be configured for an LTE UE is limited by the number of logical channels: Only logical channel IDs in the range of 3 to 10 are allowed for DRBs and hence at most 8 DRBs may be established.

## Worst case scenario for multi DRBs

With regard to the services described in [2]: One worst case scenario where 8 DRBs could be needed is when a user is using internet (e.g. retrieving maps or other information, streaming live news), is in contact with his Push to Talk (PTT) group, and then needs to make an emergency call. The described scenario would result:

- Internet web: 1 web default bearer (AM) + 1 live streaming bearer (AM)

- VoLTE: 1 signalling bearer (AM) + 1 Voice bearer (UM) (+ 1 video bearer (UM))

- MC-PTT: 1 signalling bearer (AM) + 1 voice bearer (UM) (+ 1 data bearer (AM))

- = 4 AM + 2 UM, or = 5 AM + 3 UM.

Even this extreme case (which can be seen as a very rare case) is not limited by the number of DRBs and is supported by current specifications. Supporting more than 8 DRBs seems like a theoretical corner case rather than real requirement.

## DRBs and QFIs are dynamic

5CN defines timer for a QFI in PDU session. If there is no enough traffic for a QFI, it will get removed, or at least set on hold. This results that the nature of the DRBs is not static but rather dynamic. When QFI is released, or user ends a session, DRB is released. This will free up resources to use.

Nature of QFIs & DRBs is dynamic rather than static, resulting that DRBs are released when services are no longer needed.

Additionally, UE support for multi-DRB use cases should be clear before increasing the number of DRBs in the system. It should be clear what concurrent services are practical for UE hardware and software implementations.

It may not be feasible to support all 5QI combinations. It should be clear what combinations may be supported by UE before increasing the number of DRBs.

## Impact on Increasing number of DRBs

For each DRB instantiated, there will be new RLC and PDCP entities for the UE and the eNB/gNB. As the number of DRBs increases, the number of PDCP and RLC entities, with their corresponding physical resources required will increases. Increasing the number of DRBs would result those physical resources to be shared among the DRBs. This problem is general and not tied to any specific hardware implementation. For example, when considering that each DRB will have queues in traffic management entity handling the QoS. The queues require dynamic memory as some queues may be empty while others have packets to send. When the number of DRBs is increased, there will be more queues, sharing the same resources. This may result that the initial design assumptions are not valid and some thresholds (e.g. AQM) needs to be changed. In legacy UE hardware implementations, this may not even be possible.

For each Radio Bearer (SRB or DRB) instantiated, bearer contexts must be maintained in PDCP, RLC, as well as in the scheduler and management plane software on both UE and network side.  In addition, the UE contexts must be larger to either contain all the max possible bearer contexts for that UE or have larger set of meta data to point to separate bearer contexts.

Increasing the number of DRBs may result currently unforeseen problems

It may not be feasible to increase the number of DRBs in LTE due to existing UE hardware implementations.

Based on our experience from LTE deployments, the average number of SRB+DRB connected user is 2 SRB + 3.25 DRB. For the current LTE deployments, there is not enough justification from use cases to support more than 8 DRBs.

LTE support for the number of DRBs is not changed.

When connected to in EN-DC case, the EPC has maximum of 8 DRBs per UE as described above. As the CN is responsible of e-RAB allocations and there is 1-1 mapping between e-RAB and DRB, the number of DRBs is mandated by the LTE system. Therefore, increasing the number of DRBs when NR connects to EPC (LTE+NR DC) does not seem necessary.

Increasing the number of DRBs when NR connects to EPC (LTE+NR DC) does not seem necessary.

Number of DRBs is not changed in EN-DC case.

## DRBs in NR system

In NR, network slices will be supported. A network slice consists of a RAN part and a CN part. The support of network slicing relies on the principle that traffic for different slices is handled by different PDU sessions [3]. TS 38.300 sets a lower limit to the number of slices that UE needs to support. The number of parallel slices is 8. Each network slice is uniquely identified by a S-NSSAI (Single Network Slice Selection Assistance Information) and maximum of 8 S-NSSAI may be grouped into NSSAI (Network Slice Selection Assistance Information) [4].

Based on the slice definition in 38.300 [3], there will be one PDU session per slice. Each PDU session will have at least one QFI [4] resulting that the gNB will allocate at least one DRB per slice. Even though, one PDU session is required per slice and UE supports 8 PDU sessions, when there are less than 8 PDU sessions per slice, each QFIs in PDU sessions will be able to use DRBs that are not reserved.

As the resources of each slice are dynamic in RAN, the DRB allocation in RAN may change based on the load on each slice. Currently there are no use cases foreseen that would require 8 consecutive slice instances.

NR system uses Flow based QoS Framework. Figure 1 illustrates how SDAP layer maps CN QoS Flows to DRBs. The mapping in NR allows flexibility to the system, there is no anymore requirement for having 1-1 mapping between QoS Flow and DRB.

Figure 1: NR Flow based QoS framework with QFI to DRB mapping

With QFI mapping of flows to DRBs, NR already has gained flexibility on mapping traffic on same PDU session to most appropriate DRB bearer.  The QoS scheduling at the MAC layer may not be improved by having more DRBs.  It may rather be opposite and it becomes a burden for the implementations.

Even if the CN distinguishes many QoS Flows, the NR RAN may map them onto fewer DRBs, which indicates that NR does not need to support more DRBs than LTE.

Number of DRBs supported in NR is same as in LTE, that is 8.

# Conclusion

In section 2 we made the following observations:

Observation 1	Nature of QFIs & DRBs is dynamic rather than static, resulting that DRBs are released when services are no longer needed.

Observation 2	It may not be feasible to support all 5QI combinations. It should be clear what combinations may be supported by UE before increasing the number of DRBs.

Observation 3	Increasing the number of DRBs may result currently unforeseen problems

Observation 4	It may not be feasible to increase the number of DRBs in LTE due to existing UE hardware implementations.

Observation 5	Increasing the number of DRBs when NR connects to EPC (LTE+NR DC) does not seem necessary.

Observation 6	Even if the CN distinguishes many QoS Flows, the NR RAN may map them onto fewer DRBs, which indicates that NR does not need to support more DRBs than LTE.

Based on the discussion in section 2 we propose the following:

Proposal 1	LTE support for the number of DRBs is not changed.

Proposal 2	Number of DRBs is not changed in EN-DC case.

Proposal 3	Number of DRBs supported in NR is same as in LTE, that is 8.

# References

R2-1710245. LS on the number of bearers. SA WG2 Meeting #122bis.

S2-175383. Discussion on LTE/EPS bearer limitations. Telstra. 21 - 25 August 2017, Sophia Antipolis, France

NR and NG-RAN Overall Description, 3GPP TS 38.300, V1.0.0 (2017-09).

System Architecture for the 5G System, 3GPP TS 23.501, V1.3.0 (2017-09).