3GPP TSG-RAN WG2 #99bis	Tdoc R2-1711251

Prague, Czech Republic, 9th – 13th October 2017 	Updated from R2-1708350

Agenda Item:	10.3.1.8

Source:	Ericsson

Title:	Modelling of SPS/Grant Free Scheme in NR

Document for:	Discussion, Decision

# Introduction

In RAN2-98AH, there are the following agreement regarding modelling of SPS /grant free transmission:

Agreements

=>	Modelling in the MAC for grant-free will be discussed after the difference between the two schemes is better understood pending RAN1 progress.  RAN2 will aim to have a unified MAC operation for common functionalities between grant-free and UL SPS with understanding that there can be differences after input from RAN1.

=>	RAN2 understands that to support UL SPS similar to LTE a mode of operation in which RRC configuration (with no initial PHY resources) with L1 activation/deactivation needs to be supported.  RAN2 will continue discussion on UL SPS, with LTE functionality.

=>	A common RRC signalling can be design to allow the configuration of different UL transmissions schemes.

In RAN1-89AH [1], there are the following agreements, two types of SPS/grant free scheme are supported in NR.

- Type 1: UL data transmission without grant is only based on RRC (re)configuration without any L1 signalling.

For this type, the radio resources are granted via RRC signaling for a UE. The parameters such as period, offset, MCS, HARQ configurations and power control configuration (if any) are all conveyed via RRC signaling.

- Type 2: UL data transmission without grant is based on both RRC configuration and L1 signalling to activation/deactivation for UL data transmission without grant

For this type, some parameters such as period, offset for granted resource occurrence and some HARQ operation parameters are conveyed via RRC signaling while some other parameters such as MCS are conveyed via L1 signaling. L1 signaling also acts as activation and deactivation signal for SPS operation.

Like SPS in legacy LTE, period, offset of transmission occasions and RNTI are configured by RRC for both Type 1 and type 2 SPS/grant free scheme.

Like in legacy LTE, time-frequency resources, MCS, are semi-statically configured

  - For Type 1 SPS/grant free, these parameters are conveyed via RRC signaling

  - For Type 2 SPS/grant free, these parameters are conveyed via L1 signaling

This paper investigates the modelling of SPS/grant free mechanism in NR. Compared to the last submission, this version further clarifies the proposals based on the available agreements from RAN1 and RAN2. It is concluded that RAN2 should maximize the commonality of two mechanisms to avoid unnecessary complexity for both standardization and implementation without clear benefit.

# Discussion

SPS/grant free in LTE and NR is mainly used to configure the UE semi-statically with a periodic resource allocation which ideally matches a periodic traffic pattern, where periods of approximately 20ms are typical (e.g. VoIP), or for URLLC type of services where the resource request procedure(s) incur an unwanted latency in receiving an UL grant.

In LTE, and in NR, “UL skipping” is used as for uplink scheduling as well as SPS/grant free-based scheduling. Thereby, the mandate for the UE to always utilize an UL grant – even if no UL data is available by a padding transmission – is removed. This way, the UE skips UL padding transmissions. The motivation for this feature on SPS was that the intervals between subsequent granted resources were reduced to very short intervals, and that too frequent redundant padding transmissions are thereby avoided.

It becomes evident that in NR, the SPS/grant free framework will be used for these two distinct use cases: low-latency uplink access on the one side with a minimum periodicity, and VoIP-like periodic traffic with relaxed periodicities of about 20ms.

For both Type 1 and Type 2 SPS/grant free, the essence is that the network grants a set of periodic resources for a UE. With these granted resources, the UE can transmit data when data comes. From this perspective, Type 1 and Type 2 SPS/grant free schemes should not be distinctly modelled. Consider the flexibility of Type 2 SPS/grant free for link adaptation, activation and deactivation, it is preferred to model SPS/grant free based on Type 2 SPS/grant free framework, wherein optional parameters for Type 1 SPS/grant free such as time-frequency resources, MCS can be included in RRC signaling IE as an extension.

Type 1 SPS/grant free scheme is modelled as a part of configurable options based on Type 2 SPS/grant free signaling framework.

In NR, a RRC_connected UE can be either in active or inactive state. For a UE in inactive state, the RRC context is kept in both UE and network; and the network/UE can reactivate the context (such as C-RNTI and DRB etc) when there comes a service for the UE. However, there is no benefit to still keep the allocated radio resources still effective after the UE transfers from active state to inactive state. There are the following disadvantages to allow an inactive UE to access the allocated resources during the UE in active state:

- The traffic status of an inactive UE is not known at the network, which means possible waste of radio resources.

- The network loses control of the allocated resources:

  - For congestion control when there is high traffic load from inactive UEs sharing the resources

  - For resource reallocation: e.g. adjustment of granted resource size, or reallocation of the allocated resource for other purposes.

Considering the infeasibility mentioned above, we propose:

The allocated resources for SPS/grant free transmission are released when a UE enters to RRC_inactive state from RRC_connected state.

# Conclusion

Based on the discussion in section 2 we make the following observations and propose the following:

- Like SPS in legacy LTE, period, offset of transmission occasions and RNTI are configured by RRC for both Type 1 and type 2 SPS/grant free scheme.

Like in legacy LTE, time-frequency resources, MCS, are semi-statically configured

  - For Type 1 SPS/grant free, these parameters are conveyed via RRC signaling

  - For Type 2 SPS/grant free, these parameters are conveyed via L1 signaling

Based the discussion in Section 2, we have the following proposals:

Proposal 1	Type 1 SPS/grant free scheme is modelled as a part of configurable options based on Type 2 SPS/grant free signaling framework.

Proposal 2	The allocated resources for SPS/grant free transmission are released when a UE enters to RRC_inactive state from RRC_active state.

# References

Chairman notes, 3GPP TSG RAN WG1 NR Ad-Hoc#2, Qingdao, P.R. China, 27th – 30th June 2017