3GPP TSG-RAN WG2 #99Bis	R2-1711358

Prague, Czech Republic, 9th – 13th October 2017

Agenda Item:     10.2.17

Source:	Ericsson

Title:	Text proposal for 38.305 skeleton

Document for:	Discussion and Approval

# Introduction

At RAN2#99, which was the first positioning session for NR Rel.15 [1], the following two proposals were agreed:

- Positioning network elements (UE, gNB, AMF, LMF) in the positioning related architecture in TS 23.501 [2] should be used as the basis for positioning support for NG-RAN in Release 15.

- TS 38.305 will define stage 2 positioning for NG-RAN for both NR and eLTE access.

The 38.305 skeleton in [2] was endorsed as the baseline draft. The following contribution is some text proposal suggestion for this specification. As it has been suggested by the Editor’s note in the skeleton, the intention is to model each section with respect to the structure already available in 36.305. There has been already agreements from the previous meeting on positioning methods, therefore, the text proposal here is given for Section 4 and has track changes accordingly to propose changes for NG-RAN.

# Text Proposal for Section 4 in 38.305

# Main concepts and requirements

## Assumptions and Generalities

Editor’s Note: The intention is that this section is modeled on section 4.1 in 36.305 with changes for NG-RAN.

The stage 1 description of LCS at the service level is provided in [3]; the stage 2 LCS functional description, including the LCS system architecture and message flows, is provided in [2].

Positioning functionality provides a means to determine the geographic position and/or velocity of the UE based on measuring radio signals.  The position information may be requested by and reported to a client (e.g., an application) associated with the UE, or by a client within or attached to the core network. The position information shall be reported in standard formats, such as those for cell-based or geographical co-ordinates, together with the estimated errors (uncertainty) of the position and velocity of the UE and, if available, the positioning method (or the list of the methods) used to obtain the position estimate.

Restrictions on the geographic shape encoded within the 'position information' parameter may exist for certain LCS client types.  The NG-Core, including NG-RAN, shall comply with any shape restrictions defined in NR and, in a particular country, with any shape restrictions defined for a specific LCS client type in relevant national standards. For example, in the US, national standard J-STD-036-B restricts the geographic shape for an emergency services LCS client to minimally either an "ellipsoid point" or an "ellipsoid point with uncertainty circle and confidence" as defined in [4].

It shall be possible for the majority of the UEs (active or inactive) within a network to use the LCS feature without compromising the radio transmission or signalling capabilities of the NG-RAN.

The uncertainty of the position measurement shall be network-implementation-dependent, at the choice of the network operator. The uncertainty may vary between networks as well as from one area within a network to another. The uncertainty may be less than hundred metres in some areas and only a few metres in others. In the event, that a particular position measurement is provided through a UE-assisted process, the uncertainty may also depend on the capabilities of the UE. In some jurisdictions, there is a regulatory requirement for location service accuracy that is part of an emergency service. Further details of the accuracy requirements can be found in [3].

The uncertainty of the position information is dependent on the method used, the position of the UE within the coverage area and the activity of the UE. Several design options of the NG-RAN system (e.g., size of cell, adaptive antenna technique, pathloss estimation, timing accuracy, eNode B surveys) shall allow the network operator to choose a suitable and cost-effective UE positioning method for their market.

There are many different possible uses for the positioning information. The positioning functions may be used internally by the NG-Core, by value-added network services, by the UE itself or through the network, and by "third party" services. The feature may also be used by an emergency service (which may be mandated or "value-added"), but the location service is not exclusively for emergencies.

Design of the 5G-RAN positioning capability as documented in this specification includes position methods, protocols and procedures that are either adapted from capabilities already supported for E-UTRAN, UTRAN and GERAN, or created separately from first principles.

As a basis for the operation of UE Positioning in 5G-RAN, the following assumptions apply:

-	both TDD and FDD will be supported;

-	the provision of the UE Positioning function in 5G-RAN and 5G-Core is optional through support of the specified method(s) in the gNodeB and the LMF;

-	UE Positioning is applicable to any target UE, whether or not the UE supports LCS, but with restrictions on the use of certain positioning methods depending on UE capability;

-	the positioning information may be used for internal system operations to improve system performance;

-	the UE Positioning architecture and functions shall include the option to accommodate several techniques of measurement and processing to ensure evolution to follow changing service requirements and to take advantage of advancing technology;

-

## Role of UE Positioning Methods

Editor’s Note: The intention is that this section is modeled on section 4.2 in 36.305 with changes for NG-RAN.

The 5G-RAN may utilise one or more positioning methods in order to determine the position of a UE.

Positioning the UE involves two main steps:

-	signal measurements; and

-	Position estimate and optional velocity computation based on the measurements.

The signal measurements may be made by the UE or the gNodeB. The basic signals measured for terrestrial position methods are typically the 5G radio transmissions; however, other methods may make use of other transmissions such as general radio navigation signals including those from Global Navigation Satellites Systems (GNSSs).

The positioning function should not be limited to a single method or measurement. That is, it should be capable of utilising other standard methods and measurements, as such methods and measurements are available and appropriate, to meet the required service needs of the location service client. This additional information could consist of readily available E-UTRAN and 5G-RAN measurements.

The position estimate computation may be made by the UE or by the LMF.

## Standard UE Positioning Methods

Editor’s Note: The intention is that this section is modeled on section 4.3 in 36.305 with changes foir NG-RAN.

The positioning methods described in 36.305, section 4.3, except uplink positioning, are all supported by capable 5G-RAN target devices. The set of supported positioning methods includes both RAT-dependent and RAT-independent positioning methods.

# References

RP-171485, Revised WID on New Radio Access Technology, June 2017.

R2-1709477, Skeleton for TS 38.305, Qualcomm.